Table of Contents

	Introduction	7

	Objective of the SOP Manual	7

	Scope	8

Core Values Reflected In SOP	8

Energy Consumption and Greenhouse Gas	9

	Introduction	9

	Purpose	9

	Scope	9

	Terminology Overview	9

	Procedure	10

	Technology & Equipment Selection Criteria	11

	Roles & Responsibilities	12

	Monitoring & Review	12

	Employee Engagement	13

	Review & Amendment	13

Water Management	14

	Introduction	14

	Purpose	14

	Scope	14

	Terminology Reference	14

	Practices & Technologies	15

	Technology & Equipment Selection Criteria	16

	Roles & Responsibilities	16

	Monitoring & Review	17

	Employee Engagement	17

	Review & Amendment	17

Air Pollution Prevention	18

	Terminology	19

	Air Pollutants & Contribution	19

	Roles & Responsibility	20

	Control Measures	20

	Periodical Analysis	20

	Technology Implemented	21

	Workflow	22

	Documentation & Reporting	23

	Review & Amendment	23

Material & Chemicals Management	24

	Introduction	24

	Scope	24

	Definition	24

	Reduction of Material Consumption	24

	Recovered & Eco-Friendly Input Materials	25

	Handling Hazardous Substance	25

	Monitoring & Audit Measures	26

	Continuous Improvement	26

	Review & Amendment	26

Waste Management	27

	Introduction	27

	Scope	27

	Terminology	27

	Waste Generated	27

	Roles & Responsibilities	28

	Waste Reduction & Recovery Measures	28

	Sorting & Disposal Protocol	29

	Waste Stream Mapping	30

	Traceability System	30

	Monitoring and Reporting	31

	Safe & Responsible Disposal	31

	Roles & Responsibility	31

	Key Metrics for Success	32

	Review Mechanism	32

Employee Health & Safety	33

	Purpose	33

	Scope	33

	Definitions	33

	Roles and Responsibilities	33

	Workplace Safety & Working Conditions	33

	Occupational Health and Wellness Initiatives	35

	Emergency Preparedness and Response	35

	Audit, Monitoring, and Compliance	35

	Review and Continuous Improvement	35

Career Management & Training	36

	Purpose	36

	Scope	36

	Objectives	36

	Terminology	36

	Roles and Responsibilities	37

	Career Management Framework	37

	Training and Development Framework	37

	Monitoring and Review	38

	 Key Performance Indicators (KPIs)	39

	Flowchart – Career Management and Training Process	39

	Continuous Improvement & Review	40

Employee and Buyer Performance Review	41

	Purpose	41

	Scope	41

	Objectives	41

	Terminology	41

	Roles and Responsibilities	41

	Employee Performance Review Process	42

	Buyer/Vendor Performance Review Process	42

	Documentation and Record-Keeping	43

	Monitoring and Improvement	43

	Review Cycle	43

Anti-Corruption	44

	Purpose	44

	Scope	44

	Terminology	44

	Responsibilities	44

	Operational Guidelines	45

	Handling Transactions	46

	Disciplinary Action	47

	Audits and Monitoring	47

	Review and Updates	47

Third-Party Ethics & Integrity Due Diligence Program	48

	Purpose	48

	Scope	48

	Key Ethics Risks Covered	48

	Risk-Based Approach	48

	Data Collection Requirements	49

	Due Diligence Process	49

	On-Site ESG	49

	Red Flags and Escalation Criteria	50

	Review & Improvement	50

Incident Response Plan (IRP) – Confidential Information Breaches	51

	Purpose	51

	Scope	51

	Key Definitions	51

	Roles & Responsibilities	51

	Incident Response Lifecycle	52

	Prevention & Awareness	53

	Documentation & Recordkeeping	53

	Escalation Matrix	53

	Review & Continuous Improvement	53

Responsible Information Management	54

	Purpose	54

	Scope	54

	Terminology	54

	Information Classification	54

	Responsibility Matrix	55

	Information Lifecycle	55

	Secure Storage Guidelines	55

	Access Control	56

	Data Sharing & Transfer Protocols	56

	Data Retention	56

	Secure Disposal Methods	56

	Continuous Audits and Monitoring	57

	SOP Review	57

Internal Audits of Ethics & Compliance Control Procedures	58

	Purpose	58

	Scope	58

	Accountable Entities	58

	Audit Methodology & Tools	58

	Documentation & Recordkeeping	59

	Reporting & Corrective Action	59

	Review & Continuous Improvement	59

SOP Manual Review Mechanism	60

	Frequency:	60

	Review Committee Composition:	60

	Review History	60

# Introduction

At Bedmutha Industries, we are dedicated to delivering high-performance engineering and manufacturing solutions with a focus on precision, safety, and sustainable growth. This Standard Operating Procedure (SOP) Manual outlines the framework for standardized operations across our facilities. It serves as a reference for maintaining streamlined workflows, adhering to industry benchmarks, and ensuring every team member operates with clarity and purpose. This document reinforces our core philosophy—structured execution leads to consistent excellence.

# Objective of the SOP Manual

This SOP Manual is developed with the following goals:

- To provide a unified approach for operational procedures across all units.

- To ensure strict adherence to national and international regulatory requirements.

- To mitigate risks related to safety, quality, and operational disruptions.

- To encourage a proactive and transparent work culture.

This manual is designed as a functional tool to support employees, supervisors, and stakeholders in aligning with Bedmutha Industries’ principles. It also adapts to changes in regulations, technology, and market expectations, offering a flexible yet structured system for continuous growth and reliability.

Below are the key outcomes expected from SOP implementation:

| Benefit Areas | Description |
| --- | --- |
| Operational Efficiency | Reduces Variability and streamlines Process. |
| Compliance Management | Ensures Alignment with Regulatory and legal standards. |
| Quality Assurance | Maintains Consistent Product and Service Quality. |
| Risk Mitigation | Lowers the likelihood of errors and workplace incidents. |
| Staff Empowerment | Equips employees with clear instructions and responsibilities. |

# Scope

This manual applies to all internal staff, external vendors, service providers, and affiliated teams working across Bedmutha Industries' operations. It encompasses activities related to manufacturing, quality control, equipment handling, logistics, maintenance, and health & safety protocols. The SOP Manual is a living document and will be updated periodically to remain aligned with technological advancements and organizational objectives.

| Facility | Address |
| --- | --- |
| Manufacturing Plant | Plot No. E-1, Nardana Central Government Industrial Growth Centre, Phase - II, Waghadi-Khurd, Shindhkheda, Dist - Dhule - 424309, Maharastra, India. |

# Core Values Reflected In SOP

Honesty & Accountability

Innovation and Process Optimization

Workforce Well-being and Respect

Environmental Sustainability

Customer Commitment

| Title: | # Energy Consumption and Greenhouse Gas |
| --- | --- |
| SOP No.: | ENR-GHG-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | Sustainability & Operations Team |
| Approved By: | Plant Head |
| At Bedmutha, every watt saved is a step toward a cleaner tomorrow. |  |

## Introduction

At Bedmutha Industries Limited, sustainability is not just a goal but a strategic business imperative. This Standard Operating Procedure (SOP) outlines our integrated approach to improving energy efficiency, optimizing resource utilization, and reducing greenhouse gas (GHG) emissions across all manufacturing and operational facilities. As a leading manufacturer of industrial wire products, we recognize our responsibility to mitigate environmental impact through proactive measures and sustainable innovations.

## Purpose

The primary objectives of this SOP are to:

- Standardize procedures to enhance energy efficiency and reduce GHG emissions.

- Drive adoption of low-carbon technologies and operational best practices.

- Comply with national and international environmental regulations.

- Establish performance benchmarks and foster a culture of sustainability.

## Scope

This SOP applies to all manufacturing units, warehouses, utilities, and administrative offices operated by Bedmutha Industries Limited. It is applicable to equipment operators, facility managers, engineering personnel, maintenance teams, procurement departments, and third-party contractors engaged in energy-related activities.

## Terminology Overview

| Term | Definition |
| --- | --- |
| GHG | GHG is acronym for Greenhouse Gases. These are gases that trap heat in the atmosphere, contributing to global warming. Key GHGs include carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), and fluorinated gases. |
| CCS | CCS is acronym for Carbon Capture & Storage which is a technology designed to capture carbon dioxide emissions from industrial sources and store it underground to prevent it from entering the atmosphere. |
| CHP | CHP means Combined Heat & Power. It is a system that simultaneously generates usable heat and electricity from a single energy source, increasing overall efficiency. |
| VFD | VFD is acronym for Variable frequency Drive. It is an electronic device that controls the speed and torque of electric motors by varying input frequency and voltage, leading to significant energy savings. |

## Procedure

- Energy Efficiency Improvement

- Conduct detailed energy monitoring to assess equipment performance.

- Identify inefficient motors, compressors, HVAC systems, and lighting.

- Replace outdated systems with high-efficiency models.

- Optimize processes through automation and sensor integration.

- Implement energy management software for real-time monitoring and predictive maintenance.

- Benchmark results before and after equipment upgrades to validate efficiency gains.

Equipment Upgrade & Impact

| Area | Existing Equipment | Proposed Upgrade | Expected Energy Savings |
| --- | --- | --- | --- |
| Lighting | Halogen Bulbs | LED Fixtures | 40-60% |
| Motors | IE1 Motors | IE4 Motors + VFD | 15-20% |
| Air Compression | Fixed-Speed Compressors | VFD Compressors | 18-22% |
| HVAC | Split AC Units | Inverter AC or Central HVAC | 25-30% |
| Pumps | Constant-Speed Pumps | VFD-Controlled Pumps | 20% |

- Waste Heat Recovery System

- Identify high-temperature exhaust systems (furnaces, boilers).

- Install heat exchangers to capture waste heat.

- Reuse captured heat for preheating feedwater or space heating.

- Evaluate feasibility of CHP units for facilities with thermal + electrical load.

- Monitor thermal efficiency post-installation using digital tools.

Flowchart: Waste Heat Recovery Process

[Heat Source] → [Heat Exchanger] → [Steam/Hot Air Generation] → [Use in Boilers or Cleaning Systems]

Benefits:

- Reduces fuel consumption.

- Lowers operational costs.

- Minimizes thermal losses.

- Improves system resilience and efficiency

- Carbon Reduction for Efficiency

- Analyze current fuel usage patterns and carbon intensity.

- Explore cleaner alternatives like PNG, biomass briquettes, and solar energy.

- Replace diesel-based systems with hybrid or renewable-powered units.

- Monitor emissions post-implementation using calibrated gas analyzers.

- Collaborate with fuel suppliers for consistent supply of cleaner fuels.

Outcomes:

- Significant reduction in direct CO2 emissions.

- Strengthens compliance with long-term climate goals.

## Technology & Equipment Selection Criteria

The selection of technology and equipment plays a crucial role in the success of energy and GHG management strategies. Bedmutha Industries follows a rigorous evaluation approach to ensure only the most effective and sustainable options are adopted.

Selection Considerations Include:

- Energy Efficiency:

- Preference is given to equipment with high energy performance ratings (e.g., IE3, IE4 motors, 5-star rated systems).

- Evaluation of Specific Energy Consumption (SEC) benchmarks.

- GHG Reduction Potential:

- Focus on technologies with measurable impact on carbon emissions.

- Preference for low-carbon alternatives such as electric vehicles and induction heating systems.

- Certifications and Standards:

- Equipment must meet or exceed ISO, BIS, CE, BEE, or equivalent standards.

- Preference for vendors offering certified Environmental Product Declarations (EPDs).

- Digital and Smart Integration:

- Compatibility with SCADA, EMS, IoT sensors, and AI forecasting tools.

- Support for cloud-based monitoring, automation, and performance tracking.

- Reliability and Maintenance:

- Preference for proven technologies with established track records.

- Availability of local service centers and preventive maintenance plans.

- Vendor Selection and Transparency:

- Evaluation of vendor reputation, technical expertise, and sustainability practices.

- Preference for vendors offering post-installation support, training, and documentation.

This structured selection process ensures that every investment contributes to long-term operational excellence and environmental sustainability.

## Roles & Responsibilities

| Energy Manager | - Conduct periodic energy monitoring to identify areas of improvement. - Prepare and maintain detailed energy performance reports. - Recommend and support the implementation of efficiency improvement measures. |
| --- | --- |
| Plant Head | - Approve capital investment for energy-saving technologies. - Allocate manpower and resources to sustainability initiatives. - Oversee the effectiveness of implemented measures and track progress. |
| Maintenance Team | - Ensure timely maintenance of energy-related systems. - Calibrate instruments and systems for optimal energy performance. - Replace and upgrade inefficient components. |
| HSE Officer | - Ensure adherence to environmental regulations and GHG guidelines. - Document emission data and report discrepancies. - Conduct internal audits and safety inspections linked to energy systems. |
| Procurement Team | - Manage contracts with a focus on sustainability and energy performance. - Source high-efficiency and certified energy equipment. - Evaluate vendors based on environmental criteria. |

## Monitoring & Review

| Metric | Frequency | Responsible Team |
| --- | --- | --- |
| Energy Consumption | Annually | Sustainability Team |
| GHG Emissions | Annually | EHS & Sustainability Team |
| Equipment Efficiency | Annually | Maintenance Team |
| Smart Meter Data Reporting | Annually | Data Analytics Team |
| KPI Scorecard Review | Annually | Sustainability + HR |

## Employee Engagement

To foster greater employee participation, the following initiatives have been introduced:

1. Awareness and Capacity Building

- Training & Development: All new and existing employees undergo training sessions on energy-efficient practices, GHG reduction goals, and usage of smart systems.

- Energy SOP Workshops: Special sessions are held annually to communicate changes or enhancements to the energy policy and SOP.

- Onboarding Briefings: Sustainability and energy-saving guidelines are introduced during induction for all new recruits.

2. Participation Programs

- Green Suggestion Box: Staff can submit practical ideas for energy savings. Winning suggestions are evaluated for pilot implementation.

3. Recognition and Motivation

- Gamification and Rewards: Points-based systems encourage employees to take part in challenges like “Switch-Off Hours,” “Green Desk Week,” or “Low Carbon Commutes.” Winners receive certificates and recognition.

- Sustainability Awards: Annual awards are presented to departments or individuals who significantly contribute to energy reduction efforts.

4. Engagement Platforms

- Sustainability Day: A dedicated day each year to celebrate green milestones, share project outcomes, and encourage open dialogue on improvements.

- Awareness Boards & Dashboards: Digital screens and noticeboards at plant locations display energy KPIs, live savings stats, and ongoing campaigns.

## Review & Amendment

This SOP is reviewed:

- Annually, or

- After any regulatory change, or

- If there is a change in equipment, process, or plant layout.

| Title: | # Water Management |
| --- | --- |
| SOP No.: | WTR-MGT-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | Environmental & Sustainability Department |
| Approved By: | Plant Head |
| At Bedmutha, every drop preserved paves the way for a sustainable tomorrow. |  |

## Introduction

Water is a foundational element of industrial operations. At Bedmutha Industries Limited, we recognize our responsibility to manage water efficiently and sustainably. This SOP provides a holistic roadmap for conserving water, promoting reuse, and integrating modern water management techniques across all operations. The initiative supports regulatory compliance and aligns with our commitment to ISO 14001, Zero Liquid Discharge (ZLD) standards and long-term sustainability objectives.

## Purpose

The primary objectives of this SOP are to:

- Optimize water use efficiency through modern technological interventions.

- Strengthen recycling practices to reduce freshwater dependency.

- Minimize water pollution by implementing effective wastewater management.

- Encourage a culture of water conservation at all organizational levels.

- Comply with national and international water regulations.

## Scope

This SOP is applicable to all departments, plants, facilities, and external service providers of Bedmutha Industries Limited that utilize water in their operations. It includes production lines, utilities, sanitation, HVAC systems, landscaping, and administrative areas.

## Terminology Reference

| Term | Description |
| --- | --- |
| Recycled Water | Treated process water reused for secondary applications like cooling and cleaning. |
| Rainwater Harvesting | Collecting and storing rooftop rainwater for internal non-potable use. |
| Greywater | Lightly contaminated water from wash basins, cleaning, and rinsing that can be treated and reused. |
| Closed-loop Cooling | A cooling system that recirculates the same water, reducing fresh water intake. |
| Quenching | Rapid cooling of metal products using water to increase hardness. |
| Deburring | Process to remove sharp edges or burrs from metal parts, often using water jets or slurry-based systems. |

## Practices & Technologies

  - Water Recycling Technology

- Effluent Treatment Plant (ETP): Designed to treat and detoxify industrial effluent before reuse or discharge.

- Reverse Osmosis (RO): Removes dissolved solids, ideal for reusing rinse and greywater.

- Membrane Bioreactor (MBR): Combines biological treatment and membrane filtration for high-quality treated water.

Reuse Applications

| Source | Technology Used | Application |
| --- | --- | --- |
| Rinse Water | RO | Cooling Towers, Gardening |
| Greywater | MBR | Equipment Cleaning, Flushing |
| Effluent | ETP + Sand Filter | Process Reuse, Floor Cleaning |

  - Efficient Cooling Systems

Efficient cooling mechanisms reduce water wastage and operating costs.

- Closed-loop systems recirculate cooling water to reduce evaporation losses.

- Air-cooled chillers are deployed where humidity and ambient conditions permit.

- Automated controls (float valves, timers) prevent overflow and leaks.

Flowchart: Closed-Loop Cooling Water Flow

[Chiller Unit] → [Distribution Line] → [Cooling Process] → [Return Pipe] → [Holding Tank] → [Reused]

  - Optimisation in Cleaning

Surface preparation and cleaning processes are optimized by:

- Implementing dry methods like air jet cleaning before water-based cleaning.

- Using low-toxicity, biodegradable degreasers that need fewer rinse cycles.

- Installing nozzles with flow restrictors to maintain pressure with less water.

Checklist for Opeartors:

- Dry cleaning applied before rinsing.

- Eco-friendly degreasing chemical used.

- Final rinse water sourced from greywater treatment.

  - Rainwater Harvesting System

Rainwater is an excellent alternative to reduce dependence on groundwater.

- Collection from rooftop gutters into storage tanks.

- First flush systems prevent entry of dirt and debris.

- Filtered water is used for toilets, landscaping, or sent to ETP for processing.

Illustration: Rainwater Harvesting Process

[Roof] → [Gutter] → [First Flush System] → [Storage Tank] → [Filtration] → [Non-potable Use]

  - Control of Wastewater from Finishing Operations

Finishing activities like quenching and deburring produce water waste that must be treated.

- Oil-water separators extract oils from quenching fluids.

- Settling tanks allow removal of metallic particulates from deburring wastewater.

- Reclaimed water is recycled after treatment for reuse in compatible processes.

| Process | Contaminants | Treatment | Reuse Option |
| --- | --- | --- | --- |
| Quenching | Oil & Grease | Separator | Floor Cleaning |
| Deburring | Fine Metal Particles | Sedimentation + Filtration | Equipment Washing |

## Technology & Equipment Selection Criteria

The selection of water conservation technologies and equipment is pivotal to the success of sustainable water management strategies. Bedmutha Industries adopts a structured and performance-driven evaluation process to ensure the integration of solutions that maximize reuse, minimize consumption, and align with long-term environmental goals.

- Water Efficiency: Preference for systems with lower L/unit ratings.

- Ease of Maintenance: Availability of spares and technical support.

- Lifecycle Cost: CAPEX vs OPEX for optimal ROI.

- Environmental Compliance: Conformity with CGWA and CPCB norms.

- Smart Integration: IoT-enabled devices for monitoring and automation.

## Roles & Responsibilities

| Water Management Officer | - Monitor water consumption trends and identify inefficiencies. - Coordinate implementation of reuse and recycling strategies - Maintain documentation for audits and ensure regulatory compliance. |
| --- | --- |
| Engineering & Maintenance Team | - Inspect and maintain plumbing systems to prevent leakages. - Operate and service water treatment and recycling units. - Ensure automation systems (sensors, valves) are functional and calibrated |
| EHS Team | - Conduct awareness programs on water conservation. - Carry out internal water audits and inspections. - Recommend technical upgrades based on performance gaps. |
| Procurement Team | - Source water-efficient fixtures and technologies. - Evaluate vendor performance based on water management criteria. - Ensure environmental compliance of outsourced services |

## Monitoring & Review

| Metric | Frequency | Responsible Team |
| --- | --- | --- |
| Water Use Intensity | Annually | Sustainability Team |
| % Recycled Water | Annually | EHS & Sustainability Team |
| Leak Occurence | Annually | Maintenance Team |
| Rainwater Usage | Annually | Data Analytics Team |

## Employee Engagement

To foster greater employee participation, the following initiatives have been introduced:

1. Awareness and Empowerment Initiatives

- Immersive Training Modules: Mandatory training sessions are conducted for all employees to build awareness on water conservation goals, efficient cleaning practices, and appropriate responses to leakages or water-related incidents.

- Interactive SOP Briefings: Annually workshops are organized to familiarize teams with updates to the Water Management SOP, introduce new technologies like rainwater harvesting systems and filtration units, and address on-ground implementation challenges.

## Review & Amendment

This SOP is reviewed:

- Annually, or

- After any regulatory change, or

- If there is a change in equipment, process, or plant layout.

| Title: | # Air Pollution Prevention |
| --- | --- |
| SOP No.: | AP-PVT-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | Environmental & Sustainability Department |
| Approved By: | Plant Head |
| Breathing life into the future, one clean step at a time. |  |

- Introduction

Air pollution is one of the most critical environmental challenges faced by the manufacturing sector. At Bedmutha Industries Limited, we are committed to minimizing air pollutant emissions and ensuring compliance with national and international environmental regulations. This SOP outlines our approach to controlling, monitoring, and reducing air pollution from our operations.

- Objectives

- To establish a systematic approach for preventing and controlling air pollution.

- To ensure compliance with the Air (Prevention and Control of Pollution) Act, 1981, CPCB guidelines, and MoEF&CC standards.

- To implement best practices and innovative technologies to reduce emissions.

- To safeguard the health of employees and surrounding communities.

- Scope

## Terminology

| Term | Description |
| --- | --- |
| Particulate Matter | Microscopic solid or liquid particles suspended in air, such as dust, dirt, soot, or smoke. |
| VOC | Volatile Organic Compounds are organic chemicals that easily become vapors or gases. Common in solvents, paints, and fuels. |
| NOₓ / SO₂ | Nitrogen Oxides (NOₓ) and Sulfur Dioxide (SO₂) are combustion-related gases that contribute to smog and acid rain. |
| ESP | ESP is acronym for Electrostatic Precipitator which is an equipment that uses electrical charge to remove fine particles from gas streams. |
| CEMS | Continuous Emissions Monitoring Systems are automated systems that monitor pollutant levels in stack emissions. |
| Fugitive Emissions | Emissions not released through a stack or vent, such as dust from handling materials or leaks from equipment. |
| Biofilter | A pollution control technique using living material to capture and biologically degrade pollutants. |
| Ambient Air Quality Monitoring | The assessment of outdoor air quality by measuring key pollutants at fixed locations over time. |

## Air Pollutants & Contribution

| Source | Pollutants |
| --- | --- |
| Boilers & Furnaces | PM, SO₂, NOₓ, CO |
| Material Handling & Conveying | PM₁₀, PM₂.₅ |
| Transportation & Internal Traffic | CO, NOₓ, Dust |
| DG Sets & Utility Operations | CO, NOₓ, VOCs |
| Chemical Handling / VOC Emissions | VOCs, Odor |
| Waste & Wastewater Treatment Areas | VOCs, Odor |
| Miscellaneous Fugitive Emissions | PM, VOCs, Metals |

## Roles & Responsibility

| Role | Responsibility |
| --- | --- |
| EHS Manager | SOP implementation, monitoring & compliance |
| Maintenance Head | Ensure upkeep of pollution control devices |
| Operations Supervisors | Execution of preventive measures |
| Procurement Team | Source eco-friendly equipment and materials |
| Plant Employees | Report leaks, damages, or nuisances |

## Control Measures

- Process Emission Control

- Installation of Bag Filters: Captures fine particulate matter from furnaces and grinding operations.

- Electrostatic Precipitators (ESP): Used in large-scale furnaces to remove PM from flue gases.

- Wet Scrubbers: Installed for acid gas neutralization (SO₂, HCl).

- Low-NOx Burners: Installed in boilers to reduce NOx formation during combustion.

- Fugitive Dust Control

- Water Sprinkling Systems: Regular sprinkling on roads and raw material stockyards.

- Enclosed Conveyors: Material handling systems are enclosed to prevent dust escape.

- Dust Extraction Systems: Provided at key locations like bagging and grinding units.

| Action | Implementation |
| --- | --- |
| Use of Clean Fuels | DG Sets & Boilers |
| Covered Storage Area | Raw Material covered with Sheets |
| Green Belt & Tree Buffer | Around the Site |
| Real Time Emission Monitoring | CEMS Integrated |
| Compliance Audits | Internal & External Audits |

## Periodical Analysis

- Stack Emission Monitoring: Monthly basis via third-party NABL-accredited labs.

- Ambient Air Quality Monitoring: Carried out as per CPCB norms.

## Technology Implemented

- Noise & Odor Management

- Acoustic enclosures for DG sets.

- Anti-vibration pads for heavy machinery.

- Sound barriers at plant boundary.

- Maintenance and lubrication to reduce mechanical noise.

- Biofilters for VOC and odor removal from exhaust streams.

- Sealed wastewater treatment tanks.

- Use of odor-neutralizing agents in process units.

- Housekeeping SOPs to prevent accumulation of odor-producing waste.

- Green Belt Development & Carbon Sequestration

As part of our commitment to environmental sustainability and long-term carbon management, Bedmutha Industries Limited has implemented a robust green belt and afforestation program within and around the plant premises.

- Initiative Highlights:

- A total of ~2,000 trees have been planted around our facility

- Trees were planted as part of our initial site development over 10 years ago

- Species include Neem, Banyan, Peepal, Mango, Gulmohar, and Ashoka, known for high carbon absorption and air purification

- The plantation forms a natural buffer zone to control dust and absorb gaseous pollutants.

- Environmental Benefits:

- Estimated carbon sequestration potential: ~ 40–50 metric tons of CO₂ annuall.

- Supports local biodiversity and reduces the heat island effect in the industrial zone

- Enhances ambient air quality, particularly for PM and SO₂ capture

## Workflow

Raw Material Handling

↓

Primary Process (Melting, Annealing)

↓

Flue Gas Generation

↓

Air Pollution Control Devices (ESP, Bag Filter, Scrubber)

↓

CEMS Monitoring

↓

Compliance Reporting → EHS Dept. → SPCB/CPCB

## Documentation & Reporting

| Document | Frequency | Maintained By |
| --- | --- | --- |
| Stack Monitoring Report | Monthly | EHS Officer |
| Noise Measurement Data | Quarterly | EHS Coordinator |
| Odor/Complaint Logs | Ongoing | Plant Head |
| Equipment Calibration | Monthly | Maintenance Head |
| CEMS Data & Alerts | Real-time | Control Room |

## Review & Amendment

This SOP is reviewed:

- Annually, or

- After any regulatory change, or

- If there is a change in equipment, process, or plant layout.

| Title: | # Material & Chemicals Management |
| --- | --- |
| SOP No.: | MC-MGT-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | Operation & Safety Department |
| Approved By: | Operation Head |
| Every gram managed is a step toward a greener tomorrow. |  |

## Introduction

This SOP provides a comprehensive framework for the management of materials and chemicals used in Bedmutha Industries’ operations. It ensures safe handling, storage, usage, and disposal of raw materials, hazardous substances, and chemicals, in compliance with environmental and safety standards. This includes sustainable practices like material consumption reduction, eco-friendly substitutions, and hazardous substance minimization.

## Scope

This SOP applies to all plant facilities, operational departments, warehouses, laboratories, and logistics chains at Bedmutha Industries involved in procurement, transport, storage, processing, and disposal of materials and chemicals.

## Definition

| Term | Description |
| --- | --- |
| Hazardous Chemical | Substances that pose a risk to health, safety, or environment due to flammability, toxicity, or reactivity. |
| MSDS | Material Safety Data Sheet, a document outlining safety, health, and environmental hazards associated with chemicals. |
| POPs | Persistent Organic Pollutants that resist degradation and accumulate in ecosystems. |
| Green Chemistry | Design of chemical products/processes that reduce or eliminate hazardous substances. |
| Process Optimization | Strategies to reduce material and energy use while improving output. |

## Reduction of Material Consumption

Objective: Optimize processes to minimize the raw material input required per unit of production.

Measures:

- Lean Manufacturing principles integrated in production lines.

- Material flow analysis using digital twin simulations.

- Real-time monitoring of raw material consumption via ERP.

Flowchart: Material Optimization Loop

## Recovered & Eco-Friendly Input Materials

| Material | Source of Recovery | Reuse Method |
| --- | --- | --- |
| Copper Scrap | Drawing lines | Re-melted and reused |
| Flux Residue | Annealing units | Filtered and re-circulated |
| Spent Acid | Surface Treatment | Neutralized and regenerated |

- Use of biodegradable coolants and cutting fluids in machining.

- Replacement of solvent-based degreasers with water-based variants.

- Preference to suppliers offering REACH and RoHS certified chemicals.

- Acid Rinse Recovery System : Closed-loop reuse of phosphating rinse water reducing chemical load by 80%

## Handling Hazardous Substance

Storage:

- Corrosion-resistant racks and leak-proof containers.

- Temperature-controlled storage for flammable liquids.

- Color-coded segregation: Red (Flammable), Yellow (Reactive), Blue (Corrosive)

Labeling:

- GHS-compliant labels with pictograms, hazard codes, and first aid.

Transport:

- Use of certified containers with spill-proof lids.

- Transport documents include MSDS and emergency response guide.

Safe Disposal:

- Neutralization Units: On-site units treat acidic and basic waste.

- Incineration: For non-recyclable toxic materials via authorized third parties.

- Heavy Metal Capture: Effluent treatment plant uses chelating agents.

| Waste Type | Treatment Method | Final Disposal |
| --- | --- | --- |
| Spent Solvents | Distillation | Off-site reuse |
| Acidic Rinse Water | Neutralization | Treated water reused |
| Sludge with metals | Solidification | TSDF facility |

Toxicity Reduction

- Substitution of lead-based solder with tin-silver alloys.

- Low-VOC paints and coatings in manufacturing.

- Inert atmosphere in chemical processing to reduce secondary emissions.

Old: Lead Solder (for PCB Assembly)

New: Tin-Silver Alloy → Reduction: 100%

Old: VOC Paint (for coating processes)

New: Water-Based Paint → Reduction: 92%

Actions to Eliminate Persistent Organic Pollutants (POPs)

- No use of POPs listed under Stockholm Convention.

- Product and raw material screening to detect POP traces.

- Audit suppliers for POP-free certification.

## Monitoring & Audit Measures

- Digital inventory tools track chemical inflow/outflow.

- QR-coded containers for traceability.

- Bi-annual third-party chemical management audits.

## Continuous Improvement

- Feedback loop from floor operators to R&D.

- Annual revision of SOP based on audit results.

- Benchmarking against global metal industry leaders.

## Review & Amendment

This SOP is reviewed:

- Annually, or

- After any regulatory change, or

- If there is a change in equipment, process, or plant layout.

| Title: | # Waste Management |
| --- | --- |
| SOP No.: | WA-MGT-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | EHS & Operations Coordination Department |
| Approved By: | Plant Head |
| From Waste to Worth- Driving Sustainability |  |

## Introduction

To establish a structured waste management system that ensures minimization, proper segregation, safe handling, recycling, and disposal of waste at Bedmutha Industries. The SOP also aligns with sustainability goals and compliance with environmental regulations.

## Scope

Applicable across all operational departments, manufacturing units, warehouses, and administrative buildings of Bedmutha Industries where waste is generated, handled, or disposed.

## Terminology

- Waste Streams: Categories of waste generated during operations – solid, liquid, hazardous, recyclable, organic, etc.

- EPR: Extended Producer Responsibility.

- Segregation: The separation of waste into predefined categories at the source.

## Waste Generated

| Waste Type | Source | Example | Disposal Method |
| --- | --- | --- | --- |
| Recyclable Waste | Packaging, Office | Paper, Plastic, Metal | Sent to certified recyclers |
| Hazardous Waste | Manufacturing, Chemicals | Used oils, Solvents | Treated & sent to TSDF |
| E-Waste | IT Department | Obsolete Computers | Handed over to authorized recycler |
| Organic Waste | Canteen | Food waste | Composted or bio-digested |
| Scrap Metal | Production | Copper, Aluminum | Reused or sold to scrap dealers |

## Roles & Responsibilities

| Designation | Responsibilities |
| --- | --- |
| EHS Manager | Overall compliance, audits, and reporting |
| Unit Head | Implementation at unit level, reporting deviations |
| Workers & Supervisors | Proper segregation, following disposal practices |
| Waste Contractor | Collection, transport, and processing as per norms |

## Waste Reduction & Recovery Measures

Bedmutha Industries is committed to minimizing internal waste generation through the principles of source reduction, reuse, and resource recovery. These measures are integrated into operational workflows to promote a circular approach.

Key Strategies:

- Process Optimization: Use of Lean Manufacturing tools (e.g., Kaizen, 5S) to reduce raw material wastage during production.

- Material Reuse: Return of offcuts, defective items, and packaging back into the production line where feasible.

Byproduct Recovery:

- Metal scrap from wire manufacturing is collected and remelted.

- Sludge from electroplating units is filtered and stored for recycling via authorized vendors.

Packaging Minimization: Replacing multi-layer packaging with recyclable mono-material alternatives.

| Waste Type | Treatment Method | Final Disposal |
| --- | --- | --- |
| Copper wire scrap | Manufacturing Unit | Sent for remelting and reuse |
| Wooden pallets | Raw Material Stores | Repaired and reused for next shipments |
| Plastic spools | Cable Production Line | Collected and returned to supplier |

## Sorting & Disposal Protocol

Waste Streams:

| Waste Stream | Description | Examples |
| --- | --- | --- |
| Hazardous Waste | Waste with flammable, toxic, reactive, or corrosive properties | Used machine oil, spent acid, contaminated rags, degreasing solvents |
| Non-Hazardous Industrial Waste | Waste not posing direct health/environmental risk | Paper, cardboard, plastic packaging, rejected product parts |
| Biodegradable Waste | Organic material from canteens or landscaping | Food scraps, tea waste, garden trimmings |
| E-Waste | Obsolete electrical and electronic items | Old computers, printers, cables |
| Recyclable Scrap Metal | Reusable metal remnants from production | Copper offcuts, rejected wires, aluminum trimmings |

Sorting and Disposal Steps:

- Source Segregation

- Color-coded bins are placed at generation points (e.g., production floor, offices, canteen)

- Labels include waste type and disposal instructions

- Examples:

  - 🟥 Red bin – for hazardous waste like used oil containers

  - 🟩 Green bin – for organic canteen waste

  - 🟦 Blue bin – for paper and recyclable plastics

- Weighing & Logging

- Each shift supervisor logs waste quantity at the end of shift

- Example: 35 kg of copper scrap, 2 drums of spent acid.

- Internal Storage

- Hazardous waste is stored in secondary containment tanks or spill-proof containers

- Non-hazardous materials are sorted in designated storage zones

- Example: Flammable liquids stored in ventilated, explosion-proof cabinets

- Dispatch & Disposal

- Hazardous Waste → Sent to TSDF via authorized transporters with Form 10 manifest

- Scrap Metal → Collected and sent to approved recyclers

- E-Waste → Handover to CPCB-authorized e-waste handlers

- Canteen Waste → Sent to in-house bio-compost unit or municipal disposal service

## Waste Stream Mapping

## Traceability System

Bedmutha Industries has established a robust waste traceability framework to ensure end-to-end tracking of waste from generation to final disposal.

- Tracking Mechanism: A digital tracking system using barcode and RFID tagging is implemented for key waste streams such as metal scrap, lubricants, chemical containers, and packaging waste.

- Waste Mapping: Each generation point (e.g., wire drawing units, ETP, packaging stations) is mapped with corresponding waste streams, assigned unique IDs for traceability.

- Certified Partners: All disposal and recycling are carried out via authorized waste management partners, registered with the Pollution Control Boards (SPCB/CPCB).

- Documentation: Movement of hazardous waste is recorded using Form 10 manifests under the Hazardous Waste Management Rules, 2016.

## Monitoring and Reporting

- Monthly Reports: Waste movement, treatment, and disposal data are compiled into internal reports using ERP-based waste management software.

- Compliance Checks: Regular checks ensure that disposal aligns with SPCB/CPCB protocols.

- Audit Trail: A full audit trail is maintained for high-risk wastes (e.g., ETP sludge, used oils) including date, handler, vehicle details, and receiving party acknowledgments.

## Safe & Responsible Disposal

- Certified Disposal Vendors: Only licensed recyclers, TSDF facilities, and hazardous waste handlers are partnered with for final disposal.

- Categorized Disposal: Waste is sorted into categories—hazardous, recyclable, biodegradable, and general industrial waste—and treated accordingly.

- Storage Protocols: Temporary waste storage is done in designated, labeled, and ventilated enclosures to prevent spills, contamination, or unauthorized access.

## Roles & Responsibility

## Key Metrics for Success

## Review Mechanism

- Frequency: Conduct annual reviews or upon any process/materials change

- Review Team: Includes Waste Management Team, Environmental Compliance Lead, and Operations Head

| Title: | # Employee Health & Safety |
| --- | --- |
| SOP No.: | E-H&S-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | EHS & Operations Coordination Department |
| Approved By: | Plant Head |
| Every Shift Safe – Every Life Valued |  |

## Purpose

This SOP defines the policies, procedures, and guidelines for ensuring the health, safety, and well-being of all employees at Bedmutha Industries Limited. It aims to prevent workplace injuries and illnesses through proactive planning, risk assessment, employee training, and wellness initiatives, while ensuring full compliance with legal and regulatory frameworks.

## Scope

Applies to all employees, contractors, and third-party personnel operating at or visiting Bedmutha Industries’ facilities, including administrative offices, manufacturing units, workshops, warehouses, and logistics areas.

## Definitions

- EHS: Environment, Health, and Safety

- PPE: Personal Protective Equipment

- RSI: Repetitive Strain Injury

- MSDS: Material Safety Data Sheet

- OSH: Occupational Safety and Health

- TWA: Time-Weighted Average for exposure limits to noise or substances

- LOTO: Lockout-Tagout – energy isolation safety system

## Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Manager | Policy implementation, risk assessments, and audits |
| Department Heads | Oversee daily compliance and training enforcement |
| Safety Officers | Conduct toolbox talks, monitor practices, report incidents |
| Employees | Adhere to all safety procedures, wear PPE, report hazards |

## Workplace Safety & Working Conditions

Bedmutha Industries is committed to ensuring all work areas are designed, operated, and maintained in ways that reduce risk and enhance employee well-being.

  - Physical Safety

- Slip-resistant flooring and sufficient lighting throughout the facility

- Guardrails at elevated work zones, secured platforms for maintenance

- Fire safety systems including ABC extinguishers, alarms, and clearly marked evacuation routes

- Regular inspection and maintenance of safety-critical infrastructure.

- Proper aisle marking, lighting, and emergency signage throughout the plant.

  - Chemical and Hazardous Substance Exposure Control

- MSDS available at every chemical storage or use point

- Use of sealed containers, spillage trays, and chemical-resistant PPE

- Local fume extractors and general ventilation systems

- Substitution of hazardous materials with low-toxicity or water-based alternatives where feasible

- Real-time exposure monitoring in designated hazard zones

  - Noise Exposure Control

- Noise mapping conducted for all high-noise operations

- Areas above 85 dB(A) marked as “Hearing Protection Zones”

- Use of earplugs or earmuffs is mandatory in these zones

- Acoustic enclosures installed on generators, blowers, and compressors

- Vibration dampeners and proper equipment maintenance reduce secondary sound hazards

  - Repetitive Strain Injury (RSI) Prevention:

- Ergonomic tools and adjustable workstations installed at high-risk areas.

- Anti-fatigue mats provided for prolonged standing operations.

- Job rotation and micro-breaks implemented to reduce repetitive motion strain.

- Posture awareness visuals and RSI prevention guidance displayed at workstations.

- RSI cases tracked and reviewed for early intervention.

## Occupational Health and Wellness Initiatives

- Annual health check-ups including vision, hearing, and respiratory tests.

- Tie-ups with local hospitals for emergency care.

- Onsite medical room with first-aid trained personnel.

- Weekly yoga and mental wellness programs.

- Nutrition counseling and clean drinking water stations.

## Emergency Preparedness and Response

- Fire evacuation drills every 6 months.

- Emergency Response Team (ERT) in each department.

- Clearly marked muster points and evacuation maps.

- Training on CPR, first aid, and fire extinguisher use.

Flowchart: Emergency Response Protocol

## Audit, Monitoring, and Compliance

- Daily safety inspections by line supervisors.

- Monthly internal audits using standardized checklists.

- Non-compliance is escalated to the Operations Head.

- Integration with ISO 45001:2018 for occupational health & safety management.

## Review and Continuous Improvement

- This SOP is reviewed annually or post any major incident.

- Review board includes EHS Head, Operations Manager, and an external safety consultant.

- Employee feedback is used for continual refinement.

| Title: | # Career Management & Training |
| --- | --- |
| SOP No.: | CMT-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | HR Executive |
| Approved By: | Plant Head |
| Shaping Talent, Strengthening Tomorrow |  |

## Purpose

The purpose of this SOP is to establish a robust and structured system for career management and training to support employee growth, increase job satisfaction, and ensure the strategic development of skills aligned with the business goals of Bedmutha Industries Ltd. The SOP outlines the processes to identify employee potential, build competencies, and provide opportunities for career advancement while fostering a culture of continuous learning.

## Scope

This SOP is applicable to:

- All full-time and part-time employees at all hierarchical levels.

- All departments and functional areas across Bedmutha’s manufacturing and corporate units.

- All stages of employment, from onboarding to leadership roles.

## Objectives

- To systematically develop and upgrade employee skills.

- To align individual goals with organizational growth and performance.

- To identify and nurture high-potential talent for future leadership roles.

- To create structured career paths and individual development plans (IDPs).

- To ensure employees are equipped with current and future-ready competencies.

## Terminology

| Term | Definition |
| --- | --- |
| Individual Development Plan (IDP) | A tailored career and skills development plan. |
| Training Need Identification (TNI) | Process of identifying skill gaps. |
| L&D | Learning and Development department overseeing employee training. |
| HiPo | High Potential employee identified for future leadership roles. |

## Roles and Responsibilities

| Designation | Responsibilities |
| --- | --- |
| HR Department | Maintain training records, monitor career development programs, ensure SOP implementation. |
| L&D Manager | Design training modules, coordinate programs, evaluate training outcomes. |
| Department Heads | Identify team training needs, mentor team members, support IDPs. |
| Employees | Participate actively in training and development programs. |
| Top Management | Approve strategic plans, support talent identification and leadership development. |

## Career Management Framework

- Career Planning and IDPs

- Career planning sessions to be held annually.

- Each employee create an Individual Development Plan (IDP) with their manager.

- IDPs include:

  - Short- and long-term career goals.

  - Competency development roadmap.

  - Training and certification requirements.

- Talent Assessment and Mapping

- Annual Talent Review Process using performance and potential matrix (9-box grid).

- Identify HiPos for succession planning and fast-track programs.

- Employees will be mapped against potential roles and development readiness.

- Succession Planning

- Identify critical roles across departments.

- Maintain a succession chart with at least two potential successors per role.

- Design specialized grooming and mentoring for successors.

- Mentorship Program

- Assign experienced mentors to newly hired or promoted employees.

- Quarterly mentorship sessions tracked through feedback and HR systems.

- Focus on leadership, decision-making, and problem-solving skills.

## Training and Development Framework

- Training Needs Identification (TNI)

- Conduct annual TNI survey through feedback, appraisals, audits.

- Identify gaps in:

- Technical skills

- Soft skills

- Compliance and safety

- Managerial competencies

- Types of Training

| Category | Description | Frequency |
| --- | --- | --- |
| Induction | Orientation, safety protocols, company policies | On joining |
| Technical Skills | Machines, tools, production processes | Quarterly |
| Soft Skills | Communication, time management, conflict resolution | Bi-annually |
| Compliance | ESG, safety, labor law, ethical conduct | Annually or as needed |
| Leadership | Strategic thinking, team management | Annually (HiPo focused) |
| E-learning | Online self-paced learning modules | Ongoing |

- Training Methods

- Classroom training by internal/external experts.

- On-the-job training under supervisors.

- Digital platforms and LMS (Learning Management System).

- External seminars and workshops.

- Job rotations and stretch assignments.

- Evaluation of Training

## Monitoring and Review

- Monitoring Metrics

- Monthly review of training participation.

- Quarterly IDP tracking review.

- Annual career progression analysis.

- Audit and Reporting

- Internal audits by HR every six months.

- External audits (if required by certification bodies).

- Reports presented to senior leadership annually.

## Key Performance Indicators (KPIs)

## Flowchart – Career Management and Training Process

[New Employee]

↓

[Induction Training]

↓

[Skill Assessment & IDP Formulation]

↓

[Training Need Identification]

↓

[Training Delivery – Tech/Soft/Compliance]

↓

[Assessment and Feedback]

↓

[Performance Review & Career Discussion]

↓

[Update IDP / Recommend Promotion / Identify HiPo]

↓

[Succession Planning / Leadership Training]

## Continuous Improvement & Review

- Annual benchmarking of training programs against industry standards.

- Continuous feedback loop from employees and trainers.

- Introduce new training modules based on evolving technology, sustainability goals, and market trends.

- This SOP is reviewed annually or earlier if needed.

- All changes are approved by the HR Head and Top Management.

| Title: | # Employee and Buyer Performance Review |
| --- | --- |
| SOP No.: | CMT-002 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | HR Executive |
| Approved By: | Plant Head |
| Shaping Talent, Strengthening Tomorrow |  |

## Purpose

This SOP establishes a structured framework for reviewing employee and buyer performance to ensure continuous improvement, accountability, and alignment with organizational goals. The aim is to identify high-performing talent and strategic supplier partners while providing constructive feedback and action plans for development.

## Scope

This SOP is applicable to:

- All employees (staff, line workers, managers) across departments.

- All buyers/vendors/suppliers engaged in procurement or business partnerships.

## Objectives

- To evaluate and document employee performance consistently and fairly.

- To assess supplier/buyer performance against quality, delivery, and compliance expectations.

- To provide structured feedback for improvement and future planning.

- To link performance evaluation to career progression, training needs, or contract continuation.

## Terminology

| Term | Definition |
| --- | --- |
| KRA | Key Result Area – Targeted outcome for a role or responsibility |
| KPI | Key Performance Indicator – Measurable metrics to evaluate performance |
| Appraisal | Annual or biannual performance assessment of employees |
| Vendor Scorecard | A quantitative review of supplier/buyer performance |

## Roles and Responsibilities

| Designation | Responsibilities |
| --- | --- |
| HR Department | Oversee employee appraisal cycles and maintain records |
| Department Heads | Evaluate team members and support IDP updates |
| Procurement Team | Manage buyer evaluations, documentation, and performance tracking |
| Quality & Compliance | Provide feedback on vendor quality and sustainability adherence |
| Senior Management | Approve results of performance reviews and recommend promotions or vendor contracts |

## Employee Performance Review Process

- Frequency

- Conducted biannually (Mid-Year & Annual Review)

- Forms used: “Employee Performance Evaluation Form” covering KRA, punctuality, teamwork, initiative, communication, etc.

- Evaluation Parameters

- KRA achievement (target vs. actual performance)

- Quality of work, productivity, and innovation

- Discipline, behavior, punctuality

- Teamwork and leadership potential

- Compliance with safety, EHS, and company policies.

- Process Flow

- Self-evaluation by employee

- Manager evaluation and discussion

- Review by HR

- Performance score finalization

- Recommendations: IDP update, promotion, training need, or PIP (Performance Improvement Plan).

## Buyer/Vendor Performance Review Process

- Frequency

- Conducted annually or as required by project cycles.

- Evaluation Form

- Buyer Performance Evaluation Form includes:

  - Quality of material supplied

  - On-time delivery

  - Commercial terms compliance

  - Responsiveness and communication

  - Legal and ethical compliance

- Review Criteria & Scoring

| Criteria | Weightage (%) | Minimum Acceptable Score |
| --- | --- | --- |
| Product/Service Quality | 25% | ≥70% |
| Delivery Timeliness | 20% | ≥80% |
| Communication | 15% | ≥70% |
| Pricing/Cost Effectiveness | 20% | As per agreed terms |
| Ethical & ESG Compliance | 20% | 100% mandatory |

- Action Based on Score

- ≥85%: Preferred Vendor

- 70–84%: Approved Vendor with improvement areas

- <70%: Put on Watchlist / Not Considered for New Contracts

## Documentation and Record-Keeping

- All appraisal and vendor review forms will be stored digitally via the HRMS and Vendor Management System.

- A copy will be shared with the respective employee/buyer.

- Summary reports to be reviewed in quarterly HR or procurement reviews.

## Monitoring and Improvement

- Trends in low-scoring employees or vendors will be analyzed.

- Corrective actions include training plans, contract renegotiation, or disengagement.

- Continuous feedback mechanisms (360°, customer satisfaction, etc.) will support improvements.

## Review Cycle

- SOP to be reviewed annually.

- Changes based on company policies, strategic direction, or audit recommendations.

- Amendments approved by HR Director and Procurement Head.

| Title: | # Anti-Corruption |
| --- | --- |
| SOP No.: | AC-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | Compliance Officer |
| Approved By: | Plant Head |
| Building Trust through Ethical Actions |  |

## Purpose

This SOP outlines Bedmutha Industries Ltd.'s commitment to operating with integrity, transparency, and in compliance with anti-corruption laws. It defines procedures and responsibilities to prevent, detect, and respond to any form of corruption, bribery, or unethical behavior in our business operations.

## Scope

This policy is applicable to:

- All employees, including full-time, part-time, contract, and temporary staff.

- Members of the Board of Directors and executive management.

- External stakeholders such as contractors, vendors, agents, consultants, and third-party intermediaries representing the company in any capacity.

## Terminology

- Bribery: Offering, promising, giving, accepting, or soliciting an undue advantage or anything of value with the intent to influence the actions of a person in a position of trust.

- Corruption: Abuse of entrusted power for private gain, including bribery, embezzlement, kickbacks, favoritism, and nepotism.

- Facilitation Payments: Small, unofficial payments made to expedite routine actions by officials. These are considered illegal and are strictly prohibited.

- Kickbacks: The return of a portion of money received, often secretly or illegally, in exchange for favorable treatment.

## Responsibilities

Employees

- Must comply with all anti-corruption policies and laws.

- Must report any suspected instances of corruption or unethical behavior.

- Must not accept or offer bribes in any form.

Managers/Supervisors

- Must set an example of ethical conduct.

- Must ensure employees under their supervision understand and follow this SOP.

- Must support investigations into policy breaches.

Compliance Department

- Responsible for policy implementation, updates, and training.

- Investigates all reported incidents and recommends action.

- Maintains logs of complaints, investigations, and actions taken.

Third Parties

- Must sign an agreement to follow this policy before commencing any work.

- Must not engage in any corrupt practices while working with the company.

- Subject to audits and monitoring for compliance.

## Operational Guidelines

- Gifts, Hospitality, and Entertainment

- Permissible only if:

  - Of nominal value.

  - Given transparently and occasionally.

  - Not intended to influence business decisions.

- Not permissible if:

  - Given in exchange for favorable treatment.

  - Excessive, frequent, or in the form of cash/gift cards.

  - Undocumented or intended to bypass company policies.

| Area | Clarification / Threshold |
| --- | --- |
| Gifts & Hospitality | Any gift above ₹5,000 in value (per instance or cumulative in one calendar year) is not permitted. Gifts below this value may be accepted only with prior written approval from HR. |
| Meals and Entertainment | Occasional business meals are acceptable if under ₹2,000 per person and not frequent. Lavish entertainment, personal trips, or expensive event tickets are prohibited. |
| Vendor Relations | Accepting personal favors, travel, or sponsorship from vendors under consideration for contracts is strictly prohibited, regardless of value. |
| Conflict of Interest | Example: An employee working in procurement must disclose if a family member owns or works in a supplier company — even if that company is not currently active. |
| Cash Gifts / Vouchers | Cash gifts, vouchers, or gift cards of any amount are not allowed. Even festive vouchers are treated as improper unless issued officially by the company. |

- Public Officials

- No bribes, unofficial payments, or gifts may be offered to officials.

- All interactions with officials must be recorded and approved by senior management.

- Charitable and Political Contributions

- Must be transparent, legal, and approved by senior management.

- Cannot be used as a cover for bribery or to gain any improper advantage.

- Vendor and Agent Due Diligence

- Before engaging a vendor/agent, Bedmutha perform due diligence to verify their integrity.

- Anti-corruption clauses must be included in all third-party agreements.

- Periodic monitoring and audits are mandatory.

- Recordkeeping

- All financial transactions must be recorded accurately and transparently.

- No use of off-the-books accounts or falsified records.

- All expenses, including gifts and hospitality, must be properly documented and approved.

## Handling Transactions

Step 1: Initiation of Transaction

- A transaction request must be initiated by an authorized employee.

- For purchases, a Purchase Requisition (PR) must be submitted via the internal procurement system, citing proper justification and budget approval.

Step 2: Approval Workflow

- The transaction go through multi-level approval:

  - Department Head approval

  - Budgetary control check

  - Finance Department vetting

Step 3: Vendor Verification

- Perform due diligence for all third parties involved:

  - Legal identity verification

  - Background checks for corruption or fraud history

  - Signed declaration of compliance with the Anti-Corruption SOP

- Ensure the vendor/client is onboarded in the official ERP system.

Step 4: Execution and Delivery

- Work only proceeds after formal approval and documentation.

- Monitor progress, delivery, and billing based on agreed milestones.

Step 5: Payment Authorization

- All payments must be:

  - Matched against purchase orders, invoices, and delivery receipts

  - Reviewed and verified by Accounts Payable and Finance

  - Made through traceable banking channels—no cash transactions

Step 6: Recordkeeping

- Maintain digital and physical copies of:

  - Invoices

  - Contracts

  - Approvals

  - Audit trails

- Records are stored securely for at least 7 years for audit purposes.

Step 7: Post-Transaction Review

- Finance and Internal Audit teams conduct random transaction audits annually.

- Any red flags or irregularities are reported immediately to the Compliance Officer.

## Disciplinary Action

Violations of this SOP result in:

- Disciplinary actions up to and including dismissal.

- Termination of business relationships with vendors/agents.

- Legal reporting to enforcement authorities.

- Financial penalties or reputational consequences.

## Audits and Monitoring

- Internal audits are conducted at least once a year.

- Surprise audits and forensic reviews are done in high-risk areas.

- External audits are conducted by third parties when necessary.

- All audit findings are reviewed by the Audit Committee and senior management.

## Review and Updates

This SOP is reviewed annually or when:

- Legal/regulatory changes occur.

- There are operational changes in business processes.

- Audit findings or incidents require policy revision.

Amendments are communicated to all stakeholders via emails, meetings.

| Title: | # Third-Party Ethics & Integrity Due Diligence Program |
| --- | --- |
| SOP No.: | AC-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | Compliance Officer |
| Approved By: | Plant Head |
| Building Trust through Ethical Actions |  |

## Purpose

This SOP defines a structured and risk-based framework to assess and monitor the ethical integrity of third parties associated with Bedmutha Industries Ltd. It aims to proactively prevent risks such as corruption, bribery, money laundering, information security breaches, and human rights violations through robust due diligence, audit mechanisms, and continuous monitoring.

## Scope

Applies to all third parties including:

- Raw material suppliers

- Vendors and service contractors

- Commission-based agents and consultants

- Distributors, brokers, customs agents

- Outsourced processing partners.

## Key Ethics Risks Covered

| Category | Examples of Risk |
| --- | --- |
| Corruption & Bribery | Kickbacks, facilitation payments, gifts in exchange for business |
| Information Security | Data breaches, unauthorized data sharing, lack of cyber hygiene |
| Money Laundering | Suspicious payments, shell companies, offshore banking |
| Fraud | Falsified documents, inflated invoices, forged quality certificates |
| Conflict of Interest | Undisclosed ties between employee and vendor |
| ESG Violations | Forced labor, discrimination, unsafe working conditions |

## Risk-Based Approach

a. Classification of Risk Level

| Risk Tier | Examples |
| --- | --- |
| High | Commission agents, customs brokers, third parties from high-risk countries |
| Medium | Raw material suppliers, long-term contractors |
| Low | Office supplies, one-time service providers |

b. Risk-Specific Actions

| Risk Area | Trigger | Action |
| --- | --- | --- |
| Corruption | Commission-based or high-influence roles | Full anti-bribery screening and policy signature |
| Money Laundering | Invoicing in foreign currencies or cash | Audit of payment chain + KYC documents |
| Information Security | Access to shared data or portals | Cyber due diligence checklist + NDA |

## Data Collection Requirements

All medium- and high-risk third parties must provide:

- Company registration documents

- Ownership structure and beneficial ownership declaration

- Anti-bribery and AML policy (if available)

- Information Security controls (firewall, password policy, encryption practices)

- PAN, GST, bank account verification

- Signed Code of Ethics & Confidentiality Declaration

## Due Diligence Process

| Step | Action |
| --- | --- |
| 1 | Initial screening (company checks, blacklist screening, risk classification) |
| 2 | Questionnaire: Anti-corruption, AML, data privacy, ESG compliance |
| 3 | Documentation review by Compliance Team |
| 4 | On-site audit on ESG |
| 5 | Scoring, benchmarking, and onboarding recommendation |

## On-Site ESG

Bedmutha conducts annual audits for high-risk suppliers, covering:

- Code of Conduct adherence

- Payroll transparency, no forced labor, EHS conditions

- Whistleblower mechanism availability

- Cyber hygiene (data access, password use, backup systems)

- Evidence of ethical awareness and anti-bribery training

Audit outcomes are benchmarked across vendors in the same category.

## Red Flags and Escalation Criteria

| Issue Detected | Escalation / Action |
| --- | --- |
| Refusal to sign ethics declaration | Block onboarding |
| Conflict of interest disclosure | Review by Legal and Compliance, potential recusal |
| Information security non-compliance | 30-day corrective action window, or disengagement |
| Financial red flags (AML concern) | Alert to Finance & Compliance Head, KYC verification required |
| Corruption risk or bribe offer | Immediate blacklisting and whistleblower case entry |

## Review & Improvement

- SOP is reviewed annually or after any ethics breach

- Internal audits ensure due diligence records are complete.

| Title: | # Incident Response Plan (IRP) – Confidential Information Breaches |
| --- | --- |
| SOP No.: | IS-IRP-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | Information Security Department |
| Approved By: | Head – Compliance |
| Protecting What Matters – Every Byte, Every File, Every Time |  |

## Purpose

This SOP outlines the procedure for detecting, reporting, responding to, and mitigating any breach or unauthorized access involving confidential information, including employee data, customer records, financial details, and proprietary operational data. The plan ensures quick containment and minimal impact, while protecting the interests of Bedmutha Industries Ltd and its stakeholders.

## Scope

Applies to all departments, locations, systems, and personnel (internal or external) who handle or access confidential and sensitive information, including:

  - Employee records

  - Customer/vendor data

  - Financial and legal information

  - Proprietary process, product, or R&D information

  - Network/system security credentials

## Key Definitions

| Term | Definition |
| --- | --- |
| Confidential Information | Any non-public information that can impact privacy, operations, or compliance |
| Security Incident | An attempted or successful unauthorized access, disclosure, or misuse of information |
| Breach | A confirmed event in which confidential information is lost, stolen, or exposed |

## Roles & Responsibilities

| Role | Responsibility |
| --- | --- |
| Incident Response Lead (IT Head) | Coordinates IRP activation, investigation, and technical response |
| Compliance Officer | Ensures regulatory reporting, legal review, and records of actions taken |
| HR/Department Head | Supports internal communication and corrective actions |
| Information Owner | Identifies impact of breach and supports recovery |

## Incident Response Lifecycle

1. Detection & Identification

• All employees must report suspicious activity or confirmed breach within 1 hour
• Notification to: incident@bedmutha.com 
• Examples: unusual login alerts, unauthorized data transfer, lost devices, phishing attack

2. Containment & Eradication

• IT isolates affected system/network/device
• Access rights are revoked, and firewalls updated
• Any malware is removed and logs are captured for evidence

3. Impact Assessment

• Identify type of data compromised (PII, business confidential, customer data)
• Evaluate scope: number of records, systems affected, potential harm

4. Communication

• Notify:

- Affected internal stakeholders (via email/memo)

- Regulatory bodies (if required by law – e.g., under IT Act or GDPR-like clauses)

- Customers/vendors (if relevant and under guidance of legal team)

- Communication is approved by the Director – Information Security & Compliance Officer

5. Recovery

• Restore systems from clean backups
• Change access credentials across affected endpoints
• Monitor post-incident for anomalies

6. Root Cause Analysis & Documentation

• Root cause analysis performed by IT + Compliance within 3 working days
• Report includes:

- Timeline of incident

- Actions taken

- Weaknesses exploited

- Corrective actions

7. Post-Incident Review

• Internal debrief session conducted within 7 days
• Lessons learned are applied to update this SOP, training programs, or IT policy

## Prevention & Awareness

• Quarterly employee training on data handling, phishing awareness, and reporting channels
• Access to confidential data is based on job role (least privilege principle)
• All USBs and external media are restricted
• MFA (Multi-Factor Authentication) is enabled across all cloud and email systems

## Documentation & Recordkeeping

All incident reports are logged in the Information Security Incident Register and include:

- Incident ID

- Time of detection

- Reported by

- Systems affected

- Type of data breached

- Actions taken

- Communication details

- Lessons learned

Retention period: 7 years

## Escalation Matrix

| Severity Level | Escalation Timeline | Action Owner |
| --- | --- | --- |
| Minor (limited impact) | Within 12 hours | IT Helpdesk + Dept. Supervisor |
| Moderate (system/data exposure) | Within 4 hours | IT Manager + Compliance Officer |
| Major (external breach or regulatory) | Immediate (1 hour) | Director – IS + Top Management |

## Review & Continuous Improvement

• This IRP SOP is reviewed annually or following any major breach
• Post-incident analysis directly informs:

- Firewall rule updates

- Access control adjustments

| Title: | # Responsible Information Management |
| --- | --- |
| SOP No.: | RIM-001 |
| Effective Date: | 01-01-25 |
| Review Date: |  |
| Prepared By: | Information Security Department |
| Approved By: | Plant Head |
| Protecting What Matters – Every Byte, Every File, Every Time |  |

## Purpose

The purpose of this SOP is to define the principles and procedures for managing information in a secure, ethical, and legally compliant manner. Information is a vital asset that, if mishandled, can lead to data breaches, financial loss, reputational damage, and legal implications.

## Scope

This SOP applies to all employees, vendors, contractors, consultants, and temporary workers who handle company data.

## Terminology

- Confidential Information: Non-public data including business strategies, employee records, customer/vendor information, R&D data, etc.

- Personal Data: Any information related to an identified or identifiable individual.

- Data Breach: Unauthorized access, disclosure, alteration, or destruction of information.

## Information Classification

To manage data effectively, Bedmutha Industries classifies data into four primary categories.

| Classification | Examples | Access Level | Control Measures |
| --- | --- | --- | --- |
| Public | Press releases, marketing brochures | Everyone | No restrictions |
| Internal Use Only | Internal newsletters, staff contact lists | All employees | Login-based access |
| Confidential | Project files, client agreements | Managers & teams involved | Password protection, limited sharing permissions |
| Highly Confidential | Financial reports, HR files, legal notices | Executive & authorized staff | Encryption, restricted access, audit trails |

## Responsibility Matrix

## Information Lifecycle

## Secure Storage Guidelines

A. Physical Records

- Use lockable cabinets.

- Limit key/card access to authorized personnel.

- Avoid storing important records near areas with fire or water risk.

B. Digital Records

- Use encrypted servers and secure cloud services.

- Backup critical systems daily, store backups offsite or on a secure cloud.

- Use anti-virus software and firewalls to prevent unauthorized access.

## Access Control

- Implement Role-Based Access Control (RBAC) and employees can only access what they need.

- Enable Multi-Factor Authentication (MFA) for sensitive systems.

- Regularly review and update access privileges, especially after role changes or exits.

- Use audit logs to track who accessed what data and when.

## Data Sharing & Transfer Protocols

Internal Sharing:

- Use approved platforms.

- Use read-only files where edits are not needed.

External Sharing:

- Sign NDAs before sharing sensitive data with clients.

- Transfer files through encrypted links or VPN.

- Do not send business data through personal emails or apps.

## Data Retention

Each type of data must be retained for a specified time as per compliance.

| Data Type | Retention Period | Responsible Dept. |
| --- | --- | --- |
| Employee Records | 7 years post separation | HR |
| Financial Reports | 8 years | Finance |
| Legal Contracts | 10 years or contract term | Legal |
| CCTV Footage | 30 days | Admin/Security |

## Secure Disposal Methods

Paper Records:

- Shred documents using cross-cut shredders.

- Keep shredding logs.

Digital Files:

- Use data-wiping software to erase content.

- Ensure old laptops are degaussed or destroyed before disposal.

Disposal Authorization:

- Must be approved by data owner + Compliance Officer.

- Maintain a Disposal Register for tracking.

## Continuous Audits and Monitoring

| Audit Type | Frequency | Lead Dept. |
| --- | --- | --- |
| System Security Audit | Quarterly | IT Department |
| Access Rights Review | Bi-annually | HR + Department Heads |
| Random Spot Checks | Monthly | Compliance |
| Third-party Review | Annually | External Consultant |

## SOP Review

- Reviewed annually or when new data laws are introduced.

- Changes are reviewed by IT, Legal, and Compliance team.

| Title: | # Internal Audits of Ethics & Compliance Control Procedures |
| --- | --- |
| SOP No.: | ETH-AUD-001 |
| Effective Date: | 01-01-24 |
| Review Date: | 01-01-25 |
| Prepared By: | Compliance & Internal Audit Team |
| Approved By: | Plant Head |

## Purpose

This SOP establishes the methodology for conducting internal audits to evaluate the effectiveness of Bedmutha Industries’ ethics and compliance control systems. These audits are designed to detect potential gaps, ensure continuous improvement, and support legal compliance and ethical conduct across all functions and third-party relationships.

## Scope

Audits are conducted to assess the following key areas

- Procurement and supply chain integrity

- Anti-bribery and corruption policy implementation

- Employee awareness and ethics training

- Regulatory compliance with national and global anti-corruption laws

- Whistleblower mechanism and non-retaliation practices

- Financial transparency and fraud controls

- Third-party risk management and screening

- Information security controls and data protection measures

## Accountable Entities

| Audit Focus Area | Responsible Entity |
| --- | --- |
| Policy Enforcement & Ethics Culture | Compliance Officer, HR Department |
| Third-Party Due Diligence | Procurement Head, Legal Advisor |
| Financial & Fraud Controls | Finance Controller |
| Information Security | IT Head & Data Privacy Officer |
| Audit Execution | Internal Audit Team (under Compliance Lead) |

## Audit Methodology & Tools

The audit uses a structured questionnaire-based assessment aligned with your Internal Assessment Form. This includes:

- 15+ key control questions to be answered as Yes / No

- Document review and system walkthroughs

- Interviews with department heads and key employees

- Sample testing of procurement decisions, training records, financial entries, and whistleblower logs

- Retrospective review of ethics policy effectiveness and compliance incidents

Employees involved in the audit must sign an undertaking confirming accurate disclosure.

## Documentation & Recordkeeping

- All completed forms, audit reports, and follow-up logs are stored in the Compliance Shared Drive

- Copies of final assessments are signed and acknowledged by Management

- Retention period: 7 years in line with audit and legal policy

## Reporting & Corrective Action

- Findings are categorized as: Observations, Minor Non-conformities, or Major Non-conformities

- A Corrective Action Plan (CAP) must be developed within 15 days for each finding

- High-risk or repeated issues are escalated to the Ethics Committee and Board Compliance Officer

- Follow-up is conducted within 30 to 60 days to verify closure

## Review & Continuous Improvement

- The Internal Assessment Form and audit checklist are updated annually

- Compliance team reviews trends from past audit cycles to inform training and policy updates

- Adjustments are made to improve scope, frequency, or accountability mechanisms

# SOP Manual Review Mechanism

To ensure this SOP remains relevant, up-to-date, and aligned with evolving business practices and regulatory requirements, Bedmutha Industries Limited has established an Annual SOP Manual Review Process, governed by the Compliance & IT Governance Teams.

## Frequency:

- This SOP is reviewed once every 12 months, or immediately after any critical breach, regulatory update, or major infrastructure change.

## Review Committee Composition:

| Designation | Role in SOP Review |
| --- | --- |
| IT Head / CISO | Technical accuracy and risk assessments |
| Compliance Officer | Legal/regulatory compliance |
| HR Representative (if applicable) | Data privacy and employee awareness |
| SOP Document Controller | Version control and archival |

## Review History

| Version | Effective Date | Revision Date | Changes Made | Approved By |
| --- | --- | --- | --- | --- |
| 00 | 01-01-2024 | – | Initial Release of Incident Response SOP | Plant Head |
| 01 | 01-01-2025 | 01-01-2025 | - Added some new SOP into SOP Manual - Added Review History Table | Plant Head |

Approved By : Plant Head