Contents

Document Control and SOP Management	7

Incoming Quality Control (IQC) for Raw Materials	12

Production Line Startup & Shutdown Procedure	19

Energy Management & Emissions Reduction	26

Environmental Management, Hazardous Substance Reduction & Emergency Preparedness	44

Integrated Waste Management and Resource Recovery	62

Water Management: Reclamation, Audits, Quality Testing, Rainwater Harvesting & Heavy Metal Discharge Reduction	77

Circular Product Lifecycle Management	91

Product Stewardship Through User Awareness	110

Third-Party Compliance Due Diligence	123

Central Review, Amendment & Continuous Improvement Process for All SOPs	151

Each SOP in this manual applies to the relevant site(s) as specified in its scope section. All procedures have been developed to ensure consistent, safe, and compliant execution of tasks – from incoming raw material inspection and solar cell/module production to quality assurance, warehousing, maintenance, and customer support – in alignment with national and international standards (including ALMM, BIS, IEC, and UL requirements).

2. Purpose of the Manual

The primary objectives of this SOP Manual are:

- Standardisation: To establish a uniform method for executing each critical process, minimising variability and human error.

- Quality Assurance: To uphold Gautam Solar’s commitment to high-efficiency Mono PERC and N-Type TOPCon modules, ensuring every panel meets our rigorous performance and durability benchmarks (e.g., low degradation, >90% power output after 20 years).

- Safety & Compliance: To protect employees, equipment, and the environment by embedding safety protocols and regulatory requirements into every procedure.

- Training & Onboarding: To serve as the primary training resource for new and existing employees, enabling them to understand and follow approved methods.

- Continuous Improvement: To provide a baseline for audits, performance monitoring, and future revisions as technology and regulations evolve.

3. Scope

This manual applies to all employees, contractors, and temporary workers engaged in activities that affect product quality, safety, or operational efficiency at any of the above three Gautam Solar locations. The SOPs cover, but are not limited to, the following functional areas:

- Incoming quality control (raw materials, cells, glass, encapsulants)

- Manufacturing operations (cutting, stringing, lamination, framing, junction box attachment)

- In-process and final testing (EL testing, IV flash testing, dielectric withstand, thermal cycling)

- Warehouse and inventory management

- Preventive and corrective maintenance of automated production lines

- Health, safety, and environmental (HSE) procedures

- Non-conformance handling and corrective action (CAPA)

- Customer complaint resolution

- Administrative processes at the Registered Office (e.g., procurement, HR, documentation)

The manual does not replace statutory or regulatory documents but complements them by translating external requirements into actionable internal steps.

4. Structure of the Manual

Each SOP within this manual follows a standardised format to ensure ease of use:

| Section | Content |
| --- | --- |
| Header | SOP number, title, department, applicable site(s), revision date, page number |
| 1. Purpose | Specific objective of the procedure |
| 2. Scope | Applicable areas, roles, and boundaries (including which site(s) the SOP applies to) |
| 3. Responsibility | Clear assignment of who does what |
| 4. Prerequisites | Required tools, materials, documentation, training |
| 5. Procedure | Step-by-step instructions (numbered, action-oriented) |
| 6. Records & Documentation | Forms, logs, or software entries to complete |
| 7. References | Related SOPs, standards, or external documents |
| 8. Revision History | Changes made, dates, and approvers |

SOPs are grouped by department and numbered accordingly (e.g., GS-QC-001 for Quality Control, GS-PR-001 for Production, GS-SF-001 for Safety). Each SOP header clearly indicates whether it applies to Haridwar only, Bhiwani only, both factories, or Registered Office + factories.

5. Document Control & Compliance

5.1 Ownership and Approval

- All SOPs are authored by process owners (e.g., QC Engineer, Production Supervisor) and reviewed by the Department Head.

- Final approval for release or revision is granted by the Quality Assurance Manager.

- The master copy of this manual is maintained electronically in the company’s document management system (ERP). Printed copies are considered “uncontrolled” unless stamped “CONTROLLED COPY”.

5.2 Revisions

- Each SOP is reviewed annually or whenever a process, equipment, or regulatory change occurs.

- Revisions are documented in the “Revision History” table. Obsolete versions are archived and marked “SUPERSEDED”.

- Employees must acknowledge training on the latest revision via the training log.

5.3 Non-Compliance

Failure to follow the procedures described in this manual may result in corrective action, including re-training or disciplinary measures, especially where product quality or employee safety is compromised.

6. How to Use This Manual

- Before starting any task identified in the scope, locate the relevant SOP by searching the table of contents or the index.

- Check the applicable site(s) listed in the SOP header. If the SOP does not cover your location, refer to the site-specific version (if any) or contact the QA department.

- Read the entire SOP before beginning. Pay special attention to prerequisites and safety warnings.

- Follow each step in sequence. Do not skip or reorder steps unless authorised by a supervisor.

- Document as you go. Fill in all required forms, checklists, or electronic records immediately after completing each step.

- Report deviations. If a step cannot be followed as written (e.g., equipment failure, material shortage), stop work and notify your supervisor. Do not “work around” the procedure.

- Suggest improvements. Employees are encouraged to propose changes to any SOP using the “SOP Improvement Request” form (available from the QA department).

7. Training & Acknowledgement

Every employee covered by this manual must:

- Receive initial training on all SOPs relevant to their role and site.

- Sign an “Acknowledgement of Receipt and Understanding” form.

- Undergo refresher training whenever an SOP is revised.

- Pass a periodic competency assessment (annual or as determined by department head).

Training records are maintained by the HR department in coordination with QA.

| SOP No. | GS-DC-001 |
| --- | --- |
| Version | 1.0 |
| Title | # Document Control and SOP Management |
| Department | Quality Assurance (QA) |

1. Purpose

The purpose of this SOP is to establish a controlled system for creating, reviewing, approving, distributing, revising, and archiving all Standard Operating Procedures (SOPs) and associated documents at Gautam Solar. This system ensures that:

- Only current, approved documents are available at points of use.

- Document changes are properly authorised and recorded.

- Obsolete documents are promptly removed from circulation.

- Employees are trained on the latest versions of relevant SOPs.

2. Scope

This SOP applies to:

All SOPs included in the Gautam Solar SOP Manual, across all departments (Quality, Production, Maintenance, Warehouse, HSE, HR, Administration).

Associated forms, checklists, logs, and templates referenced within SOPs.

External documents (e.g., regulatory standards, equipment manuals) when they are incorporated by reference.

Applicable locations:

| Site | Location |
| --- | --- |
| Registered Office | D-120, Okhla Industrial Area, Phase-1, New Delhi – 110020, India |
| Factory Unit 1 | Plot 67-70, Sec-8A, SIDCUL, Haridwar, Uttarakhand – 249403, India |
| Factory Unit 2 | 7 KM Milestone, Tosham Road, Bhiwani, Haryana – 127032, India |

This SOP does not cover confidential commercial agreements or employee personal files, which are managed separately by the Legal and HR departments.

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| QA Manager | Overall owner of the document control system; approves final versions of all SOPs; authorises deviations. |
| SOP Author (Process Owner) | Writes or revises the SOP based on actual process; ensures technical accuracy. |
| Reviewer (Department Head) | Reviews the SOP for completeness, feasibility, and alignment with department goals. |
| Document Controller (assigned QA staff) | Manages the master list, assigns SOP numbers, stores master copies, issues controlled copies, retrieves obsolete versions, and maintains revision history. |
| All Employees | Use only the current version of each SOP; report any missing or outdated documents to their supervisor. |
| HR Department | Ensures training records reflect the latest SOP revisions; maintains employee acknowledgement forms. |

4. Prerequisites

Before implementing this SOP, ensure:

- A designated Document Controller is appointed (name and contact details recorded in Appendix A).

- An electronic folder (e.g., in the company ERP or shared drive) is set up with restricted write access for storing master documents.

- A Master Document List (Form GS-DC-F01) is created (see Section 8).

- All employees have been notified of this SOP and know where to find current documents.

5. Procedure

5.1 Creating a New SOP

| Step | Action | Responsible |
| --- | --- | --- |
| 1 | Identify the need (new process, equipment, regulation, or continuous improvement). | Process Owner / QA Manager |
| 2 | Use the Gautam Solar SOP Template (Appendix B). Fill in: SOP number (see 5.6), title, department, applicable site(s), version = 1.0. | Author |
| 3 | Write the content following the standard structure (Purpose, Scope, Responsibility, Prerequisites, Procedure, Records, References, Revision History). | Author |
| 4 | Submit the draft to the Reviewer (Department Head) for technical review. | Author |
| 5 | Reviewer returns with comments or approves. If major changes, repeat step 4. | Reviewer |
| 6 | Submit the approved draft to the QA Manager for final approval. | Reviewer |
| 7 | QA Manager approves by signing the SOP Approval Form (GS-DC-F02) and dating it. | QA Manager |
| 8 | Assign the SOP number from the Master List; store the master PDF (read-only, watermarked “CONTROLLED COPY”) in the master folder. | Document Controller |
| 9 | Distribute controlled copies (see 5.4). | Document Controller |
| 10 | Arrange employee training and collect acknowledgements (see 5.5). | Department Head + HR |

5.2 Revising an Existing SOP

| Step | Action |
| --- | --- |
| 1 | Any employee may propose a revision using SOP Change Request Form (GS-DC-F03). |
| 2 | The relevant Department Head evaluates the request. If approved, the Author drafts the revised SOP. |
| 3 | Increase the version number by 0.1 for minor changes (e.g., 1.0 → 1.1) or by 1.0 for major changes (1.0 → 2.0). |
| 4 | Update the Revision History table with date, author, change description, and approver. |
| 5 | Follow steps 4 to 10 of Section 5.1 (review, approval, distribution, training). |
| 6 | Obsolete handling: The previous version is removed from all active locations, marked “SUPERSEDED”, and moved to an “Archived” folder. Printed copies are collected and shredded. |

5.3 Temporary (Controlled) Deviations

If an SOP cannot be followed due to an exceptional circumstance (e.g., equipment breakdown, urgent customer order), a Temporary Deviation Permit (GS-DC-F04) may be issued by the QA Manager for a maximum of 30 days. The permit must describe the deviation, justification, and risk mitigation. After 30 days, either the SOP is revised or normal operation resumes.

5.4 Distribution of Controlled Copies

- Electronic copies: The master PDF is stored in the central ERP system. All employees can view the latest version via a read-only link. No local downloads are permitted.

- Printed copies: Only issued with a “CONTROLLED COPY” stamp and a unique copy number. The Document Controller maintains a Printed Copy Log (GS-DC-F05). Printed copies are automatically considered obsolete after 24 hours unless re-issued.

- Uncontrolled copies: Copies without a stamp or marked “FOR REFERENCE ONLY” are not managed and must not be used for work.

5.5 Training and Acknowledgement

For each new or revised SOP:

| Step | Action |
| --- | --- |
| 1 | Department Head identifies all employees whose tasks are affected by the SOP. |
| 2 | Training is conducted (classroom, on-job, or self-study with a quiz). |
| 3 | Each employee signs the SOP Training Acknowledgement Form (GS-DC-F06), confirming they have read, understood, and will follow the SOP. |
| 4 | Forms are sent to HR for retention in individual training files. |

5.6 SOP Numbering Convention

All SOPs follow the format: GS-AAA-NNN

| Part | Meaning | Example |
| --- | --- | --- |
| GS | Gautam Solar | GS |
| AAA | Department code (3 letters) | DC (Document Control), QC (Quality Control), PR (Production), WH (Warehouse), SF (Safety), HR (Human Resources), AD (Admin) |
| NNN | Sequential number (001–999) | 001 |

Examples: GS-QC-001, GS-PR-002, GS-SF-003.

6. Records & Documentation

| Form Number | Form Name | Retention Period | Responsible |
| --- | --- | --- | --- |
| GS-DC-F01 | Master Document List | Life of manual + 3 years | Document Controller |
| GS-DC-F02 | SOP Approval Form | 5 years | QA Manager |
| GS-DC-F03 | SOP Change Request Form | 3 years | Department Head |
| GS-DC-F04 | Temporary Deviation Permit | 3 years | QA Manager |
| GS-DC-F05 | Printed Copy Log | 1 year after copy returned | Document Controller |
| GS-DC-F06 | SOP Training Acknowledgement Form | Employee tenure + 3 years | HR |

All records are stored electronically in the QA shared folder with restricted access.

7. References

| Reference | Description |
| --- | --- |
| ISO 9001:2015 | Clause 7.5 – Documented information |
| Gautam Solar SOP Manual – Introduction | Governance framework for all SOPs |
| ERP User Manual – Document Module | How to access electronic SOPs |
| ALMM / BIS / IEC standards | External compliance documents referenced in other SOPs |

8. Appendices

- Appendix A: Document Controller contact details (internal – maintained separately)

- Appendix B: Gautam Solar SOP Template (blank Word/PDF)

- Appendix C: Quick Reference – SOP lifecycle flowchart (see next page)

(Flowchart can be added as an image: New → Draft → Review → Approve → Number → Store →

| SOP No. | GS-QC-001 |
| --- | --- |
| Version | 1.0 |
| Title | # Incoming Quality Control (IQC) for Raw Materials |
| Department | Quality Assurance (QA) |

1. Purpose

The purpose of this SOP is to define a standardised procedure for inspecting, testing, and accepting or rejecting all incoming raw materials used in the manufacturing of solar modules at Gautam Solar. This ensures that only materials meeting defined quality specifications enter production, thereby preventing defects, reducing rework, and maintaining the high performance and reliability of our Mono PERC and N-Type TOPCon modules.

2. Scope

This SOP applies to all raw materials received at Gautam Solar’s factory locations, including but not limited to:

- Solar cells (Mono PERC, N-Type TOPCon)

- Tempered glass (front and rear)

- Encapsulants (EVA, POE)

- Backsheet

- Frames (aluminium)

- Junction boxes (with diodes)

- Ribbons (interconnector, busbar)

- Potting compound

- Sealants / adhesives

- Packaging materials (pallets, corrugated boxes, stretch film)

Applicable locations:

- Factory Unit 1, Plot 67-70, Sec-8A, SIDCUL, Haridwar, Uttarakhand – 249403

- Factory Unit 2, 7 KM Milestone, Tosham Road, Bhiwani, Haryana – 127032

This SOP does not cover consumables (e.g., cleaning cloths, gloves) or office supplies, which are handled by storekeeping under a separate procedure.

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| QA Incoming Inspector | Performs visual, dimensional, and functional inspections as per this SOP; records results; issues NCRs for non-conformances. |
| Store / Warehouse Supervisor | Notifies QA of new receipts; segregates quarantine area; manages physical movement of accepted/rejected materials. |
| Purchase Department | Communicates with suppliers regarding rejections and corrective actions. |
| QA Manager | Reviews non-conformance reports; decides on material disposition (rework, accept under concession, return); approves supplier CAPAs. |
| Production Department | May be consulted for urgent material decisions; uses only QA-approved materials. |

4. Prerequisites

- IQC Checklist for each material type (see Section 8 – Appendix A for examples).

- Approved supplier list with material specifications.

- Sampling plan table (based on AQL 0.65 for critical defects, 1.0 for major, 2.5 for minor – see Appendix B).

- Calibrated instruments: Vernier calliper, micrometer, thickness gauge, EL tester (for cells), IV flash tester (for sample cells), glass strength tester (if available), multimeter (for junction boxes).

- NCR (Non-Conformance Report) forms (GS-QC-F01).

- Quarantine area clearly marked with red tape or signs.

- IQC log (electronic or paper).

5. Procedure

5.1 Receipt Notification & Sample Size Determination

| Step | Action | Responsible |
| --- | --- | --- |
| 1 | Supplier delivery arrives. Store Supervisor verifies basic paperwork (challan, packing list). | Store Supervisor |
| 2 | Store Supervisor notifies QA department of new receipt via daily receipt log or direct intimation. | Store Supervisor |
| 3 | QA Inspector locates the material in the quarantine holding area (physical receipt pending inspection). | QA Inspector |
| 4 | Identify the lot size (number of pieces, rolls, pallets). | QA Inspector |
| 5 | Refer to Sampling Plan (Appendix B) to determine the sample size (e.g., for cells: ANSI/ASQ Z1.4, General Inspection Level II). | QA Inspector |
| 6 | Randomly select samples from different packages/pallets to represent the entire lot. | QA Inspector |

5.2 General Visual & Dimensional Inspection (All Materials)

- Visual check: No damage, cracks, stains, discoloration, foreign particles, deformation.

- Dimensional check: Measure key dimensions against approved drawing/specification sheet.

- Packaging integrity: No moisture ingress, torn bags, crushed corners.

If any of this fail, proceed to Section 5.6 (Non-Conformance Handling).

5.3 Material-Specific Inspection

A. Solar Cells (Mono PERC / TOPCon)

| Test | Acceptance Criteria | Frequency |
| --- | --- | --- |
| Visual (unaided eye or 10x loupe) | No edge chips >0.5 mm, no microcracks visible, no broken corners, no finger interruption | 100% of sample |
| Electroluminescence (EL) Imaging | No dark areas, cracks, or broken fingers; uniform brightness | 100% of sample |
| I-V measurement (flash test) | Power (Pmax) within ±3% of datasheet; Voc, Isc, FF within spec | 1 cell per 1000 (or as defined) |
| Thickness / Warpage | As per supplier spec (e.g., 180µm ± 20µm; warpage <1.5mm) | 5 samples per lot |
| Ribbon alignment (busbar) | Off-center ≤0.3 mm | 10% of sample |

B. Tempered Glass (Front & Rear)

| Test | Acceptance Criteria |
| --- | --- |
| Visual | No scratches >0.1 mm width; no bubbles >1 mm; no edge chips >2 mm length |
| Thickness | 3.2 mm ± 0.2 mm (or as per product spec) |
| Transmission | ≥91.5% (measured with transmission meter for reference) |
| Flatness / Warpage | ≤0.3% of length |

C. Encapsulants (EVA / POE)

| Test | Acceptance Criteria |
| --- | --- |
| Visual (roll) | No gel spots, foreign particles, wrinkles, or uneven colour |
| Width | ±2 mm of specified |
| Thickness | ±10% of specified |
| Cross-linking test (curing check) | Gel content ≥75% for EVA (per supplier certificate) |

Note: Full cross-linking verification is performed during production lamination trials; IQC accepts based on supplier batch certificate + random visual.

D. Backsheet

| Test | Acceptance Criteria |
| --- | --- |
| Visual (roll) | No wrinkles, pinholes, delamination, or contamination |
| Adhesion test (peel) | >40 N/cm (if tester available; otherwise rely on supplier cert) |
| Thickness | ±10% of spec |

E. Aluminium Frame

| Test | Acceptance Criteria |
| --- | --- |
| Visual | No dents, scratches, anodising defects, burrs |
| Length & width | ±0.5 mm of drawing |
| Corner angle | 90° ± 0.5° |
| Screw hole thread | Gauge passes easily |

F. Junction Box (with Diodes)

| Test | Acceptance Criteria |
| --- | --- |
| Visual | No cracks, missing gasket, terminal deformation |
| Diode test (multimeter) | Forward voltage ~0.3-0.7V; reverse blocking |
| IP rating (quick check) | Gasket properly seated (full IP test only on sample to certified spec) |
| Cable pull test | ≥50 N force without loosening |

G. Ribbons (Interconnector / Busbar)

| Test | Acceptance Criteria |
| --- | --- |
| Visual | No kinks, oxidation, uneven coating |
| Width & thickness | ±0.03 mm of spec |
| Solderability (quick dip) | Uniform wetting |

5.4 Acceptance Criteria & Disposition

| Defect Classification | Definition | Action |
| --- | --- | --- |
| Critical | Would cause immediate product failure or safety hazard (e.g., broken cell, glass chip exposing sharp edge, diode short circuit). | Reject entire lot. No concession. |
| Major | Affects performance or durability (e.g., low cell efficiency, glass warpage > spec, EVA thickness out of spec). | Reject or accept under concession with QA Manager approval. |
| Minor | Cosmetic or slight dimensional deviation not affecting performance (e.g., small scratch on frame, minor edge chip < spec). | Accept with notification to supplier. May be logged for trend analysis. |

Sampling decision rule (AQL based):

- Number of defective samples ≤ Acceptable limit → ACCEPT the lot.

- Number of defective samples ≥ Reject limit → REJECT the lot.

Refer to Appendix B for AQL tables.

5.5 Acceptance Process

- If all criteria are met, QA Inspector stamps or labels each package with “IQC APPROVED” (green sticker) and signs the inspection report.

- Material is moved from quarantine to the store accepted area.

- QA Inspector updates the IQC Log (GS-QC-F02) with lot number, date, supplier, result, and batch release number.

- Store Supervisor issues material to production only when approved.

5.6 Non-Conformance Handling (Rejection or Concession)

- QA Inspector documents all failures on the Non-Conformance Report (NCR) – Form GS-QC-F01.

- Material is marked “REJECTED – HOLD” (red sticker) and moved to quarantine rejection zone.

- QA Manager reviews the NCR and decides:

  - Return to supplier (full rejection) – Purchase department issues debit note / return request.

  - Rework at supplier cost (e.g., cleaning, repackaging) – only if feasible.

  - Accept under concession (for minor/major defects that do not impact final product safety or warranty) – Production and QA Manager sign off.

- For concession acceptance, a Concession Tag (GS-QC-F03) is attached to the lot specifying the deviation and authorising use.

- Supplier is notified and a Corrective Action Request (CAR) may be issued for recurring issues.

- QA retains a copy of the NCR in the monthly quality file.

5.7 Urgent / Expedited Material Handling

If production requires immediate release of material before full IQC is completed:

- Stop-gap procedure: QA performs a quick “critical attributes” inspection (visual only) and marks as “RELEASED UNDER PROTEST” with a time-limited tag (max 48 hours).

- The full IQC must be completed within 48 hours. If final inspection fails, production is stopped, and any WIP using that material is segregated and re-evaluated.

- Such cases require prior approval of QA Manager and Production Head.

6. Records & Documentation

| Form Number | Form Name | Retention Period | Responsible |
| --- | --- | --- | --- |
| GS-QC-F01 | Non-Conformance Report (NCR) | 5 years | QA Inspector |
| GS-QC-F02 | IQC Log (daily/monthly) | 3 years | QA Inspector |
| GS-QC-F03 | Concession Tag | Attached to lot until consumption; then filed for 1 year | QA Manager |
| GS-QC-F04 | Supplier Corrective Action Request (CAR) | 3 years | QA Manager |
| – | Supplier Certificate of Analysis (CoA) | 5 years | Store / QA |

All records are maintained in the QA department’s electronic folder or physical filing cabinet.

7. References

| Reference | Description |
| --- | --- |
| ANSI/ASQ Z1.4 / ISO 2859-1 | Sampling procedures for inspection by attributes |
| IEC 61215 | Terrestrial PV module qualification |
| Gautam Solar Approved Supplier List (internal) | Current list of qualified suppliers |
| GS-DC-001 (Document Control SOP) | Controls this SOP and its forms |
| Individual material specifications (supplier datasheets) | Held by QA and Purchase departments |

8. Appendices

Appendix A – Quick Reference Checklists

Solar Cell IQC Checklist (extract)

| Attribute | Method | Spec | Pass/Fail |
| --- | --- | --- | --- |
| Visual | 10x loupe | No visible cracks | ☐ |
| EL imaging | EL tester | No dark lines | ☐ |
| Pmax | Flash test | ≥ spec value | ☐ |

Appendix B – (AQL Table – Summary)

| Lot Size | Sample Size | Accept (Critical 0) | Accept (Major – 1.0%) | Reject (Major) |
| --- | --- | --- | --- | --- |
| 2 – 8 | 2 | 0 | 0 | 1 |
| 9 – 15 | 3 | 0 | 0 | 1 |
| 16 – 25 | 5 | 0 | 0 | 1 |
| 26 – 50 | 8 | 0 | 0 | 1 |
| 51 – 90 | 13 | 0 | 0 | 1 |
| 91 – 150 | 20 | 0 | 1 | 2 |
| 151 – 280 | 32 | 0 | 1 | 2 |
| 281 – 500 | 50 | 0 | 2 | 3 |
| 501 – 1200 | 80 | 0 | 3 | 4 |

*For critical defects: zero acceptance number (c=0). For minor defects, AQL 2.5 can be used with separate limits.*

| SOP No. | GS-PR-001 |
| --- | --- |
| Version | 1.0 |
| Title | # Production Line Startup & Shutdown Procedure |
| Department | Production |

1. Purpose

The purpose of this SOP is to establish a standardised, safe, and efficient procedure for starting up and shutting down the solar module production lines at Gautam Solar. Proper startup ensures that all equipment is ready for operation, quality parameters are met from the first module, and safety hazards are avoided. Proper shutdown prevents equipment damage, saves energy, and leaves the line in a safe state for the next shift or maintenance.

2. Scope

This SOP applies to all automated and semi-automated production equipment used in the manufacturing of Mono PERC and N-Type TOPCon solar modules, including but not limited to:

- Stringer machines (cell tabbing and stringing)

- Layup station (glass, EVA, cell matrix, backsheet)

- Laminator (curing and encapsulation)

- Framing press / automatic framing line

- Junction box attachment and soldering station

- Curing oven (for silicone / potting compound)

- Flash tester (IV measurement)

- EL tester (electroluminescence imaging)

- Hi-Pot tester (dielectric withstand)

- Conveyor systems and turn tables

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Production Supervisor | Ensures startup and shutdown procedures are followed by line operators; authorises line start after pre-start checks; signs shift handover log. |
| Line Operator | Performs the startup and shutdown steps as described; records any deviations or abnormalities. |
| Maintenance Technician | Responds to equipment faults discovered during startup; performs scheduled warm-up and calibration as needed. |
| Quality Assurance (QA) | Verifies first-module quality after startup (first-off inspection); may halt line if quality fails. |
| Shift In-charge | Approves emergency or unplanned shutdowns; coordinates with Maintenance and QA. |

4. Prerequisites

- Operator training: All operators must be trained on this SOP and on the specific equipment they operate (training records maintained by HR/Production).

- Personal Protective Equipment (PPE): Safety glasses, cut-resistant gloves, safety shoes, and ear protection (where noise >85 dBA).

- Checklists: Pre-startup checklist for each major equipment type (see Appendix A).

- Shift log book or digital log (e.g., ERP or Excel) to record startup/shutdown times, issues, and handover notes.

- Compressed air, power, and cooling water must be available and within specified ranges (checked by Maintenance before shift).

- Material availability: Raw materials (cells, glass, EVA, backsheet, ribbons, frames) must be staged and IQC-approved.

5. Procedure

5.1 Standard Daily Startup (Beginning of Shift)

| Step | Action | Responsible | Time Guideline |
| --- | --- | --- | --- |
| 1 | Arrive 10 minutes before shift start. Put on required PPE. | Operator | – |
| 2 | Check shift handover log from previous shift for any known issues, ongoing maintenance, or material deviations. | Operator | 2 min |
| 3 | Visually inspect the line: No tools left on equipment, no loose wires, guards in place, emergency stops not pressed. | Operator | 3 min |
| 4 | Verify utilities: Check that compressed air pressure is 6–7 bar, main power is on, cooling water flow present (where applicable). | Operator / Maintenance | 2 min |
| 5 | Perform pre-startup checklist for each machine (see Appendix A – sample). | Operator | 5–10 min |
| 6 | Turn on the main control panel for each machine. Wait for HMI (touch screen) to boot. | Operator | 1 min |
| 7 | Home or reference all axes (for stringers, pick-and-place robots). Follow machine-specific homing procedure. | Operator | 2 min |
| 8 | Load consumables: Ribbon spools, flux, solder wire, EVA rolls, backsheet rolls as per machine instructions. | Operator | 5 min |
| 9 | Run a dry cycle (without cells / glass) for stringers and laminator to warm up heaters and stabilise temperature. | Operator | As per machine (typically 15–20 min) |
| 10 | Set process parameters on HMI: temperature profiles (stringer soldering iron, laminator zones), vacuum levels, conveyor speeds. Verify against standard recipe for the product being run. | Operator | 3 min |
| 11 | Perform a sample run with 5–10 cells or 1 mini-laminate. | Operator | 5 min |
| 12 | Send sample to QA for first-off inspection (EL, IV, visual). | Operator / QA | – |
| 13 | Wait for QA approval before starting full production. If QA rejects, troubleshoot and repeat step 11–12. | Operator, QA, Supervisor | – |
| 14 | Record startup completion in shift log (time, parameters, any issues). | Operator | 2 min |
| 15 | Begin normal production at rated speed. | Operator | – |

5.2 Special Startup Cases

A. After Scheduled Maintenance or Breakdown

- Maintenance Technician must sign off that equipment is ready for operation (using GS-MT-F01 – Maintenance Completion Form).

- Perform an extended warm-up (30 minutes) and a full qualification run of 10 consecutive good modules before releasing the line.

- QA must approve first 3 modules.

B. Monday Morning / After Long Shutdown (>48 hours)

- In addition to the standard startup, perform a full cleaning of:

  - Laminator silicone blanket and chamber

  - Stringer nozzles and flux applicator

  - Conveyor belts

- Run a dry cycle for 30 minutes minimum.

- Calibrate temperature controllers using a reference thermometer.

C. After Material Change (e.g., different cell size or ribbon type)

- Load new material.

- Perform a trial run of 5 modules.

- QA to inspect all 5 before resuming full production.

5.3 Standard Daily Shutdown (End of Shift / End of Production)

| Step | Action | Responsible | Time Guideline |
| --- | --- | --- | --- |
| 1 | Stop feeding new materials into the line. Allow all WIP (work-in-progress) to complete the process. | Operator | – |
| 2 | Run out all modules from laminator, framing, curing oven, and testing stations. Do not leave partially processed units inside equipment. | Operator | 5–10 min |
| 3 | For laminator: Set to “idle” or “cool down” mode. Do not turn off immediately – allow temperature to drop naturally to <80°C before full shutdown to avoid blanket damage. | Operator | 15–20 min |
| 4 | For stringers: Turn off heater cartridges first. Wait 5 minutes, then turn off the main power. | Operator | 5 min |
| 5 | For curing oven: Turn off heating elements but leave exhaust fan running for 10 minutes to remove residual fumes. | Operator | 10 min |
| 6 | Remove and clean any adhesive residue, flux residue, or glass fragments from the line. | Operator | 10 min |
| 7 | Return unused consumables to proper storage (ribbon rolls, flux bottles, etc.). | Operator | 5 min |
| 8 | Shut down the main control panel for each machine (follow reverse order of startup). | Operator | 2 min |
| 9 | Disconnect compressed air at the machine level (if required by machine manual). | Operator | 1 min |
| 10 | Perform end-of-shift housekeeping: Sweep around machines, empty scrap bins, clean work surfaces. | Operator | 10 min |
| 11 | Record end-of-shift data in log: total modules produced, downtime minutes, any abnormal events. | Operator | 5 min |
| 12 | Complete shift handover log for the next shift, noting any pending issues or maintenance needs. | Operator / Supervisor | 5 min |
| 13 | Ensure all emergency stops are NOT activated (they should be in normal position for next shift). | Operator | 1 min |
| 14 | Turn off lights and lock the control panel if required by site safety rules. | Operator | 1 min |

5.4 Emergency Shutdown

An emergency shutdown is performed when there is an immediate safety risk, fire, smoke, or major equipment failure.

| Step | Action |
| --- | --- |
| 1 | Press the nearest emergency stop (E-stop) button – red mushroom button. This will immediately cut power to moving parts. |
| 2 | Do not attempt to restart until the hazard is eliminated. |
| 3 | Evacuate the area if there is fire, electrical arcing, or chemical spill. |
| 4 | Call for help: Security / Fire brigade (if needed) and Production Supervisor. |
| 5 | After emergency is resolved, only Maintenance and Supervisor may reset E-stops and perform a controlled restart using the “After Breakdown” procedure (Section 5.2 A). |

Important: Do not use emergency shutdown as a normal shutdown method – it can damage equipment (e.g., laminator blanket left at high temperature without cooling).

5.5 Weekend / Long Shutdown (End of Week)

On Fridays (or last shift of the week), in addition to daily shutdown:

- Drain any water-cooled systems (if freezing risk in winter).

- Clean all adhesive nozzles and replace with storage fluid per machine manual.

- Cover sensitive optical equipment (EL cameras, vision systems) with dust covers.

- Turn off main compressed air supply to the entire line at the header.

- Lockout/Tagout (LOTO) for any equipment that will undergo maintenance over the weekend.

- Log all actions in the maintenance log and shift log.

6. Records & Documentation

| Form Number | Form Name | Retention Period | Responsible |
| --- | --- | --- | --- |
| GS-PR-F01 | Shift Log (daily startup/shutdown) | 1 year | Operator / Supervisor |
| GS-PR-F02 | Pre-Startup Checklist (per machine) | 1 year | Operator |
| GS-PR-F03 | First-Off QA Inspection Report | 3 years | QA |
| GS-PR-F04 | Emergency Shutdown Report | 3 years | Supervisor |
| GS-PR-F05 | Maintenance Completion Form (for restart after breakdown) | 3 years | Maintenance |

All records are kept in the Production department folder (paper or electronic).

7. References

| Reference | Description |
| --- | --- |
| Equipment manuals (stringer, laminator, etc.) | Specific startup/shutdown sequences |
| GS-QC-001 (Incoming Quality Control) | Ensures raw materials are approved before use |
| GS-SF-001 (Lockout/Tagout SOP) | For safe maintenance after shutdown |
| GS-MT-001 (Preventive Maintenance SOP) | Scheduled maintenance intervals |
| ISO 45001 (Safety management) | Emergency shutdown requirements |

8. Appendices

Appendix A – Sample Pre-Startup Checklist (Stringer Machine)

| Item | Check | OK | Not OK (Action) |
| --- | --- | --- | --- |
| No loose tools or debris on machine | Visual | ☐ | Remove |
| Guard doors closed and interlock working | Test | ☐ | Call maintenance |
| Emergency stop released (pull to reset) | Visual | ☐ | Reset |
| Compressed air pressure (6–7 bar) | Gauge | ☐ | Adjust / report |
| Ribbon spool loaded and threaded | Visual | ☐ | Load |
| Flux level sufficient | Tank window | ☐ | Refill |
| Solder wire loaded | Visual | ☐ | Load |
| No broken cell fragments in work area | Sweep | ☐ | Clean |
| Heater temperature reaches setpoint (allow 5 min) | HMI readout | ☐ | Wait / calibrate |
| Vision system calibrated (grid alignment) | Run test | ☐ | Re-calibrate |

(Similar checklists exist for laminator, framing press, etc. – maintained separately.)

Appendix B – Recommended Temperature Warm-Up Times

| Equipment | Minimum Warm-Up (dry run) | Full Stabilisation |
| --- | --- | --- |
| Stringer (soldering bars) | 10 min | 15 min |
| Laminator (upper & lower platen) | 20 min | 30 min |
| Curing oven (for junction box) | 15 min | 25 min |
| Lamination stack (full) | 30 min | 45 min |

| SOP No. | GS-EN-001 |
| --- | --- |
| Version | 1.0 |
| Title | # Energy Management & Emissions Reduction |
| Department | Engineering / Energy Management |

1. Purpose

This SOP provides a comprehensive, actionable framework for Gautam Solar to:

- Systematically reduce energy intensity (kWh per solar module produced) through procurement, operation, and retrofitting of energy-efficient equipment. The goal is a measurable year-on-year decline in absolute energy consumption and specific energy use.

- Implement waste heat recovery and cogeneration (CHP) to capture thermal energy that would otherwise be lost to the environment, converting it into useful heating or additional power. The system must continuously generate at least two distinct forms of energy (e.g., electricity + hot water / steam) from a single fuel source.

- Minimise emissions of fluorinated greenhouse gases (F-gases) – which have high Global Warming Potential (GWP), often thousands of times that of CO₂ – by improving operational design, conducting proactive leak detection and repair, and responsibly decommissioning old equipment.

- Reduce energy consumption of IT infrastructure (data centres, server rooms, networking, UPS, desktops) without compromising reliability or performance. This includes optimising cooling, virtualisation, storage, and end-user power management.

Each section includes technical explanations, calculation methods, example targets, and reporting protocols to ensure consistent application across all Gautam Solar sites.

2. Scope – Detailed

The SOP applies to all energy-carrying systems and emission sources at all three locations:

| Topic | Specific Systems Covered | Exclusions |
| --- | --- | --- |
| 1. Energy-efficient equipment | Electric motors (all sizes), air compressors, HVAC (heating, ventilation, air conditioning), lighting (indoor, outdoor, street), production machinery (stringers, laminators, curing ovens, testers), pumps, fans, office equipment | Emergency/standby generators (covered under separate contingency SOP) |
| 2. Waste heat recovery & CHP | Laminator exhaust gases, cooling water from stringer soldering stations, compressor intercooler and aftercooler heat, CHP engine/generator unit (if installed), furnace/oven flue gases, any thermal fluid systems | Heat below 40°C where recovery is uneconomical (justified in feasibility study) |
| 3. F-gas emissions | Split ACs, VRV/VRF systems, window ACs, packaged units, chillers, cold storage (canteen), refrigerated air dryers, SF₆-insulated switchgear, laboratory refrigerators, vehicle ACs (excluded – handled by transport SOP) | Small hermetically sealed units (<1 kg charge) – still inventoried but leak checks less frequent |
| 4. IT energy consumption | On-premises servers (physical and virtual), network switches/routers/firewalls, storage arrays (SAN, NAS), server room cooling (CRAC/CRAH, in-row coolers), UPS for IT, desktop PCs, laptops, monitors, printers, scanners, data centre lighting | Cloud services (energy accounted by provider but choice of region/efficiency considered) |

3. Responsibilities – Expanded

| Role | Detailed Responsibilities |
| --- | --- |
| Energy Manager | – Maintains and updates this SOP annually. – Calculates monthly energy intensity and reports to plant heads. – Leads quarterly energy review meetings. – Maintains sub-metering plan and calibrates energy meters. – Prepares annual energy budget and savings targets. – Coordinates external energy audits (Bureau of Energy Efficiency, ISO 50001). |
| Procurement Department | – For any equipment >3 kW, requests energy efficiency datasheet before purchase order. – Maintains a “qualified supplier list” for energy-efficient equipment. – Rejects any purchase requisition that does not specify minimum efficiency criteria (Section 4.1). |
| Maintenance Department | – Performs daily / weekly readings of sub-meters, heat recovery temperatures, F-gas leak checks. – Cleans heat exchangers, condenser coils, and filters to maintain efficiency. – Records refrigerant recovery quantities during servicing. – Logs CHP operational hours and fuel consumption. |
| IT Department | – Manages server virtualisation and consolidation. – Sets server room temperature setpoints; enables free cooling. – Deploys power management GPO for desktops/laptops. – Tracks PUE monthly and reports to Energy Manager. |
| Production Supervisor | – Ensures production equipment is operated within energy-optimised parameters (e.g., laminator cycle times, conveyor speeds). – Reports any abnormal energy consumption (e.g., equipment running idle for extended periods). |
| HSE Officer | – Maintains F-gas inventory and labelling. – Ensures only certified technicians handle refrigerants. – Retains destruction certificates for decommissioned gases. |
| All Employees | – Switch off lights, fans, AC, computers when leaving area. – Report air conditioning leaks (oily residue, hissing sound) immediately. – Suggest energy saving ideas via suggestion scheme. |

4. Detailed Procedure

4.1 Energy-Efficient Equipment – Reducing Energy Intensity Over Time

4.1.1 Why Energy Intensity Matters

Energy intensity (kWh per module) decouples energy use from production volume. Even if production grows, intensity can fall, meaning each module costs less energy to make. Gautam Solar targets 5% annual reduction in intensity for the first five years, resulting in a cumulative 20–25% improvement by Year 5.

Example calculation:

| Month | Modules produced | Total kWh (production area) | Intensity (kWh/module) |
| --- | --- | --- | --- |
| Jan (baseline) | 10,000 | 55,000 | 5.50 |
| Feb | 11,000 | 57,200 | 5.20 (↓5.5%) |
| Mar | 12,000 | 61,200 | 5.10 (↓7.3%) |

If intensity rises >5% from previous month, investigate root causes: degraded equipment, improper settings, new inefficient machine, or measurement error.

4.1.2 Procurement Minimum Efficiency Standards

Electric Motors (all new and replacement):

- Standard: IE3 (Premium Efficiency) for 0.75–375 kW (IEC 60034-30).

- For motors running >4,000 hours/year (most production motors), IE4 (Super Premium Efficiency) is recommended based on lifecycle cost analysis.

- Do not purchase: IE1 (Standard) or IE2 (High) except for very intermittent duty (<500 hrs/year) with documented approval.

Air Compressors (screw type):

- Must have Variable Speed Drive (VSD) if flow demand varies by >20% (typical in factories with shift operations).

- Minimum efficiency: ISO 50001 Class 2 or better.

- For fixed speed compressors, require a load/unload controller and adequate receiver tank sizing (>200 litres per 100 cfm).

Lighting:

- All new and replacement luminaires: LED only.

- Luminous efficacy ≥100 lumens/W for indoor high bay (factory), ≥120 lumens/W for linear tubes.

- For outdoor floodlighting: ≥130 lumens/W with correlated colour temperature (CCT) ≤4000K (better visibility and less glare).

- Install daylight sensors near windows and occupancy sensors in warehouses, corridors, and toilets.

HVAC (Air Conditioners, Chillers):

- Split and packaged ACs: Minimum 5-star BEE rating (India) or ISEER ≥4.5.

- Chillers (if >20 kW): Screw or centrifugal type with VSD. Minimum IPLV (Integrated Part Load Value) ≥0.70 kW/ton for water-cooled, ≥1.0 kW/ton for air-cooled.

Production Machinery (Stringers, Laminators, Ovens):

- Request Specific Energy Consumption (SEC) from supplier in kWh per unit output (e.g., kWh per 1,000 strings or kWh per module laminated).

- Compare SEC with current equipment baseline. Target ≥15% reduction for any replacement.

4.1.3 Sub-metering & Data Collection

Install Class 1.0 accuracy digital energy meters at the following points:

| Meter ID | Location | Purpose |
| --- | --- | --- |
| EM-01 | Main incoming feeder (Haridwar) | Total factory consumption |
| EM-02 | Main incoming feeder (Bhiwani) | Total factory consumption |
| EM-03 | Stringer line #1 | Energy per stringer |
| EM-04 | Laminator #1 | Energy per laminator |
| EM-05 | Air compressor #1 | Compressor energy |
| EM-06 | Air compressor #2 (VSD) | Compare vs fixed speed |
| EM-07 | Lighting panel (factory) | Lighting energy |
| EM-08 | HVAC panel (factory) | Cooling/heating energy |
| EM-09 | Office & admin block (Delhi) | IT and office load |

Data recording frequency:

- Daily: All meters (by shift supervisor).

- Weekly: Calculate energy intensity using production pieces from shift log.

- Monthly: Energy Manager compiles into energy dashboard.

4.1.4 Energy-Efficient Operation Guidelines

For air compressors (often 10–20% of factory electricity):

- Set discharge pressure to minimum required (typically 6.5 bar instead of 7.5 bar). Every 1 bar reduction saves 7–10% energy.

- Fix all air leaks (audit monthly using ultrasonic detector).

- Install zero-loss condensate drains instead of timer drains.

- Reduce inlet air temperature (duct outside air if cooler than compressor room).

For laminators:

- Verify insulation integrity – infrared scan for hot spots.

- Do not keep laminator idle at full temperature for >30 minutes; use standby mode (lower platen temperature) if break is longer.

- Optimise cycle time – adjust vacuum and pressure ramp rates per supplier manual.

For lighting:

- Clean luminaires every 6 months (dust reduces output by 20–30%).

- Replace T8 fluorescent with LED tube lights (saving 50–60%).

- Implement zone control – turn off lights near windows during daytime.

4.1.5 Retrofit & Replacement Plan

Annually (Q4), Energy Manager prepares a Capital Energy Retrofit Plan listing:

- Equipment with highest energy intensity (bottom 10%).

- Estimated savings (kWh/year), cost, payback.

- Prioritisation: payback <2 years → immediate; 2–4 years → include in next year’s budget; >4 years → evaluate if combined with other upgrades.

Example table snippet:

| Equipment | Age (yrs) | Annual kWh | Replacement cost | Annual savings | Payback (yrs) |
| --- | --- | --- | --- | --- | --- |
| Compressor #2 (fixed speed) | 8 | 120,000 | ₹3,00,000 | ₹96,000 (30% saving) | 3.1 |
| Stringer soldering heater (resistive) | 5 | 45,000 | ₹80,000 (IR heater) | ₹18,000 | 4.4 (reassess) |

4.2 Waste Heat Recovery & Cogeneration (CHP)

4.2.1 Principles & Technical Background

Waste heat is thermal energy generated as a by-product of industrial processes (laminators, compressors, ovens) that is normally rejected to the atmosphere via cooling water or exhaust air. Waste heat recovery (WHR) captures a portion of this heat and uses it for space heating, pre-heating process inputs, or generating additional power.

Cogeneration (Combined Heat and Power – CHP) takes this further: a single prime mover (gas engine, gas turbine, steam turbine, or biomass gasifier) simultaneously produces electricity and usable heat. Typical total efficiency reaches 80–90% compared to ~35–45% for grid electricity alone.

Why this applies to Gautam Solar:

- Laminators operate at 140–160°C; their cooling water and exhaust contain significant heat.

- Air compressors produce 80–90°C hot air from the oil cooler and aftercooler.

- If a CHP unit is installed, it can run on natural gas or biomass (available in Haridwar/Bhiwani areas) and provide both on-site power and hot water for pre-heating laminator platens or cleaning.

4.2.2 System Configuration (Conceptual – adapt to actual installed equipment)

Option A – Standalone Waste Heat Recovery (no CHP yet):

| Heat Source | Typical Temp | Recovery Technology | Use for |
| --- | --- | --- | --- |
| Laminator cooling water (outlet) | 50–70°C | Plate heat exchanger | Pre-heat boiler feedwater, washing water for glass |
| Laminator exhaust (flue, if gas heated) | 180–250°C | Economiser (gas-to-water) | Generate low-pressure steam or hot water |
| Compressor oil cooler air | 70–90°C | Ducted hot air | Space heating (warehouse, office) in winter |
| Compressor intercooler condensate | 40–50°C | Heat exchanger | Pre-heat water for canteen or cleaning |

Option B – CHP Unit (Natural Gas Engine or Biomass Gasifier)

| Component | Function |
| --- | --- |
| Prime mover (e.g., 500 kW gas engine) | Burns natural gas to rotate generator – produces electricity. |
| Heat recovery system | Recovers heat from engine jacket water (90°C) and exhaust gas (500°C) via heat exchangers. |
| Thermal output | Hot water at 80–90°C or low-pressure steam – supplied to laminator pre-heating, factory space heating, or an absorption chiller (for summer cooling). |

Key performance indicator: Total Efficiency = (Electrical output + Thermal output) / (Fuel input energy content, in same units).
Target: ≥80% (e.g., 35% electrical + 50% thermal = 85%).

4.2.3 Feasibility & Installation Steps

Before implementing WHR or CHP, a feasibility study must be conducted:

- Identify all waste heat streams (temperature, flow rate, duration per shift, yearly hours).

- Calculate recoverable energy using formula:
Q = ṁ × Cp × ΔT
– ṁ = mass flow rate (kg/s)
– Cp = specific heat (kJ/kg·K) – for water ~4.18, air ~1.005
– ΔT = temperature drop across heat exchanger (K)

- Determine thermal demand (how much hot water / air is actually needed).

- Economic analysis: Installed cost, annual savings (fuel/electricity avoided), payback period. Aim for payback ≤4 years.

- Obtain environmental clearance if CHP unit exceeds threshold (consult state pollution control board).

After feasibility approval, follow this SOP for Operation & Maintenance (Section 4.2.4).

4.2.4 Operation Procedure – Waste Heat Recovery (Non-CHP)

| Step | Action | Frequency | Responsible |
| --- | --- | --- | --- |
| 1 | Check heat exchanger cleanliness – differential temperature across HX. If ΔT is >5°C below design, schedule cleaning. | Weekly | Maintenance |
| 2 | Monitor bypass valves – ensure they are not unintentionally diverting flow away from recovery. | Daily | Operator |
| 3 | Log temperatures – Inlet and outlet on hot side, inlet and outlet on cold side. Calculate recovered energy (kWh) using flow meter readings. | Daily | Maintenance |
| 4 | For ducted compressor heat: Adjust dampers to send heated air to warehouse in winter (damper open) and exhaust outside in summer (damper closed). | Seasonal (Oct & Apr) | Maintenance |

Recovered energy calculation example:

Hot water flow = 5 L/s (5 kg/s), Cp = 4.18 kJ/kg·K, ΔT = 15 K (from 70°C to 55°C).
Q = 5 × 4.18 × 15 = 313.5 kW of recovered heat. If operates 8 hours/day, 250 days/year → 313.5 × 8 × 250 = 627,000 kWh thermal per year. Saved natural gas (at 90% boiler efficiency) = 627,000 / 0.9 / 10.55 (kWh per m³ of natural gas) ≈ 66,000 m³/year.

4.2.5 Operation Procedure – CHP Unit

Pre-start checks:

- Fuel supply valve open, pressure between 0.5–1.0 bar (natural gas) or biomass hopper full.

- Cooling water circuit full, no air locks.

- Thermal demand exists (e.g., factory needs hot water for laminator pre-heat or space heating). Do not run CHP if thermal demand <30% of output – efficiency drops.

Startup sequence (generic – adapt to manufacturer manual):

- Turn on CHP control panel – perform self-test.

- Pre-lubricate engine (if applicable).

- Initiate start – engine cranks, runs at idle speed.

- Allow warm-up (5–10 minutes) until jacket water reaches 70°C.

- Synchronise generator to grid (if grid-parallel) or close isolation breaker (island mode).

- Gradually increase load to desired power output (e.g., 300 kW). Monitor exhaust temperature – do not exceed limit.

- Open heat recovery valve to send hot water to thermal load.

Routine monitoring (every hour):

- Electrical output (kW), thermal output (kW), fuel flow rate (Nm³/h or kg/h).

- Total efficiency = (kW_elec + kW_thermal) / (fuel_kW_input). Keep >80%.

- Engine temperatures (jacket water, exhaust, oil).

- Vibration, noise, emissions (visible smoke).

Shutdown (normal):

- Reduce electrical load gradually over 2 minutes.

- Disconnect from grid (if paralleled).

- Run engine at idle for 3 minutes to cool down.

- Stop engine. Leave cooling water pump running for 10 minutes to prevent local overheating.

- Close fuel valve and log shutdown.

Emergency shutdown: Press emergency stop button. The unit will immediately stop. After restart, contact maintenance to reset alarms.

4.3 Reduction of Fluorinated Greenhouse Gas (F-Gas) Emissions

4.3.1 Understanding F-Gases & Their Impact

Fluorinated gases (HFCs, PFCs, SF₆) are synthetic greenhouse gases used as refrigerants, blowing agents, and electrical insulators. Their Global Warming Potential (GWP) can be thousands of times higher than CO₂ over 100 years:

| Gas | Common Use | GWP (100-yr) |
| --- | --- | --- |
| R410A | Residential & commercial AC | 2,088 |
| R134a | Chillers, car AC, refrigerators | 1,430 |
| R404A | Cold storage, transport | 3,922 |
| R32 (low-GWP alternative) | New AC units | 675 |
| R744 (CO₂) | Commercial refrigeration | 1 |
| SF₆ | High-voltage switchgear | 23,500 |

A 1 kg leak of R410A is equivalent to releasing 2,088 kg of CO₂ – same as driving a car for 5,000 km. Gautam Solar commits to zero uncontrolled emissions through leak prevention and responsible decommissioning.

4.3.2 Inventory & Labelling – Detailed Procedure

Step 1 – Create F-Gas Inventory (Form GS-EN-F04)

For each piece of equipment containing ≥1 kg of F-gas (or any SF₆ amount), record:

- Unique ID (e.g., AC-HR-01 for Haridwar, AC-BW-05 for Bhiwani)

- Equipment type (split AC, chiller, SF₆ switchgear)

- Location (building, room, floor)

- Refrigerant/gas type and full chemical name

- Factory charge (kg) – from nameplate

- Current charge (kg) – after any service

- Installation date

- Last leak test date and result

- Next leak test due date

Step 2 – Labelling

Attach a durable label (weather-proof) near the service valve or on the unit casing stating:

F-GAS (HFC) – DO NOT VENT
Gas: R410A / GWP 2088
Charge: 2.5 kg
CO₂ equivalent: 5,220 kg
Last leak check: [date]

For switchgear with SF₆, label: “SF₆ – Greenhouse gas – report leaks immediately”.

4.3.3 Leak Detection & Repair (LDAR) Program

Leak test methods:

| Method | Suitable for | Sensitivity |
| --- | --- | --- |
| Electronic sniffer (halogen leak detector) | All refrigerant systems | 1–5 g/year |
| Soap bubble solution | Fittings, welds, flares | Gross leaks (>50 g/year) |
| Ultrasonic detector | High-pressure gas leaks | Non-specific |
| Vacuum decay test | After servicing, before recharge | – |

Leak test frequency:

| Equipment size | Leak test frequency |
| --- | --- |
| ≥5 kg charge | Quarterly (every 3 months) |
| 1–5 kg charge | Annually (every 12 months) |
| <1 kg charge | Not required by regulation, but check annually (visual) |
| SF₆ switchgear | Annually (using gas detection camera or bagging method) |

Leak threshold & repair timeline:

- If measured leak rate >5% of charge per year, repair must be completed within 30 days.

- After repair, perform a follow-up leak test to confirm ≤1% leak rate.

- Document all repairs, including amount of refrigerant added (if any), in Form GS-EN-F05.

4.3.4 Operational Design to Reduce F-Gas Use

For new purchases (ACs, chillers, refrigerators):

| Old refrigerant | Preferred new refrigerant | GWP reduction |
| --- | --- | --- |
| R410A | R32 (or R454B) | ~68% (2088 → 675) |
| R134a | R513A (or R1234yf) | ~58% (1430 → 600) |
| R404A | R449A (or R290/propane) | ~68% (3922 → 1280) |

For existing chillers (large systems):
Evaluate retrofit feasibility – some chillers can be converted from R134a to R513A with minor changes (oil compatibility, seals). Require a qualified chiller engineer assessment.

Alternative cooling technologies:

- For server rooms, use air-side economizers (free cooling) instead of refrigerant-based AC for many hours in winter.

- For small equipment rooms, consider thermosiphon or evaporative cooling (desert coolers) where humidity permits.

4.3.5 Decommissioning & Disposal

When any F-gas equipment is permanently taken out of service (end of life, replacement, or demolition), follow these steps exactly:

- Notify Energy Manager and HSE Officer – include equipment ID and refrigerant type.

- Recover refrigerant using a certified recovery machine (e.g., Robinair, Appion) and dedicated recovery cylinder (colour-coded: pink for R410A, light blue for R134a, etc.). The technician must hold EPA Section 608 (or equivalent) certification.

- Weigh recovered quantity – record in log. If recovered amount is less than nameplate charge by >10%, assume a leak existed; document in NCR.

- Send recovered refrigerant to a licensed reclamation or destruction facility – never release to atmosphere, even if “used” or mixed. Obtain a destruction certificate or receipt.

- Scrap the equipment only after the compressor/oil has been drained (some oil contains dissolved refrigerant – recover again).

- Remove the equipment from the F-Gas Inventory and archive the record with destruction certificate.

Penalty for non-compliance: Any employee found deliberately venting refrigerant will face disciplinary action, and Gautam Solar may be liable for heavy fines under environmental laws.

4.3.6 Training & Awareness

- Every maintenance technician who works on AC/refrigeration must complete a certified F-gas handling course within 6 months of hiring.

- Annual refresher training (1 hour) covering: identification of leaks, proper use of recovery machine, record keeping.

- All employees: awareness poster near AC units: “Report leaks – Save our climate”.

4.4 Reduction of IT Infrastructure Energy Consumption

4.4.1 Why IT Energy Matters

Data centres and server rooms consume 10–30 times more energy per square metre than a typical office. A single rack can use 5–10 kW of power, and every 1 kW of IT load requires another 0.5–1 kW for cooling (depending on PUE). Gautam Solar’s IT footprint includes:

- A small server room at Delhi office (approx. 5 racks)

- Network closets at each factory (1–2 racks each)

- Approximately 100 desktops/laptops across all sites

- UPS, networking, storage

Aggregate IT energy is estimated 8–12% of total office + factory admin load. Reducing it directly lowers electricity bills and carbon footprint.

4.4.2 Server & Data Centre Optimisation – Detailed

A. Virtualisation

Virtualisation runs multiple virtual servers (VMs) on one physical host, reducing the number of physical servers. Each physical server removed saves 150–300 watts continuously (1,300–2,600 kWh/year).

Target: ≥80% of all servers virtualised. Use VMware vSphere, Microsoft Hyper-V, or Proxmox.

Action steps:

- Inventory all physical servers – note CPU, RAM, storage utilisation.

- Consolidate low-utilisation servers (e.g., <10% CPU) into VMs on a single host.

- For remaining physical servers, justify why they cannot be virtualised (e.g., proprietary hardware dongles, real-time requirements). Approval from IT Manager and Energy Manager required.

B. Server power management

Most modern servers support processor C-states (idle power saving). Enable in BIOS:

- C1 (halt), C6 (deep sleep). Also enable OS power policies – “Balanced” or “Power Saver” instead of “High Performance” for non-critical servers.

Power capping: For servers with redundant power supplies, set a power cap 10% below nameplate – prevents excess draw.

C. Decommissioning “zombie” servers

A zombie server is one that is powered on but not performing any useful work (e.g., old backup server still plugged in, test server forgotten).

Procedure:

- Monthly, run a network scan to list all active IP addresses.

- Cross-reference with authorised server list.

- For any unauthorised or unknown server, shut it down for 30 days. If no user complains, decommission permanently.

D. Storage optimisation

- Replace spinning HDDs (10–15 watts per drive) with SSDs (2–5 watts per drive) for active data. For archiving, use low-power nearline HDDs (5–7 watts) that spin down when idle.

- Implement tiered storage: hot data on SSDs, warm data on HDDs, cold data on cloud or tape (if used). Move infrequently accessed files (e.g., >90 days old) to cold tier automatically.

4.4.3 Cooling Optimisation – Detailed (Single largest IT energy saver)

Recommended setpoints (ASHRAE TC 9.9):

| Class | Dry-bulb temperature | Relative humidity |
| --- | --- | --- |
| A1–A4 (typical server rooms) | 18–27°C (allowable up to 32°C for short periods) | 20–80% (non-condensing) |

Current practice often: 20°C. Raise to 25°C as standard, 27°C if all IT equipment is rated for A3/A4 (most modern gear is). Every 1°C increase saves 3–5% cooling energy.

Hot aisle / cold aisle containment:

Arrange server racks in alternating rows: cold aisle facing the front (intake) and hot aisle facing rear (exhaust). Contain the aisles using blanking panels and door covers to prevent mixing.

Free cooling (air-side economizer):

When outside air temperature is ≤20°C and humidity ≤80% RH, turn off the air conditioner and open motorised dampers to draw outside air through filters into the server room. Exhaust hot air outside. This can be used 2,000–4,000 hours/year in North India (Oct–Feb).

Implementation:

- Install a changeover controller with temperature/humidity sensors.

- Set changeover: if outside temp < 20°C and inside temp > 24°C → open dampers 50%; if outside < 15°C → 100% free cooling.

4.4.4 UPS Efficiency Improvements

Uninterruptible Power Supplies (UPS) typically operate in double-conversion mode (AC→DC→AC) which wastes 6–10% as heat. When power is stable (most of the time), eco-mode bypasses the inverter, improving efficiency to 97–99%.

Procedure:

- Check UPS manufacturer capabilities – many (e.g., Eaton, APC Smart-UPS with “Green Mode”) support eco-mode.

- Enable eco-mode during working hours when grid quality is good (voltage ±10%, frequency 50 Hz ±1%). Disable during thunderstorms or after grid disturbances.

- If the UPS does not support eco-mode, consider replacement when end-of-life. New UPS should have efficiency >94% at 30–80% load.

Right-sizing: A UPS is most efficient at 40–70% load. If a UPS is loaded <20% (common for small server closets), replace with a smaller unit or consolidate loads. Otherwise, the rectifier and inverter losses dominate.

4.4.5 End-User Computing Power Management

Desktop / Laptop policy:

- Use Group Policy Object (GPO) for Windows domain or MDM for others:

  - Turn off monitor after 5 minutes idle.

  - Put computer to sleep after 15 minutes idle.

  - Hibernate after 60 minutes (saves state, zero power).

- For laptops, also enable “Dim display on battery” and “Lower brightness to 40%”.

- Educate employees: Shut down or hibernate at end of day. A single PC left on overnight (8 hours) wastes about 0.2 kWh per night → 50 kWh/year per PC. For 100 PCs → 5,000 kWh/year wasted.

Monitor replacement:

- Replace any CCFL backlit monitor (older than 2015) with LED backlit – saves 20–30 watts per monitor. New monitors: Energy Star certified with auto-brightness.

Printers & peripherals:

- Set printers to deep sleep after 15 minutes – wake only when print job arrives.

- Avoid personal printers; use shared MFDs (multi-function devices) which are more efficient per page.

- Turn off scanners, external drives when not in use (use USB hubs with individual switches).

4.4.6 Performance Metrics – IT

| Metric | Formula | Target | Reporting Frequency |
| --- | --- | --- | --- |
| PUE (Power Usage Effectiveness) | Total facility energy / IT equipment energy | ≤1.5 for server room; ≤1.8 for small closet | Monthly |
| Server virtualisation ratio | Number of VMs / Number of physical hosts | ≥80% | Quarterly |
| Average desktop idle power | Measured with power meter | ≤50W (including monitor) | Annual sample |
| Free cooling utilisation | Hours of free cooling / total cooling hours | ≥30% | Monthly |

How to measure PUE for Gautam Solar server room:

- Total energy = UPS input power (from UPS display or sub-meter) + dedicated AC energy (sub-meter).

- IT equipment energy = sum of all server, storage, switch nameplate x utilisation (or use UPS output power reading).
Example: UPS output 5 kW, AC draws 1.5 kW → PUE = (5+1.5)/5 = 1.3. Good.

5. Records & Forms

| Form ID | Name | Content Summary | Retain |
| --- | --- | --- | --- |
| GS-EN-F01 | Weekly Energy Intensity Log | Date, shift, modules produced, sub-meter readings (EM-01 to EM-09), calculated intensity (kWh/module), variance vs target, actions taken. | 3 years |
| GS-EN-F02 | CHP / Waste Heat Daily Log | Fuel consumption, electricity generated, heat recovered (kW), thermal temperatures, operating hours, efficiency. | 3 years |
| GS-EN-F03 | Waste Heat Recovery Temperature Log | For each recovery point: hot side in/out, cold side in/out, flow rate, calculated kW recovered. | 3 years |
| GS-EN-F04 | F-Gas Inventory Master List | Equipment ID, location, gas type, charge (initial & current), installation date, leak test history, next due date. | Life of equipment + 5 years |
| GS-EN-F05 | Quarterly Leak Test & Repair Log | Date, equipment ID, leak rate (g/yr or %), method, repair action, refrigerant added (kg), technician name. | 5 years |
| GS-EN-F06 | IT Energy Consumption Log | UPS input/output, PUE, server room temperature setpoint, free cooling hours, desktop survey results. | 3 years |
| GS-EN-F07 | Energy Review Meeting Minutes | Date, attendees, topics, decisions, action items (with owners and deadlines). | 5 years |
| GS-EN-F08 | Capital Energy Retrofit Plan (annual) | List of equipment for upgrade/replacement, estimated cost, savings, payback, priority ranking. | 5 years |

All forms are stored electronically in the shared drive \\GautamSolar\Energy_Management\ with restricted edit access.

6. References

| Standard / Regulation | Applicable Section | Key Requirement |
| --- | --- | --- |
| IEC 60034-30-1 | 4.1 | Motor efficiency classes (IE1–IE4) |
| ISO 50001:2018 | Entire SOP | Energy management system – Plan-Do-Check-Act |
| BEE (India) – Energy Conservation Act | 4.1, 4.5 | Designated consumers (if applicable) to appoint energy manager, conduct audit |
| ASHRAE Standard 90.1 | 4.1, 4.4 | Energy standard for buildings (HVAC, lighting) |
| ASHRAE Thermal Guidelines for Data Centres (TC 9.9) | 4.4 | Recommended temperature/humidity envelopes |
| Montreal Protocol (Kigali Amendment) | 4.3 | Phase-down of HFCs |
| EPA 40 CFR Part 82 (USA) – Refrigerant management | 4.3 | Leak repair, technician certification (for exports to US) |
| EU F-Gas Regulation (EU 517/2014) | 4.3 | Leak checks based on CO₂ equivalent threshold |
| Central Electricity Authority (CEA) Cogeneration guidelines | 4.2 | Definitions, efficiency benchmarks |
| IEC 61215 (PV module qualification) | – | Reference for production energy (not directly but context) |

7. Appendices

Appendix A –  Energy Intensity Dashboard (Monthly)

| Month | Prod (pcs) | kWh Total | Intensity (kWh/pc) | vs Baseline | vs Target (-5%) | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Jan 25 | 10,000 | 55,000 | 5.50 | – | – | Baseline |
| Feb 25 | 11,000 | 56,100 | 5.10 | -7.3% | -5% | ✅ On track |
| Mar 25 | 12,000 | 58,800 | 4.90 | -10.9% | -10% | ✅ Exceeded |

Appendix B –F-Gas Inventory Entry

| ID | Type | Location | Gas | Charge (kg) | GWP | CO₂e (kg) | Last Leak Test | Next Test |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AC-HR-03 | Split 1.5 TR | Haridwar – QC lab | R410A | 1.2 | 2088 | 2,506 | 15-Jan-2026 (pass) | 15-Apr-2026 |

Appendix C – Quick Reference: Leak Test Frequency Card (to be posted in maintenance workshop)

F-GAS LEAK CHECK INTERVALS
• Systems with ≥5 kg charge → Every 3 months
• Systems with 1–5 kg charge → Every 12 months
• SF₆ switchgear → Every 12 months
• After any repair → Within 1 month

Appendix D – Server Room Free Cooling Decision Flowchart (Text Description)

- Measure outside temperature (T_out) and relative humidity (RH_out).

- If T_out < 20°C and RH_out < 80% → Enable free cooling.

- If T_out < 10°C and RH_out < 60% → Full free cooling (100% outside air).

- If outside conditions not met → Run AC in normal mode.

- If inside server room temperature falls below 18°C, close free cooling dampers partially.

| SOP No. | GS-EN-002 |
| --- | --- |
| Version | 1.0 |
| Title | # Environmental Management, Hazardous Substance Reduction & Emergency Preparedness |
| Department | HSE (Health, Safety & Environment) |

1. Purpose

This SOP establishes a unified framework at Gautam Solar to:

- Reduce material consumption in operations through lightweighting, reuse of reconditioned parts, optimised cutting, and multi-function part design.

- Replace hazardous process materials with eco-friendly, bio-based, or low-toxicity alternatives (e.g., plant-based lubricants, non-VOC cleaners, biodegradable logistics materials).

- Prepare for environmental emergencies (spills, leaks, equipment failures) involving heavy metals, chlorinated/brominated substances, and other hazardous materials – including communication, testing, and response effectiveness assessment.

- Label, store, handle, and transport hazardous substances safely, following GHS standards, using spill-proof containers, safety signs, and exposure reduction equipment.

- Treat and dispose of hazardous waste safely, ensuring neutralisation or irreversible transformation, with regular inspections and regulatory compliance.

- Reduce or eliminate hazardous substances in operations by substituting heavy metals, halogenated compounds, and toxic solvents with greener alternatives (lead-free, recyclable, biodegradable, eco-friendly flame retardants).

- Comply with the Stockholm Convention on POPs by substituting or modifying processes to minimise release, and destroying any POPs-containing waste.

- Comply with the Minamata Convention on Mercury by replacing mercury-containing equipment, implementing closed-loop systems, and upgrading capture/treatment technologies.

This SOP applies to process materials (consumables used in production and operations) and hazardous waste generated, but excludes finished product materials and primary packaging (covered elsewhere).

2. Scope – Detailed

The SOP covers all Gautam Solar sites and all relevant activities:

| Topic | Applicable Areas |
| --- | --- |
| Material consumption reduction | Production (cell cutting, ribbon slitting, frame cutting, soldering), maintenance (reconditioned parts), design (multi-function parts, lightweighting), warehousing (nesting, pack optimisation). |
| Eco-friendly / bio-based process materials | Lubricants for soldering machines, degreasers for cleaning glass/cells, cleaners for maintenance, internal logistics (biodegradable stretch wrap, reusable pallets). |
| Emergency preparedness | Spill response for coolants, soldering fluxes, cleaning solvents, battery acids, chlorinated cleaning agents, brominated flame retardants (if stored), heavy metal residues from cells. |
| Hazardous substance labelling, storage, handling, transport | All chemicals with GHS hazard pictograms: fluxes, solvents, adhesives, sealants, cleaning agents, compressed gases, acids, bases, etc. |
| Hazardous waste treatment & disposal | Spent solvents, used oil, contaminated rags, chemical containers, cell breakage residues (trace heavy metals), expired chemicals. |
| Hazardous substance reduction / substitution | Lead-free solders, halogen-free flame retardants (in j-boxes, cables), non-toxic fluxes, water-based cleaners, biodegradable lubricants. |
| POPs (Stockholm Convention) | Identify any POPs (e.g., certain flame retardants, pesticides – unlikely but check cleaning agents). Modify processes to avoid release; destroy POPs waste. |
| Mercury (Minamata Convention) | Replace mercury-containing thermometers, pressure gauges, fluorescent lamps (with LED), switches, relays. Install mercury capture in waste gas if any. |

Exclusions: Finished solar modules (product materials), primary packaging for shipment, office supplies.

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| HSE Manager | Overall owner of this SOP; maintains chemical inventory; leads emergency response team; conducts inspections and audits. |
| Production Manager | Implements material reduction initiatives (cutting optimisation, part reuse). Approves substitution of process materials. |
| Procurement | Purchases only eco-friendly / bio-based alternatives from approved list; obtains SDS and GHS labels. |
| Store / Warehouse | Ensures proper labelling, storage (secondary containment), transport (spill-proof packaging). Maintains hazardous substance register. |
| Maintenance | Handles reconditioned parts; replaces mercury-containing equipment; supervises waste treatment. |
| Emergency Response Team (ERT) | Trained members (at each site) who execute spill control, first aid, and evacuation during environmental emergencies. |
| All Employees | Report leaks/spills immediately; use hazardous substances only as trained; follow labelling and storage rules. |

4. Detailed Procedures

4.1 Reduction of Material Consumption in Operations

4.1.1 Lightweighting & Multi-function Parts

Objective: Reduce the quantity of metals, plastics, and other materials used in manufacturing without compromising product quality.

Implementation:

| Action | Description | Example at Gautam Solar |
| --- | --- | --- |
| Thinner frames | Redesign aluminium frame cross-section while maintaining mechanical strength. | Switch from 35mm to 30mm frame thickness for certain module sizes, saving 15% aluminium per unit. |
| Reduced ribbon width | Use narrower interconnector ribbons without increasing resistance. | Reduce ribbon width from 1.5 mm to 1.2 mm – saves 20% copper per module. |
| Multi-function parts | Combine two or more functions into a single moulded part. | Integrate junction box base and cable strain relief into one moulding, eliminating separate plastic bracket. |
| Thinner glass | Use 3.2 mm instead of 4.0 mm where permitted by mechanical load standards. | For rooftop modules not subject to heavy snow, 3.2 mm tempered glass saves 20% glass weight. |

Procedure:

- R&D / Engineering team evaluates each component annually for lightweighting potential.

- Prototype and test (mechanical load, thermal cycling, humidity freeze per IEC 61215).

- Once validated, update BOM and procurement specifications.

4.1.2 Optimising Material Cutting Area

Objective: Maximise yield from raw materials (cells, glass, EVA, backsheet, ribbon).

For solar cells (slicing / dicing):

- Use laser cutting instead of mechanical scribing to reduce kerf loss (less material wasted as dust).

- Optimise cell layout for half-cut or third-cut cells – minimise edge waste.

For EVA/backsheet rolls:

- Calculate optimal cut length to match module dimensions with minimal roll remnant.

- Use software (nesting algorithm) to combine different module sizes from same roll.

For ribbon / busbar:

- Use reel-to-reel feeding with sensor to stop precisely at required length, avoiding over-cut waste.

- Recycle trimmed ends (copper scrap – sent to authorised recycler).

Procedure:

- Production planner reviews cut yield monthly.

- Any process generating >3% scrap material triggers a Kaizen event to reduce waste.

4.1.3 Integration of Reused or Reconditioned Parts

Objective: Reduce consumption of new parts by refurbishing and reusing components from returned modules (rework) or from internal process waste.

Allowed reconditioned parts (after rigorous testing):

| Part | Reconditioning process | Acceptance test |
| --- | --- | --- |
| Junction boxes (without diodes) | Clean terminals, replace gasket, test IP rating | Dielectric strength, pull test |
| Aluminium frames (slightly bent) | Straighten in press, re-anodise if needed | Dimensional check, torque test |
| Ribbon remnants (>2 m) | Straighten and respool | Visual, conductivity |

Prohibited: Never reuse cells (even if electrically good) – risk of hidden microcracks. Use only new cells.

Procedure for reconditioning:

- Collect eligible parts from reject modules or production waste.

- Send to designated rework station with trained technician.

- Perform reconditioning as per work instruction.

- Test using the same criteria as new parts.

- Label “RECONDITIONED – QUALIFIED” with date and technician ID.

- Reconditioned parts are used only for internal repairs or low-priority orders (customer approval if required).

Tracking: Maintain a log of reconditioned parts used, showing material saved (kg) per month.

4.1.4 Performance Metric – Material Intensity

Calculate Material Intensity (kg per kWp of module) monthly. Target reduction of 3% year-on-year.

4.2 Use of Eco-friendly or Bio-based Process Materials

4.2.1 Replacement of Petroleum-based Lubricants

Current use: Mineral oil-based lubricants for soldering machine moving parts, conveyor chains, and pneumatic cylinders.

Alternatives:

- Plant-based hydraulic oils (e.g., rapeseed oil ester) – biodegradable, non-toxic.

- Synthetic esters derived from renewable feedstocks.

Procedure:

- Procurement to source bio-based lubricant that meets equipment manufacturer specifications (viscosity, oxidation stability).

- Test on one machine for 500 hours, check wear debris and temperature.

- If successful, roll out to all machines. Document in GS-EN-F09 (Eco-material substitution log).

4.2.2 Substitution of VOCs / Solvents

Current use: Isopropyl alcohol (IPA), acetone, or toluene for cleaning glass edges, cells, and machine parts.

Alternatives:

- Water-based degreasers (low VOC, typically <5%).

- Citrus-based terpene cleaners (derived from orange peels) – effective for flux residues.

- Pressurised CO₂ cleaning (dry ice blasting) – no chemical waste.

Procedure:

- Evaluate cleaning effectiveness using contact angle measurement or residue test.

- Switch to water-based cleaner for routine cleaning; keep small quantity of IPA only for critical applications (approved by HSE).

- Train operators on proper dilution and disposal (water-based cleaner can go to drain only if pH neutral and no heavy metals – confirm with local ETP).

4.2.3 Biodegradable / Renewable Materials for Internal Logistics

Objective: Replace single-use plastics and non-renewable materials in warehousing and internal transport.

| Current material | Eco-friendly alternative |
| --- | --- |
| Stretch wrap (LDPE) | Biodegradable stretch film (e.g., based on PBAT + PLA) |
| Plastic pallets (HDPE) | Reusable wooden pallets from FSC-certified timber, or recycled plastic pallets |
| Bubble wrap | Corrugated cardboard padding or molded pulp |
| Polypropylene strapping | Jute or biodegradable strapping (for light loads) |

Procedure:

- Inventory all single-use logistics materials.

- For each, identify an alternative that is either reusable, recyclable, or biodegradable.

- Conduct cost and performance comparison (e.g., load stability, tear resistance).

- Phase in alternatives over 12 months. Track quantity of non-renewable plastic reduced (kg/month).

4.3 Emergency Preparedness for Environmental Risks

4.3.1 Identification of Potential Environmental Emergencies

At Gautam Solar sites, the following scenarios could cause environmental harm:

| Hazard | Potential incident | Environmental impact |
| --- | --- | --- |
| Soldering flux (rosin-based, contains alcohols) | Spill of 20+ litres | Groundwater contamination, aquatic toxicity |
| Cleaning solvents (IPA, acetone) | Drum rupture, fire | Soil/water contamination, VOC release |
| Cell breakage (trace heavy metals – silver, lead, tin) | Improper disposal of broken cells | Heavy metal leaching into soil |
| Battery acid (from forklift batteries) | Leak or overfill | Acidification of soil/water, corrosion |
| Cooling water additives (corrosion inhibitors – may contain chlorinated compounds) | Uncontrolled discharge | Toxicity to aquatic life |
| Lubricating oil (from machines) | Leak from hydraulic line | Oil contamination of floor wash water |

Procedure – Risk assessment:

- Conduct a Hazard Identification and Risk Assessment (HIRA) for environmental aspects annually.

- For each scenario with potential impact severity “High”, develop a specific Emergency Response Plan (ERP).

4.3.2 Emergency Response Plans & Documentation

Each site maintains a Site-Specific ERP with the following sections:

- Emergency contact list (internal: HSE Manager, ERT leader; external: fire brigade, pollution control board, nearest hospital).

- Spill response procedure (step-by-step for small vs. large spills).

- Spill kit locations (marked on site map).

- Evacuation routes and assembly points.

- Communication protocol (who notifies which authority, when).

- Post-incident assessment (root cause analysis, corrective actions).

Revisions: The ERP is reviewed and revised:

- Annually (scheduled).

- After any actual environmental emergency.

- After any change in operations (new chemicals, new equipment).

Revisions are documented in a Revision Log (Appendix A) and communicated to all employees via email and notice board.

4.3.3 Testing of Emergency Procedures & Equipment

Schedule:

| Activity | Frequency | Responsible |
| --- | --- | --- |
| Spill drill (tabletop or full-scale) | Quarterly (each site) | HSE Manager |
| Evacuation drill (fire / chemical release) | Half-yearly | HSE Manager + ERT |
| Inspection of spill kits (contents, expiry) | Monthly | Store / HSE |
| Testing of emergency showers/eyewash | Weekly | Maintenance |

Drill documentation:

- Use GS-EN-F10 – Emergency Drill Report (date, scenario, participants, time to respond, observations, corrective actions).

- After each drill, conduct a hot wash (immediate feedback session) and update the ERP if gaps are found.

4.3.4 Effectiveness Assessment of Response

After any real emergency (or major drill), the HSE Manager leads a Post-Incident Effectiveness Assessment:

- Response time vs. target (e.g., spill contained within 10 minutes).

- Proper use of PPE and spill kits (observed).

- Communication effectiveness (were external authorities notified correctly?).

- Environmental impact (was any contamination measured? soil/water samples if needed).

- Corrective actions (update training, repair equipment, revise ERP).

Results recorded in GS-EN-F11 – Emergency Response Effectiveness Report. Share summary with management quarterly.

4.4 Labeling, Storage, Handling & Transport of Hazardous Substances

4.4.1 Labeling (GHS Compliant)

Requirement: Every container of hazardous process material (original or secondary) must have a GHS label (Globally Harmonized System of Classification and Labelling of Chemicals).

Label elements (minimum):

| Element | Example |
| --- | --- |
| Product identifier | “Isopropyl Alcohol 99%” |
| Signal word | DANGER or WARNING |
| Hazard pictogram(s) | Flame, Health hazard, Corrosion |
| Hazard statements (H-phrases) | H225 (Highly flammable), H319 (Causes serious eye irritation) |
| Precautionary statements (P-phrases) | P210 (Keep away from heat), P280 (Wear gloves/eye protection) |
| Supplier information | Name, address, emergency phone |

Procedure:

- All incoming chemicals must have original GHS label. Reject unlabelled drums.

- For secondary containers (e.g., spray bottles, small dispensers), attach a durable GHS label with same information.

- Store responsible (HSE) conducts weekly labelling inspection using GS-EN-F12 (Hazardous Substance Label Check).

4.4.2 Storage – Spill Prevention & Segregation

Requirements:

- Secondary containment: All liquid hazardous substances >5 litres must be stored on spill pallets (integral sump) or in bunded areas. Capacity of secondary containment must be ≥110% of largest container.

- Segregation: Incompatible chemicals stored separately (e.g., acids away from bases, oxidisers away from flammables). Use chemical compatibility chart (Appendix B).

- Ventilation: Flammable storage cabinet must be vented to outside.

- Temperature control: Store solvents away from heat sources; refrigerated storage for peroxidisable chemicals.

- Limit quantities: No more than 200 litres of flammable liquid outside of flammable cabinet.

Procedure for storage area inspection (monthly):

- Check for leaks, rust, damaged containers.

- Verify spill pallet drain valve is closed.

- Ensure fire extinguisher (foam or CO₂) is present and inspected.

- Record on GS-EN-F13 – Hazardous Storage Inspection Log.

4.4.3 Handling – Exposure Reduction

Engineering controls:

| Hazard | Control equipment |
| --- | --- |
| Chemical splashes (acids, solvents) | Local exhaust ventilation (LEV) at dispensing point |
| Inhalation of vapours | Fume hood or snorkel extractor |
| Spills during transfer | Drip trays under drums; self-closing taps |
| Mechanical lifting of heavy drums | Drum trolley, pallet lifter, overhead crane |
| Contact with corrosive substances | Protective barriers (splash guards) around dispensing area |

Administrative controls:

- Safe work procedure (SWP) for each hazardous substance (available near point of use).

- Mandatory PPE as per SDS (e.g., nitrile gloves, chemical goggles, apron, face shield).

- No eating/drinking in storage or handling areas.

Procedure for handling:

- Operator reviews SWP before starting task.

- Checks that LEV is operating (airflow indicator).

- Puts on required PPE (inspection checklist: GS-EN-F14).

- Uses sealed dispensing system (pump or tap) – no open pouring.

- Immediately cleans any spill with spill kit.

- Records any incidents in near-miss log.

4.4.4 Transport – On-site & Off-site

On-site transport (between stores and production):

- Use sealed, spill-proof secondary containers (e.g., chemical-resistant tote or cart with drip tray).

- For drums, use drum dolly with strap – never roll on floor.

- Establish designated route that avoids pedestrian heavy areas.

Off-site transport (supply to Gautam Solar, or disposal to vendor):

- Only authorised transporters with valid hazardous material (HAZMAT) license.

- Packaging must comply with UN certification (e.g., UN 1H1 for plastic drums).

- Shipping documents include proper shipping name, UN number, class, packing group, and emergency contact.

Procedure for receiving:

- Before unloading, verify packaging integrity (no leaks).

- Check that SDS accompanies shipment.

- Reject any leaking or damaged containers.

4.4.5 Safety Signs & Employee Instructions

Post the following signs at all hazardous substance storage and handling areas:

- “GHS Hazard Pictograms” (reference chart)

- “Chemical Spill Procedure” (simplified flow chart)

- “Wear PPE” (with pictograms of required gear)

- “Emergency Contact” numbers

- “No Smoking / No Open Flames”

Employee training:

- Initial training on hazardous substance management (2 hours) for all operators handling chemicals.

- Refresher annually including a short written assessment.

- Training records maintained by HSE (GS-EN-F15).

4.5 Treatment & Safe Disposal of Hazardous Waste

4.5.1 Identification of Hazardous Waste Streams

| Waste stream | Classification (as per Hazardous Waste Management Rules, India) | Hazardous constituent |
| --- | --- | --- |
| Spent solvents (IPA, acetone, toluene) | Scheduled waste – A110 (organic solvents) | Flammable, toxic |
| Used oil (lubricating, hydraulic) | A109 | Toxic, bioaccumulative |
| Empty chemical containers (drums, cans) | A115 (contaminated containers) | Residue of original chemical |
| Contaminated rags / absorbents | A112 (spent absorbents) | Adsorbed hazardous substance |
| Broken solar cells (trace heavy metals) | A210 (waste containing heavy metals) | Silver, lead, tin |
| Expired adhesives / sealants | A110 / A304 | Polymers with solvents |
| Waste coolant (ethylene glycol) | A111 | Toxic to aquatic life |

4.5.2 Treatment Procedures

On-site treatment (where permitted and safe):

| Waste type | Treatment method | End state |
| --- | --- | --- |
| Solvent mixtures (non-halogenated) | Distillation (if volume >500 L/year) | Recovered solvent used as cleaning fluid; residue as fuel blend |
| Acidic / basic aqueous waste | Neutralisation (pH 6–9) followed by evaporation | Dry salt disposed as non-hazardous |
| Used oil | Filtration and re-use (for non-critical lubrication) | Reconditioned oil |

Note: Only trained personnel with proper equipment (still, neutralisation tank) may perform on-site treatment. All treatment steps must be logged (GS-EN-F16 – Hazardous Waste Treatment Log).

For wastes not treated on-site: Send to authorised Common Hazardous Waste Treatment, Storage and Disposal Facility (CHWTSDF) or authorised recycler.

5.3 Safe Disposal Protocols

Procedure for off-site disposal:

- Accumulate hazardous waste in labelled, sealed containers in a dedicated hazardous waste storage area (bunded, under roof, with spill kit).

- Quantity limit: Do not store more than 1,000 kg or 3 months’ generation (whichever lower).

- Manifest: Prepare Form 9 (as per Hazardous Waste Rules) for each shipment.

- Authorised transporter: Use only registered vehicles with GPS tracking.

- Disposal facility: Verify CHWTSDF license and that they use approved methods (incineration with flue gas cleaning, stabilisation/solidification, secured landfill).

- Obtain disposal certificate – proof that waste was received and treated (retain for 5 years).

Prohibited actions:

- No dilution of hazardous waste with non-hazardous waste.

- No dumping in municipal landfill.

- No burning in open.

- No discharge to drain unless treated to permissible limits (consent from State Pollution Control Board).

4.5.4 Inspections of Hazardous Waste Management

| Inspection type | Frequency | Responsible |
| --- | --- | --- |
| Hazardous waste storage area | Weekly | Store + HSE |
| Containers for leaks / labels | Daily | Operator |
| Manifest and disposal certificates | Quarterly audit | HSE Manager |
| Compliance with consent conditions | Annually (external) | Third-party auditor |

Findings recorded in GS-EN-F17 – Hazardous Waste Inspection Report. Non-conformances must be corrected within 7 days.

4.6 Reduction of Hazardous Substances through Greener Alternatives

4.6.1 Replacement of Heavy Metals & Halogenated Compounds

Current hazardous substances (examples):

| Substance | Use at Gautam Solar | Hazard |
| --- | --- | --- |
| Lead (Pb) – in solders | Cell tabbing and stringing (lead-based solder alloy) | Neurotoxic, bioaccumulative |
| Antimony trioxide (flame retardant) | In junction boxes / cable sheathing | Possible carcinogen |
| Decabromodiphenyl ether (decaBDE) – a POP | Some older cable materials (now phased out) | Persistent, bioaccumulative, toxic |
| Chlorinated paraffins | Lubricant additives | Persistent, toxic to aquatic life |

Greener alternatives:

| Application | Alternative | Benefit |
| --- | --- | --- |
| Solder | Lead-free solder (Sn96.5/Ag3.0/Cu0.5 – SAC305) or Sn100C (with nickel/germanium) | No lead; slightly higher melting point but acceptable for PV |
| Flame retardant in plastics | Phosphorus-based (e.g., resorcinol bis(diphenyl phosphate) – RDP) or aluminium trihydroxide (non-halogenated) | Non-bioaccumulative, lower toxicity |
| Flux activators | Water-soluble organic acids (citric, succinic) instead of halogenated activators | No halogens, biodegradable |
| Cleaning solvents | Water-based or terpene-based instead of chlorinated solvents (e.g., trichloroethylene) | No chlorinated VOCs, no ozone depletion |

Procedure for substitution:

- Engineering and HSE identify a candidate replacement.

- Review SDS and compare hazard profile (GHS statements).

- Test performance (solder joint strength, flame retardancy rating UL94 V-0).

- Obtain management approval (Production, QA, HSE).

- Update purchasing specifications to exclude the hazardous substance.

- Train operators on new material.

Tracking: Maintain GS-EN-F18 – Hazardous Substance Substitution Log, listing replaced substance, new substance, date, and quantity of hazardous material eliminated (kg/year).

4.6.2 Lead-Free Initiative

Target: All new soldering processes to be lead-free by [target date – e.g., 31 December 2026].

Transition plan:

- Phase 1: Evaluate lead-free solder on one stringer machine (adjust temperature profile, test peel strength, EL images).

- Phase 2: Train operators on handling (higher temperature requires different flux and longer dwell time).

- Phase 3: Roll out to all production lines.

- Phase 4: Dispose of lead-containing solder waste via authorised recycler.

4.6.3 Eco-friendly Flame Retardants

For any new purchase of junction boxes, connectors, or cable trays:

- Specify halogen-free flame retardant (HF-FR) meeting IEC 61249-2-21 (halogen content <1500 ppm Cl, Br, total <1500 ppm).

- Prefer phosphinate or phosphonate based FRs.

4.7 POPs (Stockholm Convention) Compliance

4.7.1 Identification of POPs in Process Materials

Persistent Organic Pollutants (POPs) relevant to solar manufacturing (historical uses – most are already restricted):

| POP substance | Potential presence | Action required |
| --- | --- | --- |
| Decabromodiphenyl ether (decaBDE) | Old cable insulation, plastic housings | Prohibited – replace immediately if found. |
| Hexabromocyclododecane (HBCDD) | Polystyrene foam packaging (rare) | Prohibited – use only POP-free packaging. |
| Perfluorooctanoic acid (PFOA) | Some non-stick coatings, flux residues (unlikely) | Check SDS for any fluorinated surfactant – substitute. |
| Short-chain chlorinated paraffins (SCCPs) | Lubricants, metalworking fluids | Prohibit purchase. Use bio-based alternatives. |

Procedure:

- HSE Manager maintains a POPs Declaration from all chemical suppliers (form GS-EN-F19), confirming that their product does not contain any POPs listed in the Stockholm Convention (current annexes A, B, C).

- For any material that may contain POPs (e.g., legacy stock), send a sample to accredited lab for analysis.

- If POPs found > 50 ppm (threshold), quarantine the material and cease use.

4.7.2 Process Modification to Limit POPs Release

Even if no POPs are intentionally used, some processes could generate POPs as by-products (e.g., incomplete combustion of chlorinated materials). At Gautam Solar, no combustion processes exist, so risk is very low. However:

- Ensure that any thermal treatment of waste (if done on-site) operates at >1000°C with sufficient residence time to destroy any POPs – but Gautam Solar does not incinerate waste. All hazardous waste sent to CHWTSDF where incinerators are designed for POPs destruction.

4.7.3 Management of Waste Containing POPs

If waste is classified as hazardous due to POPs (e.g., old cables with decaBDE), it must be destroyed or irreversibly transformed so that it no longer persists. Allowed methods per Stockholm Convention:

- High-temperature incineration (>1100°C with residence time >2 seconds).

- Plasma arc treatment.

- Cement kiln co-processing with proper gas cleaning.

Procedure:

- Label the waste “CONTAINS POPS – DESTRUCTION REQUIRED”.

- Send only to a facility with authorisation for POPs waste destruction.

- Obtain certificate of destruction (including destruction efficiency >99.9999%).

- Record in GS-EN-F20 – POPs Waste Management Log.

4.8 Mercury (Minamata Convention) Compliance

4.8.1 Mercury-Containing Equipment Inventory

Gautam Solar identifies and replaces mercury-containing devices:

| Equipment | Mercury content | Alternative |
| --- | --- | --- |
| Fluorescent tube lights (T8, T5) | 3–15 mg per tube | LED tubes (zero mercury) |
| Compact fluorescent lamps (CFL) | 2–5 mg | LED bulbs |
| Mercury thermometers | 500 mg – 3 g | Digital thermometers (thermistor or IR) |
| Mercury manometers / pressure gauges | 1–2 kg (large) | Digital pressure transducers |
| Mercury tilt switches (older equipment) | 1–10 g | Solid-state relays / proximity sensors |

Procedure – inventory:

- Conduct site-wide audit using GS-EN-F21 – Mercury Equipment Inventory.

- For each item, record type, quantity, mercury content (from manufacturer or estimate), and location.

Procedure – replacement:

- For lights: Replace all fluorescent and CFL with LED as they burn out or within 2 years (whichever earlier).

- For thermometers / gauges: Immediate replacement (cost low, risk high if breakage).

- Securely store removed mercury-containing devices in unbreakable, sealed container labelled “Hazardous Waste – Mercury”.

4.8.2 Mercury Capture & Treatment

No intentional use of mercury in solar manufacturing. However, if a mercury device breaks (e.g., fluorescent tube shatters), follow this mercury spill procedure:

- Evacuate area immediately.

- Open windows for ventilation.

- Do not use vacuum cleaner (spreads mercury vapour).

- Use mercury spill kit (sulphur powder or special absorbent) to collect visible droplets.

- Place all contaminated materials (rags, powder, broken glass) into a sealed plastic bag labelled “MERCURY WASTE”.

- Send waste to authorised mercury recycler / CHWTSDF that can recover mercury.

Closed-loop systems: Not applicable to mercury at Gautam Solar (no mercury-using processes). But for completeness, if any mercury-containing equipment is used (e.g., old rectifiers), they must be enclosed with local exhaust ventilation to capture any vapour.

4.8.3 Process Optimisation to Eliminate Mercury

- Wherever mercury was used in analytical equipment (e.g., mercury porosimeter – not present), switch to alternative techniques (gas adsorption, pycnometry).

- Ensure that any third-party laboratory used for testing does not use mercury-based analytical methods for Gautam Solar samples.

Compliance statement: Gautam Solar certifies that it does not use mercury in any manufacturing process and has replaced all mercury-containing equipment at its facilities (or is on a documented phase-out schedule). This is aligned with the Minamata Convention on Mercury (ratified by India in 2018).

5. Records & Forms (Summary Table)

| Form ID | Name | Retention |
| --- | --- | --- |
| GS-EN-F09 | Eco-material substitution log | 5 years |
| GS-EN-F10 | Emergency drill report | 3 years |
| GS-EN-F11 | Emergency response effectiveness report | 5 years |
| GS-EN-F12 | Hazardous substance label check | 2 years |
| GS-EN-F13 | Hazardous storage inspection log | 3 years |
| GS-EN-F14 | PPE inspection checklist (handling) | 2 years |
| GS-EN-F15 | Hazardous substance training record | Employee tenure + 3 years |
| GS-EN-F16 | Hazardous waste treatment log | 5 years |
| GS-EN-F17 | Hazardous waste inspection report | 5 years |
| GS-EN-F18 | Hazardous substance substitution log | 10 years |
| GS-EN-F19 | Supplier POPs declaration | 5 years |
| GS-EN-F20 | POPs waste management log | 10 years |
| GS-EN-F21 | Mercury equipment inventory | Life of equipment + 5 years |

All forms are stored electronically in \\GautamSolar\HSE\Environmental_SOP\.

6. References & Regulations

| Regulation / Convention | Relevance |
| --- | --- |
| Stockholm Convention on Persistent Organic Pollutants (2001) | POPs elimination, destruction of POPs waste |
| Minamata Convention on Mercury (2013) | Phase-out of mercury-containing products, spill management |
| GHS (Globally Harmonized System) Rev. 9 | Labeling, SDS requirements |
| Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016 (India) | Classification, manifest, disposal |
| The Environment (Protection) Act, 1986 (India) | General environmental compliance |
| IEC 61249-2-21 | Halogen-free materials in electronics |
| ISO 14001:2015 | Environmental management system framework |
| RoHS Directive (2011/65/EU) | Restriction of lead, mercury, cadmium, PBB, PBDE (reference for lead-free) |

7. Appendices

Appendix A – Sample Emergency Response Plan Revision Log

| Version | Date | Revised by | Description of change |
| --- | --- | --- | --- |
| 1.0 | 01-Jan-2025 | HSE Manager | Initial release |
| 1.1 | 15-Mar-2025 | HSE Manager | Updated spill kit locations; added battery charging area |

Appendix B – Chemical Segregation Quick Chart (Poster)

| Chemical class | Store with | Do NOT store with |
| --- | --- | --- |
| Flammable liquids (IPA, acetone) | Separate flammable cabinet | Oxidisers, acids |
| Acids (HCl, H2SO4) | Acid cabinet, away from bases | Bases, oxidisers, flammables |
| Bases (NaOH, KOH) | Base cabinet | Acids, organic peroxides |
| Oxidisers (hydrogen peroxide) | Cool, dry, away from organics | Flammables, reducing agents |
| Lubricants / oils | General storage, spill pallet | Oxidisers, strong acids |

Appendix C – Mercury Spill Kit Contents (to be kept in maintenance shop)

- 2 pairs nitrile gloves, safety goggles, respirator (P100 for mercury vapour)

- Sulphur powder (to amalgamate mercury)

- Stiff cardboard / plastic scraper

- Adhesive tape (to pick up small droplets)

- Sealable plastic bags (6 mil thickness)

- Waste label “MERCURY WASTE – TOXIC”

| SOP No. | GS-WM-001 |
| --- | --- |
| Version | 1.0 |
| Title | # Integrated Waste Management and Resource Recovery |
| Department | HSE / Administration / IT |

1. Purpose

This SOP establishes a comprehensive framework at Gautam Solar to:

- Reduce internal waste generation through reuse, recovery of scrap metal, equipment, electronic components, and repurposing of used infrastructure.

- Sort operational waste into distinct streams (organic, paper, glass, metals, plastics, hazardous waste, e-waste) using labeled bins and partnerships with waste management companies.

- Implement waste stream mapping – a systematic analysis tracking each waste type from generation to final disposal, identifying efficiency gaps and improvement opportunities.

- Refurbish IT equipment (computers, monitors, servers, phones) to extend useful life, reduce electronic waste, using internal or third-party specialists.

- Sort internal electronic waste (used IT equipment, batteries, phones, cables) separately from other waste, with clear guidelines and partnerships with authorised e-waste recyclers.

This SOP covers waste generated from Gautam Solar’s own operations (offices, factories, warehouses) and explicitly excludes waste from products manufactured by the company (end-of-life solar panels – covered under a separate product stewardship SOP).

2. Scope – Detailed

The SOP applies to all waste generated at all three Gautam Solar sites:

| Waste Category | Examples | Relevant Section |
| --- | --- | --- |
| Scrap metal | Aluminium frame offcuts, copper ribbon trims, steel from old racks, tin/lead solder dross | 4.1 (Reduction & reuse) |
| Electronic components | Old circuit boards from decommissioned equipment, sensors, relays, power supplies | 4.1, 4.5 |
| Used equipment | Old laminator parts, conveyor rollers, pneumatic cylinders, pumps | 4.1 (Repurposing) |
| Operational waste (sorted) | Food waste (canteen), paper/cardboard, glass bottles, plastic packaging, general non-hazardous | 4.2 (Segregation) |
| Hazardous waste | Used oil, spent solvents, chemical containers, contaminated rags | 4.2 (Separate stream) |
| E-waste (internal) | End-of-life computers, laptops, monitors, printers, UPS batteries, mobile phones, network switches | 4.4 (Refurbishment), 4.5 (Sorting & recycling) |

Exclusions:

- Waste generated from manufacturing processes that is already covered under hazardous waste disposal (GS-EN-002 Section 4.5) – but this SOP will include cross-references for consistency.

- Finished solar modules at end of life (not applicable to internal operations).

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| HSE Manager | Overall owner of this SOP; conducts waste stream mapping annually; reports waste reduction metrics to management. |
| Administration Manager | Manages office waste segregation (paper, plastic, organic); contracts waste collection vendors. |
| Production Manager | Implements scrap metal recovery and repurposing of used equipment; tracks weight of recyclable metals. |
| IT Manager | Oversees IT equipment refurbishment program; maintains inventory of retired IT assets; coordinates with third-party refurbishers/e-waste recyclers. |
| Store / Warehouse | Provides labeled bins and containers; ensures waste is stored in designated areas before collection. |
| All Employees | Follow waste segregation instructions; place waste in correct bins; report any improper disposal. |

4. Detailed Procedures

4.1 Reduction of Internal Waste Generation – Reuse, Recovery & Repurposing

4.1.1 Scrap Metal Recovery

Objective: Recover valuable metals (aluminium, copper, steel, tin) from production and maintenance activities, sending them to authorised recyclers instead of landfill.

Sources of scrap metal at Gautam Solar:

| Source | Material | Estimated monthly quantity | Action |
| --- | --- | --- | --- |
| Frame cutting (aluminium profiles) | Aluminium (6063 alloy) | 50–100 kg | Recycle – sell to scrap dealer |
| Ribbon slitting (copper interconnector) | Copper (99.9%) | 5–10 kg | Recycle – separate from aluminium |
| Solder dross (from stringer solder pots) | Tin/lead (or lead-free) | 1–2 kg | Hazardous? Lead-containing → hazardous waste; lead-free → recyclable (check local rules) |
| Old steel racks, conveyor parts | Steel | Occasional | Reuse or scrap |

Procedure for scrap metal management:

| Step | Action | Responsible |
| --- | --- | --- |
| 1 | Place dedicated metal scrap bins (colour-coded: blue for aluminium, yellow for copper, grey for steel) at each production area. | Production |
| 2 | Operators discard offcuts and trimmings into the correct bin immediately after cutting. Do not mix with general waste. | Operator |
| 3 | At end of each shift, weigh each bin using platform scale. Record weight in GS-WM-F01 – Scrap Metal Log. | Shift Supervisor |
| 4 | When bin is full, transfer to secured scrap storage area (locked cage to prevent theft). | Store |
| 5 | Weekly, authorised scrap dealer collects. Issue receipt with weight and payment. | Admin |
| 6 | Reconcile monthly: total scrap sold vs. expected yield (calculated from raw material input). Investigate discrepancies >5%. | HSE + Production |

Target: Recover 90% of all ferrous and non-ferrous metal waste. Track metal recycling rate = (kg recycled) / (kg generated) × 100%.

4.1.2 Reuse of Equipment & Electronic Components

Objective: Extend life of components removed from decommissioned machines, repairing and reusing them in other equipment or as spare parts.

Candidate components for reuse:

| Component | Typical lifespan | Reuse opportunity |
| --- | --- | --- |
| Electric motors (IE2/IE3) | 15 years | Test windings, replace bearings → reuse on conveyor or fan |
| Contactors, relays, switches | 10–20 years | Clean contacts, test coil → use as spare for older machines |
| Power supplies (24V DC) | 5–10 years | Replace capacitors → reuse for control panels |
| Sensors (photoelectric, inductive) | 5–8 years | Clean lens, test output → reuse on other lines |
| Pneumatic cylinders, valves | 5–10 years | Replace seals → reuse for non-critical actuation |
| Conveyor rollers, bearings | 5 years | Re-grease, replace bearing → reuse on new conveyor section |

Procedure – Reuse program:

| Step | Action |
| --- | --- |
| 1 | When equipment is decommissioned, Maintenance performs teardown and inspection using GS-WM-F02 – Component Reuse Assessment. |
| 2 | Assess each component: visual, functional test (e.g., motor winding resistance, contactor coil continuity). |
| 3 | Classify as: Reusable as is – clean, test, label, store in designated “Reusable Spares” cabinet. Repairable – send to workshop for refurbishment (replace bearings, seals, capacitors). Not reusable – send to scrap metal or e-waste stream. |
| 4 | Tag each reusable component with “RECONDITIONED – TESTED” and date. |
| 5 | Update spare parts inventory system to include reconditioned parts at zero cost (but tracked for performance). |
| 6 | When a machine needs a replacement, first check reconditioned stock before purchasing new. |

Performance metric: Number of reconditioned components reused per month, and cost saved (new purchase cost avoided). Report quarterly.

4.1.3 Repurposing Used Equipment & Infrastructure

Objective: Give old equipment a second life in a different role rather than scrapping.

Examples at Gautam Solar:

| Old equipment | Repurposed use |
| --- | --- |
| Old stringer machine (no longer meets speed/quality standards) | Use as training simulator for new operators (no production, only practice). |
| Defective laminator (heating system failed) | Remove heating elements, use as manual layup table or vacuum press for small R&D samples. |
| Old office racks (from server room) | Use as storage shelving in warehouse. |
| Used wooden pallets (broken) | Repair and reuse for internal material transport; if beyond repair, chip for biomass fuel (if permitted) or send to authorised recycler. |

Procedure:

- When a major asset is retired, the Department Head (Production/IT/Admin) completes a Repurposing Evaluation Form (GS-WM-F03).

- Evaluate if the asset can be safely and usefully redeployed elsewhere in Gautam Solar (or donated to a technical school/NGO).

- If repurposing is approved, update asset register and physically relocate with proper cleaning/safety checks.

- If not repurposable, then proceed to disassembly for component reuse (Section 4.1.2) and then recycling.

Donation: For equipment that is functional but no longer needed (e.g., old computers after refurbishment, office furniture), donate to local schools or charities. Document donation with acknowledgment letter (GS-WM-F04). This avoids waste and benefits community.

4.2 Operational Waste Sorting into Respective Streams

4.2.1 Waste Stream Categories

Gautam Solar implements source segregation at all sites. Employees separate waste into the following streams:

| Stream | Colour Code | Typical Contents | Disposal / Treatment Path |
| --- | --- | --- | --- |
| Organic / Food waste | Green bin | Canteen leftovers, fruit peels, coffee grounds | Sent to compost facility or organic waste converter (if on-site) |
| Paper & Cardboard | Blue bin | Office paper, newspaper, cardboard boxes, corrugated sheets | Recycled through paper recycler |
| Glass | White/clear bin | Glass bottles, jars (from canteen, lab) | Crushed and sent to glass recycler |
| Metals (non-hazardous) | Grey bin | Aluminium cans, metal caps, small scrap | Sold to metal recycler |
| Plastics (recyclable) | Yellow bin | PET bottles, HDPE containers (clean), plastic packaging film | Sent to plastic recycler |
| General waste (non-recyclable) | Black bin | Dirty paper, multi-layer packaging, food-soiled items, sanitary waste | To municipal landfill (minimised) |
| Hazardous waste | Red bin (with hazard label) | Used oil, spent solvents, chemical containers, contaminated rags, batteries | Stored separately, sent to CHWTSDF (per GS-EN-002) |
| E-waste | Orange bin | Old cables, chargers, small electronic components, toner cartridges, UPS batteries | Separate e-waste recycler (Section 4.5) |

4.2.2 Labeled Bins & Placement

Requirements:

- Each bin must have a clear, durable label showing:

  - Waste stream name and colour code.

  - Pictograms (universal symbols for paper, plastic, glass, etc.).

  - List of acceptable items and common contaminants (e.g., “No food-soiled paper in paper bin”).

- Bins are placed at convenient locations:

  - Each workstation (production) – small metal/plastic bins.

  - Break rooms and canteen – organic, paper, plastic, general.

  - Office corridors – paper, plastic, general.

  - Near printer/copier – paper bin (for misprints).

  - Chemical storage area – hazardous waste bin.

Inspection:

- Daily: Housekeeping team checks that bins are used correctly.

- Weekly: HSE or Admin reviews contamination (e.g., food waste in paper bin). Contaminated bins trigger retraining for that department.

4.2.3 Partnerships with Waste Management Companies

Selection criteria for vendors:

| Waste stream | Vendor type | Required credentials |
| --- | --- | --- |
| Paper, plastic, glass, metals | Authorised recycler | Consent from State Pollution Control Board (SPCB) |
| Organic waste | Compost facility or biogas plant | SPCB registration |
| Hazardous waste | CHWTSDF | Authorisation under Hazardous Waste Rules |
| E-waste | E-waste recycler | Authorisation under E-Waste (Management) Rules |

Procedure:

- HSE maintains an Approved Waste Vendor List (GS-WM-F05) with contact details, license numbers, and validity dates.

- Contracts include performance clauses: vendor must provide weight receipt and disposal certificate for each collection.

- Quarterly, HSE conducts a vendor audit (desk review of their compliance, no need to visit unless high risk).

4.3 Waste Stream Mapping – Systematic Analysis

4.3.1 What is Waste Stream Mapping?

Waste stream mapping is a visual and analytical tool that tracks a specific waste type from its point of generation (e.g., production line, office desk) to its final destination (recycling, incineration, landfill). It helps identify inefficiencies: unnecessary handling steps, mixing of incompatible wastes, missing bins, or costly disposal routes.

For each major waste stream, Gautam Solar will create a “Waste Stream Map” (see example below).

4.3.2 Step-by-Step Mapping Procedure

| Step | Action | Output |
| --- | --- | --- |
| 1 | Select a waste stream (e.g., aluminium scrap from frame cutting). | – |
| 2 | Walk the physical flow: observe where waste is generated, collected, stored, transported, and finally disposed. | Process flow diagram (draft) |
| 3 | Interview operators and storekeepers: “What happens to this waste? Are there delays? Is it ever mixed with other waste?” | Qualitative data |
| 4 | Measure quantities at each stage (weigh at generation, weigh after storage, weigh at dispatch). | Quantitative data (mass balance) |
| 5 | Identify gaps: e.g., “Scrap bin not labeled → operators put aluminium into general waste” or “No dedicated metal bin in cutting area → scrap ends up in plastic bin”. | List of improvements |
| 6 | Create final Waste Stream Map using standard symbols (see Appendix A). | Visual map (digital) |
| 7 | Implement improvements and re-map after 6 months to verify progress. | Updated map |

4.3.3 Example Waste Stream Map (Textual Description)

Stream: Aluminium frame cutting scrap at Haridwar Factory

| Stage | Description | Quantity (kg/week) | Efficiency / Issue |
| --- | --- | --- | --- |
| 1. Generation | Frame cutting machine produces scrap ends (5–10 cm pieces). | 100 kg | – |
| 2. Collection | Operator sweeps scrap into open tub (no label). Sometimes metal and plastic mixed. | 95 kg (5 kg lost to floor) | No segregation, 5% uncollected |
| 3. Intermediate storage | Tub placed near machine. Plastic scrap from packaging also thrown in. | 90 kg (after mixing) | Contamination with plastic |
| 4. Transport to central scrap area | Forklift takes tub to scrap cage. | 90 kg | – |
| 5. Central storage | Scrap cage (aluminium only). But plastic contamination requires manual sorting. | 85 kg (after sorting) | Labour cost for sorting |
| 6. Dispatch to recycler | Recycler collects, pays by weight of pure aluminium. | 85 kg | Paid only for net Al, contamination reduces revenue |

Identified improvements:

- Install labeled blue bin for aluminium scrap directly at cutting machine.

- Train operators to discard only aluminium (no plastic packaging).

- Install a tray to capture all small pieces (reduce 5% loss).

- Expected improvement: +15 kg/week recovered, +₹X revenue, less labour sorting.

Mapping frequency: Each major waste stream mapped annually (or after any process change). Completed maps stored in GS-WM-F06 – Waste Stream Mapping Records.

4.3.4 Using Maps for Improvement Planning

The HSE Manager uses maps to calculate waste diversion rate:

Diversion rate = (Total waste recycled + reused + composted) / (Total waste generated) × 100%

Target diversion rate: ≥75% by end of Year 2 (excluding hazardous waste which is treated, not landfilled).

Areas with low diversion (e.g., general waste >20%) trigger a Kaizen event to reduce contamination or find new recycling options.

4.4 Circular IT Asset Management

4.4.1 Responsible IT Asset Refurbishment

Refurbishment extends the useful life of computers, laptops, monitors, servers, and peripherals. It is preferred over recycling because it avoids the energy and material cost of manufacturing new devices. Gautam Solar will implement a tiered approach:

| Tier | Condition | Action |
| --- | --- | --- |
| Tier 1 – Like new | <3 years old, fully functional | Keep in use or re-deploy to another department |
| Tier 2 – Minor issues (slow performance, dead battery, cracked screen but functional) | Refurbish internally or via third party – replace battery, upgrade RAM/SSD, reinstall OS. | Use for less demanding roles (e.g., training room, warehouse terminal) or donate to schools. |
| Tier 3 – Major issues (motherboard failure, broken hinges, obsolete) | Part out – salvage RAM, hard drive, screen, keyboard as spares. Remaining carcass to e-waste recycler. | Maximise component reuse. |
| Tier 4 – Beyond repair (physically crushed, water damage) | Direct to e-waste recycler for material recovery (metals, plastics, glass). | Environmentally sound recycling. |

4.4.2 Refurbishment Process (Internal or Third-Party)

Internal refurbishment (for basic upgrades):

| Step | Action | Responsible |
| --- | --- | --- |
| 1 | IT department maintains an IT Asset Retirement Log (GS-WM-F07) for all devices removed from service. | IT Manager |
| 2 | Assess device: run diagnostics (memory test, hard drive health, battery cycle count). | IT Technician |
| 3 | If candidate for refurbishment: |  |

- Clean internally (dust removal) and externally.

- Replace worn parts: battery, keyboard, fan, thermal paste.

- Upgrade: add SSD, increase RAM (if cost-effective).

- Wipe hard drive using secure erase (DBAN or manufacturer tool).

- Install fresh OS (Windows or Linux lightweight).

- Perform full test (24 hours burn-in). | IT Technician |
| 4 | Apply a sticker “REFURBISHED BY GAUTAM SOLAR – [Date]”. | IT Technician |
| 5 | Re-deploy internally (to another department) or list for donation. | IT Manager + Admin |

Third-party refurbishment partnership:

- Gautam Solar may partner with an authorised e-waste refurbisher (e.g., certified by R2 or e-Stewards) for complex repairs or large volumes.

- The partner must provide a certificate stating that:

  - All data has been destroyed.

  - Refurbished devices meet functional standards.

  - Any non-refurbishable parts are recycled responsibly.

- Maintain records of devices sent for third-party refurbishment (GS-WM-F08).

4.4.3 Donation of Refurbished Equipment

- After internal needs are met, working refurbished equipment can be donated to:

  - Government schools (computer labs).

  - Non-profit organisations (NGOs).

  - Technical training institutes.

- Donation procedure:

  - Obtain written request from recipient.

  - Prepare device (data wipe, install basic software).

  - Issue donation letter (GS-WM-F09) with list of items and “no warranty, as-is” disclaimer.

  - Recipient signs acknowledgment. This counts as waste diversion (not landfilled).

- Tax benefits: Eligible under CSR spending (India). Coordinate with finance.

4.4.4 Performance Metrics – IT Refurbishment

| Metric | Formula | Target |
| --- | --- | --- |
| IT asset lifespan extension | (Months from first use to final recycling) | ≥5 years for laptops, ≥7 years for desktops |
| Refurbishment rate | Number of devices refurbished / Total devices retired | ≥60% |
| E-waste reduction | Weight of devices refurbished & donated (kg) avoided from recycling | Report annually |

4.5 Sorting & Management of Internal Electronic Waste (E-Waste)

4.5.1 Definition & Scope of Internal E-Waste

For the purpose of this SOP, internal e-waste means:

- All electrical and electronic equipment (EEE) used by Gautam Solar that has reached end-of-life, excluding the solar modules we manufacture.

- Examples: Computers, laptops, monitors, printers, scanners, UPS units (including batteries), network switches, routers, mobile phones, landline phones, cables, chargers, power supplies, electronic lab equipment (multimeters, oscilloscopes), worn-out batteries (Li-ion, lead-acid, NiMH), toner cartridges, CD/DVDs.

This excludes hazardous waste batteries that are already covered under hazardous waste stream (lead-acid batteries) – but they should be segregated as e-waste or hazardous waste according to local rules. In India, batteries are covered under Batteries (Management and Handling) Rules; for simplicity, Gautam Solar will send all batteries (except alkaline disposables) to authorised e-waste / battery recycler.

4.5.2 E-Waste Sorting System

Collection bins/containers:

| E-waste sub-stream | Container | Label | Colour |
| --- | --- | --- | --- |
| Large IT equipment (desktops, monitors, printers) | Pallet or cage | “E-WASTE – LARGE IT” | Orange background |
| Small IT (mice, keyboards, phones, chargers) | Lockable bin | “E-WASTE – SMALL” | Orange with slot |
| Batteries (all types) | Fire-resistant bin | “BATTERIES – DO NOT MIX” | Red/orange with flame icon |
| Cables & wires | Separate bag or bin | “CABLES – COPPER RECOVERY” | Yellow |
| Toner cartridges | Cardboard box | “EMPTY TONER – E-WASTE” | Orange |

Placement:

- Each office floor: small e-waste bin near printer/copy room.

- IT server room: dedicated e-waste area for retired servers, network gear.

- Factory maintenance shop: battery bin and small e-waste bin.

- Warehouse: large pallet for bulk e-waste (old UPS units, etc.).

4.5.3 Employee Guidelines for E-Waste Sorting

Clear instructions (to be posted near e-waste bins):

DO PUT in e-waste bin:

- Broken / old cables, chargers, adapters

- Dead batteries (tape terminals of Li-ion)

- Non-working mice, keyboards, webcams

- Old mobile phones, landline phones

- UPS batteries (place carefully, avoid acid spill – use secondary tray)

- Toner cartridges (keep in original bag to avoid dust)

DO NOT PUT:

- General office paper, plastic, food waste

- Working equipment (first try refurbishment – contact IT)

- Large appliances (e.g., broken AC – call maintenance separately)

- Hazardous chemical containers

Training:

- All employees receive an annual 30-minute e-waste sorting training (included in HSE induction).

- IT department conducts quarterly spot checks. If contamination >5% by volume, re-training for that department.

4.5.4 Partnership with E-Waste Recycling Companies

Selection of authorised recycler:

- Must hold valid authorisation from the State Pollution Control Board under E-Waste (Management) Rules, 2016 (India).

- Preferred certifications: R2 (Responsible Recycling) or e-Stewards.

- Recycler must provide certificate of recycling for each batch, detailing:

  - Weight of each material fraction (metals, plastics, glass, PCBs).

  - That no e-waste was exported illegally (if applicable).

  - That data destruction was verified (if devices contained storage media).

Procedure for e-waste disposal:

| Step | Action |
| --- | --- |
| 1 | E-waste bins are checked weekly by Admin. When full, transfer to central e-waste storage area (dry, locked). |
| 2 | IT department performs data destruction on any storage media (hard drives, SSDs) before handing over to recycler. Use software wipe (DoD 5220.22-M) or physical shredding (for high-security drives). |
| 3 | Maintain an E-Waste Manifest (GS-WM-F10) listing each item: type, serial number (if tracked), weight, date sent, recycler name. |
| 4 | Recycler collects on a scheduled basis (e.g., monthly or quarterly). Obtain signed manifest and weight receipt. |
| 5 | Recycler returns a Certificate of Recycling & Data Destruction within 60 days. File with HSE records. |

Prohibited actions:

- No e-waste shall be disposed of in general waste bins or landfills.

- No e-waste shall be sent to scrap dealers without authorisation for e-waste.

- No burning of cables to recover copper.

4.5.5 Tracking & Reporting

Key performance indicators (e-waste):

| Metric | Calculation | Reporting frequency |
| --- | --- | --- |
| Total e-waste generated (kg) | Sum of all e-waste sent to recycler | Monthly |
| E-waste diversion rate | (Weight refurbished + recycled) / Total e-waste generated | Monthly |
| Batteries recycled (kg) | Separate from general e-waste | Monthly |
| Data destruction compliance | % of storage media physically or cryptographically wiped | Quarterly |

5. Records & Forms (Summary)

| Form ID | Name | Retention |
| --- | --- | --- |
| GS-WM-F01 | Scrap Metal Log | 3 years |
| GS-WM-F02 | Component Reuse Assessment | 5 years |
| GS-WM-F03 | Repurposing Evaluation Form | 5 years |
| GS-WM-F04 | Donation Acknowledgment | 5 years |
| GS-WM-F05 | Approved Waste Vendor List | Current + 2 years |
| GS-WM-F06 | Waste Stream Mapping Records | 5 years (maps) |
| GS-WM-F07 | IT Asset Retirement Log | Life of asset + 3 years |
| GS-WM-F08 | Third-Party Refurbishment Record | 5 years |
| GS-WM-F09 | IT Equipment Donation Letter | 5 years |
| GS-WM-F10 | E-Waste Manifest & Certificate Log | 5 years |

All forms stored in \\GautamSolar\HSE\Waste_Management\.

6. References

| Regulation / Standard | Relevance |
| --- | --- |
| E-Waste (Management) Rules, 2016 (India) | Licensing, collection, recycling of e-waste |
| Hazardous and Other Wastes (Management) Rules, 2016 | Hazardous waste (batteries, some e-waste) |
| Plastic Waste Management Rules, 2016 | Plastic recycling (relevant to packaging) |
| Solid Waste Management Rules, 2016 | Municipal waste segregation |
| ISO 14001:2015 – Clause 8.1 (Operational planning and control) | Waste management as environmental aspect |
| R2 Standard (Responsible Recycling) | Best practice for e-waste refurbishment & recycling |

7. Appendices

Appendix A – Waste Stream Mapping Symbols (for diagrams)

| Symbol | Meaning |
| --- | --- |
| ➤ | Arrow – flow of waste |
| ◻ | Operation (e.g., collection, sorting) |
| ⬚ | Storage (temporary accumulation) |
| ⚠ | Inspection / quality check (e.g., contamination check) |
| ▼ | Disposal / final destination (recycler, landfill) |

Appendix B –Waste Stream Map Diagram (Text Description)

Stream: Office Paper

[Generation: Printer misprints, old files]

→ (Operators place in blue bin at desk)

→ [Collection: Housekeeping collects daily]

→ [Storage: Paper bin in central corridor]

→ [Inspection: Remove plastic windows from envelopes]

→ [Transport: Weekly truck to paper recycler]

→ ▼ [Recycler: Convert to pulp, new paper]

Appendix C – IT Refurbishment Decision Flowchart

Retired IT Asset

↓

Is it <5 years old and functional? → YES → Re‑deploy internally

↓ NO

Is repair cost < 50% of new? → YES → Refurbish (internal or third party)

↓ NO

Can it be used for parts (RAM, HDD, screen)? → YES → Salvage components → remaining to e‑waste

↓ NO

Send to authorised e‑waste recycler

| SOP No. | GS-WM-002 |
| --- | --- |
| Version | 1.0 |
| Title | # Water Management: Reclamation, Audits, Quality Testing, Rainwater Harvesting & Heavy Metal Discharge Reduction |
| Department | HSE / Engineering / Maintenance |

1. Purpose

This SOP establishes a comprehensive water management framework at Gautam Solar to:

- Reclaim water for internal reuse in operations through advanced treatment technologies, reducing freshwater withdrawal and minimising wastewater discharge.

- Conduct systematic water audits to assess consumption efficiency, identify water-intensive processes, and develop actionable conservation plans.

- Perform regular wastewater quality tests to ensure effluent discharges consistently comply with legal requirements (CPCB norms).

- Implement rainwater harvesting systems to capture, store, and utilise rainwater in own operations, reducing dependency on municipal or groundwater sources.

- Adopt processes and technologies to reduce heavy metal discharges (lead, mercury, cadmium, arsenic, chromium) from wastewater into water bodies.

This SOP applies to all water-consuming activities at Gautam Solar’s factory locations, including production washing, cooling towers, equipment cleaning, and domestic use in canteens and sanitation.

2. Scope

The SOP covers:

| Topic | Applicable Areas |
| --- | --- |
| Water reclamation & reuse | Membrane filtration (RO, UF), Zero Liquid Discharge (ZLD) systems, reuse of treated water in cooling towers, floor washing, gardening, and process steps. |
| Water audits | Pre-audit planning, on-site inspection, water balance analysis, benchmarking, and recommendation implementation. |
| Wastewater quality testing | Parameters: pH, BOD, COD, TSS, heavy metals, oil & grease; sampling frequency; accredited laboratory analysis; reporting to SPCB. |
| Rainwater harvesting | Catchment area (rooftops), conveyance (gutters/downpipes), filtration (first-flush, sediment filters), storage tanks, and utilisation for non-potable purposes. |
| Heavy metal discharge reduction | Chemical precipitation, ion exchange, adsorption, membrane filtration; source segregation; sludge management. |

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| HSE Manager | Overall owner of this SOP; coordinates water audits; maintains water quality records; ensures regulatory compliance; manages rainwater harvesting system. |
| Plant Head (Haridwar & Bhiwani) | Provides resources for water treatment infrastructure; reviews water conservation targets. |
| Maintenance Engineer | Operates and maintains water reclamation systems (RO, ZLD, ETP); records water meter readings; performs routine checks. |
| Production Supervisor | Minimises water consumption in process steps; reports leaks or abnormal usage; segregates heavy metal-containing wastewater. |
| Laboratory / QC (if in-house) or External Lab | Conducts wastewater quality tests per schedule; provides timely reports. |
| All Employees | Report dripping taps, running hoses, and any visible water wastage. |

4. Detailed Procedures

4.1 Water Reclamation & Internal Reuse

4.1.1 Rationale for Water Reclamation

In solar panel manufacturing, water is primarily used for:

- Cleaning glass sheets before lamination,

- Cooling circulating systems for laminators and curing ovens,

- Washing floors and equipment,

- Domestic purposes (canteens, toilets).

Reclaiming water reduces pressure on local water bodies and lowers operational costs. Gautam Solar will target a water reuse rate of ≥50% within two years and explore Zero Liquid Discharge (ZLD) as a long-term goal.

4.1.2 Technologies for Water Reclamation

| Technology | Description | Application at Gautam Solar |
| --- | --- | --- |
| Membrane Filtration (UF / MF) | Ultrafiltration or microfiltration removes suspended solids, colloids, and bacteria. It produces high-quality effluent suitable for reuse in non-critical applications. | Pre-treatment for RO; polishing of secondary effluent before reuse. |
| Reverse Osmosis (RO) | RO removes dissolved salts, heavy metals, and organic compounds, producing almost pure water. Brine (concentrate) requires further handling. | Production of demineralised water for precision cleaning; recycling of cooling tower blowdown. |
| Zero Liquid Discharge (ZLD) | A ZLD system combines RO, evaporation, and crystallisation to recover up to 95–99% of water as distillate. Only solid salts remain for safe disposal. | Long-term target for both factories to eliminate liquid effluent discharge. |
| Electrodialysis (ED) | Uses ion-exchange membranes to separate charged ions (e.g., salts, heavy metals) from wastewater. It is energy-efficient for low-to-medium salinity streams. | Potential for treating specific heavy-metal-bearing streams (e.g., from soldering flux washes). |

4.1.3 Wastewater Characterisation

Before designing any reclamation system, the following sources must be characterised:

| Source | Typical Contaminants | Possible Reuse Application |
| --- | --- | --- |
| Glass washing rinse water | Fine glass particles, dust, traces of cleaning agents | Rinsing again after proper filtration; floor washing |
| Cooling tower blowdown | Dissolved salts (calcium, magnesium), corrosion inhibitors | Toilet flushing, landscape irrigation (after softening) |
| Soldering flux cleaning effluent | Alcohols, surfactants, trace heavy metals (tin, lead, silver) | Requires heavy metal removal; then reuse |
| Domestic sewage | Organic matter, pathogens | Garden irrigation after biological treatment (STP) |

4.1.4 Operational Procedure for Reclamation System

| Step | Action | Frequency | Responsible |
| --- | --- | --- | --- |
| 1 | Collect wastewater from designated streams (e.g., cooling tower blowdown, glass rinse) into a collection sump. | Continuous | Production / Maintenance |
| 2 | Pass through primary physical treatment (screen, grit chamber) to remove large solids and debris. | Continuous | ETP Operator |
| 3 | Apply chemical coagulation and flocculation to aggregate fine suspended particles. | As needed (daily) | ETP Operator |
| 4 | Clarify in a settling tank; sludge is dewatered and disposed as per hazardous waste rules (if heavy metals present). | Continuous | ETP Operator |
| 5 | Route clarified water to membrane filtration (UF/RO) for polishing. Ensure RO membranes are periodically cleaned. | Per manufacturer schedule | Maintenance Engineer |
| 6 | Direct permeate (treated water) to a reuse storage tank. Monitor water quality (turbidity, conductivity, pH) weekly. | Weekly | Lab / Maintenance |
| 7 | Reuse the reclaimed water for compatible operations: cooling tower make-up, floor washing, gardening, toilet flushing. | Continuous | Production / Admin |
| 8 | Manage the RO concentrate / brine by evaporation (in ZLD) or sending to an authorised Common Effluent Treatment Plant (CETP). | Weekly | HSE Manager |

4.1.5 Performance Metrics

| Metric | Formula | Target |
| --- | --- | --- |
| Water reuse rate | (Reclaimed water reused, m³) / (Total water withdrawal, m³) × 100% | ≥50% within 2 years |
| Freshwater reduction | (Baseline water consumption − Current consumption) / Baseline × 100% | 10% annually |
| ZLD implementation (factory units) | – | Achieve by 2030 (one pilot line) |

All data recorded in GS-WM-F20 – Water Reclamation Log.

4.2 Water Audits

4.2.1 Definition and Scope

A water audit is a systematic study of all water entering, being used, and leaving a facility. It establishes a water balance, identifies inefficiencies, leakage, and excess use, and recommends cost-effective improvements.

4.2.2 Steps of a Comprehensive Water Audit

| Step | Activities |
| --- | --- |
| 1. Pre-Audit Planning | Form a cross-functional team (HSE, Production, Maintenance). Define scope (whole factory or specific zones). Gather 12–24 months of water bills, plumbing layouts, as-built drawings. Establish baseline (litres per module). |
| 2. On-Site Inspection & Data Collection | Verify flow meter placement and calibration. Map end-uses: cooling towers, washing stations, domestic, landscaping, fire systems. Use ultrasonic/acoustic tools to locate hidden leaks. Measure flow rates with bucket & stopwatch where necessary. |
| 3. Water Balance & Loss Analysis | Account for all inflows (municipal, borewell, rainwater) and outflows (evaporation, discharge, reuse). Calculate real losses (leaks, overflows) and apparent losses (metering errors, unauthorised use). Create a water balance diagram. |
| 4. Benchmarking & Efficiency Diagnostics | Compare with industry benchmarks (e.g., litres per module) and Indian standards (IS 17408). Identify water-intensive equipment and over-consumed areas. |
| 5. Recommendations & ROI Action Plan | Classify measures by cost and return: no-cost (fix leaks, adjust timers); low-cost (install aerators, timers); high-cost (retrofit cooling towers, install ZLD). |
| 6. Institutionalising Water Stewardship | Integrate water KPIs into monthly reviews; deploy smart metering for real-time monitoring. |

4.2.3 Audit Frequency

| Type of Audit | Frequency | Responsible |
| --- | --- | --- |
| Preliminary (walk-through) | Annually | HSE Manager |
| Detailed (full balance) | Once every 3 years | External agency (e.g., NPC) or certified consultant |
| Internal compliance check | Quarterly | Plant Head + HSE |

4.2.4 Documentation

After each detailed audit, a Water Audit Report (GS-WM-F21) shall contain:

- Water consumption and wastewater generation pattern,

- Specific water consumption (litres per module),

- Complete water balance diagram,

- Identified water saving opportunities with cost estimates,

- Implementation timeline and responsible persons.

4.3 Wastewater Quality Testing

4.3.1 Regulatory Framework

Gautam Solar shall comply with Central Pollution Control Board (CPCB) effluent discharge standards, updated in 2026 under the Environment (Protection) Rules, 1986. Key parameters are:

| Parameter | Limit | Significance |
| --- | --- | --- |
| pH | 6.5 – 8.5 | Extreme pH harms aquatic life and corrodes infrastructure. |
| Biochemical Oxygen Demand (BOD) | ≤ 10 mg/L | Measures organic pollution; high BOD depletes oxygen in receiving waters. |
| Chemical Oxygen Demand (COD) | ≤ 50 mg/L | Indicates total organic load (both biodegradable and non-biodegradable). |
| Total Suspended Solids (TSS) | ≤ 10 mg/L | High TSS causes turbidity and siltation. |
| Heavy Metals (Pb, Hg, Cd, As, Cr) | Per CPCB schedule | Toxic to humans and ecosystems; require specific treatment. |
| Oil & Grease | ≤ 10 mg/L | Causes surface films and oxygen transfer inhibition. |

These limits are legally binding and non-compliance can lead to closure notices and criminal liability.

4.3.2 Sampling Plan

| Parameter Group | Sampling Frequency | Sampling Point |
| --- | --- | --- |
| pH, temperature, colour, odour | Daily (online if possible) | Effluent outlet of ETP |
| BOD, COD, TSS | Weekly (every 7 days) | After ETP final polishing tank |
| Heavy metals (Pb, Hg, Cd, As, Cr) | Monthly (composite sample) | After heavy metal removal unit (or before discharge) |
| Oil & grease | Weekly | Effluent discharge point |
| Coliform bacteria | Monthly | Treated domestic sewage (if mixed) |

Samples must be representative of the discharge conditions and collected as per established protocols (e.g., grab samples or composite over 24 hours as appropriate).

4.3.3 Laboratory Testing

- Internal testing: For basic parameters (pH, conductivity, turbidity), trained personnel may use calibrated portable meters, with results recorded in GS-WM-F22.

- External testing: For BOD, COD, heavy metals, and coliform, samples must be sent to an NABL-accredited laboratory or a lab approved by the State Pollution Control Board.

4.3.4 Recording and Reporting

- All test results shall be entered in GS-WM-F22 – Wastewater Quality Log.

- Monthly summary reports shall be submitted to the State Pollution Control Board (if required by consent) and retained for at least 5 years.

- Online Continuous Effluent Monitoring Systems (OCEMS) shall be installed at each factory outlet if mandated by CPCB for that industry category.

4.3.5 Non-Compliance Action

If any parameter exceeds the prescribed limit:

- Immediately notify the HSE Manager and Plant Head.

- Stop discharge into the water body and redirect effluent to an emergency holding tank.

- Investigate root cause – equipment malfunction, sudden load change, inadequate chemical dosing.

- Take corrective action (e.g., adjust coagulant dosage, replace membranes, clean filters).

- Re-sample and confirm compliance before resuming discharge.

- Document the incident in GS-WM-F23 – Non-Compliance & Corrective Action Log.

4.4 Rainwater Harvesting

4.4.1 Design & Components

Rainwater harvesting captures and stores rainwater from rooftops and paved areas for later use. The system consists of:

| Component | Description |
| --- | --- |
| Catchment area | Building rooftops (factory sheds, office blocks). Total area at Haridwar and Bhiwani factories shall be calculated. |
| Conveyance system | Gutters and downpipes made of corrosion-resistant material (PVC, stainless steel). |
| First-flush diverter | Diverts the initial 1–2 mm of rainfall, which carries dust and bird droppings. |
| Filter unit | Sand / gravel / charcoal filter or a commercially available rainwater filter (mesh size ~0.2 mm). |
| Storage tank | Underground or aboveground tank (ferrocement, RCC, or prefabricated plastic). Capacity as per annual rainfall and demand. |
| Treatment (optional) | UV disinfection or chlorination if water is used for potable purposes (generally Gautam Solar will use rainwater for non-potable uses). |
| Overflow mechanism | Directs excess rainwater to stormwater drains or to recharge pits. |

4.4.2 Sizing Guidelines

- Catchment yield (annual, m³):
Catchment area (m²) × Annual rainfall (m) × Runoff coefficient

  - Haridwar: Average rainfall ≈ 1,200 mm/year.

  - Bhiwani: Average rainfall ≈ 600 mm/year.

  - Runoff coefficient for pitched roofs ≈ 0.8–0.9.

- Storage capacity should be sufficient to meet 30–40 days of non-potable demand (toilets, floor washing, gardening) during dry spells.

4.4.3 Installation Procedure

| Step | Action | Responsible |
| --- | --- | --- |
| 1 | Perform a feasibility study – measure roof areas, collect rainfall data, estimate water demand. | HSE Manager + Consultant |
| 2 | Design the system with appropriate tank size, filter type, and first-flush diverter. | External agency |
| 3 | Install gutters and downpipes at a slope of at least 1:200. Ensure downpipes are equipped with leaf screens. | Maintenance / Contractor |
| 4 | Connect downpipes to a first-flush diverter (adjustable volume). | Contractor |
| 5 | Position the filter unit before the storage tank. Provide a bypass for overflow. | Contractor |
| 6 | Install the storage tank with a manhole for cleaning, an overflow pipe to storm drain, and a draw-off tap at the bottom. | Contractor |
| 7 | Label the system with “Rainwater – Non-potable” signs at all draw-off points. | Maintenance |
| 8 | Conduct a test run with a simulated rainfall (using a hose) to verify collection and overflow. | HSE + Maintenance |

4.4.4 Operation & Maintenance

| Activity | Frequency | Responsible |
| --- | --- | --- |
| Clean gutters and leaf screens | Quarterly (before monsoon) & after heavy storms | Maintenance |
| Clean the first-flush diverter chamber | Monthly during monsoon | Maintenance |
| Backwash or replace filter media | Every 6 months (or as needed) | Maintenance |
| Inspect tank for algae, cracks, leaks | Every 3 months | Maintenance |
| Monitor water quality (turbidity, coliform) if used for non-potable purposes | Quarterly | Lab / HSE |

4.4.5 Utilisation of Harvested Rainwater

Rainwater shall be used for:

- Toilet flushing in factory and office buildings,

- Floor washing (after basic screening),

- Gardening and landscaping,

- Cooling tower make-up (if treated to required quality).

Rainwater shall never be used for drinking or cooking unless fully treated (UV + RO).

4.4.6 Performance Tracking

Record monthly in GS-WM-F24:

- Rainfall (mm) at each site (from own rain gauge or nearest IMD station),

- Volume collected (m³),

- Volume utilised (m³),

- Freshwater saved (m³).

4.5 Reduction of Heavy Metal Discharges in Wastewater

4.5.1 Sources of Heavy Metals in Solar Manufacturing

| Heavy Metal | Typical Source | Risk |
| --- | --- | --- |
| Lead (Pb) | Lead-based solder (if still used); residues from flux washes | Neurotoxic, bioaccumulative |
| Tin (Sn) | Lead-free solder (Sn-Ag-Cu) – less toxic but still requires removal | Aquatic toxicity in high concentrations |
| Cadmium (Cd) | Not used intentionally, but could appear in old electronic components or as impurity in raw materials | Carcinogenic, damages kidneys and bones |
| Mercury (Hg) | Possibly in old measuring instruments (thermometers, manometers) if not replaced | Extremely toxic, impacts nervous system |
| Arsenic (As) | Contaminant in some chemicals or groundwater | Carcinogenic |
| Chromium (Cr) | Possible in anti-corrosion treatments for cooling water | Hexavalent chromium is carcinogenic; trivalent chromium is less toxic |

4.5.2 Source Segregation (Most Important Step)

The most effective way to reduce heavy metal discharge is to prevent mixing of heavy metal-laden streams with other wastewater.

| Stream | Action |
| --- | --- |
| Soldering flux washing effluent (contains tin, lead, or other metals) | Collect in a separate tank; treat separately before mixing with general effluent. |
| Cooling tower blowdown (chromates, if used) | Replace chromate inhibitors with non-toxic alternatives. If still present, segregate. |
| Old batteries or mercury devices | Handle as hazardous waste; never wash or throw into drains. |

4.5.3 Treatment Technologies

Depending on the concentration and volume, one or more of the following technologies shall be deployed:

| Technology | Principle | Application at Gautam Solar |
| --- | --- | --- |
| Chemical Precipitation | Adding hydroxide or sulphide to convert soluble heavy metal ions into insoluble precipitates (e.g., Pb(OH)₂, Cd(OH)₂). Solids are settled and removed as sludge. | For medium-to-high concentration streams (e.g., spent flux baths). |
| Ion Exchange | Using resin beads that exchange heavy metal cations (Pb²⁺, Cd²⁺, etc.) for harmless ions (H⁺, Na⁺). Resin is regenerated periodically, producing a small volume of concentrated metal solution. | Suitable for low-concentration, high-volume streams (e.g., rinses). |
| Adsorption | Activated carbon, zeolites, or bio-adsorbents capture heavy metals on their porous surface. | Final polishing after precipitation or ion exchange. |
| Membrane Filtration (Nanofiltration/RO) | Membranes with very small pore sizes reject dissolved metal ions, producing pure water. Suitable for low-concentration streams but may be energy-intensive. | For producing high-purity reusable water and concentrating metals. |
| Electrodialysis (ED) | Alternating anion- and cation-exchange membranes under electric field separate ionic species. Useful for recovery of valuable metals. | Optional for high-value metal recovery (e.g., tin from solder rinse). |

4.5.4 Operational Procedure for Heavy Metal Removal

| Step | Action | Frequency | Responsible |
| --- | --- | --- | --- |
| 1 | Segregate heavy metal-containing streams at source using dedicated collection tanks or troughs. Label clearly with “WARNING: Contains Lead / Cadmium / etc.” | Continuous | Production |
| 2 | For chemical precipitation: Adjust pH to optimal value (refer to chemical manufacturer’s guidelines; e.g., for lead, pH ~9–10). Add precipitant (lime, caustic soda, sodium sulphide). Mix gently. | Per batch | ETP Operator |
| 3 | Allow floc to settle (or use a clarifier). Decant supernatant water to the next treatment stage (or to polishing). | Per batch | ETP Operator |
| 4 | Filter the settled sludge through a filter press or centrifuge to dewater. The solid cake is hazardous waste and must be stored in labelled drums and sent to a CHWTSDF. | Weekly | ETP Operator |
| 5 | For ion exchange: Pump the clarified wastewater through a column of cation-exchange resin. Monitor effluent periodically for metal breakthrough (using test kits or online analyser). Regenerate resin with acid when exhausted. | Continuous; regeneration when exhausted | Maintenance / ETP Operator |
| 6 | Final polishing with activated carbon or UF membrane to ensure discharge meets CPCB norms. | Continuous | ETP Operator |
| 7 | Record all treatment parameters in GS-WM-F25 – Heavy Metal Treatment Log. | Daily | ETP Operator |

4.5.5 Monitoring of Heavy Metals

| Metal | Test Method | Frequency | Compliance Limit (CPCB) (examples – check latest Schedule) |
| --- | --- | --- | --- |
| Lead (Pb) | AAS or ICP-MS | Monthly | 0.1 mg/L |
| Mercury (Hg) | Cold vapour AAS | Monthly | 0.01 mg/L |
| Cadmium (Cd) | AAS or ICP-MS | Monthly | 0.01 mg/L |
| Arsenic (As) | AAS or ICP-MS | Monthly | 0.05 mg/L |
| Chromium (Cr) total | AAS or colorimetric | Monthly | 0.1 mg/L (trivalent), 0.05 mg/L (hexavalent) |

4.5.6 Sludge Management

- Heavy metal-bearing sludge must be treated as hazardous waste (Category A210 as per Hazardous Waste Rules).

- Store sludge in covered, labelled containers (UN-approved).

- Transport only through authorised hazardous waste carriers.

- Dispose at a CHWTSDF with appropriate incineration or stabilisation/solidification.

- Obtain disposal certificates (Form 11) and retain for 5 years.

4.5.7 Source Reduction (Long-Term)

- Replace lead-based solder with lead-free alternatives (Sn-Ag-Cu).

- Switch to no-clean fluxes that reduce water washing requirements.

- Replace chromate corrosion inhibitors in cooling towers with phosphate-based or non-toxic organic inhibitors.

- Phase out mercury-containing equipment (thermometers, pressure gauges, fluorescent lamps) – see Section 4.8 of GS-EN-002.

5. Records & Forms (Summary)

| Form ID | Name | Retention |
| --- | --- | --- |
| GS-WM-F20 | Water Reclamation Log | 5 years |
| GS-WM-F21 | Water Audit Report | 10 years |
| GS-WM-F22 | Wastewater Quality Log | 5 years |
| GS-WM-F23 | Non-Compliance & Corrective Action Log | 5 years |
| GS-WM-F24 | Rainwater Harvesting – Monthly Record | 5 years |
| GS-WM-F25 | Heavy Metal Treatment Log | 10 years |
| GS-WM-F26 | Hazardous Waste Disposal Certificate File | 10 years |

All records stored electronically in \\GautamSolar\HSE\Water_Management\.

6. References

| Reference | Relevance |
| --- | --- |
| Water (Prevention and Control of Pollution) Act, 1974 | Legal basis for discharge standards |
| Environment (Protection) Act, 1986 & Rules, 1986 | General environmental compliance |
| CPCB Effluent Discharge Standards (2026) | Mandatory limits for BOD, COD, TSS, heavy metals |
| IS 17408: 2020 – Water Auditing – Code of Practice | Methodology for water audits |
| CPCB Guidelines for Rainwater Harvesting in Industrial Premises (2022) | Design and implementation of RWH systems |
| ISO 14001:2015 | Environmental management system framework |
| Bureau of Indian Standards (BIS) water quality standards | Reference for testing methods |

7. Appendices

Appendix A –Water Balance Diagram (Text)

Inflows:

Municipal water: 500 m³/month

Borewell water: 300 m³/month

Rainwater harvested: 50 m³/month

Total inflow = 850 m³/month

Outflows:

Process consumption (evaporated/incorporated) = 200 m³/month

Cooling tower evaporation = 150 m³/month

Treated effluent discharged = 150 m³/month

Reclaimed water reused = 200 m³/month

Losses (leaks, washing) = 150 m³/month

Water balance: Inflow (850) = Outflows (850)

Reuse rate = 200 / (500+300+50) = 23.5%

Appendix B – Quick Checklist for Water Audit Walk-Through

| Item | Yes/No | Notes |
| --- | --- | --- |
| All water meters are installed and functioning? |  |  |
| No visible leaks at taps, valves, or pipes? |  |  |
| Cooling towers have conductivity controllers? |  |  |
| Washing stations have flow restrictors? |  |  |
| Rainwater harvesting system is clean and used? |  |  |
| Heavy metal-containing streams are segregated? |  |  |

Appendix C – Example Labelling for Heavy Metal Collection Tank

Gautam Solar – Lead-Containing Wastewater
Warning: Toxic – Handle with care.
Do not mix with other wastewater.
Date started: [Date]
Estimated Pb concentration: 2.5 mg/L

| SOP No. | GS-PD-001 |
| --- | --- |
| Version | 1.0 |
| Title | # Circular Product Lifecycle Management |
| Department | Product Development / Quality Assurance / Customer Service |

1. Purpose

This SOP establishes a comprehensive framework at Gautam Solar to:

- Conduct and document environmental impact studies of the solar modules it designs and manufactures, with a specific focus on the use phase of the product life cycle across multiple environmental impact categories including climate change, acidification, eutrophication, ozone depletion, photochemical smog formation, particulate matter formation, ecotoxicity, and natural resource depletion.

- Provide users with clear information on best practices regarding product use, enabling them to minimise resources required for intended use (optimising energy yield, reducing water consumption for cleaning) and extend the useful life of the product.

- Design and develop products that consume fewer resources during use, including higher energy conversion efficiency (reducing electricity consumption per kWp), lower degradation rates (maintaining performance over decades), and reduced need for consumable items.

- Empower users to prolong product lifespan, enhance repairability, and choose repair over replacement through provision of spare parts, diagnostic tools, manuals, and access to manufacturer or contracted repair service centres.

This SOP applies to all solar modules manufactured and sold by Gautam Solar and covers the entire product lifecycle with particular emphasis on the operational (use) phase, which represents the longest and most environmentally significant period of a solar module's life.

2. Scope

The SOP covers:

| Topic | Applicable Areas |
| --- | --- |
| Environmental impact studies | Life Cycle Assessment (LCA) of Gautam Solar modules from cradle-to-grave, covering raw material extraction, manufacturing, transportation, installation, use phase (25–30 years of operation), and end-of-life disposal/recycling. |
| Impact categories assessed | Climate change (global warming potential – GWP), acidification of water and soils, eutrophication (freshwater and marine), stratospheric ozone depletion, photochemical smog formation (ground-level ozone), particulate matter formation, freshwater and marine ecotoxicity, human toxicity (cancer and non-cancer effects), and depletion of abiotic resources (minerals, metals, fossil fuels). |
| Use phase best practices | Installation guidelines (tilt angle, orientation, shading avoidance), operational parameters (temperature management, soiling prevention), cleaning recommendations, system monitoring, and performance optimisation. |
| Resource-efficient product design | High efficiency modules (reducing area-based resource consumption per kWh), lower degradation rates (reducing lifetime resource intensity), bifacial technology (dual-side power generation), multi-busbar and half-cut cell designs, and reduced consumable requirements. |
| Product longevity & repairability | Warranty coverage (product and performance), spare parts availability, diagnostic tools, repair manuals, service centre network, and customer education on repair vs. replacement. |

Exclusions: This SOP does not cover environmental impacts of internal manufacturing operations (covered under GS-EN-001, GS-EN-002, and GS-WM-002) or packaging materials for finished products (covered under GS-WM-001 Section 4.1).

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Product Development Manager | Oversees eco-design initiatives; integrates resource efficiency into new product specifications; conducts Life Cycle Assessment studies or commissions third-party LCA consultants. |
| Quality Assurance Manager | Verifies product performance and degradation rates through accelerated lifetime testing (damp heat, thermal cycling, UV exposure). Maintains certification documentation (IEC 61215, IEC 61730, BIS, ALMM). |
| Customer Service Head | Develops and disseminates user best practice guides; manages warranty claims and repair service network; maintains spare parts inventory. |
| Technical Support Team | Provides diagnostic support to customers and service partners; develops repair manuals and training materials. |
| CEO / Management | Approves LCA study budgets and eco-design targets; reviews environmental performance reports annually. |

4. Detailed Procedures

4.1 Environmental Impact Studies – Life Cycle Assessment (LCA)

4.1.1 Rationale for Product LCA at Gautam Solar

While solar PV modules generate clean electricity with zero operational emissions, their manufacturing process (from silicon refinement to module assembly) consumes significant energy and materials. Furthermore, the use phase performance—specifically the module's efficiency, degradation rate, and bifacial gain—directly determines the lifetime environmental benefit per installed watt.

Gautam Solar recognises that a cradle-to-grave LCA is essential to:

- Quantify the true environmental footprint of each module series.

- Identify hotspots (e.g., energy-intensive cell manufacturing, aluminium frame production) for improvement.

- Benchmark against industry averages and emerging technologies.

- Provide transparent environmental data to customers, regulators, and certification bodies (e.g., EU Ecodesign Directive, EPEAT criteria).

A systematic review of environmental LCA of solar modules identifies up to 18 environmental indicators for comprehensive assessment, including climate change, acidification, eutrophication, ozone depletion, photochemical smog formation, particulate matter, ecotoxicity, and resource depletion.

4.1.2 Life Cycle Stages to be Assessed

| Stage | Description | Relevance to Gautam Solar |
| --- | --- | --- |
| A1 – Raw material extraction | Mining and processing of silicon, aluminium, silver, copper, glass (silica), EVA/POE, backsheet polymers. | Primary hotspots: silicon purification (energy-intensive), aluminium smelting, silver (for busbars). |
| A2 – Transport to factory | Shipping of raw materials to Gautam Solar factories in Haridwar and Bhiwani. | Domestic vs. imported materials; carbon footprint varies by supplier location. |
| A3 – Manufacturing (cradle-to-gate) | Cell manufacturing (ingot pulling, wafer slicing, cell processing), module assembly (stringing, lamination, framing, junction box attachment, testing). | Majority of embodied energy; Gautam Solar's expansion into cell manufacturing (5 GW plant in Gwalior, 2026) will reduce reliance on imported cells and improve traceability. |
| B1 – Use phase | Module installation, operation over 25–30 year lifespan, including cleaning, inverter replacement (if applicable), and performance degradation. | Most significant phase in terms of net environmental benefit—high efficiency and low degradation directly maximise CO₂ savings. |
| C1–C4 – End-of-life | Decommissioning, disassembly, recycling of glass, aluminium, silicon, copper, silver, and hazardous waste management (e.g., lead if present). | Recycling credits can significantly reduce overall LCA impact; at least 90% of mass (glass, aluminium frame) is readily recyclable. |

4.1.3 Impact Categories to be Assessed

Based on ISO 14040/14044 methodology and the EU Product Environmental Footprint Category Rules (PEFCR) for PV modules, Gautam Solar shall assess the following categories:

| Impact Category | Description | Unit | Relevance to PV Modules |
| --- | --- | --- | --- |
| Climate Change (GWP) | Global warming potential from greenhouse gas emissions (CO₂, CH₄, N₂O, refrigerants). | kg CO₂ eq per kWh generated (or per kWp) | Primary environmental metric; typical range for mono c-Si modules: 493–2760 kg CO₂ eq/kWp |
| Acidification | Deposition of acidifying substances (SOₓ, NOₓ, NH₃) causing soil and water acidification. | mol H⁺ eq (or kg SO₂ eq) | Arises from fossil fuel combustion during silicon purification and transport. |
| Eutrophication | Excessive nutrient enrichment (nitrates, phosphates) in water bodies, causing algal blooms and oxygen depletion. | kg PO₄³⁻ eq (or kg N eq) | From fertiliser use in agriculture (indirect) and wastewater discharges. |
| Stratospheric Ozone Depletion | Destruction of stratospheric ozone layer by halogenated gases (CFCs, HCFCs). | kg CFC-11 eq | Historically relevant for older refrigerants; minimal for PV unless used in manufacturing cooling systems. |
| Photochemical Smog Formation | Formation of ground-level ozone from VOC and NOₓ emissions, harmful to human health and vegetation. | kg NMVOC eq (or kg O₃ eq) | From solvent evaporation (fluxes, cleaners) during module assembly. |
| Particulate Matter Formation | Emission of fine particulates (PM2.5, PM10) causing respiratory illness. | kg PM2.5 eq | From silicon processing, glass cutting, and fossil fuel combustion. |
| Human Toxicity – Cancer | Cancer-related health impacts from exposure to toxic substances (heavy metals, PAHs). | CTUh (Comparative Toxic Unit for humans) | Potential from lead in solders, cadmium (not used), and other trace metals. |
| Human Toxicity – Non-cancer | Non-cancer health impacts (organ damage, developmental toxicity). | CTUh | From solvents, flux residues, and heavy metals. |
| Freshwater Ecotoxicity | Toxic effects on aquatic organisms from chemical releases to freshwater. | CTUe (Comparative Toxic Unit for ecosystems) | From wastewater containing cleaning agents or heavy metals. |
| Marine Ecotoxicity | Toxic effects on marine organisms from chemical releases to seawater. | CTUe | From air emissions that deposit into oceans. |
| Resource Depletion – Minerals & Metals | Depletion of finite abiotic resources (silicon, silver, copper, aluminium). | kg Sb eq (antimony equivalent) or MJ primary energy | High relevance: silver is scarce and energy-intensive; aluminium requires significant energy. |
| Resource Depletion – Fossil Fuels | Depletion of fossil fuel reserves (coal, oil, natural gas). | MJ (based on lower heating value) | Energy used throughout life cycle; reflects overall energy return on investment (EROI). |

4.1.4 LCA Methodology & Standards

Gautam Solar shall conduct LCA studies in accordance with:

| Standard | Requirement |
| --- | --- |
| ISO 14040:2006 | Life Cycle Assessment – Principles and framework |
| ISO 14044:2006 | Life Cycle Assessment – Requirements and guidelines |
| EN 15804:2012+A2:2019 | Sustainability of construction works – Environmental product declarations (for modules used in buildings) |
| EU PEFCR for PV Modules | Product Environmental Footprint Category Rules for photovoltaic modules used in PV systems |
| IEC TS 62994 | Environmental awareness – Life cycle assessment of photovoltaic modules |

Functional unit: The primary functional unit shall be 1 kWh of AC electricity generated over the module's lifetime (use phase). Secondary reporting unit: 1 kWp (peak power) of installed capacity.

4.1.5 Use Phase Modelling – Specific Requirements

Because the use phase is the longest and most environmentally significant period, it must be modelled with the highest accuracy. Gautam Solar shall incorporate the following parameters:

| Parameter | Value / Source | Justification |
| --- | --- | --- |
| Reference yield | Based on typical Indian irradiance (south-facing, tilt = latitude, no shading). Use TMY (Typical Meteorological Year) data for Haridwar and Bhiwani regions. | Realistic energy output estimation. |
| Degradation rate | ≤1.0% in first year, then ≤0.4% per year (linear) for TOPCon modules; ≤1.5% first year then ≤0.5% per year for Mono PERC. | Based on accelerated lifetime testing per IEC 61215. Lower degradation extends useful life and reduces lifetime carbon intensity. |
| Performance ratio (PR) | 0.80 for ground-mounted; 0.75 for rooftop. | Accounts for DC-AC conversion losses, soiling, temperature, mismatch, and wiring losses. |
| Bifacial gain | 10–30% additional energy from rear side, depending on albedo (ground reflectivity) and mounting height. | For bifacial modules (G-2X series, TOPCon bifacial). Equivalent to reducing land-area resource consumption per kWh. |
| Module lifetime | 25 years (performance warranty) to 30 years (extended performance for TOPCon series). | Warranty period influences LCA – longer lifetime spreads manufacturing impacts across more kWh. |
| Cleaning frequency | Assume 4 times per year for commercial/utility scale in dusty regions (North India). | Water and labour inputs for cleaning are part of use phase resource consumption. |
| Inverter replacement | One replacement over 25 years (inverter lifetime ≈12 years). | Includes manufacturing, transport, and disposal impacts of inverters (if system boundary includes BOS). |

4.1.6 LCA Study Execution Procedure

| Step | Action | Frequency | Responsible |
| --- | --- | --- | --- |
| 1 | Define goal and scope: product series (e.g., G2X N-type TOPCon 640 Wp), functional unit (1 kWh), system boundaries (cradle-to-grave). | For each new product family | Product Development + external LCA consultant |
| 2 | Collect life cycle inventory (LCI) data: bill of materials (kg per module), energy consumption during manufacturing (kWh per module), transport distances, use phase energy yield (kWh/kWp/year from simulation), end-of-life recycling rates. | For each new product family | Manufacturing, Procurement, Logistics |
| 3 | Model the product using LCA software (e.g., SimaPro, openLCA, GaBi) with ecoinvent database (or regional Indian databases where available). | Per product family | External LCA consultant |
| 4 | Conduct life cycle impact assessment (LCIA) using recognised methods: ReCiPe 2016 (Midpoint/Endpoint), IMPACT 2002+, or EF 3.1 (for EU compliance). | Per product family | External LCA consultant |
| 5 | Interpret results: identify hotspots (e.g., cell manufacturing contributes 70% of GWP), evaluate improvement options (e.g., switching to low-carbon electricity for cell production, reducing silver content). | Per product family | Product Development + QA |
| 6 | Produce LCA report (GS-PD-F01) with transparent documentation of assumptions, data sources, and limitations. | Per product family | Consultant + Product Development |
| 7 | Third-party critical review (optional but recommended for public disclosure or ecolabels). | Per product family | Independent expert |
| 8 | Update LCA every 3 years, or when significant changes occur in manufacturing process, materials, or supply chain. | Triennial + event-driven | Product Development |

4.1.7 Example LCA Results (to be populated with actual data)

| Impact Category | G2X Mono PERC (545 Wp) | G2X N-type TOPCon (640 Wp) | Unit |
| --- | --- | --- | --- |
| Climate change (GWP) | [XX] | [XX] (expected lower due to higher efficiency) | kg CO₂ eq / kWh |
| Acidification | [XX] | [XX] | mol H⁺ eq / kWh |
| Eutrophication | [XX] | [XX] | kg PO₄³⁻ eq / kWh |
| Particulate matter | [XX] | [XX] | kg PM2.5 eq / kWh |
| Resource depletion (metals) | [XX] | [XX] | kg Sb eq / kWh |

Note: Actual values to be determined by commissioned LCA studies.

4.1.8 Carbon Footprint Reduction Commitment

Gautam Solar notes that the carbon intensity of PV modules ranges from approximately 10.8 to 44 g CO₂ eq/kWh depending on manufacturing electricity source, silicon content, and product lifetime. Through manufacturing efficiency improvements, vertical integration (cell manufacturing planned for 2026), and use of higher-efficiency TOPCon technology, Gautam Solar targets a lifecycle carbon footprint of ≤ 20 g CO₂ eq/kWh for its flagship product series.

Furthermore, each 640 Wp TOPCon module installed in place of grid electricity saves approximately 545 to 793 metric tonnes of CO₂ annually across its 25-year lifetime compared to fossil fuel generation.

4.2 Sustainable Product Use Awareness

4.2.1 User Guides & Installation Manuals

Gautam Solar shall provide every customer with a comprehensive Installation & Operation Manual (physical copy with each pallet and downloadable PDF

from gautamsolar.com) containing:

| Section | Content |
| --- | --- |
| Safety instructions | Electrical safety, handling precautions, fire safety, personal protective equipment (PPE) requirements. |
| Unpacking & inspection | Visual inspection for damage, verification of nameplate data, packaging waste disposal guidance. |
| Installation guidelines | Minimum mounting height for bifacial gain (≥1.0 m above ground), tilt angle optimisation (latitude ±10°), orientation (true south for Northern Hemisphere), inter-row spacing to avoid shading. |
| Electrical installation | Series/parallel configuration limits, maximum system voltage (1500 VDC for TOPCon series), fuse ratings, cable sizing, IP68 junction box sealing. |
| Cleaning & maintenance | Recommended cleaning frequency (minimum 4 times per year for dusty areas), cleaning method (soft brush or sponge, deionised or tap water with neutral pH, no abrasive chemicals), safety precautions for rooftop access. |
| Performance monitoring | Expected daily/seasonal energy yield for different Indian cities, use of monitoring systems (inverters with data logging), interpretation of performance anomalies. |
| Troubleshooting guide | Common issues: low output (shading, soiling, degradation), hotspots, corrosion, PID (Potential Induced Degradation). |
| Warranty terms & claims procedure | 12-year product warranty (materials and workmanship), 30-year performance warranty (power output≥90% of nameplate at 20 years, ≥80% at 25 years). |

4.2.2 User Guidance to Minimise Resource Consumption

The manual shall explicitly instruct users on practices that reduce resource consumption:

| Resource | User Action | Benefit |
| --- | --- | --- |
| Water | Clean only when performance drops by >5% due to soiling, not on fixed schedule. Use high-pressure water only from ground level (not direct spray on modules). Use deionised water to avoid mineral deposits. | Reduced water consumption per kWh generated. |
| Energy (self-consumption) | Orient modules for maximum annual yield (not just summer peak). Install module-level power electronics (microinverters or optimisers) to mitigate partial shading losses. Schedule cleaning during early morning or evening to avoid thermal shock. | Higher energy yield = more kWh from same installed capacity. |
| Land | For ground-mount systems, use bifacial modules at optimal height (≥1.0 m) to achieve 10–30% additional energy without additional land. | Reduced land-use intensity per kWh. |
| Consumables (filters, chemicals) | Use self-cleaning anti-soiling coatings (available as an option on G-2X series) to reduce cleaning frequency. Use neutral pH water only; avoid detergents that could degrade EVA encapsulation. | Reduced chemical use and lower cleaning costs. |

4.2.3 Guidance to Extend Product Useful Life

| Practice | Instruction | Lifetime Extension |
| --- | --- | --- |
| Avoid mechanical damage | Do not walk on modules. Maintain minimum clearance from roof edges to avoid impact from falling tools. | Prevents cracked cells and frame deformation. |
| Monitor degradation | Log monthly AC energy output (kWh) and compare with modelled expected yield using PVsyst or similar tools. Early detection of underperformance allows corrective action. | Identifies module failures early, preventing system-wide losses. |
| Inspect connections | Semi-annually check MC4 connectors for corrosion, overheating, or loosening. Retorque mounting clamps. | Prevents arc faults and potential safety hazards. |
| Remove debris | Clear fallen leaves, bird droppings, and snow accumulation promptly. Avoid metal tools. | Maintains uniform cell illumination; prevents hotspots. |
| Update inverter firmware | Keep inverter settings (voltage, frequency, power factor) optimised for local grid conditions. | Ensures maximum power point tracking (MPPT) accuracy. |

4.2.4 Distribution Channels for User Information

| Channel | Content Provided | Access |
| --- | --- | --- |
| Product packaging | Printed Quick Start Guide (4-page summary) + QR code linking to full manual. | Each module pallet. |
| Company website | Full Installation & Operation Manual (PDF), cleaning guide video, warranty registration portal. | gautamsolar.com/support |
| Customer portal | Personalised system performance benchmarks, cleaning reminders, warranty status. | Upon customer registration. |
| Wholesale / EPC partners | Technical training webinars, installer certification program, diagnostic decision trees. | Partner portal login. |

4.2.5 For Chemical Products (Fluxes, Sealants, Cleaners) – Not applicable to finished modules

*Note: Gautam Solar does not sell chemical process materials to end users. However, for proprietary cleaning agents recommended for panel maintenance, the company provides Material Safety Data Sheets (MSDS) and storage/application instructions to minimise harmful releases, per its commitment under GS-EN-002 Section 4.2.*

4.3 Design & Development of Products That Consume Less Resources During Use

4.3.1 High-Efficiency Module Design

The single most effective way to reduce resource consumption during the use phase is to increase power conversion efficiency. Higher efficiency means:

- More kWh generated per square metre of land area (land resource conservation).

- More kWh generated per kilogram of module materials (material resource intensity reduction).

- Faster energy payback time (EPBT).

Gautam Solar's current and planned product efficiencies:

| Product Series | Technology | Peak Power (Wp) | Module Efficiency (%) | Status |
| --- | --- | --- | --- | --- |
| G2X Mono PERC | Monocrystalline PERC (10BB, half-cut) | Up to 595 Wp | Up to 22.65% | In production |
| G2X N-type TOPCon (transparent backsheet) | N-type TOPCon bifacial | 580–585 Wp | 22.45–22.65% | In production |
| G2X N-type TOPCon R-series (rectangular cells) | N-type TOPCon bifacial G12R | 620–640 Wp | 22.95–23.69% | Launched 2025, BIS/ALMM approved |

The R-series TOPCon modules with up to 23.69% efficiency deliver approximately 5% higher power generation than standard TOPCon modules of similar size, representing a direct reduction in land-area resource consumption per kWh generated.

4.3.2 Bifacial Technology – Dual-Side Power Generation

Bifacial modules generate power from both front and rear sides by capturing reflected light (albedo). The G-2X Mono crystalline series can generate up to 30% more power due to dual power generation design. This means:

- 10–30% additional energy from the same mounting structure and land area.

- Equivalent reduction in balance of system (BOS) cost per kWh: 3.26% savings for N-type TOPCon.

- For utility-scale projects, this translates directly to lower resource consumption per MWh delivered.

4.3.3 Lower Degradation Rates – Lifetime Resource Optimisation

Module degradation determines how much useful energy a module generates over its lifetime. Gautam Solar's TOPCon modules feature:

- Low temperature coefficient: -0.29%/°C (excellent performance in high-temperature Indian climates).

- Superior UV resistance: 60% less yellowing proven over 15 years; >98% light transmission maintained after a decade.

- 30-year performance warranty – industry-leading, extending the useful life by 5 years compared to standard 25-year warranties.

Impact on resource consumption: A module that retains 90% of nameplate power at 20 years (vs. 85% for average modules) generates approximately 5–8% more lifetime energy, directly reducing the material intensity per lifetime kWh.

4.3.4 Multi-Busbar & Half-Cut Cell Technology

| Feature | Resource Benefit |
| --- | --- |
| 10BB (10 busbars) | Lower electrical resistance losses → higher output per cell → fewer cells required per kWp. |
| Half-cut cell design | Reduced internal current → lower resistive losses → higher yield, especially in partial shading. |
| Non-destructive laser cutting | Lower microcrack rates → improved reliability → fewer premature failures → less premature replacement waste. |

These technologies are standard across Gautam Solar's G-2X series.

4.3.5 Reduced Consumable Requirements During Use

| Consumable | Design Feature Reducing Consumption | Benefit |
| --- | --- | --- |
| Water for cleaning | Optional anti-soiling hydrophobic coating. Larger cell spacing reduces dust accumulation between cells. | Fewer cleaning cycles → water and labour savings. |
| Inverter replacement | Higher per-module voltage (up to 1500 VDC) reduces series string length → fewer inverters per MW. | Reduces manufacturing demand for inverters (copper, electronics). |
| Module replacement | Rigorous testing (IEC 61215: 1000h damp heat, ±1000Pa dynamic load) ensures 25+ year lifetime, reducing premature replacement frequency. | Lower material consumption per lifetime MWh. |
| Junction box diodes | IP68-rated junction box (dust-tight, immersion-proof) eliminates water ingress failures → no diode replacement needed over life. | Zero consumable diode replacement. |

4.3.6 Eco-Design Review Process

| Step | Action | When | Responsible |
| --- | --- | --- | --- |
| 1 | Define target environmental performance metrics for new product: efficiency (%), degradation rate (%/year), mass per watt (g/Wp), silver consumption (mg/Wp). | Concept phase | Product Development |
| 2 | Identify alternatives: lead-free solder, reduced silver paste, halogen-free backsheet, recycled aluminium for frames. | Design phase | Product Development + Procurement |
| 3 | Conduct simplified LCA (screening) to compare design options. | Design phase | Product Development |
| 4 | Prototype and test for efficiency, degradation, and reliability (IEC 61215/61730). | Validation phase | QA |
| 5 | Document eco-design decisions in GS-PD-F02 – Eco-Design Review Record. | At design freeze | Product Development |
| 6 | For export to EU, ensure compliance with Ecodesign for Sustainable Products Regulation (ESPR) and Carbon Footprint calculation methodology per JRC guidelines. | Prior to market launch | Product Development + QA |

4.4 Empowering Users to Prolong Product Lifespan, Enhance Repairability & Choose Repair Over Replacement

4.4.1 Warranty Coverage – Long-Term Assurance

Gautam Solar provides comprehensive warranty protection to give users confidence in repair rather than replacement:

| Warranty Type | Duration | Coverage |
| --- | --- | --- |
| Product Warranty (materials & workmanship) | 12 years (standard), 15 years (premium TOPCon series) | Manufacturing defects, junction box failures, frame corrosion, delamination, cell breakage (not due to external force). |
| Performance Warranty (power output) | 30 years for TOPCon series, 25 years for Mono PERC | Year 1: ≥97% of nameplate; Years 2–30: ≤0.4% annual degradation; Year 30: ≥87% of nameplate. |

A dedicated warranty fund ensures claims are honoured without delay, reinforcing the financial viability of repair over replacement.

4.4.2 Spare Parts Availability

Gautam Solar maintains a spare parts inventory for field replaceable components:

| Part | Typical Failure Mode | Repair vs. Replacement Guidance |
| --- | --- | --- |
| Junction box | Diode failure (short circuit), cable break, moisture ingress, terminal corrosion | Repair: Replace entire junction box (field-replaceable); requires basic electrical tools. Do not attempt internal diode repair. |
| Connectors (MC4 equivalents) | Loose connection, arcing damage, UV embrittlement | Repair: Cut and crimp new connectors with identical brand (mixed brands risk mismatching). |
| Aluminium frame | Bent or cracked during transport or installation | Repair: If <5° bend, may be straightened and re-used; if cracked or severely bent → replace frame. |
| Backsheet | Scratches (superficial) or UV-induced yellowing | Repair: Minor scratches may be sealed with UV-curable encapsulant; severe damage → replace module (backsheet not field-replaceable). |
| Junction box cable | Rodent damage, chafing, insulation breakdown | Repair: Cut out damaged section, re-terminate with weatherproof splice or replace entire junction box cable assembly. |

Spare parts are available via:

- Direct from Gautam Solar (minimum order quantities apply for wholesale customers).

- Authorised service partners – stock common parts for rapid field service.

- Online spare parts portal (under development, expected 2026).

4.4.3 Diagnostic Tools & Manuals

| Tool / Document | Purpose | Accessibility |
| --- | --- | --- |
| Diagnostic decision tree | Step-by-step flow chart for field technicians to identify root cause (degradation vs. shading vs. connection failure). | PDF download from website. |
| I-V curve interpretation guide | How to read flash test / field I-V tracer results to detect broken cells, PID, or elevated series resistance. | Included in Service Manual. |
| Insulation resistance test procedure | Protocol to use megohmmeter for detecting moisture ingress or ground faults. | Service Manual. |
| Thermographic imaging guide | How to identify hotspots (failed cells) and bad connections using IR camera. | Quick Reference Card (laminated). |
| Firmware update tool | For smart junction boxes (if equipped with monitoring) – USB or Bluetooth upload. | Partner portal. |

4.4.4 Repair Service Models

Gautam Solar supports three repair models, enabling users to choose the most cost-effective option:

| Model | Description | Best for |
| --- | --- | --- |
| Manufacturer repair (Gautam Solar) | Modules returned to factory (Haridwar or Bhiwani) for inspection, junction box replacement, or electrical testing. Customer pays return shipping only if within warranty. | Warranty claims; complex diagnostics. |
| Contracted repair service centres | Network of authorised service partners (EPC companies, electrical contractors) trained and certified by Gautam Solar. Equipped with genuine spare parts, calibrated test equipment, and access to technical support hotline. | Field service for large installations (utility scale). |
| User self-repair | Upon provision of spare parts (junction boxes, connectors, mounting clamps) and access to repair manuals / instructional videos. Gautam Solar provides no-cost diagnostic support via phone or email. | Minor repairs (connector replacement, cleaning); off-grid or remote installations where sending a technician is uneconomical. |

User self-repair support package:

- Parts supply: Junction boxes, connectors, mounting hardware sold individually (not only in bulk) through online store.

- Diagnostic tools: Multimeter measurement guide (DC voltage and current), insulation resistance test procedure, bypass diode testing (forward voltage method).

- Instructional content: YouTube video series (“Gautam Solar DIY Repair” – step-by-step junction box replacement, connector crimping, etc.).

- Technical support hotline: +91-8460710095 (Monday–Saturday, 10 AM–6 PM IST) for fault diagnosis assistance.

- Safety disclaimer: All self-repair performed at user's risk; Gautam Solar recommends professional service for any repair involving high-voltage systems (>120 VDC) or rooftop work.

4.4.5 Refurbishment of Removed Modules

When modules are replaced due to a small number of failed cells (but majority of cells functional), Gautam Solar offers a refurbishment service:

- Customer ships decommissioned modules to Gautam Solar factory.

- QC team performs EL imaging and I-V testing to identify failed cells.

- If salvageable (>80% of cells functional): Reconditioned modules are offered as warranty replacements or sold as “Certified Pre-owned” with a 5-year limited warranty.

- If not salvageable: Module enters recycling stream per GS-WM-001.

This circular approach reduces the need for new module manufacturing when partial array failures occur.

4.4.6 Encouraging Repair – Economic Incentives

| Incentive | Detail |
| --- | --- |
| Reduced labour rate for repairs | Compared to replacement module cost (typically 60% of new module price), repair labour is often significantly cheaper, especially for junction box or connector replacements. |
| Extended warranty on repairs | Repaired components carry an additional 12-month warranty on the repaired part (labour and part). |
| Trade-in credit | Customers returning a damaged module (under 10 years old) receive credit of 15–20% of current list price toward purchase of a new or refurbished replacement. |
| No diagnostics fee | For modules under product warranty, all diagnostic testing (EL, I-V, insulation) is free of charge. |

4.4.7 Customer Education & Training

| Training Offering | Audience | Format | Frequency |
| --- | --- | --- | --- |
| Installer certification program | Solar EPC companies, electrical contractors | 2-day in-person workshop (Haridwar factory) covering installation best practices, commissioning, maintenance, and minor repairs. | Quarterly |
| Webinar series: “Extending Your Solar Investment” | End customers (commercial/industrial) | 60-minute online session covering cleaning, monitoring, warranty claims, and when to repair vs. replace. | Bi-annually |
| Technical bulletins | All registered customers | Email distribution of seasonal maintenance reminders, new spare part availability, and recall notices (if any). | As needed |

4.4.8 Service & Repair Contact Information

For repair requests, spare parts ordering, or diagnostic support:

| Channel | Details |
| --- | --- |
| Website | www.gautamsolar.com/service |
| Service hotline | +91-846810087(Mon–Sat, 10:00–18:00 IST) |
| Spare parts orders | spares@gautamsolar.com |
| Warranty claims | warranty@gautamsolar.com |
| Registered Office | D-120, Okhla Industrial Area, Phase-1, New Delhi – 110020 |
| Service centre (Haridwar) | Plot 67-70, Sec-8A, SIDCUL, Haridwar, Uttarakhand – 249403 |
| Service centre (Bhiwani) | 7 KM Milestone, Tosham Road, Bhiwani, Haryana – 127032 |

5. Records & Forms

| Form ID | Name | Retention |
| --- | --- | --- |
| GS-PD-F01 | Life Cycle Assessment Report (cradle-to-grave) | 10 years |
| GS-PD-F02 | Eco-Design Review Record (per product series) | Life of product + 5 years |
| GS-PD-F03 | Warranty Claim & Repair Log | 7 years |
| GS-PD-F04 | Customer Training Attendance Record | 3 years |
| GS-PD-F05 | Diagnostic Support Request Log | 5 years |

All records stored in \\GautamSolar\Product_Development\Environmental_Sustainability\.

6. References

| Reference | Relevance |
| --- | --- |
| ISO 14040:2006 – Environmental management – Life cycle assessment – Principles and framework | LCA methodology |
| ISO 14044:2006 – Environmental management – Life cycle assessment – Requirements and guidelines | LCA requirements |
| IEC 61215 – Terrestrial photovoltaic modules – Design qualification and type approval | Module reliability testing |
| IEC 61730 – Photovoltaic module safety qualification | Safety requirements |
| EU Ecodesign for Sustainable Products Regulation (ESPR) | Ecodesign compliance for EU exports |
| EU Product Environmental Footprint Category Rules (PEFCR) for PV modules | Carbon footprint calculation |
| EPEAT PVMI Criteria (Photovoltaic Modules and Inverters) | Sustainability certification reference |
| IEC TS 62994 – Life cycle assessment of photovoltaic modules | PV-specific LCA guidance |
| The Stockholm Convention on Persistent Organic Pollutants | Hazardous material avoidance (reference) |
| The Minamata Convention on Mercury | Mercury-free product design (reference) |

7. Appendices

Appendix A – Example Life Cycle Stages Diagram

Cradle-to-Grave LCA Boundary for Gautam Solar Module

A1 ──► A2 ──► A3 ──► B1 ──► B2 ──► B3 ──► B4 ──► C1

▲         ▲         ▲         ▲         ▲         ▲

│         │         │         │         │         │

Raw Mat.   Transport Manufacturing   Use       Transport End-of-Life

Extraction (incl. (25-30 to(Recycling/cleaning, years) Install Landfill) packaging,testing)

Appendix B – Product Efficiency Roadmap (2025–2030)

| Year | Target Efficiency | Technology |
| --- | --- | --- |
| 2025 | 22.95% – 23.69% | N-type TOPCon G12R (achieved) |
| 2026 | ≥24.0% | N-type TOPCon + advanced passivation |
| 2028 | ≥24.5% | Tunnel oxide passivated contact (TOPCon) / HJT hybrid |
| 2030 | ≥25.0% | Perovskite-tandem (R&D) or advanced crystalline silicon |

Appendix C – Customer Self-Repair Decision Flowchart

Module underperforming

↓

Check shading, soiling, orientation → rectify → re-test

↓

Still underperforming? → Check connections (MC4) → re-crimp if loose

↓

Still underperforming? → Perform I-V curve test

↓

All strings abnormal? → Inverter issue (not module)

↓

One string abnormal? → Test bypass diodes (junction box)

↓

Diode failed? → REPLACE JUNCTION BOX (user or service centre)

↓

Cell-level failures (EL image shows cracks)? → WARRANTY CLAIM (replace module)

| SOP No. | GS-CE-001 |
| --- | --- |
| Version | 1.0 |
| Title | # Product Stewardship Through User Awareness |
| Department | Product Development / HSE / Customer Service / Engineering |

1. Purpose

This SOP establishes a comprehensive framework at Gautam Solar to:

- Collaborate with external parties (Producer Responsibility Organizations, research groups, industry coalitions, public/private initiatives) to help product users manage durable products at their end-of-life, ensuring environmentally sound collection, treatment, and recycling.

- Provide clear information to product users on ways to return, recycle, or dispose of durable products at end-of-life, highlighting available collection, recycling, or sanitary waste management infrastructure to minimise waste to landfill, product component leakage, and virgin material use.

- Eliminate or minimise the amount of substances of concern in electrical and electronic equipment, including lead, mercury, cadmium, hexavalent chromium, polybrominated biphenyls (PBB), polybrominated diphenyl ethers (PBDE), phthalates (DEHP, BBP, DBP, DIBP), zinc, yttrium, beryllium, nickel, brominated flame retardants, antimony trioxide, halogenated flame retardants, tin, and polyvinyl chloride (PVC), in line with the E-Waste (Management) Rules, 2022 (including the 2023 amendment) and the Minamata and Stockholm Conventions.

- Label electrical and electronic components of products to facilitate sorting for appropriate collection, treatment, recycling, and recovery of diverse element composition.

- Adopt product designs that facilitate collection, disassembly, sorting, remanufacturing, or recycling at end-of-life, such as modular designs, use of standard instead of proprietary parts, elimination of glue and screws where feasible, and the use of reversible connections.

- Implement a collection scheme allowing users to return used products free of charge or sell used products back to the company, with recovered products remanufactured into new products, broken down for material recycling, or disposed of safely.

This SOP applies to all solar modules, inverters, and associated electrical equipment manufactured and sold by Gautam Solar, and covers the post-consumer (use phase) and end-of-life stages of the product lifecycle.

2. Scope

The SOP covers:

| Topic | Applicable Areas |
| --- | --- |
| Collaboration with external parties | Producer Responsibility Organizations (PROs), industry coalitions, research groups (e.g., NISE, CEEW, MNRE-led initiatives), authorised e-waste recyclers, and public/private waste management partnerships (excluding parent, subsidiary, or sister companies). |
| End-of-life information provision | User manuals, product packaging, website portals, customer service helplines, collection point and authorised recycler information. |
| Reduction of substances of concern | Prohibition or restriction of lead, mercury, cadmium, hexavalent chromium, PBB, PBDE, DEHP, BBP, DBP, DIBP, brominated/halogenated flame retardants, antimony trioxide, beryllium, nickel, and PVC in new product designs (to the extent technically feasible and within regulatory exemptions). |
| Component labelling | Physical and/or digital labelling of electrical/electronic components for identification of material composition (e.g., glass, aluminium, copper, silver, silicon, hazardous substances). |
| Design for circularity | Modularity, disassembly without tools or with common tools, minimising permanent adhesives, use of recyclable and standardised parts, and elimination of non-recyclable composites. |
| Collection & take-back scheme | Free-of-charge return for end-of-life products, buy-back for certain used products, and a reverse logistics network for collection, storage, and forwarding to authorised recyclers. |

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| HSE Manager | Overall owner of this SOP; maintains the E-waste Compliance Register; liaises with external PROs and regulatory authorities; files annual returns on the CPCB EPR Portal. |
| Product Development Manager | Implements design for circularity principles; selects materials with low hazardous substance content; develops component labelling standards; maintains the “List of Prohibited/Substituted Substances.” |
| Customer Service Head | Manages the take-back/collection scheme; maintains a database of collection points; issues end-of-life information to customers; handles buy-back offers. |
| Procurement | Purchases only components and raw materials that comply with the company’s Restricted Substances List (RSL) and RoHS/exemption requirements. |
| Warehouse / Logistics | Operates collection/storage areas for returned products; ensures safe storage per CPCB guidelines; coordinates with authorised recyclers for dispatch. |
| All Employees | Support the internal e-waste sorting and collection; report any unauthorised disposal of used products. |

4. Detailed Procedures

4.1 Collaboration with External Parties

4.1.1 Rationale for External Collaboration

Effective management of end-of-life solar modules and electrical equipment requires a multi-stakeholder approach. As India’s installed solar capacity is projected to generate over 11 million tonnes of solar waste by 2047, no single company can build all the necessary collection, dismantling, and recycling infrastructure alone. Collaborations enable:

- Economies of scale in reverse logistics,

- Access to specialised recycling technologies,

- Compliance with Extended Producer Responsibility (EPR) obligations under the E-Waste (Management) Rules, 2022,

- Standardisation of waste handling across the industry.

4.1.2 Producer Responsibility Organizations (PROs)

Gautam Solar shall actively participate in SolaREcycle India, the country’s first solar-specific Producer Responsibility Organization, launched by the National Solar Energy Federation of India (NSEFI). Through this PRO, Gautam Solar will collaborate with other manufacturers, recyclers, technology providers, and developers to:

- Establish common collection networks across states,

- Share best practices on dismantling and recycling,

- Work with the Ministry of New and Renewable Energy (MNRE) and the Ministry of Environment, Forest and Climate Change (MoEFCC) on policy development,

- Aggregate waste volumes to make recycling economically viable.

4.1.3 Research & Innovation Partnerships

Gautam Solar shall collaborate with research groups to advance solar waste management:

| Partner | Collaboration Focus |
| --- | --- |
| National Institute of Solar Energy (NISE) | Joint research on recycling technologies and development of domestic recycling capacity (similar to Attero’s MoU with NISE). |
| Council on Energy, Environment and Water (CEEW) | Data sharing and policy advocacy for circular economy in solar panels. |
| MNRE Innovation Challenge on Circularity | Participation in government-led R&D programmes for recycling and second-life applications of solar modules. |

4.1.4 Industry Coalitions

Gautam Solar shall be an active member of the Solar Waste Management Coalition under the Indian Solar Manufacturers Association (ISMA), working to:

- Develop voluntary industry standards for collection and recycling,

- Create a shared reverse logistics platform,

- Engage with the Central Pollution Control Board (CPCB) on regulatory updates.

4.1.5 Authorised Recyclers & Dismantlers

Gautam Solar shall empanel and maintain a list of authorised e-waste recyclers and dismantlers (registered under the E-Waste (Management) Rules, 2022) for the handling of returned solar waste. Priority will be given to recyclers with high resource recovery rates (e.g., >95% recovery) and those that have achieved certifications such as R2 (Responsible Recycling) or e-Stewards. All recyclers must be listed on the CPCB’s EPR Portal.

4.1.6 Public/Private Waste Management Initiatives

Gautam Solar shall work with state-level government agencies (e.g., State Pollution Control Boards, municipal corporations) to integrate its take-back scheme with existing e-waste collection centres, particularly in the states where it has major customer bases (Uttarakhand, Haryana, Delhi NCR). This will be done in a manner consistent with the CPCB’s draft guidelines for solar waste.

4.2 Provision of End-of-Life Information to Product Users

4.2.1 Information Channels

Gautam Solar shall provide end-of-life management information to all product users through the following channels:

| Channel | Content Provided |
| --- | --- |
| Product packaging | A printed leaflet (in English and Hindi) with the heading: “End of Life Management for Your Gautam Solar Module”. It includes a QR code linking to the online take-back portal. |
| User manual | A dedicated chapter on “End-of-Life Disposal,” listing collection points, authorised recyclers, and the free take-back procedure. |
| Company website | A dedicated page: gautamsolar.com/end-of-life, containing: - List of collection points by state, - Helpline number for pick-up requests, - Instructions on how to prepare modules for return, - Types of products accepted (modules, inverters, mounting structures, electrical cables), - Information on hazardous substances and why modules should not be landfilled. |
| Customer portal (registered users) | Personalised reminders for system owners when their modules approach the end of their expected useful life (25–30 years). |
| Customer service helpline | A dedicated phone number (+91-8460710074) for end-of-life inquiries, with trained staff to guide users on return options. |

4.2.2 Information on Collection, Recycling and Disposal Infrastructure

The end-of-life information shall explicitly describe the available infrastructure to ensure that users can return modules free of charge (for residential and small commercial systems) or at low cost (for large utilities). The information includes:

- Drop-off collection points at authorised dealer locations (list provided on the website).

- Pick-up service for bulk volumes (≥10 modules) – users contact the helpline to schedule a free pick-up.

- Authorised recyclers – a list of CPCB-authorised recyclers to whom collected waste will be sent, including their contact details and recycling capabilities (e.g., recovery of glass, aluminium, copper, silver, silicon).

- Prohibition of landfill – clear warnings that solar modules must never be disposed of in open areas or landfills, as they contain hazardous materials such as lead, cadmium, and antimony. The information explains that improper disposal can lead to soil and water contamination and poses health risks to communities and workers handling the waste.

4.2.3 Highlighting the Circular Economy Benefits

The end-of-life information shall emphasise that returning modules for recycling contributes to:

- Minimising waste to landfill – diverting thousands of tonnes of waste.

- Preventing leakage of hazardous substances into the environment.

- Reducing virgin material use – recycling aluminium, glass, copper, and silver from old modules can supply a significant portion of the materials needed for new modules. By 2047, recycling could meet approximately 60% of India’s silicon demand and 50% of its aluminium demand.

4.2.4 Use of Trademarked Product Names

All end-of-life information materials may include the Gautam Solar brand name and product trademarks (e.g., “G2X”, “G2X N-type TOPCon”, “R-Series”) to ensure clear identification of the products covered.

4.3 Reduction of Substances of Concern in Electrical and Electronic Equipment

4.3.1 Regulatory Framework

The E-Waste (Management) Rules, 2022 (and the 2023 amendment) aim to reduce the use of hazardous substances such as lead, mercury, and cadmium in the manufacture of electrical and electronic equipment. Rule 16 specifies a maximum concentration value (MCV) of 0.1% by weight for lead, mercury, hexavalent chromium, PBB, and PBDE in homogenous materials, and 0.01% for cadmium.

However, the 2023 amendment added two exemptions to Schedule II of the rules, including cadmium and lead in solar panels/cells, solar photovoltaic panels/cells/modules. These exemptions acknowledge the technical difficulties in immediately replacing lead in solders and cadmium in certain thin-film technologies. Nevertheless, Gautam Solar commits to voluntarily phasing out these substances wherever technically feasible.

4.3.2 Targeted Substances and Substitution Plan

| Substance | Primary Use in Solar Modules | Substitution Action | Target Completion |
| --- | --- | --- | --- |
| Lead (Pb) | Solder alloy for cell interconnection (typically Sn-Pb). | Transition to lead-free solders (e.g., Sn-Ag-Cu, SAC305). Lead-free alternatives have slightly higher melting points but are now commercially available for PV assembly. | By 31 December 2026 (for all new product lines). |
| Cadmium (Cd) | Not intentionally used by Gautam Solar. However, some imported cells may contain trace cadmium. | Supplier declaration of cadmium-free cells. Independent testing for cadmium using X-ray fluorescence (XRF) spectrometry per IS 16197. | Already in effect (zero tolerance). |
| Mercury (Hg) | No intentional use. May be present in old fluorescent lamps or thermometers in ancillary equipment. | All such mercury-containing devices have already been replaced (as per GS-EN-002, Section 4.8). | Completed. |
| Hexavalent Chromium (Cr⁶⁺) | May be used in anti-corrosion treatments for aluminium frames. | Switch to chromate-free conversion coatings (e.g., trivalent chromium or zirconium-based). | By 30 June 2027. |
| PBB / PBDE (brominated flame retardants) | Used in backsheet, junction box plastic housing, and cable insulation. | Switch to halogen-free flame retardants (e.g., phosphorus-based or inorganic mineral fillers) that meet UL 94 V-0 without bromine or chlorine. | By 31 December 2027. |
| DEHP, BBP, DBP, DIBP (phthalates) | Plasticisers in cables and some polymer components. | Use phthalate-free, bio-based plasticisers or alternative polymers (e.g., cross-linked polyethylene). | By 31 December 2027. |
| Zinc, Yttrium, Beryllium, Nickel | Present in some alloys or as trace elements. | Not restricted by regulation, but Gautam Solar will avoid intentional addition of beryllium (a known carcinogen) and will use low-nickel stainless steel only where necessary. | Continuous improvement. |
| Brominated flame retardants (BFRs) | As above (PBB/PBDE). Additionally, other BFRs (e.g., HBCD) are POPs regulated under the Stockholm Convention. | Use non-halogenated FRs. Supplier declaration that no POPs are present (Form GS-EN-F19). | 31 December 2027. |
| Antimony trioxide | Used as a flame retardant synergist in backsheets and cables. | Substitute with zinc stannate or other non-antimony synergists. Antimony is also a substance of concern under the E-Waste Rules. | 31 December 2027. |
| Tin (Sn) | Used in lead-free solder; not restricted but consumption should be minimised. | Optimise solder joint design to reduce tin volume per module. Monitor consumption per MW. | Continuous reduction. |
| Polyvinyl chloride (PVC) | Used in some cable insulation. | Switch to halogen-free, recyclable alternatives such as cross-linked polyethylene (XLPE) or thermoplastic elastomers (TPE). | 30 June 2026. |

4.3.3 Compliance and Verification

- Supplier declaration: All suppliers of materials (cells, ribbons, backsheets, cables) must complete a Restricted Substances Declaration form (GS-CE-F01), confirming compliance with Gautam Solar’s RSL and the E-Waste Rules, 2022.

- Incoming testing: Random samples of incoming materials shall be sent to an NABL-accredited laboratory for testing of restricted substances (e.g., lead in ribbons, antimony in backsheets).

- Annual review: The list of restricted substances and substitution targets shall be reviewed annually by the Product Development Manager and updated as new safer alternatives become commercially viable.

4.4 Component Labelling for Collection and Sorting

4.4.1 Importance of Labelling

Solar modules contain a diverse mix of materials: glass (≈70% by weight), aluminium frames (≈15%), encapsulation polymers (≈5–8%), cells (≈3–5%, containing silicon, silver, copper, and trace metals), junction boxes (≈2%, containing plastics, copper, diodes), and cables (≈2%). Clear labelling enables downstream recyclers to sort components efficiently, maximising material recovery.

4.4.2 Label Specifications

Gautam Solar shall implement the following labelling practices:

- Aluminium frame: Engraved or printed label with the material code Al and the recycling symbol (♻️) with number 40 (aluminium).

- Junction box: Moulded-in symbol indicating PP (polypropylene) and PB FREE (lead-free solder) or HALOGEN-FREE as applicable.

- Cable insulation: Periodic printing along the cable indicating XLPE (cross-linked polyethylene) or TPE (thermoplastic elastomer) and FREE OF PHTHALATES.

- Glass: Permanent marking on the edge of the glass (laser-etched) indicating GLASS – RECYCLABLE and the thickness (e.g., 3.2mm).

- Backsheet: Printed code indicating the type (e.g., PET or PVDF) and HALOGEN-FREE if applicable.

For modules that contain restricted substances above the threshold but within the regulatory exemption (e.g., lead-containing solder in older product lines), a visible label shall state:

“CONTAINS LEAD – NOT FOR LANDFILL – RETURN TO GAUTAM SOLAR FOR RECYCLING”

4.4.3 Digital Labelling (QR Code)

Every Gautam Solar module shall have a permanent QR code (laser-etched on the junction box or printed on the backsheet) that encodes:

- Product model number,

- Manufacturing date,

- Material composition (glass, aluminium, polymer, cell type),

- Location of hazardous substances (e.g., solder joints contain lead if pre-2027),

- End-of-life handling instructions.

Recyclers can scan the QR code at the time of decommissioning to instantly access detailed material data, facilitating automated sorting.

4.4.4 Identification of Hazardous Components

Components that contain restricted substances (e.g., soldered interconnections, diodes) shall be physically located and marked (via a diagram in the user manual and on the product label) to enable safe removal before shredding.

4.5 Design for Circularity (Collection, Disassembly, Sorting, Remanufacturing, Recycling)

4.5.1 Guiding Principles

Gautam Solar adopts the following design principles to facilitate end-of-life circularity:

| Principle | Description | Implementation |
| --- | --- | --- |
| Modularity | Break down the product into independent, replaceable modules. | Junction box, frame, and connector are designed as separate, replaceable modules. |
| Standardised parts | Use industry-standard connectors (e.g., Stäubli MC4-EVO2 equivalents) and frame profiles to maximise interoperability and repairability. | All junction boxes use standard MC4-compatible connectors. Frame dimensions follow the industry standard for 60-cell and 72-cell formats. |
| Reversible connections | Avoid permanent adhesives or welds where disassembly is required. Use screws, clips, or snap-fits. | Frame is attached using mechanical clamps, not glued to the backsheet. Junction box is secured with clips and sealant (but sealant is peelable). |
| Material purity | Avoid composites that cannot be separated (e.g., metal-polymer laminates). Use mono-material components where possible. | Frame is pure aluminium; junction box body is pure polypropylene; cable is XLPE. |
| Use of recycled content | Incorporate post-consumer recycled (PCR) materials, such as recycled aluminium for frames and recycled glass for new glass. | Target: 30% recycled aluminium in frames by 2028. |

4.5.2 Design for Disassembly (DfD)

Gautam Solar’s modules are designed to be disassembled using common hand tools without damaging reusable components. The disassembly steps are:

| Step | Component | Tool | Time (approx.) |
| --- | --- | --- | --- |
| 1 | Remove frame clamps | Hex key (Allen) | 1 minute per module |
| 2 | Detach aluminium frame from laminate | Pull apart (no adhesive) | 2 minutes |
| 3 | Remove junction box from backsheet | Cut sealant with knife, unclip | 1 minute |
| 4 | Cut off cables from junction box | Wire cutter | 30 seconds |
| 5 | Remove residual polymer if needed | Thermal delamination (optional) | – |

A disassembly manual (GS-CE-F02) is available on the company website for use by recyclers.

4.5.3 Remanufacturing

Modules that are returned under the take-back scheme and are still functional (e.g., after minor cell failure but overall >80% of original output) shall be routed to a remanufacturing line at the Haridwar factory. Remanufacturing includes:

- Replacing failed bypass diodes,

- Repairing minor junction box damage,

- Re-framing if required,

- Retesting (EL, IV, insulation) to certify output and safety.

Remanufactured modules are sold with a limited 5-year warranty as “Certified Pre-owned” for applications where absolute long-term reliability is not required (e.g., off-grid systems, temporary installations).

4.5.4 Recycling of Non-Remanufacturable Modules

Modules that cannot be remanufactured are sent to authorised recyclers who recover materials using processes such as:

- Mechanical recycling: Glass crushing, metal separation, polymer pyrolysis.

- Thermal delamination: Removing the encapsulant (EVA/POE) to free silicon cells.

- Chemical leaching: Recovering silver and copper from cells.

Gautam Solar requires recyclers to achieve material recovery rates of at least 90% for glass and aluminium, and >95% for silver and copper, wherever technically feasible.

4.6 Collection Scheme – Free Return and Buy-Back

4.6.1 Overview of Collection Scheme

Gautam Solar implements a national take-back scheme that allows any consumer (residential, commercial, or industrial) to return end-of-life Gautam Solar modules free of charge (for residential and small commercial systems) or at a low, negotiated cost (for large utility volumes). The scheme complies with the CPCB’s mandate for producers to establish collection mechanisms and publicise take-back systems.

4.6.2 Return Methods

| Customer Type | Return Method | Cost to Customer |
| --- | --- | --- |
| Residential (≤5 modules) | Drop-off at any authorised dealer’s collection point | Free |
| Small commercial (6–50 modules) | Drop-off or request a pick-up (helpline) | Free (for pick-up) |
| Large commercial / utility (>50 modules) | Scheduled pick-up by authorised logistics partner | Negotiated (typically ≤₹500 per module) |

4.6.3 Buy-Back Offer

For modules that are still functional but being replaced for upgrade purposes (e.g., upgrading to higher-efficiency TOPCon modules), Gautam Solar offers a buy-back programme:

- Age of module | Buy-back percentage of current market price (at the time of return)

- ≤5 years | 40%

- 5 and ≤10 years | 25%

- 10 and ≤15 years | 15%

- 15 years | 5% (or free return only)

The buy-back amount is credited to the customer’s account and can be used toward the purchase of new Gautam Solar modules.

4.6.4 Handling of Returned Products

| Stage | Action |
| --- | --- |
| Collection point | Dealer or collection centre stores returned modules in a covered, dry, well-ventilated area, stacked no more than 20 layers or 2 metres high (whichever is lower), on impervious flooring. |
| Transport | Modules are transported in covered trucks that prevent breakage and leakage of hazardous substances. |
| Central storage | At the Haridwar and Bhiwani factories, dedicated e-waste storage areas are maintained (meeting CPCB specifications: fire protection, emergency exits, clear labelling, spill kits). |
| Screening | QA team performs visual inspection and basic electrical testing to determine if the module is: (a) remanufacturable, (b) suitable for part salvage, or (c) only for material recycling. |
| Disposition | – Remanufacturable → Section 4.5.3– Part salvage → dismantle for reusable components (junction boxes, connectors, cables) – Material recycling → forward to authorised recycler. |

4.6.5 Record Keeping and Reporting

All returned modules are logged in the Take-Back Register (GS-CE-F03), which records:

- Customer name and contact details,

- Number and model of modules returned,

- Date of return,

- Condition (visual, test results),

- Disposition path (remanufacture, salvage, recycle),

- Final destination (recycler name, certificate reference).

Annual returns are filed on the CPCB EPR Portal as required under the E-Waste (Management) Rules, 2022.

5. Records & Forms

| Form ID | Name | Retention |
| --- | --- | --- |
| GS-CE-F01 | Restricted Substances Declaration (Supplier) | 5 years |
| GS-CE-F02 | Module Disassembly Manual (published) | Life of product + 5 years |
| GS-CE-F03 | Take-Back Register | 10 years |
| GS-CE-F04 | End-of-Life Information Leaflet (master copy) | 3 years |
| GS-CE-F05 | Authorised Recycler List (approved by CPCB) | Current + 2 years |

All records stored in \\GautamSolar\Product_Development\Circular_Economy\.

6. References

| Reference | Relevance |
| --- | --- |
| E-Waste (Management) Rules, 2022 (including 2023 amendment) | Legal framework for e-waste management, RoHS provisions, and EPR |
| CPCB Draft Guidelines for Storage and Handling of Waste Solar PV Modules (2025) | Storage, transport, and labelling requirements |
| Minamata Convention on Mercury (2013) | Phase-out of mercury-containing equipment |
| Stockholm Convention on Persistent Organic Pollutants (2001) | Prohibition of POPs (e.g., certain flame retardants) |
| ISO 14040 / 14044 – Life Cycle Assessment | Environmental assessment methodology |
| IEC 62759 – Transportation testing of PV modules | Design for transportation and handling (indirectly informs durability) |
| RoHS Directive (2011/65/EU) – reference for restricted substances | Best practice for hazardous substance reduction |
| E-Waste (Management) Rules, 2016 / 2022 – India | Legal basis for PROs, collection, and recycling targets |

7. Appendices

Appendix A – Example Take-Back Request Form (for customers)

| Field | Input |
| --- | --- |
| Customer name |  |
| Contact number |  |
| Email |  |
| Installation address |  |
| Product model(s) |  |
| Approximate number of modules |  |
| Age of modules (years) |  |
| Reason for return (e.g., end of life, damaged, upgrade) |  |
| Preferred return method (drop-off/pick-up) |  |

Appendix B – Quick Reference: Restricted Substances & Substitution Status

| Substance | Threshold (MCV) | Gautam Solar Status | Target Completion |
| --- | --- | --- | --- |
| Lead (Pb) | 0.1% w/w | Phase-out in progress | 31 Dec 2026 |
| Cadmium (Cd) | 0.01% w/w | Already compliant (not used) | Completed |
| Mercury (Hg) | 0.1% w/w | Compliant (not used) | Completed |
| Hexavalent Chromium | 0.1% w/w | Phase-out in progress | 30 Jun 2027 |
| PBB / PBDE | 0.1% w/w | Phase-out in progress | 31 Dec 2027 |
| DEHP, BBP, DBP, DIBP | 0.1% w/w (each) | Phase-out in progress | 31 Dec 2027 |
| Antimony trioxide | – | Phase-out in progress | 31 Dec 2027 |
| PVC (in cables) | – | Phase-out in progress | 30 Jun 2026 |

| SOP No. | GS-COMP-001 |
| --- | --- |
| Version | 1.0 |
| Title | # Third-Party Compliance Due Diligence |
| Department | Legal & Compliance / Information Security (ISMS) |

1. Purpose

This SOP establishes a comprehensive due diligence framework at Gautam Solar to:

- Investigate third parties (suppliers, vendors, commission agents, brokers, sales representatives, distributors, contractors, customs brokers, consultants, and all other business partners) to ensure anti-corruption compliance and prevent third-party corruption incidents, in accordance with applicable laws including the Prevention of Corruption Act, 1988 (as amended in 2018), the Companies Act, 2013, and the UN Convention against Corruption;

- Investigate third parties to ensure information security compliance and prevent third-party information security incidents, in accordance with ISO/IEC 27001:2022 (Controls A.5.19, A.5.20, A.5.21, A.5.22, A.5.23) and the CERT-In Comprehensive Cyber Security Audit Policy Guidelines (2025);

- Operationalise a risk-based approach to third-party due diligence, underpinned by systematic data collection and benchmarking against industry standards; and

- Embed contractual protections, ongoing monitoring, and audit rights to detect and mitigate compliance failures throughout the lifecycle of each third-party relationship.

This SOP applies to all third parties that (a) act on behalf of Gautam Solar, (b) interact with public officials or handle government-related transactions on Gautam Solar´s behalf, or (c) have access to Gautam Solar´s information assets, systems, networks, or confidential data.

2. Scope

The SOP covers all categories of third parties engaged by Gautam Solar, including but not limited to:

- Raw material suppliers (silicon, glass, aluminium, ribbons, EVA/POE, backsheet, junction boxes)

- Equipment manufacturers (stringers, laminators, testers)

- Commission agents and sales representatives

- Brokers (export/import)

- Customs brokers

- Consultants (legal, technical, ESG)

- Contractors (construction, maintenance, logistics)

- IT service providers (cloud hosting, data centre services, software vendors)

- Any other third party providing goods or services on behalf of, or in connection with, Gautam Solar’s operations

Exclusions: This SOP does not apply to one-off, low-value purchases of standard off-the-shelf items where the supplier has no ongoing business relationship, no access to sensitive information, and no contact with government officials. However, any such exemption must be documented and approved by the Compliance Officer.

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Compliance Officer (Legal & Compliance Department) | Overall owner of this SOP; maintains the Third-Party Risk Register; performs high-risk due diligence; reviews red flags; approves or rejects third-party engagements; files annual compliance certifications. |
| Information Security Manager (ISMS) | Owns the information security due diligence process; maintains supplier security assessment checklists; reviews ISO 27001 certifications and SOC reports; manages remediation tracking. |
| Procurement Department | Collects initial due diligence information from all third parties before engagement; flags high-risk third parties for enhanced due diligence. |
| Internal Audit | Conducts periodic independent reviews of the third-party due diligence process; reviews audit rights exercised against high-risk third parties. |
| All Employees (especially procurement, sales, and logistics) | Must not engage any third party without completing due diligence per this SOP; report any suspicious third-party activity through the whistleblower channel. |

4. Detailed Procedures

4.1 Anti-Corruption Due Diligence

4.1.1 Legal Framework

India’s anti-corruption legal framework provides the baseline for Gautam Solar’s due diligence obligations:

| Legal Instrument | Key Provisions Relevant to Third-Party Due Diligence |
| --- | --- |
| Prevention of Corruption Act, 1988 (as amended in 2018) | Criminalises bribery of public officials; extends corporate liability to companies whose employees or third parties bribe officials; the 2018 amendment introduced an “adequate procedures” defence for commercial organisations that have implemented reasonable anti-bribery procedures. |
| Companies Act, 2013 – Section 447 (Fraud) | Defines fraud broadly, including acts committed “by means of corruption”. Mandates directors and senior management to maintain systems for ensuring compliance with applicable law. |
| UN Convention against Corruption | India is a signatory; reinforces the obligation to implement transparent and verifiable third-party due diligence. |
| Prevention of Money Laundering Act, 2002 | Imposes customer due diligence (CDD) obligations on certain entities to prevent money laundering and terrorist financing. |

While Gautam Solar is a private company and not a designated reporting entity under PMLA, it adopts best-practice CDD principles as a prudent business standard.

4.1.2 Risk-Based Due Diligence Approach

Third parties are categorised into three risk tiers based on the following risk indicators:

| Risk Tier | Definition | Due Diligence Level | Approval Authority |
| --- | --- | --- | --- |
| Tier 1 – Low Risk | Suppliers/vendors with no interaction with public officials; no access to Gautam Solar’s confidential data; low transaction value (<₹10 lakh/year); located in low-corruption jurisdictions (Transparency International CPI score >70). | Standard DD – basic identity verification; watchlist screening; compliance declaration form. | Procurement Manager |
| Tier 2 – Medium Risk | Third parties with occasional contact with government authorities (e.g., customs brokers); moderate transaction value (₹10–50 lakh/year); located in jurisdictions with moderate corruption risk (CPI score 40–70). | Standard DD + – enhanced documentation; review of financial statements; conflict-of-interest declaration from third-party principals; three trade references. | Compliance Officer |
| Tier 3 – High Risk | Agents, consultants, distributors, or contractors with frequent interaction with public officials (including government procurement); any third party engaged to obtain permits, licences, or regulatory approvals on Gautam Solar’s behalf; any third party in a high-corruption jurisdiction (CPI score <40). | Enhanced DD (Level 2) – all of the above, plus beneficial ownership verification (ownership structure chart, ultimate beneficial owners); on-site visit or remote video inspection; forensic background checks through external agency; reputational review using open-source intelligence (OSINT); annual compliance recertification. | Compliance Committee (Legal, Finance, Internal Audit) |

Indicators for risk escalation (red flags):

- Third party requests payment to a bank account in a different country from the one in which it operates;

- Third party refuses to provide complete ownership information or references;

- Third party has been named in any regulatory enforcement action (SEBI, CBI, ED, Interpol);

- Third party has pending litigations for fraud, corruption, or money laundering;

- Third party is located in a jurisdiction with high corruption risk, as per Transparency International’s Corruption Perceptions Index (CPI) or the World Bank’s Control of Corruption indicator;

- Third party is related to (or recommended by) a Gautam Solar employee (potential conflict of interest);

- Third party requests that payments be made in cash, through offshore accounts, or to a third-party beneficiary;

- Third party insists on unusual confidentiality terms that would prevent Gautam Solar from conducting compliance checks.

4.1.3 Data Collection on Third Parties

For all third parties (regardless of tier), Gautam Solar collects the following baseline information before contract execution:

| Data Element | Documentation Required | Collection Method |
| --- | --- | --- |
| Legal identity | Certificate of Incorporation; PAN; GST registration | Third-Party Onboarding Form (Form GS-COMP-F01) |
| Business address and primary place of operation | Utility bill or lease agreement (not older than 3 months); bank statement confirming address | Upload to Vendor Portal |
| Ownership and control (up to 20% beneficial ownership) | Declaration of beneficial ownership (Form GS-COMP-F02); shareholding pattern for companies; partnership deed for partnerships | Third-Party Self-Declaration |
| Key principals (directors, partners, authorised signatories) | List of names, nationalities, passport copies; biographical information | Third-Party Self-Declaration |
| Financial stability | Audited financial statements (last 2 years) for Tier 2 & Tier 3; banker’s reference | Upload to Vendor Portal |
| Regulatory compliance history | Self-declaration of any past or pending litigation, regulatory sanctions, or convictions for corruption, fraud, or money laundering (Form GS-COMP-F03) | Third-Party Self-Declaration |
| References | Two professional references from existing clients for Tier 2; three for Tier 3 | Direct contact by Procurement |
| Conflict of interest | Declaration that neither the third party nor its principals has any family or business relationship with any Gautam Solar employee involved in the selection or contracting process (Form GS-COMP-F04) | Third-Party Self-Declaration |

For High-Risk (Tier 3) third parties, in addition to the above:

- Beneficial ownership verification – ownership structure chart identifying all individuals with 10% or more direct or indirect ownership interest;

- Adverse media screening – search of publicly available databases, news archives, and regulatory enforcement records for the third party entity and each principal;

- Background verification – for agents, sales representatives, and consultants, Gautam Solar may engage an external due diligence provider (e.g., Kroll, Exiger, local Indian equivalent) to conduct enhanced background checks;

- Site visit – physical or virtual (video) inspection of the third party’s office and operations;

- Government official check – confirmation that no principal holds a position as a public official (or, if so, a detailed risk assessment and mitigation plan approved by the Compliance Committee).

4.1.4 Benchmarking and Comparative Assessment

Gautam Solar benchmarks its anti-corruption due diligence programme against industry standards, global best practices, and regulatory requirements:

| Benchmark | Application |
| --- | --- |
| India Disclosure Index – assesses voluntary non-financial disclosures of India’s top 100 listed companies across transparency, governance maturity, and preparedness for emerging risks | Gautam Solar aligns its due diligence disclosures with market-leading transparency benchmarks, even though it is not a listed company, to ensure best-in-class governance. |
| FCPA and UK Bribery Act “adequate procedures” principles – six principles: proportionate procedures, top-level commitment, risk assessment, due diligence, communication, monitoring and review. | Gautam Solar’s due diligence framework incorporates all six principles, with particular emphasis on risk-based due diligence and ongoing monitoring. The Foreign Corrupt Practices Act (FCPA) imposes liability on companies for corrupt activities performed by third parties on their behalf, while the UK Bribery Act requires businesses to implement “adequate procedures” to prevent bribery within third-party networks. Gautam Solar applies these international standards to its Indian operations. |
| Basel Institute on Governance Good Practice Guidelines on Conducting Third Party Due Diligence – a practitioner’s guide covering corruption risks and anti-corruption strategies | Gautam Solar follows the step-by-step methodology recommended by the Basel Institute for all high-risk third-party assessments. |
| Peer benchmarking – comparison with procurement practices of similar-sized manufacturing companies in India (solar and allied industries) | Annual benchmarking exercise conducted by Internal Audit using publicly available disclosure reports and industry surveys. |

Screening tools and watchlists: Gautam Solar screens all new third parties (and annually for existing) against the following global watchlists using commercially available screening software or external agency:

- UN Sanctions List

- World Bank Listing of Ineligible Firms and Individuals

- OFAC (U.S. Office of Foreign Assets Control) – SDN List

- Indian Ministry of Home Affairs – banned entities

- Interpol – Red Notices

4.1.5 Documentation of Results

All due diligence findings are documented in the Third-Party Risk Register (GS-COMP-F05), which records:

- Third-party name, type, risk tier, and date of risk assessment;

- Compliance scores for each data element collected;

- Any red flags identified and the rationale for acceptance/rejection;

- Approval sign-off and date;

- Date of next scheduled monitoring review.

Where a red flag is identified but the business case for engaging the third party nonetheless exists, the Compliance Committee must approve a Risk Mitigation Plan (GS-COMP-F06) detailing:

- Specific measures to reduce the risk (e.g., reduced scope, quarterly compliance certifications, escrow payment arrangements);

- Enhanced monitoring frequency (e.g., monthly instead of quarterly);

- Exit triggers – conditions under which Gautam Solar will terminate the relationship.

4.1.6 Ongoing Monitoring (Including Audit Rights)

Due diligence is not a one-time event. The following ongoing monitoring obligations apply:

| Risk Tier | Monitoring Frequency | Audit Rights Exercise |
| --- | --- | --- |
| Tier 1 (Low) | Annual recertification (Form GS-COMP-F03 – updated declaration) | None, unless triggered by a red flag |
| Tier 2 (Medium) | Semi-annual recertification; quarterly review of any payments to government authorities or related parties | Right to audit (financial records) once every 2 years, exercisable at Gautam Solar’s option with 30 days’ notice |
| Tier 3 (High) | Quarterly recertification; quarterly compliance report certified by third party’s compliance officer; annual on-site compliance audit | Right to audit (all financial records, contracts, compliance policies, and training logs) annually, exercisable at Gautam Solar’s option with 15 days’ notice |

Payment review (all tiers): Finance Department must review all payments to third parties against the contract and approved commercial terms. Any deviation (e.g., payment to a bank account in a different jurisdiction) must be flagged to the Compliance Officer before disbursement.

4.1.7 Contractual Protections

Every third-party contract shall contain the following non-negotiable clauses (except for Tier 1 low-risk third parties where the Compliance Officer may waive):

- Anti-corruption representation and warranty – third party confirms compliance with all applicable anti-corruption laws;

- Audit clause – Gautam Solar reserves the right to audit third-party books and records relevant to the engagement, at Gautam Solar’s expense, with reasonable notice;

- Termination for cause – material breach of the anti-corruption provisions entitles Gautam Solar to terminate the contract immediately without liability;

- Reporting obligation – third party must immediately report to Gautam Solar any request for improper payments or any actual or suspected violation of anti-corruption laws in connection with Gautam Solar’s business;

- Subcontractor flow-down – if third party subcontracts any part of the work, the same anti-corruption obligations must be flowed down to the subcontractor, and third party remains fully liable.

4.2 Information Security Due Diligence

4.2.1 Legal Framework and Industry Standards

Gautam Solar’s information security due diligence process is aligned with:

| Standard / Regulation | Relevance |
| --- | --- |
| ISO/IEC 27001:2022 – Controls A.5.19 to A.5.23 | Provides the framework for managing information security risks in supplier relationships throughout the supplier lifecycle. Control A.5.19 requires policies and procedures to protect organisational assets accessible by suppliers; A.5.20 mandates that security requirements be agreed with each supplier based on risk; A.5.21 extends these requirements to the ICT supply chain; A.5.22 requires ongoing monitoring and review; A.5.23 governs cloud service providers. |
| CERT-In Comprehensive Cyber Security Audit Policy Guidelines (2025) | Mandates annual third-party cybersecurity audits for all public and private enterprises operating in India’s digital ecosystem, effective 25 July 2025. The audit scope must align with ISO/IEC 27001 and reflect actual business risk. In particular, all companies must now undergo annual third-party cybersecurity audits that are not surface-level compliance checks but must reflect actual business risk. The full audit lifecycle—including risk-based planning, control validation, audit execution, and remediation tracking—is now required. |
| ISO/IEC 27001:2022 – Annex A Control 5.19 (Information security in supplier relationships) | Establishes that supplier relationships must be governed to protect the organisation’s information and services, requiring risk-based segmentation of suppliers—by data sensitivity, service criticality, connectivity, and substitution difficulty—and due diligence that assesses security posture before onboarding. |

4.2.2 Risk-Based Supplier Segmentation

Under ISO/IEC 27001:2022 Control A.5.19, Gautam Solar classifies third parties into information security risk tiers based on:

| Classification Factor | Criteria |
| --- | --- |
| Data sensitivity | Does the third party have access to personal data (employee, customer), trade secrets, financial data, or intellectual property? |
| Service criticality | Would a disruption to the third party’s services materially impact Gautam Solar’s operations? |
| Connectivity | Does the third party have network connectivity to Gautam Solar’s systems (VPN, API, remote access)? |
| Substitution difficulty | Can the service be easily replaced by another vendor without operational disruption? |
| Subcontractor dependencies | Does the third party rely on subcontractors that also have access to Gautam Solar’s information assets? |

Risk classification matrix:

4.2.3 Data Collection for Information Security

Gautam Solar collects the following information security data from all third parties (tier-appropriate):

| Data Element | Collection Method | Minimum Frequency of Update |
| --- | --- | --- |
| Information security policies (access control, data classification, incident management, business continuity, supplier relationships) | Security questionnaire (Form GS-COMP-IS-F01) – based on ISO 27001 control families | Onboarding; annual recertification |
| ISO 27001 certification status (valid certificate and scope) | Request copy of certificate | Onboarding; immediately upon expiry/renewal |
| SOC 2 Type II report (or equivalent) | Request report (with redactions permitted) | Onboarding; upon refresh (typically annually) |
| Incident history – number of security incidents in last 24 months involving Gautam Solar data | Self-declaration (Form GS-COMP-IS-F02) | Onboarding; upon any material incident |
| Data breach notification process – timeline and method for notifying Gautam Solar | Review of contractual clause and incident response plan | Onboarding |
| Data location – geographical location(s) where Gautam Solar data will be stored or processed | Declaration (Form GS-COMP-IS-F03) | Onboarding; upon any change |
| Subcontractors – list of all subcontractors with access to Gautam Solar data | Declaration (Form GS-COMP-IS-F04) | Onboarding; upon any addition |
| Vulnerability management – frequency of scanning, penetration testing, and remediation SLAs | Security questionnaire / interview | Onboarding |
| Business continuity and disaster recovery – RTO and RPO for services supporting Gautam Solar | Security questionnaire / contractual annex | Onboarding |

For High-Risk (IS-Tier 3) third parties, Gautam Solar also requires:

- Certified audit report – the third party must undergo an independent cybersecurity audit annually (consistent with CERT-In 2025 guidelines) using a CERT-In-empanelled auditor or equivalent recognised certification body.

- Right-to-audit – Gautam Solar reserves the right to conduct its own security audit (or appoint a third-party security assessor) at Gautam Solar’s expense, with reasonable notice.

- Evidence of staff security training – copy of security awareness training records for employees who will have access to Gautam Solar systems or data.

4.2.4 Benchmarking and Comparative Assessment

Gautam Solar benchmarks its information security due diligence programme against industry-recognised standards:

| Benchmark | Application |
| --- | --- |
| Standardised Information Gathering (SIG) Questionnaire – developed by the Shared Assessments community; the leading standard for third-party risk assessments, covering 21 risk domains and aligned with 31+ global frameworks including ISO 27001, NIST CSF, SOC 2, and GDPR. Benefits include consistency (every vendor measured against the same yardstick), defensibility (standing up to regulators), comparability (results tracked over time), and efficiency. | Gautam Solar uses the SIG framework as the baseline for its security questionnaire; the questionnaire is reviewed annually against the latest SIG 2025 release, which incorporates NIST CSF principles. |
| ISO 27001:2022 mapping – A.5.19 (risk-based segmentation of suppliers by data sensitivity, service criticality, connectivity, and substitution difficulty). | Gautam Solar’s risk-based segmentation (Section 4.2.2) directly maps to this control. |
| CERT-In 2025 Guidelines – mandating annual third-party cybersecurity audits for all enterprises, aligning with ISO/IEC 27001. | Gautam Solar’s high-risk third parties must undergo annual third-party cybersecurity audits by CERT-In-empanelled auditors. |
| ISO 27001 Control A.5.20 – requires that supplier agreements explicitly define security requirements and responsibilities, including data classification, encryption, access controls, breach notification timelines, audit and right-to-audit clauses, vulnerability disclosure duties, and exit provisions. | All IS-Tier 3 contracts incorporate these required clauses (see Section 4.2.6). |

4.2.5 Security Questionnaires

Gautam Solar uses a standardised Information Security Due Diligence Questionnaire (GS-COMP-IS-F01) that covers the following ISO 27001 control families (consistent with SIG 2025):

- A.5 – Information security policies

- A.6 – Organisation of information security

- A.7 – Human resource security

- A.8 – Asset management

- A.9 – Access control

- A.10 – Cryptography

- A.11 – Physical and environmental security

- A.12 – Operations security

- A.13 – Communications security

- A.14 – System acquisition, development and maintenance

- A.15 – Supplier relationships

- A.16 – Information security incident management

- A.17 – Information security aspects of business continuity management

- A.18 – Compliance

The questionnaire is provided to all third parties before onboarding. Responses are reviewed by the Information Security Manager. For ISO 27001-certified third parties (certificate in scope), the questionnaire may be reduced to a limited set of controls not covered by the certificate scope.

4.2.6 Contractual Protections for Information Security

All contracts with IS-Tier 2 and IS-Tier 3 third parties shall include (non-negotiable):

- Data processing agreement (DPA) – compliant with applicable data protection laws (including India’s Digital Personal Data Protection Act, 2023, if applicable);

- Confidentiality and non-disclosure obligations – surviving termination;

- Security schedule – attaching the information security questionnaire responses as a contractual commitment (not merely informative);

- Incident notification – third party must notify Gautam Solar within 24 hours of discovery of any security incident affecting Gautam Solar data (exceeding CERT-In’s 6-hour requirement for high-risk data);

- Data return and deletion – upon termination, third party must return all Gautam Solar data and provide certification of secure deletion;

- Subcontractor flow-down – any subcontractor handling Gautam Solar data must be bound by equivalent information security obligations, and third party must provide a list of all subcontractors (which must be pre-approved by Gautam Solar for high-risk data).

4.2.7 Ongoing Monitoring of Information Security

| Risk Tier | Monitoring Activities | Frequency |
| --- | --- | --- |
| IS-Tier 2 (Medium) | Review of ISO 27001 certificate validity (if certified); review of security questionnaire recertification; review of any reported incidents. | Annually |
| IS-Tier 3 (High) | Review of ISO 27001 certificate validity; review of SOC 2 Type II report; review of independent security audit report (CERT-In compliant); verify that penetration testing has been performed within the last 12 months; review of any subcontractor changes. | Semi-annually |
| All tiers | Automated continuous monitoring of third-party cybersecurity risk using third-party risk management platform (e.g., SecurityScorecard, UpGuard, BitSight) – monitoring of publicly observable security posture indicators (TLS/SSL configuration, domain reputation, open ports, malware presence, patching cadence). | Continuous |

Where a material deficiency is identified (e.g., ISO 27001 certificate expired, security incident involving Gautam Solar data, penetration testing not performed), Gautam Solar invokes the right-to-audit clause, requests a remediation plan, and may suspend the third party’s access to Gautam Solar systems until the issue is resolved. Repeated or uncorrected deficiencies result in contract termination.

4.2.8 Cloud Service Providers

For any third party providing cloud services (IaaS, PaaS, SaaS) to Gautam Solar, in addition to the above:

- The provider’s compliance with ISO 27001:2022 Control A.5.23 must be verified, including policies on confidentiality, integrity, availability, and information handling requirements for cloud services.

- The provider must agree to the Cloud Security Alliance (CSA) Security, Trust, Assurance and Risk (STAR) registration or equivalent.

- The provider’s adherence to the CSA’s SaaS Security Benchmark (SSCF) shall be reviewed, which replaces endless vendor questionnaires with a consistent, technical baseline and faster, more objective assessments.

5. Records & Forms

| Form ID | Name | Retention Period | Responsible |
| --- | --- | --- | --- |
| GS-COMP-F01 | Third-Party Onboarding Form (basic identity & contact) | Duration of relationship + 5 years | Procurement |
| GS-COMP-F02 | Beneficial Ownership Declaration | Duration of relationship + 5 years | Compliance Officer |
| GS-COMP-F03 | Compliance History Self-Declaration (anti-corruption) | Duration of relationship + 7 years | Compliance Officer |
| GS-COMP-F04 | Conflict of Interest Declaration | Duration of relationship + 5 years | Compliance Officer |
| GS-COMP-F05 | Third-Party Risk Register (consolidated) | 10 years (permanent archive) | Compliance Officer |
| GS-COMP-F06 | Risk Mitigation Plan (for accepted red flags) | Duration of relationship + 7 years | Compliance Committee |
| GS-COMP-IS-F01 | Information Security Due Diligence Questionnaire (SIG-based) | Duration of relationship + 3 years | Information Security Manager |
| GS-COMP-IS-F02 | Incident History Declaration | Duration of relationship + 3 years | Information Security Manager |
| GS-COMP-IS-F03 | Data Location Declaration | Duration of relationship + 5 years | Information Security Manager |
| GS-COMP-IS-F04 | Subcontractor Declaration | Duration of relationship + 5 years | Information Security Manager |

All records stored in \\GautamSolar\Compliance\ThirdParty_DueDiligence\ with access restricted to Legal, Finance, Information Security, and Internal Audit.

6. References

| Reference | Relevance |
| --- | --- |
| Prevention of Corruption Act, 1988 (as amended 2018) | Criminalises bribery of public officials; corporate liability; adequate procedures defence |
| Companies Act, 2013 – Section 447 (Fraud) | Defines fraud, including corruption; director’s duty to ensure compliance |
| UN Convention against Corruption (UNCAC) – 2003 | International framework for anti-corruption measures |
| ISO/IEC 27001:2022 – Information security, cybersecurity and privacy protection – Controls A.5.19 to A.5.23 | Supplier information security management framework |
| CERT-In Comprehensive Cyber Security Audit Policy Guidelines (2025) | Mandatory annual third-party cybersecurity audits for all enterprises |
| Shared Assessments – Standardised Information Gathering (SIG) 2025 | Industry benchmark security questionnaire; aligned with 31+ frameworks |
| Basel Institute on Governance – Good Practice Guidelines on Conducting Third Party Due Diligence | Practitioner’s guide for anti-corruption due diligence |
| Transparency International – Corruption Perceptions Index (CPI) | Country risk assessment for risk-based due diligence tiering |
| World Bank – Listing of Ineligible Firms and Individuals | Watchlist screening for sanctions |

7. Appendices

Appendix A – Anti-Corruption Due Diligence Flowchart (Text)

text

Third party identified for engagement

↓

Assign risk tier (Low / Medium / High) based on:

- Interaction with public officials?

- Country corruption risk (CPI score)?

- Transaction value?

- Access to confidential information?

↓

Collect baseline data (Onboarding Form F01 + self-declarations F02–F04)

↓

Check watchlists (UN, World Bank, OFAC, MHA, Interpol)

↓

Any red flags? → YES → Risk Mitigation Plan (F06) approved by Compliance Committee

↓ NO

Approved for engagement

↓

Contract signed (with required anti-corruption and audit clauses)

↓

Ongoing monitoring (tier-based recertification and audit rights)

Appendix B – Information Security Due Diligence Flowchart (Text)

Third party identified (IT / data access / cloud services)

↓

Classify IS risk tier (IS-Tier 1 / 2 / 3) based on:

- Data sensitivity?

- Service criticality?

- Network connectivity?

- Substitution difficulty?

↓

Collect information security data (Questionnaire F01 + declarations F02–F04)

↓

Review findings against ISO 27001 / SIG 2025 benchmark

↓

Material deficiency? → YES → Remediation plan required; may suspend access

↓ NO

Contract signed (with security schedule, incident notification clause, data deletion obligation)

↓

Ongoing monitoring (tier-based: continuous automated monitoring for IS-Tier 3)

Appendix C – Anti-Corruption Red Flags Quick Reference (to be displayed in Procurement Department)

| Red Flag | Action |
| --- | --- |
| Third party has a different country bank account from its country of operation | Flag to Compliance Officer; enhanced DD required |
| Third party refuses to disclose beneficial ownership | Do NOT engage |
| Third party or principal appears on watchlist | Do NOT engage |
| Third party has been convicted of corruption | Do NOT engage |
| Third party requests cash payment | Flag to Compliance Officer; enhance monitoring |
| Third party asks for confidentiality clause preventing compliance checks | Reject clause; if insisted, do NOT engage |
| Principal is a government official (or close family member) | Enhanced DD; obtain Compliance Committee approval |

Appendix D – Information Security Red Flags Quick Reference

| Red Flag | Action |
| --- | --- |
| No ISO 27001 certification AND unable to complete security questionnaire | Do NOT engage (IS-Tier 2 or above) |
| Refuses right-to-audit clause (IS-Tier 3) | Do NOT engage |
| History of data breaches involving client data in last 24 months | Enhanced DD; seek detailed incident report; may engage with additional controls |
| Data location in a high-risk jurisdiction for data protection | Assess legal and technical safeguards; may restrict data type |
| Subcontractors not disclosed and not pre-approved | Suspend access; enforce contractual provision |

Appendix E – Glossary of Terms

| Term | Definition |
| --- | --- |
| Adequate procedures defence | Defence available to a commercial organisation under the UK Bribery Act (and analogous principles under Indian law following the 2018 Prevention of Corruption Act amendment) if it can demonstrate that it had “adequate procedures” in place to prevent bribery. |
| Beneficial ownership | Any natural person who ultimately owns or controls a legal entity (directly or indirectly). Typically includes ownership of 10% or more of shares or voting rights. |
| CERT-In | Indian Computer Emergency Response Team – the national nodal agency for cybersecurity under the Ministry of Electronics and Information Technology (MeitY). |
| CPI | Corruption Perceptions Index – an annual index published by Transparency International ranking countries by perceived levels of public sector corruption (scale 0–100, where 0 = highly corrupt). |
| DPA | Data Processing Agreement – a legally binding contract between a data controller and a data processor that sets out the terms for processing personal data. |
| FCPA | Foreign Corrupt Practices Act (USA) – prohibits bribing foreign government officials to obtain or retain business. |
| ISMS | Information Security Management System – the systematic approach to managing sensitive company information under ISO 27001. |
| OFAC | Office of Foreign Assets Control (U.S. Treasury) – administers and enforces economic and trade sanctions. |
| PCA | Prevention of Corruption Act, 1988 (India) – principal anti-corruption law in India. |
| Red flag | A circumstance or indicator suggesting a heightened risk of corruption or security incident. |
| RTO | Recovery Time Objective – the maximum acceptable time to restore services after a disruption. |
| RPO | Recovery Point Objective – the maximum acceptable age of data that must be recovered after a disruption. |
| SIG | Standardised Information Gathering – questionnaire developed by Shared Assessments for third-party risk assessment. |
| SOC 2 Type II | Service Organisation Control 2 report – an attestation report covering a service organisation’s controls over a period of time (typically 12 months). |
| UNCAC | United Nations Convention against Corruption – an international anti-corruption treaty to which India is a signatory. |

| SOP No. | GS-AUD-001 |
| --- | --- |
| Version | 1.0 |
| Title | Internal Audit & Monitoring of Ethics, Social, Environmental, Sustainable Procurement and Compliance Control Systems |
| Department | Internal Audit / Compliance |

1. Purpose

This SOP establishes a systematic framework at Gautam Solar to:

- Regularly monitor the effectiveness of the company’s ethics, social, environmental, sustainable procurement, and compliance control systems through internal testing and audits.

- Ensure that these control systems are operating as intended, identify deficiencies, and drive continuous improvement.

- Document audit scope, accountable persons/entities, and retrospective controls for each audit activity, in line with industry best practices and regulatory expectations (e.g., Companies Act, 2013 – Section 177 and 179; ISO 19011:2018 guidelines for auditing management systems).

This SOP applies to all internal audits covering the following management systems:

- Ethics control system (anti-corruption, whistleblower, code of conduct)

- Social control system (labour practices, human rights, community engagement)

- Environmental control system (ISO 14001, waste management, emissions, water, energy)

- Sustainable procurement control system (supplier ESG screening, green procurement criteria)

- Compliance control system (legal and regulatory compliance, licences, permits, filings)

The SOP defines audit scope, accountable persons, and retrospective controls as mandatory elements of every audit plan and report, thereby meeting the required criteria.

2. Scope

The SOP covers internal audits (conducted by Gautam Solar’s own Internal Audit team or outsourced to an external audit firm but managed internally) of the following control systems:

| Control System | Key Processes Covered | Relevant SOPs / Standards |
| --- | --- | --- |
| Ethics | Anti-corruption due diligence (GS-COMP-001), whistleblower mechanism, code of conduct training, gift and hospitality policy, conflict of interest declarations. | GS-COMP-001; Companies Act 2013; UN Convention against Corruption |
| Social | Worker health & safety (ISO 45001), grievance mechanism, child labour prohibition, non-discrimination, community liaison, fair wages. | ISO 45001; ILO conventions; Factories Act, 1948 |
| Environmental | Waste management (GS-WM-001), water & wastewater (GS-WM-002), energy & emissions (GS-EN-001, GS-EN-002), emergency preparedness, hazardous substances, POPs, mercury. | ISO 14001; GS-EN-001/002; GS-WM-001/002; CPCB norms |
| Sustainable Procurement | Supplier ESG screening, green material substitution, life cycle assessment input, vendor code of conduct, third-party due diligence for anti-corruption & information security. | GS-COMP-001; GS-CE-001; ISO 20400 (Sustainable Procurement) |
| Compliance | Legal register (all applicable laws), licence/permit renewals, regulatory filings (pollution control, e-waste, hazardous waste, factory inspectorate), training records, document control (GS-DC-001). | GS-DC-001; all statutory requirements |

Exclusions: Financial audits (covered under separate annual statutory audit) and IT security penetration testing (covered under GS-COMP-001 – Information Security Due Diligence, Section 4.2). However, compliance aspects of IT (e.g., data protection law adherence) are included.

3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Internal Audit Head | Overall owner of this SOP; prepares the annual internal audit plan; assigns auditors; approves audit scope; reviews audit reports; tracks corrective action closure; presents findings to the Audit Committee. |
| Compliance Officer | Supports audits of ethics and compliance control systems; provides legal register and prior compliance test results; implements agreed corrective actions. |
| HSE Manager | Supports audits of environmental and social (HSE) control systems; provides records of waste, water, emissions, safety; implements corrective actions. |
| Procurement Manager | Supports audits of sustainable procurement control system; provides supplier due diligence records, green procurement metrics. |
| Process Owners (Department Heads) | Cooperate with auditors; provide access to records; implement corrective actions for findings in their area. |
| Audit Committee | Reviews the effectiveness of the internal audit function; approves the annual audit plan; reviews significant findings and management responses. |

4. Detailed Procedures

4.1 Internal Audit Planning

4.1.1 Risk-Based Audit Frequency

Gautam Solar conducts internal audits of each control system according to a risk-based schedule:

| Control System | Audit Frequency | Rationale |
| --- | --- | --- |
| Ethics | Annually (full audit) + quarterly spot checks on high-risk areas (e.g., third-party due diligence) | High reputational and legal risk |
| Social (HSE) | Half-yearly (full audit) | Critical for worker safety; high regulatory scrutiny |
| Environmental | Annually (full audit) | Regulatory compliance obligations (CPCB, SPCB) |
| Sustainable Procurement | Annually | Growing importance for ESG reporting and customer requirements |
| Compliance (legal register) | Half-yearly (rolling) – different legal clusters each half | Broad scope; proactive identification of new obligations |

In addition to scheduled audits, trigger-based audits are conducted when:

- A material non-compliance or incident occurs (e.g., environmental spill, reported corruption allegation, serious accident);

- A new law or regulation with significant impact comes into force;

- A major change in operations (new product line, new factory layout, new hazardous substance) occurs;

- At the request of the Audit Committee or CEO.

4.1.2 Audit Scope Definition (Required Element 1)

For every internal audit, the audit scope must be explicitly documented in the Audit Plan (GS-AUD-F01). The scope shall include:

| Scope Element | Description |
| --- | --- |
| System(s) audited | Which control system(s) – ethics, social, environmental, sustainable procurement, compliance – and specific sub-processes (e.g., “Anti-corruption due diligence for Tier 3 third parties” or “Hazardous waste storage and disposal”). |
| Audit period | The time period covered (e.g., “1 April 2025 to 31 March 2026” or “the last 6 months”). |
| Sites / locations covered | Delhi registered office, Haridwar factory, Bhiwani factory, or a combination. |
| Key control objectives | Specific requirements to be tested (e.g., “All Tier 3 third parties have been subject to enhanced due diligence before contract signing” or “Effluent discharge parameters have remained within CPCB limits for 95% of daily samples”). |
| Audit criteria | The standards, policies, SOPs, legal requirements against which compliance is assessed (e.g., “GS-COMP-001, Section 4.1.3” or “CPCB Schedule – Effluent Discharge Standards”). |
| Exclusions (if any) | Any processes or areas specifically excluded from the audit (e.g., “R&D laboratory records excluded due to concurrent patent filing”). |

Example audit scope (ethics control system, annual audit):

Scope: Audit of the ethics control system at all three Gautam Solar sites for the period 1 July 2025 – 30 June 2026. The audit will cover the following sub-processes: (1) third-party anti-corruption due diligence (GS-COMP-001, Section 4.1); (2) whistleblower mechanism (case logging, investigation closure); (3) employee code of conduct training completion records; (4) conflict of interest declarations (annual filing). Audit criteria: GS-COMP-001; Prevention of Corruption Act, 1988 (as amended); company Code of Conduct. Excluded: Internal audit of information security due diligence (covered under separate IT audit).

4.1.3 Accountable Person or Entity (Required Element 2)

For every internal audit, the accountable person or entity – who is responsible for the effective functioning of the audited control system – must be identified and documented in the audit plan and audit report.

| Control System | Accountable Entity | Accountable Person (Executive) |
| --- | --- | --- |
| Ethics | Compliance Committee (comprising Compliance Officer, CFO, Legal Head) | Compliance Officer (day-to-day) |
| Social (HSE) | HSE Department | HSE Manager |
| Environmental | HSE Department | HSE Manager |
| Sustainable Procurement | Procurement Department in coordination with ESG Steering Committee | Procurement Manager |
| Compliance (legal register) | Legal & Compliance Department | Compliance Officer |

In the Audit Plan (GS-AUD-F01), the accountable person is named (e.g., “Compliance Officer – Mr. XYZ”). In the Audit Report (GS-AUD-F02), the audit findings and recommendations are directed to that accountable person, who is responsible for implementing corrective actions.

4.1.4 Retrospective Control of the Existing System (Required Element 3)

For every internal audit, the audit shall include a retrospective control assessment – a review of how the existing control system has performed over a past period, not merely a snapshot at the time of the audit. The retrospective control shall cover:

- Historical records – sampling of past transactions, incidents, inspections, training records, etc., from the audit period.

- Trend analysis – identification of recurring issues or patterns (e.g., repeated failure to update legal register on time, persistent lack of supplier declarations).

- Root cause analysis for any previously identified non-conformities that have recurred.

- Effectiveness of previous corrective actions – whether actions taken after previous audits have actually eliminated the root cause.

The retrospective control is evidenced by:

- Sampling plan – e.g., “Select 30 Tier 3 third-party contracts executed in the last 12 months to verify due diligence completion dates.”

- Review of corrective action closure – e.g., “All 5 high-priority findings from the previous audit (July 2025) were marked closed. Retrospective check confirms that 4 of the 5 have remained effective; 1 recurred (hazardous waste label missing).”

- Trend charts – e.g., monthly percentage of employees completing ethics training, quarterly wastewater BOD exceedance count.

The retrospective control findings are documented in the Audit Report as a distinct section titled “Retrospective Control Assessment”.

4.1.5 Audit Team & Independence

Internal audits are conducted by the Internal Audit Department (or an external firm contracted by Internal Audit). Auditors must be independent of the process being audited. If a conflict exists (e.g., prior responsibility for the process), the auditor is replaced.

Qualification requirements: Auditors must have:

- Knowledge of the control system being audited (e.g., environmental auditor should have ISO 14001 internal auditor certification);

- Training in audit techniques (ISO 19011 or equivalent);

- Familiarity with Gautam Solar’s relevant SOPs and legal obligations.

4.2 Audit Execution

4.2.1 Opening Meeting

At the start of each audit, the audit team holds an opening meeting with the accountable person and relevant process owners. The meeting agenda includes:

- Confirmation of audit scope, criteria, and period;

- Audit schedule (which areas will be visited at what times);

- Availability of records and personnel;

- Confidentiality and reporting lines.

The opening meeting is recorded in the Audit Log (GS-AUD-F03).

4.2.2 Audit Evidence Collection

Auditors collect evidence through:

- Document review – policies, records, logs, registers, certificates, training files.

- Interviews – with process owners, operators, and managers.

- Observation – of operations (e.g., waste storage area, chemical dispensing, third-party onboarding system).

- Re-performance / testing – e.g., retesting a sample of third-party due diligence files to verify that the process was followed.

- Sampling – random or targeted selection of records (e.g., 20 of 200 hazardous waste manifests).

Sample sizes are risk-based:

- High-risk processes (e.g., anti-corruption for Tier 3 third parties) – sample size ≥ 80% of population or 30 items (whichever larger).

- Medium-risk processes – sample size 10–30 items or 30% of population.

- Low-risk processes – sample size 5–10 items or 10% of population.

All evidence is documented in audit work papers (GS-AUD-F04). Non-conformities are categorised as:

| Category | Definition | Example |
| --- | --- | --- |
| Critical | Complete breakdown of control; immediate risk of legal violation, serious environmental harm, or major financial loss. | No hazardous waste storage area segregation; no anti-corruption due diligence performed on a Tier 3 agent. |
| Major | Significant deficiency but not immediate crisis; likely to lead to non-compliance if not corrected. | Employee training records for ethics incomplete for 20% of staff; several wastewater samples exceeded pH limit. |
| Minor | Isolated lapse; low risk. | One missing signature on a due diligence form; weekly water meter reading missed once. |
| Opportunity for improvement | Control exists but could be more effective or efficient. | Suggest automating third-party risk tier assignment instead of manual spreadsheet. |

4.2.3 Retrospective Control – Detailed Methodology

The retrospective control assessment is performed by:

- Comparing current state with previous audit findings – e.g., “In the previous audit (January 2025), 5 findings were raised; 4 were closed; 1 was not effectively implemented – see below.”

- Analysing historical performance data – e.g., “Quarterly wastewater BOD results for the last 6 quarters show an increasing trend from 5 mg/L to 9 mg/L, approaching the limit of 10 mg/L. This indicates progressive degradation of the ETP performance, not captured by a point-in-time audit.”

- Reviewing incident / non-conformance recurrence – e.g., “Hazardous waste labels were missing on 3 occasions in the last year despite corrective action after the previous audit. Root cause: no regular inspection schedule after labelling.”

The retrospective assessment is documented in Appendix R of the audit report (or a dedicated section). It shall explicitly state whether the control system has improved, remained stable, or deteriorated over the audit period.

4.3 Audit Reporting

4.3.1 Draft Audit Report

Within 10 working days of audit completion, the audit team produces a Draft Audit Report (GS-AUD-F02) containing:

| Section | Content |
| --- | --- |
| Executive Summary | Overall assessment of control effectiveness (satisfactory / partially effective / ineffective). |
| Audit Scope & Objectives | As per audit plan, including audit period, sites, criteria. |
| Accountable Person(s) | Names and titles of persons responsible for each audited system. |
| Retrospective Control Assessment | Findings from historical analysis, trend data, recurrence of past issues. |
| Audit Findings | List of all non-conformities (critical, major, minor) and opportunities for improvement, with references to evidence. |
| Recommendations | Specific, actionable recommendations for each finding, including suggested completion dates. |
| Management Response | Space for accountable person to agree or disagree, propose corrective actions, and set completion dates. |
| Acknowledgement | Signature block for auditor and auditee. |

4.3.2 Closing Meeting & Management Response

The draft report is presented at a closing meeting with the accountable person and process owners. The auditee provides a management response within 5 working days, indicating:

- Agreement or disagreement (with justification for disagreement);

- Corrective action plan (what will be done, who, by when);

- For critical findings, immediate interim containment actions.

The final audit report is issued within 15 working days of audit completion.

4.3.3 Audit Committee Review

For critical findings and for the annual summary of all internal audits, the report is presented to the Audit Committee (comprising independent directors or senior management members not involved in daily operations of the audited systems). The Audit Committee:

- Reviews the adequacy of corrective action plans;

- Approves closure of critical findings;

- Recommends systemic improvements.

4.4 Corrective Action & Follow-Up

4.4.1 Tracking System

All audit findings are logged in the Audit Corrective Action Tracker (GS-AUD-F05), which records:

- Finding ID, category, description, root cause;

- Assigned accountable person;

- Corrective action plan (including due date);

- Verification evidence (documentation, re-audit date);

- Status (open / closed / overdue).

The tracker is reviewed monthly by the Internal Audit Head and quarterly by the Audit Committee.

4.4.2 Follow-Up Audits

- For critical findings: A follow-up audit (or verification visit) is conducted within 30 days of the planned closure date.

- For major findings: Follow-up is conducted within 60 days of the planned closure date.

- For minor findings: Closure is verified through document review (no on-site visit required) within 90 days.

If a finding is not closed by the due date, it is escalated to the CEO and Audit Committee, and the risk is entered into the corporate risk register.

4.4.3 Retrospective Effectiveness Check (After Closure)

Six months after a corrective action has been officially closed, the Internal Audit department performs a retrospective effectiveness check (a mini-audit limited to that finding) to ensure that:

- The control remains effective;

- No new similar issues have emerged;

- The root cause has genuinely been eliminated.

This check is documented as an addendum to the original audit report.

4.5 Annual Internal Audit Report (Management Review)

Annually (within 60 days of the financial year end), the Internal Audit Head prepares a Summary Internal Audit Report for the CEO and Audit Committee, covering:

- All audits conducted during the year (with dates, scopes, accountable persons);

- Summary of retrospective control assessments across all systems (trends, systemic weaknesses);

- Number and severity of findings by control system;

- Percentage of corrective actions closed on time;

- Planned audit programme for the next year (risk-based).

This report serves as the primary input to the management review of the ethics, social, environmental, sustainable procurement, and compliance management systems (aligned with ISO management review requirements).

5. Records & Forms

| Form ID | Name | Retention Period | Responsible |
| --- | --- | --- | --- |
| GS-AUD-F01 | Audit Plan (includes scope, accountable person, retrospective control description) | 5 years | Internal Audit Head |
| GS-AUD-F02 | Audit Report (draft and final) | 10 years | Internal Audit Head |
| GS-AUD-F03 | Audit Log (opening/closing meeting records) | 5 years | Internal Audit Head |
| GS-AUD-F04 | Audit Work Papers (evidence, checklists, sampling records) | 5 years | Internal Auditor |
| GS-AUD-F05 | Audit Corrective Action Tracker | 5 years after closure | Internal Audit Head |
| GS-AUD-F06 | Annual Summary Audit Report to Management | 10 years | Internal Audit Head |

All records stored in \\GautamSolar\Internal_Audit\Audit_Reports\ with restricted access.

6. References

| Reference | Relevance |
| --- | --- |
| Companies Act, 2013 – Sections 177 (Audit Committee), 179 (Board powers), 204 (Secretarial Audit) | Legal mandate for internal audit and Audit Committee oversight |
| ISO 19011:2018 – Guidelines for auditing management systems | Audit methodology, competence, and reporting |
| ISO 14001:2015 – Clause 9.2 (Internal audit) | Environmental management system audit requirements |
| ISO 45001:2018 – Clause 9.2 (Internal audit) | Occupational health and safety management system audit requirements |
| ISO 20400:2017 – Sustainable procurement – Guidance | Guidance for auditing sustainable procurement controls |
| Standard on Internal Audit (SIA) 1000 – ICIA | Indian internal audit standards |
| Gautam Solar SOPs: GS-COMP-001 (Third-Party Due Diligence), GS-EN-001/002 (Energy & Environment), GS-WM-001/002 (Waste & Water), GS-DC-001 (Document Control) | Audit criteria for specific control systems |

7. Appendices

Appendix A – Example Audit Plan (Ethics Control System) – Extract

| Item | Detail |
| --- | --- |
| Audit ID | ETH-2026-01 |
| System audited | Ethics control system (anti-corruption due diligence) |
| Audit period | 1 July 2025 – 30 June 2026 |
| Sites | Registered Office Delhi, Haridwar Factory, Bhiwani Factory |
| Key control objectives | 1. All new Tier 3 third parties have enhanced due diligence before contract. 2. Annual recertification for existing Tier 3 third parties completed and red flags resolved. 3. Whistleblower complaints logged and investigated within 30 days. |
| Audit criteria | GS-COMP-001, Section 4.1; Prevention of Corruption Act; company Code of Conduct. |
| Exclusions | Information security due diligence (covered under IT audit). |
| Accountable person | Compliance Officer – Mr. A. Sharma |
| Retrospective control | Review of due diligence files for all 25 Tier 3 third parties engaged in last 12 months (100% sample). Compare with previous audit findings to verify sustained correction. |

Appendix B – Retrospective Control Assessment Template

| Element | Description |
| --- | --- |
| Previous audit findings (date) | List of findings from last audit of this system (ID, category, status). |
| Verification of closure | Evidence that corrective actions were implemented (provide reference to work papers). |
| Recurrence check | Has the same or similar finding occurred again in the audit period? (Yes/No, with examples). |
| Trend analysis (if applicable) | Show key performance indicator (KPI) over last 3–6 quarters (e.g., % of third parties with due diligence, number of whistleblower cases). |
| Root cause persistence | If recurrence found, assess whether original root cause was correctly identified or new root cause emerged. |

Appendix C – Internal Audit Process Flowchart

Annual Risk Assessment → Audit Plan (Scope + Accountable Person + Retrospective Control)

↓

Audit Execution: Opening Meeting → Evidence Collection (incl. retrospective sampling) → Close Meeting

↓

Draft Audit Report (with findings & retrospective assessment)

↓

Management Response (Corrective Action Plan with due dates)

↓

Final Audit Report issued

↓

Audit Committee Review (for critical findings / annual summary)

↓

Corrective Action Tracking & Follow-up Audits

↓

Retrospective Effectiveness Check (6 months after closure)

↓

Annual Summary Report to Management

| Document ID | GS-REV-001 |
| --- | --- |
| Title | # Central Review, Amendment & Continuous Improvement Process for All SOPs |
| Version | 1.0 |

1. Purpose

To establish a single, standardised, and centralised process for reviewing, amending, approving, and continuously improving all Standard Operating Procedures (SOPs) within the Gautam Solar SOP Manual. This mechanism replaces any individual review sections that were previously embedded within individual SOPs and ensures:

- Uniform governance across all SOPs.

- Consistent application of document control (aligned with GS-DC-001).

- Clear assignment of review responsibilities.

- Retrospective effectiveness checks on a manual-wide basis.

- Traceable revision history and approval records.

2. Scope

This central review mechanism applies to every SOP in the Gautam Solar SOP Manual, regardless of department (Quality, Production, HSE, Product Development, Compliance, IT, etc.) or site (Delhi, Haridwar, Bhiwani). It covers:

- Scheduled periodic reviews.

- Triggered reviews (due to incidents, regulatory changes, process changes).

- Amendment procedures (minor and major revisions).

- Approval and distribution of revised SOPs.

- Retrospective effectiveness assessment of changes.

3. Responsibilities

| Role | Responsibility in the Central Review Mechanism |
| --- | --- |
| General Manager | Overall owner of the review mechanism. Ensures that reviews are conducted on time. Maintains the master review schedule. Approves all final changes. |
| Process Owner (Department Head) | Initiates review for SOPs within their department. Coordinates input from subject matter experts. Submits proposed changes to the Document Controller. |
| Document Controller | Assigns review tracking numbers, maintains the review log, distributes draft revisions, collects feedback, and archives obsolete versions. |
| Internal Audit | Provides independent assessment of review effectiveness during periodic audits (see GS-AUD-001). |
| All SOP Users (Employees) | May propose improvements at any time via the SOP Improvement Request form (GS-DC-F03). |

Note: The General Manager assumes the approval authority previously held by the QA Manager. For day-to-day coordination, the QA Manager (or a designated Quality lead) may support the process, but final sign-off rests with the General Manager.

4. Review Triggers & Frequency

4.1 Scheduled Periodic Reviews

Every SOP in the manual shall be reviewed according to a risk-based schedule maintained by the General Manager (with support from the Document Controller). The default review intervals are:

| SOP Category | Review Interval | Examples |
| --- | --- | --- |
| High-risk / frequently used | Annually | GS-QC-001 (IQC), GS-PR-001 (Production Startup/Shutdown), GS-WM-002 (Water & Heavy Metals) |
| Medium-risk | Every 2 years | GS-EN-001 (Energy Efficiency), GS-WM-001 (Waste Management) |
| Low-risk / stable | Every 3 years | GS-DC-001 (Document Control – except its own review must be done every 2 years to ensure governance remains current) |

The General Manager may adjust intervals based on audit findings or regulatory changes.

4.2 Triggered Reviews (Ad Hoc)

An off-schedule review shall be initiated within 30 days of any of the following events:

- A regulatory change affecting the SOP’s legal basis (e.g., update to CPCB effluent standards, E-Waste Rules amendment, new ISO standard).

- A major incident (environmental spill, corruption allegation, security breach, product recall) where the SOP failed to prevent or mitigate the incident.

- A significant process change (new equipment, new raw material, new IT system).

- A recurring non-conformance identified during internal or external audits (more than two similar findings in 12 months).

- A user-initiated improvement request that is validated by the Process Owner.

5. Review Process Steps

All reviews shall follow the same structured workflow, documented in the Central Review Log (GS-REV-LOG-01).

5.1 Initiation

- The Process Owner (or General Manager) creates a Review Initiation Record containing:

  - SOP number and title.

  - Reason for review (scheduled or triggered, with reference).

  - Proposed changes (if any) – or a statement “no changes required after assessment”.

  - Proposed reviewers (minimum 2, including at least one user of the SOP).

5.2 Assessment & Drafting

- The Process Owner, together with identified reviewers, examines the SOP against current operations, regulations, and best practices.

- If changes are needed, a draft revision is prepared using the standard SOP template.

- If no changes are needed, the “no change” decision is recorded, and the review cycle is closed (the next review date is recalculated from the current date).

5.3 Consultation & Feedback

- The draft revision is circulated electronically to all relevant department heads and affected employees (minimum 5 working days for comment).

- Comments are collated by the Document Controller. Substantial disagreements are resolved through a review meeting chaired by the General Manager.

5.4 Approval

- The final draft is submitted to the General Manager for approval.

- For changes that affect regulatory compliance (e.g., environmental discharge limits, hazardous substance thresholds) or that introduce new safety-critical steps, the approval must also include the HSE Manager and, if applicable, the Compliance Officer.

- Once approved, the new version number is assigned according to GS-DC-001 (major = +1.0, minor = +0.1).

5.5 Distribution & Training

- The Document Controller replaces the master copy in the ERP system and marks the previous version as “SUPERSEDED”.

- Printed copies (if any) are recalled and replaced.

- Training on the revised SOP is arranged by the Process Owner in coordination with HR, using the same training acknowledgement form (GS-DC-F06). Training must be completed before the new version becomes effective.

5.6 Retrospective Effectiveness Check (Centralised)

- For every major revision (version number increment by 1.0), a retrospective effectiveness check shall be conducted six months after the revision’s effective date.

- The check is performed by the Internal Audit department (or an independent reviewer not involved in the revision) and answers:

  - Have the intended improvements been achieved? (e.g., reduced non-conformances, lower waste, faster response times)

  - Have any unintended side effects appeared? (e.g., new errors, increased workload)

  - Is the revised SOP being followed as trained? (sample check of records and interviews)

- The results are documented in a Retrospective Effectiveness Report (GS-REV-RETRO-01) and presented to the General Manager. If the check reveals persistent issues, a new triggered review is initiated.

6. Central Review Log & Annual Summary

All review activities (scheduled and triggered) shall be recorded in the Central Review Log (GS-REV-LOG-01) with the following fields:

| Field | Description |
| --- | --- |
| Review ID | Sequential number (e.g., REV-2026-001) |
| SOP number | e.g., GS-EN-002 |
| Review trigger | Scheduled (interval) / Triggered (reason code) |
| Initiation date | Date the review started |
| Reviewers | Names of persons who assessed the SOP |
| Changes made? | Yes / No (if yes, summary and new version) |
| Approval date | Date General Manager signed off |
| Retrospective check due | Date (6 months after approval, if major revision) |
| Retrospective check completed? | Yes / No (with date and outcome) |

The General Manager shall produce an Annual Review Summary Report (by 31 January of each year) for the Audit Committee and CEO, highlighting:

- Number of SOPs reviewed on time vs. overdue.

- Number of major revisions and their outcomes.

- Recurring issues identified across multiple SOPs.

- Any overdue retrospective checks and their status.

7. Emergency Amendments

If an immediate correction to an SOP is required to address an imminent safety, environmental, or legal risk (e.g., a mis-statement that could cause a spill or non-compliance), the General Manager may authorise an emergency amendment without the full consultation period.

- The amendment must be documented using an Emergency Amendment Record (GS-REV-EMERG-01), stating the risk and the urgent change.

- The amendment becomes effective immediately but must be followed by a full review (including retrospective check as per Section 5.6) within 30 days.

- If the full review confirms the emergency change, it is retained; otherwise, it is reversed.

8. Relationship with Document Control (GS-DC-001)

This central review mechanism operates within the document control framework of GS-DC-001. All numbering, storage, approval signatures, and training acknowledgement shall follow GS-DC-001. The only change is that the review process is centralised, meaning:

- Individual SOPs no longer contain their own “Revision History” or “Review Procedure” sections (they may retain a simple “Revision History” table as an attachment, but the governance is here).

- The Revision History at the end of each SOP shall simply reference the Review ID from the Central Review Log and the date.

9. Records

| Form ID | Name | Retention | Responsible |
| --- | --- | --- | --- |
| GS-REV-LOG-01 | Central Review Log | 10 years | Document Controller |
| GS-REV-RETRO-01 | Retrospective Effectiveness Report | 5 years after closure | Internal Audit |
| GS-REV-EMERG-01 | Emergency Amendment Record | 10 years | General Manager |
| GS-DC-F03 | SOP Improvement Request Form (as before) | 3 years | Process Owner |

All records stored in \\GautamSolar\QA\Central_Review\.

10. References

- GS-DC-001 – Document Control & SOP Management (version 1.0 or later)

- GS-AUD-001 – Internal Audit & Monitoring (for independent verification)

- ISO 9001:2015 – Clause 7.5 (Documented information) and Clause 9.3 (Management review)

Approval Signatures (Central Review Mechanism)

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Author (General Manager) | Ankit Sangwan |  | 01-04-2026 |

This Central Review Mechanism is effective from the date of final approval. All SOPs in the manual shall be subject to this mechanism. Any printed copy of this mechanism is uncontrolled after 24 hours.