NAVITAS SOLAR PVT LTD

STANDARD OPERATING PROCEDURES MANUAL

Environment, Product Stewardship and Governance

| Document Control |  |
| --- | --- |
| Manual No. | NSPL/SOP/MNL/001 |
| Issue date | 01-01-2026 |
| Revision No. | 01 |
| Revision date | 01-04-2026 |
| Copy | Control Copy |
| Number of SOPs | 14 |

Contents

1. PURPOSE	4

2. SCOPE	4

3. OBJECTIVES	4

4. DEFINITIONS AND ABBREVIATIONS	5

5. RESPONSIBILITIES	6

6. PROCEDURE	7

7. REVIEW AND CORRECTIVE ACTION	21

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	21

9. RECORDS	22

1. PURPOSE	23

2. SCOPE	23

3. OBJECTIVES	23

4. DEFINITIONS AND ABBREVIATIONS	24

5. RESPONSIBILITIES	24

6. PROCEDURE	25

7. DEVIATION AND CORRECTIVE ACTION	29

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	29

9. RECORDS	30

1. PURPOSE	31

2. SCOPE	31

3. OBJECTIVES	31

4. DEFINITIONS AND ABBREVIATIONS	32

5. RESPONSIBILITIES	32

6. PROCEDURE	33

7. REVIEW AND CORRECTIVE ACTION	37

8. REFERENCE DOCUMENTS	37

9. RECORDS	37

1. PURPOSE	39

2. SCOPE	39

3. OBJECTIVES	39

4. DEFINITIONS AND ABBREVIATIONS	39

5. RESPONSIBILITIES	40

6. PROCEDURE	41

7. REVIEW AND CORRECTIVE ACTION	46

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	46

9. RECORDS	46

1. PURPOSE	48

2. SCOPE	48

3. OBJECTIVES	48

4. DEFINITIONS AND ABBREVIATIONS	48

5. RESPONSIBILITIES	49

6. PROCEDURE	50

7. MONITORING AND CORRECTIVE ACTION	53

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	54

9. RECORDS	54

1. PURPOSE	55

2. SCOPE	55

3. OBJECTIVES	55

4. DEFINITIONS AND ABBREVIATIONS	55

5. RESPONSIBILITIES	56

6. PROCEDURE	56

7. MONITORING AND CORRECTIVE ACTION	59

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	60

9. RECORDS	60

1. PURPOSE	61

2. SCOPE	61

3. OBJECTIVES	61

4. DEFINITIONS AND ABBREVIATIONS	61

5. RESPONSIBILITIES	62

6. PROCEDURE	62

7. REVIEW AND CORRECTIVE ACTION	66

8. REFERENCE DOCUMENTS	66

9. RECORDS	67

1. PURPOSE	68

2. SCOPE	68

3. OBJECTIVES	68

4. DEFINITIONS AND ABBREVIATIONS	68

5. RESPONSIBILITIES	69

6. PROCEDURE	69

7. REVIEW AND CORRECTIVE ACTION	72

8. REFERENCE DOCUMENTS	72

9. RECORDS	73

1. PURPOSE	74

2. SCOPE	74

3. OBJECTIVES	74

4. DEFINITIONS AND ABBREVIATIONS	74

5. RESPONSIBILITIES	75

6. PROCEDURE	75

7. REVIEW AND CORRECTIVE ACTION	77

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	78

9. RECORDS	78

1. PURPOSE	79

2. SCOPE	79

3. OBJECTIVES	79

4. DEFINITIONS AND ABBREVIATIONS	79

5. RESPONSIBILITIES	80

6. PROCEDURE	80

7. REVIEW AND CORRECTIVE ACTION	85

8. REFERENCE DOCUMENTS	85

9. RECORDS	85

1. PURPOSE	86

2. SCOPE	86

3. OBJECTIVES	86

4. DEFINITIONS AND ABBREVIATIONS	86

5. RESPONSIBILITIES	87

6. PROCEDURE	87

7. REVIEW AND CORRECTIVE ACTION	91

8. REFERENCE DOCUMENTS	91

9. RECORDS	91

1. PURPOSE	92

2. SCOPE	92

3. OBJECTIVES	92

4. DEFINITIONS AND ABBREVIATIONS	92

5. RESPONSIBILITIES	93

6. PROCEDURE	93

7. VIOLATIONS	95

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	96

9. RECORDS	96

1. PURPOSE	97

2. SCOPE	97

3. OBJECTIVES	97

4. DEFINITIONS AND ABBREVIATIONS	97

5. RESPONSIBILITIES	98

6. PROCEDURE	98

7. REVIEW	101

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	101

9. RECORDS	101

1. PURPOSE	102

2. SCOPE	102

3. OBJECTIVES	102

4. DEFINITIONS AND ABBREVIATIONS	102

5. RESPONSIBILITIES	103

6. PROCEDURE	103

7. REVIEW AND CORRECTIVE ACTION	107

8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS	107

9. RECORDS	107

INDEX OF STANDARD OPERATING PROCEDURES

| Sr. | SOP Title | Department | Areas Covered |
| --- | --- | --- | --- |
| 1 | Low-Carbon Energy and Emissions Stewardship | Utility | Carbon baseline; green power incl. 1,701 kWp rooftop solar; equipment modernisation; thermal energy capture; refrigerant stewardship; low-power digital systems; workforce engagement |
| 2 | Water Circularity and Effluent Quality Control | HSE | Treated water circulation (50 KLD MBR STP); effluent performance checks; toxic metal safeguards |
| 3 | Lean Material Use and Greener Process Inputs | HSE | Resource-lean production; sustainable consumables and plant-derived inputs; safer chemistry replacement |
| 4 | Hazardous Substance Control and Safe Handling | HSE | Chemical register; GHS marking; storage conditions; in-plant movement; protective measures; authorised disposal routes; competence building |
| 5 | Resource Recovery and Discard Flow Management | HSE | Circular use of surplus materials; colour-coded collection; origin-to-destination flow tracing; zero-waste awareness |
| 6 | Digital Asset Longevity and Retired Electronics Control | HSE | Restoration and upgrade of computing equipment; end-of-use electronics tracking and handover |
| 7 | Product Care, Longevity and Repair Support | Quality Assurance | Owner's guidance from storage to upkeep; higher output per module; fix-first support model |
| 8 | Circular Product Design and Restricted Substance Control | Quality Assurance | Controlled chemistry in components; identification marks for separation; separable construction |
| 9 | Module Return, Recycling and End-of-Life Guidance | Quality Assurance | Retirement guidance for module owners; free return pathway through EPR partners |
| 10 | Customer Safety Assurance and Field Action Management | Quality Assurance | Use-stage hazard evaluation; safe-use guidance and outreach; field incident escalation and market withdrawal |
| 11 | Business Partner ESG Screening and Monitoring | Purchase | Code acknowledgement; risk screening; self-assessments, internal assessments and on-site ESG audits; corrective action |
| 12 | Gifts, Hospitality and Discretionary Expense Approval | Admin | Principles; value thresholds and approval matrix; register; monitoring |
| 13 | Information Security Breach Handling | IT | Detection, containment, recovery, notification and learning cycle |
| 14 | Personal and Partner Data Stewardship | IT | Access restriction controls; holding periods and secure disposal; notice, permission and choice |

### 1. PURPOSE

To define a structured method for shifting the Company's operations towards cleaner energy and lower greenhouse gas (GHG) emissions by increasing renewable power, upgrading to energy-efficient technology, recovering waste energy, controlling fluorinated gas emissions, reducing IT energy use and building employee capability for a low-carbon workplace.

### 2. SCOPE

This SOP applies to all departments, employees and contract workers of Navitas Solar Pvt Ltd, and covers:

- Purchased grid electricity, electricity from the 1,701.00 kWp rooftop solar PV plant and any renewable electricity procured from external sources

- Production equipment and utilities, including stringers, laminators, framing lines, testers, compressors, pumps, motors, HVAC and lighting

- Equipment that produces recoverable heat, such as air compressors, laminators, chillers and DG sets (where installed)

- Equipment containing fluorinated gases, such as air conditioners, chillers, refrigerators, vehicle air conditioning, SF6-insulated switchgear and clean-agent fire suppression systems

- IT infrastructure, including the server room / network racks, UPS systems, computers, printers and other office electronics

- Procurement of new or replacement energy-using equipment and new energy projects

- Training and awareness of all employees on energy and climate topics

### 3. OBJECTIVES

The Company aims to:

- Raise the share of renewable electricity in total energy consumption year on year.

- Reduce specific energy consumption (kWh per MW of modules produced) through technology and equipment upgrades.

- Recover usable heat from processes and utilities wherever technically and economically viable.

- Prevent leakage of refrigerants and other fluorinated gases, and move to low-GWP alternatives.

- Reduce the energy consumed by IT infrastructure and office electronics.

- Build employee knowledge and ownership of energy saving and low-carbon practices.

- Measure, report and continually reduce Scope 1 and Scope 2 GHG emissions.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| RE | Renewable Energy – electricity from renewable sources such as solar, wind or hydro |
| SEC | Specific Energy Consumption – energy used per unit of output (kWh/MW of modules produced) |
| Open Access / GEOA | Supply of electricity from a generator of the consumer's choice through the grid; Green Energy Open Access applies to renewable sources |
| PPA | Power Purchase Agreement – long-term contract to buy electricity from a generator |
| REC / I-REC | Renewable Energy Certificate – a tradable certificate representing the renewable attribute of 1 MWh of electricity |
| WHR | Waste Heat Recovery – capture and reuse of heat that would otherwise be released to the atmosphere |
| CHP / Cogeneration | Combined Heat and Power – simultaneous generation of two or more useful forms of energy (e.g. electricity and heat/cooling) from a single fuel source |
| F-gases | Fluorinated greenhouse gases – HFCs, PFCs, SF6 and NF3, used as refrigerants, insulating gases and fire suppressants |
| GWP | Global Warming Potential – warming effect of a gas relative to the same mass of CO2 |
| PUE | Power Usage Effectiveness – total server room energy ÷ IT equipment energy (closer to 1.0 is better) |
| M&V | Measurement and Verification of energy savings |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Top Management / Plant Head | Approve energy and climate objectives, targets, renewable energy and efficiency investments; review performance |
| Utility / Maintenance | Operate and maintain energy systems, renewable generation and heat recovery units; maintain registers; carry out M&V; manage refrigerant servicing and leak checks |
| HSE Department | Coordinate this SOP; compile energy and GHG data; track KPIs; coordinate awareness campaigns |
| Production | Operate equipment as per optimal parameters; avoid idle running; implement agreed improvements |
| Purchase | Include energy efficiency, low-GWP refrigerants and renewable options in specifications and vendor evaluation |
| IT Department | Implement IT energy-saving measures; maintain server room conditions; manage IT asset disposal |
| Finance / Accounts | Support financial evaluation (cost, payback) of energy projects and renewable power options |
| HR | Plan and record training; include energy topics in induction |
| All Employees | Follow energy-saving practices, report wastage and leaks, and take part in training |

### 6. PROCEDURE

6.1 Energy and Carbon Baseline

Before planning improvements, the Utility and HSE teams shall establish an annual baseline covering:

- Total electricity consumed (kWh) – grid and renewable separately

- Fuel consumed (diesel for DG sets, LPG, vehicle fuel, as applicable)

- Specific Energy Consumption (kWh/MW of modules produced)

- Scope 1 emissions (fuels and F-gas leakage) and Scope 2 emissions (purchased electricity) in tCO2e

Grid electricity emissions shall be calculated using the latest grid emission factor published in the Central Electricity Authority (CEA) CO2 Baseline Database. Day-to-day energy monitoring shall continue as per NSPL/UTY/SOP/013 – Energy Efficiency and Energy Management.

The baseline shall give particular attention to the significant energy users recorded in the Master List of Equipment (NSPL/UTY/L/002) and the Master List of Machineries (NSPL/MNT/L/002):

| Equipment | Qty | Rated Power (each) | Source List |
| --- | --- | --- | --- |
| Laminators (Visual KSL27870AC-ED) | 4 | 620 kW | NSPL/MNT/L/002 |
| Auto stringers (ATW AM050FH) | 6 | 41 kW | NSPL/MNT/L/002 |
| Auto bussing machines | 2 | 50 kW | NSPL/MNT/L/002 |
| Auto JB soldering machines | 2 | 20 kW | NSPL/MNT/L/002 |
| Air compressors (Wen Bell) – 2 VFD and 5 fixed-speed | 7 | 75 kW | NSPL/UTY/L/002 |
| Air handling units / LG VRF outdoor units | 15 / 47 | Various | NSPL/UTY/L/002 |
| Air washers (JPC) | 3 | 11–13 kW | NSPL/UTY/L/002 |
| Cooling towers (160 TR) with circulation pumps | 2 | 3.2–3.7 kW fan; 15–18.3 kW pumps | NSPL/UTY/L/002 |
| UPS systems (Eaton 93PR) | 3 | 600 kVA | NSPL/UTY/L/002 |
| Transformer (Mahathi) | 1 | 4.4 MVA | NSPL/UTY/L/002 |
| Electric forklifts and BOPTs (Heli) | 5 + 4 | 15–16 kW (forklifts) | NSPL/UTY/L/002 |

6.2 Clean Power Sourcing and On-Site Generation

6.2.1 Existing On-Site Solar Generation

The Company generates renewable electricity at its manufacturing facility through a captive rooftop solar PV power plant:

| Parameter | Details |
| --- | --- |
| Installation | Rooftop Solar PV Power Plant |
| Installed capacity | 1,701.00 kWp |
| Location | Navitas Solar Private Limited, Ankleshwar, Sisodara, Gujarat – 394810 |
| EPC contractor | M/s Navitas Planet Private Limited (turnkey: design, supply, installation, testing and commissioning) |
| Commissioning date | 31 May 2026, as per the commissioning protocol |
| Evidence | Work completion certificate issued by the Company's authorised signatory |
| Metering arrangement (net / gross / captive) | [To be confirmed by the Company] |
| Expected annual generation (kWh) | [To be confirmed by the Company] |
| O&M responsibility | [To be confirmed by the Company] |

All electricity generated by this plant and consumed in operations shall be recorded as renewable energy as per Section 6.2.4.

6.2.2 Expansion Pathways for Green Power

To further increase the renewable share beyond the existing rooftop plant, the Utility department, with Finance, shall assess the following options at least once a year and whenever tariffs or regulations change significantly:

| Option | Description | Key Considerations |
| --- | --- | --- |
| On-site solar (rooftop / ground-mounted) | Captive solar plant installed on factory roofs, sheds or open land | Roof strength and area, shading, net/gross metering rules, O&M |
| Off-site captive / group captive | Share in an off-site renewable plant with power wheeled through the grid | Equity and consumption conditions, wheeling and banking charges |
| Green Energy Open Access / third-party PPA | Purchase of renewable power from a developer through open access | Eligibility, contract term, open access charges, approval timelines |
| Green tariff | Renewable power supplied by the distribution company at a green tariff, where offered | Tariff premium, availability, documentation for claims |
| Renewable Energy Certificates (REC / I-REC) | Certificates purchased to cover consumption not met by physical RE supply | Use only to supplement physical RE; certificates must be retired in the Company's name |
| Battery energy storage | Storage combined with solar to increase self-consumption | Cost, safety, space, battery end-of-life |

6.2.3 Evaluation and Approval

- Collect 12 months of load profile and electricity bills.

- Carry out a technical feasibility check (available area, structural stability, grid connectivity, shading).

- Prepare a techno-commercial comparison: cost per unit vs. grid tariff, payback, expected annual generation and tCO2e reduction.

- Confirm regulatory requirements (connectivity, metering arrangement, Electrical Inspector approval, open access approval as applicable).

- Submit the proposal to the Plant Head / Top Management for approval.

- On approval, execute the project or contract through the Purchase department and commission it with documented test results.

6.2.4 Metering and Renewable Energy Accounting

- Every on-site RE system shall have a dedicated generation meter; purchased RE shall be supported by invoices, energy accounts or certificates.

- Meter readings shall be recorded monthly in the Renewable Energy Register.

- RE share (%) = Renewable electricity consumed (kWh) ÷ Total electricity consumed (kWh) × 100

- Renewable attributes that are sold or transferred to another party (e.g. RECs sold) shall not be counted as the Company's renewable consumption.

| Month | Source | Capacity (kWp) | RE Units (kWh) | Total Units (kWh) | RE Share (%) | Evidence |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rooftop solar (on-site) | 1,701.00 |  |  |  | Generation meter reading |
|  | Open access / PPA |  |  |  |  | Invoice / energy account |
|  | REC / I-REC |  |  |  |  | Retirement certificate |

6.2.5 Operation and Maintenance of the Rooftop Solar Plant

- Retain the commissioning protocol, test reports, as-built drawings and warranty documents of the 1,701.00 kWp plant.

- Clean modules at a defined frequency ([To be confirmed by the Company]) using methods that do not damage the glass.

- Inspect modules, cables, junction boxes, inverters and earthing as per the preventive maintenance schedule.

- Compare actual generation with expected generation monthly; investigate any shortfall above the defined tolerance.

- Ensure safe access to the roof (fall protection, permit-to-work) for all cleaning and maintenance activities.

6.3 Modernisation of Plant and Utility Equipment

6.3.1 Sources of Modernisation Ideas

Opportunities for technology or equipment upgrades shall be identified from:

- Energy audit findings and machine-wise consumption data

- Equipment approaching end of life or showing high breakdown frequency

- Abnormal consumption trends identified in monthly monitoring

- Suggestions from employees, OEMs and energy service companies

- Capacity expansion and new line installations

6.3.2 Equipment Modernisation Options

| Area | Upgrade Option | Expected Benefit |
| --- | --- | --- |
| Motors and drives | Premium-efficiency (IE3/IE4) motors; Variable Frequency Drives (VFDs) on variable loads | Lower losses; power matched to actual load |
| Compressed air | Run the VFD compressors as trim units and fixed-speed units at full load; evaluate VFD retrofit or replacement of the 5 fixed-speed 75 kW compressors; pressure optimisation; leak-free piping; zero-loss drains | Less unloaded running and leakage loss |
| HVAC and cooling | Inverter / high-efficiency ACs and chillers; EC fans; automatic temperature controls | Lower cooling energy per unit of cooling |
| Lighting | LED fixtures with occupancy and daylight sensors; translucent roof sheets | Lower lighting load and running hours |
| Electrical distribution | Automatic power factor correction; harmonic filters; energy-efficient (star-rated) transformers | Reduced losses and demand charges |
| Production equipment | Laminator insulation and heater optimisation; standby / sleep modes on idle machines | Lower idle and heat-loss consumption |
| Monitoring | Sub-metering and an online energy monitoring system for major loads | Faster detection of abnormal consumption |

6.3.3 Evaluation and Approval

Each proposal shall be documented with: baseline consumption, expected annual saving (kWh and ₹), investment, simple payback, expected tCO2e reduction, impact on production and safety, and implementation timeline. Proposals shall be approved by the Plant Head / Top Management as per the delegated financial authority.

6.3.4 Measurement and Verification of Savings

- Measure consumption of the equipment or area for a representative period before the change.

- Implement the upgrade and allow it to stabilise.

- Measure consumption for an equivalent period under comparable production conditions.

- Calculate savings: Energy saved (kWh) = (Baseline SEC − New SEC) × Production in the period.

- Record the result in the Energy Efficiency Project Register and update the baseline.

| No. | Project / Equipment | Baseline (kWh/yr) | Verified Saving (kWh/yr) | Investment (₹) | Payback (yrs) | tCO2e Reduced / Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |

Energy-efficient procurement of new equipment shall follow Section 6 of NSPL/UTY/SOP/013. Where available, BEE star-labelled or equivalent high-efficiency equipment shall be preferred.

6.4 Thermal Energy Capture and Multi-Output Generation

6.4.1 Mapping of Surplus Heat Sources

| Heat Source | Form of Recoverable Heat | Possible Use |
| --- | --- | --- |
| Air compressors (7 × 75 kW Wen Bell) | Hot oil / hot discharge air | Hot water for canteen, washing or process pre-heating |
| Laminators (4 × 620 kW rated) | Hot exhaust air / heating system losses | Pre-heating, space or water heating |
| Chillers / large AC units | Condenser heat (desuperheater) | Hot water generation |
| DG sets (where installed) | Exhaust gas and jacket water heat | Hot water or absorption cooling |

6.4.2 Feasibility Assessment

For each source, the Utility department shall evaluate: quantity and temperature of available heat, matching heat demand nearby, distance between source and use, investment and payback, and safety implications. A Combined Heat and Power (CHP) or cogeneration / trigeneration system (for example, a gas engine with a vapour absorption chiller) shall be assessed where the facility has a simultaneous and steady demand for electricity and heat or cooling, and a suitable fuel supply is available.

The outcome of every assessment, including cases found not feasible, shall be documented with reasons and reviewed again when operating conditions change.

6.4.3 Installation and Commissioning

- Obtain engineering design approval and OEM confirmation that heat recovery will not affect equipment performance or warranty.

- Provide temperature and pressure safety controls, interlocks and insulation of hot surfaces.

- Carry out installation under the permit-to-work system and record commissioning test results.

6.4.4 Operation and Performance Monitoring

- Install heat (BTU) meters or temperature and flow measurement on the recovery circuit.

- Record recovered heat (kWh-thermal) monthly, along with fuel or electricity displaced.

- For CHP units: Overall efficiency (%) = (Electrical output + Useful thermal output) ÷ Fuel energy input × 100.

- Include WHR / CHP equipment in the preventive maintenance schedule.

Existing WHR / CHP installations at the facility: [To be confirmed by the Company].

6.5 Refrigerant Stewardship and Leak-Tight Cooling Systems

6.5.1 Register of Gas-Charged Equipment

The Utility department shall maintain an F-Gas Register covering all equipment containing refrigerants or other fluorinated gases, including split and ducted ACs, chillers, precision ACs, refrigerators and water coolers, vehicle ACs, SF6-insulated switchgear / ring main units and clean-agent fire suppression systems.

Refrigerant-containing equipment currently recorded in the Master List of Equipment (NSPL/UTY/L/002):

| Equipment | Qty | Refrigerant (as recorded) | Action |
| --- | --- | --- | --- |
| LG VRF outdoor units serving AHUs and the admin building | 47 | R-401A | Verify refrigerant from nameplates (VRF systems commonly use R-410A) and correct the list |
| Refrigerated compressed-air dryers (Wen Bell 40-HTF) | 4 | R-22 (HCFC, ozone-depleting) | Priority for leak checks and phased replacement |
| Precision AC, SF6 switchgear, fire suppression, water coolers | [TBC] | [TBC] | Add to register after site survey |

| Equip. ID | Location | Type | Gas | Charge (kg) | GWP | Last Leak Check | Top-up (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |

6.5.2 Leak-Tightness Checks and Repairs

- Carry out leak checks at a frequency based on gas charge and criticality (minimum frequency: [To be confirmed by the Company]).

- Use trained and competent technicians (in-house or service provider) and electronic leak detectors.

- Locate and repair leaks before recharging; repeated top-ups on the same unit require root-cause analysis.

- Never vent refrigerants or SF6 to the atmosphere; monitor SF6 gas density / pressure gauges on switchgear.

- Record every top-up quantity in the F-Gas Register.

6.5.3 Shift to Lower-Impact Refrigerants

- New or replacement cooling equipment shall preferably use lower-GWP refrigerants (for example R-32, R-290 or HFO-based blends), subject to safety and technical suitability.

- Avoid purchasing new equipment using HCFC R-22 or high-GWP HFCs (such as R-404A) where suitable alternatives exist, in line with India's commitments under the Montreal Protocol and its Kigali Amendment.

- Plan phased replacement of old, leak-prone or high-GWP equipment as part of the capital plan, starting with the R-22 refrigerated air dryers.

6.5.4 Servicing and Decommissioning

- Recover refrigerant during servicing using a recovery machine and approved cylinders.

- At decommissioning, recover all gas before dismantling and hand it over to an authorised reclaimer / recycler; obtain a recovery or disposal certificate.

- Update the F-Gas Register when equipment is added, serviced or removed.

6.5.5 Quantifying Refrigerant Losses in tCO2e

F-gas emissions (tCO2e) = Quantity topped up or lost (kg) × GWP ÷ 1,000

| Gas | Typical Use | GWP (IPCC AR5, 100-yr) |
| --- | --- | --- |
| R-22 (HCFC) | Older ACs, refrigerated air dryers | 1,760 |
| R-32 | Split / inverter ACs | 677 |
| R-410A | Split / ducted ACs | 1,924 |
| R-134a | Chillers, vehicle ACs | 1,300 |
| R-407C | Older ACs / chillers | 1,624 |
| HFC-227ea | Clean-agent fire suppression | 3,350 |
| SF6 | HT switchgear / ring main units | 23,500 |

Note: GWP values shall be aligned with the methodology adopted for the Company's GHG inventory.

6.6 Low-Power Digital Systems and Server Room Practices

6.6.1 Server Room and Network Racks

- Maintain server room temperature within the ASHRAE recommended range of 18–27 °C instead of overcooling.

- Arrange racks in hot aisle / cold aisle layout where possible; use blanking panels and seal cable openings.

- Maintain precision ACs and keep server room doors closed.

- Meter server room energy separately where practical and track PUE.

6.6.2 Hardware and Infrastructure

- Consolidate and virtualise servers; switch off and decommission idle or unused servers.

- Use right-sized, high-efficiency UPS systems and enable energy-saving modes where safe.

- Prefer ENERGY STAR or BEE star-rated computers, monitors and printers when purchasing.

- Consider cloud or hosted services powered by renewable energy when replacing on-premise servers.

6.6.3 End-User Practices

- Apply a central power management setting: display off and system sleep after a defined idle time.

- Shut down computers, monitors and printers at the end of the working day unless required to run.

- Use shared network printers with duplex printing as default.

6.6.4 Disposal of IT Equipment

Obsolete IT equipment shall be disposed of only through authorised e-waste recyclers as per the E-Waste (Management) Rules, 2022, after secure data wiping.

| IT Energy Check | Frequency | Responsibility |
| --- | --- | --- |
| Server room temperature and AC performance | Daily | IT Department |
| Idle / unused server review | Quarterly | IT Department |
| Power management settings on PCs | Half-yearly | IT Department |
| Server room energy and PUE review | Monthly | IT / Utility |

6.7 Workforce Engagement for a Low-Carbon Workplace

6.7.1 Identifying Learning Needs

HR, with HSE and Utility, shall identify training needs annually based on job roles, new technologies, audit findings and energy performance results, and include them in the annual training calendar.

6.7.2 Role-Wise Learning Plan

| Target Group | Key Topics | Method | Frequency |
| --- | --- | --- | --- |
| All new joiners | Company energy and climate commitments, energy-saving practices, reporting wastage | Induction | At joining |
| Production operators | Optimal machine settings, avoiding idle running, reporting abnormal consumption | Toolbox talk / on-the-job | Quarterly |
| Utility & maintenance staff | Compressed air, motors, HVAC efficiency, RE system O&M, M&V | Classroom / OEM training | Annually |
| HVAC / refrigeration technicians | Leak detection, refrigerant recovery, safe handling of F-gases and SF6 | Technical training | Annually |
| IT staff | Server room efficiency, power management, e-waste handling | Classroom / online | Annually |
| Purchase team | Energy-efficient and low-GWP procurement, life-cycle cost | Classroom | Annually |
| Managers and heads of department | Climate risks, GHG emissions (Scope 1 & 2), targets and investment decisions | Workshop | Annually |

6.7.3 Campaigns and Engagement

- Observe National Energy Conservation Day (14 December), World Environment Day (5 June) and other campaigns.

- Display posters, energy performance boards and monthly RE / energy KPIs at the shop floor.

- Run an energy-saving suggestion scheme and recognise implemented ideas.

- Nominate department energy champions to support awareness and monitoring.

6.7.4 Evaluating Learning Impact

Effectiveness shall be checked through post-training assessments, shop-floor observations, number of energy suggestions received and reduction in reported wastage. Attendance and evaluation records shall be maintained by HR.

### 7. REVIEW AND CORRECTIVE ACTION

- Utility and HSE shall review energy, renewable, F-gas and IT energy data monthly; Top Management shall review performance, projects and targets at least quarterly.

- Any adverse trend, missed target, repeated F-gas top-up or under-performance of RE / WHR systems shall be treated as a deviation.

- Deviations shall be handled through the corrective action steps defined in Section 12 of NSPL/UTY/SOP/013 (identify, verify, root cause, action, responsibility, verification).

- This SOP shall be reviewed annually or when there is a significant change in operations, technology or regulations.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- NSPL/UTY/SOP/013 – SOP: Energy Efficiency and Energy Management

- Energy Conservation Act, 2001

- Electricity (Promoting Renewable Energy Through Green Energy Open Access) Rules, 2022

- Ozone Depleting Substances (Regulation and Control) Rules, 2000

- E-Waste (Management) Rules, 2022

- CEA CO2 Baseline Database for the Indian Power Sector (latest version)

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Energy & Carbon Baseline | Utility / HSE | As applicable |
| Renewable Energy Register and supporting invoices / certificates | Utility | As applicable |
| Rooftop solar plant work completion certificate, commissioning protocol and O&M records | Utility | As applicable |
| Energy Efficiency Project Register (incl. M&V) | Utility | As applicable |
| WHR / CHP feasibility studies and monitoring records | Utility | As applicable |
| F-Gas Register, leak check and recovery certificates | Utility | As applicable |
| IT energy checklist and e-waste disposal records | IT Department | As applicable |
| Training calendar, attendance and effectiveness records | HR | As applicable |
| Performance review and corrective action records | HSE / Concerned Department | As applicable |

### 1. PURPOSE

To define how the Company reclaims and reuses water within its operations, monitors the quality of its treated wastewater and prevents heavy metals from reaching water bodies, so that freshwater demand and the risk of water pollution are kept to a minimum.

### 2. SCOPE

This SOP applies to all water sources, water-using activities and wastewater streams of the manufacturing facility, including:

- Domestic sewage from toilets, canteen and washrooms treated in the Sewage Treatment Plant (STP)

- Rainwater collected on the premises

- Utility water from cooling, air conditioning condensate, RO / water treatment rejects and equipment cleaning

- Any wash water or residue from production, rework, laboratory or maintenance activities that may contain chemicals or metals

### 3. OBJECTIVES

The Company aims to:

- Reduce freshwater withdrawal by reusing treated and reclaimed water wherever its quality allows.

- Ensure treated wastewater consistently meets applicable discharge / reuse norms.

- Prevent heavy metals such as lead, mercury, cadmium, arsenic and chromium from entering wastewater.

- Maintain reliable records of water reuse and wastewater quality.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| STP | Sewage Treatment Plant – treats domestic wastewater generated on the premises |
| MBR | Membrane Bio-Reactor – biological treatment combined with membrane filtration, producing high-quality treated water |
| KLD | Kilolitres per day |
| Reclaimed water | Water recovered from internal or external sources and treated to a quality suitable for reuse |
| BOD / COD | Biochemical / Chemical Oxygen Demand – indicators of organic load in water |
| TSS / TDS | Total Suspended Solids / Total Dissolved Solids |
| Heavy metals | Toxic metals of concern in wastewater, including lead (Pb), mercury (Hg), cadmium (Cd), arsenic (As) and chromium (Cr) |
| NABL | National Accreditation Board for Testing and Calibration Laboratories |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Plant Head | Approve water reuse projects and resources; review water and wastewater performance |
| HSE Department | Coordinate this SOP; arrange wastewater testing; review results; maintain records; report deviations |
| Utility / Maintenance | Operate and maintain the STP and water reuse systems; record flows; carry out daily checks |
| Production / Maintenance / QC | Prevent chemicals and metal-bearing residues from entering drains; follow segregation instructions |
| Housekeeping | Use reclaimed water for permitted applications; report leaks and overflows |
| All Employees | Use water responsibly and report leaks, overflows or abnormal discharges |

### 6. PROCEDURE

6.1 Treated Water Recovery and Circulation

6.1.1 Sewage Treatment and Reuse

The facility operates a 50 KLD MBR-based Sewage Treatment Plant, designed, supplied, erected and commissioned by M/s Aldee Water Pvt. Ltd. (completion and handover certificate against PO No. 2425-00343 dated 21.12.2024). The MBR technology produces treated water of a quality that allows non-potable reuse.

- Collect domestic sewage from all toilets, washrooms and the canteen and route it to the STP.

- Operate the STP as per the supplier's O&M instructions (aeration, membrane cleaning, sludge removal, disinfection).

- Store treated water in a dedicated treated-water tank, clearly marked "Treated Water – Not for Drinking".

- Reuse treated water only for approved non-potable applications (listed in 6.1.3) after confirming that quality meets the reuse criteria.

- Record daily inlet / outlet flow and the quantity of treated water reused.

6.1.2 Additional Recoverable Water Sources

| Source | Reclamation Method | Possible Reuse |
| --- | --- | --- |
| Rainwater | Collection in the on-site catchment pond; filtration before use | Gardening, housekeeping, groundwater recharge |
| AC / AHU condensate | Collection through drain lines into a holding tank | Cooling tower / utility make-up, gardening |
| RO / softener reject | Separate collection and blending where quality allows | Floor washing, toilet flushing |
| Equipment rinse water (non-chemical) | Settling and filtration | Rinse reuse, floor cleaning |

6.1.3 Permitted Uses and Safeguards

- Approved reuse applications: [To be confirmed by the Company] (for example landscaping, toilet flushing, floor washing, utility make-up).

- Reclaimed water lines shall be separate from drinking water lines and identified by colour coding or labels.

- Reclaimed water shall never be used for drinking, cooking or personal hygiene.

- Any new reuse application shall be approved by the HSE Department and Plant Head after confirming water quality.

| Date / Month | STP Inlet (KL) | Treated Water (KL) | Reused (KL) | Reuse Application | Sign |
| --- | --- | --- | --- | --- | --- |

6.2 Effluent Performance Checks

The Company regularly monitors the quality of its treated wastewater through a NABL-accredited / MoEF&CC-recognised environmental laboratory, in addition to daily operational checks of the STP.

- Treated wastewater (STP outlet) shall be sampled and tested at a defined frequency ([To be confirmed by the Company]).

- Results shall be compared with the applicable limits and reviewed by the HSE Department.

- Test reports shall be retained as records and made available during audits and inspections.

Most recent STP outlet test (ENPRO Enviro Tech and Engineers Pvt. Ltd., Test Report No. NABLT0626GJ181572600000142, issued 11-07-2026):

| Parameter | Unit | Result | Limit |
| --- | --- | --- | --- |
| pH | – | 7.26 | 6.5 – 9.0 |
| Total Suspended Solids | mg/L | 26.0 | < 30 |
| Total Dissolved Solids | mg/L | 228 | – |
| Chemical Oxygen Demand | mg/L | 59.5 | – |
| Biochemical Oxygen Demand (3 days, 27 °C) | mg/L | 15.7 | < 20 |
| Chlorine Residual | mg/L | 0.64 | Minimum 0.5 |

6.3 Toxic Metal Safeguards for Discharge Streams

6.3.1 Possible Points of Metal Release

The HSE Department shall identify activities that could release heavy metals into wastewater, such as:

- Cleaning of soldering equipment, ribbons or tools carrying solder / flux residues

- Washing of floors or equipment in rework, soldering and cell handling areas

- Laboratory, testing or maintenance activities using metal-bearing chemicals

- Leachate from scrap, battery or hazardous waste storage areas exposed to rain

6.3.2 Prevention at Source

- Metal-bearing residues shall be removed by dry cleaning (vacuum, wipes) and managed as hazardous waste, not washed into drains.

- Process or chemical wash water shall not be mixed with domestic sewage or sent to the STP.

- Scrap, battery and hazardous waste storage areas shall be covered, with impervious flooring and spill containment, so that rainwater does not carry metals into drains.

6.3.3 Metal Capture Techniques

Where a wastewater stream is found to contain heavy metals, it shall be collected separately and treated using a suitable technology before reuse or disposal, or handed over to an authorised treatment facility:

| Technology | How it Works | Typical Application |
| --- | --- | --- |
| Chemical precipitation | pH adjustment and addition of precipitating / coagulating chemicals convert dissolved metals into solid particles that are settled and removed as sludge | Streams with higher metal concentrations |
| Ion exchange | Resin beads exchange harmless ions for dissolved metal ions; resins are regenerated or replaced | Polishing of low-concentration streams |
| Adsorption | Activated carbon or specialised adsorbent media capture metal ions on their surface | Final polishing; removal of trace metals |

Sludge, spent resins and spent adsorbent media from metal treatment shall be managed as hazardous waste through authorised facilities.

6.3.4 Verification

- Heavy metal parameters (Pb, Hg, Cd, As, Cr) shall be included in wastewater testing at a defined frequency ([To be confirmed by the Company]).

- Any detection above applicable limits shall be treated as a deviation and handled as per Section 7.

### 7. DEVIATION AND CORRECTIVE ACTION

- Stop or divert the affected discharge / reuse where results exceed limits or abnormal conditions are observed.

- Inform the HSE Head and Plant Head.

- Identify the root cause (process upset, chemical ingress, equipment failure).

- Implement corrective action and re-test to confirm effectiveness.

- Record the deviation, action and re-test result.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- Water (Prevention and Control of Pollution) Act, 1974 and consent conditions issued by GPCB

- Environment (Protection) Act, 1986 and applicable discharge standards

- STP completion and handover certificate – M/s Aldee Water Pvt. Ltd.

- NSPL/HSE/SOP/004 – Waste Management

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| STP daily operation log (flows, checks) | Utility / Maintenance | As applicable |
| Treated water reuse register | Utility / HSE | As applicable |
| Wastewater test reports (incl. heavy metals) | HSE Department | As applicable |
| Deviation and corrective action records | HSE Department | As applicable |

### 1. PURPOSE

To define how the Company reduces the quantity of materials consumed in manufacturing, prefers sustainable, plant-derived consumables and moves to safer chemistry, so that resource use, cost and environmental impact are reduced without affecting product quality.

### 2. SCOPE

This SOP applies to:

- Production materials used in module manufacturing, such as glass, cells, ribbons / busbars, EVA / encapsulant, backsheet, aluminium frames, junction boxes, sealants and potting

- Process and operational materials that do not form part of the finished product, such as cleaning agents, solvents, lubricants, greases, consumables and internal logistics materials (pallets, crates, wraps)

- Selection of new materials and changes to existing materials

- Rework, repair and reuse of parts and equipment

Hazardous residues are covered under the Hazardous Substance Control and Safe Handling SOP and the Resource Recovery and Discard Flow Management SOP.

### 3. OBJECTIVES

The Company aims to:

- Reduce material consumption per MW of modules produced.

- Reduce process scrap and rejections through process optimisation.

- Reuse, rework or recondition parts wherever quality permits.

- Increase the use of sustainable, low-toxicity and plant-derived consumables.

- Progressively substitute hazardous substances with less hazardous alternatives.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Process material | Material consumed in production or operations that does not remain in the finished product or its primary packaging (e.g. cleaners, lubricants, wipes, pallets) |
| Bio-based material | Material wholly or partly derived from renewable biological feedstock (e.g. plant-based lubricants) |
| Low-VOC | Product that releases low levels of volatile organic compounds |
| Yield | Proportion of input material converted into good output |
| Rework | Correcting a defective unit so that it meets specification |
| SDS | Safety Data Sheet of a chemical substance or mixture |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Plant Head | Approve material optimisation and substitution projects; review material performance |
| Production | Monitor material consumption and scrap; implement process optimisation; follow rework procedures |
| Quality Assurance | Approve rework / reuse and alternative materials from a quality standpoint; validate trials |
| Process / Maintenance Engineering | Optimise machine settings, cutting and dispensing; recondition spare parts |
| Purchase | Source sustainable, plant-derived and safer materials; obtain SDS and technical data from suppliers |
| HSE Department | Review SDS and hazards of new materials; coordinate substitution assessment; maintain the chemical register |
| Stores | Issue materials on FIFO basis; control shelf life; record consumption |

### 6. PROCEDURE

6.1 Resource-Lean Production Practices

6.1.1 Monitoring Consumption and Scrap

- Record material issue and consumption by line and shift through the store / ERP records.

- Record scrap daily by process, type, quantity and reason (as per the Company's Scrap Policy).

- Review monthly material consumption per MW and scrap trends jointly with Production and Quality.

6.1.2 Shop-Floor Saving Techniques

| Material / Process | Optimisation Practice | Benefit |
| --- | --- | --- |
| EVA / encapsulant and backsheet | Optimise roll width and cutting length to module size; reduce trimming allowance; nest cutting patterns | Less trimming waste |
| Cells | Control stringer parameters; careful handling and automated transfer to avoid micro-cracks | Fewer broken / cracked cells |
| Ribbon / busbar | Optimise ribbon length and solder settings; avoid over-feeding | Lower copper and solder usage |
| Glass | Correct handling equipment, racks and suction lifters; controlled stacking | Lower breakage |
| Sealant and potting | Calibrated dispensing equipment; correct bead size; avoid purging losses | Lower sealant consumption |
| Aluminium frames | Optimise cutting plans and order frames to exact length where possible | Fewer off-cuts |
| Product design | Adopt larger-format and higher-efficiency cell technologies that deliver more power per unit of material | Less material per watt |

6.1.3 Life Extension of Components and Spares

- Modules with repairable defects shall be reworked instead of scrapped where technically feasible and approved by QA (e.g. junction box replacement as per NSPL/QAD/CC/SOP/002 – Junction Box Rework At Site).

- Usable materials recovered from rework (frames, glass, junction boxes) shall be reused when they meet specification.

- Machine spare parts (motors, pumps, gearboxes, heaters, PCBs) shall be repaired or reconditioned before a new part is purchased, where safe and economical.

- Reusable packaging and logistics items (e.g. glass pallets redesigned for reuse in module packaging, returnable crates) shall be used and tracked.

6.1.4 Saving Initiatives Register

Material reduction ideas shall be recorded, evaluated for quality, cost and environmental benefit, trialled under QA supervision and, if successful, standardised in work instructions.

| No. | Initiative | Material | Baseline Use | Use After | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |

6.2 Sustainable Consumables and Plant-Derived Inputs

When purchasing or replacing process materials, the Purchase and HSE Departments shall give preference to sustainable and plant-derived options that perform adequately:

| Category | Conventional Material | Preferred Alternative |
| --- | --- | --- |
| Lubricants and greases | Petroleum-based oils and greases | Plant-derived / bio-based biodegradable lubricants, where compatible with equipment |
| Cleaners and degreasers | Solvent-based or high-VOC cleaners | Water-based, low-toxicity, low-VOC cleaners |
| Wipes and cleaning cloth | Disposable synthetic wipes | Reusable or cotton cloths; biodegradable wipes |
| Internal logistics | Single-use plastic wraps, foam and non-returnable pallets | Returnable crates and pallets; recycled-content or biodegradable wraps |
| Housekeeping chemicals | Conventional detergents | Biodegradable / eco-labelled detergents |

- Request SDS and environmental information (biodegradability, VOC content, eco-labels) from suppliers before approval.

- Trial alternatives on a small scale, with QA and Maintenance confirming performance and equipment compatibility.

- Maintain a list of approved sustainable consumables and update it when changes are approved.

6.3 Safer Chemistry Replacement Programme

6.3.1 Replacement Steps

- Identify hazardous substances in use from the chemical register and SDS (e.g. substances that are toxic, carcinogenic, flammable or contain heavy metals or halogens).

- Search for alternatives with lower hazard, in discussion with suppliers.

- Compare alternatives on hazard, performance, product quality and certification impact, and cost.

- Conduct trials and obtain QA and HSE approval.

- Update specifications, approved material list, SDS file and work instructions.

- Train affected employees before introducing the new material.

6.3.2 Priority Replacement Areas

| Area | Hazardous Substance / Concern | Less Hazardous Option to Evaluate |
| --- | --- | --- |
| Soldering | Cell and string ribbons are tinned copper with Sn60Pb40 (tin-lead) coating, as listed in the IEC 61730 bill of materials | Lead-free ribbon coatings and solder, subject to qualification and re-certification |
| Cleaning | Isopropyl alcohol (IPA) and solvent cleaners | Reduced quantities; water-based or low-VOC cleaners where suitable |
| Electrical components | Brominated / chlorinated flame retardants, PVC | Halogen-free cables, connectors and junction box materials |
| Sealants and adhesives | High-VOC formulations | Low-VOC or solvent-free silicone sealants and potting |
| Maintenance | Chlorinated degreasers, mercury-containing lamps and devices | Non-chlorinated degreasers; LED lighting and mercury-free instruments |

Current status of substitution for each area: [To be confirmed by the Company].

### 7. REVIEW AND CORRECTIVE ACTION

- Production, Quality and HSE shall review material consumption, scrap and substitution progress monthly.

- Abnormal increase in consumption or scrap shall be investigated for root cause and corrected.

- Any quality issue arising from an alternative material shall be escalated to QA before further use.

### 8. REFERENCE DOCUMENTS

- Scrap Policy of the Company

- NSPL/QAD/CC/SOP/002 – Junction Box Rework At Site

- Hazardous Substance Control and Safe Handling SOP (this Manual)

- Safety Data Sheets of materials in use

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Material consumption and scrap records | Production / Stores | As applicable |
| Material optimisation initiative register | Production | As applicable |
| Approved sustainable consumables list | Purchase / HSE | As applicable |
| Substitution assessments and trial reports | HSE / QA | As applicable |

### 1. PURPOSE

To define the controls for labelling, storing, handling, transporting, treating and disposing of hazardous substances and hazardous waste, and for training employees, so that exposure of people and releases to the environment are prevented.

### 2. SCOPE

This SOP applies to all hazardous substances used as process materials at the facility (for example IPA, solvents, cleaning chemicals, flux, sealants and potting compounds, adhesives, oils, lubricants, gases and maintenance chemicals), and to the treatment and disposal of hazardous substances and hazardous waste arising from them. It covers all employees, contract workers, transporters and service providers handling these materials.

### 3. OBJECTIVES

The Company aims to:

- Ensure every hazardous substance is identified, labelled and accompanied by its Safety Data Sheet (SDS).

- Store, handle and transport hazardous substances without leaks, spills or exposure.

- Treat and dispose of hazardous substances and hazardous waste only through safe and authorised routes.

- Ensure all relevant employees are trained and competent.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Hazardous substance | Chemical or material with properties such as toxicity, flammability, corrosiveness or reactivity that can harm people or the environment |
| GHS | Globally Harmonized System of Classification and Labelling of Chemicals |
| SDS / MSDS | (Material) Safety Data Sheet – document giving hazards, safe handling, storage and emergency information |
| Secondary containment | Tray, bund or pallet that holds leaks or spills from the primary container |
| TSDF | Treatment, Storage and Disposal Facility authorised for hazardous waste |
| Manifest | Document that tracks hazardous waste from the generator to the receiving facility |
| PPE | Personal Protective Equipment |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Plant Head | Ensure resources and infrastructure for safe chemical management; approve disposal arrangements |
| HSE Department | Maintain chemical register and SDS file; inspect storage areas; arrange training; manage hazardous waste disposal and manifests |
| Stores | Receive, label check, store and issue hazardous substances; maintain stock and storage records |
| Production / Maintenance / QC | Use only labelled and approved chemicals; follow SDS and work instructions; report spills |
| Purchase | Obtain current SDS with every new chemical; ensure suppliers' packaging and labelling meet requirements |
| Contractors / Transporters | Follow site rules, use proper vehicles and containers, carry emergency information |
| All Employees | Wear PPE, follow instructions and report unsafe conditions |

### 6. PROCEDURE

6.1 Chemical Register and Safety Data

- HSE shall maintain a Hazardous Substance Register listing each substance, its location, maximum quantity, hazard class and SDS reference.

- A current SDS (in a language understood by users) shall be available at the point of storage and use.

- No new hazardous substance shall be introduced without HSE review of its SDS.

- Suppliers of chemicals shall provide the MSDS with the supply and TREM cards for hazardous chemicals in transit, as required under the EHS clause (clause 22) of the Company's purchase order terms.

6.2 Container Identification and GHS Marking

- Every container, including small dispensing bottles and secondary containers, shall carry a clear, legible label as per GHS showing: product name, hazard pictogram(s), signal word, hazard and precautionary statements, and supplier details.

- Labels shall be resistant to the substance and to handling; damaged labels shall be replaced immediately.

- Unlabelled containers shall not be used and shall be reported to HSE.

- Pipelines and tanks containing hazardous substances shall be identified with content and flow direction.

6.3 Chemical Storage Conditions

- Store hazardous substances only in designated, ventilated, covered areas with impervious flooring, away from sunlight, heat and ignition sources, as per SDS.

- Segregate incompatible substances (e.g. flammables away from oxidisers, acids away from bases).

- Use sealed, original or approved containers; keep lids closed when not in use.

- Provide secondary containment (spill pallets / bunds) of adequate capacity.

- Flammable storage shall have flameproof electrical fittings, earthing and fire extinguishers.

- Keep only the minimum quantity required for the shift at the point of use.

- Display storage area signage, SDS and emergency contact details.

6.4 In-Plant Movement and Dispensing

- Move containers in closed condition using trolleys, drum lifters, forklifts or other mechanical aids to avoid manual lifting and dropping.

- Use spill-proof carriers and secure containers against tipping during movement.

- Transfer and dispense only in designated areas using pumps, funnels or dispensing systems with drip trays.

- Protective barriers and restricted access shall be provided around chemical storage and dispensing areas.

6.4.1 Off-Site Consignment Requirements

- Hazardous substances shall be transported only in suitable, labelled, leak-proof packaging by vehicles meeting applicable rules.

- Drivers shall carry emergency information (TREM card / SDS) and spill kit as applicable.

- Loading and unloading shall be supervised and carried out with the vehicle stationary and secured.

6.5 Protective Measures and Visual Warnings

| Control | Requirement |
| --- | --- |
| Engineering controls | Local exhaust ventilation at soldering, cleaning and dispensing points; enclosed dispensing where practical |
| Mechanical aids | Drum lifters, trolleys, pumps and lifts to avoid manual handling of heavy containers |
| Protective barriers | Bunds, guards and restricted-access barriers around storage and dispensing areas |
| PPE | Chemical-resistant gloves, goggles / face shield, apron and respirator as specified in the SDS |
| Signage and instructions | Hazard signs, PPE signs, no-smoking signs and pictorial work instructions displayed at storage and use points |
| Emergency equipment | Eyewash / safety shower, spill kits and fire extinguishers near storage and use areas |

6.6 Spill and Emergency Response

- Raise the alarm and isolate the area; remove ignition sources.

- Wear appropriate PPE and contain the spill with absorbents from the spill kit.

- Prevent the spill from entering drains, soil or water.

- Collect contaminated absorbent as hazardous waste.

- Report the incident to HSE; investigate the cause and record corrective action.

6.7 Neutralisation, Containment and Authorised Disposal Routes

6.7.1 Pre-Dispatch Conditioning

- Where safe, residues shall be neutralised or stabilised as per SDS before storage for disposal (e.g. neutralising acid / alkali residues, absorbing liquid residues).

- Empty chemical containers shall be drained completely and kept closed; they shall be decontaminated only by a GPCB-licensed decontamination vendor.

- Used / spent oil shall be collected in closed containers and reused for suitable alternative applications where possible before disposal.

6.7.2 Holding and Dispatch of Hazardous Residues

- Store hazardous waste in a designated, covered, labelled area with leak-proof containers and secondary containment.

- Storage of hazardous waste shall not exceed 90 days.

- Dispose of hazardous waste only through GPCB-authorised vendors, recyclers or TSDFs, according to waste type:

| Hazardous Waste | Disposal Route |
| --- | --- |
| Empty chemical barrels / containers | Vendor licensed by GPCB for decontamination of HDPE / MS drums and carboys |
| Used / spent oil | Certified used / spent oil recycler |
| Chemical-contaminated wipes, rags, PPE, filters | Authorised hazardous waste handler / incineration |
| Sealant and potting material waste, chemical sludge | Authorised hazardous waste handler |
| IPA and solvent waste | Authorised hazardous waste handler / recycler |
| Biohazard (first-aid) waste | Certified biomedical waste collector |

6.7.3 Manifests, Verification and Area Inspections

- All disposals shall be made through the online GPCB XGN system, entering waste details, receiving vendor and transport vehicle number; vehicles shall have GPCB-approved GPS.

- Manifests, disposal certificates and the annual return to the pollution control board shall be maintained.

- HSE shall verify the validity of vendor authorisations before each contract and periodically thereafter.

- HSE shall inspect hazardous substance and hazardous waste storage areas monthly using a checklist (labels, containment, leaks, segregation, quantity, signage).

6.8 Chemical Safety Competence Building

The Company provides training to all employees and contract workers who store, handle, transport or dispose of hazardous substances.

| Target Group | Training Content | Frequency |
| --- | --- | --- |
| New joiners and contract workers | Hazard pictograms and labels, PPE, emergency actions, reporting | At induction |
| Chemical handlers (stores, production, maintenance) | SDS reading, safe storage, handling, transfer, spill response | Annually and when a new chemical is introduced |
| HSE and Stores staff | Hazardous waste rules, manifests, vendor verification, inspections | Annually |
| Emergency response team | Spill and fire drills involving chemicals | Half-yearly |

Attendance and evaluation records shall be maintained by HR / HSE.

### 7. REVIEW AND CORRECTIVE ACTION

- Findings from inspections, incidents and audits shall be recorded with root cause, corrective action, responsibility and target date.

- Effectiveness of actions shall be verified by HSE.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016

- Manufacture, Storage and Import of Hazardous Chemical Rules, 1989

- Factories Act, 1948 and applicable State Factories Rules

- Central Motor Vehicles Rules, 1989 (transport of hazardous goods)

- NSPL/HSE/SOP/004 – Waste Management

- Waste Disposal Record (NSPL/HSE/FM/024) and Waste Disposal Passbook (NSPL/HSE/FM/025)

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Hazardous Substance Register and SDS file | HSE Department | As applicable |
| Storage area inspection checklists | HSE Department | As applicable |
| Spill / incident reports | HSE Department | As applicable |
| Hazardous waste manifests, disposal certificates and annual returns | HSE / Stores | As applicable |
| Waste Disposal Record and Passbook | Stores | As applicable |
| Training records | HR / HSE | As applicable |

### 1. PURPOSE

To define how the Company gives surplus materials and assets a second life, separates discards at the point of generation, traces each discard flow from origin to final destination and builds workforce awareness of zero-waste practices.

### 2. SCOPE

This SOP applies to all waste generated in production, utilities, maintenance, stores, offices, canteen and other areas of the facility, by employees, contract workers and service providers. Detailed controls for hazardous waste disposal are given in the Hazardous Substance Control and Safe Handling SOP, and retired electronics in the Digital Asset Longevity and Retired Electronics Control SOP.

### 3. OBJECTIVES

The Company aims to:

- Reduce waste generation by reusing, recovering and repurposing materials, equipment and infrastructure.

- Segregate all waste at source into defined waste streams.

- Maintain a waste stream map showing the route of every stream from generation to final destination.

- Increase the share of waste sent for recycling and recovery instead of disposal.

- Ensure employees know how to reduce and sort waste correctly.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Waste stream | Group of waste with similar characteristics that is collected and managed together (e.g. paper, metals, hazardous) |
| Reuse | Using an item again for the same or a similar purpose without reprocessing |
| Recovery / Recycling | Reprocessing waste into materials or energy |
| Repurpose | Using an item or structure for a different purpose than originally intended |
| Flow tracing | Systematic tracing of each waste stream from point of generation through collection, storage and transport to final recycling, recovery or disposal |
| EPR | Extended Producer Responsibility |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Plant Head | Ensure this SOP is implemented; approve waste reduction initiatives and recycling partners |
| HSE Department | Coordinate the SOP; maintain the waste stream map; monitor segregation; verify recycler authorisations; arrange training |
| Production / Maintenance / QC | Minimise scrap; identify reusable items; segregate waste at source and hand it over to Stores |
| Stores | Take custody of waste; store by stream; arrange disposal / sale to authorised parties; maintain disposal records |
| Housekeeping | Collect waste from bins by stream; keep collection points clean |
| All Employees | Reduce waste and use the correct bin |

### 6. PROCEDURE

6.1 Circular Use of Surplus Materials and Assets

6.1.1 Order of Preference

Every material that is no longer needed shall be evaluated in the following order of priority:

- Reuse – use again without quality risk.

- Rework / Repair – restore to usable condition where technically feasible and cost-effective.

- Repurpose – use for a different internal purpose.

- Recycle / Recover – send to authorised recyclers.

- Dispose – only where none of the above is possible.

6.1.2 Second-Life Practices by Material

| Material / Item | Practice |
| --- | --- |
| Module and cell scrap | Rework where approved by QA; otherwise sent for recycling to authorised recyclers (e.g. Regain, Giga Salvage) under the Company's EPR arrangements |
| Aluminium frame off-cuts, copper ribbon / busbar, steel parts | Segregated as valuable metal scrap and sent to authorised metal recyclers |
| Glass breakage | Collected safely and sent to glass recyclers |
| Glass pallets and packaging | Pallets redesigned for reuse in module packaging; cartons, wooden pallets and crates reused internally before recycling |
| Machine spare parts and equipment | Repaired / reconditioned and reused; surplus equipment redeployed to other lines or areas |
| Electronic components | Usable components recovered from discarded equipment for maintenance spares where safe |
| Used oil | Reused for suitable alternative applications before disposal to certified recyclers |
| Infrastructure (racks, cabins, furniture) | Repurposed for storage, offices or other areas |

6.1.3 Recording Second-Life Quantities

Quantities reused, recovered and repurposed shall be recorded monthly by Stores and reviewed by HSE.

6.2 Point-of-Generation Separation and Colour-Coded Collection

Waste shall be separated at the point of generation into labelled, colour-coded bins or designated areas, as below:

| Waste Stream | Examples | Collection Container |
| --- | --- | --- |
| Organic / wet waste | Canteen and food waste | Green bin |
| Dry / office waste | Paper, office waste, dry domestic waste | Blue bin |
| Paper and cardboard | Cartons, interleaf paper, packaging | Labelled bin / bale area |
| Plastics | Wraps, strapping, EVA / backsheet trims, PVC | Labelled bin, segregated by plastic type |
| Glass | Broken solar glass | Labelled, covered glass bin (handled with PPE) |
| Metals | Aluminium off-cuts, copper ribbon, steel parts | Labelled scrap bins by metal type |
| Wood | Damaged pallets, crates | Designated area |
| Cell / module scrap | Broken cells, rejected modules | Labelled scrap bins / racks |
| E-waste and batteries | IT equipment, electronics, batteries | Designated e-waste and battery storage |
| Hazardous waste | Chemical containers, used oil, contaminated cloth, sealant waste | Hazardous waste storage area |
| Biohazard waste | First-aid waste | Red bin with biohazard symbol |

- Bins shall be placed at strategic points to cover the entire plant, with pictorial labels.

- Mixed or wrongly sorted waste shall be re-sorted by Housekeeping and the issue reported to the area in-charge.

- Each stream shall be handed over only to authorised recyclers, collectors or disposal facilities appropriate to that stream.

6.3 Origin-to-Destination Flow Tracing

HSE shall prepare and maintain a waste stream map that traces each stream from where it is generated to its final destination. The map shall be reviewed at least annually and whenever a process, waste type or vendor changes.

| Waste Stream | Generation Point | Collection & Storage | Handled By | Receiver | Final Fate |
| --- | --- | --- | --- | --- | --- |
| Cell / module scrap | Stringer, layup, lamination, testing | Scrap bins → scrap yard | Production → Stores | Authorised recycler | Recycling / material recovery |
| Metal scrap | Framing, maintenance | Metal bins → scrap yard | Production → Stores | Authorised metal recycler | Recycling |
| Glass | Glass loading, lamination | Glass bin → scrap yard | Production → Stores | Glass recycler | Recycling |
| Plastics / EVA / backsheet trims | Layup, packing | Plastic bins → store | Housekeeping → Stores | Authorised recycler | Recycling |
| Paper / cardboard | Stores, packing, offices | Bins → bale area | Housekeeping → Stores | Authorised recycler | Recycling |
| Hazardous waste | Production, maintenance | HW storage area (≤ 90 days) | Stores / HSE | GPCB-authorised vendor / TSDF | Treatment / recycling / disposal |
| E-waste & batteries | Offices, IT, maintenance | E-waste store | IT / Stores | Authorised e-waste recycler | Recycling |
| Food / wet waste | Canteen | Green bins | Housekeeping | Approved / municipal collector | Composting / disposal |
| Biohazard waste | First-aid room | Red bin | Housekeeping / HSE | Biomedical waste collector | Treatment |

The map shall be supported by quantities (kg / tonnes per month) for each stream, so that the efficiency of each step can be assessed and improvement areas identified.

6.4 Workforce Awareness for Zero-Waste Practices

| Target Group | Training Content | Frequency |
| --- | --- | --- |
| All employees and contract workers | Waste hierarchy, bin colour coding, correct sorting, reporting | At induction and annually |
| Production and maintenance | Scrap reduction, reuse and rework options, scrap recording | Annually |
| Housekeeping staff | Collection by stream, handling glass and hazardous waste safely | Half-yearly |
| Stores and HSE | Vendor authorisation checks, disposal documentation, waste mapping | Annually |

Awareness shall be reinforced through toolbox talks, posters at bins and periodic sorting audits.

### 7. MONITORING AND CORRECTIVE ACTION

- HSE shall carry out periodic segregation audits of bins and storage areas.

- Monthly waste quantities by stream and by final fate (reuse, recycling, recovery, disposal) shall be reviewed.

- Non-conformities shall be corrected and their root causes addressed.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- NSPL/HSE/SOP/004 – Waste Management

- Scrap Policy of the Company

- Solid Waste Management Rules, 2016; Plastic Waste Management Rules, 2016

- Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016

- E-Waste (Management) Rules, 2022; Battery Waste Management Rules, 2022

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Waste stream map | HSE Department | As applicable |
| Scrap registers and reuse / recovery records | Production / Stores | As applicable |
| Waste Disposal Record (NSPL/HSE/FM/024) and Passbook (NSPL/HSE/FM/025) | Stores | As applicable |
| Recycler authorisations, agreements and certificates | HSE / Stores | As applicable |
| Segregation audit reports | HSE Department | As applicable |
| Training records | HR / HSE | As applicable |

### 1. PURPOSE

To extend the useful life of the Company's IT hardware through refurbishment and remanufacturing, and to ensure that e-waste generated from the Company's own use is sorted, tracked and handed over for proper recycling, so that electronic waste is reduced and fully traceable.

### 2. SCOPE

This SOP applies to all IT and electronic equipment used internally by the Company, including desktops, laptops, servers, monitors, printers, network devices, mobile phones, tablets, UPS units, batteries, cables and accessories.

It does not cover electronic waste arising from the products the Company manufactures (solar modules and production scrap), which are managed under the Company's EPR arrangements and the Resource Recovery and Discard Flow Management SOP.

### 3. OBJECTIVES

The Company aims to:

- Extend the life of IT equipment through repair, upgrade and refurbishment.

- Reduce the quantity of electronic waste generated.

- Sort internal e-waste by type for effective recycling.

- Maintain traceability of every e-waste item from retirement to recycling.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Refurbishing | Inspecting, cleaning, repairing, upgrading and replacing components to restore equipment to original working condition |
| Remanufacturing | Rebuilding equipment from used and new parts to a specification equivalent to new |
| E-waste | Electrical and electronic equipment, whole or in part, that is discarded |
| IT Asset Register | List of all IT assets with tag number, user, location, status and history |
| Data sanitisation | Secure wiping or destruction of data storage so data cannot be recovered |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| IT Department | Maintain IT Asset Register; assess, repair, refurbish and redeploy equipment; sanitise data; retire assets; initiate e-waste handover |
| HSE Department | Verify e-waste recycler authorisation; monitor e-waste storage and documentation; file returns as required |
| Stores | Store e-waste by category; arrange dispatch to authorised recycler; maintain dispatch records |
| Finance / Accounts | Update asset records on write-off and disposal |
| All Employees | Return unused or faulty IT equipment to IT; do not discard electronics in general bins |

### 6. PROCEDURE

6.1 Asset Tagging and Register

Every IT asset shall be tagged with a unique asset number and recorded in the IT Asset Register with its make, model, serial number, purchase date, user / location, condition and repair or upgrade history.

6.2 Restoration and Upgrade of Computing Equipment

6.2.1 Condition Assessment

When equipment is faulty, slow or returned by a user, IT shall assess it before considering replacement:

| Assessment Result | Action | Example |
| --- | --- | --- |
| Repairable | Repair / replace faulty component | Replace keyboard, battery, power supply, fan |
| Upgradable | Upgrade to extend life | Add RAM, replace HDD with SSD, reinstall OS |
| Usable for lower-demand role | Refurbish and redeploy | Use as shop-floor terminal, training PC, kiosk |
| Parts usable | Harvest working parts as spares | Memory, drives, power supplies, screens |
| Beyond economical repair | Retire as e-waste (Section 6.3) | Damaged motherboard, obsolete servers |

6.2.2 Restoration Steps

- Back up any required data and sanitise the storage device.

- Clean the equipment internally and externally.

- Inspect and test all components; replace or upgrade defective or outdated parts.

- Reinstall licensed operating system and software; apply security updates.

- Test for performance and safety; label as refurbished with date.

- Update the IT Asset Register and redeploy.

6.2.3 External Restoration Partners

Where refurbishment is outsourced, IT shall use specialist service partners or OEM programmes, require written confirmation of data sanitisation and work performed, and record the job in the IT Asset Register. Equipment that can still be used but is no longer needed by the Company may be given to an authorised refurbisher for reuse.

6.3 End-of-Use Electronics Tracking and Responsible Handover

6.3.1 Retirement and Data Sanitisation

- Equipment declared beyond use shall be approved for retirement by the IT Head.

- All data storage shall be sanitised or physically destroyed before the item leaves IT custody, and a sanitisation record kept.

- Each retired item shall be marked "E-Waste" with its asset number.

6.3.2 Categorisation of Retired Electronics

| E-Waste Category | Examples | Storage |
| --- | --- | --- |
| IT and telecom equipment | Desktops, laptops, servers, monitors, printers, routers, phones | Labelled e-waste bins / racks |
| Batteries | Laptop and phone batteries, UPS batteries | Separate, cool, dry area away from ignition sources; terminals protected |
| Cables and accessories | Cables, chargers, keyboards, mice | Labelled containers |
| Lamps and small electronics | Tube lights, LED lamps, small devices | Separate labelled containers |

6.3.3 Recording and Handover

- Enter each item in the E-Waste Register: asset number, category, quantity / weight, date of retirement, storage location.

- Store e-waste in the designated e-waste area until handover.

- Hand over e-waste only to an authorised / registered e-waste recycler. The Company has an Agreement / MOU dated 01-04-2025 with M/s Regain Energies Solutions Pvt. Ltd. for collection, transportation, handling and recycling of e-waste (valid for 7 years, subject to the recycler's GPCB authorisation).

- Before each handover, HSE shall confirm that the recycler's authorisation is valid.

- Prepare dispatch documents listing items, weights and asset numbers; obtain the recycler's acknowledgement and recycling certificate.

- Close each asset in the IT Asset Register and E-Waste Register with the recycler's certificate reference.

- Maintain records and file returns as required under the E-Waste (Management) Rules, 2022 and Battery Waste Management Rules, 2022.

6.3.4 Do's and Don'ts for Employees

- Never throw electronic items or batteries into general waste bins.

- Return all unused, faulty or old devices to the IT Department.

- Report damaged, swollen or leaking batteries immediately to IT / HSE.

### 7. MONITORING AND CORRECTIVE ACTION

- IT and HSE shall reconcile the IT Asset Register with the E-Waste Register at least half-yearly.

- Any untraced asset or e-waste item shall be investigated and corrective action recorded.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- E-Waste (Management) Rules, 2022

- Battery Waste Management Rules, 2022

- Agreement / MOU with M/s Regain Energies Solutions Pvt. Ltd. dated 01-04-2025

- NSPL/HSE/SOP/004 – Waste Management

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| IT Asset Register (incl. repair / refurbishment history) | IT Department | As applicable |
| Data sanitisation records | IT Department | As applicable |
| E-Waste Register | IT / Stores | As applicable |
| Dispatch documents and recycling certificates | Stores / HSE | As applicable |
| Recycler agreement and authorisation copies | HSE Department | As applicable |

### 1. PURPOSE

To define how the Company guides customers on getting the best performance and longest life from its solar PV modules, designs modules that deliver more energy with fewer resources, and supports repair so that customers choose repair over replacement wherever possible.

### 2. SCOPE

This SOP applies to all solar PV modules manufactured and sold by the Company, including the N-Type TOPCon bifacial dual-glass G12R series (NSG-BFT60 to NSG-BFT132), and to the information, technical support, spare parts and repair services provided to customers, installers, EPC companies and channel partners.

### 3. OBJECTIVES

The Company aims to:

- Give customers clear guidance on unloading, storage, handling, installation, cleaning and maintenance of modules.

- Continuously improve module efficiency and durability so that each module generates more electricity over a longer life.

- Make repair practical through diagnostics, spare parts, trained technicians and the warranty process.

- Reduce premature replacement of modules in the field.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| JB | Junction Box – enclosure on the back of the module containing terminals and bypass diodes |
| Voc / Isc | Open-circuit voltage / Short-circuit current of the module |
| EL test | Electroluminescence imaging used to detect cell cracks and defects |
| STC | Standard Test Conditions – 1000 W/m², 25 °C cell temperature, AM 1.5 |
| Degradation | Gradual reduction in module power output over time |
| Residual Price | Purchase price after 5% depreciation per year, as defined in the Limited Warranty Statement |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Quality Assurance | Own this SOP; approve product literature, cleaning instructions, repair procedures and spare parts; analyse field failures and warranty claims |
| R&D / Technical | Improve module efficiency and durability; maintain datasheets and the Installation and User Manual |
| Technical Support / Service | Respond to customer queries; diagnose issues; carry out or guide repairs |
| Stores / Logistics | Pack and dispatch modules as per NSPL/ST/SOP/005; maintain stock of approved spare parts |
| Sales & Marketing | Share product documents with customers and channel partners; collect feedback |

### 6. PROCEDURE

6.1 Owner's Guidance from Storage to Upkeep

Every customer shall receive, with the product or digitally, the product datasheet, the Installation and User Manual (NS/IM/001), the Module Cleaning Instruction (NSPL/QA/CI/01), the Unloading & Unpacking Instructions (NSPL/ST/SOP/005) and the Limited Warranty Statement. These documents give the following guidance:

| Stage | Guidance Provided |
| --- | --- |
| Unloading | Check packaging, model, serial numbers and quantity against the delivery order; prefer forklift over crane; lift only one pallet at a time; drive forklifts at ≤ 5 km/h straight and ≤ 3 km/h when turning; do not unload in winds above Beaufort 6, heavy rain or snow; take photos / videos of unloading for warranty registration |
| Transport and storage | Keep modules in the supplied packaging until installation; secure pallets against movement and falling; do not exceed the marked stacking height (vertical landscape packages not more than two layers); store in a cool, dry place |
| Handling | Handle by the frame, never by the junction box or cables; never step on modules or place loads on them; do not install modules damaged in transit; do not paint, modify or disassemble modules |
| Installation | Keep 10–15 mm clearance between modules; avoid tilt angles below 10–15° so that rain can wash off dirt; avoid shading; follow electrical ratings, grounding and connector instructions of the manual |
| Cleaning | Clean during low-light hours (early morning / late afternoon) when modules are cool; use soft brush, sponge or micro-fibre cloth and clean (de-ionised / low-mineral) water at ambient temperature; do not use pressure washers, steam, corrosive chemicals, metal objects or abrasives; never clean modules with broken glass or exposed wiring; do not step on modules |
| Periodic inspection | Check that cables and connectors are undamaged and secured, no sharp objects touch the module, no unwanted shading, and mounting and grounding parts are tight and free of corrosion; address defects immediately |
| Warranty registration | Register on the "PV Module(s) Owner" portal at www.navitassolar.in within 30 days of purchase |

Note: the water-pressure limit for cleaning shall be the same in NS/IM/001 and NSPL/QA/CI/01 [To be confirmed by the Company].

6.2 More Energy per Module, Fewer Inputs per Watt

The product range is being moved to technologies that generate more electricity per module and per square metre, so that fewer modules, less mounting structure, cabling and land are needed for the same plant capacity. For example, the NSG-BFT132 G12R module (as per its datasheet):

| Design Feature | Benefit During Use |
| --- | --- |
| 132 half-cut N-Type TOPCon cells, 16 busbars | Higher efficiency (up to about 24%) and better shading tolerance |
| Bifacial dual-glass construction (bifaciality 80 ± 5%) | Additional generation from reflected light; durable glass on both sides |
| Low temperature coefficient of Pmax (−0.29 %/°C) | Less power loss in hot climates |
| Low degradation – 1% in year 1, 0.40% per year thereafter | More energy over a 30-year life; performance warranty to at least 85% of nameplate power at year 30 |
| PID resistance; 5400 Pa snow / 2400 Pa wind load rating | Reliable output over the operating life |
| No consumables needed in operation | Only periodic cleaning water; no batteries, filters or chemicals |

- New cell and module technologies (such as half-cut, multi-cut, IBC and perovskite-TOPCon) shall be evaluated for higher efficiency and lower degradation.

- Every new product design review shall record expected efficiency, degradation rate and material use per watt compared with the previous model.

- Modules shall be qualified to IEC 61215 and IEC 61730 before release.

6.3 Fix-First Support Model

6.3.1 Fault Reporting and Diagnosis

- The customer reports the issue in writing as per Clause G of the Limited Warranty Statement (to info@navitassolar.com or the registered address) within 30 days of discovering it, with model, serial numbers, installation date and location, description, photographs, monitoring data, test report and invoice details.

- Technical Support reviews the information and, where required, arranges a site inspection or testing at a Company facility or a recognised third-party laboratory (measurements per IEC 60904 at STC).

- Diagnosis uses suitable tools such as multimeter (Voc / Isc), IR thermography, IV-curve tracing and EL testing.

- The fault is classified as repairable on site, repairable at the factory, or not repairable.

6.3.2 Repair Before Replacement

| Option | Description |
| --- | --- |
| Field repair by Company technician | Replacement of junction box, connectors or cables as per NSPL/QAD/CC/SOP/002 – Junction Box Rework At Site |
| Repair by approved technician / installer | Carried out only by a licensed service technician or installer approved by the Company, using approved parts (repairs by others void the warranty) |
| Factory repair | Module returned with prior written authorisation and as per packaging and shipping instructions |
| Replacement / supplementary module / compensation | Only where repair is not possible, as per the Limited Warranty Statement |

- Repair is the first option under the Limited Warranty Statement; remanufactured or refurbished parts may be used when repairing or replacing modules.

- Modules replaced under warranty become the property of the Company and are routed for repair, reuse or recycling (see Module Return, Recycling and End-of-Life Guidance SOP).

6.3.3 Replacement Components and Test Equipment

- Stores shall maintain approved spare parts such as junction boxes, connectors, cables, sealant and potting compound.

- Approved diagnostic and repair tools shall be available to field technicians.

- Spare part availability period after a model is discontinued: [To be confirmed by the Company].

6.3.4 Learning from Repairs

Quality Assurance shall analyse warranty claims, repair and failure data monthly to identify recurring causes and feed improvements into design, materials, manufacturing and customer documents.

### 7. REVIEW AND CORRECTIVE ACTION

- Customer documents shall be reviewed whenever a product, standard or installation practice changes, and kept consistent with each other.

- Recurring field issues shall trigger corrective and preventive action through Quality Assurance.

### 8. REFERENCE DOCUMENTS

- NS/IM/001 – Installation and User Manual

- NSPL/QA/CI/01 – Module Cleaning Instruction

- NSPL/ST/SOP/005 – SOP Unloading & Unpacking Instructions

- Limited Warranty Statement for Photovoltaic Module Products

- Product datasheets (e.g. NSG-BFT132 G12R)

- NSPL/QAD/CC/SOP/002 – Junction Box Rework At Site

- IEC 61215 and IEC 61730 test reports

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Current versions of customer documents | Quality Assurance | As applicable |
| Warranty claims and diagnosis records | Technical Support / QA | As applicable |
| Repair / rework records | Technical Support / QA | As applicable |
| Design review records | R&D / QA | As applicable |

### 1. PURPOSE

To define how the Company controls restricted substances in its electrical and electronic products, identifies product components to enable sorting and recovery, and designs modules so that they can be easily dismantled, remanufactured or recycled at end of life.

### 2. SCOPE

This SOP applies to the design, material selection, supplier approval and labelling of all solar PV modules manufactured by the Company and their electrical and electronic components, including cells, ribbons and busbars, solder, junction boxes, bypass diodes, cables, connectors, encapsulants, glass, sealants, adhesives and frames.

### 3. OBJECTIVES

The Company aims to:

- Eliminate or minimise restricted substances in products and components.

- Ensure every material supplier provides verified material declarations.

- Mark products and components so that they can be identified and sorted for recovery.

- Increase the share of the module that can be separated and recycled at end of life.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Restricted substance | Substance whose use is prohibited or limited due to health or environmental risk |
| RSL | Restricted Substance List maintained by the Company |
| BOM | Bill of Materials – list of materials and components of a module, as certified |
| Sn60Pb40 | Tin-lead alloy (60% tin, 40% lead) used as a coating on copper ribbons |
| PBB / PBDE | Polybrominated biphenyls / Polybrominated diphenyl ethers (brominated flame retardants) |
| DEHP, BBP, DBP, DIBP | Phthalate plasticisers |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| R&D / Technical | Apply design-for-recovery principles; select materials; maintain the RSL and the certified BOM |
| Quality Assurance | Verify declarations and test reports; approve components; control product labelling |
| Purchase | Obtain declarations and test reports from suppliers; include restricted-substance requirements in purchase orders |
| Production | Use only approved materials; apply product labels correctly |

### 6. PROCEDURE

6.1 Controlled Chemistry in Components

6.1.1 Current Material Composition

The certified bill of materials of the G12R TOPCon bifacial series (IEC 61730 test report No. 6250204.50Bv1.1, Annex 2) shows the following main materials:

| Component | Material | Restricted Substance Note |
| --- | --- | --- |
| Front glass | 2.0 mm AR-coated tempered glass | – |
| Rear glass | 2.0 mm low-iron semi-tempered glass (no polymer backsheet) | – |
| Encapsulant | EPE (front and rear) | Declaration to be obtained |
| Cell and string ribbons | Tinned copper with Sn60Pb40 coating | Contains lead |
| Frame | 6005-T6 anodised aluminium alloy | – |
| Junction box | IP68, Class II, potted | Declaration to be obtained (flame retardants) |
| Cables | 4 mm² copper, IEC 62930 certified | Declaration to be obtained (PVC, phthalates, halogens) |
| Connectors | IEC 62852 certified | Declaration to be obtained |
| Adhesives / potting | Silicone-based (frame and junction box) | Declaration to be obtained |
| Edge sealing tape | Polymer tape | Declaration to be obtained |

6.1.2 Restricted Substance List

The RSL shall include at least the following substances, with applicable limits:

| Substance Group | Substances |
| --- | --- |
| Heavy metals | Lead, mercury, cadmium, hexavalent chromium |
| Other metals of concern | Zinc, yttrium, beryllium, nickel, tin, antimony trioxide |
| Brominated / halogenated flame retardants | PBB, PBDE and other brominated or halogenated flame retardants |
| Phthalates | DEHP, BBP, DBP, DIBP |
| Polymers of concern | Polyvinyl chloride (PVC) |

6.1.3 Control and Reduction Steps

- Purchase shall obtain a material declaration and, where applicable, a third-party test report for every component before approval.

- Quality Assurance shall review declarations against the RSL; new components exceeding limits shall not be approved unless technically unavoidable and justified by R&D.

- Purchase orders and specifications shall require compliance with the RSL.

- Declarations shall be renewed when a supplier, material or process changes, and periodically ([To be confirmed by the Company]).

- Lead reduction: R&D shall evaluate lead-free ribbon coatings and solders to replace the current Sn60Pb40 coating, including qualification and re-certification to IEC 61215 / IEC 61730. Target date: [To be confirmed by the Company].

- Halogen-free and PVC-free cables, junction box and connector materials shall be preferred when selecting or changing suppliers.

6.2 Identification Marks for Downstream Separation

- Each module shall carry a durable label (marking durability verified in IEC 61730 testing) showing manufacturer, model, serial number, electrical ratings, certifications and the crossed-out wheelie bin symbol.

- The serial number shall remain intact and legible; it links each module to its certified BOM so that recyclers can be informed of its materials.

- Connectors and wiring shall be marked as required by IEC 61730.

- A material composition sheet for each model, including the presence of lead in ribbon coating, shall be shared with recycling partners.

- Packaging shall be marked with its material type to support recycling.

6.3 Separable Construction Principles

| Design Principle | Application to Modules |
| --- | --- |
| Separable frame | 6005-T6 aluminium frame bonded with silicone adhesive, which can be removed mechanically for direct metal recycling |
| Removable junction box | Junction box can be removed and replaced (NSPL/QAD/CC/SOP/002), allowing repair and separation |
| Standard parts | Industry-standard IEC 62852 connectors and IEC 62930 cables instead of proprietary parts |
| Material simplicity | Glass–glass construction without a polymer backsheet; main materials are glass, aluminium, silicon and copper, which are recyclable |
| Recyclable packaging | Cartons and pallets that can be reused or recycled |
| Information for recyclers | Dismantling guidance and material composition in the Module Recycling Manual |

Every new design review shall include a check of these principles, and the expected recoverable fraction of the module by weight shall be recorded.

### 7. REVIEW AND CORRECTIVE ACTION

- The RSL and progress on lead reduction shall be reviewed annually.

- Any component found to exceed RSL limits shall be investigated and replaced; affected products shall be assessed by Quality Assurance.

### 8. REFERENCE DOCUMENTS

- IEC 61730 test report No. 6250204.50Bv1.1 (DEKRA, issued 22-12-2025) – Annex 2: Bill of Materials

- Restricted Substance List (RSL)

- Module Recycling Manual of the Company

- E-Waste (Management) Rules, 2022

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Restricted Substance List and certified BOM | R&D / QA | As applicable |
| Supplier material declarations and test reports | Purchase / QA | As applicable |
| Lead-reduction evaluation and qualification records | R&D / QA | As applicable |
| Material composition sheets per model | R&D | As applicable |

### 1. PURPOSE

To define how the Company informs users about the correct end-of-life handling of its solar modules and collects used modules back for remanufacturing, recycling or safe disposal, so that materials stay in circulation and are not sent to landfill.

### 2. SCOPE

This SOP applies to all solar PV modules sold by the Company that are damaged, obsolete or at the end of their useful life, wherever they are installed, and to the information provided to customers, installers, EPC companies and channel partners.

### 3. OBJECTIVES

The Company aims to:

- Ensure users know how to return or hand over used modules responsibly.

- Collect end-of-life modules free of charge through authorised partners.

- Maximise the recovery of glass, aluminium, silicon, copper and other materials.

- Prevent uncontrolled disposal and leakage of module components to the environment.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| End-of-life (EoL) module | Module that is damaged, defective beyond repair, obsolete or decommissioned |
| Take-back | Collection of used products from users by or on behalf of the manufacturer |
| EPR | Extended Producer Responsibility under the E-Waste (Management) Rules, 2022 |
| Authorised recycler | Recycler registered / authorised by CPCB / SPCB to process e-waste |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Quality Assurance / EPR Coordinator | Own this SOP; maintain EPR registration and recycler arrangements; maintain take-back records |
| Sales & Marketing / Customer Service | Inform customers about return options; receive return requests |
| Logistics | Arrange collection and transport with recycling partners |
| HSE Department | Verify recycler authorisations; ensure safe handling of returned modules |

### 6. PROCEDURE

6.1 Retirement Guidance for Module Owners

The Installation and User Manual (NS/IM/001) already directs users, under Decommissioning and Recycling, to dismantle PV systems with the same care as installation (modules can generate hazardous voltage even after disconnection) and to contact Navitas Solar or a reputed recycling agency for proper recycling. The manual, website and warranty documents shall include end-of-life information covering:

- Do not dispose of modules with household or general waste, by burning or by dumping (crossed-out wheelie bin symbol on the module label).

- Safe removal: disconnect and isolate the system, cover the module face to stop generation, wear gloves and handle broken glass carefully.

- Temporary storage: keep removed modules stacked flat or upright in a dry, covered area; keep broken modules wrapped to prevent glass and material loss.

- How to return: contact details and procedure for the Company's free collection service.

- What happens next: modules are sent to authorised recyclers where glass, aluminium, silicon, copper and other materials are recovered for new products.

Customer contact for returns: info@navitassolar.com / +91 70690 70990 (as published in the Limited Warranty Statement); dedicated return contact, if any: [To be confirmed by the Company].

6.2 Free Return Pathway

6.2.1 Arrangement

The Company has a formal arrangement with an EPR-approved firm, as per Central Pollution Control Board requirements, under which end-of-life solar modules are collected for recycling at no cost to the end user. Authorised recycling partners include Regain and Giga Salvage, who also recycle the Company's production-stage module and cell scrap.

6.2.2 Return Process

- User submits a return request with quantity, model / serial numbers, location and condition of modules.

- Customer Service registers the request and confirms whether modules can be repaired (refer Product Care, Longevity and Repair Support SOP) or are end-of-life.

- Logistics and the recycling partner schedule collection; modules are packed to prevent breakage.

- Recycling partner collects modules and issues a receipt with quantity and weight.

- Partner dismantles and recycles modules and issues a recycling / disposal certificate.

- Quality Assurance / EPR Coordinator updates the Take-Back Register and EPR records.

6.2.3 Recovery Routes

| Condition of Returned Module | Route |
| --- | --- |
| Repairable / functional | Repair or remanufacture for reuse, after QA testing |
| Non-functional, intact | Recycling – separation of frame, junction box, glass, cells and polymers |
| Broken | Recycling with contained handling of glass and cell fragments |
| Residues that cannot be recycled | Safe disposal through authorised facilities |

Modules replaced under the Limited Warranty become the property of the Company and follow the same routes; remanufactured or refurbished parts may be used for warranty repairs and replacements.

Buy-back of used modules: [To be confirmed by the Company].

### 7. REVIEW AND CORRECTIVE ACTION

- Take-back requests, quantities collected and recycling certificates shall be reviewed periodically.

- Any delay in collection or missing certificate shall be followed up with the partner and corrected.

- Recycler authorisations and agreements shall be verified before expiry.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- E-Waste (Management) Rules, 2022 (EPR for solar PV modules, panels and cells)

- Module Recycling Manual of the Company

- NS/IM/001 – Installation and User Manual (Decommissioning and Recycling)

- Limited Warranty Statement (Clause K – Miscellaneous)

- Agreements with EPR / recycling partners

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Take-Back Register (requests, collection, quantities) | QA / EPR Coordinator | As applicable |
| Collection receipts and recycling certificates | QA / EPR Coordinator | As applicable |
| EPR registration and returns | QA / EPR Coordinator | As applicable |
| Recycler agreements and authorisations | HSE / QA | As applicable |

### 1. PURPOSE

To define how the Company studies the health and safety risks of its products during use, informs customers about safe use, and responds quickly to product safety events, including withdrawal or recall of affected products, so that people and property are protected.

### 2. SCOPE

This SOP applies to all solar PV modules supplied by the Company, from dispatch through transport, installation, operation and maintenance at customer sites, and to all customers, installers, EPC companies, distributors and channel partners.

### 3. OBJECTIVES

The Company aims to:

- Identify and evaluate potential health and safety hazards of products during use.

- Provide clear safety information and awareness to everyone who handles the products.

- Detect, escalate and contain product safety events without delay.

- Communicate promptly and transparently with affected customers and authorities.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Product safety event | Any incident, complaint or finding indicating that a product may cause injury, fire, electric shock or property damage |
| Field action | Corrective action on products already supplied, such as inspection, repair, replacement, withdrawal or recall |
| Recall | Retrieval of affected products from customers or the market |
| Safety Response Team (SRT) | Cross-functional team that manages product safety events |
| Traceability | Ability to identify affected products by serial number, batch and customer |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Plant Head / Top Management | Decide on recall or withdrawal; approve external communication |
| Quality Assurance | Own this SOP; lead the Safety Response Team; carry out risk evaluation, investigation and traceability |
| R&D / Technical | Conduct product safety studies; support root-cause analysis |
| Sales / Customer Service | Receive safety complaints; communicate with customers and partners |
| Logistics | Arrange retrieval and segregation of recalled products |
| Legal / Compliance | Advise on regulatory notification and contractual obligations |

### 6. PROCEDURE

6.1 Use-Stage Hazard Evaluation

The G12R TOPCon bifacial module series (NSG-BFT60 to NSG-BFT132) has been tested by DEKRA Testing and Certification (Shanghai) Ltd. under the IECEE CB Scheme, with test reports issued on 22-12-2025:

| Standard | Report No. | Safety-Relevant Tests Included |
| --- | --- | --- |
| IEC 61215-1/-1-1/-2:2021 (design qualification) | 6250204.50Av1.1 | Hot-spot endurance, potential-induced degradation (PID), thermal cycling, damp heat, mechanical load, hail |
| IEC 61730-1/-2:2023 (safety qualification) | 6250204.50Bv1.1 | Fire test (spread of flame, burning brand), sharp edge, accessibility, bypass diode function, equipotential bonding, insulation and wet leakage current |

Quality Assurance and R&D shall assess potential health and safety impacts of each product family and of any significant design or material change. The assessment shall cover:

| Risk Area | Points Studied |
| --- | --- |
| Electrical | Electric shock from DC voltage, insulation failure, arcing at connectors, earthing |
| Fire | Hot spots, bypass diode failure, junction box overheating, fire classification of materials |
| Mechanical | Glass breakage, frame failure, wind and snow load, sharp edges during handling |
| Material and chemical | Release of substances from damaged or burning modules; restricted substance content |
| Installation and maintenance | Working at height on rooftops, handling heavy modules, cleaning practices |

- Inputs: certification test results, reliability and accelerated-ageing tests, field failure data, customer complaints and published research.

- Outputs: documented risk assessment with controls in design, manufacturing and product information; reviewed at least annually.

6.2 Safe-Use Guidance and Outreach

- The installation / user manual shall contain safety warnings and instructions on transport, handling, installation, electrical connection, operation, cleaning and maintenance.

- Safety labels on modules shall show electrical ratings and warnings against touching live parts and working on energised systems.

- Datasheets shall state certifications, application and safety class (Class A, Class II), fire rating and mechanical load ratings; the fire rating shall be stated consistently across datasheet, manual and test report [To be confirmed by the Company].

- The Installation and User Manual (NS/IM/001) includes safety warnings: do not disconnect connectors under load; do not touch modules with broken glass unless disconnected and wearing PPE; modules can produce dangerous voltage during and after a fire – inform the fire crew; install rooftop systems only after structural assessment and with fall protection; do not work in strong wind or on wet roofs.

- Awareness sessions or materials (training for installers and channel partners, safety leaflets, website guidance) shall be provided on safe handling, installation and maintenance.

- Product safety information shall be updated when the risk assessment or a safety event identifies new risks.

6.3 Field Incident Escalation and Market Withdrawal

6.3.1 Receipt and Initial Evaluation

- Customers report product concerns to info@navitassolar.com / +91 70690 70990 or through their dealer / EPC partner; any person receiving a product safety complaint or incident report shall inform Quality Assurance within [To be confirmed by the Company].

- Quality Assurance records the event and collects details: product, serial numbers, site, photographs and nature of harm.

- Quality Assurance classifies the event by severity.

| Severity | Description | Response |
| --- | --- | --- |
| Critical | Injury, fire or risk to life, or many products potentially affected | Convene Safety Response Team immediately; consider recall |
| Major | Potential for injury or property damage, limited number of products | Investigate within [To be confirmed by the Company]; field inspection or repair |
| Minor | No safety risk; performance or cosmetic issue | Handle through normal complaint process |

6.3.2 Investigation and Containment

- Hold affected stock in the warehouse and stop dispatch of suspect batches.

- Use traceability records to identify all affected serial numbers, batches and customers.

- Carry out root-cause analysis (component, process, transport or installation).

- Give affected customers interim safety instructions, such as isolating the system.

6.3.3 Decision and Field Action

Based on the investigation, Top Management shall decide on the appropriate action: field inspection, repair, replacement, withdrawal or recall.

6.3.4 Communication Plan

| Audience | Message | Channel |
| --- | --- | --- |
| Affected customers and EPC partners | Nature of risk, immediate precautions, action plan and contact | Phone, e-mail, letter, site visit |
| Distributors and channel partners | Stop sale, quarantine stock, return process | E-mail, written notice |
| Regulatory authorities / certification bodies | Notification where required by law or certification rules | Formal letter |
| Employees | Containment instructions and changes | Internal communication |

6.3.5 Retrieval, Closure and Learning

- Retrieved products shall be segregated and assessed for repair, recycling or safe disposal.

- Effectiveness of the field action shall be monitored until all identified products are addressed.

- Corrective actions shall be built into design, manufacturing and product information, and the event closed by Quality Assurance with Top Management approval.

6.3.6 Readiness Drills

The Safety Response Team members and contact list shall be kept up to date, and a mock recall / traceability exercise shall be conducted at least once a year to test that affected products can be identified quickly.

### 7. REVIEW AND CORRECTIVE ACTION

- Product safety events, complaint trends and mock-recall results shall be reviewed in management review.

- This SOP shall be reviewed annually and after any recall.

### 8. REFERENCE DOCUMENTS

- IEC 61215 test report No. 6250204.50Av1.1 and IEC 61730 test report No. 6250204.50Bv1.1 (DEKRA, 22-12-2025)

- NS/IM/001 – Installation and User Manual

- Limited Warranty Statement

- Product installation / user manuals and datasheets

- Product Care, Longevity and Repair Support SOP (this Manual)

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Product health and safety risk assessments | QA / R&D | As applicable |
| Customer awareness materials and training records | Sales / QA | As applicable |
| Product safety event log and investigation reports | Quality Assurance | As applicable |
| Field action / recall records and communications | Quality Assurance | As applicable |
| Mock recall exercise reports | Quality Assurance | As applicable |

### 1. PURPOSE

To define the steps the Company takes to screen, assess, audit and monitor the third parties it works with on environmental, social and governance (ESG) matters, so that ESG risks are identified early and third-party ESG incidents are prevented.

### 2. SCOPE

This SOP applies to all third parties the Company engages with, including raw material and component suppliers, service providers, contractors, labour contractors, transporters, distributors, sales representatives, commission agents, brokers, customs brokers and consultants.

### 3. OBJECTIVES

The Company aims to:

- Communicate ESG expectations to every third party.

- Screen third parties for ESG risk before and during the relationship.

- Verify ESG performance through self-assessments, internal assessments and on-site audits.

- Ensure non-conformities are corrected and improvements are supported.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Third party | Any external organisation or individual providing goods or services to, or acting on behalf of, the Company |
| SAQ | Self-Assessment Questionnaire completed by the third party |
| CAP | Corrective Action Plan agreed with the third party to close non-conformities |
| High-risk third party | Third party with elevated ESG risk due to category, geography, spend or past findings |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Top Management | Approve the programme; decide on suspension or termination of high-risk third parties |
| Purchase | Own this SOP; obtain Code of Conduct acknowledgement and SAQs; conduct risk screening; maintain third-party ESG records |
| HSE / Quality Assurance | Participate in on-site audits; evaluate environmental, safety and quality aspects |
| HR / Admin | Support labour and human rights checks of contractors and labour suppliers |
| Finance | Support integrity checks of agents, distributors and intermediaries |

### 6. PROCEDURE

6.1 Onboarding and Code Acknowledgement

- Every new third party shall sign / acknowledge the Company's Supplier Sustainability Code of Conduct, covering legal compliance, environmental protection, health and safety, labour and human rights, prohibition of child and forced labour, non-discrimination, ethical conduct and anti-bribery.

- Every purchase order shall carry the Company's standard terms, which include: clause 22 (EHS) – supplier to minimise environmental impact, provide MSDS and TREM cards for hazardous chemicals, not employ child or forced labour, maintain safe working conditions, non-discrimination, freedom of association, legal working hours and at least minimum wages; clause 23 (Supplier Due Diligence & Compliance) – supplier to comply with anti-corruption, labour and data protection laws and the Supplier Code of Conduct / Due Diligence Policy, and to submit to periodic due diligence assessments, audits and supply chain transparency requests; and clause 6 (Confidentiality).

- Supplier PO acceptance is required within 2 days, failing which the PO terms are deemed accepted.

- Vehicles delivering to the site shall carry the documents listed in the PO (invoice, packing list, material test certificate, e-way bill, LR copy, driver ID card, vehicle fitness certificate, insurance); vehicles without documents shall not enter the premises.

- Basic integrity checks (legal registration, licences, sanctions / adverse media search) shall be completed for agents, distributors and intermediaries.

6.2 ESG Risk Screening

Purchase shall classify each third party by ESG risk, considering:

| Risk Factor | Examples |
| --- | --- |
| Category | Raw materials and components, labour supply, hazardous waste handling, transport, intermediaries acting on the Company's behalf |
| Geography | Countries or regions with higher labour, environmental or corruption risk |
| Business relevance | Spend, criticality and single-source dependency |
| History | Past assessment results, incidents, complaints or adverse media |

Result: Low, Medium or High risk. The risk level decides the depth of due diligence in Section 6.3.

6.3 Assessment Methods

| Method | Description | Applied To |
| --- | --- | --- |
| Supplier Self-Assessment on ESG | Third party completes the Company's ESG SAQ covering environment, labour and human rights, health and safety, ethics and management systems, and provides supporting documents (policies, certificates, licences) | All key suppliers; mandatory for Medium and High risk |
| Internal ESG Assessment | Purchase and HSE review SAQ responses, documents, certifications, published ESG reports and public information; score the third party | All assessed third parties |
| On-Site ESG Audit | Company team (or appointed auditor) visits the third-party site: document review, site inspection, management discussion and worker interviews; announced or unannounced | High-risk third parties and those with significant SAQ gaps |

6.3.1 On-Site Audit Steps

- Plan the audit: scope, checklist, team and date.

- Conduct an opening meeting with the third party's management.

- Inspect the facility (workplace safety, chemical storage, waste and effluent handling, dormitories where applicable).

- Review records (wages, working hours, age proofs, licences, environmental consents, safety records).

- Interview workers confidentially.

- Hold a closing meeting and share findings.

- Issue the audit report with findings rated Critical, Major or Minor.

6.4 Corrective Action and Capacity Building

| Finding | Required Action | Closure Timeline |
| --- | --- | --- |
| Critical (e.g. child / forced labour, bribery, serious safety or pollution risk) | Immediate escalation to Top Management; business may be suspended until resolved | [To be confirmed by the Company] |
| Major | CAP with responsibility and dates; follow-up verification | [To be confirmed by the Company] |
| Minor | Improvement recommendation; checked at next assessment | [To be confirmed by the Company] |

- The Company shall support third parties with guidance, awareness sessions and good-practice sharing to close gaps.

- Repeated or unresolved critical findings may lead to termination of the relationship as per contract.

6.5 Ongoing Monitoring

- Re-assess third parties periodically according to risk level (High: [To be confirmed by the Company]; Medium: [To be confirmed by the Company]; Low: [To be confirmed by the Company]).

- Monitor incidents, complaints and adverse news involving third parties.

- Provide training to the Purchase team on ESG risk identification, due diligence and audit methods.

### 7. REVIEW AND CORRECTIVE ACTION

- Purchase shall report the status of screening, assessments, audits and CAPs to management periodically.

- This SOP shall be reviewed annually.

### 8. REFERENCE DOCUMENTS

- Supplier Sustainability Code of Conduct

- ESG Self-Assessment Questionnaire and On-Site Audit Checklist

- Purchase order terms and conditions (clauses 6, 22 and 23)

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Code of Conduct acknowledgements | Purchase | As applicable |
| Third-party ESG risk register | Purchase | As applicable |
| Completed SAQs and internal assessment scores | Purchase | As applicable |
| On-site audit reports and CAPs | Purchase / HSE | As applicable |
| Training records of the Purchase team | HR / Purchase | As applicable |

### 1. PURPOSE

To define the reporting and approval process for gifts, hospitality, travel, sponsorships and other discretionary transactions with external parties that may be seen as favours, so that business decisions are never influenced, or seen to be influenced, by personal benefit.

### 2. SCOPE

This SOP applies to all directors, employees, contract staff and anyone acting on behalf of the Company, and covers transactions with customers, suppliers, contractors, government officials, consultants, agents and other external parties, both given and received.

### 3. OBJECTIVES

The Company aims to:

- Prevent bribery, corruption and conflicts of interest arising from discretionary transactions.

- Set clear limits and an approval process for each type of transaction.

- Maintain a transparent register of all such transactions.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Sensitive transaction | Discretionary transaction with an external party where there is, or may appear to be, a personal benefit in addition to the business benefit |
| Gift | Anything of value given or received without payment (goods, vouchers, services) |
| Hospitality | Meals, entertainment, events or accommodation offered or received |
| Sponsored travel | Travel or accommodation paid by or for an external party |
| Government official | Employee or representative of any government body, public sector undertaking or regulator |
| Compliance Officer | Person designated by Management to administer this SOP |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Top Management | Approve limits; approve high-value or exceptional transactions |
| Compliance Officer / Admin | Own this SOP; maintain the register; review requests; report to management |
| Heads of Department | First-level approval of requests within limits |
| Finance | Pay only approved expenses; flag unusual payments |
| All Employees | Seek approval before giving or accepting; declare all items received |

### 6. PROCEDURE

6.1 General Principles

- Never offer, give, request or accept anything of value to obtain or reward a business decision or improper advantage.

- Cash, cash equivalents (gift cards, vouchers, loans) and personal services are never permitted.

- Nothing shall be offered to or accepted from a party during an active tender, negotiation or audit.

- All transactions must be modest, infrequent, transparent and properly recorded.

- Transactions with government officials always require prior approval of the Compliance Officer.

6.2 Thresholds and Approval Matrix

| Type of Transaction | Value per Person | Approval Required | Register Entry |
| --- | --- | --- | --- |
| Promotional items with Company logo (diary, pen, calendar) | Up to ₹[TBC] | No prior approval | Not required |
| Gifts given or received | Up to ₹[TBC] | Head of Department | Yes |
| Gifts given or received | Above ₹[TBC] | Compliance Officer + Top Management | Yes |
| Business meals / hospitality | Up to ₹[TBC] | Head of Department | Yes |
| Entertainment, events, sponsored travel or accommodation | Any value | Compliance Officer (prior approval) | Yes |
| Anything involving a government official | Any value | Compliance Officer (prior approval) | Yes |
| Donations and sponsorships | Any value | Top Management | Yes |

[TBC] = monetary limits to be set by the Company.

6.3 Request and Approval Steps

- Employee completes the Sensitive Transaction Request / Declaration Form before giving or accepting (or within [To be confirmed by the Company] working days of receipt, where the item was received unexpectedly).

- Details: date, external party, relationship, type of item, estimated value, business purpose and any ongoing dealings with the party.

- Approver reviews against the principles and thresholds and approves, rejects or sets conditions.

- Approved transaction is recorded in the Gifts and Hospitality Register by the Compliance Officer.

- Finance processes payment only with the approved form attached.

6.4 Handling Items That Cannot Be Accepted

- Return the item politely with a note explaining the Company policy.

- If return is not practical (e.g. perishable items), hand it over to the Compliance Officer for sharing among staff or donation, and record the action.

6.5 Monitoring and Awareness

- The Compliance Officer shall review the register periodically for patterns (repeat parties, high values, timing near decisions).

- Finance shall review hospitality and travel expense claims for compliance with this SOP.

- This SOP shall be covered, together with the Code of Conduct and Anti-Bribery Policy, in induction training (where HR policies are communicated) and in periodic ethics training.

- Concerns or violations may be reported under the Company's Whistleblower Policy or to the grievance committee, without fear of retaliation.

### 7. VIOLATIONS

Violation of this SOP shall be investigated and may lead to disciplinary action, as per the Company's code of conduct and applicable law.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- Code of Conduct, Anti-Bribery Policy and Whistleblower Policy of the Company

- Prevention of Corruption Act, 1988 (as amended)

- Whistleblower / grievance mechanism of the Company

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Sensitive Transaction Request / Declaration Forms | Compliance Officer / Admin | As applicable |
| Gifts and Hospitality Register | Compliance Officer / Admin | As applicable |
| Review reports and investigation records | Compliance Officer | As applicable |
| Training records | HR | As applicable |

### 1. PURPOSE

To define how the Company detects, contains and recovers from security incidents affecting confidential information, limits further damage and informs and reassures affected parties and authorities as required.

### 2. SCOPE

This SOP applies to all information held or processed by the Company in any form (electronic or paper), including customer, supplier, employee and business-confidential data, and to all IT systems, devices, networks, cloud services and third parties handling Company information.

### 3. OBJECTIVES

The Company aims to:

- Detect security incidents quickly.

- Contain incidents and limit their impact.

- Restore normal operations safely.

- Notify affected parties and authorities within required timelines.

- Learn from incidents to prevent recurrence.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Security incident | Event that threatens the confidentiality, integrity or availability of information or systems |
| Data breach | Unauthorised access to, or disclosure, alteration, loss or destruction of, personal or confidential data |
| IRT | Incident Response Team |
| CERT-In | Indian Computer Emergency Response Team |
| Containment | Actions to stop an incident spreading or causing further damage |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Top Management | Approve major decisions and external communication |
| IT Head (IRT Lead) | Own this SOP; lead the Incident Response Team; coordinate technical response |
| IT Department | Monitor systems; contain, investigate and recover; preserve evidence |
| HR / Admin / Legal | Handle employee-related aspects, legal advice and regulatory notifications |
| Department Heads | Support assessment of business impact for their data |
| All Employees | Report suspected incidents immediately; do not attempt own investigation |

### 6. PROCEDURE

6.1 Incident Response Team

The IRT shall comprise the IT Head (lead), IT staff, a representative of HR / Admin, Legal / Compliance and the head of the affected department. The IRT contact list shall be displayed and kept current.

6.2 Detection and Reporting

- Sources of detection: antivirus / firewall alerts, system logs, unusual system behaviour, user reports, third-party or customer notifications.

- Examples to report: phishing e-mail clicked, lost or stolen laptop / phone / pen drive, malware or ransomware, unauthorised access, e-mail sent to wrong recipient with confidential data, misplaced confidential documents.

- Any employee who suspects an incident shall report it immediately to the IT Department by phone or e-mail ([To be confirmed by the Company]).

6.3 Seven-Stage Handling Cycle

| Phase | Actions |
| --- | --- |
| 1. Assess and classify | Log the incident; confirm whether it is genuine; classify severity (High: personal / confidential data exposed or critical systems down; Medium: limited impact; Low: no data exposure) |
| 2. Contain | Isolate affected devices or accounts from the network; disable compromised accounts; change passwords; block malicious addresses; remotely lock / wipe lost devices |
| 3. Preserve evidence | Keep logs, images and affected items; record all actions with time |
| 4. Investigate | Identify cause, method of attack, systems and data affected, and number of people affected |
| 5. Eradicate and recover | Remove malware; patch vulnerabilities; restore systems and data from clean backups; monitor closely after restoration |
| 6. Notify | Inform affected parties and authorities as per Section 6.4 |
| 7. Close and learn | Prepare incident report with root cause and corrective actions; update controls and training |

6.4 Notification and Stakeholder Communication

| Recipient | When | Responsibility |
| --- | --- | --- |
| Top Management | Immediately for High severity incidents | IT Head |
| CERT-In | Within 6 hours of noticing reportable cyber incidents, as per CERT-In directions (2022) | IT Head |
| Data Protection Board and affected individuals | For personal data breaches, as required under the Digital Personal Data Protection Act, 2023 and rules made under it | Legal / Compliance with IT |
| Affected customers / suppliers | Without undue delay where their data is involved, explaining what happened, likely impact, actions taken and steps they can take | Department Head with Legal |
| Employees | When their data is affected or action is needed from them | HR |

6.5 Readiness Measures

- Maintain regular backups of ERP, MES and production data (EL, flash test and traceability data) on the Company's NAS server, and test restoration at defined intervals ([To be confirmed by the Company]).

- Keep antivirus and firewall up to date on all systems.

- Keep system logs for investigation purposes.

- Continue IT and cybersecurity policy awareness training for users, covering phishing, password security and incident reporting.

- Record all IT incidents and system failures in the incident log; analyse them and present IT performance and incidents in management review (MRM).

- Cover incident handling in the IT internal audit (checklist NSPL/MR/F/013); the internal audit of 23-04-2026 recorded no IT incidents to date.

- Include information security risks in the risk register (NSPL/MR/F/011).

- Conduct at least one incident response drill per year.

- Require third parties handling Company data to report incidents to the Company promptly (contract clause).

### 7. REVIEW

Every High severity incident and the annual drill shall be reviewed by management, and this SOP updated as needed.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- IT Policy of the Company (approved by Top Management)

- Information Technology Act, 2000 and CERT-In Directions dated 28 April 2022

- Digital Personal Data Protection Act, 2023 and rules made thereunder

- Personal and Partner Data Stewardship SOP (this Manual)

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| Incident log | IT Department | As applicable |
| Incident investigation reports and evidence | IT Department | As applicable |
| Notifications sent to authorities and affected parties | Legal / IT | As applicable |
| Drill and training records | IT / HR | As applicable |

### 1. PURPOSE

To define how the Company protects personal and confidential data of customers, suppliers and other external parties against unauthorised access, keeps it only for defined periods, and informs and obtains the consent of the people whose data it collects, uses and shares.

### 2. SCOPE

This SOP applies to all personal and confidential data of customers, channel partners, suppliers, contractors, visitors and other external parties held by the Company, in electronic or paper form, and to all employees and third parties who access it.

### 3. OBJECTIVES

The Company aims to:

- Allow access to external party data only to authorised persons for legitimate business purposes.

- Keep data only for as long as necessary and dispose of it securely.

- Collect and use personal data transparently and with valid consent.

- Respond to requests from individuals about their data.

### 4. DEFINITIONS AND ABBREVIATIONS

| Term | Meaning |
| --- | --- |
| Personal data | Any data about an individual who can be identified by or in relation to that data |
| Data principal | The individual to whom personal data relates |
| Confidential data | Business information of an external party shared in confidence (pricing, drawings, contracts) |
| Data owner | Department head responsible for a category of data |
| Least privilege | Giving users only the access they need for their role |
| DPDP Act | Digital Personal Data Protection Act, 2023 |

### 5. RESPONSIBILITIES

| Function | Responsibilities |
| --- | --- |
| Top Management | Approve this SOP and the retention schedule |
| IT Department | Own this SOP; implement technical access controls, backups and secure deletion |
| Data Owners (Department Heads) | Approve access; ensure retention and disposal for their data |
| HR / Admin / Legal | Prepare privacy notices and consent forms; handle data principal requests; physical security |
| All Employees | Access only what they need; keep data confidential; follow clean desk practices |

### 6. PROCEDURE

6.1 Access Restriction Controls

6.1.1 Digital Controls

| Control | Requirement |
| --- | --- |
| User accounts | Unique user ID and password for every user; in ERP, only authorised persons have access to data; no shared accounts; multi-factor authentication for e-mail and critical systems where available |
| Role-based access | Access to ERP, CRM, e-mail and shared folders granted on least-privilege basis with data owner approval |
| Joiners, movers, leavers | Access created on joining, changed on transfer and removed on the last working day |
| Access review | Data owners review user access rights at least half-yearly |
| Device security | Antivirus, screen lock, encryption of laptops and removable media; USB restrictions where required |
| Network security | Firewall, secure Wi-Fi, separate guest network, VPN for remote access |
| Logging | Access to critical systems logged and reviewed |
| Backup | Business and production data backed up on the NAS server to prevent loss |

6.1.2 Physical Controls

- Server room and record rooms locked, with access limited to authorised persons and a visitor log.

- Confidential paper records kept in locked cabinets; clean desk practice at the end of the day.

- Visitors escorted in areas where confidential data is handled.

6.1.3 Third Parties

- Third parties who process Company-held data (IT vendors, consultants, transporters) shall be bound by confidentiality and data protection obligations; the Company's purchase order terms already include a confidentiality clause (clause 6) and require suppliers to comply with data protection laws (clause 23).

- Data shall be shared only to the extent necessary and through secure channels.

6.2 Holding Periods and Secure Disposal

Each category of external party data shall be kept for the period below and then securely deleted or destroyed, unless a legal hold applies. The periods below are proposed and shall be confirmed by the Company against applicable laws and contracts.

| Data Category | Examples | Retention Period | Disposal Method |
| --- | --- | --- | --- |
| Customer contracts and orders | Agreements, POs, quotations | Contract term + 8 years [TBC] | Secure deletion / shredding |
| Customer invoices and accounting records | Tax invoices, ledgers | 8 years (Companies Act, 2013 s.128) [TBC] | Secure deletion / shredding |
| GST records | Invoices, returns | 72 months from due date of annual return (CGST Act s.36) [TBC] | Secure deletion / shredding |
| Warranty and product traceability data | Serial numbers, customer site details | Warranty period + [TBC] years | Secure deletion |
| Customer and partner contact data (CRM) | Names, phone, e-mail | While relationship is active + [TBC] years, or until consent withdrawn | Secure deletion |
| Supplier records | Registration, KYC, bank details, assessments | Relationship + 8 years [TBC] | Secure deletion / shredding |
| Visitor records and CCTV | Visitor register, footage | [TBC] | Overwrite / shredding |
| Complaint and service records | Complaint forms, site reports | [TBC] | Secure deletion |

- Data owners shall review data due for disposal at least annually.

- Disposal shall be recorded in the Data Disposal Log (category, date, method, authorised by).

6.3 Notice, Permission and Choice for Individuals

6.3.1 Privacy Notice

Personal data is collected, for example, when module owners register on the "PV Module(s) Owner" portal at www.navitassolar.in (owner details, installation photographs, purchase invoice and seller details), when warranty claims are submitted, and at visitor entry. Before or at the time of collection, the Company shall inform the individual in clear language of:

- The type of personal data collected and the purpose of use.

- Third parties with whom it may be shared (e.g. logistics partners, service partners, authorities where legally required).

- How long it will be kept.

- How to withdraw consent, request access, correction or deletion, and raise a grievance (contact: [To be confirmed by the Company]).

6.3.2 Capturing and Logging Permission

- Collect only data that is necessary for the stated purpose.

- Obtain clear, affirmative consent (signed form, tick box or electronic confirmation) where consent is the basis for processing.

- Record the consent with date, purpose and version of the notice.

- Do not use the data for a new purpose without fresh notice and consent.

- On withdrawal of consent, stop processing within a reasonable time and delete data unless retention is required by law.

6.3.3 Requests from Individuals

Requests for access, correction, deletion or grievances shall be logged, verified and responded to within [To be confirmed by the Company] days by HR / Admin / Legal with support from IT.

### 7. REVIEW AND CORRECTIVE ACTION

- IT and data owners shall review access rights, retention compliance and consent records periodically.

- Any unauthorised access or disclosure shall be handled under the Information Security Breach Handling SOP.

### 8. REFERENCE DOCUMENTS AND LEGAL REQUIREMENTS

- Digital Personal Data Protection Act, 2023 and rules made thereunder

- Information Technology Act, 2000

- Companies Act, 2013 and CGST Act, 2017 (record retention)

- Information Security Breach Handling SOP (this Manual)

### 9. RECORDS

| Record | Responsibility | Retention |
| --- | --- | --- |
| User access approvals and access review records | IT Department | As applicable |
| Data retention schedule and Data Disposal Log | IT / Data Owners | As applicable |
| Privacy notices and consent records | HR / Admin / Legal | As applicable |
| Data principal request log | HR / Admin / Legal | As applicable |
| Confidentiality agreements with third parties | Purchase / Legal | As applicable |