Contents

SECTION 1: INTRODUCTION	3

SECTION 2: WASTE MANAGEMENT	6

ETP Sludge Management: Sludge handled through authorised treatment or co-processing facilities.	11

SECTION 3: HAZARDOUS WASTE MANAGEMENT	13

SECTION 4: INTEGRATED WATER MANAGEMENT PROCEDURES	27

SECTION 5: API DISCHARGE REDUCTION & PHARMACEUTICAL WASTEWATER MANAGEMENT	39

SECTION 6: AIR EMISSIONS COMPLIANCE AND COMMUNITY IMPACT PREVENTION	53

SECTION 7: ODOR CONTROL & FILTRATION MANAGEMENT SYSTEM	61

SECTION 8: ACTIONS TO AVOID ROAD CONGESTION AND MINIMIZE TRANSPORT IMPACTS	75

SECTION 9: PRODUCT USE AND END-OF-LIFE (EOL) MANAGEMENT	84

SECTION 10: PHARMACEUTICAL DEMAND FORECASTING & RESPONSIBLE INVENTORY MANAGEMENT	98

SECTION 11: SAFE HANDLING OF CHEMICALS	110

SECTION 12: ERGONOMIC RISK MANAGEMENT	119

SECTION 13: SECURITY CONDUCT AND USE-OF-FORCE PREVENTION	124

SECTION 14: CARBON CAPTURE, STORAGE & EMISSIONS REDUCTION	133

SECTION 15: OPERATIONAL ADAPTATION TO ENVIRONMENTAL DISRUPTIONS	146

SECTION 16: CONFIDENTIAL DATA MANAGEMENT AND RECORD RETENTION PROTOCOL	154

SECTION 17: THIRD-PARTY ETHICAL COMPLIANCE DUE DILIGENCE	163

SECTION 18: RECRUITMENT PROCEDURE	179

SECTION 19: ENERGY MANAGEMENT MEASURES	190

SECTION 20: CUSTOMER FEEDBACK MANAGEMENT SYSTEM	201

SECTION 21: SUSTAINABLE SOURCING MANUAL	212

SECTION 22: CUSTOMER HEALTH & SAFETY MEASURES	217

SECTION 23: EMPLOYEE PERFORMANCE EVALUATION AND CAREER PROGRESSION FRAMEWORK	223

SECTION 24: INTERNAL COMMUNICATION PROTOCOL	229

SECTION 25: EMERGENCY PREPAREDNESS & RESPONSE FOR ENVIRONMENT, HEALTH & SAFETY (EHS)	234

SECTION 26: ENVIRONMENTAL DISRUPTION CONTINGENCY & FINANCIAL RESILIENCE MEASURES	246

SECTION 27: SENSITIVE TRANSACTION APPROVAL AND MANAGEMENT PROTOCOL	257

SECTION 28: INCIDENT RESPONSE PROCEDURE	265

SECTION 29: INTERNAL ASSESSMENT PROCEDURE	273

SECTION 30: SOP MANUAL REVIEW MECHANISM	280

# SECTION 1: INTRODUCTION

Introduction

Alchem International Private Limited is a leading manufacturer in the pharmaceutical, phytochemical, botanical extraction, nutraceutical, and cosmeceutical ingredient industry, specializing in high-quality plant-derived active ingredients and APIs for global pharmaceutical, cosmetic, and nutraceutical brands. With extensive experience in botanical extraction, isolation, purification, synthesis, and finishing, Alchem International is committed to product quality, responsible production, worker welfare, and environmentally conscious operations across its manufacturing facilities.

This Standard Operating Procedure (SOP) Manual has been developed to ensure that all operational activities across the organization are carried out in a consistent, safe, and efficient manner, while meeting applicable legal, customer, and regulatory requirements. The manual establishes a unified operational framework that supports Alchem International's manufacturing processes—from botanical raw material sourcing and production planning to extraction, isolation, purification, quality control, packing, and dispatch—while reinforcing compliance with occupational health and safety, environmental management, and social responsibility standards relevant to the pharmaceutical and phytochemical sector.

The SOP Manual serves as a comprehensive reference for employees, supervisors, managers, and technical personnel by clearly defining roles, responsibilities, and standardized work practices. It promotes operational discipline, ethical conduct, product consistency, and continuous improvement, thereby strengthening Alchem International's commitment to responsible manufacturing, workplace safety, and long-term sustainability across all its operations.

Objective of the SOP Manual

The primary objective of this SOP Manual is to:

- Establish uniform and standardized procedures across all functional areas of Alchem International operations.

- Ensure full compliance with applicable national laws, customer codes of conduct, and relevant international standards applicable to the pharmaceutical, phytochemical, and nutraceutical industry, including occupational health and safety, environmental management, chemical handling, and social compliance requirements.

- Minimize process variability while enhancing product quality, workplace safety, operational reliability, and manufacturing efficiency.

- Provide clear and structured guidance to employees at all levels to ensure consistent, accurate, and accountable execution of roles and responsibilities.

- Serve as a reference document for internal audits, third-party social and quality audits, customer assessments, regulatory inspections, and ongoing employee training and capacity-building initiatives.

Scope of the Manual

This SOP Manual applies to all business units and operational functions within Alchem International, including but not limited to:

- Botanical Raw Material Sourcing and Procurement

- Extraction, Isolation, and Purification Operations

- Synthesis and Semi-Synthesis Operations

- Quality Control (In-process, Final, and Stability Testing)

- Quality Assurance Systems and Documentation

- Chemical Management and Storage

- Environmental Management, Health & Safety (EHS)

- Production Planning and Industrial Engineering

- Procurement, Supply Chain, and Inventory Management

- Human Resources, Payroll, and Administrative Services

- Engineering, Utilities, Machinery Operation, and Maintenance

- Packing, Warehousing, and Dispatch

This manual is applicable to all Alchem International locations, including manufacturing facilities, warehouses, utility areas, and administrative offices. It covers all employees, contract workers, temporary staff, service providers, and any third parties directly engaged in or supporting Alchem International's operations, ensuring consistent implementation of responsible manufacturing practices across the organization.

| Sr. No. | Manufacturing Plant | Address |
| --- | --- | --- |
| 1. | Factory | Alchem International Private Limited, 25/2, Main Mathura Road, Village Kaili, Ballabhgarh, Faridabad, Haryana 121004 |

Core Values of the SOP Manual

Alchem International operates on a strong set of core values that guide decision-making, operational behavior, and responsible manufacturing practices across all levels of the organization. These values form the foundation of every Standard Operating Procedure defined in this manual and support consistent, ethical, and high-quality operations within the pharmaceutical and phytochemical industry.

1. Integrity

Upholding ethical conduct, transparency, and fairness in all manufacturing, employment, and business practices, including interactions with workers, customers, suppliers, and other stakeholders.

2. Quality Excellence

Maintaining a strong commitment to consistent product quality through standardized processes, robust quality control systems, and continuous improvement across extraction, isolation, purification, and packing operations.

3. Health & Safety

Prioritizing the health, safety, and well-being of all employees by maintaining safe workplaces, well-maintained machinery, effective safety protocols, and a culture of accident prevention.

4. Operational Efficiency & Innovation

Embracing efficient production methods, modern equipment, and process improvements to enhance productivity, reduce waste, and meet evolving customer expectations in the pharmaceutical and nutraceutical sector.

5. Accountability

Ensuring clear ownership, responsibility, and traceability at every stage of operations, from raw material sourcing to final shipment, supported by accurate documentation and defined roles.

6. Customer Focus

Delivering reliable, timely, and value-driven products by understanding customer requirements, maintaining open communication, and consistently meeting quality, delivery, and compliance expectations.

7. Compliance

Strictly adhering to applicable laws, environmental regulations, customer codes of conduct, and industry standards relevant to pharmaceutical and phytochemical manufacturing.

8. Sustainability & Social Responsibility

Promoting environmentally responsible practices, efficient resource use, responsible chemical management, and respect for workers' rights to minimize environmental impact and support long-term social sustainability.

# SECTION 2: WASTE MANAGEMENT

Introduction

Alchem International is committed to reducing its environmental footprint across all stages of pharmaceutical, phytochemical, and nutraceutical manufacturing through a structured, practical, and compliant approach to resource use and waste management. As a manufacturer serving global pharmaceutical and nutraceutical markets, Alchem International generates multiple waste streams including spent botanical biomass, extraction residues, rejected batches, packaging materials, chemical containers, sludge from effluent treatment, and general solid waste which require systematic and responsible handling.

This Waste Management Standard Operating Procedure (SOP) establishes Alchem International's framework for the identification, segregation, storage, reuse, recycling, treatment, and disposal of waste generated from extraction, isolation, purification, synthesis, finishing, utilities, warehouses, and administrative areas. The SOP integrates principles of waste minimization, resource efficiency, circular economy, and regulatory compliance, ensuring that environmental responsibility remains embedded in day-to-day manufacturing operations. It supports pollution prevention, continual improvement, and alignment with internationally recognised environmental management practices, including ISO 14001 and responsible sourcing expectations of global pharmaceutical and nutraceutical customers.

Objectives

The objectives of this Waste Management SOP are to:

- Minimise material consumption and waste generation through efficient production planning, process optimisation, yield improvement, and process control.

- Identify, segregate, collect, store, transport, and dispose of all waste streams in accordance with applicable environmental regulations and local authority requirements in India, as well as international customer expectations.

- Promote circular resource practices, including spent biomass utilization, solvent recovery, reuse of packaging materials, safe recycling of chemical containers, and responsible handling of effluent treatment plant (ETP) waste.

- Ensure clear accountability of employees, supervisors, contractors, and waste handlers in implementing environmentally responsible waste management practices.

- Support effective implementation of the Environmental Management System (EMS) and reinforce Alchem International's broader sustainability and ESG commitments.

Applicability & Coverage

This Waste Management SOP applies to:

All departments within Alchem International, including extraction, isolation, purification, synthesis, finishing, quality control, quality assurance, production planning, industrial engineering, utilities, engineering and maintenance, warehousing, packing, administration, human resources, and sustainability functions. All categories of waste generated across operations, including but not limited to solid waste, liquid waste, hazardous and non-hazardous waste, chemical waste, sludge from effluent treatment plants (ETP), spent botanical biomass, packaging waste, recyclable materials, and electronic waste (e-waste). All personnel involved in material handling and waste-related activities, including employees, supervisors, contract workers, housekeeping staff, waste handlers, and third-party service providers responsible for segregation, storage, transport, recycling, treatment, or disposal of waste. All Alchem International operational locations, including manufacturing facilities, production floors, utility areas, chemical storage rooms, effluent treatment facilities, warehouses, packing areas, and administrative offices, ensuring consistent implementation of waste management practices across the organization.

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Department | • Identify and classify all waste streams generated from pharmaceutical and phytochemical manufacturing activities, including extraction, purification, synthesis, utilities, and maintenance activities. • Ensure safe storage, segregation, labelling, and handling of waste containers in designated areas. • Coordinate with government-authorised waste recyclers, treatment facilities, and disposal vendors as per Indian environmental regulations and customer requirements. • Maintain complete documentation, including waste registers, manifests, handover records, disposal certificates, and regulatory compliance records. • Provide mandatory PPE and conduct regular training on waste handling, spill response, and emergency preparedness. |
| Department Heads / Line Managers | • Ensure department-wise compliance with waste management procedures, including segregation, labelling, temporary storage, and safe internal transfer. • Nominate trained and responsible personnel for waste handling within their departments. • Promptly report any spills, leaks, near-miss incidents, or unsafe conditions to the EHS team for immediate action. |
| Warehouse / Stores Team | • Provide and maintain compliant containers for waste storage, including drums, barrels, containers, and spill pallets as applicable. • Ensure all waste containers are clearly labelled, sealed, leak-proof, and stored in designated areas with secondary containment. • Control access to waste storage areas, restrict unauthorised entry, and maintain good housekeeping standards. |
| HR / Administration | • Coordinate and schedule periodic training on waste awareness, emergency handling, spill response, and correct PPE usage. • Support procurement and availability of PPE, spill kits, absorbent materials, containment pallets, and storage infrastructure required for safe waste management. |
| All Personnel | • Strictly follow SOPs and safety instructions while handling waste or hazardous materials. • Use assigned PPE correctly and ensure its proper condition. • Immediately report unsafe practices, spills, improper disposal, or deviations from procedures to supervisors and the EHS team. |

Mapping of Waste Streams

| Type of Waste | Source Area(s) | Disposal / Handling Method |
| --- | --- | --- |
| Spent botanical biomass | Extraction, Isolation | Composting / authorised disposal / biomass energy recovery |
| Rejected / off-spec batches | Production, QC | Rework / authorised disposal |
| Packaging waste (cartons, paper) | Packing, Stores, Admin | Recycling |
| Plastic waste (bags, liners, tapes) | Packing, Admin, QC | Recycling through authorised vendors |
| Chemical containers (empty) | Extraction, Purification, Chemical Stores | Decontamination and recycling / authorised disposal |
| Extraction and purification residues | Extraction, Purification Units | Authorised hazardous waste disposal |
| ETP sludge | Effluent Treatment Plant (ETP) | Secure landfilling / authorised disposal |
| Boiler ash | Utilities | Authorised disposal |
| Used oils and lubricants | Maintenance, Utilities | Authorised oil recyclers |
| Scrap metal | Engineering, Maintenance | Metal recycling |
| Wooden pallets | Stores, Warehousing | Reuse / Recycling |
| Food waste | Canteen | Composting / Municipal collection |
| E-waste | IT, Admin | Authorised e-waste recyclers |
| Damaged tools and equipment | Maintenance, Production | Scrap / authorised disposal |
| Plastic water bottles | Offices, Production Floors | Recycling |

Waste Categorization

| Category | Description |
| --- | --- |
| Hazardous Waste | Chemical residues, extraction and purification residues, contaminated containers, ETP sludge, used oils requiring controlled disposal. |
| Non-Hazardous Waste | General office waste, housekeeping waste, uncontaminated packaging materials, spent biomass not exposed to chemicals. |
| Recyclable Waste | Spent biomass, paper, cardboard, clean plastics, metal scrap, wooden pallets, glass. |
| Chemical Waste | Solvents, reagents, auxiliaries, and contaminated materials requiring authorised treatment. |
| E-Waste | Computers, electronic devices, control panels, cables, batteries, damaged electronic equipment. |
| Organic Waste | Food waste and biodegradable waste from canteen and common areas. |

Waste Segregation Protocol

Alchem International enforces strict waste segregation at the point of generation to prevent contamination, improve recyclability, and ensure compliance with applicable environmental regulations and customer expectations within the pharmaceutical and phytochemical sector.

Color-Coded Bin System (Standardized Across All Facilities)

Red Bin - Hazardous waste (chemical-contaminated materials, extraction residues, contaminated containers, oily waste).

Blue Bin - Recyclable waste (clean plastic bottles, bags, polyethylene films, cartons, paper, glass, metal scrap).

Yellow Bin - Contaminated PPE and cleaning materials (chemical-contaminated gloves, absorbents, wipes).

Green Bin - Organic waste (canteen food waste, spent biomass, and biodegradable waste).

All bins are clearly labelled in English and the local language, supported by visual signage and icons to ensure easy identification by all employees and contract workers.

Staff Training & Awareness

- Mandatory waste segregation and environmental awareness training is conducted at least once every six months for employees, contract workers, housekeeping staff, and facility personnel.

- Training includes practical demonstrations, visual posters, on-floor guidance, and toolbox talks.

- All new employees receive waste segregation and environmental awareness training as part of their onboarding process.

- Refresher sessions are conducted periodically through departmental meetings and internal communication channels.

Bin Maintenance & Monitoring

- Waste bins are inspected and cleaned daily by housekeeping supervisors.

- Waste transfer is carried out using designated, colour-coded trolleys to prevent cross-contamination.

- Damaged, overflowing, or incorrectly labelled bins are immediately replaced or corrected.

Compliance

- Waste management practices comply with applicable Indian environmental regulations, local authority requirements, and customer environmental standards for the pharmaceutical industry.

- Processes are aligned with environmental and occupational health and safety management systems, including ISO 14001 and ISO 45001 principles.

- Periodic internal audits and third-party assessments are conducted to verify compliance.

- Any non-compliance is addressed through Root Cause Analysis (RCA) and Corrective and Preventive Actions (CAPA).

- Continuous monitoring and internal reviews support ongoing improvement.

5R Approach for Waste Reduction in Pharmaceutical Manufacturing

1. Reduce

- Production Planning & Process Optimization: Process optimization and yield improvement techniques to minimise spent biomass and extraction residues.

- Defect Reduction: Inline quality checks to reduce rework and rejected batches.

- Inventory Control: Optimised procurement and FIFO practices to minimise expired chemicals, reagents, and packaging materials.

- Solvent Recovery: Recovery and reuse of solvents through distillation and purification to reduce fresh solvent consumption and waste generation.

- Process Optimisation: Efficient equipment settings, reduced changeover waste, energy-efficient equipment and lighting.

2. Reuse

- Reusable Packaging: Reuse of cartons, pallets, bubble wrap, and poly sheets for internal material movement.

- Container Reuse: Reuse of chemical drums and containers after proper cleaning and decontamination where permitted.

- Reusable PPE: Use of washable gloves, aprons, and uniforms in non-critical areas.

3. Recycle

- Biomass Recycling: Spent botanical biomass sent to authorised composting or biomass energy recovery facilities.

- Paper & Carton Recycling: Office paper and packaging waste baled and recycled.

- Plastic Recycling: Clean plastic waste and bottles sent to authorised recyclers.

- Metal Recycling: Scrap metal from maintenance and engineering activities recycled through authorised vendors.

- Solvent Recovery: Recovery and purification of solvents for reuse in processes.

4. Recovery

# ETP Sludge Management: Sludge handled through authorised treatment or co-processing facilities.

- Heat & Energy Efficiency: Optimised boiler and utility operations to reduce energy losses.

- Water Reuse: Treated ETP water reused for floor cleaning, gardening, or utilities where permitted.

- Solvent Recovery: Distillation and purification of spent solvents for reuse.

5. Repurpose

- Repurposed Containers: Damaged drums and bins reused for waste collection or storage.

- Wooden Pallet Reuse: Scrap pallets reused for maintenance supports and internal logistics.

- Training Use: Obsolete or non-operational equipment used for maintenance and safety training demonstrations.

Continuous Improvement

Alchem International promotes continuous environmental improvement through:

- Regular environmental performance reviews and monitoring.

- Department-wise waste reduction targets and tracking.

- Employee suggestion schemes and Kaizen initiatives.

- Gradual adoption of cleaner technologies and process improvements.

- Benchmarking against global pharmaceutical industry best practices and customer sustainability expectations

# SECTION 3: HAZARDOUS WASTE MANAGEMENT

Introduction

Alchem International recognises the potential environmental, health, and safety risks associated with hazardous waste generated during pharmaceutical, phytochemical, and nutraceutical manufacturing activities. Hazardous waste at Alchem International may arise from processes such as extraction, purification, synthesis, chemical handling, effluent treatment operations, maintenance work, and utility functions. Typical hazardous wastes include chemical-contaminated sludge, spent solvents, used oils and lubricants, contaminated containers, oily rags, chemical residues, and contaminated personal protective equipment (PPE).

This Hazardous Waste Management Standard Operating Procedure (SOP) establishes a structured and comprehensive framework for the identification, segregation, handling, storage, internal movement, transportation, and final disposal of hazardous waste generated across all Alchem International facilities. The SOP is designed to prevent environmental pollution, reduce occupational health and safety risks, ensure legal compliance, and protect employees, contractors, surrounding communities, and the environment.

Aligned with applicable environmental regulations in India—including the Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016, the Factories Act, 1948, and CPCB/SPCB guidelines—recognised international environmental management practices, and the principles of ISO 14001, this SOP ensures that hazardous waste is managed in a manner that:

- Minimises environmental impact and pollution risks

- Promotes safe handling and controlled disposal

- Upholds employee health, safety, and well-being

- Supports Alchem International's sustainability and ESG commitments

Objectives

The primary objectives of this Hazardous Waste Management SOP are to:

- Ensure safe, compliant, and traceable handling of all hazardous waste generated at Alchem International facilities.

- Establish clear procedures for identification, labelling, containment, and storage of hazardous waste streams.

- Reduce hazardous waste generation through process optimisation, efficient chemical usage, preventive maintenance, and material substitution where feasible.

- Define clear roles and responsibilities for hazardous waste handling, internal transfer, and final disposal.

- Support compliance with national environmental regulations (Hazardous and Other Wastes Rules, 2016), customer environmental requirements, and environmental management system principles aligned with ISO 14001.

Applicability & Coverage

This SOP applies to:

All functional departments involved in the generation, storage, or handling of hazardous waste, including but not limited to:

- Extraction, Purification, and Synthesis Operations

- Chemical Storage and Handling Areas

- Effluent Treatment Plant (ETP) and Utilities

- Engineering and Maintenance

- Production Floors and Support Areas

- Quality Control and Inspection Areas

- Warehouse, Raw Material, and Finished Goods Stores

- EHS, Compliance, and Sustainability Functions

All forms of hazardous waste, including but not limited to:

- Extraction and purification residues

- ETP sludge and chemical residues

- Spent solvents and distillation residues

- Used oils, lubricants, and grease

- Chemical-contaminated containers, drums, and packaging

- Contaminated PPE, absorbents, and cleaning materials

- Flammable, corrosive, or toxic chemical wastes

- Oily rags and maintenance waste

All personnel involved in hazardous waste activities, including:

- Permanent employees

- Contract and temporary workers

- Housekeeping and maintenance staff

- Third-party logistics providers and authorised waste disposal vendors

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Department | • Identify and classify all hazardous waste streams generated from pharmaceutical and phytochemical manufacturing activities, including extraction, purification, synthesis, chemical handling, ETP operations, utilities, and maintenance activities. • Ensure safe storage, segregation, labelling, and handling of hazardous waste containers in designated areas. • Coordinate with government-authorised hazardous waste recyclers, treatment facilities, and disposal vendors as per Indian environmental regulations (Hazardous and Other Wastes Rules, 2016) and customer requirements. • Maintain complete documentation, including hazardous waste registers (Form 3), manifests (Form 6/7), handover records, disposal certificates, and regulatory compliance records. • Provide mandatory PPE and conduct regular training on hazardous waste handling, spill response, and emergency preparedness. |
| Department Heads / Line Managers | • Ensure department-wise compliance with hazardous waste management procedures, including segregation, labelling, temporary storage, and safe internal transfer. • Nominate trained and responsible personnel for hazardous waste handling within their departments. • Promptly report any spills, leaks, near-miss incidents, or unsafe conditions to the EHS team for immediate action. |
| Warehouse / Stores Team | • Provide and maintain compliant containers for hazardous waste storage, including drums, barrels, containers, and spill pallets as applicable. • Ensure all hazardous waste containers are clearly labelled, sealed, leak-proof, and stored in designated areas with secondary containment. • Control access to hazardous waste storage areas, restrict unauthorised entry, and maintain good housekeeping standards. |
| HR / Administration | • Coordinate and schedule periodic training on hazardous waste awareness, emergency handling, spill response, and correct PPE usage. • Support procurement and availability of PPE, spill kits, absorbent materials, containment pallets, and storage infrastructure required for safe hazardous waste management. |
| All Personnel | • Strictly follow SOPs and safety instructions while handling hazardous waste or hazardous materials. • Use assigned PPE correctly and ensure its proper condition. • Immediately report unsafe practices, spills, improper disposal, or deviations from procedures to supervisors and the EHS team. |

Use of Sustainable Raw Materials

Alchem International actively integrates circular economy principles and safer chemical management practices into its hazardous waste management framework. The focus is on reducing hazardous waste at source, improving material efficiency, recovering usable inputs, and transitioning to less hazardous and more sustainable alternatives wherever technically and commercially feasible within the pharmaceutical and phytochemical industry.

1. Recovery of Input Materials

- Solvent Recovery & Reuse in Extraction and Purification Operations: Residual solvents from extraction, purification, and synthesis processes are recovered through distillation and purification for reuse where feasible. Controlled reuse of low-risk solvents is carried out after internal verification to reduce fresh solvent consumption and waste generation.

- Reuse of Cleaning Solutions: Cleaning solutions used for equipment, reactors, and tools are segregated by type and contamination level. Where quality and safety parameters allow, these solutions are reused for non-critical cleaning or pre-cleaning activities.

- Recovery of Packaging Inputs: Chemical drums, containers, and IBCs are cleaned and reused internally where permitted or returned to suppliers under take-back arrangements, reducing hazardous container waste.

- ETP By-product Management: Sludge and residues generated from effluent treatment are monitored for calorific value and disposal suitability. Where permitted, co-processing or authorized treatment routes are explored to minimize landfill disposal.

- Side Stream Utilization: Wash water or rinse streams with acceptable quality parameters are reused for secondary washing, floor cleaning, or utility purposes, subject to internal controls and regulatory allowances.

2. Adoption of Eco-Friendly Raw Materials

- Safer Chemical Audits: Periodic reviews are conducted to identify opportunities to replace hazardous chemicals used in extraction, purification, and synthesis with lower-toxicity or environmentally preferable alternatives.

- Low-VOC and Water-Based Chemicals: Preference is given to water-based solvents, reagents, and auxiliaries, as well as low volatile organic compound (VOC) formulations, to reduce air emissions and health risks.

- Bio-Based and Enzyme-Based Products: Where feasible, bio-based catalysts, enzyme-based reagents, and biodegradable auxiliaries are adopted to reduce chemical load in effluents and hazardous waste streams.

- Phosphate-Free and Eco-Labelled Detergents: Phosphate-free detergents and eco-labelled cleaning agents are used in cleaning and equipment maintenance to minimize aquatic toxicity.

- Water-Based Degreasers: Maintenance workshops use water-based degreasers for machinery and component cleaning, reducing reliance on petroleum-based solvents.

- Sustainable Packaging for Chemicals: Preference is given to suppliers providing chemicals in recyclable, reusable, or returnable packaging such as reusable drums and IBCs, with minimal secondary packaging.

3. Use of Alternative and Less Hazardous Substances

- Chemical Substitution: High-hazard chemicals used in extraction, purification, and synthesis are systematically reviewed and replaced with safer alternatives where process performance and product quality are not compromised.

- Reduction of Hazardous Additives: Chemicals classified as carcinogenic, mutagenic, reprotoxic, or highly sensitizing are avoided wherever possible, and use is restricted only to essential applications with strict controls.

- Elimination of Restricted Substances: Substances restricted under international pharmaceutical standards and customer RSL/MRSL requirements are phased out, with strong monitoring of incoming chemicals.

- Reduced Corrosive Chemical Usage: Strong acids and alkalis are replaced with milder formulations in cleaning and finishing processes where feasible, reducing corrosion risks and hazardous effluent loads.

4. Procurement Strategy for Safer Substitutes

- Substitution-First Procurement Policy: The Procurement team follows a substitution-first approach, prioritising suppliers offering safer, environmentally responsible chemical alternatives that meet performance and compliance requirements.

- Hazard and Lifecycle Assessment: Chemical sourcing decisions consider hazard classification, environmental fate, and lifecycle impacts, particularly for high-volume extraction and purification chemicals.

- Supplier Sustainability Performance: Supplier evaluations include environmental and safety criteria such as reduction of hazardous content, waste minimization initiatives, packaging take-back programs, and regulatory compliance.

- Long-Term Responsible Partnerships: Alchem International builds long-term partnerships with suppliers demonstrating strong sustainability credentials, transparent chemical disclosure, and alignment with responsible manufacturing practices.

Labelling, Storage, Handling & Transport Protocols

Proper identification, secure containment, and compliant movement of hazardous waste are critical to minimising risks related to chemical exposure, spills, fire hazards, environmental damage, and regulatory non-compliance. Alchem International has established detailed and rigorous protocols for hazardous waste labelling, storage, handling, and transportation, aligned with applicable Indian environmental regulations (Hazardous and Other Wastes Rules, 2016), recognised international chemical safety practices, and customer environmental compliance expectations. These protocols ensure full operational control, traceability, and accountability throughout the hazardous waste lifecycle.

1. Labelling System

Each hazardous waste container—whether drum, barrel, carboy, IBC, or bag—is labelled immediately at the point of generation and remains fully traceable until final treatment or disposal.

Labelling Standards

All hazardous waste containers must be labelled in accordance with the Globally Harmonized System (GHS) and applicable national standards. Labels must be:

- Clearly printed, laminated, and weather-resistant

- Firmly affixed to the container

- Easily visible from all accessible sides

Label Requirements

Each label must include the following information:

- Material / Waste Stream Name (e.g., Extraction Residue, Spent Solvent, Used Oil, ETP Sludge)

- Chemical Composition / Primary Hazardous Constituents (where applicable)

- Physical State (solid / liquid / sludge)

- GHS Hazard Class Symbols (flammable, corrosive, toxic, reactive, etc.)

- Generating Department & Responsible Contact Person

- Accumulation Start Date & Maximum Storage Duration (as per regulatory limits)

- Waste Quantity (gross, net, and container weight where applicable)

- Proposed Disposal Method (authorised recycler, incineration, secure landfill, co-processing)

- Container Identification Number / Barcode

Label Implementation Notes

- All labels are bilingual (English and Hindi) to ensure universal understanding.

- Barcodes or unique container numbers are logged into the Hazardous Waste Tracking Register or equivalent system maintained by the EHS team.

- Internal colour-coded label borders are used as visual identification aids:

  - Red Border - Toxic or highly hazardous waste

  - Orange Border - Flammable waste

  - Yellow Border - Reactive or unstable waste

  - Black Border - Corrosive waste

In cases where containers are reused (e.g., cleaned chemical drums), all previous labels must be completely removed and destroyed before new labels are applied to prevent misidentification and accidental misuse.

2. Storage Practices

Hazardous waste is stored only in designated, secured, and engineered storage areas designed to prevent leakage, incompatible mixing, fire risk, unauthorised access, and environmental exposure.

Physical Infrastructure Requirements

- Chemical-resistant, epoxy-coated flooring to prevent seepage and allow easy cleaning.

- Secondary containment systems, bund walls, or spill pallets with a minimum capacity of 110% of the largest container stored.

- Fire detection systems, including smoke detectors and heat sensors, connected to the central fire alarm system.

- Restricted access controls, allowing entry only to trained and authorised personnel.

- Adequate ventilation to prevent accumulation of hazardous vapours.

- Intrinsically safe, spark-proof lighting and electrical fittings in areas storing flammable waste.

Storage Management Controls

- Hazardous wastes are segregated based on compatibility (e.g., acids separated from alkalis; flammables isolated from oxidisers).

- Containers are kept sealed at all times except during waste addition or inspection.

- Storage areas are clearly marked with hazard signage and emergency contact information.

- Maximum storage duration limits (as per Hazardous and Other Wastes Rules, 2016 – typically 90 days) are strictly monitored and documented by the EHS team.

Segregation Protocols

At Alchem International, hazardous waste is segregated based on chemical compatibility, physical properties, and risk profile to prevent reactions, fires, spills, and cross-contamination. Dedicated zones are clearly demarcated and controlled.

| Waste Type | Dedicated Zone / Marking |
| --- | --- |
| Reactive Chemicals / Reactive Wastes | Isolated cabinets or enclosures with mechanical ventilation, restricted access, and prominent hazard signage |
| Oxidizers | Dedicated stainless-steel shelving with yellow floor and shelf markings and clear "Oxidizer" signage |
| Flammable Chemical Wastes | Flame-proof storage cabinets or rooms with proper earthing and bonding provisions |
| Heavy Metal-Containing Wastes | Bundled sections with warning tape, secondary containment, and specific hazard labels |
| Incompatible Sludges / Mixed Chemical Residues | Physically segregated areas with barriers and clear incompatibility signage |

Operational Practices

- Hazardous waste containers are stored above ground level on polymer or chemical-resistant pallets to prevent corrosion and contact with floor spillages.

- FIFO (First In, First Out) principles are strictly followed. Accumulation start dates are clearly mentioned to ensure waste is not stored beyond permitted time limits.

- Daily inspections are conducted and documented by authorised personnel to check for:

  - Leaks, corrosion, bulging, or deterioration of containers

  - Integrity, accuracy, and readability of labels

  - Housekeeping standards and pest control status

- Direction arrows, storage area layouts, emergency contact details, and GHS hazard signage are prominently displayed throughout hazardous waste storage and movement zones.

3. Handling Protocols

To prevent chemical burns, inhalation exposure, fire or reactivity incidents, and ergonomic injuries, Alchem International enforces strict handling procedures for all hazardous waste-related activities.

Personnel Qualification

- Only trained and authorised personnel are permitted to handle hazardous waste containers.

- Training covers:

  - Correct PPE selection, use, and limitations

  - Chemical compatibility, segregation rules, and incompatibility risks

  - Emergency spill response and use of spill kits

  - Fire safety measures, extinguisher use, and evacuation procedures

PPE Requirements by Waste Type

| Waste Type | Required PPE Set |
| --- | --- |
| Flammable chemical waste | Antistatic gloves, chemical splash goggles, FFP2 mask or equivalent, chemical-resistant apron, ESD safety shoes |
| Acidic / Corrosive waste | Acid-resistant gloves, full face shield, chemical splash suit or apron, gumboots |
| Reactive / Toxic residues | Flame-retardant coverall, appropriate respirator, nitrile gloves, use of local exhaust ventilation or hood where required |

Material Handling Tools

- Dedicated trolleys with secondary containment trays for safe movement of drums and containers.

- Drum lifters, grabbers, or rotators for handling large containers; manual lifting of full drums is strictly prohibited.

- Spill-resistant pumps or closed transfer systems for liquid movement; manual tipping or pouring is not permitted.

Documentation

- A Material Movement Register (MMR) is maintained for every internal movement of hazardous waste.

- Records include container ID, waste type, quantity, source department, destination, handler name, PPE confirmation, and time/date of movement.

4. Transportation Procedures

Transportation of hazardous waste, both within Alchem International premises and to authorised off-site facilities, is carried out with maximum safety and regulatory control.

Internal Transfer (Within Premises)

- Only approved, corrosion-resistant, and antistatic trolleys are used on designated routes.

- Containers are:

  - Properly sealed and secured with lid locks or restraints

  - Covered where there is potential for vapour release or odour

- Movement is preferably planned during non-peak production hours to minimise interaction with routine operations.

- Routes are pre-mapped, clearly marked, and kept free from obstructions.

External Transfer (To Authorised Treatment / Disposal Facilities)

- Only government-authorised transporters and vehicles are engaged (holding valid consent from SPCB/HPCB).

- The EHS team prepares and verifies required waste manifests (Form 6/7 under Hazardous and Other Wastes Rules, 2016) and transport documentation in line with regulatory and customer requirements.

- Containers are loaded on:

  - Covered vehicles with appropriate hazard placards

  - UN-approved sealed containers where applicable

  - Vehicles equipped with spill kits, MSDS folders, and emergency contact information

- Disposal receipts and weight reconciliation slips are obtained and recorded in the EHS compliance system within defined timelines.

5. Transport Safety Measures

All vehicles used for hazardous waste transport must:

- Be fitted with ABC-type fire extinguishers accessible to the driver and loading area.

- Carry spill containment kits appropriate to the waste type.

- Provide adequate ventilation to prevent vapour accumulation.

- Use GPS-based tracking systems, where applicable, to enable monitoring of route adherence and transport status.

6. Emergency Procedures During Transport

In the event of an accident, spill, leakage, fire, or any transport-related emergency involving hazardous waste:

- The Transport Emergency Response Plan (ERP) is immediately activated by the transport team and EHS.

Primary actions include:

- Securing the area and restricting unauthorised access

- Eliminating ignition sources and stopping vehicle engines where safe

- Deploying spill kits to contain or neutralise the release

- Using appropriate PPE based on the hazard class involved

Secondary actions include:

- Informing local emergency services, authorities, and medical facilities as required

- Notifying the EHS Head and Site Incident Controller

- Initiating internal incident reporting procedures

Following stabilisation, a detailed Root Cause Analysis (RCA) is conducted. Findings and corrective actions are recorded, and CAPA measures are implemented to prevent recurrence.

7. Transport Recordkeeping

All hazardous waste transport activities form part of Alchem International's compliance and audit trail.

Transport records capture:

- Transporter name, licence details, and vehicle registration

- Driver details and contact information

- Route and key checkpoints

- Date and time of departure and arrival

- Container identification numbers, waste categories, and quantities

Records are retained for a minimum of five years, or longer where required by regulations, customer requirements, or Alchem International's internal data-retention policy.

Specialized Treatment & Safe Disposal of Hazardous Substances

1. Onsite Pre-Treatment Activities

To minimise environmental impact and reduce dependency on external disposal facilities, Alchem International implements multiple in-plant pre-treatment measures before hazardous waste is sent off-site:

- pH Neutralisation Systems: Installed in extraction, purification, utilities, and maintenance wash areas. Treat acidic and alkaline wastewater generated during equipment cleaning and process washdowns. Equipped with automated pH monitoring and controlled dosing systems to bring wastewater within acceptable limits before routing to the Effluent Treatment Plant (ETP).

- Oil & Chemical Separation Units: Used oils, greases, and oily wastewater from maintenance activities pass through oil-water separators to isolate hydrocarbons prior to further treatment or disposal.

- Filtration & Adsorption Systems: Activated carbon and filtration units are used at selected discharge points to remove colour, residual chemicals, surfactants, and trace contaminants from wash water prior to ETP/STP treatment.

- Solvent Recovery Systems: Distillation and purification units recover spent solvents for reuse in processes, reducing hazardous waste generation.

2. Authorized Disposal Facilities

All hazardous waste that cannot be safely reused, recycled, or treated on-site is transferred only to government-authorised hazardous waste treatment and disposal facilities, in line with Indian environmental regulations (Hazardous and Other Wastes Rules, 2016) and customer expectations.

Treatment, Storage & Disposal Facilities (TSDF / Authorized Facilities)

Non-recoverable hazardous waste streams include:

- Extraction and purification residues

- ETP sludge

- Spent activated carbon and filter media

- Used oils and contaminated absorbents

- Chemical-contaminated packaging and PPE

Final Treatment Methods

- High-Temperature Incineration: Used for high-calorific and toxic wastes such as contaminated rags, absorbents, oily residues, and chemical sludge, ensuring destruction of hazardous constituents.

- Secure Engineered Landfilling: Used for inorganic, non-recyclable hazardous solids, including filter cakes, ash, and stabilised residues, with engineered liners and leachate control systems.

Pre-Dispatch Controls

Before dispatch to authorised facilities:

- Waste is fully segregated by category

- Containers are sealed, labelled, and documented

- Waste characteristics (where required) are reviewed to determine the appropriate treatment route

Reduction of Toxic Chemical Emissions

Alchem International adopts multiple source-level and system-level strategies to minimise hazardous emissions and reduce downstream treatment loads:

- Process Parameter Optimisation: Extraction, purification, and synthesis parameters (chemical dosage, temperature, dwell time, solvent ratios) are optimised to reduce chemical losses and hazardous residues.

- In-Process Recycling & Reuse: Controlled reuse of wash water, low-risk chemical streams, and internal recycling initiatives are implemented where validated and permitted.

- Standardised Operating Protocols: Detailed SOPs govern chemical addition, concentration limits, storage, and usage to prevent overdosing, spills, and unintended reactions.

- Continuous Monitoring: Monitoring of pH, temperature, flow rates, and chemical usage at critical points enables early intervention and prevents off-spec emissions.

- Targeted Contaminant Isolation: Filtration, sedimentation, and phase separation techniques are used to isolate hazardous constituents before they enter bulk effluent streams.

- Onsite Hazard Reduction: Neutralisation, dilution, or controlled treatment is applied at source to reduce hazard classification before final disposal.

Documentation

Alchem International maintains comprehensive, secure, and auditable documentation for hazardous waste and environmental management:

- Manufacturing and support operations documentation covering chemical receipt, usage, storage, and disposal

- Hazardous waste records including:

  - Generation logs

  - Storage and inspection records

  - Internal movement registers

  - Transport manifests (Form 6/7)

  - Disposal certificates

  - Form 3 returns to SPCB/HPCB

Records are maintained digitally with:

- Regular backups

- Role-based access

- Traceable approval and audit trails

Documentation supports:

- Regulatory compliance

- End-to-end waste traceability

- Customer and third-party audits

- Continuous improvement initiatives

Continuous Improvement

Alchem International fosters a strong culture of continuous improvement in hazardous waste management and overall ESG performance through:

- Regular internal audits and inspections

- Structured CAPA tracking for identified gaps

- Cross-functional involvement from Production, EHS, Quality, Procurement, Maintenance, and HR

- Ongoing training on regulatory updates, safer chemical alternatives, and pollution prevention

- Integration of audit feedback, KPI trends, and stakeholder inputs into system enhancements

Regulatory Compliance

All hazardous waste management systems at Alchem International are designed, implemented, and reviewed to meet or exceed:

- Applicable Indian environmental laws and local authority requirements (Hazardous and Other Wastes Rules, 2016; Factories Act, 1948; CPCB/SPCB guidelines)

- International chemical safety and transport standards, as applicable to export markets and customer requirements

Dedicated EHS personnel:

- Monitor regulatory updates and evolving compliance obligations

- Update SOPs, training modules, and operational controls accordingly

- Engage with accredited laboratories, authorised vendors, consultants, and authorities to maintain high standards of safety, compliance, and sustainability

# SECTION 4: INTEGRATED WATER MANAGEMENT PROCEDURES

Introduction

Alchem International recognises water as a critical and finite resource that underpins its pharmaceutical, phytochemical, and nutraceutical manufacturing operations, including extraction, purification, synthesis, utilities, and domestic use. As a manufacturer supplying global markets, responsible water stewardship is central to Alchem International's sustainability strategy, regulatory compliance, customer expectations, and long-term operational resilience.

This Integrated Water Management SOP defines Alchem International's structured and comprehensive approach to water abstraction, consumption, quality monitoring, conservation, treatment, reuse, and wastewater discharge. It addresses operational and environmental risks associated with water use, including risks related to solvents, reagents, auxiliaries, oils, suspended solids, and other pharmaceutical-processing contaminants present in wastewater streams. The SOP ensures that water management is proactive, preventive, and aligned with responsible manufacturing and environmental protection principles.

Objectives

The objectives of this Integrated Water Management SOP are to:

- Ensure efficient, controlled, and minimised use of water across all operational areas, including production floors, extraction and purification units, utilities, laboratories, and domestic services.

- Monitor, analyse, and manage wastewater responsibly through in-house Effluent Treatment Plant (ETP) and Sewage Treatment Plant (STP) systems, and through authorised external discharge arrangements where applicable.

- Implement source-level controls to capture, isolate, and reduce contaminants such as solvents, chemicals, oils, and suspended solids before they enter common effluent streams.

- Integrate routine water audits, metered consumption tracking, and water quality monitoring into operational controls, management reviews, and sustainability reporting frameworks.

- Ensure compliance with applicable local and national water regulations (Water Act, 1974; Environment Protection Act, 1986; CPCB/SPCB norms), environmental discharge standards, customer requirements, and international sustainability expectations relevant to the pharmaceutical and phytochemical industry.

Applicability & Coverage

This SOP applies to all departments and activities that consume water or generate wastewater, including but not limited to:

- Production Operations (extraction, isolation, purification, synthesis)

- Quality Control and Inspection Areas

- Utilities and HVAC Systems (boilers, cooling towers, chillers, air washers, compressors)

- Effluent Treatment Plant (ETP) and Sewage Treatment Plant (STP)

- Maintenance, equipment cleaning, and floor washing operations

- Sanitary and domestic uses (canteen, washrooms, housekeeping, drinking water systems)

Organizational Responsibilities

| Role | Responsibilities |
| --- | --- |
| EHS Manager / EHS Head | Ensure effective implementation of this SOP across all Alchem International facilities; conduct employee and contractor training; perform internal audits; monitor compliance; and prepare reports related to water use, wastewater treatment, reuse, and regulatory adherence. |
| Production Managers / Line Supervisors | Monitor water consumption within extraction, purification, synthesis, and finishing areas; ensure local conservation measures and SOP controls are followed; identify inefficiencies; and promptly report deviations or abnormal usage trends to EHS and Utilities teams. |
| Maintenance / Utilities Team | Maintain plumbing systems, pipelines, pumps, valves, cooling towers, boilers, RO plants, STP and ETP units; conduct leak detection and repairs; and perform routine inspections to ensure uninterrupted and efficient water management infrastructure. |
| Procurement Team | Source and procure water-efficient fixtures (low-flow taps, aerators), efficient valves and fittings, metering devices, RO membranes, treatment chemicals, and consumables required for water treatment and conservation initiatives. |
| All Employees & Contract Staff | Use water responsibly during operations and domestic use; promptly report leaks, overflows, or abnormal consumption; comply with department-specific water usage guidelines; and actively participate in water conservation awareness programmes. |

Water in Processes

- Water used in extraction, purification, synthesis, quality testing, and supporting production activities is metered at department or process level wherever feasible to enable consumption tracking and optimisation.

- Reverse Osmosis (RO) systems used for process and utility water are operated to maximise recovery rates and minimise reject water generation, supported by preventive maintenance and membrane health monitoring.

- Equipment, reactor, and floor cleaning procedures are designed using defined water quantities or validated cleaning cycles, avoiding uncontrolled free-flow washing and unnecessary water losses.

Process Cooling and Thermal Systems

1. Scheduled Inspection and Preventive Maintenance

Regular preventive maintenance is carried out on cooling towers, chillers, condensers, air washers, and heat exchangers to ensure optimal thermal performance, minimise water loss due to leaks or drift, and maintain energy efficiency.

2. Leak Monitoring and Immediate Repair Protocol

Moisture sensors, level indicators, and routine visual inspections are used to detect leaks in cooling and utility water circuits. Identified leaks are logged and rectified within 24 hours to prevent water wastage and operational disruption.

3. Closed-Loop Cooling Systems

Where technically and economically feasible, Alchem International operates closed-loop cooling systems for chillers and process equipment, reducing blowdown losses, preventing contamination, and minimising wastewater discharge from cooling operations.

4. Adaptive Thermal Load Management

Cooling intensity is automatically adjusted based on real-time production load, ambient conditions, and machine requirements through control systems, preventing overcooling and unnecessary water and energy consumption.

5. Water-Conscious Cooling Technologies

Water-efficient alternatives such as air-cooled chillers, hybrid cooling systems, drift eliminators, and high-efficiency tower internals are implemented or evaluated to reduce reliance on water-intensive cooling methods.

6. Energy Water Nexus Optimisation

Technologies such as Variable Frequency Drives (VFDs), improved insulation, condensate recovery, and optimised pump operations are used to simultaneously reduce water abstraction and electricity consumption.

7. Data-Driven Cooling System Management

Daily and monthly logs covering make-up water usage, blowdown volumes, cycles of concentration, and energy consumption are maintained to identify inefficiencies and support continuous improvement initiatives.

Wastewater Treatment and Reuse

- Pre-treatment of process wastewater from extraction, purification, and synthesis operations is carried out using:

  - Activated carbon filters for colour and organic load reduction

  - pH neutralisation tanks to correct acidic or alkaline streams

  - Solvent recovery and separation units

- Chemical-laden liquid waste (e.g., high-strength extraction residues, spent solvents) is segregated at source, stored in labelled containers, and handed over to authorised hazardous waste handlers instead of being mixed with general effluent.

- All industrial wastewater is routed to the in-house Effluent Treatment Plant (ETP) and domestic wastewater to the Sewage Treatment Plant (STP), where appropriate physical, chemical, and biological treatment processes are employed to reduce pollutants.

- Where applicable, treated effluent meeting prescribed standards is discharged to an authorised Common Effluent Treatment Plant (CETP) or approved discharge points, ensuring compliance with regulatory norms.

- Greywater streams, including RO reject, cooling tower blowdown (where permissible), and wash water from handwash stations, are collected separately and reused for non-critical applications such as toilet flushing, floor cleaning, or gardening (where permitted).

- Alchem International maintains formal agreements with authorised CETP operators and treatment partners, supported by periodic effluent sampling and testing through accredited laboratories (NABL/MoEFCC-approved) to verify compliance.

Groundwater Protection and Monitoring

- No untreated or partially treated industrial or domestic wastewater is discharged to soil, open drains, or natural water bodies. This requirement is strictly enforced across all facilities.

- Effluent is managed through closed collection systems and routed only to authorised treatment facilities (ETP/STP/CETP).

- Secondary containment such as trays and drip pans are installed beneath chemical storage, extraction chemical areas, fuel storage, and critical equipment.

- Spill kits containing absorbents, neutralisers, and booms are positioned at strategic locations for immediate response.

- Containment systems, bunds, trenches, and barriers are inspected periodically to ensure integrity.

- Groundwater abstraction (where applicable) is monitored monthly, and groundwater levels and quality are reviewed annually through internal checks or third-party testing, in compliance with Central Ground Water Authority (CGWA) guidelines.

Rainwater Harvesting and On-Site Water Reuse Initiative

System Implementation

A dedicated rainwater harvesting system has been installed across suitable roof catchment areas of the facility. Rainwater is collected through a network of gutters and downpipes, passed through initial filtration units to remove debris and sediments, and directed into designated storage tanks.

The system has been designed considering local rainfall patterns, roof area availability, and on-site water demand to ensure effective capture and utilization.

Utilization of Harvested Rainwater

Collected rainwater is safely reused for non-potable applications within the facility, including:

- Floor washing and general cleaning activities

- Utility and housekeeping purposes

- Gardening and greenbelt maintenance

- Other approved operational uses where potable water quality is not required

This reuse directly reduces freshwater consumption from external sources and lowers operational water costs.

Monitoring and Performance Tracking

Alchem International maintains periodic records of harvested rainwater volumes and its end-use applications. Usage data is reviewed to assess system effectiveness and contribution to overall water reduction targets.

Seasonal comparisons are conducted to evaluate performance during monsoon and non-monsoon periods.

System Inspection and Maintenance

Regular inspections and preventive maintenance are carried out to ensure the system remains functional and efficient. Activities include:

- Cleaning of gutters and filters

- Inspection of storage tanks for leaks or contamination

- Verification of water flow paths and drainage integrity

Maintenance and inspection records are documented to demonstrate system upkeep and long-term reliability.

Supporting Documentation Maintained

The following documents are available for verification and audit purposes:

- Rainwater harvesting system installation report

- Harvested rainwater use and monitoring reports

- System inspection and maintenance logs

Emergency Water Supply and Contingency Planning

- Overhead and underground storage tanks are designed to provide a minimum two-day backup for essential production, sanitation, and emergency needs.

- Alternate arrangements such as tanker water supply and bottled drinking water are maintained for emergency scenarios, including supply failure or contamination incidents.

- The EHS team maintains a Water Scarcity and Contamination Emergency Response Plan (ERP) covering:

  - Raw water contamination events

  - Failure of primary water sources

  - Regulatory restrictions on water withdrawal

Water Footprint and Sustainability Initiatives

Alchem International actively monitors and improves its water footprint through structured conservation, reuse, and efficiency initiatives aligned with responsible pharmaceutical and phytochemical manufacturing practices.

| Initiative | Status |
| --- | --- |
| Installation of low-flow faucets and aerators | Implemented in 100% of washrooms, canteen areas, and key production wash points |
| Condensate recovery for reuse | Ongoing (phased implementation across boilers, steam lines, and utility systems) |
| Rainwater harvesting feasibility study | Conducted in 2024 for selected Alchem International facilities |
| Awareness training on water conservation | Conducted quarterly for employees and contract staff |

Regular Water Audits and Quality Assessment

Annual water audits are conducted jointly by internal EHS/Utilities teams and certified third-party environmental consultants to assess water-use efficiency, identify high-consumption areas, and detect leaks or system losses. Routine wastewater assessments are carried out by accredited third-party laboratories to verify compliance with discharge standards prescribed by applicable authorities (CPCB/SPCB) and customer requirements for the pharmaceutical sector.

Key metrics monitored include:

- Department-wise water consumption

- Losses due to leakages, overflows, or inefficiencies

- Percentage of water reuse and recycling achieved

- Monthly water quality testing is performed for applicable streams and includes parameters such as:

  - pH

  - Total Dissolved Solids (TDS)

  - Total Suspended Solids (TSS)

  - Oil & Grease (O&G)

  - Chemical Oxygen Demand (COD)

  - Biological Oxygen Demand (BOD)

  - Colour intensity

  - Sulphide

  - Ammoniacal Nitrogen

  - Total Chromium

  - Hexavalent Chromium

  - Heavy metals (Lead, Nickel, Zinc, Copper)

  - Presence of solvents, reagents, auxiliaries, or other pharmaceutical-processing residues (as part of special studies or risk-based monitoring)

Documentation

Alchem International maintains comprehensive and systematic records related to water and wastewater management, including:

- Daily, monthly, and department-wise water consumption logs

- Maintenance and repair records for pipelines, valves, pumps, RO systems, cooling towers, and plumbing infrastructure

- Wastewater and water quality test reports (internal and third-party laboratory results)

- Pre-treatment and ETP/STP logbooks covering neutralisation, filtration, sludge handling, and off-site disposal routing

- Water audit reports, CETP agreements (where applicable), and vendor contracts related to water supply and effluent management

All records are retained in line with legal requirements and Alchem International's internal document control and data-retention policies, supporting audit readiness and traceability.

Continuous Improvement

- Kaizen events and idea campaigns are organised annually to capture employee suggestions on water-saving measures across production floors, utilities, and common areas.

- Technology upgrades under evaluation or implementation include:

  - RO systems with higher recovery efficiency and lower specific energy consumption

  - Flow restrictors and automatic shut-off valves at high-use points

  - Smart meters and IoT-enabled water monitoring systems

- Alchem International benchmarks water intensity (m³ per unit of product output) and uses these benchmarks to set progressive reduction targets over defined timelines.

Compliance

Alchem International complies with:

- Applicable Indian water and environmental protection regulations (Water Act, 1974; Environment Protection Act, 1986; CPCB/SPCB norms)

- Local authority standards for industrial wastewater discharge

- National drinking water quality standards where relevant (BIS 10500)

- Customer-specific wastewater and pharmaceutical industry-aligned requirements

- Vendor manifests, waste tracking, and disposal documentation requirements for hazardous and non-hazardous wastewater residues

Dedicated EHS personnel continuously monitor regulatory updates and ensure timely updates to SOPs, operational controls, and training content.

Source-Level Isolation and Segregation

Periodic water risk assessments (e.g., baseline studies) are conducted to identify:

- High-consumption processes and areas

- Points with elevated risk of solvent, chemical, or oil contamination entering wastewater

- Equipment, pipelines, cooling systems, and ETP components vulnerable to leaks, fouling, or inefficiencies

Key needs identified include:

- Enhanced removal of colour, solvents, and chemical residues at source

- Expansion of wastewater pre-treatment capacity

- Strategic engagement with advanced treatment providers and authorised CETPs where applicable

Removal of Solvents, Chemicals, and Hazardous Pharmaceutical Residues from Wastewater

Alchem International applies a multi-layered strategy to minimise and eliminate solvents, reagents, auxiliaries, and other hazardous pharmaceutical-processing residues from wastewater, preventing contamination of surface water, soil, and aquatic ecosystems.

1. Source-Level Isolation and Segregation

High-strength streams containing solvents, chemicals, oils, or purification auxiliaries—such as extraction wash water, reactor cleaning effluents, concentrated spill washings, and chemical mixing residues—are identified and segregated at the point of generation. These streams are not discharged into stormwater drains or untreated biological systems.

2. Targeted Contaminant Removal Technologies

- Granular Activated Carbon (GAC) systems are used to remove colour, organic compounds, and residual chemicals from high-risk wastewater streams.

- Multi-barrier polishing units, combining filtration, adsorption, and sedimentation, are evaluated or deployed to enhance final effluent quality prior to discharge or reuse.

- Advanced oxidation processes (AOPs) are evaluated for breakdown of recalcitrant pharmaceutical compounds.

3. Secure Handling and Disposal

- Concentrated chemical- or solvent-bearing wastewater and sludge are collected in leak-proof, labelled containers specifying:

  - Chemical or solvent type

  - Approximate concentration

  - Volume

  - Generation source (process, department, batch reference)

- Containers are tracked through internal registers to ensure full traceability.

- Such waste is stored in designated hazardous areas and disposed of through authorised treatment and disposal facilities in compliance with environmental regulations.

4. Research and Innovation Initiatives

- Alchem International's EHS and Engineering teams continuously evaluate advanced treatment options, including:

  - Enhanced biological treatment for improved degradation of pharmaceutical organics

  - Advanced oxidation processes (AOPs) for colour and chemical breakdown

  - Improved adsorption media and filtration technologies

- Collaborations with technology providers, laboratories, and industry experts are pursued to assess scalability, efficiency, and economic feasibility.

5. Regulatory Alignment

- All wastewater management practices align with evolving national regulations (CPCB/SPCB norms) and international benchmarks for pharmaceutical wastewater, including customer requirements and recognised industry guidance on colour, COD, heavy metals, and chemical residue control.

Water Efficiency, Low-Impact Processing, and Responsible Cleaning Practices

Context and Commitment

- Alchem International is committed to minimizing freshwater consumption and reducing wastewater pollution arising from extraction, purification, cleaning, and finishing operations associated with pharmaceutical manufacturing. As an India-based facility operating in a water-stressed region (Faridabad, Haryana), the company prioritizes water efficiency, chemical optimization, and pollution prevention as core environmental performance objectives.

- This integrated approach focuses on reducing water use at source, improving cleaning efficiency, and limiting the discharge of high-impact chemicals, including those contributing to AOX in wastewater.

Water-Efficient Cleaning and Washing Processes

- Alchem International has implemented optimized cleaning and washing practices to reduce excessive water consumption without compromising product quality or workplace hygiene. Traditional high-volume wash practices are replaced, where feasible, with controlled and efficient cleaning methods.

- Key measures include the use of low-flow nozzles, pressure-assisted cleaning equipment, and optimized wash cycles that limit water use to only what is operationally necessary. Cleaning processes are standardized through defined work instructions to avoid prolonged or repeated washing.

- Sequential washing practices are applied so that lightly contaminated rinse water may be reused for initial cleaning stages, reducing overall freshwater demand.

Optimization of Cleaning Process Parameters

- Cleaning processes are designed with defined parameters for time, pressure, temperature, and chemical concentration. Operators follow approved work instructions that prevent over-washing, uncontrolled hose use, or unnecessary re-cleaning.

- Routine monitoring of water consumption helps identify deviations and improvement opportunities, while maintenance checks ensure that valves, hoses, and spray systems remain leak-free and efficient.

Reduction of AOx-Generating Chemicals

- In parallel with water reduction initiatives, Alchem International actively works to minimize AOx load in wastewater by reducing the use of chlorine-based chemicals and sodium chlorate in cleaning, bleaching, or purification-related applications.

- Where technically feasible, alternative low-AOx or non-chlorinated chemicals are used. Chemical selection decisions prioritize environmental compatibility, lower toxicity, and reduced halogen content.

- Controlled chemical dosing systems ensure accurate application, preventing excess chemical discharge during washing or cleaning stages.

Water-Saving Equipment and Technology

- Water-efficient machinery and fixtures are installed in relevant production and cleaning areas. These include controlled spray systems, recirculation-supported wash units, and equipment designed to deliver effective cleaning with reduced water volumes.

- Installation records and equipment manuals are maintained to demonstrate correct setup, operation, and maintenance of water-saving systems.

Monitoring, Reporting, and Operational Control

- Alchem International maintains operational efficiency reports tracking water consumption trends, cleaning process performance, and chemical usage. These records support internal reviews and provide documented evidence for audits and customer assessments.

- Effluent parameters are periodically reviewed to assess the effectiveness of water-saving and low-AOx measures, ensuring alignment with regulatory limits and customer expectations.

Ongoing Continuous Improvement

Alchem International ensures continuous improvement in water stewardship and wastewater management through:

- Quarterly EHS reviews of water usage trends, leak reports, and reuse performance

- Annual third-party audits of water systems and effluent quality

- Regular evaluation of high-recovery RO systems, improved ETP performance, and advanced treatment technologies

- Employee-driven initiatives

- Kaizen programmes, awareness training, and internal communication embed responsible water use into daily operations.

- Data from consumption logs, reuse metrics, leak repair records, and treatment performance is systematically analysed to guide targeted upgrades and long-term sustainability planning.

# SECTION 5: API DISCHARGE REDUCTION & PHARMACEUTICAL WASTEWATER MANAGEMENT

Introduction

Alchem International recognises that the discharge of Active Pharmaceutical Ingredients (APIs), intermediates, and other pharmaceutical residues into wastewater streams represents a significant environmental and public health concern. APIs—including those manufactured by Alchem International such as Digoxin, Atropine, Hyoscine Butyl Bromide, Colchicine, and Paclitaxel—are biologically active compounds designed to produce therapeutic effects in humans and animals. When released into aquatic environments, even at trace concentrations, these compounds can adversely affect aquatic organisms, contaminate drinking water sources, contribute to antimicrobial resistance, and disrupt ecosystem balance.

As a manufacturer and exporter of pharmaceutical APIs, phytochemicals, and nutraceutical ingredients serving regulated markets across 35 countries, Alchem International acknowledges its responsibility to prevent, minimise, and control the discharge of pharmaceutical compounds into wastewater. This responsibility extends beyond regulatory compliance to encompass environmental stewardship, community protection, and alignment with global pharmaceutical industry best practices, including the AMR Industry Alliance framework, the Pharmaceutical Supply Chain Initiative (PSCI), and customer-specific API discharge requirements.

This API Discharge Reduction & Pharmaceutical Wastewater Management Standard Operating Procedure (SOP) establishes Alchem International's structured framework for the identification, characterisation, treatment, and monitoring of pharmaceutical compounds in wastewater. The SOP incorporates advanced treatment technologies—including Advanced Oxidation Processes (AOPs) and membrane filtration—to achieve effective removal of APIs and other pharmaceutical residues before discharge or reuse.

Objectives

The objectives of this SOP are to:

- Prevent and minimise the discharge of APIs, intermediates, and pharmaceutical residues into wastewater streams from all Alchem International manufacturing operations.

- Characterise and quantify API concentrations in process wastewater streams to establish baseline data and treatment requirements.

- Implement advanced treatment technologies—including Advanced Oxidation Processes (AOPs), membrane filtration (nanofiltration, reverse osmosis), and activated carbon adsorption—to achieve effective API removal.

- Establish API-specific discharge limits and monitoring protocols aligned with CPCB/SPCB norms, customer requirements, and international pharmaceutical industry standards.

- Ensure compliance with applicable environmental regulations, including the Water Act (1974), Environment Protection Act (1986), and CPCB guidelines for pharmaceutical industry effluent.

- Protect aquatic ecosystems, drinking water sources, and public health from the adverse effects of pharmaceutical compounds.

- Support Alchem International's sustainability commitments, ESG performance, and customer audit requirements related to pharmaceutical discharge management.

- Promote continuous improvement through research, innovation, and adoption of emerging treatment technologies for pharmaceutical removal.

Applicability & Coverage

This SOP applies to:

All Alchem International manufacturing facilities, production units, and operational areas involved in the synthesis, extraction, isolation, purification, formulation, and handling of APIs, intermediates, and pharmaceutical compounds, including:

- Active Pharmaceutical Ingredient (API) Manufacturing Units

- Botanical Extraction and Purification Units

- Synthesis and Semi-Synthesis Operations

- Quality Control and Analytical Laboratories

- Research and Development Laboratories

- Pilot Plant and Scale-Up Operations

- Effluent Treatment Plant (ETP) and Wastewater Treatment Systems

- Utilities and Equipment Cleaning Areas

- Warehouse and Raw Material Storage Areas

- Packing and Finishing Operations

All wastewater streams generated from Alchem International operations, including:

- Process wastewater from extraction, purification, and synthesis

- Equipment and reactor cleaning wastewater

- Laboratory and analytical waste streams

- Floor washing and general cleaning wastewater

- Cooling tower blowdown and boiler blowdown (where contaminated)

- Stormwater runoff from chemical handling areas

- Domestic wastewater from employee facilities (where mixed with process streams)

- Any other wastewater stream that may contain APIs or pharmaceutical residues

All personnel involved in pharmaceutical manufacturing, wastewater treatment, environmental management, and compliance activities, including:

- Production and Manufacturing Personnel

- EHS and Environmental Management Personnel

- Quality Control and Quality Assurance Personnel

- Engineering and Maintenance Personnel

- Wastewater Treatment Plant Operators

- Research and Development Personnel

- Contractors and Service Providers engaged in wastewater-related activities

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Department | • Develop and implement the API Discharge Reduction & Pharmaceutical Wastewater Management Program. • Conduct wastewater characterisation studies to identify API concentrations and treatment requirements. • Monitor and report API discharge concentrations and removal efficiency. • Coordinate with regulatory authorities (CPCB/SPCB) for compliance reporting. • Maintain documentation of wastewater testing, treatment performance, and compliance records. • Provide training on API discharge prevention, treatment technologies, and spill response. |
| Production / Manufacturing Department | • Implement source-level controls to prevent API entry into wastewater streams. • Optimise production processes to reduce API losses and waste generation. • Segregate high-strength API-containing waste streams at source. • Report any spills, leaks, or unintended API discharges to EHS immediately. • Support wastewater characterisation and treatment optimisation efforts. |
| Engineering / Maintenance Team | • Install, operate, and maintain advanced treatment systems (AOPs, membrane filtration, activated carbon units). • Conduct preventive maintenance of wastewater treatment equipment. • Ensure proper calibration and functioning of treatment system instrumentation. • Support technology upgrades and process modifications for improved API removal. |
| Quality Control / Quality Assurance | • Develop and validate analytical methods for API detection in wastewater. • Conduct regular sampling and testing of wastewater streams for API concentrations. • Verify compliance with API discharge limits and treatment performance standards. • Maintain analytical records and testing documentation. |
| Research & Development | • Evaluate emerging treatment technologies for API removal. • Conduct pilot studies on advanced oxidation processes, membrane filtration, and other treatment options. • Optimise existing treatment processes for improved efficiency and cost-effectiveness. • Collaborate with external technology providers and research institutions. |
| Procurement Team | • Source treatment chemicals, membranes, adsorbents, and equipment required for API removal. • Evaluate suppliers based on environmental performance and technology capability. • Ensure procurement specifications include API removal requirements where applicable. |
| All Personnel | • Follow SOPs and safety instructions to prevent API releases to wastewater. • Report spills, leaks, or improper disposal of API-containing materials immediately. • Participate in training on API discharge prevention and environmental protection. • Support continuous improvement initiatives for wastewater management. |

Sources of API Discharge in Wastewater

API discharge into wastewater at Alchem International may originate from multiple sources across manufacturing operations. The following table identifies key sources and their characteristics:

| Source | Description | Typical API Concentration | Treatment Priority |
| --- | --- | --- | --- |
| Process Wastewater | Wastewater generated during extraction, purification, and synthesis operations | High | High |
| Equipment Cleaning | Wastewater from cleaning reactors, vessels, filters, and processing equipment | High | High |
| Spent Solvents | Solvent-water mixtures from extraction and purification processes | High | High |
| Laboratory Wastewater | Wastewater from analytical testing, quality control, and R&D laboratories | Variable | High |
| Floor Washing | Wastewater from floor cleaning in production and chemical handling areas | Low to Medium | Medium |
| Cooling Tower Blowdown | Blowdown water from cooling systems (may contain trace contaminants) | Low | Low |
| Stormwater Runoff | Runoff from chemical storage and handling areas | Variable | Medium |
| Domestic Wastewater | Wastewater from employee facilities (where mixed with process streams) | Low | Low |
| Filter and Membrane Cleaning | Wastewater from cleaning of filtration and membrane systems | Medium | Medium |
| Spill Cleanup | Wastewater from spill cleanup activities | Variable | High |

API Characterisation and Risk Assessment

1. Wastewater Characterisation Studies

Alchem International conducts periodic wastewater characterisation studies to identify and quantify APIs, intermediates, and pharmaceutical residues present in wastewater streams. These studies include:

- Sampling Protocol: Representative samples are collected from all major wastewater streams, including process wastewater, cleaning wastewater, and combined effluent.

- Analytical Methods: Validated analytical methods—including HPLC, LC-MS/MS, GC-MS, and UV-Vis spectrophotometry—are used to detect and quantify API concentrations.

- Frequency: Characterisation studies are conducted quarterly, or more frequently during process changes, new product introductions, or regulatory requirements.

- Documentation: All characterisation data is documented in the API Wastewater Characterisation Register maintained by the EHS and QC departments.

2. API Risk Assessment

APIs identified in wastewater are assessed for environmental and health risks based on:

- Toxicity: Acute and chronic toxicity to aquatic organisms (fish, daphnia, algae).

- Persistence: Resistance to degradation in the environment.

- Bioaccumulation: Potential to accumulate in aquatic organisms.

- Environmental Concentration: Predicted environmental concentration (PEC) vs. predicted no-effect concentration (PNEC).

- Regulatory Classification: Classification under CPCB/SPCB norms, customer requirements, and international standards.

3. Risk Prioritisation

APIs are prioritised for treatment based on their risk profile:

| Priority Level | Criteria | Examples |
| --- | --- | --- |
| High Priority | High toxicity, high concentration, persistent, bioaccumulative | Cytotoxic APIs, hormones, antibiotics |
| Medium Priority | Moderate toxicity, moderate concentration | Analgesics, anti-inflammatory compounds |
| Low Priority | Low toxicity, low concentration, readily degradable | Some botanical extracts, nutrients |

Advanced Treatment Technologies for API Removal

Alchem International employs a multi-barrier treatment approach for API removal, combining multiple technologies to achieve effective removal of pharmaceutical compounds. The following sections describe the key treatment technologies implemented.

1. Advanced Oxidation Processes (AOPs)

Advanced Oxidation Processes (AOPs) are chemical treatment technologies that generate highly reactive hydroxyl radicals (•OH) to oxidise and degrade pharmaceutical compounds. AOPs are particularly effective for recalcitrant APIs that resist conventional biological treatment.

Types of AOPs Implemented

- Fenton and Photo-Fenton Processes: Iron-catalysed hydrogen peroxide oxidation for degradation of APIs. Effective for a wide range of pharmaceutical compounds.

- Ozone-Based AOPs (O₃/H₂O₂, O₃/UV): Ozone combined with hydrogen peroxide or UV light for enhanced oxidation of APIs.

- UV/Hydrogen Peroxide: UV light activation of hydrogen peroxide for hydroxyl radical generation.

- Photocatalytic Oxidation: Titanium dioxide (TiO₂) or other photocatalysts activated by UV light for API degradation.

- Electrochemical Oxidation: Electrochemical generation of oxidants for API removal.

- Wet Air Oxidation (WAO): High-temperature, high-pressure oxidation for high-strength API waste streams.

AOP System Design and Operation

- AOP systems are designed based on wastewater characteristics, API types, and treatment objectives.

- Reaction conditions (pH, temperature, oxidant dosage, residence time) are optimised for maximum API removal.

- AOP systems are integrated with pre-treatment (filtration, pH adjustment) and post-treatment (biological treatment, polishing) for comprehensive treatment.

- Performance is monitored through API concentration measurement, COD/TOC reduction, and toxicity testing.

Performance Monitoring

- AOP system performance is monitored through:

  - API concentration reduction (influent vs. effluent)

  - COD and TOC reduction

  - Hydroxyl radical generation efficiency

  - Oxidant consumption and cost

  - By-product formation and toxicity

2. Membrane Filtration Technologies

Membrane filtration technologies physically separate pharmaceutical compounds from wastewater based on molecular size and charge. These technologies are highly effective for API removal and can be integrated with other treatment processes.

Types of Membrane Filtration Implemented

- Reverse Osmosis (RO): High-pressure membrane process for removal of dissolved APIs and salts. Effective for a wide range of pharmaceutical compounds.

- Nanofiltration (NF): Membrane process for removal of organic compounds and multivalent ions. Effective for APIs with molecular weight > 200 Da.

- Ultrafiltration (UF): Membrane process for removal of macromolecules, colloids, and suspended solids. Used as pre-treatment for RO/NF.

- Microfiltration (MF): Membrane process for removal of suspended solids and bacteria. Used as pre-treatment.

- Forward Osmosis (FO): Emerging membrane technology for API concentration and removal.

- Membrane Bioreactor (MBR): Combination of biological treatment and membrane filtration for enhanced API removal.

Membrane System Design and Operation

- Membrane systems are designed based on wastewater flow rate, API characteristics, and treatment objectives.

- Pre-treatment (filtration, pH adjustment, anti-scalant dosing) is provided to protect membranes and extend membrane life.

- Membrane cleaning and maintenance schedules are established to maintain performance.

- Concentrate (reject) streams are managed through authorised disposal or further treatment.

Performance Monitoring

- Membrane system performance is monitored through:

  - API rejection rate (influent vs. permeate)

  - Permeate flux and recovery rate

  - Transmembrane pressure (TMP)

  - Membrane fouling and cleaning frequency

  - Concentrate management and disposal

3. Activated Carbon Adsorption

Activated carbon adsorption is a proven technology for removal of organic compounds, including APIs, from wastewater. Activated carbon has a high surface area and affinity for organic molecules.

Types of Activated Carbon Systems Implemented

- Granular Activated Carbon (GAC) Columns: Fixed-bed columns for continuous treatment of wastewater.

- Powdered Activated Carbon (PAC) Addition: Addition of PAC to wastewater for API adsorption, followed by separation.

- Biological Activated Carbon (BAC): Combination of adsorption and biological degradation for enhanced API removal.

System Design and Operation

- Activated carbon systems are designed based on wastewater characteristics, API concentrations, and treatment objectives.

- Carbon selection (type, particle size, surface area) is based on API characteristics and treatment requirements.

- Carbon replacement and regeneration schedules are established to maintain performance.

- Spent carbon is managed as hazardous waste or regenerated through authorised facilities.

Performance Monitoring

- Activated carbon system performance is monitored through:

  - API removal efficiency (influent vs. effluent)

  - Carbon exhaustion rate and replacement frequency

  - Pressure drop and flow rate

  - Breakthrough curve analysis

4. Biological Treatment Enhancement

Biological treatment—including activated sludge, trickling filters, and membrane bioreactors—can be enhanced for improved API removal through:

- Acclimation of Biomass: Gradual exposure of biomass to APIs to develop degradation capability.

- Nutrient Supplementation: Addition of nutrients to support microbial growth and API degradation.

- Co-metabolism: Addition of primary substrates to enhance API degradation.

- Bioaugmentation: Addition of specialised microbial cultures for API degradation.

- Extended Solids Retention Time (SRT): Longer SRT for improved degradation of recalcitrant compounds.

5. Integrated Treatment Train

Alchem International employs an integrated treatment train combining multiple technologies for comprehensive API removal:

| Treatment Stage | Technology | Purpose |
| --- | --- | --- |
| Pre-treatment | Screening, equalisation, pH adjustment | Remove solids, homogenise flow, optimise pH |
| Primary Treatment | Chemical coagulation, sedimentation | Remove suspended solids, some APIs |
| Secondary Treatment | Activated sludge, MBR | Biological degradation of APIs |
| Tertiary Treatment | AOPs, membrane filtration, activated carbon | Advanced removal of recalcitrant APIs |
| Polishing | UV disinfection, final filtration | Final polishing before discharge/reuse |
| Sludge Treatment | Dewatering, drying, disposal | Manage treatment residuals |

API Discharge Limits and Monitoring

1. Regulatory Discharge Limits

Alchem International complies with applicable API discharge limits prescribed by:

- CPCB/SPCB Norms: Central and State Pollution Control Board standards for pharmaceutical industry effluent.

- Environment Protection Act (1986): General standards for discharge of environmental pollutants.

- Water Act (1974): Standards for discharge of sewage and industrial waste.

- Customer Requirements: Buyer-specific API discharge limits and reporting requirements.

- International Standards: WHO guidelines, EU pharmaceutical effluent standards, and other applicable international benchmarks.

2. Internal Discharge Limits

Alchem International establishes internal API discharge limits that are more stringent than regulatory requirements, reflecting the company's commitment to environmental protection:

| API Category | Internal Limit (mg/L) | Regulatory Limit (mg/L) | Monitoring Frequency |
| --- | --- | --- | --- |
| High Priority APIs | 0.01 | 0.1 | Daily |
| Medium Priority APIs | 0.1 | 1.0 | Weekly |
| Low Priority APIs | 1.0 | 10.0 | Monthly |
| Total API Load | 5.0 | 50.0 | Daily |

3. Monitoring Protocol

- Sampling Points: Wastewater samples are collected at:

  - Individual process streams (before mixing)

  - Combined effluent (before treatment)

  - Treated effluent (after treatment, before discharge)

  - Final discharge point

- Sampling Frequency: Based on API priority level and regulatory requirements.

- Analytical Methods: Validated methods (HPLC, LC-MS/MS, GC-MS) with appropriate detection limits.

- Documentation: All monitoring data is recorded in the API Discharge Monitoring Register maintained by EHS and QC.

- Reporting: Monthly and annual reports on API discharge performance are prepared for management review and regulatory submission.

Source-Level Controls for API Discharge Prevention

Alchem International implements source-level controls to prevent APIs from entering wastewater streams:

1. Process Optimisation

- Yield Improvement: Optimise extraction, purification, and synthesis processes to maximise API recovery and minimise losses to wastewater.

- Solvent Recovery: Recover and reuse solvents to reduce API-containing waste streams.

- Catalyst Recovery: Recover and reuse catalysts to reduce API losses.

- Water Reduction: Minimise water use in processes to reduce wastewater volume and API load.

2. Segregation of High-Strength Streams

- Source Segregation: Segregate high-strength API-containing waste streams from low-strength streams.

- Dedicated Collection: Collect high-strength streams in dedicated containers for separate treatment.

- Pre-treatment: Pre-treat high-strength streams before mixing with general effluent.

3. Spill Prevention and Response

- Secondary Containment: Provide secondary containment for chemical storage and handling areas.

- Spill Kits: Maintain spill kits at strategic locations for immediate response.

- Training: Train personnel on spill prevention and response procedures.

- Reporting: Report all spills and near-misses to EHS for investigation and corrective action.

4. Equipment Cleaning Optimisation

- Validated Cleaning Procedures: Use validated cleaning procedures to minimise water and chemical use.

- Cleaning Solution Recovery: Recover and reuse cleaning solutions where feasible.

- Rinse Water Reuse: Reuse final rinse water for initial cleaning stages.

5. Stormwater Management

- Diversion: Divert stormwater away from chemical handling and storage areas.

- Treatment: Treat contaminated stormwater before discharge.

- Monitoring: Monitor stormwater for API contamination.

Documentation and Recordkeeping

Alchem International maintains comprehensive documentation related to API discharge reduction and pharmaceutical wastewater management:

- API Wastewater Characterisation Reports: Results of wastewater characterisation studies.

- API Risk Assessment Records: Documentation of API risk assessment and prioritisation.

- Treatment System Performance Records: Monitoring data for AOPs, membrane filtration, activated carbon, and biological treatment.

- API Discharge Monitoring Records: Sampling and analytical data for API concentrations.

- Regulatory Compliance Reports: Reports submitted to CPCB/SPCB and other authorities.

- Customer Reporting Records: API discharge reports provided to customers.

- Spill and Incident Reports: Documentation of spills, leaks, and corrective actions.

- Training Records: Documentation of personnel training on API discharge prevention and treatment.

All records are retained for a minimum of five years, or longer where required by regulations, customer requirements, or Alchem International's internal data-retention policy.

Continuous Improvement

Alchem International fosters a strong culture of continuous improvement in API discharge reduction and pharmaceutical wastewater management through:

- Regular Performance Reviews: Monthly and annual reviews of API discharge performance, treatment efficiency, and compliance status.

- Technology Evaluation: Ongoing evaluation of emerging treatment technologies for API removal.

- Pilot Studies: Conducting pilot studies on new treatment technologies before full-scale implementation.

- Research Collaboration: Collaborating with technology providers, research institutions, and industry experts.

- Employee Engagement: Encouraging employee suggestions and Kaizen initiatives for API discharge reduction.

- Benchmarking: Benchmarking against pharmaceutical industry best practices and customer expectations.

- Training and Awareness: Regular training on API discharge prevention, treatment technologies, and environmental protection.

Regulatory Compliance

All API discharge reduction and pharmaceutical wastewater management systems at Alchem International are designed, implemented, and reviewed to meet or exceed:

- Applicable Indian Environmental Laws: Water Act (1974), Environment Protection Act (1986), CPCB/SPCB norms, and other applicable regulations.

- International Standards: WHO guidelines, EU pharmaceutical effluent standards, and other applicable international benchmarks.

- Customer Requirements: Buyer-specific API discharge limits, reporting requirements, and audit expectations.

- Industry Best Practices: AMR Industry Alliance framework, PSCI requirements, and other industry initiatives.

Dedicated EHS personnel:

- Monitor regulatory updates and evolving compliance obligations.

- Update SOPs, training modules, and operational controls accordingly.

- Engage with accredited laboratories, authorised vendors, consultants, and authorities to maintain high standards of safety, compliance, and sustainability.

# SECTION 6: AIR EMISSIONS COMPLIANCE AND COMMUNITY IMPACT PREVENTION

Introduction

Alchem International recognises that maintaining clean air within and around its manufacturing facilities is a critical element of environmental stewardship, worker health and safety, and responsible community engagement. As a pharmaceutical and phytochemical manufacturer, Alchem International operates a range of activities—including extraction, purification, synthesis, utilities, and material handling—that can generate air emissions such as dust, particulate matter, volatile organic compounds (VOCs), chemical vapours, combustion gases, odours, and noise if not properly managed.

This Standard Operating Procedure (SOP) defines the preventive strategies, engineering controls, monitoring systems, and management practices adopted by Alchem International to control atmospheric emissions, process-generated vapours, workplace dust, odours, and noise. These measures are designed to ensure compliance with applicable environmental and occupational health regulations—including the Air (Prevention and Control of Pollution) Act, 1981, and CPCB/SPCB norms—meet international customer expectations, and protect employees, neighbouring communities, and the surrounding environment.

Objectives

The objectives of this SOP are to:

- Prevent and minimise emissions of VOCs, particulate matter, chemical vapours, and combustion gases from production and utility operations.

- Monitor indoor air quality, process emissions, and ambient air quality at regular intervals and take prompt corrective action in case of deviations.

- Minimise workplace and boundary-level noise generated from utilities and production machinery through engineering and administrative controls.

- Control and mitigate nuisance odours arising from extraction, purification, chemical storage, wastewater treatment, and waste handling activities.

- Maintain full compliance with applicable environmental regulations, occupational hygiene standards, and customer environmental and social responsibility requirements relevant to the pharmaceutical and phytochemical industry.

Applicability & Coverage

This SOP applies to:

- All manufacturing operations involving chemical usage, extraction, purification, synthesis, heating, drying, ventilation, or exhaust systems.

- All personnel involved in production activities, operation of fume hoods and local exhaust ventilation (LEV), chemical handling, material transfer, and waste handling.

- All noise- and emission-generating equipment, including utilities (boilers, generators, compressors, chillers, cooling towers), production machinery, dust extraction systems, and exhaust stacks at Alchem International facilities.

Organizational Responsibilities

| Role | Key Responsibilities |
| --- | --- |
| EHS Coordinator / Environmental Manager | Develop and implement air emission and noise monitoring programmes; assess emission risks; consolidate monitoring data; ensure compliance with statutory limits and customer requirements; and drive corrective and preventive actions. |
| Production Managers / Line Supervisors | Enforce air-handling and ventilation protocols; ensure proper use of LEV systems and fume extraction units; and implement emission control SOPs at shop-floor level. |
| Maintenance Supervisor / Utilities Head | Maintain air pollution control equipment, exhaust systems, scrubbers, filters, silencers, and noise insulation; ensure operational uptime; and conduct calibration and preventive maintenance. |
| Production Operators & Technicians | Use ventilation, dust extraction, and odour control systems correctly; follow SOPs for chemical handling; and report abnormal emissions, odours, excessive dust, or noise to supervisors and EHS. |

Emission Prevention Actions

- Multi-Stage Wet Scrubbers: Installed on exhausts from extraction, purification, and utility operations where acidic or water-soluble vapours may be generated. Scrubbers are designed to neutralise odorous compounds and chemical vapours. Scrubber performance is verified through routine monitoring of pH, conductivity, and total dissolved solids (TDS) of recirculating scrubbing liquor.

- Closed and Enclosed Process Operations: Chemical preparation, mixing, and transfer activities are carried out in enclosed or semi-enclosed systems wherever feasible. Containers are kept sealed, and vapours are routed to exhaust and treatment systems to minimise fugitive emissions.

- Local Exhaust Ventilation (LEV) Systems: LEV systems are installed at high-risk points such as reactors, distillation units, chemical dosing areas, and maintenance workstations. These systems capture vapours and dust at source and discharge them through filtration or scrubbing units.

- Vapour Adsorption Systems: Activated carbon-based adsorption units are deployed in selected high-VOC areas such as solvent storage rooms and chemical handling zones. Carbon media is monitored for breakthrough and replaced or regenerated as per defined schedules.

- Leak Detection and Repair (LDAR): Routine inspections are conducted to identify leaks from pipelines, valves, pumps, chemical containers, and exhaust ducts. Detected leaks or abnormal odours are logged, investigated, and rectified within defined corrective action timelines.

- Combustion Emission Controls: Emissions from boilers, generators, and thermic fluid heaters are controlled through proper burner design, fuel quality control, optimised combustion settings, and regular tuning to minimise NOx, SOx, CO, and particulate emissions.

- Stack Design and Monitoring Readiness: Utility and process stacks are designed in accordance with regulatory guidelines for height and exit velocity to ensure effective dispersion. Sampling ports and safe access platforms are provided for stack emission monitoring.

- Continuous and Periodic Emission Monitoring: Where applicable, continuous or periodic monitoring is carried out for parameters such as particulate matter, SOx, NOx, CO, and VOCs. Alert thresholds trigger immediate investigation and corrective action if values approach permissible limits.

- Preventive Maintenance of Air Pollution Control (APC) Equipment: Scrubbers, filters, fans, ducts, silencers, and chimneys are included in preventive maintenance schedules. Activities include inspection of duct integrity, filter replacement, fan balancing, demister cleaning, and verification of airflow rates.

- Ambient Air Quality Monitoring (AAQM): Ambient air quality monitoring is conducted at designated upwind and downwind boundary locations to assess PM2.5, PM10, VOCs, NOx, SOx, and other relevant parameters. Trends are reviewed periodically and benchmarked against applicable ambient air quality standards to ensure negligible off-site impact.

Control of Particulate Matter

At Alchem International, particulate matter (PM) generated from pharmaceutical manufacturing, material handling, utilities, and material movement is managed through layered engineering controls, real-time monitoring, and preventive maintenance to protect worker health, ensure regulatory compliance, and prevent off-site impacts.

- Baghouse Dust Collectors with HEPA-Grade Filters: Baghouse systems fitted with high-efficiency (HEPA-grade) filters are installed on particulate-generating equipment such as dryers, mills, blenders, and material handling points where fine powders and dust are generated. These systems provide filtration efficiency exceeding 99.9% for fine particulates, maintaining clean work environments and controlling stack emissions.

- Wet Scrubbing Systems with Packed Bed Columns: Wet scrubbers are connected to exhausts from extraction, purification, and chemical preparation areas where mist, aerosols, or particulate-bound vapours may be generated. Scrubber effectiveness is assessed through pressure drop monitoring, pH checks, conductivity/TDS trends, and visual inspections of packing media and spray distribution.

- Electrostatic Precipitators (ESPs): ESPs are deployed on boiler stacks and other high-temperature exhaust sources to remove sub-micron particulate matter (< 1 μm). ESP performance indicators—including inlet/outlet dust load, voltage (kV), spark rate, and plate condition—are reviewed and documented during quarterly EHS and engineering audits.

- Continuous Particulate Monitoring: PM10 and PM2.5 sensors are installed at critical exhaust ducts and selected ambient monitoring locations. These sensors are integrated with alarm-linked control panels that alert operators to abnormal emission spikes, triggering immediate investigation, load reduction, or controlled shutdown actions.

- Preventive Maintenance & Calibration: All particulate control systems follow defined maintenance schedules, including:

  - Daily or shift-wise pulse-jet cleaning (where applicable)

  - Weekly filter and hopper inspections

  - Periodic fan, motor, damper, and duct integrity checks

  - Calibration of differential pressure gauges and sensors to ensure sustained capture efficiency

CO, SO₂, NOₓ and Heavy Metal Emission Mitigation

- CEMS on Critical Stacks: Continuous Emission Monitoring Systems (CEMS) are installed on major stacks associated with boilers, generators, thermic fluid heaters, and key exhaust systems. Parameters monitored include SO₂, NOₓ, CO, CO₂, particulate matter, and VOCs where applicable.

- Ambient Air Quality Monitoring Stations (AAQMS): AAQMS units are installed at strategic perimeter locations to monitor PM10, PM2.5, NOₓ, SOₓ, CO, and VOCs. The EHS team reviews monitoring data weekly or monthly, depending on operational risk levels.

- Third-Party Manual Stack Testing: Manual stack emission testing is conducted at least quarterly by accredited third-party laboratories (NABL/MoEFCC-approved). Parameters tested include SO₂, NOₓ, CO, particulate matter, and total VOCs, with results certified and retained for compliance reporting.

- Real-Time Data Analytics & Alarming: Emission and ambient air data are consolidated into an Environmental Data Management System. Analytics tools evaluate trends, detect anomalies, and auto-generate alarms when values approach defined action thresholds, enabling prompt corrective measures.

- VOC Capture and Mitigation: VOC emissions from chemical storage, extraction, purification, and maintenance activities are minimised through:

  - Vapour balancing and return lines

  - Enclosed chemical transfer systems

  - Activated carbon adsorption units

  - Optimised ventilation and exhaust capture

Operational parameters are adjusted based on monitoring data to further reduce fugitive releases.

Closed-Circuit Sulphur Control Measures

- Sulphur-emitting processes are fully enclosed using sealed reactors, ducts, and processing chambers to prevent fugitive emissions.

- Exhaust air from processing units is routed through sulphur absorption and recovery equipment rather than being released directly into the atmosphere.

- The closed-loop system allows sulphur vapors to be captured, neutralized, and recovered for reuse or safe disposal.

- Negative pressure is maintained within enclosed areas to prevent leakage into the work environment.

Sulphur Recovery and Absorption Technology

- Dedicated emission control equipment is installed to absorb sulphur compounds through chemical scrubbing or adsorption media.

- Recovered sulphur or sulphur-containing solutions are either reused in controlled processes or managed as per approved waste handling procedures.

- The system operates continuously during sulphur-related production activities to ensure uninterrupted emission control.

Operational Control and Efficiency

- Standard operating procedures define start-up, operation, monitoring, and shutdown of sulphur recovery systems.

- Emission control equipment performance is monitored through operational efficiency checks and process parameters.

- Process optimization initiatives are implemented to reduce sulphur losses and improve recovery rates.

Maintenance and Inspection

- Preventive maintenance schedules are followed to ensure integrity of enclosures, ducts, scrubbers, and recovery units.

- Regular inspections are conducted to detect leaks, corrosion, or performance degradation.

- Maintenance records are maintained to demonstrate system reliability and continuous operation.

Environmental and Occupational Protection

- The closed-circuit design significantly reduces sulphur emissions to ambient air, improving air quality within and outside the facility.

- Employee exposure to sulphur fumes is minimized through engineering controls rather than reliance on PPE alone.

- Emergency response procedures are in place for any abnormal emission or system failure.

Routine Calibration & Equipment Maintenance

- CEMS and AAQMS systems are calibrated monthly or as per OEM guidance using certified calibration gases.

- Preventive maintenance is conducted by trained technicians, with calibration and service records maintained under the instrumentation management system.

Monthly Compliance Review

- Emission trends, alarms, and any exceedances are reviewed by a cross-functional environmental management team.

- Root cause analyses, corrective actions, and improvement projects are tracked through a central Environmental Performance Dashboard.

Regulatory Reporting & Disclosure

- Annual emission reports covering stack data, APC performance, CEMS logs, and monitoring summaries are submitted to regulatory authorities (CPCB/SPCB) as required.

- Reports include pollutant loads (PM, SOₓ, NOₓ, CO, VOCs) and implemented control measures.

Odor Mitigation and Biofiltration

- Chemical and Solvent Storage Ventilation: Chemical and auxiliary storage areas are equipped with dedicated exhaust systems designed for adequate air changes per hour (ACH) to prevent vapour and odour buildup.

- Biofiltration Systems: Compact biofilter units using activated media and microbial biofilms are installed near odor-prone zones such as wastewater treatment areas, chemical handling zones, and waste storage points.

- Passive Odor Control: Solid or gel-based neutralisers are placed at identified hotspots (only where safe and compliant) to manage localised odours.

- Air Curtains and Zoning: Air curtains and pressure zoning are used at entrances to prevent cross-migration of odours between production, utility, and administrative areas.

- Preventive Maintenance: Exhaust fans, filters, scrubbers, and biofilters are maintained as per schedule, including airflow calibration, media replacement, and performance verification.

Ambient Air and Emission Monitoring

Monitoring Schedule

- Ambient air quality is monitored continuously or at defined intervals at facility boundaries and critical internal locations.

- Third-party manual monitoring is conducted at least monthly or as required.

- During high-risk periods (seasonal inversions, peak production, major shutdowns/start-ups), monitoring frequency is increased.

Pollutant Evaluation and Compliance

- All monitoring data is analysed by the Environmental Monitoring Team and benchmarked against applicable national ambient air quality standards (NAAQS) and customer expectations.

- Abnormal results trigger root cause analysis and targeted mitigation actions.

Noise Monitoring and Control

1. Routine Noise Monitoring and Mapping

Periodic noise surveys are conducted across production floors, utilities, compressor rooms, boiler houses, cooling towers, and boundary areas. Results are benchmarked against occupational exposure limits (e.g., 85 dB(A) over 8-hour TWA) and ambient noise standards (CPCB). Noise contour maps are prepared to identify high-noise zones.

2. Engineering Controls

- Vibration isolation for compressors, pumps, blowers, and rotating equipment

- Acoustic enclosures and barriers providing 10-25 dB noise reduction

- Use of acoustic panels and baffles in high-noise areas

3. Administrative Controls

- Scheduling of noise-intensive operations to minimise worker and community impact

- Zoning of utilities away from offices and sensitive areas

4. Personal Protective Equipment

- Provision of earplugs or earmuffs in high-noise zones

- Clear signage indicating mandatory hearing protection

- PPE selected for comfort and compatibility with other safety gear

5. Preventive Maintenance

- Routine lubrication, balancing, and alignment

- Inspection of panels, guards, and fasteners to prevent rattling and vibration

6. Employee Health & Feedback

- Audiometric testing for exposed employees where required

- Simple reporting mechanisms for abnormal noise

- Regular training on noise risks and PPE use

Equipment Maintenance and Air Control Systems

| System | Frequency | Activity |
| --- | --- | --- |
| Fume hoods & carbon filters | Monthly | Airflow testing, sash checks, filter inspection/replacement |
| Ventilation ductwork & exhaust lines | Quarterly | Blockage, corrosion, leak inspections |
| HEPA/ULPA filters (critical areas) | Every 6 months or as validated | Pressure drop checks, integrity testing |
| Odor scrubbers & biofilters | Every 2 months | Media condition review, replacement |
| Noise insulation systems | Annually | Acoustic integrity inspection and repair |

Documentation

Alchem International maintains detailed documentation including:

- Air quality and emission monitoring reports

- Noise survey results

- Maintenance and calibration records for APC systems

- Incident and grievance logs related to emissions or noise

- Filter change logs and equipment service records

Continuous Improvement

- Regular training for production and maintenance teams on emission prevention and ventilation practices

- Evaluation of low-VOC chemicals and auxiliaries

- Progressive implementation of real-time dashboards for air and noise performance

# SECTION 7: ODOR CONTROL & FILTRATION MANAGEMENT SYSTEM

Introduction

Alchem International recognises that odour emissions from pharmaceutical, phytochemical, and nutraceutical manufacturing operations can cause nuisance, discomfort, and complaints from neighbouring communities, employees, and other stakeholders. Odours may arise from various sources including botanical raw material processing, solvent handling, extraction and purification operations, effluent treatment, waste storage, and chemical reactions. These odours, if not properly controlled, can impact community relations, regulatory compliance, and the company's social licence to operate.

This Odor Control & Filtration Management System Standard Operating Procedure (SOP) establishes Alchem International's structured framework for the identification, prevention, capture, treatment, and monitoring of odorous emissions from all manufacturing and support operations. The SOP focuses on the installation, maintenance, and replacement of odour-minimizing filters and associated air pollution control equipment to prevent the release of odours to the company's premises and surrounding environment.

Aligned with applicable environmental regulations in India—including the Air (Prevention and Control of Pollution) Act, 1981, CPCB/SPCB guidelines on odour control, and the Environment Protection Act, 1986—as well as international best practices and customer expectations, this SOP ensures that odour emissions are managed responsibly and effectively.

Objectives

The objectives of this Odor Control & Filtration Management System SOP are to:

- Identify all significant odour sources across Alchem International manufacturing facilities and operations.

- Prevent and minimise the generation of odorous compounds through process optimisation, material substitution, and operational controls.

- Capture and treat odorous emissions at source using appropriate filtration, scrubbing, and adsorption technologies.

- Install, maintain, and replace odour-minimizing filters on a regular and defined schedule to ensure continuous effective odour control.

- Monitor odour emissions and ambient odour levels at facility boundaries and sensitive receptor locations.

- Ensure compliance with applicable odour regulations, CPCB/SPCB norms, and customer requirements.

- Respond promptly to odour complaints and incidents through investigation and corrective action.

- Protect the health, safety, and comfort of employees, neighbouring communities, and other stakeholders.

Applicability & Coverage

This SOP applies to:

All Alchem International manufacturing facilities, production units, and operational areas that may generate odorous emissions, including:

- Botanical Raw Material Handling and Storage Areas

- Extraction, Isolation, and Purification Units

- Synthesis and Semi-Synthesis Operations

- Solvent Storage and Handling Areas

- Chemical Storage and Mixing Areas

- Effluent Treatment Plant (ETP) and Wastewater Treatment Systems

- Solid Waste Storage and Handling Areas

- Boiler and Utility Areas

- Drying and Finishing Operations

- Quality Control and Analytical Laboratories

- Research and Development Laboratories

- Packing and Warehousing Areas

- Any other area where odorous compounds may be generated or released

All personnel involved in operations that may generate odours, including:

- Production and Manufacturing Personnel

- EHS and Environmental Management Personnel

- Maintenance and Engineering Personnel

- Wastewater Treatment Plant Operators

- Waste Handlers and Housekeeping Staff

- Contractors and Service Providers

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Department | • Identify and classify all odour sources across Alchem International facilities. • Develop and implement odour control and monitoring programmes. • Conduct regular odour monitoring at facility boundaries and sensitive receptor locations. • Investigate odour complaints and incidents, and implement corrective actions. • Maintain documentation of odour control equipment, monitoring data, and complaints. • Provide training on odour prevention, control technologies, and complaint response. |
| Production / Manufacturing Department | • Implement process controls to minimise odour generation. • Ensure odour capture and treatment systems are operated correctly. • Report any abnormal odours or equipment malfunctions to EHS immediately. • Support odour source identification and control efforts. |
| Engineering / Maintenance Team | • Install, operate, and maintain odour control equipment including filters, scrubbers, biofilters, and adsorption units. • Conduct preventive maintenance and filter replacement as per defined schedules. • Ensure proper calibration and functioning of odour control system instrumentation. • Maintain records of maintenance activities, filter changes, and equipment performance. |
| Procurement Team | • Source odour control filters, media, chemicals, and equipment as per specifications. • Evaluate suppliers based on performance, reliability, and environmental compliance. • Ensure timely availability of replacement filters and media. |
| HR / Administration | • Coordinate community engagement and grievance redressal related to odour issues. • Support training and awareness programmes on odour control. • Maintain records of community complaints and responses. |
| All Personnel | • Follow SOPs and operational controls to prevent odour generation. • Report any odour observations, equipment malfunctions, or concerns to supervisors. • Participate in training on odour prevention and control. • Support continuous improvement initiatives for odour management. |

Sources of Odour Emissions

Odour emissions at Alchem International may originate from various sources across manufacturing operations. The following table identifies key odour sources, their characteristics, and associated control measures:

| Odour Source | Description | Typical Odour Characteristics | Control Measures |
| --- | --- | --- | --- |
| Botanical Raw Material Processing | Odours from grinding, milling, and processing of botanical materials | Earthy, musty, pungent | Enclosed processing, LEV, biofilters |
| Solvent Handling and Storage | Vapours from solvents used in extraction and purification | Chemical, solvent-like | Closed systems, vapour recovery, activated carbon |
| Extraction and Purification | Odours from extraction, distillation, and purification operations | Variable, chemical, organic | Enclosed reactors, condensers, scrubbers |
| Synthesis Operations | Odours from chemical reactions and synthesis processes | Chemical, pungent, sulphurous | Enclosed reactors, scrubbers, thermal oxidisers |
| Effluent Treatment Plant (ETP) | Odours from wastewater treatment processes | H₂S (rotten eggs), ammonia, organic | Covered tanks, biofilters, chemical scrubbing |
| Waste Storage and Handling | Odours from solid and hazardous waste storage | Decomposing, chemical, pungent | Enclosed storage, ventilation, biofilters |
| Boiler and Utility Operations | Odours from fuel combustion and utility operations | Combustion, exhaust | Proper combustion, stack dispersion |
| Drying and Finishing | Odours from drying and finishing operations | Organic, chemical | Enclosed dryers, exhaust treatment |
| Chemical Storage and Mixing | Odours from chemical storage and mixing areas | Chemical, pungent | Ventilation, scrubbers, activated carbon |
| Laboratory Operations | Odours from analytical and R&D laboratories | Chemical, solvent-like | Fume hoods, LEV, activated carbon |

Odour Prevention and Source Controls

Alchem International implements source-level controls to prevent and minimise odour generation at the point of origin:

1. Process Optimisation

- Reaction Parameter Optimisation: Optimise temperature, pressure, pH, and reaction time to minimise odour generation.

- Raw Material Selection: Select raw materials with lower odour potential where feasible.

- Solvent Selection: Prefer low-odour, low-VOC solvents where technically feasible.

- Process Enclosure: Enclose odour-generating processes in closed reactors, vessels, and systems.

- Condensation and Recovery: Use condensers and recovery systems to capture volatile compounds before release.

2. Material Substitution

- Replace high-odour chemicals with lower-odour alternatives where technically and economically feasible.

- Use water-based or low-VOC formulations where possible.

- Avoid use of sulphur-containing compounds where alternatives exist.

- Evaluate odour potential during chemical procurement and substitution decisions.

3. Operational Controls

- Good Housekeeping: Maintain clean production areas to prevent accumulation of odorous materials.

- Prompt Waste Removal: Remove odorous waste from production areas promptly.

- Spill Prevention: Prevent and clean up spills immediately to avoid odour generation.

- Equipment Maintenance: Maintain equipment to prevent leaks and odour releases.

- Ventilation Management: Ensure proper ventilation and air changes in odour-prone areas.

4. Enclosure and Containment

- Enclose odour-generating equipment and processes in closed systems.

- Use local exhaust ventilation (LEV) to capture odours at source.

- Cover open tanks, vessels, and containers to prevent odour release.

- Maintain negative pressure in enclosed areas to prevent fugitive emissions.

Odour Capture and Treatment Technologies

Alchem International employs a multi-barrier approach for odour capture and treatment, combining multiple technologies based on odour source characteristics and treatment requirements.

1. Local Exhaust Ventilation (LEV) Systems

LEV systems are installed at odour-generating points to capture odours at source and prevent their release into the workplace and environment.

Design and Installation

- LEV systems are designed based on odour source characteristics, capture velocity requirements, and airflow needs.

- Hoods, ducts, and fans are sized and positioned to ensure effective capture of odorous emissions.

- LEV systems are integrated with odour treatment equipment (filters, scrubbers, biofilters) for comprehensive control.

Operation and Maintenance

- LEV systems are operated continuously during odour-generating activities.

- Airflow and capture efficiency are verified periodically.

- Filters, ducts, and fans are inspected and maintained as per defined schedules.

- Performance issues are addressed promptly to prevent odour release.

2. Odour-Minimizing Filters

Odour-minimizing filters are installed in LEV systems, exhaust ducts, and air handling units to remove odorous compounds from air streams before release. These filters are maintained and replaced on a regular basis to ensure continuous effective odour control.

Types of Odour-Minimizing Filters

- Activated Carbon Filters: Activated carbon is highly effective for adsorption of a wide range of odorous organic compounds, including solvents, VOCs, and sulphur compounds.

- Activated Carbon Impregnated with Chemicals: Chemically impregnated activated carbon (e.g., with potassium hydroxide, potassium iodide, or sulphur) is used for enhanced removal of specific odorous compounds such as H₂S, ammonia, and amines.

- HEPA Filters: High-efficiency particulate air (HEPA) filters remove particulate-bound odours and fine dust.

- ULPA Filters: Ultra-low penetration air (ULPA) filters provide higher filtration efficiency for fine particulates and odours.

- Zeolite Filters: Zeolites are used for adsorption of specific odorous compounds, including ammonia and volatile organic compounds.

- Biofilters: Biofilters use microbial action to degrade odorous compounds and are effective for organic odours.

Filter Installation

- Odour-minimizing filters are installed at all identified odour sources, including LEV exhausts, stack emissions, and air handling units serving odour-prone areas.

- Filters are selected based on odour characteristics, airflow rate, and treatment objectives.

- Filters are installed in appropriately designed housings with proper sealing to prevent bypass.

- Installation records are maintained, including filter type, size, location, and installation date.

Filter Maintenance and Replacement

- Regular Inspection: Odour-minimizing filters are inspected at defined intervals (e.g., weekly, monthly) to check for:

  - Physical damage, clogging, or saturation

  - Pressure drop across the filter

  - Airflow and capture efficiency

  - Sign of odour breakthrough

- Performance Monitoring: Filter performance is monitored through:

  - Pressure drop measurements

  - Odour detection at exhaust outlets

  - Regular odour monitoring at facility boundaries

  - Analytical testing for specific odorous compounds

- Filter Replacement Schedule: Filters are replaced on a defined schedule based on:

  - Manufacturer recommendations

  - Operating hours

  - Pressure drop thresholds

  - Odour breakthrough detection

  - Regular inspection findings

- Replacement Frequency: As a general guideline:

  - Activated carbon filters: Every 3–6 months or when breakthrough is detected

  - HEPA/ULPA filters: Every 6–12 months or when pressure drop exceeds limits

  - Zeolite filters: Every 6–12 months or as per manufacturer recommendations

  - Biofilter media: Every 12–24 months or when performance degrades

- Documentation: Filter replacement activities are documented, including:

  - Filter type and location

  - Date of replacement

  - Reason for replacement

  - New filter specifications

  - Disposal method for spent filters

Spent Filter Management

- Spent odour-minimizing filters are managed as hazardous or non-hazardous waste based on their contamination.

- Spent activated carbon is sent to authorised recyclers or regenerators where feasible.

- Contaminated filters are disposed of through authorised waste disposal facilities.

- Disposal records are maintained for compliance and audit purposes.

3. Wet Scrubbers

Wet scrubbers are used to remove water-soluble odorous compounds and particulate matter from air streams. They are particularly effective for acidic or alkaline odours.

Types of Wet Scrubbers

- Packed Bed Scrubbers: Use packing media to provide a large surface area for gas-liquid contact.

- Venturi Scrubbers: Use high-velocity gas flow to atomise scrubbing liquid and capture fine particulates.

- Spray Tower Scrubbers: Use spray nozzles to create fine droplets for gas absorption.

- Impingement Scrubbers: Use impingement plates to create turbulence and enhance gas-liquid contact.

Operation and Maintenance

- Scrubbing liquid (water, caustic, acid, or oxidizing solution) is selected based on odour characteristics.

- Scrubbing liquid pH, concentration, and flow rate are monitored and maintained.

- Packing media, nozzles, and mist eliminators are inspected and cleaned regularly.

- Blowdown and make-up water are managed to maintain scrubbing efficiency.

- Performance is monitored through pH, pressure drop, and odour removal efficiency.

4. Biofilters

Biofilters use microbial action to degrade odorous organic compounds. They are effective for a wide range of organic odours and are environmentally friendly.

Design and Operation

- Biofilters consist of a filter bed (e.g., compost, wood chips, or synthetic media) with microbial populations.

- Odorous air is passed through the filter bed, where microorganisms degrade odorous compounds.

- Moisture, temperature, pH, and nutrient levels are maintained for optimal microbial activity.

- Biofilters are designed based on airflow rate, odour characteristics, and required removal efficiency.

Maintenance

- Media is inspected periodically for compaction, channeling, and drying.

- Moisture is maintained through irrigation systems.

- Nutrients are supplemented as needed to support microbial growth.

- Media is replaced or replenished when performance degrades.

- Performance is monitored through odour removal efficiency and microbial activity.

5. Thermal Oxidisers

Thermal oxidisers are used for destruction of odorous compounds at high temperatures. They are effective for high-concentration, high-flow odour streams.

Types of Thermal Oxidisers

- Direct-Fired Thermal Oxidisers: Use direct flame contact for odour destruction.

- Regenerative Thermal Oxidisers (RTOs): Use heat recovery for energy efficiency.

- Catalytic Thermal Oxidisers: Use catalysts to lower oxidation temperature.

Operation and Maintenance

- Operating temperature, residence time, and oxygen concentration are monitored and controlled.

- Burners, heat exchangers, and catalysts are inspected and maintained.

- Performance is monitored through odour removal efficiency and emission monitoring.

- Energy consumption is optimised through heat recovery and process integration.

6. Activated Carbon Adsorption Systems

Activated carbon adsorption systems are used for removal of a wide range of odorous organic compounds.

Design and Operation

- Activated carbon is selected based on odour characteristics and treatment requirements.

- Adsorption systems are designed with appropriate contact time, airflow rate, and carbon quantity.

- Multiple beds are used for continuous operation during carbon replacement.

- Performance is monitored through odour removal efficiency and carbon exhaustion.

Maintenance

- Carbon is replaced based on breakthrough detection or scheduled intervals.

- Spent carbon is regenerated or disposed of through authorised facilities.

- System pressure drop and airflow are monitored to detect clogging or channeling.

Odour Monitoring and Assessment

1. Olfactometry Monitoring

Olfactometry is used to measure odour concentration in terms of odour units (OU/m³). Samples are collected from odour sources, exhaust outlets, and facility boundaries, and analysed by trained panellists using standard methods (e.g., EN 13725).

- Sampling Frequency: Quarterly or as required based on operational changes or complaints.

- Sampling Locations: Odour sources, exhaust outlets, facility boundaries, and sensitive receptor locations.

- Parameters: Odour concentration, intensity, character, and hedonic tone.

2. Ambient Odour Monitoring

Ambient odour monitoring is conducted at facility boundaries and sensitive receptor locations to assess off-site odour impact.

- Monitoring Frequency: Monthly or as required based on complaints or operational changes.

- Monitoring Locations: Upwind and downwind facility boundaries, nearby residential areas, and sensitive receptors.

- Assessment Criteria: Odour concentration, intensity, and duration compared to applicable standards and community expectations.

3. Odour Complaint Management

Alchem International maintains a structured odour complaint management system to receive, investigate, and respond to odour complaints from employees, neighbours, and other stakeholders.

Complaint Receipt

- Complaints may be received through:

  - Direct communication (phone, email, in-person)

  - Community grievance mechanisms

  - Regulatory authorities

  - Employee reports

- All complaints are logged in the Odour Complaint Register with details including date, time, location, description, and complainant information.

Complaint Investigation

- EHS team acknowledges complaints within 24 hours and initiates investigation.

- Investigation includes:

  - Site inspection of odour sources

  - Review of odour control equipment performance

  - Meteorological conditions assessment

  - Sampling and analysis if required

  - Interviews with relevant personnel

- Root cause analysis is conducted to identify the source and cause of the odour.

Corrective Action

- Corrective actions are implemented to address the identified cause.

- Actions may include equipment repair or replacement, process modification, increased filter replacement frequency, or additional odour control measures.

- Corrective actions are documented, and their effectiveness is verified.

Communication and Feedback

- The complainant is informed of the investigation findings and corrective actions.

- Follow-up is conducted to ensure the issue is resolved and no further complaints are received.

- Complaint trends are reviewed periodically to identify recurring issues and improvement opportunities.

Regulatory Compliance

Alchem International complies with applicable odour regulations and standards, including:

- Air (Prevention and Control of Pollution) Act, 1981: Provisions related to air pollution control, including odour.

- Environment Protection Act, 1986: General standards for environmental protection.

- CPCB/SPCB Guidelines: State and central pollution control board guidelines on odour control and ambient air quality.

- National Ambient Air Quality Standards (NAAQS): Standards for ambient air quality, including odour-related parameters.

- Local Authority Requirements: Municipal and local authority requirements for odour control.

- Customer Requirements: Buyer-specific odour control and reporting requirements.

Documentation and Recordkeeping

Alchem International maintains comprehensive documentation related to odour control and filtration management:

- Odour Source Inventory: Identification and classification of all odour sources.

- Odour Control Equipment Register: Details of all odour control equipment, including filters, scrubbers, biofilters, and thermal oxidisers.

- Filter Replacement Records: Documentation of filter installation, maintenance, and replacement.

- Odour Monitoring Reports: Results of olfactometry and ambient odour monitoring.

- Odour Complaint Register: Records of odour complaints, investigations, and corrective actions.

- Maintenance Records: Documentation of maintenance activities for odour control equipment.

- Training Records: Documentation of personnel training on odour prevention and control.

- Regulatory Compliance Reports: Reports submitted to CPCB/SPCB and other authorities.

All records are retained for a minimum of five years, or longer where required by regulations, customer requirements, or Alchem International's internal data-retention policy.

Training and Awareness

Alchem International provides regular training and awareness programmes on odour prevention and control:

- Induction Training: All new employees receive training on odour sources, prevention, and control.

- Annual Refresher Training: All employees receive annual refresher training on odour management.

- Specialised Training: Personnel involved in odour control equipment operation and maintenance receive specialised training.

- Community Awareness: Alchem International engages with neighbouring communities to raise awareness of odour control efforts and grievance mechanisms.

- Training Records: Documentation of all training activities, including attendance, content, and assessment.

Continuous Improvement

Alchem International fosters a strong culture of continuous improvement in odour control and filtration management through:

- Regular Performance Reviews: Monthly and annual reviews of odour control performance, monitoring data, and complaint trends.

- Technology Evaluation: Ongoing evaluation of emerging odour control technologies for improved efficiency and effectiveness.

- Pilot Studies: Conducting pilot studies on new odour control technologies before full-scale implementation.

- Research Collaboration: Collaborating with technology providers, research institutions, and industry experts.

- Employee Engagement: Encouraging employee suggestions and Kaizen initiatives for odour reduction.

- Benchmarking: Benchmarking against pharmaceutical industry best practices and community expectations.

- Training and Awareness: Regular training on odour prevention, control technologies, and community engagement.

Compliance

All odour control and filtration management systems at Alchem International are designed, implemented, and reviewed to meet or exceed:

- Applicable Indian Environmental Laws: Air (Prevention and Control of Pollution) Act, 1981; Environment Protection Act, 1986; CPCB/SPCB norms; and other applicable regulations.

- International Standards: WHO guidelines, EU odour standards, and other applicable international benchmarks.

- Customer Requirements: Buyer-specific odour control limits, reporting requirements, and audit expectations.

- Industry Best Practices: Pharmaceutical industry odour control guidelines and community engagement standards.

Dedicated EHS personnel:

- Monitor regulatory updates and evolving compliance obligations.

- Update SOPs, training modules, and operational controls accordingly.

- Engage with accredited laboratories, authorised vendors, consultants, and authorities to maintain high standards of odour control, compliance, and community relations.

# SECTION 8: ACTIONS TO AVOID ROAD CONGESTION AND MINIMIZE TRANSPORT IMPACTS

Introduction

Alchem International recognizes that transportation activities can contribute to road congestion, community disturbance, safety risks, and indirect greenhouse gas emissions. As part of its responsible logistics and environmental management approach, structured measures are implemented to minimize traffic impact around the Company's premises at 25/2, Main Mathura Road, Village Kaili, Ballabhgarh, Faridabad, Haryana 121004, and along key transport routes.

These actions support operational efficiency, community well-being, road safety, and environmental performance. Given the location of Alchem International's facility on Main Mathura Road—a major arterial road with significant traffic density—managing transportation impacts is particularly important for maintaining good community relations and ensuring smooth logistics operations.

Objectives

The objectives of this procedure are to:

- Prevent traffic congestion in and around Alchem International's facilities, particularly on Main Mathura Road and adjacent areas.

- Reduce vehicle idling, delays, and unnecessary fuel consumption.

- Enhance road safety for employees, contractors, and local communities.

- Ensure responsible logistics operations aligned with environmental and social expectations.

- Minimize the impact of transportation activities on local residents, businesses, and other road users.

- Support Alchem International's sustainability commitments and community engagement objectives.

Applicability & Coverage

This SOP applies to:

- All Alchem International employees, contractors, and service providers involved in transportation, logistics, and material movement activities.

- All vehicles and transport operations associated with Alchem International, including:

  - Raw material deliveries

  - Finished goods dispatches

  - Inter-facility transfers

  - Employee transportation

  - Waste transportation

  - Contractor and visitor vehicles

  - Service and maintenance vehicles

- All transport routes used by Alchem International vehicles and transporters, with particular focus on:

  - Main Mathura Road (NH-2) and adjacent roads

  - Local roads in Village Kaili and surrounding areas

  - Routes to and from major highways and logistics hubs

  - Routes to and from the National Capital Region (NCR) and other destinations

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Department | • Develop and implement traffic management and congestion avoidance measures. • Conduct traffic impact assessments and monitor traffic-related incidents. • Coordinate with local authorities on traffic management issues. • Maintain records of traffic-related complaints and incidents. • Provide training on safe and responsible driving practices. |
| Logistics / Supply Chain Department | • Plan and schedule deliveries and dispatches to avoid peak traffic hours. • Coordinate with transporters and drivers on route planning and scheduling. • Optimize loads and consolidate shipments to reduce vehicle movements. • Monitor logistics performance and identify improvement opportunities. |
| Administration Department | • Manage employee transportation and parking. • Coordinate with security on gate management and vehicle movement. • Handle community complaints and grievances related to traffic. • Support traffic management during peak hours and special events. |
| Security Team | • Control vehicle entry and exit at facility gates. • Manage vehicle queuing and parking within the premises. • Coordinate with logistics and administration on traffic flow. • Report traffic-related incidents and issues. |
| Transporters / Drivers | • Follow designated routes and schedules. • Comply with speed limits and traffic rules. • Avoid unnecessary idling and honking. • Report traffic issues, road closures, and incidents promptly. • Follow the Driver Code of Conduct. |
| All Employees | • Use designated parking areas and follow traffic rules within the premises. • Report traffic-related concerns or suggestions to administration. • Support carpooling and other sustainable transportation initiatives. |

Scheduled and Optimized Logistics Operations

Alchem International plans and schedules logistics operations to avoid peak traffic hours and minimize congestion impacts.

1. Delivery and Dispatch Scheduling

- Dispatch and receipt of raw materials and finished goods are planned to avoid peak traffic hours on Main Mathura Road, particularly during:

  - Morning rush hours (8:00 AM – 10:30 AM)

  - Evening rush hours (5:00 PM – 8:00 PM)

  - School opening and closing times

  - Local market days and festivals

- Deliveries and dispatches are scheduled during off-peak hours wherever feasible.

- Advance slot booking is followed for loading and unloading activities to prevent queueing at entry gates.

- Delivery schedules are coordinated with transporters to stagger vehicle arrivals and departures.

2. Load Consolidation

- High-volume shipments are consolidated where feasible to reduce the number of vehicle movements.

- Return loads are arranged to minimize empty vehicle trips.

- Shared transportation is explored with nearby industries and logistics providers.

- Load optimization is conducted to maximize vehicle utilization and reduce trips.

3. Advance Coordination

- Transporters are informed of delivery schedules and slot bookings in advance.

- Drivers are provided with clear instructions on routes, entry gates, and parking areas.

- Changes in schedules are communicated promptly to avoid confusion and delays.

- Coordination with local authorities is maintained for major movements or special events.

Route Planning and Traffic Management

Alchem International identifies and uses preferred transport routes based on traffic density, road conditions, and proximity to residential areas.

1. Preferred Route Identification

- Preferred transport routes are identified based on:

  - Traffic density and congestion levels

  - Road conditions and infrastructure

  - Proximity to residential areas, schools, and hospitals

  - Emergency access requirements

  - Time of day and seasonal variations

- Routes are reviewed periodically and updated based on changing conditions.

2. GPS-Enabled Tracking

- GPS-enabled tracking systems are used to monitor vehicle movement and dynamically adjust routes in case of congestion, roadblocks, or accidents.

- Real-time traffic information is used to identify alternative routes and avoid delays.

- Transporters are encouraged to use navigation apps and traffic alerts for route optimization.

3. Route Restrictions

- Transport partners are instructed to avoid:

  - Narrow residential roads

  - Sensitive community zones (e.g., near schools, hospitals, places of worship)

  - Roads with weight or height restrictions

  - Congestion-prone areas during peak hours

- Alternative routes are identified and communicated to drivers.

4. Internal Traffic Management

- Entry and exit points within the facility are clearly demarcated to ensure smooth vehicle circulation.

- Designated parking areas are provided for different vehicle types (trucks, cars, two-wheelers).

- Traffic signals, signage, and speed limits are implemented within the premises.

- Security personnel manage vehicle movement during peak hours and special events.

Traffic Impact Risk Assessment

Periodic traffic impact risk assessments are conducted to identify congestion-prone periods, routes, and operational bottlenecks.

1. Assessment Scope

The assessment considers:

- Peak-hour traffic patterns on Main Mathura Road and adjacent roads

- Seasonal demand fluctuations and their impact on logistics

- Nearby schools, markets, hospitals, and residential areas

- Emergency access requirements

- Road conditions, construction activities, and infrastructure constraints

- Community concerns and complaints related to traffic

2. Assessment Frequency

- Comprehensive traffic impact assessments are conducted annually.

- Additional assessments are conducted when:

  - Significant changes in operations or logistics occur

  - New products or processes are introduced

  - Road infrastructure changes or construction activities occur

  - Community complaints increase

  - Regulatory requirements change

3. Risk Identification and Mitigation

- Identified risks are documented, and mitigation measures are implemented.

- Mitigation measures may include:

  - Revised delivery windows

  - Alternative routing

  - Load consolidation

  - Additional parking or holding areas

  - Coordination with local authorities

- Assessment findings are reviewed by Logistics, Administration, and EHS teams.

Driver Training and Code of Conduct

All drivers, including third-party logistics personnel, must comply with Alchem International's Driver Code of Conduct.

Driver Code of Conduct

The Code of Conduct covers:

- Safe and Defensive Driving Practices:

  - Adherence to speed limits and traffic rules

  - Defensive driving techniques to avoid accidents

  - No driving under the influence of alcohol or drugs

  - No use of mobile phones while driving

- Speed Limits:

  - Compliance with speed limits inside and around the facility

  - Reduced speed in congested areas and near sensitive receptors

  - Safe speed based on road and weather conditions

- Prohibition of Unnecessary Honking and Idling:

  - No honking in silence zones (near schools, hospitals, residential areas)

  - Engine shut-off during loading/unloading and waiting

  - Minimize idling to reduce noise and emissions

- Respectful Behavior:

  - Respectful behavior toward pedestrians, cyclists, and local communities

  - Courteous interaction with security, staff, and other drivers

  - No aggressive driving or road rage

Training and Briefing

- Periodic driver briefings are conducted on congestion avoidance, route discipline, and road safety.

- Training is provided on defensive driving, fuel-efficient driving, and environmental awareness.

- Any violations are recorded and addressed through corrective actions with transport service providers.

- Recognition and incentives are provided for safe and responsible drivers.

Emergency and Contingency Measures

Alchem International has established contingency plans to manage traffic disruptions and emergencies.

1. Traffic Disruption Contingency Plans

- Contingency plans are established to manage traffic disruptions caused by:

  - Road closures or construction

  - Weather conditions (floods, storms, fog)

  - Accidents or breakdowns

  - Strikes or protests

  - Special events or festivals

- Temporary vehicle holding areas are designated to prevent congestion on public roads.

- Alternative routes are identified and communicated to drivers.

- Coordination with local authorities and emergency services is maintained.

2. Emergency Response

- Security and logistics teams coordinate traffic flow during emergency situations.

- Emergency access routes are kept clear for ambulances, fire engines, and other emergency vehicles.

- Communication protocols are established for notifying drivers and transporters of emergencies.

- Post-emergency reviews are conducted to identify lessons learned and improve future response.

3. Community Engagement

- Alchem International engages with local communities and authorities on traffic-related issues.

- Community complaints and grievances are addressed promptly and transparently.

- Traffic management measures are communicated to the community through appropriate channels.

- Feedback from the community is considered in traffic management planning.

Monitoring and Performance Review

Alchem International monitors logistics and traffic performance to identify improvement opportunities.

1. Performance Indicators

Logistics performance indicators include:

- Vehicle waiting time at gates

- On-time dispatch and delivery rates

- Number of vehicle trips per day

- Average trip distance and fuel consumption

- Congestion-related incidents and delays

- Traffic-related complaints from community

- Driver violations and incidents

2. Data Collection and Analysis

- Data is collected through GPS tracking, gate logs, and driver reports.

- Data is analysed periodically to identify trends and improvement opportunities.

- Benchmarking is conducted against industry best practices.

3. Feedback and Improvement

- Feedback from employees, transporters, and local stakeholders is considered for improvement.

- Audit findings and incident reports are used to strengthen traffic management controls.

- Improvement actions are implemented and their effectiveness is monitored.

Documentation and Records

The following records are maintained:

- Logistics operational procedures

- Driver training and briefing records

- Route planning and scheduling documents

- Traffic impact risk assessment reports

- Incident logs and corrective action records

- Community complaint records

- GPS tracking data and performance reports

- Driver Code of Conduct acknowledgements

All records are retained in line with legal requirements and Alchem International's internal document retention policies.

Continuous Improvement

Alchem International reviews traffic management and logistics planning practices regularly to address changes in operational scale, infrastructure conditions, and community expectations. Improvements are aligned with environmental management, occupational safety, and responsible business conduct commitments.

Specific continuous improvement initiatives include:

- Regular review of delivery schedules and routes to optimize efficiency and minimize congestion.

- Evaluation of alternative fuel and electric vehicles for logistics operations.

- Adoption of digital tools for real-time traffic monitoring and route optimization.

- Collaboration with local authorities and industry peers on traffic management solutions.

- Employee and community awareness programmes on responsible transportation.

- Annual review of the Driver Code of Conduct and training programmes.

Compliance

All traffic management and logistics practices at Alchem International are aligned with:

- Applicable Indian motor vehicle and traffic laws

- Local authority traffic regulations and restrictions

- CPCB/SPCB guidelines on vehicular emissions

- Customer requirements and audit expectations

- Alchem International's environmental and social responsibility commitments

Dedicated EHS and Logistics personnel:

- Monitor regulatory updates and evolving compliance obligations.

- Update SOPs, training modules, and operational controls accordingly.

- Engage with transporters, authorities, and communities to maintain high standards of safety, compliance, and community relations.

# SECTION 9: PRODUCT USE AND END-OF-LIFE (EOL) MANAGEMENT

Introduction

As part of its environmental stewardship and responsible manufacturing commitment, Alchem International integrates responsible product-use and end-of-life (EOL) practices into the design, packaging, distribution, and post-use management of APIs, botanical extracts, nutraceutical ingredients, intermediates, and pilot or development batches. This SOP outlines Alchem International's measures to reduce environmental burdens associated with packaging waste, expired or obsolete products, returned items, damaged stock, and surplus inventory, while promoting circular material flows and transparency across the pharmaceutical value chain.

Alchem International recognizes that pharmaceutical products—including APIs, botanical extracts, and nutraceutical ingredients—require responsible management throughout their lifecycle to prevent environmental contamination, protect public health, and ensure regulatory compliance. This SOP addresses the unique challenges of pharmaceutical product end-of-life management, including the need for secure disposal of expired or unused products, prevention of environmental release of pharmaceutical compounds, and promotion of circular economy principles where feasible.

Objectives

The objectives of this SOP are to:

- Minimise post-consumer and post-distribution waste arising from pharmaceutical products, botanical extracts, and packaging.

- Promote proper reuse, recycling, and disposal methods to prevent environmental contamination.

- Enable circular material flows through collaboration with customers, recyclers, and suppliers.

- Avoid overproduction of short-lifecycle or trend-driven products.

- Improve packaging efficiency to deliver environmental and cost benefits.

- Ensure secure and compliant disposal of expired, unused, or rejected pharmaceutical products.

- Prevent environmental release of pharmaceutical compounds from product end-of-life management.

- Maintain full traceability and documentation for product end-of-life activities.

- Support Alchem International's sustainability commitments and customer audit requirements.

Applicability & Coverage

This SOP applies to:

All Alchem International products and materials, including:

- Active Pharmaceutical Ingredients (APIs)

- Botanical Extracts and Phytochemicals

- Nutraceutical Ingredients

- Cosmeceutical Ingredients

- Intermediates and Semi-Synthesised Compounds

- Pilot and Development Batches

- Samples and Reference Standards

- Packaging Materials and Ancillaries

- Returned, Expired, or Obsolete Products

- Damaged or Off-Specification Products

All Alchem International facilities and operations, including:

- Manufacturing Facilities

- Quality Control and Analytical Laboratories

- Research and Development Laboratories

- Warehousing and Storage Areas

- Packing and Dispatch Areas

- Administrative Offices

All personnel involved in product design, packaging, distribution, and end-of-life management, including:

- Production and Manufacturing Personnel

- Quality Control and Quality Assurance Personnel

- Research and Development Personnel

- Supply Chain and Logistics Personnel

- Warehouse and Stores Personnel

- EHS and Sustainability Personnel

- Customer Service and Business Development Personnel

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Department | • Develop and implement product end-of-life management procedures. • Ensure compliance with environmental regulations for product disposal. • Coordinate with authorised recyclers and disposal facilities. • Maintain records of product end-of-life activities and disposal certificates. • Provide training on responsible product end-of-life management. |
| Production / Manufacturing Department | • Implement packaging optimization and waste reduction measures. • Segregate expired, obsolete, and rejected products for proper disposal. • Support product take-back and recovery programmes. • Report any product end-of-life issues to EHS. |
| Quality Control / Quality Assurance | • Assess returned, expired, or damaged products for reuse, recycling, or disposal. • Ensure compliance with pharmacopoeial standards for product disposition. • Maintain documentation of product disposition decisions. • Support recall and incident response activities. |
| Research & Development | • Incorporate design for environment principles into product development. • Evaluate packaging materials for recyclability and sustainability. • Develop product take-back and recovery programmes. • Conduct lifecycle assessments for new products. |
| Supply Chain / Logistics | • Coordinate product take-back and recovery logistics. • Manage returned and expired product inventory. • Ensure proper transportation of products for disposal or recycling. • Maintain records of product movements for end-of-life management. |
| Procurement Team | • Source sustainable packaging materials. • Evaluate suppliers based on environmental performance. • Ensure procurement specifications include recyclability and sustainability criteria. • Support packaging optimization initiatives. |
| All Personnel | • Follow SOPs for product handling, storage, and disposal. • Report expired, obsolete, or damaged products to supervisors. • Support product take-back and recovery programmes. • Participate in training on product end-of-life management. |

Actions Implemented to Reduce Environmental Impacts

| Focus Area | Actions Taken |
| --- | --- |
| Optimised Packaging Design | Adoption of minimalistic, right-size packaging for samples, swatches, and shipments; avoidance of PVC and complex plastic-metal composites. |
| Material Simplification | Preference for mono-material, recyclable bags, paper-based wraps, and cardboard cartons; labels printed using water-based inks. |
| Eco-Friendly Materials | Transition to biodegradable cushioning, corrugated inserts, and recyclable packaging materials. |
| Sustainable Packaging for APIs | Use of recyclable, reusable, or returnable packaging for API shipments where feasible; reduction of secondary packaging. |
| Tamper-Evident Packaging | Use of recyclable tamper-evident seals and closures where feasible. |
| Cold Chain Packaging | Use of recyclable or reusable cold chain packaging for temperature-sensitive products. |

1. Reduced Material Usage in Primary Packaging

- Redesign of bags, liners, and protective wraps using thinner yet durable grades, reducing plastic usage while maintaining product protection.

- Elimination of unnecessary inserts and excess layers in secondary packaging, particularly for sampling and customer submissions.

- Optimization of packaging sizes to reduce void space and material usage.

2. Dimension & Weight Optimisation

- Standardisation of carton sizes for common product types to improve container utilisation and reduce packaging volume per shipment.

- Introduction of lightweight packaging materials that maintain integrity while reducing shipping weight and transport emissions.

- Optimization of drum and container sizes for bulk API shipments.

3. Simplified Material Composition

- Shift towards mono-material packaging formats to enhance recyclability and reduce composite waste.

- Avoidance of PVC-based laminates and metallised films in new packaging introductions.

- Use of compatible materials for caps, closures, and labels to facilitate recycling.

4. Sustainable Ink and Labelling Practices

- Use of water-based or soy-based inks and non-toxic adhesives for labels.

- Minimalistic label designs with essential information only, printed on recyclable or recycled paper where feasible.

- Digital labelling options where feasible to reduce paper usage.

Forecast-Based Production & Inventory Optimisation

To prevent surplus production and reduce associated material, storage, and disposal impacts, Alchem International follows robust planning and inventory controls:

1. Forecast-Driven Production Planning

- Integration of historical order data, customer forecasts, seasonal trends, and sampling feedback into demand planning tools to guide production quantities.

- Monthly cross-functional reviews with business development, production, and supply chain teams to proactively adjust plans.

- Use of statistical forecasting models and demand planning software for improved accuracy.

- Consideration of regulatory approvals, market trends, and customer contracts in demand planning.

2. Batch Flexibility & Agile Manufacturing

- Modular production planning that allows flexible order sizing and product changes with minimal waste and downtime.

- Small-scale pilot runs for new products or customer trials to validate demand before scaling up.

- Quick changeover capabilities to respond to changing customer requirements.

3. Inventory Control for Raw and Finished Goods

- Implementation of FIFO/FEFO principles for raw materials, intermediates, and finished products to avoid ageing and obsolescence.

- Digital monitoring of stock levels and ageing to flag slow-moving or excess inventory.

- Regular inventory reviews and disposition of expired or near-expiry products.

- Optimization of safety stock levels to balance supply security and waste reduction.

4. Pilot Quantities for New Products

- Limited initial production runs for new products or customer trials to validate demand before scaling up.

- Customer collaboration for sample evaluation and feedback before full-scale production.

- Evaluation of market demand and customer commitments before large-scale production.

5. Waste Reduction from Returns or Obsolescence

- Collaboration with customers and logistics partners to consolidate shipments, reduce returns, and align packaging volumes with real-time demand.

- Return authorization and tracking systems to manage product returns efficiently.

- Assessment of returned products for reuse, recycling, or disposal based on quality and regulatory requirements.

Regular Disposal of Products and Wastes

Alchem International ensures that expired, obsolete, or unsellable products are disposed of responsibly and in compliance with regulatory requirements.

1. Segregation and Assessment

- Damaged, obsolete, or unsellable products are segregated and assessed for reuse, donation (where permitted), recycling, or authorised disposal.

- Assessment considers:

  - Product quality and stability

  - Regulatory requirements

  - Environmental impact

  - Reuse or recycling potential

  - Disposal options

2. Secure Disposal of Pharmaceutical Products

- Products contaminated with chemicals, solvents, or hazardous substances are treated as hazardous waste and disposed of through authorised vendors.

- Expired or unused pharmaceutical products are disposed of through:

  - High-temperature incineration at authorised facilities

  - Secure landfilling at authorised facilities

  - Co-processing at authorised cement kilns

  - Other authorised disposal methods

- Disposal practices align with applicable environmental regulations and authorised waste management procedures.

- Disposal certificates and manifests are maintained for all pharmaceutical product disposals.

3. Prevention of Environmental Release

- No pharmaceutical products are disposed of in regular municipal waste streams.

- No pharmaceutical products are discharged to soil, water, or open drains.

- No pharmaceutical products are sent to unauthorised recyclers or disposal facilities.

- All disposal activities are conducted through authorised and audited vendors.

Stakeholder Collaboration for Product End-of-Life

Alchem International collaborates with stakeholders to promote responsible product end-of-life management:

- Authorised Recyclers and Waste Processors: For product and packaging waste recycling and disposal.

- Customers and Brand Partners: On take-back, reuse, or recycling initiatives for unsold or returned items.

- Packaging Suppliers: To develop sustainable alternatives and provide end-of-life guidance.

- Logistics Partners: For safe and compliant transportation of products for disposal or recycling.

- Regulatory Authorities: For compliance with product disposal and environmental regulations.

- Industry Associations: For sharing best practices and developing industry standards.

Continuous coordination with authorised transporters and recyclers ensures traceability of product and packaging waste.

Information Provided to Stakeholders

Alchem International provides information to customers and stakeholders on responsible product end-of-life management:

- Packaging and shipment documentation includes guidance on reuse, recycling, and responsible disposal where applicable.

- Safety Data Sheets (SDS) or chemical handling information is provided for any auxiliary materials supplied with products.

- Customers and partners receive summaries of return, reuse, or disposal procedures for samples and pilot batches.

- Product stewardship information is provided to customers to support responsible use and disposal.

- Training and awareness materials are available on request.

Circular Flow Promotion

Alchem International promotes circular material flows through:

- Reusable cartons, pallets, and protective materials are circulated internally where condition permits.

- Non-contaminated paper and cardboard packaging is segregated and sent for recycling.

- Spent biomass, extraction residues, and rejected batches are routed to authorised recyclers or disposal facilities.

- Pilot batch return protocols are available for customers to return unsold or unused items for consolidated reuse or recycling.

- Solvent recovery and reuse programmes reduce the need for fresh solvent procurement and waste generation.

- Packaging take-back programmes with suppliers for drums, IBCs, and other containers.

Optimization of Primary Packaging Design

The Company has implemented structured processes to optimize primary packaging designed and specified internally. These measures include:

- Light weighting of Primary Packaging: Packaging designs are reviewed to reduce wall thickness, overall dimensions, and material mass while maintaining required protection and product compatibility.

- Material Simplification and Mono-Material Use: Wherever feasible, primary packaging is designed using single-material formats to improve recyclability and reduce sorting complexity for end users.

- Elimination of Unnecessary Components: Non-essential inserts, layers, or mixed-material elements in primary packaging are removed during design reviews.

- Design for Disposal and Recycling: Packaging is designed to be easily emptied, cleaned (where applicable), and disposed of through standard recycling streams available to customers.

All packaging optimization activities are governed through documented SOPs, work instructions, and technological change records.

Design for Disassembly, Recycling, and Remanufacturing

The Company incorporates design features that support circularity at the product's end-of-life, including:

- Ease of Disassembly: Products are designed to allow separation of key components or materials without complex tools, adhesives, or irreversible joints.

- Use of Standardized Materials and Components: Preference is given to standard material grades and components to improve compatibility with recycling and remanufacturing processes.

- Material Identification and Transparency: Product material specifications clearly identify material composition to support sorting, recycling, or recovery.

- Recycling and Disposal Guidance: Disposal or recycling instructions are provided to guide customers on appropriate end-of-life handling.

These design considerations help ensure that valuable materials are recovered and reused rather than discarded.

Take-Back and Product Recovery Programs

To further support circularity, the Company has established mechanisms to recover products after use:

- Product Take-Back / Buy-Back Programs: Customers are enabled to return used or end-of-life products either free of charge or through defined buy-back arrangements.

- Material Recovery and Remanufacturing: Returned products are assessed for:

  - Remanufacturing into new products

  - Dismantling for material recycling

  - Safe disposal where recovery is not feasible

- Program Governance: Recovery programs are supported by documented implementation records, defined responsibilities, and tracking mechanisms.

These programs help close material loops and reduce waste generation downstream.

Information on Efficient Use, Storage, and Maintenance to Extend Product Life

Objective

The objectives of this initiative are to:

- Enable customers to use products efficiently with minimal waste and resource consumption

- Prevent product degradation, leakage, or unintended emissions through proper storage and handling

- Extend product shelf life and functional performance

- Reduce environmental and health risks associated with improper use or disposal

- Support customers in meeting regulatory and sustainability requirements

Product Use Guidance

The Company provides clear instructions to ensure optimal and efficient product use, including:

- Recommended dosage or application rates to avoid overuse

- Appropriate process conditions (temperature, pH, mixing sequence, contact time)

- Compatibility guidance with other chemicals or materials

- Instructions to prevent spills, splashing, or uncontrolled release during handling and application

- Warnings against misuse that could lead to inefficiency, safety risks, or environmental harm

These instructions are included on product labels, technical data sheets, and user manuals.

Safe Storage Instructions

To prevent leaks, emissions, and premature degradation, customers are provided with detailed storage guidance, such as:

- Recommended storage temperature and humidity ranges

- Protection from direct sunlight, moisture, or incompatible substances

- Proper container sealing and positioning to prevent leakage

- Shelf-life information and first-in-first-out (FIFO) storage practices

- Guidance on segregation of incompatible chemicals

Storage instructions are clearly stated on labels, safety data sheets (SDS), and storage guidelines.

Maintenance and Handling Practices

For durable goods, containers, or reusable chemical handling systems, the Company provides guidance on:

- Routine inspection of containers, valves, hoses, and dispensing equipment

- Cleaning procedures to avoid cross-contamination or residue build-up

- Preventive maintenance practices to ensure safe and prolonged usability

- Early identification of wear, damage, or leakage

Where applicable, service or repair manuals are provided to customers or service partners.

Environmental Protection Measures

The information provided enables customers to:

- Minimize accidental releases into air, water, or soil

- Reduce emissions arising from improper storage or handling

- Avoid unnecessary disposal of unused or degraded products

- Apply products efficiently to reduce wastewater load and chemical loss

For chemical products, instructions emphasize closed handling, controlled dosing, and spill-prevention practices.

Communication Channels

Best-practice information is communicated through multiple channels, including:

- Product labels with clear symbols and instructions

- User manuals and technical guidelines

- Safety Data Sheets (SDS)

- Service and repair manuals (where applicable)

- Documented customer guidelines shared digitally or during onboarding

Documentation and Evidence

The following documents demonstrate compliance with this approach:

- Product labels with usage and storage instructions

- User manuals and technical data sheets

- Service or repair manuals (where applicable)

- Written document guidelines on efficient use and maintenance

- SDS and product stewardship documentation

Resource-Efficient Product Design and Emission Reduction

Design Strategies for Reducing Resource Input During Use

The Company integrates the following principles into product design and development:

- Optimized Material Selection: Raw materials and formulations are selected to achieve required performance at lower dosage or lower process intensity, reducing water, energy, and auxiliary chemical consumption during use.

- Efficiency-Driven Functional Design: Products are engineered to perform effectively under reduced operational parameters, such as lower temperatures, shorter process cycles, or reduced water volumes.

- Reduced Consumable Dependency: Product designs aim to minimize the need for frequent replacement or additional consumables (e.g., reduced need for reprocessing, additives, or repeated treatments).

- Extended Product Life: Enhanced stability and durability reduce product degradation, rework, and premature disposal, improving overall resource efficiency.

Performance validation is supported through internal testing, benchmarking, and technical evaluations.

Design and Treatment Measures to Minimize Environmental Impact

To address environmental pollution risks, the Company implements targeted design and treatment measures, including:

- Material Engineering: Use of optimized material structures and controlled formulations to reduce waste during handling and processing.

- Surface and Finish Optimization: Application of treatments and finishes that improve material cohesion and durability without increasing environmental burden.

- Strength and Durability: Design improvements aimed at enhancing tensile strength and reducing mechanical stress that leads to degradation or material release over time.

- Use Durability Consideration: Products are evaluated for performance stability across repeated use cycles, reducing progressive material loss during normal use.

Testing and Validation

Product performance and environmental improvements are verified through:

- Product design and development reports

- Laboratory and pilot-scale product testing

- Comparative technical performance assessments

- Durability and stability testing

- Resource efficiency benchmarking against similar products

Where applicable, findings are incorporated into Environmental Product Declarations (EPDs) or internal sustainability documentation.

Customer Guidance and Transparency

To ensure effective implementation during use, customers are provided with:

- Product brochures highlighting efficiency and durability benefits

- Technical specifications and comparison data

- Documented usage and care guidelines to support low-impact use

- Recommendations for handling, storage, and maintenance practices that reduce environmental impact

Documentation

Maintained records include:

- Production and batch logs for products and samples

- Disposal and recycling certificates

- Vendor waste manifests and transporter records

- Packaging material specifications and weight data

- Demand forecasting records and inventory ageing reports

Continuous Improvement

Alchem International fosters continuous improvement in product end-of-life management through:

- Annual EOL and packaging reviews to assess reductions in waste generation and improvements in recyclability.

- Participation in supplier innovation programmes and eco-design workshops.

- Periodic evaluation of reusable or returnable packaging options for future customer collaborations.

- Employee suggestion programmes and quarterly reviews focused on material efficiency and waste reduction.

- Benchmarking against pharmaceutical industry best practices and customer expectations.

- Technology evaluation for improved recycling and disposal methods.

- Customer feedback integration for packaging and product design improvements.

Compliance

All product use and end-of-life management practices at Alchem International are aligned with applicable environmental regulations, customer requirements, and pharmaceutical industry best practices. The company ensures full traceability, responsible handling, and transparent communication across the product lifecycle through collaboration with authorised vendors and stakeholders.

Specific compliance requirements include:

- Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016: For disposal of hazardous pharmaceutical waste.

- Environment Protection Act, 1986: For environmental protection and pollution prevention.

- Drugs and Cosmetics Act, 1940: For pharmaceutical product quality and disposal.

- CPCB/SPCB Guidelines: For waste management and environmental compliance.

- Customer Requirements: For product stewardship, take-back programmes, and end-of-life reporting.

- International Standards: For pharmaceutical product disposal and environmental protection.

# SECTION 10: PHARMACEUTICAL DEMAND FORECASTING & RESPONSIBLE INVENTORY MANAGEMENT

Introduction

Alchem International recognises that accurate demand forecasting and responsible inventory management are critical to minimising the risk of overproduction, excess inventories of unused or expired pharmaceutical products, and the eventual release of pharmaceutical compounds to the environment. As a manufacturer of APIs, botanical extracts, and nutraceutical ingredients serving global markets, Alchem International must balance the need to meet customer demand reliably with the imperative to minimise waste, reduce environmental impact, and optimise resource utilisation.

Overproduction and excess inventory in the pharmaceutical industry can lead to:

- Expiration of pharmaceutical products, requiring costly and environmentally sensitive disposal.

- Increased risk of environmental release of pharmaceutical compounds through improper disposal.

- Unnecessary consumption of raw materials, energy, and water.

- Increased storage and handling costs.

- Potential regulatory non-compliance and financial losses.

- Contribution to the global challenge of pharmaceutical waste in the environment.

This Pharmaceutical Demand Forecasting & Responsible Inventory Management Standard Operating Procedure (SOP) establishes Alchem International's structured framework, tools, and strategies for estimating demand for pharmaceutical products, optimising inventory levels, and minimising the risk of overproduction and pharmaceutical waste. The SOP aligns with Indian regulatory requirements, customer expectations, and international best practices for responsible pharmaceutical manufacturing.

Objectives

The objectives of this SOP are to:

- Establish structured tools and strategies for accurate demand forecasting of pharmaceutical products.

- Minimise the risk of overproduction and excess inventories of APIs, extracts, and nutraceutical ingredients.

- Reduce the generation of unused or expired pharmaceutical products requiring disposal.

- Prevent the environmental release of pharmaceutical compounds through responsible inventory management.

- Optimise inventory levels to balance supply security with waste reduction.

- Improve production planning and resource utilisation through accurate demand forecasting.

- Enhance customer satisfaction through reliable supply and reduced lead times.

- Support Alchem International's sustainability commitments, ESG performance, and customer audit requirements.

- Ensure compliance with applicable regulations and customer requirements for pharmaceutical demand forecasting and inventory management.

Applicability & Coverage

This SOP applies to:

All Alchem International products and materials, including:

- Active Pharmaceutical Ingredients (APIs)

- Botanical Extracts and Phytochemicals

- Nutraceutical Ingredients

- Cosmeceutical Ingredients

- Intermediates and Semi-Synthesised Compounds

- Finished Products and Formulations

- Raw Materials and Packaging Materials

- Samples and Reference Standards

All Alchem International facilities and operations, including:

- Manufacturing Facilities

- Quality Control and Analytical Laboratories

- Research and Development Laboratories

- Warehousing and Storage Areas

- Packing and Dispatch Areas

- Administrative Offices

All personnel involved in demand forecasting, production planning, inventory management, and supply chain activities, including:

- Business Development and Sales Personnel

- Production Planning Personnel

- Supply Chain and Logistics Personnel

- Warehouse and Stores Personnel

- Procurement Personnel

- Finance and Accounting Personnel

- Quality Control and Quality Assurance Personnel

- EHS and Sustainability Personnel

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| Business Development / Sales | • Provide customer forecasts, order history, and market intelligence. • Communicate customer requirements, contract details, and demand trends. • Collaborate with production planning on demand forecasting. • Support customer engagement on demand planning. |
| Production Planning | • Develop and maintain demand forecasting models. • Integrate historical data, market trends, and customer forecasts into production planning. • Coordinate with procurement and production on inventory requirements. • Monitor forecast accuracy and adjust plans as needed. • Optimise production schedules to minimise waste and excess inventory. |
| Supply Chain / Logistics | • Manage inventory levels based on demand forecasts. • Coordinate with production and procurement on inventory replenishment. • Monitor inventory ageing and identify slow-moving or excess stock. • Support demand forecasting with lead time and supply chain data. • Coordinate product disposal for expired or obsolete inventory. |
| Procurement | • Source raw materials and packaging based on production plans. • Coordinate with suppliers on lead times and delivery schedules. • Support demand forecasting with supplier capacity and lead time data. • Optimise procurement to minimise excess inventory. |
| Quality Control / Quality Assurance | • Monitor product shelf life and stability data. • Assess inventory for quality and compliance with specifications. • Support disposition decisions for expired or near-expiry products. • Provide input on product stability for demand forecasting. |
| Finance / Accounting | • Provide financial data and analysis for demand forecasting. • Monitor inventory costs and carrying costs. • Support inventory valuation and disposition decisions. • Track financial performance against forecast. |
| EHS / Sustainability | • Monitor environmental impact of inventory management. • Ensure compliance with environmental regulations for product disposal. • Support waste reduction and circular economy initiatives. • Track and report on pharmaceutical waste generation. |
| All Personnel | • Follow SOPs for demand forecasting and inventory management. • Report demand changes, inventory issues, or potential waste to supervisors. • Support continuous improvement initiatives for demand forecasting and inventory management. |

Demand Forecasting Tools and Strategies

Alchem International employs a range of tools and strategies to estimate demand for pharmaceutical products accurately and minimise the risk of overproduction and excess inventory.

1. Historical Data Analysis

- Order History Analysis: Analysis of historical order data, including order quantities, frequencies, seasonality, and customer patterns.

- Sales Trend Analysis: Analysis of sales trends over time to identify growth, decline, or cyclical patterns.

- Customer Segmentation: Segmentation of customers based on order patterns, product preferences, and demand characteristics.

- Product Lifecycle Analysis: Analysis of product lifecycle stages (introduction, growth, maturity, decline) to anticipate demand changes.

- Data Sources: ERP systems, sales records, customer databases, and historical production data.

2. Customer Forecast Integration

- Customer Forecasts: Collection and integration of customer forecasts, contracts, and demand commitments.

- Customer Collaboration: Regular engagement with customers on demand planning, product requirements, and market trends.

- Contract Analysis: Analysis of customer contracts for committed volumes, delivery schedules, and pricing.

- Customer Relationship Management (CRM): Use of CRM systems to track customer interactions, requirements, and forecasts.

- Demand Sensing: Use of real-time customer signals to adjust forecasts dynamically.

3. Statistical Forecasting Models

- Time Series Analysis: Use of statistical methods (moving average, exponential smoothing, ARIMA) to forecast demand based on historical patterns.

- Regression Analysis: Use of regression models to identify relationships between demand and influencing factors (e.g., price, seasonality, economic indicators).

- Causal Models: Use of causal models to forecast demand based on underlying drivers.

- Simulation Models: Use of simulation to evaluate different demand scenarios and their impact on production and inventory.

- Forecast Accuracy Tracking: Regular tracking of forecast accuracy and adjustment of models as needed.

4. Market Intelligence and Trend Analysis

- Market Research: Collection and analysis of market data, industry trends, and competitive intelligence.

- Regulatory Intelligence: Monitoring of regulatory changes that may impact demand (e.g., new approvals, patent expirations, regulatory restrictions).

- Economic Indicators: Analysis of economic indicators (GDP, inflation, exchange rates) that may impact demand.

- Seasonal Trends: Analysis of seasonal patterns in demand for different products.

- Emerging Trends: Identification of emerging trends (e.g., new therapeutic areas, changing consumer preferences) that may impact demand.

5. Collaborative Planning, Forecasting, and Replenishment (CPFR)

- Cross-Functional Collaboration: Collaboration between Business Development, Production Planning, Supply Chain, and Finance on demand forecasting.

- Customer Collaboration: Collaboration with key customers on demand planning and replenishment.

- Supplier Collaboration: Collaboration with key suppliers on lead times, capacity, and delivery schedules.

- Regular Review Meetings: Monthly or quarterly review meetings to align forecasts and plans.

- Information Sharing: Sharing of demand forecasts, inventory data, and production plans across the supply chain.

6. Demand Planning Software and Tools

- ERP Systems: Use of ERP systems for demand planning, inventory management, and production planning.

- Advanced Planning Systems (APS): Use of APS for demand forecasting, production scheduling, and inventory optimization.

- Spreadsheet Models: Use of spreadsheet models for demand forecasting and scenario analysis.

- Business Intelligence (BI) Tools: Use of BI tools for data analysis, visualization, and reporting.

- Artificial Intelligence (AI) and Machine Learning (ML): Use of AI/ML algorithms for improved demand forecasting accuracy.

7. Scenario Planning and Risk Assessment

- Scenario Analysis: Development of different demand scenarios (best case, base case, worst case) to evaluate production and inventory strategies.

- Sensitivity Analysis: Analysis of demand sensitivity to key variables (e.g., price, customer demand, regulatory changes).

- Risk Assessment: Identification and assessment of risks that may impact demand (e.g., supply chain disruptions, regulatory changes, market shifts).

- Contingency Planning: Development of contingency plans to address demand variability and uncertainty.

- Flexibility and Agility: Design of production and supply chain systems to be flexible and responsive to demand changes.

Inventory Management Strategies

Alchem International employs responsible inventory management strategies to balance supply security with waste reduction.

1. Inventory Classification

- ABC Analysis: Classification of inventory based on value and turnover (A = high value, low turnover; B = medium value, medium turnover; C = low value, high turnover).

- VED Analysis: Classification of inventory based on criticality (V = vital, E = essential, D = desirable).

- FSN Analysis: Classification of inventory based on movement (F = fast-moving, S = slow-moving, N = non-moving).

- Shelf Life Classification: Classification of inventory based on shelf life and stability.

- Regulatory Classification: Classification of inventory based on regulatory requirements and restrictions.

2. Inventory Optimization

- Economic Order Quantity (EOQ): Determination of optimal order quantities to minimize ordering and carrying costs.

- Safety Stock Optimization: Determination of optimal safety stock levels to balance supply security and waste reduction.

- Reorder Point (ROP): Determination of reorder points based on lead times and demand variability.

- Just-in-Time (JIT) Inventory: Adoption of JIT principles where feasible to minimize inventory levels.

- Vendor-Managed Inventory (VMI): Collaboration with suppliers on VMI arrangements for improved inventory efficiency.

- Consignment Inventory: Use of consignment inventory arrangements for critical or slow-moving items.

3. FIFO/FEFO Inventory Management

- First-In, First-Out (FIFO): Strict adherence to FIFO principles to ensure older inventory is used first.

- First-Expired, First-Out (FEFO): Implementation of FEFO principles for products with limited shelf life.

- Inventory Ageing Monitoring: Regular monitoring of inventory ageing to identify slow-moving or near-expiry stock.

- Expiry Date Tracking: Systematic tracking of expiry dates and proactive management of near-expiry inventory.

- Disposition Planning: Proactive planning for disposition of near-expiry or obsolete inventory.

4. Inventory Visibility and Control

- Real-Time Inventory Tracking: Use of ERP systems and barcode/RFID technology for real-time inventory tracking.

- Inventory Dashboards: Use of dashboards for inventory visibility and monitoring.

- Cycle Counting: Regular cycle counting to ensure inventory accuracy.

- Physical Inventory Audits: Periodic physical inventory audits to verify inventory records.

- Inventory Reconciliation: Regular reconciliation of inventory records with physical stock.

5. Slow-Moving and Obsolete Inventory Management

- Identification: Regular identification of slow-moving and obsolete inventory through ageing analysis and demand review.

- Disposition Options: Evaluation of disposition options, including:

  - Use in production (where feasible)

  - Sale to other customers (where permitted)

  - Return to supplier (where permitted)

  - Recycling or recovery

  - Authorised disposal

- Disposition Decision: Decision on disposition based on quality, regulatory, and environmental considerations.

- Disposition Execution: Execution of disposition through authorised channels with proper documentation.

- Preventive Actions: Identification of root causes and implementation of preventive actions to avoid future slow-moving or obsolete inventory.

6. Safety Stock and Buffer Inventory

- Safety Stock Determination: Determination of safety stock levels based on demand variability, lead time variability, and service level requirements.

- Buffer Inventory for Critical Items: Maintenance of buffer inventory for critical items to prevent production disruptions.

- Review and Adjustment: Regular review and adjustment of safety stock levels based on changing demand and supply conditions.

- Trade-off Analysis: Analysis of trade-offs between inventory carrying costs and supply security.

7. Collaborative Inventory Management

- Customer Collaboration: Collaboration with customers on inventory planning and replenishment.

- Supplier Collaboration: Collaboration with suppliers on lead times, delivery schedules, and inventory management.

- Distributor Collaboration: Collaboration with distributors on inventory management and demand planning.

- Information Sharing: Sharing of inventory data and demand forecasts across the supply chain.

- Joint Planning: Joint planning and decision-making on inventory management.

Prevention of Overproduction and Pharmaceutical Waste

Alchem International implements specific measures to prevent overproduction and pharmaceutical waste generation.

1. Demand-Driven Production

- Production Scheduling Based on Demand: Production scheduling aligned with demand forecasts and customer orders.

- Make-to-Order (MTO): Adoption of MTO production where feasible to avoid excess inventory.

- Make-to-Stock (MTS) Optimization: Optimization of MTS production to balance supply security and waste reduction.

- Demand Review Before Production: Review of demand forecasts before initiating production runs.

- Production Flexibility: Flexibility to adjust production volumes based on changing demand.

2. Batch Size Optimization

- Optimal Batch Sizing: Determination of optimal batch sizes to balance production efficiency and inventory waste.

- Small-Scale Pilot Runs: Use of small-scale pilot runs for new products or market trials.

- Scale-Up Based on Demand: Scale-up of production based on confirmed demand.

- Batch Consolidation: Consolidation of small orders into larger batches where feasible to improve efficiency and reduce waste.

- Batch Flexibility: Flexibility to produce smaller or larger batches based on demand.

3. Shelf Life Management

- Shelf Life Assessment: Assessment of product shelf life and stability during development.

- Shelf Life Extension: Evaluation of shelf life extension options where feasible.

- Stability Studies: Ongoing stability studies to support shelf life claims.

- Expiry Date Management: Proactive management of expiry dates and near-expiry inventory.

- Disposition Planning: Proactive planning for disposition of near-expiry inventory.

4. Customer Order Management

- Order Validation: Validation of customer orders for accuracy and feasibility.

- Order Consolidation: Consolidation of customer orders to optimize production and reduce waste.

- Order Flexibility: Flexibility to adjust order quantities based on production and inventory constraints.

- Customer Communication: Proactive communication with customers on order status, lead times, and changes.

- Returns Management: Efficient management of customer returns to minimize waste.

5. Regulatory and Quality Compliance

- Regulatory Requirements: Compliance with regulatory requirements for product shelf life, labelling, and disposal.

- Quality Standards: Compliance with quality standards for product release and disposition.

- Documentation: Complete documentation of production, inventory, and disposition activities.

- Audit Readiness: Maintenance of audit-ready records for regulatory and customer audits.

- Compliance Monitoring: Ongoing monitoring of regulatory changes and compliance requirements.

Environmental Benefits of Responsible Demand Forecasting and Inventory Management

Alchem International's responsible demand forecasting and inventory management practices deliver significant environmental benefits:

- Reduced Pharmaceutical Waste: Minimisation of unused or expired pharmaceutical products requiring disposal.

- Prevention of Environmental Release: Prevention of pharmaceutical compounds from entering the environment through improper disposal.

- Resource Conservation: Reduced consumption of raw materials, energy, and water through optimized production.

- Reduced Carbon Footprint: Lower greenhouse gas emissions from reduced production, storage, and transportation.

- Waste Reduction: Reduced solid and hazardous waste generation from expired or obsolete inventory.

- Circular Economy: Promotion of circular economy principles through reuse, recycling, and recovery.

- Regulatory Compliance: Enhanced compliance with environmental regulations and customer requirements.

- Community Protection: Protection of communities and ecosystems from pharmaceutical contamination.

Documentation and Recordkeeping

Alchem International maintains comprehensive documentation related to demand forecasting and inventory management:

- Demand Forecasts: Records of demand forecasts, assumptions, and accuracy tracking.

- Production Plans: Records of production plans, schedules, and adjustments.

- Inventory Records: Records of inventory levels, ageing, and disposition.

- Customer Orders: Records of customer orders, contracts, and forecasts.

- Disposition Records: Records of disposition decisions, actions, and certificates.

- Disposal Certificates: Certificates for disposal of expired or obsolete inventory.

- Forecast Accuracy Reports: Reports on forecast accuracy and improvement actions.

- Inventory Reports: Reports on inventory levels, turnover, and waste.

- Compliance Records: Records of regulatory compliance and customer audits.

All records are retained in line with legal requirements and Alchem International's internal document retention policies.

Training and Awareness

Alchem International provides regular training and awareness programmes on demand forecasting and inventory management:

- Induction Training: All new employees receive training on demand forecasting and inventory management.

- Annual Refresher Training: All relevant employees receive annual refresher training.

- Specialised Training: Personnel involved in demand forecasting and inventory management receive specialised training.

- Software Training: Training on ERP and demand planning software.

- Best Practices Sharing: Sharing of best practices and lessons learned.

- Training Records: Documentation of all training activities.

Continuous Improvement

Alchem International fosters a strong culture of continuous improvement in demand forecasting and inventory management through:

- Regular Performance Reviews: Monthly and annual reviews of forecast accuracy, inventory levels, and waste generation.

- Technology Evaluation: Ongoing evaluation of emerging demand forecasting and inventory management technologies.

- Process Improvement: Identification and implementation of process improvements.

- Employee Engagement: Encouraging employee suggestions and Kaizen initiatives.

- Benchmarking: Benchmarking against pharmaceutical industry best practices and customer expectations.

- Training and Awareness: Regular training on demand forecasting and inventory management.

- Customer Feedback: Integration of customer feedback into demand forecasting and inventory management.

Compliance

All demand forecasting and inventory management practices at Alchem International are designed, implemented, and reviewed to meet or exceed:

- Applicable Indian Laws and Regulations: Drugs and Cosmetics Act, 1940; Environment Protection Act, 1986; Hazardous and Other Wastes Rules, 2016; and other applicable regulations.

- International Standards: ICH guidelines, WHO guidelines, and other applicable international standards.

- Customer Requirements: Buyer-specific demand forecasting, inventory management, and reporting requirements.

- Industry Best Practices: Pharmaceutical industry demand forecasting and inventory management guidelines.

Dedicated personnel:

- Monitor regulatory updates and evolving compliance obligations.

- Update SOPs, training modules, and operational controls accordingly.

- Engage with customers, suppliers, and authorities to maintain high standards of demand forecasting, inventory management, and environmental compliance.

# SECTION 11: SAFE HANDLING OF CHEMICALS

Purpose and Scope

This Standard Operating Procedure (SOP) establishes the mandatory requirements for the safe handling, storage, transfer, use, and disposal of chemicals at all Alchem International Private Limited facilities. It applies to every employee, contract worker, trainee, visitor, and service provider who may come into contact with or work near chemicals—including acids, bases, solvents, reagents, catalysts, auxiliaries, cleaning agents, and specialty chemicals—used in extraction, isolation, purification, synthesis, quality control, utilities, and maintenance operations. The SOP is designed to eliminate or minimise the risk of chemical exposure, burns, inhalation, fires, explosions, and environmental contamination.

Chemical Inventory and Hazard Classification

1. Chemical Register

The EHS Department maintains a master Chemical Register listing every chemical used, stored, or generated at the facility. The register is updated whenever a new chemical is introduced or an existing chemical is discontinued. Each entry includes:

- Chemical name (IUPAC and trade name)

- CAS number

- Supplier and SDS version

- Physical state and appearance

- Hazard classification (GHS)

- Storage location and maximum quantity

- Department(s) of use

- Assigned custodian

2. GHS Hazard Classification

All chemicals are classified according to the Globally Harmonized System (GHS) and labelled with the appropriate hazard pictograms, signal words, and precautionary statements. The following hazard classes are recognised and managed:

| Hazard Class | Description | Examples at Alchem International |
| --- | --- | --- |
| Flammable Liquids | Liquids with flash point below 60°C | Methanol, ethanol, acetone, hexane |
| Corrosive Substances | Substances that cause skin corrosion or eye damage | Sulphuric acid, sodium hydroxide, hydrochloric acid |
| Acute Toxicity | Substances toxic by inhalation, ingestion, or skin contact | Certain alkaloids, cyanide salts |
| Oxidisers | Substances that may cause or intensify fire | Hydrogen peroxide, nitric acid |
| Carcinogens / Mutagens | Substances known or suspected to cause cancer or genetic damage | Certain solvents, some botanical extracts |
| Sensitizers | Substances that may cause allergic reactions | Some reagents, formaldehyde |
| Environmental Hazards | Substances hazardous to aquatic life | Solvents, heavy metal compounds |

3. Safety Data Sheets (SDS)

An SDS (Safety Data Sheet) compliant with Indian and international standards (16-section format) is available for every chemical at the point of use and storage. SDSs are:

- Obtained from suppliers before first delivery

- Reviewed by EHS before chemical approval

- Stored physically and digitally

- Made accessible to all personnel

- Updated when supplier provides revised versions

Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Head | Owns this SOP; approves chemical introductions; conducts training; audits compliance; maintains chemical register; investigates incidents. |
| Department Heads | Ensure departmental compliance; nominate trained chemical handlers; verify PPE availability; report deviations to EHS. |
| Chemical Handlers | Follow safe handling procedures; use PPE correctly; report spills, leaks, and near-misses immediately; participate in training. |
| Stores / Warehouse | Ensure proper storage, segregation, labelling, and inventory control; maintain secondary containment; control access. |
| HR / Admin | Coordinate training schedules; maintain training records; support PPE procurement. |
| All Personnel | Report unsafe conditions; do not handle chemicals without training; follow emergency procedures. |

General Chemical Safety Requirements

- No chemical shall be handled by untrained personnel. Only employees who have completed chemical safety induction and task-specific training are authorised to handle chemicals.

- SDS must be reviewed before first use of any chemical.

- PPE is mandatory for all chemical handling activities. The minimum PPE for general chemical handling is:

  - Safety goggles or face shield

  - Chemical-resistant gloves (nitrile, neoprene, or as specified)

  - Lab coat or chemical-resistant apron

  - Closed-toe safety shoes

  - Additional PPE (respirator, full suit) as specified in the SDS or task risk assessment.

- No eating, drinking, smoking, or chewing in chemical handling areas.

- No mouth pipetting is permitted under any circumstances.

- Containers must be kept closed when not in use.

- Secondary containment must be used for all liquid chemicals in storage and during transfer.

- Incompatible chemicals must never be stored together. Segregation is based on the chemical compatibility matrix maintained by EHS.

- Spill kits are available at designated locations and inspected monthly.

- Emergency eyewash and safety showers are available within 10 seconds' travel distance from chemical handling areas and tested weekly.

Specific Handling Procedures

A. Acids

| Step | Requirement |
| --- | --- |
| 1 | Wear acid-resistant gloves, face shield, acid-resistant apron, and safety boots. |
| 2 | Work in a fume hood or well-ventilated area. |
| 3 | Always add acid to water—never water to acid—to control heat generation. |
| 4 | Use secondary containment trays for all transfers. |
| 5 | Use dedicated, labelled containers and funnels. |
| 6 | Never store acids with bases, cyanides, or reactive metals. |
| 7 | Clean up spills immediately using acid neutraliser (sodium bicarbonate or commercial acid spill kit). |
| 8 | In case of skin contact, flush with copious water for at least 15 minutes and seek medical attention. |
| 9 | In case of eye contact, use eyewash for at least 15 minutes and seek immediate medical attention. |
| 10 | Empty containers must be triple-rinsed and sent to designated decontamination area. |

B. Bases (Alkalis)

| Step | Requirement |
| --- | --- |
| 1 | Wear alkali-resistant gloves, goggles or face shield, protective clothing, and safety shoes. |
| 2 | Work in a ventilated area or fume hood. |
| 3 | Add bases slowly to water with continuous stirring to control heat. |
| 4 | Use secondary containment for all transfers. |
| 5 | Never store bases with acids or incompatible substances. |
| 6 | Clean up spills using appropriate neutraliser (citric acid or commercial base spill kit). |
| 7 | In case of skin or eye contact, flush with copious water for at least 15 minutes and seek medical attention. |
| 8 | Empty containers must be decontaminated before disposal. |

C. Solvents

| Step | Requirement |
| --- | --- |
| 1 | Wear solvent-resistant gloves, goggles, and, where required, respiratory protection. |
| 2 | Use only in well-ventilated areas, fume hoods, or closed systems. |
| 3 | Keep away from all ignition sources—no open flames, sparks, or hot surfaces. |
| 4 | Use explosion-proof equipment and bonding/earthing during transfer. |
| 5 | Store in approved flammable storage cabinets or rooms. |
| 6 | Limit quantities at point of use to the minimum required for the task. |
| 7 | Clean up spills using inert absorbent and dispose as hazardous waste. |
| 8 | Never use solvents near hot surfaces or in confined spaces without ventilation. |

D. Toxic and Highly Active Substances (e.g., APIs, Alkaloids)

| Step | Requirement |
| --- | --- |
| 1 | Handle only in designated, controlled areas with restricted access. |
| 2 | Wear full PPE including respirator, coverall, and double gloves. |
| 3 | Use closed systems (glove boxes, isolators) wherever feasible. |
| 4 | Decontaminate all equipment and surfaces after handling. |
| 5 | Dispose of contaminated PPE and materials as hazardous waste. |
| 6 | Pregnant workers and workers with compromised immunity must not handle these substances. |
| 7 | Medical surveillance is provided for personnel routinely handling highly active substances. |

E. Chemicals During Analysis and Testing

| Step | Requirement |
| --- | --- |
| 1 | Wear lab coat, gloves, goggles, and mask as required. |
| 2 | Ensure fume hood airflow is verified before use. |
| 3 | Label all prepared solutions with name, concentration, date, and preparer. |
| 4 | Keep containers closed when not in use. |
| 5 | Do not pipette by mouth. |
| 6 | Clean work area before and after analysis. |
| 7 | Dispose of analytical waste in designated containers. |
| 8 | Report any spill or exposure immediately. |

Spill Response Procedures

1. Classification of Spills

| Category | Definition | Response |
| --- | --- | --- |
| Minor Spill | Small volume (< 1 L), low hazard, trained personnel can handle safely | Handle with spill kit by trained personnel |
| Moderate Spill | Moderate volume (1–10 L), moderate hazard | Evacuate area, call EHS, respond with trained team |
| Major Spill | Large volume (> 10 L), high hazard, or unknown material | Evacuate facility, call emergency services, activate ERP |

2. Spill Response Steps

- Alert – Warn others and notify supervisor/EHS.

- Evacuate – Move people away from the spill area.

- Assess – Identify the chemical (from label or SDS) and the extent of the spill.

- Protect – Don appropriate PPE before approaching.

- Contain – Use absorbent booms or sand to prevent spreading.

- Absorb – Use absorbent pads or appropriate material to soak up the spill.

- Neutralise – Use appropriate neutraliser if required.

- Collect – Place contaminated absorbent in hazardous waste container.

- Clean – Wash the area with appropriate cleaning agent.

- Report – Document the spill and response in the incident register.

- Investigate – Conduct root cause analysis and implement corrective actions.

Emergency Response

1. Chemical Exposure

- Skin Contact: Remove contaminated clothing, flush skin with water for 15 minutes, seek medical attention.

- Eye Contact: Use eyewash for 15 minutes, seek immediate medical attention.

- Inhalation: Move to fresh air, administer oxygen if needed, seek medical attention.

- Ingestion: Do not induce vomiting unless instructed by medical personnel; seek immediate medical attention.

2. Chemical Fire

- Use appropriate extinguisher (CO₂, dry powder, or foam) based on chemical involved.

- Never use water on water-reactive chemicals.

- Evacuate and call fire services if fire is not immediately controllable.

- Follow the Emergency Preparedness & Response SOP.

3. Chemical Release to Environment

- Contain the release immediately.

- Prevent entry into drains, soil, or water bodies.

- Notify EHS and regulatory authorities if required.

- Clean up and dispose of contaminated material as hazardous waste.

Training and Competency

- All chemical handlers receive induction training before first handling.

- Task-specific training is provided for acids, bases, solvents, and highly active substances.

- Refresher training is conducted annually.

- Competency is assessed through written test and practical demonstration.

- Training records are maintained by HR and EHS.

Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| Chemical Register | EHS | 5 years |
| SDS Files | EHS | Current + 3 years |
| Training Records | HR | 5 years |
| Spill Reports | EHS | 5 years |
| PPE Issue Records | Stores | 3 years |
| Medical Surveillance Records | HR / Medical | As per law |
| Chemical Waste Disposal Manifests | EHS | 5 years |

Audit and Review

- Internal audits are conducted quarterly by EHS.

- Third-party audits are conducted annually.

- Audit findings are tracked through CAPA system.

- This SOP is reviewed annually or upon regulatory changes, incidents, or process modifications.

# SECTION 12: ERGONOMIC RISK MANAGEMENT

Background and Rationale

Alchem International Private Limited recognises that repetitive tasks, forceful exertions, awkward postures, prolonged standing or sitting, and static work positions—common in pharmaceutical and phytochemical manufacturing—can result in Repetitive Strain Injuries (RSIs). These injuries may affect muscles, tendons, nerves, joints, and other soft tissues. Activities such as reactor charging, drum handling, filtration, packing, quality inspection, laboratory analysis, and computer-based work are associated with ergonomic risk factors. This SOP establishes structured measures to prevent, identify, and mitigate ergonomic and RSI risks across all work areas, in line with ISO 45001 principles, the Factories Act, 1948, and internationally accepted ergonomic practices.

Definitions

| Term | Definition |
| --- | --- |
| Repetitive Strain Injury (RSI) | A musculoskeletal disorder caused by repetitive motion, overuse, awkward posture, vibration, or excessive force affecting muscles, tendons, ligaments, joints, or nerves. |
| Ergonomic Assessment | A structured evaluation of tasks, tools, workstation design, and worker posture to identify risk factors that may lead to musculoskeletal disorders. |
| Microbreaks | Short, frequent breaks (typically 30–60 seconds) taken during repetitive work to reduce muscle fatigue and cumulative strain. |
| Manual Material Handling (MMH) | Any activity involving lifting, lowering, pushing, pulling, carrying, or holding objects. |
| Awkward Posture | Any fixed or constrained body position that deviates significantly from neutral. |

Ergonomic Risk Assessment Methodology

1. Task Identification

All tasks are screened to identify those with ergonomic risk factors. High-priority tasks include:

- Manual lifting of drums, bags, and containers

- Reactor charging and discharging

- Filtration and centrifugation operations

- Packing and labelling of small containers

- Continuous standing at packing lines

- Continuous sitting at HPLC/GC workstations

- Repetitive pipetting in laboratories

- Use of hand tools and valves

- Computer work for extended periods

2. Risk Assessment Tools

The EHS team uses recognised ergonomic assessment tools, including:

- REBA (Rapid Entire Body Assessment) – for whole-body posture

- RULA (Rapid Upper Limb Assessment) – for upper limb and neck

- NIOSH Lifting Equation – for manual lifting tasks

- OCRA Index – for repetitive upper limb tasks

- Checklist-based assessments – for workstation design

3. Risk Grading

| Risk Level | Action Required |
| --- | --- |
| Low | No action; routine monitoring |
| Medium | Review and improve workstation or task; provide training |
| High | Immediate corrective action; engineering controls; job rotation; medical surveillance |

4. Re-evaluation

High-risk tasks are re-assessed at least once every six months or after any significant process change, new equipment installation, or reported injury.

Prevention and Control Measures

1. Engineering Controls

- Adjustable worktables, chairs, and stools

- Anti-fatigue mats for standing work

- Lifting aids (trolleys, hoists, drum lifters)

- Ergonomic hand tools with cushioned grips

- Adjustable monitor stands and keyboard trays for computer work

- Footrests for seated workstations

- Proper lighting to reduce eye strain

- Vibration-dampening tools and equipment

2. Administrative Controls

- Job rotation between high-repetition and low-strain tasks

- Scheduled microbreaks during continuous repetitive work

- Work-rest cycles based on task intensity

- Limiting overtime for high-risk tasks

- Pre-shift stretching exercises

- Training on proper lifting, posture, and body mechanics

3. PPE

- Anti-vibration gloves for power tool use

- Supportive belts for heavy lifting (where medically appropriate)

- Proper footwear for standing work

4. Workstation Design Principles

- Work height at elbow level for standing tasks

- Work height at seated elbow level for seated tasks

- Reach distance within 40–50 cm

- Adequate legroom and foot support

- Materials positioned to avoid twisting and bending

- Good visibility and lighting

Health Surveillance and Early Intervention

- Pre-employment medical examination includes musculoskeletal screening for high-risk roles.

- Periodic medical examinations (annual or biennial) for workers in high-risk tasks.

- Medical records are confidential and maintained by HR/Medical.

- Employees are encouraged to report discomfort or early symptoms without fear of retaliation.

- Early reporting triggers ergonomic review and intervention.

- Return-to-work programmes for employees recovering from RSI.

Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Department | Conduct ergonomic risk assessments; recommend controls; provide training; maintain records; review effectiveness. |
| Department Heads / Supervisors | Identify high-risk tasks; implement job rotation; ensure controls are followed; report ergonomic issues to EHS. |
| HR Department | Integrate ergonomic awareness into induction and refresher training; monitor health trends; coordinate medical surveillance. |
| Employees | Follow ergonomic practices; use workstations and tools correctly; report discomfort or early symptoms promptly; participate in training. |
| Maintenance / Engineering | Install and maintain ergonomic equipment; modify workstations as recommended by EHS. |

Training and Awareness

- Induction training includes ergonomic awareness for all employees.

- Task-specific training is provided for high-risk activities (lifting, repetitive work, computer use).

- Refresher training is conducted at least annually.

- Training covers:

  - Proper sitting and standing posture

  - Correct lifting and handling techniques

  - Hand, wrist, shoulder, and back exercises

  - Early signs and symptoms of RSI

  - Importance of workstation adjustment

  - Use of microbreaks and job rotation

Recordkeeping

| Record | Responsibility | Retention |
| --- | --- | --- |
| Ergonomic Risk Assessment Reports | EHS | 5 years |
| RSI and Musculoskeletal Complaint Logs | EHS / HR | 5 years |
| Corrective and Preventive Action Records | EHS | 5 years |
| Training Attendance Records | HR | 3 years |
| Occupational Health Screening Records | HR / Medical | As per law |
| Job Rotation Schedules | Department Heads | 2 years |

Review and Continuous Improvement

- This SOP is reviewed annually or whenever new machinery, processes, or layouts are introduced.

- Employee feedback, medical trends, and audit findings are used to strengthen ergonomic controls.

- Improvements are integrated into Alchem International's Occupational Health & Safety Management System to ensure long-term worker well-being.

- Benchmarking against industry best practices and emerging ergonomic standards is conducted periodically.

- The EHS Department reports ergonomic performance to the Management Review Committee annually.

# SECTION 13: SECURITY CONDUCT AND USE-OF-FORCE PREVENTION

1. Why This SOP Exists

Alchem International Private Limited operates manufacturing facilities that handle high-value pharmaceutical actives, controlled substances, proprietary botanical extracts, and sensitive intellectual property. Security personnel are deployed to protect people, assets, and information. However, the presence of security personnel also creates a power dynamic that—if misused—can result in fear, intimidation, coercion, and human rights violations.

This SOP exists to draw a clear, enforceable line: security personnel are protectors, not disciplinarians. They safeguard life, property, and dignity. They do not punish, discipline, restrain, or humiliate. This SOP defines prohibited conduct, the narrow conditions under which force may ever be used, and the accountability mechanisms that ensure violations are detected and addressed.

This SOP is aligned with:

- The UN Basic Principles on the Use of Force and Firearms by Law Enforcement Officials

- The UN Guiding Principles on Business and Human Rights

- The Factories Act, 1948

- The Bharatiya Nyaya Sanhita (BNS), 2023

- ILO Convention 190 on Violence and Harassment

- Customer social compliance requirements (PSCI, Sedex, EcoVadis)

2. Who This Applies To

This SOP applies to:

- All in-house security guards employed by Alchem International

- All third-party security agency personnel deployed at any Alchem International site

- Security supervisors, shift in-charges, and site security leads

- HR, Administration, and Compliance teams with oversight responsibility

- All persons present on Alchem International premises, including permanent employees, contract workers, trainees, apprentices, interns, visitors, vendors, service providers, logistics personnel, and community members visiting the site

Location coverage: Main plant at 25/2, Main Mathura Road, Village Kaili, Ballabhgarh, Faridabad, Haryana 121004; all warehouses, offices, and any future Alchem International site.

3. Definitions That Matter

| Term | Meaning |
| --- | --- |
| Excessive Force | Any physical, verbal, or psychological action beyond what is strictly necessary to address an immediate safety threat. |
| Reasonable Force | The minimum level of force required solely to protect life or prevent serious harm. Never punitive, retaliatory, or disciplinary. |
| Freedom of Movement | The right of individuals to enter, exit, and move freely within the workplace without obstruction, except where temporary restrictions are lawfully required for safety, security, or legal reasons. |
| Detention | Any act of holding, confining, or preventing a person from leaving company premises without lawful authority. |
| Coercion | Any form of pressure, intimidation, manipulation, or threat used to influence an individual's actions or decisions. |
| Silence Zone | Areas near schools, hospitals, places of worship, and residential clusters where honking and loudspeaker use is prohibited. |

4. What Security Personnel Must NEVER Do

The following actions are strictly prohibited under all circumstances. Any violation is treated as a serious human rights violation and will result in immediate removal, contract termination, and where warranted, referral to law enforcement.

4.1 Physical Prohibitions

- Pushing, grabbing, twisting arms, or restraining any person

- Forcing individuals to sit, stand, or remain in one place against their will

- Blocking exits, gates, or pathways

- Dragging, pulling, or physically moving a person

- Any form of corporal punishment

- Unnecessary or inappropriate physical contact

4.2 Verbal and Psychological Prohibitions

- Shouting, threatening, or intimidating language

- Abusive, discriminatory, or derogatory remarks

- Humiliation, body shaming, or aggressive posturing

- Mocking, taunting, or belittling behaviour

- Threats of termination, arrest, or harm

4.3 Procedural Prohibitions

- Confiscation of personal belongings, documents, tools, or mobile phones

- Intentional delays at gates without valid justification

- Acting as disciplinarians or enforcing HR-related matters

- Forcing employees to stand in lines or wait excessively

- Preventing exit during emergencies or medical situations

- Demanding bribes, favours, or personal benefits

- Conducting body searches beyond approved procedures

- Retaining employee identity documents

5. The Only Conditions Under Which Force May Be Used

Force may be used only when all five of the following conditions are simultaneously met:

- There is an immediate threat to life or serious physical harm.

- All lesser measures—verbal instruction, distancing, de-escalation—have failed.

- The force used is minimal, proportionate, and stops immediately once the threat is neutralised.

- Medical assistance is arranged immediately if injury occurs.

- The incident is reported without delay and fully documented.

Force must never be used to:

- Discipline or punish

- Restrict movement

- Assert authority

- Enforce company rules

- Respond to verbal insults or non-compliance

6. Conduct and Communication Standards

6.1 Daily Interaction Standards

| Requirement | Standard |
| --- | --- |
| Tone | Calm, respectful, polite |
| Body language | Non-threatening, open, professional |
| Questions | Courteous inquiry, not interrogation |
| Gestures | Avoid fear-inducing or aggressive gestures |
| Confidentiality | Maintain confidentiality of all interactions |
| Cultural sensitivity | Respect diversity, language, and customs |
| Gender sensitivity | Heightened respect and care with female employees |

6.2 Communication Protocol

- Speak clearly, professionally, neutrally, and without judgment.

- Do not raise voice unless absolutely necessary for safety.

- Do not use sarcasm, threats, or ultimatums.

- Do not discuss employee matters with other employees.

- Do not share gate entry/exit information with unauthorised persons.

7. Freedom of Movement: What Security Must Allow

Security personnel must allow:

- Entry and exit at scheduled shift times without obstruction

- Exit during emergencies, medical needs, or personal crises

- Access to authorised work areas without unjustified restrictions

- Movement within the premises as per operational permissions

Strictly prohibited:

- Blocking gates during shift changes

- Preventing exit during labour issues or disputes

- Confiscating gate passes

- Locking exits without authorisation

- Group detainment for "checking"

Movement restrictions may only be authorised by HR/Admin during:

- Emergency lockdowns

- Verified security breaches

- Legally mandated investigations

8. Training Requirements

8.1 Mandatory Training Modules

| Module | Content | Frequency |
| --- | --- | --- |
| Human Rights and Ethical Treatment | Dignity, respect, non-discrimination | Before deployment |
| Legal Consequences of Excessive Force | BNS provisions, civil liability | Before deployment |
| Grievance Reporting Mechanisms | How to report, how to respond | Before deployment |
| Conflict De-escalation | Verbal techniques, distancing, distraction | Before deployment |
| Communication and Empathy | Active listening, respectful inquiry | Before deployment |
| Gender Sensitivity | Respectful conduct, avoiding harassment | Before deployment |
| Emergency Response and First Aid | Basic life support, incident response | Before deployment |
| CCTV Use and Documentation | Legal use, privacy, evidence handling | Before deployment |

8.2 Training Frequency

- Before deployment: Full induction training

- Quarterly: Refresher sessions

- After any misconduct: Mandatory retraining

- Annually: Advanced de-escalation and human rights training

9. Monitoring, Audits, and Oversight

| Activity | Frequency | Responsibility |
| --- | --- | --- |
| Daily conduct verification | Daily | Security Supervisor |
| CCTV review of gates and access points | Weekly | HR / Admin |
| Behavioural audits and random employee interviews | Monthly | HR |
| Surprise checks | Quarterly | HR / Administration |
| Third-party security agency compliance audit | Quarterly | HR / Compliance |
| Review of incident logs | Monthly | HR / EHS |

Key records maintained:

- Attendance and duty rosters

- Briefing and training registers

- Incident and escalation logs

- Corrective action and follow-up records

- CCTV footage (retained as per policy)

10. Incident Reporting Process

10.1 Reporting Channels

Any employee, contract worker, or visitor who experiences or witnesses excessive force, harassment, intimidation, or obstruction by security personnel may report through:

- Direct report to HR (in person, phone, email)

- Report to Administration Head

- Anonymous drop box

- Grievance committee

- Third-party ethics hotline (where available)

10.2 Investigation Steps

| Step | Action | Timeline |
| --- | --- | --- |
| 1 | Incident reported to HR, Admin, or Security Supervisor | Immediate |
| 2 | Immediate safety and support provided if required | Immediate |
| 3 | HR acknowledges complaint | Within 24 hours |
| 4 | CCTV footage secured and reviewed | Within 48 hours |
| 5 | Statements collected from all parties | Within 72 hours |
| 6 | Investigation completed | Within 3 working days (unless severe) |
| 7 | Corrective actions finalised and communicated | Within 5 working days |
| 8 | Follow-up to ensure no retaliation | Within 30 days |

10.3 Incident Classification

| Classification | Description | Action |
| --- | --- | --- |
| Minor Misconduct | Rude behaviour, minor procedural deviation | Verbal/written warning, retraining |
| Major Misconduct | Intimidation, obstruction, unauthorised detention | Suspension, retraining, possible removal |
| Human Rights Violation | Physical force, harassment, coercion | Immediate removal, contract termination, legal referral |
| Criminal Offence | Assault, extortion, unlawful confinement | Immediate termination, police referral |

11. Disciplinary Actions

Depending on severity, actions may include:

- Verbal or written warning

- Immediate suspension

- Mandatory retraining

- Removal by security agency

- Blacklisting of guard or agency

- Termination of security contract

- Reporting to authorities where legally required

Alchem International maintains zero tolerance for violence, harassment, or intimidation.

12. Additional Safeguards

- Security personnel must never hold employee identity documents.

- Female employees must be treated with heightened sensitivity and respect.

- Bag checks are conducted only as per approved procedures, by trained personnel, and with dignity.

- Security must not intervene in HR or grievance matters.

- Entry into production areas requires authorisation.

- Guards must remain at assigned posts unless instructed otherwise.

- All suspicious or unsafe situations must be reported immediately.

- Security personnel must not carry firearms or lethal weapons on premises.

- Any use of force must be reported within 1 hour to HR and EHS.

13. Community and Neighbour Relations

Alchem International's facility on Main Mathura Road is located near residential and commercial areas. Security personnel at gates and perimeter must:

- Be courteous to neighbours, commuters, and visitors

- Avoid unnecessary honking or loudspeaker use

- Manage traffic politely during shift changes

- Report any community concerns to Administration

- Not engage in arguments or confrontations with community members

14. Review and Continuous Improvement

- This SOP is reviewed annually or after any major incident.

- Feedback from employees, security personnel, and community is considered.

- Training modules are updated based on emerging best practices.

- Audit findings and incident trends are analysed for systemic improvements.

- The HR Head presents an annual security conduct report to the Management Review Committee.

# SECTION 14: CARBON CAPTURE, STORAGE & EMISSIONS REDUCTION

1 Preamble: Why Carbon Capture Matters to Alchem International

Alchem International Private Limited operates energy-intensive pharmaceutical and phytochemical manufacturing processes—including botanical extraction, solvent distillation, purification, drying, and utilities—that generate carbon dioxide (CO₂) emissions from:

- Boilers and thermic fluid heaters (fuel combustion)

- Diesel generators (backup power)

- Solvent recovery and distillation (energy demand)

- Manufacturing processes (chemical reactions)

- Transportation and logistics

As India commits to its Net Zero by 2070 target and as global pharmaceutical customers increasingly demand Scope 1 and Scope 2 emissions reductions, Alchem International recognises that carbon capture and storage (CCS) is a critical technology for decarbonising hard-to-abate industrial emissions. This SOP establishes the framework for evaluating, implementing, operating, and maintaining carbon capture, utilisation, and storage (CCUS) technologies at Alchem International facilities.

This SOP is aligned with:

- The Paris Agreement and India's Nationally Determined Contributions (NDCs)

- The Environment Protection Act, 1986

- CPCB guidelines on industrial emissions

- Bureau of Indian Standards (BIS) guidelines on CO₂ capture and storage

- ISO 27914:2017 (Carbon dioxide capture, transportation and geological storage)

- Customer requirements for carbon footprint reduction and ESG reporting

2 Carbon Emission Inventory: Baseline and Sources

2.1 Scope 1 Emissions (Direct)

| Emission Source | Type of Fuel / Process | Estimated CO₂ Emissions (tCO₂e/year) | Capture Potential |
| --- | --- | --- | --- |
| Boiler #1 | Natural Gas / Diesel | [To be filled] | High (flue gas capture) |
| Boiler #2 | Natural Gas / Diesel | [To be filled] | High (flue gas capture) |
| Thermic Fluid Heater | Diesel / Furnace Oil | [To be filled] | High (flue gas capture) |
| Diesel Generators (DG Set 1) | Diesel | [To be filled] | Medium (intermittent operation) |
| Diesel Generators (DG Set 2) | Diesel | [To be filled] | Medium (intermittent operation) |
| Solvent Recovery Unit | Steam / Electricity | [To be filled] | Indirect (energy source) |
| Process Reactions | Chemical | [To be filled] | Low (diffuse) |
| Company Vehicles | Diesel / Petrol | [To be filled] | Low (mobile source) |

2.2 Scope 2 Emissions (Indirect – Purchased Electricity)

| Source | Estimated Consumption (kWh/year) | Estimated CO₂ Emissions (tCO₂e/year) |
| --- | --- | --- |
| Grid Electricity (Haryana) | [To be filled] | [To be filled] |
| Captive Power (DG) | [To be filled] | [To be filled] |
| Renewable Energy (Solar) | [To be filled] | 0 (offset) |

2.3 Scope 3 Emissions (Value Chain – Optional / Estimated)

| Category | Description | Estimated Emissions |
| --- | --- | --- |
| Purchased Goods & Services | Raw materials, chemicals, packaging | [To be filled] |
| Upstream Transportation | Inbound logistics | [To be filled] |
| Waste Generated | Waste treatment and disposal | [To be filled] |
| Business Travel | Employee travel | [To be filled] |
| Downstream Transportation | Outbound logistics | [To be filled] |

3 Carbon Capture Technologies: Assessment and Selection

Alchem International has evaluated the following carbon capture technologies for applicability to its operations. The selection matrix is based on technical feasibility, cost, energy penalty, and integration potential.

3.1 Post-Combustion Capture (PCC)

| Parameter | Specification |
| --- | --- |
| Technology | Amine-based chemical absorption |
| Application | Flue gas from boilers, thermic fluid heaters, DG sets |
| CO₂ Capture Efficiency | 85–95% |
| Energy Penalty | 2.5–4.0 GJ/tCO₂ |
| Solvent | MEA (Monoethanolamine) or advanced amine blend |
| Integration | Retrofit to existing stacks |
| Footprint | Moderate – absorber, regenerator, reboiler, heat exchangers |
| Status | Technically feasible; under detailed engineering evaluation |

3.2 Oxy-Fuel Combustion

| Parameter | Specification |
| --- | --- |
| Technology | Combustion in oxygen-rich environment |
| Application | New boilers / heaters (not retrofit) |
| CO₂ Capture Efficiency | > 90% |
| Energy Penalty | High (air separation unit) |
| Integration | Requires new equipment |
| Footprint | High (ASU + boiler modification) |
| Status | Not feasible for existing assets; considered for future expansion |

3.3 Pre-Combustion Capture

| Parameter | Specification |
| --- | --- |
| Technology | Gasification / reforming + CO₂ separation |
| Application | Synthesis gas production |
| CO₂ Capture Efficiency | 80–90% |
| Energy Penalty | Moderate |
| Integration | Requires process modification |
| Footprint | High |
| Status | Not applicable to current operations; evaluated for future synthesis blocks |

3.4 Direct Air Capture (DAC)

| Parameter | Specification |
| --- | --- |
| Technology | Chemical absorption from ambient air |
| Application | Offsetting residual emissions |
| CO₂ Capture Efficiency | 70–90% (per unit) |
| Energy Penalty | Very high |
| Integration | Standalone |
| Footprint | High |
| Status | Evaluated as long-term option; not immediate due to cost and energy demand |

4 Carbon Utilisation Pathways

Captured CO₂ can be utilised in the following ways, either on-site or through partnerships:

| Utilisation Pathway | Description | Applicability at Alchem International |
| --- | --- | --- |
| Enhanced Oil Recovery (EOR) | Injection into oil reservoirs | Not applicable (no oil fields) |
| Mineral Carbonation | Reaction with minerals to form stable carbonates | Feasible for construction materials |
| CO₂ to Chemicals | Conversion to methanol, urea, polymers | Feasible with technology partners |
| CO₂ to Fuels | Conversion to synthetic fuels | Long-term option |
| Food & Beverage Grade CO₂ | Purification for use in food/beverage industry | Feasible if purity achieved |
| Greenhouse Enrichment | CO₂ for greenhouse agriculture | Feasible for nearby farms |
| Algae Cultivation | CO₂ for algae growth (biofuels, nutraceuticals) | Feasible with partners |

13.5 Carbon Storage Options

Where utilisation is not feasible, captured CO₂ is stored using the following methods:

| Storage Option | Description | Applicability |
| --- | --- | --- |
| Geological Storage | Injection into deep saline aquifers or depleted oil/gas fields | Not feasible on-site; requires regional storage hub |
| Mineral Storage | Reaction with basalt or olivine to form carbonates | Long-term; requires partner facilities |
| Deep Ocean Storage | Injection into deep ocean | Not permitted under Indian regulations |
| On-Site Temporary Storage | Pressurised tanks for short-term storage before transport | Feasible for small volumes |

Note: For Alchem International, given the absence of on-site geological storage, the primary pathway will be capture + utilisation or capture + transport to a regional storage hub (where available).

6 CCS System Design and Integration

6.1 System Components

| Component | Function |
| --- | --- |
| Flue Gas Pre-Treatment | Cooling, particulate removal, SOx/NOx polishing |
| CO₂ Absorber | Chemical absorption of CO₂ from flue gas |
| Rich Solvent Pump | Transfer of CO₂-rich solvent to regenerator |
| Regenerator / Stripper | Release of CO₂ from solvent using heat |
| Reboiler | Provides heat for solvent regeneration |
| Condenser | Condenses water and solvent vapours |
| CO₂ Compressor | Compresses CO₂ for transport/storage |
| CO₂ Dryer | Removes moisture from CO₂ stream |
| CO₂ Storage Tanks | Temporary storage of compressed CO₂ |
| Lean Solvent Cooler | Cools regenerated solvent for reuse |
| Solvent Filtration | Removes degradation products and particulates |
| Control System | DCS/PLC-based monitoring and control |

6.2 Integration with Existing Utilities

- Steam Supply: Regeneration heat is supplied from existing boilers or waste heat recovery.

- Cooling Water: Cooling water is supplied from existing cooling towers.

- Power Supply: Additional power demand met through existing electrical infrastructure or captive solar.

- Control Room: Integration with existing DCS/SCADA.

7 Operational Procedures

7.1 Start-Up Procedure

- Verify all equipment is in safe condition.

- Confirm solvent inventory and concentration.

- Start flue gas pre-treatment system.

- Start cooling water circulation.

- Start lean solvent pump.

- Gradually introduce flue gas to absorber.

- Start regenerator reboiler.

- Monitor CO₂ purity and capture efficiency.

- Start CO₂ compressor when CO₂ flow stabilises.

- Record all parameters in the CCS logbook.

7.2 Normal Operation

| Parameter | Normal Range | Alarm Setpoint |
| --- | --- | --- |
| Flue Gas Flow | [Design value] | ±10% |
| Flue Gas Temperature | 40–60°C | > 70°C |
| Solvent Flow Rate | [Design value] | ±10% |
| CO₂ Capture Efficiency | > 85% | < 80% |
| Regenerator Temperature | 110–130°C | > 140°C |
| Reboiler Steam Pressure | [Design value] | ±10% |
| CO₂ Purity | > 95% | < 90% |
| CO₂ Compressor Pressure | [Design value] | ±10% |
| Solvent pH | 8.5–10.5 | < 8.0 |
| Solvent Concentration | [Design value] | ±5% |

7.3 Shutdown Procedure

- Stop flue gas feed to absorber.

- Continue lean solvent circulation for 15 minutes.

- Stop reboiler steam supply.

- Stop CO₂ compressor.

- Stop cooling water circulation.

- Drain and store solvent as per procedure.

- Inspect equipment for corrosion, fouling, or damage.

- Record shutdown parameters.

7.4 Emergency Shutdown

| Trigger | Action |
| --- | --- |
| High CO₂ concentration in treated gas | Stop flue gas feed; increase solvent flow; investigate |
| High regenerator temperature | Cut steam; increase cooling; check for solvent degradation |
| Solvent leak | Stop system; contain spill; use spill kit; notify EHS |
| CO₂ leak | Evacuate area; ventilate; use SCBA; notify EHS |
| Power failure | Activate emergency shutdown; close valves; secure system |

8 Monitoring, Reporting, and Verification (MRV)

8.1 Monitoring Parameters

| Parameter | Frequency | Method |
| --- | --- | --- |
| Flue gas flow rate | Continuous | Flow meter |
| Flue gas CO₂ concentration | Continuous | NDIR analyser |
| Treated gas CO₂ concentration | Continuous | NDIR analyser |
| CO₂ capture efficiency | Calculated | Mass balance |
| Solvent concentration | Daily | Titration / HPLC |
| Solvent degradation | Weekly | GC-MS / ion chromatography |
| CO₂ purity | Continuous | GC / NDIR |
| Energy consumption | Continuous | Power meter |
| Steam consumption | Continuous | Flow meter |
| Water consumption | Continuous | Flow meter |

8.2 Reporting

- Daily: CCS operation log

- Weekly: Performance summary

- Monthly: CO₂ captured, utilised, stored; energy penalty; solvent consumption

- Quarterly: CCS performance report to management

- Annually: CCS report for ESG disclosure and regulatory compliance

8.3 Verification

- Internal verification by EHS and Engineering

- Third-party verification by accredited agency (annually or as required)

- Verification includes:

  - CO₂ mass balance

  - Energy consumption

  - Solvent management

  - Storage / utilisation documentation

9 Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Managing Director | Approves CCS strategy and investment |
| EHS Head | Owns CCS environmental compliance; monitors emissions; reports to management |
| Engineering Head | Designs, installs, operates, and maintains CCS systems |
| Production Head | Integrates CCS with production operations |
| Maintenance Team | Maintains CCS equipment; calibration; spare management |
| Procurement | Sources solvents, spares, and consumables |
| Finance | Tracks CCS costs and carbon credits (if applicable) |
| All Operators | Follow CCS SOPs; monitor parameters; report deviations |

10 Training and Competency

| Training Module | Audience | Frequency |
| --- | --- | --- |
| CCS Fundamentals | All relevant staff | Induction |
| CCS Operation | Operators | Before deployment |
| CCS Maintenance | Maintenance staff | Before deployment |
| CCS Emergency Response | All relevant staff | Annually |
| Solvent Handling | Operators | Annually |
| CO₂ Safety | All relevant staff | Annually |

11 Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| CCS Design Documents | Engineering | Life of asset |
| CCS Operating Logs | Operations | 5 years |
| CCS Maintenance Records | Maintenance | 5 years |
| Solvent Analysis Reports | QC / EHS | 5 years |
| CO₂ Monitoring Data | EHS | 5 years |
| CCS Performance Reports | EHS | 5 years |
| Third-Party Verification Reports | EHS | 5 years |
| Training Records | HR | 5 years |

12 Regulatory Compliance

| Regulation | Requirement | Compliance |
| --- | --- | --- |
| Environment Protection Act, 1986 | Emission standards | Compliant |
| CPCB Guidelines | Industrial emissions | Compliant |
| ISO 27914:2017 | CO₂ geological storage | Followed where applicable |
| Customer ESG Requirements | Carbon footprint reduction | Reported |
| BIS Guidelines | CO₂ capture and storage | Followed |

13 Continuous Improvement

- Annual review of CCS performance and technology options

- Evaluation of next-generation solvents (lower energy penalty)

- Evaluation of membrane-based capture (lower footprint)

- Integration with renewable energy for regeneration heat

- Participation in industry CCS consortia

- Carbon credits and offset opportunities

- Employee suggestion schemes for emission reduction

# SECTION 15: OPERATIONAL ADAPTATION TO ENVIRONMENTAL DISRUPTIONS

1 Purpose and Context

Alchem International Private Limited operates in Faridabad, Haryana—a region that experiences:

- Extreme heat waves (summer temperatures exceeding 45°C)

- Monsoon flooding (localized waterlogging and drainage issues)

- Air quality emergencies (severe AQI episodes, especially in winter)

- Water scarcity (declining groundwater tables, erratic supply)

- Dust storms (pre-monsoon)

- Unseasonal rainfall (affecting logistics and raw material supply)

These environmental disruptions can directly impact operations, including production schedules, employee safety, utility availability, logistics, and community relations. This SOP establishes the operational adaptations Alchem International has implemented to manage and mitigate the effects of environmental disruptions while ensuring business continuity.

This SOP is aligned with:

- Disaster Management Act, 2005

- National Disaster Management Authority (NDMA) guidelines

- Haryana State Disaster Management Authority directives

- Factories Act, 1948 (safety provisions)

- ISO 22301 (Business Continuity Management)

- Customer requirements for supply chain resilience

2 Environmental Disruption Scenarios and Operational Adaptations

2.1 Extreme Heat Waves

| Aspect | Adaptation |
| --- | --- |
| Production Scheduling | Outdoor maintenance, material movement, and non-critical activities are shifted to early morning (6:00–10:00 AM) or late evening (6:00–9:00 PM). Critical indoor production continues with enhanced cooling. |
| Employee Safety | Heat stress awareness training; hydration stations at multiple locations; electrolytes provided; mandatory rest breaks every 90 minutes for outdoor workers; cooling rooms with ACs and fans. |
| Cooling Systems | Chillers and HVAC systems are serviced pre-summer; backup chillers available; cooling tower efficiency monitored; additional fans installed in production areas. |
| Utilities | Standby DG sets tested; power factor correction; UPS for critical controls; solar panels (where installed) provide shading and power. |
| Raw Materials | Heat-sensitive raw materials stored in temperature-controlled warehouses; cold chain integrity checked more frequently. |
| Logistics | Deliveries scheduled during cooler hours; drivers provided with hydration and rest; vehicle AC checked; transport of heat-sensitive products in insulated/refrigerated vehicles. |
| Community | Drinking water provided to nearby residents if requested; community awareness on heat safety. |

2.2 Monsoon Flooding

| Aspect | Adaptation |
| --- | --- |
| Site Drainage | Stormwater drains cleaned pre-monsoon; additional drains created in low-lying areas; pumping systems installed; flood barriers at critical entry points. |
| Production | Raw materials and finished goods stored on elevated platforms; critical equipment raised; water-sensitive processes rescheduled. |
| Utilities | Electrical panels raised; backup power ready; water supply protected from contamination. |
| Logistics | Alternate routes pre-identified; GPS tracking for real-time rerouting; delivery schedules adjusted for road conditions. |
| Employee Safety | Flood warnings issued; employees advised to avoid waterlogged areas; emergency contact numbers circulated; work-from-home options where feasible. |
| Community | Coordination with local authorities; assistance to nearby residents if safe and feasible. |

2.3 Air Quality Emergencies

| Aspect | Adaptation |
| --- | --- |
| Production | Non-critical outdoor activities suspended; indoor air quality monitored; air purifiers deployed in offices and control rooms. |
| Employee Safety | N95 masks provided; outdoor work limited; vulnerable employees (respiratory issues) given indoor duties; medical support available. |
| Utilities | Boiler and DG emissions monitored; stack heights optimised; APC systems operated at maximum efficiency. |
| Logistics | Vehicle idling reduced; electric vehicles prioritised for short trips; route optimisation to reduce emissions. |
| Community | Company participates in local air quality management initiatives; awareness programmes. |

2.4 Water Scarcity

| Aspect | Adaptation |
| --- | --- |
| Production | Water-intensive processes rescheduled or reduced; water recycling and reuse maximised; closed-loop cooling systems prioritised. |
| Utilities | Cooling tower cycles of concentration optimised; blowdown minimised; rainwater harvesting utilised. |
| Employee Facilities | Low-flow fixtures; water-saving awareness; leak detection and repair within 24 hours. |
| Community | Water conservation awareness; no competition with community water needs; alternative supply arrangements (tankers) if required. |

2.5 Dust Storms

| Aspect | Adaptation |
| --- | --- |
| Production | Outdoor activities suspended; windows and doors closed; ventilation systems set to recirculation mode with filters. |
| Employee Safety | Masks provided; outdoor workers moved indoors; eye protection available. |
| Equipment | Sensitive equipment covered; air filters checked and replaced more frequently. |
| Logistics | Deliveries postponed if visibility is low; vehicles checked for air filter condition. |

3 Business Continuity Framework

3.1 Business Continuity Plan (BCP)

Alchem International maintains a documented BCP that covers:

- Risk assessment for environmental disruptions

- Critical business functions and dependencies

- Recovery time objectives (RTOs) and recovery point objectives (RPOs)

- Alternate operating procedures

- Emergency contact lists

- Resource requirements

- Communication protocols

3.2 Critical Business Functions

| Function | RTO | RPO | Dependencies |
| --- | --- | --- | --- |
| API Manufacturing | 48 hours | 24 hours | Power, water, raw materials |
| Quality Control | 24 hours | 12 hours | Power, lab supplies |
| Utilities (Boiler, Cooling) | 12 hours | 6 hours | Fuel, water, power |
| Warehousing | 24 hours | 12 hours | Power, access |
| Logistics | 48 hours | 24 hours | Roads, vehicles, fuel |
| IT Systems | 4 hours | 2 hours | Power, internet, servers |

3.3 Alternate Operating Procedures

| Disruption | Alternate Procedure |
| --- | --- |
| Power failure | DG backup; critical loads prioritised; manual operations where feasible |
| Water failure | Stored water; tanker supply; process adjustments |
| Flooding | Elevated storage; alternate routes; remote operations |
| Heat wave | Night operations; reduced load; enhanced cooling |
| Air quality emergency | Indoor operations; air purification; remote work |

4 Ecosystem Services and Natural Buffers

Alchem International recognises that local ecosystem services—such as soil stability, natural stormwater drainage, and green buffers—are essential for operational resilience. The following measures are implemented:

| Ecosystem Service | Adaptation |
| --- | --- |
| Soil Erosion Control | Green belt development; vegetation on slopes; erosion barriers |
| Stormwater Management | Natural drainage channels maintained; permeable surfaces; rainwater harvesting |
| Green Buffers | Plantation around perimeter; dust and noise absorption; biodiversity support |
| Local Water Recharge | Rainwater harvesting; recharge pits; avoiding groundwater over-abstraction |
| Air Quality | Green belt; emission controls; dust suppression |

5 Logistics and Access Route Adaptation

Alchem International has identified alternate access routes to avoid flood-prone areas and maintain logistics during disruptions:

| Route | Normal Use | Alternate Route |
| --- | --- | --- |
| Main Mathura Road (NH-2) | Primary access | Sector 58–59 bypass; Old Faridabad Road |
| Village Kaili Road | Secondary access | Kheri Gujran Road; Mujesar Road |
| Ballabhgarh Bypass | Heavy vehicle route | Mohna Road; Sohna Road (for southbound) |
| NH-2 to Delhi | Northbound | Faridabad Bypass; Surajkund Road |

Routes are reviewed pre-monsoon and updated based on road conditions.

6 Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Managing Director | Approves BCP; allocates resources; reviews performance |
| EHS Head | Owns BCP; monitors environmental disruptions; coordinates response |
| Operations Head | Implements production adaptations; ensures employee safety |
| HR Head | Communicates with employees; manages welfare; training |
| Logistics Head | Manages transport adaptations; alternate routes; driver safety |
| Maintenance Head | Maintains backup equipment; flood protection; cooling systems |
| All Employees | Follow adaptation procedures; report disruptions; support response |

7 Training and Awareness

- Annual BCP training for all relevant staff

- Mock drills for flood, heat wave, and power failure scenarios

- Awareness programmes on environmental disruptions

- Community awareness on company preparedness

- Training records maintained

8 Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| BCP Document | EHS | 5 years |
| Risk Assessment Reports | EHS | 5 years |
| Mock Drill Reports | EHS | 3 years |
| Adaptation Implementation Records | Operations | 3 years |
| Training Records | HR | 3 years |
| Incident Reports | EHS | 5 years |
| Community Communication Records | Admin | 3 years |

9 Continuous Improvement

- Annual review of BCP and adaptation measures

- Post-event reviews after each disruption

- Integration of lessons learned into BCP

- Benchmarking against industry best practices

- Employee feedback and suggestions

- Community feedback integration

# SECTION 16: CONFIDENTIAL DATA MANAGEMENT AND RECORD RETENTION PROTOCOL

1. Introduction

Alchem International Private Limited is entrusted with confidential, proprietary, and sensitive information by employees, customers, buyers, suppliers, contractors, auditors, regulators, and other stakeholders. As a manufacturer operating within the pharmaceutical, phytochemical, nutraceutical, and cosmeceutical value chain—serving global customers with strict compliance, quality, and social responsibility expectations—robust data governance is essential to ensure regulatory compliance, operational integrity, and stakeholder trust.

This Standard Operating Procedure (SOP) establishes structured requirements for the collection, processing, storage, safeguarding, retention, and secure disposal of confidential information. It aligns with:

- The Digital Personal Data Protection Act (DPDP Act), 2023

- The Information Technology Act, 2000 and IT (Reasonable Security Practices) Rules, 2011

- The Drugs and Cosmetics Act, 1940

- Customer confidentiality agreements

- ISO 27001 principles

- Contractual confidentiality obligations

- Applicable international data protection requirements

2. Objectives

This SOP aims to:

- Ensure that all confidential and sensitive data is collected and processed lawfully, transparently, and for legitimate business purposes.

- Protect internal and third-party data through controlled access, secure IT systems, physical safeguards, and monitored handling practices.

- Establish a clear and compliant record retention schedule aligned with statutory, customer, audit, and business requirements.

- Maintain transparency by informing stakeholders how their data is collected, used, stored, shared, and retained.

- Restrict access to confidential information through role-based permissions, need-to-know principles, and data minimisation.

- Continuously review and strengthen data protection measures to address emerging risks, regulatory updates, and technological changes.

- Provide defined internal channels for employees and stakeholders to raise queries or concerns related to data privacy and handling.

- Ensure secure, irreversible destruction of records once retention periods expire, preventing unauthorised recovery or misuse.

3. Applicability and Coverage

3.1 Who This SOP Applies To

- All permanent employees

- Contract workers and temporary staff

- Consultants and service providers

- Security personnel and outsourced vendors

- Interns, apprentices, and trainees

who access, process, manage, or store confidential or sensitive information on behalf of Alchem International.

3.2 What This SOP Covers

This protocol applies to both digital and physical records handled across:

- Corporate and administrative offices

- Production floors and manufacturing units

- HR, payroll, and compliance departments

- Quality, compliance, and audit functions

- Warehouses, logistics, and supply-chain operations

- Customer liaison and commercial communication

- Research and development laboratories

- Remote or field-based work environments

This SOP remains applicable during routine operations, audits, inspections, emergencies, and business continuity situations.

4. Data Classification Framework

All information at Alchem International is classified into one of four tiers. The classification determines handling, access, storage, and disposal requirements.

| Classification | Definition | Examples | Access Level |
| --- | --- | --- | --- |
| Public | Information approved for public release | Website content, brochures, press releases | Open |
| Internal | Information for internal use only | Internal circulars, meeting minutes, general policies | All employees |
| Confidential | Sensitive business information | Customer contracts, pricing, production plans, quality reports | Need-to-know basis |
| Restricted | Highly sensitive information | API formulations, proprietary processes, employee personal data, payroll, buyer specifications | Named individuals only |

5. Data Collection and Consent

5.1 Lawful Basis for Collection

Alchem International collects personal and business data only for legitimate purposes, including:

- Employment and payroll administration

- Regulatory compliance (Drugs and Cosmetics Act, Factories Act, labour laws)

- Customer contract fulfilment

- Supplier and vendor management

- Audit and compliance verification

- Health and safety management

5.2 Consent Collection

Stakeholders are provided with consent documentation that clearly explains:

- Purpose and scope of data collection

- Categories of data processed

- Legal basis and retention duration

- Conditions for sharing with buyers, auditors, regulators, or service providers

- Rights related to access, correction, and withdrawal

All forms use simple, transparent language.

5.3 Consent Capture Channels

Consent may be obtained through:

- HR onboarding documentation

- Digital forms or secure portals

- Email confirmations

- Vendor onboarding processes

- Buyer or auditor data-sharing agreements

All consent records are dated, version-controlled, and securely archived.

5.4 Digital Consent Management

- Managed through secure HR or ERP systems

- Access restricted to authorised personnel

- Audit trails maintained for all updates or withdrawals

5.5 Periodic Consent Review

- Conducted annually or when policies change

- Stakeholders notified if renewed consent is required

5.6 Consent Withdrawal

- Requests processed within defined timelines

- Systems updated promptly

- Prior lawful processing remains valid

6. Operational Transparency and Stakeholder Communication

- Stakeholders are informed of data practices through notices, contracts, policies, and official communication.

- Relevant documentation is shared upon legitimate request.

- Queries are handled by HR, Compliance, or designated data custodians.

- Significant changes in data practices are formally communicated.

7. Measures to Protect Third-Party and Internal Data

7.1 Role-Based Access Control

- Access limited to operational necessity

- Approved by department heads and IT

- Periodic access reviews conducted

7.2 Digital Security Controls

| Control | Description |
| --- | --- |
| Secure servers | Physically secured, access-controlled server rooms |
| Encryption | Data at rest and in transit encrypted using industry-standard algorithms |
| Firewalls | Network perimeter protection |
| Antivirus | Endpoint protection on all devices |
| Intrusion detection | Monitoring for unauthorised access attempts |
| Controlled backups | Regular backups with encrypted storage |
| Disaster recovery | Documented and tested recovery procedures |
| Vulnerability reviews | Regular system and application vulnerability assessments |

7.3 Physical Security Controls

| Control | Description |
| --- | --- |
| Restricted access | Records rooms and server areas accessible only to authorised personnel |
| CCTV surveillance | Coverage of sensitive locations |
| Secure cabinets | Lockable cabinets for personnel, payroll, and compliance files |
| Visitor management | Escorted access for visitors |
| Clean desk policy | No confidential documents left unattended |

7.4 Awareness and Monitoring

- Regular employee awareness sessions

- Monitoring of access logs and unusual activity

- Immediate escalation of suspected breaches

8. Internal Controls and Audit Oversight

- Quarterly access reviews and system checks

- Internal audits aligned with quality, compliance, and customer requirements

- Corrective actions tracked for any gaps identified

- Annual third-party security audit (where required by customers)

9. Record Retention Schedule

| Document Type | Retention Period | Legal / Business Basis |
| --- | --- | --- |
| Employee Records | 5–7 years after separation | Factories Act, labour laws |
| Payroll & Financial Records | 7–8 years | Income Tax Act, Companies Act |
| Buyer & Supplier Contracts | Contract term + 5–7 years | Contractual, audit |
| Legal & Compliance Records | As per legal requirements | Statutory |
| Health & Safety Records | 5 years | Factories Act, OHS |
| Training Records | 3 years | Audit, compliance |
| Incident & Investigation Reports | 5 years | OHS, legal |
| Quality & Audit Records | 5 years | GMP, customer |
| Equipment & Maintenance Logs | 5 years | GMP, OHS |
| Environmental & Social Compliance Records | 3–5 years | CPCB/SPCB, customer |
| Consent Forms | 7 years or until withdrawn | DPDP Act |
| ERP Transaction Logs | Minimum 3 years | Audit, compliance |
| CCTV Footage | 30–90 days (unless incident) | Security, legal |
| Laboratory Records | 5 years | GMP, regulatory |
| Batch Manufacturing Records | 5 years or as per Drugs & Cosmetics Act | GMP |
| Stability Study Records | Product shelf life + 1 year | GMP |

10. Secure Disposal of Records

10.1 Physical Records

- Shredded or destroyed through authorised methods

- Cross-cut shredding for confidential documents

- Incineration for highly sensitive documents

- Disposal logged and verified by responsible personnel

10.2 Digital Records

- Permanently deleted using secure deletion protocols

- Hard drives and storage media wiped or physically destroyed

- Cloud data deleted as per provider protocols and verified

- Disposal logged and verified by IT and Compliance

10.3 Disposal Verification

- Disposal certificates obtained from authorised vendors

- Records maintained for audit purposes

11. Data Breach Response

11.1 Immediate Actions

- Contain the breach (disconnect affected systems, change passwords)

- Notify the Data Protection Officer / Compliance Head

- Preserve evidence (logs, screenshots)

- Do not attempt independent fixes

11.2 Investigation

- Assess the scope and impact

- Identify root cause

- Determine legal and contractual notification requirements

11.3 Notification

- Notify affected individuals if personal data is compromised

- Notify regulators if required under DPDP Act

- Notify customers if contractual data is affected

4 Corrective Actions

- Implement corrective and preventive actions

- Strengthen controls to prevent recurrence

- Document lessons learned

Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Managing Director | Approves data governance policy; reviews major breaches |
| Compliance Head / DPO | Owns this SOP; conducts audits; handles breach response; liaises with regulators |
| IT Manager | Implements technical controls; manages access; monitors systems |
| HR Manager | Manages employee data; ensures consent; maintains records |
| Department Heads | Ensure departmental compliance; classify data; restrict access |
| All Employees | Follow data handling rules; report breaches; complete training |

13. Training and Awareness

- All employees receive induction training on data protection.

- Annual refresher training is mandatory.

- Role-specific training for HR, IT, Finance, and Compliance.

- Training covers:

  - Data classification

  - Access controls

  - Phishing and social engineering

  - Breach reporting

  - Secure disposal

  - DPDP Act requirements

14. Continuous Improvement

Alchem International strengthens its data governance framework through:

- Annual internal reviews and management oversight

- Buyer and third-party audit feedback

- IT security enhancements

- Regulatory monitoring and policy updates

- Employee awareness and reporting mechanisms

Findings from audits, incidents, or reviews are used to enhance system security, update retention schedules, and reinforce compliance with evolving legal and buyer expectations.

# SECTION 17: THIRD-PARTY ETHICAL COMPLIANCE DUE DILIGENCE

1 Opening Statement

Alchem International Private Limited does not operate in isolation. Every API batch, botanical extract, and nutraceutical ingredient that leaves the Faridabad facility carries with it the cumulative conduct of every supplier, contractor, transporter, consultant, and service provider in the value chain. A single unethical supplier can expose the company to regulatory action, customer delisting, reputational damage, and human rights liability.

This SOP establishes a risk-based due diligence framework for all third parties engaged by Alchem International. It is not a one-time onboarding formality. It is a continuous cycle of screening, assessment, monitoring, and corrective action that applies throughout the business relationship.

This SOP is aligned with:

- UN Guiding Principles on Business and Human Rights

- OECD Due Diligence Guidance for Responsible Business Conduct

- Pharmaceutical Supply Chain Initiative (PSCI) Principles

- Sedex SMETA requirements

- Indian Contract Act, 1872 and Bharatiya Nyaya Sanhita (BNS), 2023

- Prevention of Corruption Act, 1988

2 Scope of Third Parties Covered

| Category | Examples | Risk Level |
| --- | --- | --- |
| Raw Material Suppliers | Botanical raw materials, solvents, reagents, excipients | High |
| Chemical Suppliers | Acids, bases, solvents, catalysts, auxiliaries | High |
| Packaging Suppliers | Drums, bags, cartons, labels, liners | Medium |
| Logistics Providers | Transporters, freight forwarders, couriers | Medium |
| Contract Manufacturers | Outsourced processing, milling, blending | High |
| Waste Management Vendors | Hazardous waste handlers, recyclers, TSDF operators | High |
| Labor Contractors | Manpower supply, housekeeping, security | High |
| IT & Software Vendors | ERP, LIMS, cloud services, cybersecurity | Medium |
| Consultants | Regulatory, technical, financial, legal | Medium |
| Maintenance Contractors | Equipment servicing, calibration, fabrication | Medium |
| Construction Contractors | Civil works, expansion, renovation | Medium |
| Canteen & Facility Vendors | Food service, housekeeping, gardening | Low |

3 Due Diligence Process Flow

The following steps describe the sequential due diligence process for engaging and managing third parties. Each step has a defined owner, timeline, and output.

Step 1: Business Need Identification

| Element | Description |
| --- | --- |
| Owner | Requesting Department |
| Action | Identify need for third-party engagement; define scope, duration, and criticality |
| Output | Third-Party Engagement Request Form |
| Timeline | Before initiating procurement |

Step 2: Risk Categorisation

| Element | Description |
| --- | --- |
| Owner | Procurement + Compliance |
| Action | Categorise third party based on risk matrix (see Section 29.4) |
| Output | Risk Category (Low / Medium / High) |
| Timeline | Within 2 working days |

Step 3: Pre-Qualification Screening

| Element | Description |
| --- | --- |
| Owner | Procurement |
| Action | Collect basic documents: GST, PAN, MSME, incorporation, references |
| Output | Pre-Qualification Checklist |
| Timeline | Within 5 working days |

Step 4: Ethical Due Diligence Assessment

| Element | Description |
| --- | --- |
| Owner | Compliance + EHS + HR (as applicable) |
| Action | Assess against ethical criteria (see Section 29.5) |
| Output | Due Diligence Assessment Report |
| Timeline | 10–15 working days (depending on risk) |

Step 5: Site Audit (High-Risk Only)

| Element | Description |
| --- | --- |
| Owner | EHS + Compliance |
| Action | Conduct on-site audit of supplier premises |
| Output | Audit Report with findings and CAPA |
| Timeline | Within 30 days of assessment |

Step 6: Approval or Rejection

| Element | Description |
| --- | --- |
| Owner | Cross-Functional Panel (Procurement, Compliance, Finance, EHS) |
| Action | Review assessment; approve, conditionally approve, or reject |
| Output | Approval Decision Record |
| Timeline | Within 5 working days |

Step 7: Contractual Commitments

| Element | Description |
| --- | --- |
| Owner | Legal + Procurement |
| Action | Insert ethical compliance clauses into contract |
| Output | Signed Contract with Supplier Code of Conduct |
| Timeline | Before work commences |

Step 8: Onboarding and Orientation

| Element | Description |
| --- | --- |
| Owner | Procurement + Compliance |
| Action | Orient third party on Alchem International expectations |
| Output | Orientation Record |
| Timeline | Before work commences |

Step 9: Ongoing Monitoring

| Element | Description |
| --- | --- |
| Owner | Procurement + Compliance |
| Action | Monitor performance, incidents, complaints, audit findings |
| Output | Monitoring Reports |
| Timeline | Continuous; reviewed quarterly |

Step 10: Periodic Re-Assessment

| Element | Description |
| --- | --- |
| Owner | Compliance |
| Action | Re-assess based on risk level (see Section 29.6) |
| Output | Re-Assessment Report |
| Timeline | High: Annual; Medium: Biennial; Low: Triennial |

Step 11: Exit Management

| Element | Description |
| --- | --- |
| Owner | Procurement + Compliance |
| Action | Ensure orderly exit; recover assets; settle dues; document learnings |
| Output | Exit Checklist |
| Timeline | Before contract closure |

4 Risk Categorisation Matrix

| Risk Factor | Low (1) | Medium (2) | High (3) |
| --- | --- | --- | --- |
| Nature of Work | Non-critical, low hazard | Moderate hazard, moderate criticality | Critical, high hazard |
| Geographic Location | Low-risk region | Moderate-risk region | High-risk region (conflict, corruption) |
| Labour Intensity | Low (few workers) | Moderate (some workers) | High (many workers, contract labour) |
| Environmental Impact | Minimal | Moderate | High (chemicals, waste, emissions) |
| Regulatory Exposure | Low | Moderate | High (environmental, labour, health) |
| Financial Value | < ₹10 lakh | ₹10–50 lakh | > ₹50 lakh |
| Past Conduct | No known issues | Minor issues resolved | Major issues, unresolved |
| Subcontracting | None | Limited | Extensive |
| Public Visibility | Low | Moderate | High (brand-sensitive) |

Total Score: Sum of all factors

| Score | Risk Category |
| --- | --- |
| 9–15 | Low |
| 16–21 | Medium |
| 22–27 | High |

5 Ethical Due Diligence Criteria

5.1 Integrity and Anti-Corruption

| Criterion | Assessment Method |
| --- | --- |
| No history of bribery or corruption | Declaration + background check |
| No pending litigation for fraud | Legal search |
| Commitment to anti-corruption laws | Policy review |
| No conflicts of interest | Declaration |
| Transparent ownership structure | KYC documentation |

5.2 Labour and Human Rights

| Criterion | Assessment Method |
| --- | --- |
| No child labour | Age verification records |
| No forced or bonded labour | Worker interviews, documentation |
| No retention of identity documents | Worker interviews |
| Fair wages (minimum wage or above) | Payroll records |
| Working hours within legal limits | Time records |
| Freedom of association | Worker interviews, policy review |
| No discrimination | Policy review, worker interviews |
| Grievance mechanism available | Policy review, worker interviews |

5.3 Occupational Health and Safety

| Criterion | Assessment Method |
| --- | --- |
| Safety policy and procedures | Document review |
| PPE provided and used | Site observation |
| Training records | Document review |
| Incident records | Document review |
| Emergency preparedness | Site observation, drills |
| Medical surveillance | Document review |
| Chemical safety | Site observation, SDS review |

5.4 Environmental Responsibility

| Criterion | Assessment Method |
| --- | --- |
| Environmental permits | Document review |
| Waste management practices | Site observation |
| Effluent treatment | Site observation, test reports |
| Emission controls | Site observation, test reports |
| Chemical storage and handling | Site observation |
| Environmental incidents | Document review |
| ISO 14001 certification | Certificate review (if applicable) |

5.5 Data Privacy and Information Security

| Criterion | Assessment Method |
| --- | --- |
| Data protection policy | Document review |
| Confidentiality agreements | Document review |
| Access controls | Site observation, IT review |
| Cybersecurity measures | IT review |
| Incident response plan | Document review |
| Employee training | Training records |

5.6 Business Ethics and Governance

| Criterion | Assessment Method |
| --- | --- |
| Code of conduct | Document review |
| Whistleblower mechanism | Document review |
| Conflict of interest policy | Document review |
| Gift and hospitality policy | Document review |
| Anti-money laundering | Document review |
| Financial transparency | Financial statements |

6 Monitoring and Re-Assessment Schedule

| Risk Category | Monitoring Frequency | Re-Assessment Frequency | Audit Requirement |
| --- | --- | --- | --- |
| High | Quarterly | Annually | On-site annual audit |
| Medium | Half-yearly | Biennially | On-site biennial audit |
| Low | Annually | Triennially | Self-assessment |

7 Incident Reporting and Response

7.1 Reporting Triggers

Third parties must report to Alchem International within 24 hours:

- Any fatality or serious injury

- Any child labour or forced labour allegation

- Any environmental spill or release

- Any regulatory notice or penalty

- Any bribery or corruption allegation

- Any data breach

- Any labour unrest or strike

- Any major accident or fire

7.2 Response Protocol

| Step | Action | Owner | Timeline |
| --- | --- | --- | --- |
| 1 | Receive incident report | Compliance | Immediate |
| 2 | Acknowledge and assess severity | Compliance + EHS | Within 4 hours |
| 3 | Deploy investigation team (if high severity) | Compliance | Within 24 hours |
| 4 | Conduct root cause analysis | Investigation Team | Within 7 days |
| 5 | Develop CAPA | Third Party + Alchem | Within 14 days |
| 6 | Implement CAPA | Third Party | As per plan |
| 7 | Verify effectiveness | Compliance | Within 30 days |
| 8 | Close incident | Compliance | After verification |

7.3 Escalation Matrix

| Severity | Escalation To |
| --- | --- |
| Low | Procurement Head |
| Medium | Compliance Head |
| High | Managing Director |
| Critical | Board / Management Committee |

8 Corrective and Preventive Actions (CAPA)

8.1 CAPA Triggers

- Audit findings

- Incident reports

- Worker complaints

- Regulatory notices

- Customer complaints

- Monitoring deviations

8.2 CAPA Process

- Identify the non-conformance

- Contain the immediate issue

- Investigate root cause (5 Why, Fishbone)

- Develop corrective action

- Develop preventive action

- Assign responsibility and timeline

- Implement actions

- Verify effectiveness

- Close CAPA

- Document lessons learned

8.3 CAPA Tracking

| Element | Description |
| --- | --- |
| Tracking Tool | Central CAPA Register (digital) |
| Review Frequency | Monthly |
| Escalation | Overdue CAPAs escalated to Management |
| Closure | Only after verification |

9 Supplier Development and Capacity Building

Alchem International does not simply penalise non-compliance. The company invests in supplier development:

| Initiative | Description | Frequency |
| --- | --- | --- |
| Training Workshops | EHS, labour rights, chemical safety, waste management | Annual |
| Guidance Materials | SOP templates, checklists, best practice guides | Ongoing |
| Mentoring | One-on-one support for high-risk suppliers | As needed |
| Shared Learning | Supplier forums, case studies, lessons learned | Half-yearly |
| Recognition | Awards for outstanding ESG performance | Annual |

10 Documentation and Recordkeeping

| Record | Responsibility | Retention |
| --- | --- | --- |
| Pre-Qualification Checklists | Procurement | 5 years |
| Due Diligence Assessment Reports | Compliance | 5 years |
| Site Audit Reports | EHS / Compliance | 5 years |
| Approval Decision Records | Procurement | 5 years |
| Signed Supplier Code of Conduct | Procurement | Contract + 5 years |
| Orientation Records | Compliance | 3 years |
| Monitoring Reports | Procurement | 5 years |
| Incident Reports | Compliance | 5 years |
| CAPA Records | Compliance | 5 years |
| Training Records | HR | 3 years |
| Exit Checklists | Procurement | 5 years |

11 Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Managing Director | Approves high-risk engagements; reviews escalated issues |
| Compliance Head | Owns due diligence process; conducts assessments; monitors compliance |
| Procurement Head | Manages supplier relationships; ensures contractual clauses |
| EHS Head | Conducts environmental and safety audits; reviews incidents |
| HR Head | Assesses labour practices; reviews worker welfare |
| Finance Head | Assesses financial stability; monitors payments |
| Legal Head | Reviews contracts; ensures regulatory compliance |
| Department Heads | Identify needs; monitor performance; report issues |
| All Employees | Report third-party misconduct; support audits |

12 Training and Awareness

| Training Module | Audience | Frequency |
| --- | --- | --- |
| Due Diligence Process | Procurement, Compliance | Annual |
| Supplier Code of Conduct | All relevant staff | Annual |
| Ethical Sourcing | Procurement, Business Development | Annual |
| Incident Reporting | All relevant staff | Annual |

13 Continuous Improvement

- Annual review of due diligence criteria and risk matrix

- Integration of audit findings and incident learnings

- Benchmarking against PSCI, Sedex, EcoVadis

- Supplier feedback integration

- Technology upgrades for monitoring and tracking

- Expansion of supplier development programmes

14 Regulatory and Customer Compliance

| Requirement | Source | Compliance |
| --- | --- | --- |
| UN Guiding Principles | International | Followed |
| OECD Due Diligence Guidance | International | Followed |
| PSCI Principles | Industry | Followed |
| Sedex SMETA | Customer | Supported |
| Prevention of Corruption Act, 1988 | India | Compliant |
| Factories Act, 1948 | India | Compliant |
| Hazardous and Other Wastes Rules, 2016 | India | Compliant |

# SECTION 18: RECRUITMENT PROCEDURE

1 Introduction

Alchem International Private Limited is committed to maintaining a fair, transparent, and ethical recruitment process that attracts competent talent while fully respecting human rights, labour laws, and equal opportunity principles. As a company operating in the pharmaceutical and phytochemical manufacturing industry at Faridabad, Haryana, Alchem International recognizes that responsible recruitment is fundamental to building a diverse, skilled, and compliant workforce across production units, offices, and support functions.

This Recruitment Procedure SOP establishes a structured framework to ensure that all hiring activities are conducted objectively, without discrimination, and in strict compliance with applicable labour laws. Particular emphasis is placed on equal opportunity during hiring, non-discrimination, and robust age verification measures to prevent underage employment across all operations and employment categories.

This SOP is aligned with:

- Factories Act, 1948

- Contract Labour (Regulation and Abolition) Act, 1970

- Minimum Wages Act, 1948

- Equal Remuneration Act, 1976

- Child Labour (Prohibition and Regulation) Act, 1986

- Rights of Persons with Disabilities Act, 2016

- Sexual Harassment of Women at Workplace (Prevention, Prohibition and Redressal) Act, 2013 (POSH Act)

- ILO Conventions on Child Labour and Discrimination

- Customer Codes of Conduct and social compliance standards

2 Objectives

The objectives of this Recruitment Procedure SOP are to:

- Ensure a fair, merit-based, and transparent recruitment process for all roles at Alchem International.

- Promote equal opportunity and prevent discrimination during hiring, selection, and onboarding.

- Prevent child labour and underage employment through systematic age verification and legal compliance.

- Ensure recruitment practices align with national labour laws, customer codes of conduct, and international standards such as ILO Conventions.

- Maintain proper documentation, traceability, and accountability throughout the recruitment lifecycle.

- Attract and retain a diverse, skilled, and motivated workforce.

- Support business continuity by filling positions with qualified candidates in a timely manner.

- Protect the company from legal and reputational risks associated with unethical recruitment practices.

3 Scope

This SOP applies to:

- All recruitment activities undertaken by Alchem International for permanent, temporary, contract, apprentice, and trainee positions.

- All departments, including production, quality, maintenance, administration, HR, business development, and management functions.

- All hiring locations, including the factory, offices, and third-party labour sourcing arrangements.

- All personnel involved in recruitment, including HR staff, department heads, supervisors, and authorized recruitment agents.

4 Definitions

| Term | Definition |
| --- | --- |
| Equal Opportunity | Fair treatment of all job applicants without discrimination based on gender, age, religion, caste, ethnicity, disability, marital status, sexual orientation, political opinion, or any other protected characteristic. |
| Underage Employment | Employment of any person below the legally permitted minimum working age as defined under applicable labour laws. |
| Child Labour | Any employment of a person below the minimum legal working age, strictly prohibited at Alchem International. |
| Recruitment Agency | Any external agency engaged to source, screen, or supply candidates for employment. |
| Recruitment Fee | Any fee, charge, or payment required from a candidate as a condition of employment. |

5 Recruitment Process Overview

The recruitment process at Alchem International follows these standardized steps:

- Manpower requisition and approval

- Job description and role definition

- Candidate sourcing and outreach

- Screening and shortlisting

- Interview and evaluation

- Age verification and document validation

- Final selection and offer issuance

- Onboarding and induction

6 Organizational Responsibilities

| Role | Responsibilities |
| --- | --- |
| HR Department | Manage end-to-end recruitment, ensure compliance with this SOP, verify documents, maintain records, and conduct training. |
| Department Heads / Hiring Managers | Define job requirements, participate in interviews, ensure non-discriminatory selection, and verify workforce needs. |
| EHS / Compliance Team | Support audits, verify age compliance during inspections, and monitor adherence to labour standards. |
| Recruitment Agents / Contractors (if any) | Follow Alchem International recruitment standards, submit verified documentation, and comply with child labour and non-discrimination requirements. |
| All Interview Panel Members | Conduct fair and unbiased interviews and adhere strictly to equal opportunity principles. |

7 Step-by-Step Recruitment Procedure

7.1 Step 1: Manpower Requisition and Approval

| Element | Description |
| --- | --- |
| Initiator | Department Head |
| Action | Raise Manpower Requisition Form with justification, role details, and budget approval |
| Approval | HR Head + Finance Head |
| Documentation | Manpower Requisition Form |
| Timeline | Before initiating recruitment |

7.2 Step 2: Job Description and Role Definition

| Element | Description |
| --- | --- |
| Initiator | Department Head + HR |
| Action | Define job description, qualifications, experience, skills, and competencies |
| Approval | HR Head |
| Documentation | Job Description Document |
| Timeline | Within 2 days of requisition approval |

7.3 Step 3: Candidate Sourcing and Outreach

| Element | Description |
| --- | --- |
| Initiator | HR |
| Action | Source candidates through job portals, referrals, campus placements, recruitment agencies, walk-ins |
| Channels | Naukri, LinkedIn, Indeed, employee referrals, local ITI/polytechnic |
| Documentation | Sourcing Record |
| Timeline | As per hiring urgency |

7.4 Step 4: Screening and Shortlisting

| Element | Description |
| --- | --- |
| Initiator | HR |
| Action | Screen applications against job criteria; shortlist candidates |
| Criteria | Qualifications, experience, skills, cultural fit |
| Documentation | Screening Sheet |
| Timeline | Within 3 days of sourcing |

7.5 Step 5: Interview and Evaluation

| Element | Description |
| --- | --- |
| Initiator | HR + Department Head |
| Action | Conduct interviews (technical, HR, panel); assess candidates |
| Methods | Structured interviews, technical tests, practical assessments |
| Documentation | Interview Evaluation Sheets |
| Timeline | Within 7 days of shortlisting |

7.6 Step 6: Age Verification and Document Validation

| Element | Description |
| --- | --- |
| Initiator | HR |
| Action | Verify age, identity, qualifications, and experience |
| Documents | Birth certificate, Aadhaar, school leaving certificate, passport, degree certificates, experience letters |
| Documentation | Verification Checklist |
| Timeline | Before final selection |

7.7 Step 7: Final Selection and Offer Issuance

| Element | Description |
| --- | --- |
| Initiator | HR + Department Head |
| Action | Finalize selection; issue offer letter |
| Approval | HR Head + Department Head |
| Documentation | Offer Letter, Employment Contract |
| Timeline | Within 3 days of interview |

7.8 Step 8: Onboarding and Induction

| Element | Description |
| --- | --- |
| Initiator | HR |
| Action | Conduct induction covering company policies, code of conduct, health & safety, labour rights |
| Documentation | Induction Record |
| Timeline | On joining day |

8 Equal Opportunity and Non-Discrimination During Hiring

Alchem International is an equal opportunity employer and strictly prohibits discrimination at any stage of recruitment.

8.1 Non-Discriminatory Hiring Principles

- All recruitment decisions are based solely on job-related criteria such as skills, experience, qualifications, and suitability for the role.

- No applicant shall be discriminated against on the basis of:

  - Gender or gender identity

  - Age (above legal minimum)

  - Religion, caste, ethnicity, or nationality

  - Disability or health condition (unless job-critical and legally justified)

  - Marital status, pregnancy, or family responsibilities

  - Sexual orientation

  - Political opinion

- Job advertisements and vacancy notices use neutral and inclusive language.

8.2 Fair Screening and Interview Practices

- Screening criteria are standardized and documented prior to candidate evaluation.

- Interview questions focus strictly on professional competence and role requirements.

- Personal questions unrelated to job performance (e.g., family planning, religion, political beliefs) are strictly prohibited.

- Interview panels are trained on unconscious bias and ethical recruitment practices.

8.3 Equal Access and Inclusion

- Women candidates are encouraged to apply for both operational and supervisory roles.

- Reasonable accommodations are provided to candidates with disabilities during interviews where feasible.

- Migrant and local workers are treated equally in hiring, wages, and employment conditions.

9 Age Verification and Prevention of Underage Employment

Alchem International maintains zero tolerance for child labour and strictly enforces age verification prior to employment.

9.1 Legal Compliance

- The minimum age for employment is strictly in line with applicable national labour laws and factory regulations.

- No person below the legally permitted working age is allowed to work at any Alchem International facility, directly or indirectly.

9.2 Mandatory Age Verification Documents

Before final selection and onboarding, HR must verify at least one valid government-issued proof of age, such as:

- Birth Certificate

- Aadhaar Card

- School Leaving Certificate

- Passport

- Any other legally recognized age proof

9.3 Verification Procedure

- Original documents are physically checked by HR personnel.

- Copies of verified documents are retained in the employee personal file.

- Date of birth is recorded in the HR management system and cross-verified during audits.

- In case of doubt regarding age authenticity, employment is withheld until clarification or independent verification is completed.

9.4 Prevention Controls

- Recruitment agents and contractors are contractually obligated to comply with age verification requirements.

- Random age verification checks are conducted during internal audits and customer inspections.

- Supervisors are trained to report any suspected underage employment immediately to HR and Compliance teams.

9.5 Remediation (If Risk Identified)

- If any underage worker is identified, immediate removal from work is ensured without penalty to the individual.

- A remediation plan is initiated, which may include support for education or vocational training, as per legal and ethical guidelines.

- Root cause analysis is conducted, and corrective actions are implemented to prevent recurrence.

10 Prohibition of Recruitment Fees

- No recruitment fees are charged to candidates at any stage.

- Recruitment agencies are contractually prohibited from charging fees to candidates.

- Any violation leads to immediate termination of the agency contract.

- Candidates who have paid fees are reimbursed by the agency.

11 Selection, Offer, and Onboarding

- Final selection is approved jointly by HR and the concerned Department Head.

- Offer letters clearly define role, wages, working hours, benefits, and employment terms.

- No recruitment fees are charged to candidates at any stage.

- All employees undergo induction covering company policies, code of conduct, health & safety, and labour rights.

- Background verification is conducted for sensitive positions (finance, IT, security).

12 Documentation and Record-Keeping

Alchem International maintains the following recruitment records:

- Manpower requisition and approval forms

- Job descriptions

- Sourcing records

- Screening sheets

- Interview evaluation sheets

- Age verification documents

- Offer letters and joining forms

- Recruitment agent agreements (if applicable)

- Induction records

Records are retained in line with legal requirements and internal document retention policies.

13 Training and Awareness

HR and hiring managers receive periodic training on:

- Equal opportunity and non-discrimination

- Child labour laws and age verification

- Ethical recruitment practices

- Unconscious bias

- POSH Act requirements

Awareness sessions are conducted for supervisors and contractors involved in labour hiring.

14 Monitoring, Audit, and Continuous Improvement

- Internal audits periodically review recruitment records and age verification compliance.

- Customer audits and third-party assessments are supported with complete documentation.

- Feedback from audits, grievances, or incidents is used to strengthen recruitment controls.

- This SOP is reviewed periodically and updated to reflect legal, regulatory, or operational changes.

- Recruitment metrics (time to fill, source effectiveness, diversity) are tracked and reviewed.

15 Compliance

This Recruitment Procedure SOP aligns with:

- Applicable national labour and factory laws

- ILO Conventions on Child Labour and Discrimination

- Customer Codes of Conduct and social compliance standards

- Alchem International's Human Rights and Ethical Business Commitments

- POSH Act, 2013

- Rights of Persons with Disabilities Act, 2016

- Equal Remuneration Act, 1976

16 Grievance Redressal

- Candidates and employees may raise recruitment-related grievances with HR.

- Grievances are acknowledged within 48 hours.

- Investigation is completed within 7 working days.

- Corrective actions are implemented and communicated.

- Records of grievances and resolutions are maintained.

# SECTION 19: ENERGY MANAGEMENT MEASURES

Energy Policy Statement

Alchem International Private Limited is committed to reducing its energy consumption, improving energy efficiency, and transitioning to cleaner energy sources across all manufacturing operations at its Faridabad facility. The company recognises that energy management is not merely a cost-saving exercise but a critical component of environmental responsibility, climate action, and long-term business resilience.

This Energy Management SOP establishes a structured framework for monitoring, controlling, and reducing energy consumption in compliance with the Energy Conservation Act, 2001 (as amended in 2022), Bureau of Energy Efficiency (BEE) guidelines, and ISO 50001 principles. The SOP covers electricity, thermal energy, fuel, and compressed air across all production, utility, and administrative areas.

Energy Sources

| Energy Source | Primary Use | Annual Consumption | Unit |
| --- | --- | --- | --- |
| Grid Electricity | Production machinery, HVAC, lighting, IT | [To be filled] | kWh |
| Diesel | DG sets, boilers, thermic fluid heaters | [To be filled] | Litres |
| Natural Gas / LPG | Boilers, canteen | [To be filled] | SCM |
| Solar PV | Captive power generation | [To be filled] | kWh |
| Compressed Air | Pneumatic systems, tools | [To be filled] | Nm³ |

Major Energy-Consuming Areas

| Area | Approximate Share of Total Energy | Key Equipment |
| --- | --- | --- |
| Extraction & Purification | 35% | Reactors, distillation columns, agitators |
| Utilities (Boiler, Chiller, Compressor) | 25% | Boilers, chillers, air compressors, cooling towers |
| HVAC & Ventilation | 15% | AHUs, exhaust fans, fume hoods |
| Lighting | 8% | LED fixtures, emergency lighting |
| Drying & Finishing | 7% | Dryers, ovens, mills |
| Quality Control Labs | 4% | HPLC, GC, stability chambers |
| Offices & Administration | 3% | Computers, printers, ACs |
| Wastewater Treatment | 3% | Pumps, blowers, aerators |

Energy Efficiency Measures by Area

Lighting Systems

| Measure | Description | Status |
| --- | --- | --- |
| LED Replacement | All conventional fluorescent, metal halide, and halogen fixtures replaced with high-efficiency LED lighting | Implemented |
| Occupancy Sensors | Motion sensors installed in corridors, storage rooms, washrooms, staircases | Implemented |
| Daylight Harvesting | Photocell-based controls in areas with natural light | Implemented |
| Zonal Switching | Separate switches for different zones to avoid unnecessary lighting | Implemented |
| Timers | Automatic switch-off for external lighting during daytime | Implemented |
| Periodic Cleaning | Luminaire cleaning to maintain light output | Ongoing |

HVAC Optimization

| Measure | Description | Status |
| --- | --- | --- |
| Variable Frequency Drives (VFDs) | Installed on AHUs, exhaust fans, and ventilation systems | Implemented |
| Temperature Setpoint Optimization | Production areas maintained at 24–26°C; offices at 24°C | Implemented |
| Humidity Control | Optimized humidity levels (50–60% RH) to balance product quality and energy | Implemented |
| Air Change Rate Optimization | Air changes adjusted based on occupancy and process requirements | Implemented |
| Duct Insulation | Thermal insulation on ducts to minimize heat gain/loss | Implemented |
| Filter Maintenance | Regular filter cleaning/replacement to reduce fan load | Ongoing |
| Economizer Cycles | Use of outside air for cooling when ambient conditions permit | Implemented |

Refrigeration & Cooling Systems

| Measure | Description | Status |
| --- | --- | --- |
| Inverter-Based Chillers | Older chillers replaced with energy-efficient inverter-based systems | Implemented |
| Right-Sizing | Cooling systems sized based on actual load, not peak | Implemented |
| Condenser Cleaning | Regular cleaning to maintain heat transfer efficiency | Ongoing |
| Cooling Tower Optimization | Cycles of concentration optimized; drift eliminators installed | Implemented |
| Chilled Water Reset | Chilled water temperature reset based on load | Implemented |
| Thermal Storage | Evaluation of chilled water storage for peak shaving | Under evaluation |

Process Ventilation & Exhaust Systems

| Measure | Description | Status |
| --- | --- | --- |
| Energy-Efficient Fans | High-efficiency fans and blowers installed | Implemented |
| Duct Design Optimization | Reduced pressure drop through improved duct design | Implemented |
| Timers & Interlocks | Ventilation operated only when required | Implemented |
| VFD on Fans | Variable speed control based on demand | Implemented |

Building Envelope

| Measure | Description | Status |
| --- | --- | --- |
| Roof Insulation | Thermal insulation on roofs to reduce heat gain | Implemented |
| Wall Insulation | Insulated cladding on walls | Implemented |
| Window Glazing | Double-glazed or tinted windows to reduce solar heat gain | Implemented |
| Reflective Roofing | Reflective roof coating to reduce cooling load | Implemented |
| Door Sealing | Sealed doors and windows to prevent air leakage | Implemented |
| Vestibules | Air-lock entries to reduce infiltration | Implemented |

Equipment & Technology Upgrades

| Measure | Description | Status |
| --- | --- | --- |
| IE3/IE4 Motors | High-efficiency motors installed for pumps, fans, compressors | Implemented |
| Digitally Controlled Machines | Programmable cycles, load sensors, auto shut-off | Implemented |
| Energy-Efficient Compressors | Screw compressors with VFD and heat recovery | Implemented |
| High-Efficiency Pumps | Energy-efficient pumps with proper sizing | Implemented |
| Variable Frequency Drives | Installed on major motor loads | Implemented |
| Preventive Maintenance | Regular maintenance to minimize frictional losses, air leaks, overheating | Ongoing |

Fuel Switching Measures

| Measure | Description | Status |
| --- | --- | --- |
| Low-Sulphur Diesel | Backup DG sets operated on low-sulphur diesel | Implemented |
| Biodiesel Blends | Evaluation of biodiesel blends for DG sets | Under evaluation |
| Electrification of Heating | Gas burners replaced with electric hot plates, induction heaters | Partially implemented |
| Solar Water Heating | Solar-assisted water heating for canteen and washrooms | Implemented |
| Electric Material Handling | Battery-operated forklifts and pallet movers | Implemented |
| Electric Drying | Programmable electric dryers with temperature and time controls | Implemented |
| Natural Gas | Evaluation of natural gas for boilers (if pipeline available) | Under evaluation |

Waste Heat Recovery & Energy Reuse

| Measure | Description | Status |
| --- | --- | --- |
| HVAC Heat Recovery | Heat exchangers to recover thermal energy from exhaust air | Implemented |
| Closed-Loop Cooling | Closed-loop cooling circuits for machinery and panels | Implemented |
| Compressor Waste Heat | Heat recovery from air compressors for water pre-heating | Implemented |
| Steam Condensate Recovery | Condensate recovered and reused for boiler feed water | Implemented |
| Process Heat Integration | Residual heat from dryers and reactors reused for pre-heating | Partially implemented |
| Thermic Fluid Heat Recovery | Heat recovery from thermic fluid systems | Under evaluation |

Renewable Energy Initiatives

| Initiative | Description | Status |
| --- | --- | --- |
| Rooftop Solar PV | Feasibility assessment for rooftop solar installation | Completed |
| Solar Water Heating | Solar water heating for canteen and washrooms | Implemented |
| Solar Street Lighting | Solar-powered street lights in campus | Implemented |
| Power Purchase Agreement (PPA) | Evaluation of solar PPA for captive consumption | Under negotiation |
| Green Power Purchase | Evaluation of green tariff options from DISCOM | Under evaluation |
| Battery Energy Storage | Evaluation of BESS for peak shaving and backup | Under evaluation |

Monitoring, Measurement, and Reporting

Metering and Sub-Metering

| Metering Level | Description |
| --- | --- |
| Main Incomer | Total electricity consumption |
| Department-Level | Sub-meters for production, utilities, HVAC, lighting |
| Equipment-Level | Sub-meters for major energy-consuming equipment |
| Fuel Meters | Measurement of diesel, gas, and LPG consumption |
| Thermal Meters | Measurement of steam and thermic fluid consumption |
| Compressed Air Meters | Measurement of compressed air generation and consumption |

Energy Performance Indicators (EnPIs)

| EnPI | Unit | Frequency |
| --- | --- | --- |
| Specific Energy Consumption (SEC) | kWh/kg of product | Monthly |
| Electricity per Production Hour | kWh/hour | Monthly |
| Fuel per Unit of Steam | Litres/tonne of steam | Monthly |
| HVAC Energy per m² | kWh/m² | Monthly |
| Lighting Energy per m² | kWh/m² | Monthly |
| Compressed Air per Nm³ | kWh/Nm³ | Monthly |
| Water Pumping Energy per m³ | kWh/m³ | Monthly |

Monitoring and Reporting

| Activity | Frequency | Responsibility |
| --- | --- | --- |
| Meter reading | Daily | Utilities Team |
| Data entry and compilation | Daily | Utilities Team |
| EnPI calculation | Monthly | EHS / Utilities |
| Energy dashboard update | Monthly | EHS |
| Management review | Quarterly | Management Review Committee |
| Internal energy audit | Annually | EHS / Internal Audit |
| Third-party energy audit | Biennially | External Auditor |
| Regulatory reporting | As required | EHS |

Training and Awareness

| Training Module | Audience | Frequency |
| --- | --- | --- |
| Energy Conservation Basics | All employees | Induction |
| Energy-Efficient Operations | Operators, Technicians | Annual |
| Energy Management Systems | EHS, Utilities, Maintenance | Annual |
| ISO 50001 Awareness | Relevant staff | Annual |
| Energy Awareness Week | All employees | Annual |

Energy Awareness Week includes:

- Posters and digital campaigns on energy-saving practices

- Toolbox talks and interactive sessions

- Employee feedback forums and idea collection

- Recognition of best energy-saving suggestions

Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Managing Director | Approves energy policy and targets; reviews performance |
| EHS Head | Owns energy management programme; monitors EnPIs; reports to management |
| Utilities Head | Operates and maintains energy systems; implements efficiency measures |
| Production Head | Ensures energy-efficient production operations |
| Maintenance Head | Maintains equipment for optimal energy performance |
| Procurement Head | Sources energy-efficient equipment and materials |
| Finance Head | Tracks energy costs; evaluates ROI of efficiency projects |
| Department Heads | Ensure departmental energy efficiency; report deviations |
| All Employees | Follow energy-saving practices; report wastage; suggest improvements |

Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| Energy Policy | EHS | 5 years |
| Energy Consumption Logs | Utilities | 5 years |
| EnPI Reports | EHS | 5 years |
| Energy Audit Reports | EHS | 5 years |
| Equipment Maintenance Records | Maintenance | 5 years |
| Training Records | HR | 3 years |
| Capital Expenditure Records for Energy Projects | Finance | 7 years |
| Regulatory Filings | EHS | 5 years |

Continuous Improvement

- Annual review of energy performance and targets

- Evaluation of new energy-efficient technologies

- Employee suggestion schemes and Kaizen initiatives

- Benchmarking against industry best practices

- Participation in BEE programmes and recognition schemes

- Integration of energy performance into management review

- Setting progressive reduction targets for EnPIs

Compliance

| Regulation | Requirement | Compliance |
| --- | --- | --- |
| Energy Conservation Act, 2001 | Energy efficiency compliance | Compliant |
| BEE Guidelines | Energy labelling, PAT scheme (if applicable) | Compliant |
| Factories Act, 1948 | Safe operation of energy systems | Compliant |
| CPCB/SPCB Norms | Emission standards for boilers/DG sets | Compliant |
| Customer Requirements | Energy reporting, carbon footprint | Reported |
| ISO 50001 | Energy management system | Followed |

# SECTION 20: CUSTOMER FEEDBACK MANAGEMENT SYSTEM

Introduction

Customer feedback is the single most valuable source of intelligence for any manufacturing organisation. It tells us what we are doing right, what we are doing wrong, and what our customers need before they articulate it themselves. For Alchem International Private Limited—a manufacturer of APIs, botanical extracts, and nutraceutical ingredients serving 35 countries—customer feedback is not a complaint to be managed. It is data to be mined, a relationship to be strengthened, and an opportunity to improve.

This Standard Operating Procedure establishes a structured, transparent, and effective process for collecting, documenting, evaluating, and resolving customer feedback related to product quality, compliance, delivery performance, documentation, health and safety, and environmental aspects. It is aligned with ISO 9001:2015 requirements and global customer audit expectations.

Scope

This SOP applies to:

- All customer interactions, both domestic and international

- Buyers, brand customers, trading houses, agents, and supply-chain partners

- Feedback received through any formal or informal communication channel

- Feedback related to:

  - Product quality and specifications

  - Documentation and certificates of analysis

  - Packaging and labelling

  - Delivery timeliness and logistics

  - Health and safety concerns

  - Environmental and sustainability aspects

  - Regulatory compliance

  - Customer service responsiveness

The SOP covers the complete feedback lifecycle:

- Feedback collection

- Evaluation and categorization

- Escalation of critical issues

- Investigation and root-cause analysis

- Corrective and preventive action (CAPA)

- Documentation, communication, and management review

Definitions

| Term | Definition |
| --- | --- |
| Customer Feedback | Any comment, complaint, suggestion, inquiry, or observation related to Alchem International's products, workmanship, packaging, documentation, delivery timelines, safety, environmental performance, or compliance requirements. |
| Critical Quality Issue (CQI) | Feedback involving serious product non-conformance, safety risk, regulatory breach, shipment rejection, or significant customer impact. |
| Moderate Issue | Feedback affecting service levels, documentation accuracy, delivery coordination, or minor quality deviations that do not pose safety or regulatory risk. |
| General Feedback | Suggestions, compliments, or minor service-related inputs with no direct impact on quality, safety, or compliance. |
| Escalation | A formal process for notifying senior management and relevant departments when a high-risk, time-sensitive, or customer-critical issue is reported. |

Feedback Collection Channels

Alchem International provides multiple channels for customers to share feedback:

| Channel | Description | Responsible |
| --- | --- | --- |
| Email | Dedicated customer service, quality, and business development email IDs | Customer Service / QA |
| Customer Feedback Form | Digital or physical forms shared during audits or customer visits | QA / Business Development |
| Website Contact Form | Online form on company website | IT / Customer Service |
| Direct Communication | Feedback received by Business Development, QA, Production, or Compliance teams | All customer-facing staff |
| Customer Audits & Inspections | Observations raised during factory audits or assessments | QA / Compliance |
| Post-Delivery Follow-Up | Structured follow-up calls or emails | Customer Service |
| Third-Party Audits | Feedback received through agents, distributors, or auditors | Business Development |

All received feedback must be formally recorded in the Customer Feedback Register (digital or physical).

Feedback Processing Procedure

Step 1: Feedback Receipt

| Element | Description |
| --- | --- |
| Action | Customer feedback received through any approved channel is immediately forwarded to the Customer Service, Business Development, or Quality team. |
| Responsibility | Receiving Personnel |
| Timeline | Immediate |

Step 2: Initial Review

| Element | Description |
| --- | --- |
| Action | Review feedback for completeness; assign a unique reference number for tracking. |
| Responsibility | Customer Service Representative |
| Timeline | Within 24 hours |

Step 3: Classification

| Category | Description | Examples |
| --- | --- | --- |
| General / Minor | No risk to product quality or compliance | Packaging preference, minor suggestion, documentation format |
| Moderate | Potential impact on service or customer satisfaction | Delay in shipment documents, minor workmanship deviation |
| Critical | Serious quality, safety, or compliance risk | Shipment rejection, major quality failure, safety non-compliance |

Step 4: Response Timelines

| Category | Response Requirement |
| --- | --- |
| General | Response within 5 working days |
| Moderate | Investigation initiated within 48 hours |
| Critical | Immediate escalation; action within 4 hours |

Step 5: Escalation Procedure for Critical Issues

Critical feedback activates the Rapid Escalation Protocol, requiring:

Immediate Notification to:

- Head – Quality Assurance

- Business Development Head

- Production Head

- Senior Management

- EHS Team (if safety or environmental related)

Root Cause Analysis (RCA):

- Initiated within 24–48 hours

- Tools may include Fishbone Analysis, 5-Why Analysis, or FMEA

Corrective Action Implementation:

- Quality Assurance coordinates corrective and preventive actions addressing process, material, workmanship, or system gaps.

Buyer / Regulatory Communication:

- Where required, customers are formally informed of findings, actions taken, and preventive measures.

Step 6: Follow-Up & Resolution

| Action | Responsibility | Timeline |
| --- | --- | --- |
| Acknowledgement sent to customer | Customer Service | Within 24 hours |
| Status updates during investigation | Customer Service / QA | Weekly or as agreed |
| Final corrective actions and closure communicated | QA | Within agreed timeline |
| All closures approved by QA | QA Head | Before closure |
| Feedback trends reviewed quarterly | QA | Quarterly |

Roles and Responsibilities

| Role / Department | Key Responsibilities |
| --- | --- |
| Customer Service / Business Development | Receive, log, and communicate feedback; coordinate responses |
| Quality Assurance (QA) | Lead investigations, RCA, CAPA, and verification |
| Production & QC | Support investigations and implement corrective actions |
| EHS Team | Address safety or environmental-related feedback |
| IT / ERP Support | Maintain digital tracking systems |
| Senior Management | Review critical cases and approve major CAPAs |
| Procurement | Address supplier-related feedback and corrective actions |
| Logistics | Address delivery and transportation-related feedback |

Documentation & Recordkeeping

The following records are maintained:

| Record | Responsibility | Retention |
| --- | --- | --- |
| Customer Feedback / Complaint Forms | Customer Service | 5 years |
| Customer Feedback Register | Customer Service | 5 years |
| Investigation and RCA Reports | QA | 5 years |
| CAPA Records | QA | 5 years |
| Customer Communication Logs | Customer Service | 5 years |
| Trend Analysis and Review Reports | QA | 5 years |
| Escalation Records | QA | 5 years |
| Closure Approvals | QA | 5 years |

All records are retained in line with Alchem International's Record Retention Policy and customer requirements.

Customer Satisfaction Measurement

Beyond complaint handling, Alchem International measures customer satisfaction through:

| Method | Frequency | Responsibility |
| --- | --- | --- |
| Customer Satisfaction Survey | Annually | Business Development |
| Net Promoter Score (NPS) | Annually | Business Development |
| Customer Visits and Meetings | As scheduled | Business Development / QA |
| Post-Delivery Follow-Up | Quarterly | Customer Service |
| Customer Audit Feedback | As conducted | QA / Compliance |
| Repeat Order Analysis | Quarterly | Business Development |
| Customer Retention Rate | Annually | Business Development |

Trend Analysis and Reporting

| Analysis | Frequency | Responsibility |
| --- | --- | --- |
| Complaint trend analysis | Quarterly | QA |
| Root cause category analysis | Quarterly | QA |
| CAPA effectiveness review | Quarterly | QA |
| Customer satisfaction trend | Annually | Business Development |
| Top complaints by product/customer | Quarterly | QA |
| Management review presentation | Annually | QA Head |

Training and Awareness

| Training Module | Audience | Frequency |
| --- | --- | --- |
| Customer Feedback Handling | Customer Service, QA, BD | Annual |
| Root Cause Analysis | QA, Production, QC | Annual |
| CAPA Writing and Tracking | QA, Production | Annual |
| Customer Communication | Customer Service, BD | Annual |
| Complaint Investigation | QA, Production | Annual |

Continuous Improvement

- Annual review of feedback handling process

- Integration of customer feedback into product development

- Benchmarking against industry best practices

- Customer feedback-driven process improvements

- Recognition of employees who resolve complaints effectively

- Quarterly review of feedback trends by Management Review Committee

Review & Updates

This SOP is reviewed annually or earlier in case of:

- Buyer or regulatory requirement changes

- Introduction of new product lines or customers

- Audit findings or recurring customer complaints

- Significant process or system changes

- Changes in customer communication channels

Findings from reviews are used to strengthen customer satisfaction, compliance performance, and operational excellence.

Appendix: Sample Customer Feedback Form

A. Customer Details

| Field | Description |
| --- | --- |
| Name |  |
| Company |  |
| Contact Number |  |
| Email Address |  |
| Date of Experience |  |
| Product / Service Used |  |

B. Feedback Type

(Please check ✓ the most relevant option)

| Option | Description |
| --- | --- |
| ☐ | General Suggestion |
| ☐ | Minor Inconvenience |
| ☐ | Product or Service Quality Issue |
| ☐ | Health or Safety Concern |
| ☐ | Documentation / Compliance Issue |
| ☐ | Delivery / Logistics Issue |
| ☐ | Other: |

C. Description of Feedback

Please describe your feedback in detail. Include any symptoms, specific incidents, batch numbers, or suggestions for improvement.

D. Severity Classification (Office Use Only)

| Classification | Description | Tick (✓) |
| --- | --- | --- |
| Minor | Suggestion, minor issue | ☐ |
| Moderate | Service impact, minor quality deviation | ☐ |
| Critical | Safety threat, major quality failure, regulatory breach | ☐ |

E. Desired Resolution (Optional)

Please specify if you expect any specific response or resolution:

| Option | Description |
| --- | --- |
| ☐ | Follow-up call/email |
| ☐ | Replacement / Credit |
| ☐ | Investigation & update |
| ☐ | No action needed – for awareness only |
| ☐ | Other: |

F. Signature

| Field | Description |
| --- | --- |
| Customer Signature |  |
| Date |  |

G. Internal Use Only (To Be Completed by Staff)

| Field | Description |
| --- | --- |
| Received By |  |
| Date Received |  |
| Reviewed By |  |
| Escalation Required? | ☐ Yes ☐ No |
| Escalated To |  |
| Action Taken |  |
| Closure Date |  |
| Logged in Feedback Register | ☐ Yes ☐ No |

# SECTION 21: SUSTAINABLE SOURCING MANUAL

Introduction

Alchem International is committed to responsible and sustainable sourcing practices that uphold environmental integrity, protect human rights, and promote ethical business conduct across all tiers of its supply chain. As a pharmaceutical and phytochemical manufacturer, Alchem International sources a wide range of inputs including botanical raw materials, solvents and reagents, chemicals, excipients, packaging materials, utilities, engineering spares, logistics services, and outsourced processing services.

Given this diverse sourcing landscape, procurement decisions directly influence environmental performance (water, energy, chemicals), worker welfare, community impacts, and long-term supply resilience. This Sustainable Sourcing Manual establishes Alchem International's expectations for ESG compliance, ethical conduct, quality consistency, environmental protection, and inclusive business relationships with suppliers and service providers.

Objectives

- Identify and mitigate environmental, social, and human rights risks across Alchem International's upstream supply chain, including botanical raw materials, chemicals, processing services, and logistics.

- Promote ethical, transparent, and inclusive sourcing practices aligned with internationally recognized principles, ISO standards, and Alchem International's internal ESG framework.

- Evaluate, engage, and improve supplier performance through structured ESG assessments, audits, and capacity-building initiatives.

- Strengthen supplier accountability on labor rights, non-discrimination, safe workplaces, environmental protection, and responsible chemical management.

- Support long-term sustainability and resilience through responsible material sourcing, traceability, and low-impact production practices.

Key Sustainable Procurement Principles

1. Supplier Code of Conduct Compliance

All suppliers—whether providing botanical raw materials, chemicals, excipients, packaging, services, or logistics—must review, sign, and comply with Alchem International's Supplier Code of Conduct, which covers:

- Human rights and decent work

- Ethical business conduct

- Environmental performance

- Safe and responsible chemical handling

- Zero tolerance for corruption, bribery, harassment, or discrimination

Annual reaffirmation ensures continued commitment.

2. Environmental Stewardship

Alchem International gives preference to suppliers who:

- Demonstrate environmental management systems aligned with ISO 14001 (or equivalent).

- Minimize emissions, hazardous waste, chemical losses, and water pollution.

- Implement energy-efficient technologies and, where feasible, renewable energy.

- Follow responsible material sourcing practices, including botanical traceability, reduced environmental impact, and protection of ecosystems.

Chemical suppliers must comply with applicable chemical management standards and restricted substance requirements.

3. Social Responsibility & Ethical Labor

Suppliers must adhere to:

- Prohibition of child labor, forced labor, and human trafficking

- Fair wages meeting or exceeding local statutory requirements

- Safe and healthy working conditions aligned with ISO 45001 principles

- Equal opportunity, diversity, and non-discrimination

- Freedom of association and effective grievance mechanisms

Worker welfare is a mandatory audit criterion.

4. Supplier Diversity and Inclusion (DEI)

Alchem International promotes inclusive sourcing by encouraging business with:

- Women-owned enterprises

- Small and medium enterprises (SMEs/MSMEs)

- Minority-owned or socially disadvantaged businesses

Supplier diversity information is collected during onboarding and tracked as a sustainability indicator.

5. Governance, Transparency, and Ethical Procurement

Procurement decisions follow a zero-tolerance approach to:

- Corruption, bribery, and facilitation payments

- Conflicts of interest

- Unfair vendor favoritism

All sourcing decisions must be transparent, documented, and traceable.

Supplier Evaluation and Engagement

1. Pre-Qualification and ESG Screening

New suppliers undergo structured onboarding that assesses:

- Legal compliance and certifications

- Environmental, labor, and ethical risks

- Chemical management practices (where applicable)

- Ownership structure and DEI status

- Commitment to Alchem International's Supplier Code of Conduct

- History of non-compliances or sanctions

High-risk categories (chemicals, solvents, botanical raw materials, waste handlers, logistics) receive enhanced scrutiny.

2. On-Site Sustainability Audits

Periodic audits (annual or risk-based) assess:

- Environmental controls (effluents, emissions, waste, chemicals)

- Occupational health and safety, PPE use, ergonomics, and emergency preparedness

- Labor practices (wages, working hours, harassment prevention)

- Chemical storage, handling, and compliance with restricted substances

- Environmental and community impact risks

Findings result in Corrective and Preventive Action Plans (CAPA) with timelines and follow-up verification.

3. Worker Feedback Mechanisms at Supplier Sites

Anonymous feedback is collected via:

- Digital surveys

- Physical forms during audits

- Interviews with worker representatives

Focus areas include safety perception, discrimination or harassment concerns, grievance access, and management practices. Inputs inform supplier risk scoring.

4. Annual ESG Performance Review

Active suppliers are reviewed annually based on:

- Audit outcomes and CAPA closure

- ESG scorecard performance

- Incident history

- Worker feedback results

- DEI contribution

- Supply reliability and quality consistency

Persistent non-compliance may lead to warnings, enhanced monitoring, suspension, or phase-out.

Procurement Process Flow

| Step | Activity | Responsible Function |
| --- | --- | --- |
| 1 | Raise purchase request with sustainability requirements | Requesting Department |
| 2 | Screen suppliers for ESG & Code of Conduct adherence | Procurement |
| 3 | Review supplier DEI ownership/diversity status | Procurement |
| 4 | Share and collect signed Supplier Code of Conduct | Procurement |
| 5 | Conduct ESG risk screening and supplier audits | Procurement + EHS |
| 6 | Evaluate supplier DEI practices annually | Procurement |
| 7 | Collect worker feedback from supplier sites | Procurement / EHS |
| 8 | Approve final supplier shortlist | Procurement Head |
| 9 | Insert sustainability clauses into contracts | Procurement + Legal |
| 10 | Monitor KPIs, compliance, and ESG performance | Procurement / EHS |

Supplier Development and Capacity Building

Alchem International focuses on improvement as well as compliance by providing:

- Workshops and training for suppliers on:

  - Environmental management and water stewardship

  - Safe chemical handling and pharmaceutical industry-aligned practices

  - Waste reduction and energy efficiency

  - Human rights, DEI, and grievance mechanisms

- Targeted support for women-led or minority-owned SMEs

- Guidance materials explaining expectations, documentation, and audit readiness

- Structured re-evaluation cycles to support continuous improvement

Continuous Improvement

- Annual review of supplier ESG scoring models and risk matrices

- Cross-functional sustainability reviews involving Procurement, EHS, Quality, and Operations

- Annual ESG training webinars for key suppliers

- Quarterly internal feedback loops to identify emerging risks and resolve supplier issues

- Development of long-term sustainability partnerships with high-performing suppliers

# SECTION 22: CUSTOMER HEALTH & SAFETY MEASURES

Introduction

Alchem International is committed to ensuring that all customers, buyers, brands, and downstream users can safely handle, process, and use the pharmaceutical, phytochemical, nutraceutical, and cosmeceutical ingredients manufactured and supplied by the company. As a manufacturer of APIs, botanical extracts, and specialty ingredients, Alchem International supplies products that ultimately reach global consumers, including patients, workers, and sensitive user groups.

Although Alchem International does not manufacture finished pharmaceutical products for direct consumer sale, its operations involve solvents, reagents, botanical raw materials, intermediates, and packaging materials that can pose health and safety risks if not properly controlled. Therefore, Alchem International takes full responsibility for identifying potential customer health and safety risks, controlling them at source, and communicating relevant information transparently throughout the product lifecycle.

This SOP defines the systems and controls adopted to protect customer health and safety from material selection and chemical management to labeling, documentation, restricted substance compliance, recall readiness, and customer communication.

Objectives

- Ensure that all customers and downstream users receive accurate, complete, and timely health and safety information related to Alchem International's products.

- Establish structured systems for hazard identification, chemical risk assessment, and health impact evaluation across APIs, extracts, intermediates, and packaging.

- Maintain transparent disclosure regarding chemical use, restricted substances, allergenic risks, and safe use instructions.

- Proactively assess risks related to new materials, innovative formulations, and functional ingredients.

- Ensure compliance with applicable global and customer-specific standards, including GHS, ICH, pharmacopoeial standards (USP, EP, BP, IP), REACH, and ISO-aligned management systems.

Customer Health & Safety Protection Measures

| Area | Measures Implemented |
| --- | --- |
| Product Safety Communication | Customers receive full product safety documentation, including chemical compliance declarations and testing summaries. |
| Material & Chemical Disclosure | Disclosure of solvents, reagents, auxiliaries, and restricted substances compliance through RSL/MRSL documentation. |
| Labeling & Hazard Identification | Products and packaging carry correct composition, batch traceability, and safety labels. |
| Health Risk Assessment | Customer Health & Safety Risk Assessment (CHSRA) conducted for new products or product categories. |
| Customer Safety Guidance | Technical sheets and customer manuals provided for safe handling, storage, and use of products. |
| Recall & Incident Management | Documented Product Recall & Incident Response SOP ensuring traceability and rapid customer notification. |
| Storage & Handling Guidance | Defined storage conditions for finished goods to prevent chemical migration, degradation, or contamination. |

Product Safety Information and Documentation

Alchem International maintains safety documentation for all products, including APIs, extracts, and intermediates supplied to customers.

Documentation includes:

- Product composition and specification details

- Chemical compliance declarations (RSL/MRSL)

- Residual chemical and heavy metal test results

- Purity, potency, and stability data

- Packaging material safety information

All documents are:

- Reviewed prior to shipment

- Shared digitally with customers

- Maintained under Quality and Compliance control

Provision of Information on Product Composition

Alchem International ensures transparent disclosure through the following practices:

1. Comprehensive Labeling and Documentation

Labels and technical documents clearly disclose:

- Product name, batch number, and specification

- Manufacturing date and expiry date

- Storage conditions

- Known allergens or sensitizing substances, where applicable

- Packaging labels include batch numbers, manufacturing dates, and country of origin.

2. Batch-Wise Traceability

Each production batch is traceable to:

- Botanical raw material source

- Solvent and reagent batch

- Processing parameters (extraction, purification, synthesis)

- Final inspection and testing results

This supports rapid investigation during complaints, audits, or recalls.

3. Customer and Regulatory Support

For customers requiring regulatory submissions or certifications, Alchem International provides:

- Test reports from accredited laboratories

- Regulatory compliance statements (USP, EP, BP, IP, ICH)

- Certifications (where applicable)

- Chemical inventory summaries aligned with customer requirements

Provision of Information on Potential Health Impacts

Alchem International proactively evaluates and communicates health-related risks:

1. Hazard Identification

Technical documentation highlights:

- Skin irritation or sensitization risks

- Chemical residue thresholds

- Use limitations (e.g., handling requirements, protective equipment)

2. Risk-Based Product Classification

Products are categorized as:

- Low risk: Standard extracts and untreated materials

- Moderate risk: Purified compounds or chemically processed materials

- Higher risk: Potent APIs or functional ingredients requiring enhanced control

3. Health and Safety Alerts

Additional advisories are issued for:

- Products with special handling requirements

- High-potency or cytotoxic compounds

- Moisture-sensitive or light-sensitive materials

4. Product Stewardship for Customers

Customers receive:

- Handling and storage instructions

- Personal protective equipment recommendations

- Packaging disposal guidance

- Environmental and recyclability information

5. Incident and Complaint Reporting

Customers can report:

- Adverse reactions or sensitivities

- Quality issues or deviations

- Chemical compliance concerns

Each case is investigated by Quality, EHS, and Compliance teams.

6. Employee Training

Internal teams are trained on:

- Chemical hazards

- Restricted substances

- Cross-contamination prevention

- Customer communication protocols

Customer Awareness and Safety Communication

Alchem International operates a structured Customer Safety Communication Program, providing:

- Compliance updates

- Regulatory change notifications

- Customer-specific advisories

Product Safety Information Sheets include:

- Restricted substance compliance status

- Safe handling and storage guidance

- Recommended disposal practices

Customer Health & Safety Risk Assessment

Each new or modified product undergoes CHSRA covering:

- Chemical exposure likelihood

- Skin contact and sensitization risk

- Chronic exposure concerns

- Environmental persistence

- Packaging safety

Risk categories:

- Low: Safe under normal use

- Moderate: Controlled processing and handling required

- High: Restricted applications or additional customer approvals

Documentation and Record-Keeping

Alchem International maintains records of:

- Customer safety communications

- Signed customer declarations and acknowledgments

- Chemical compliance files

- CHSRA evaluations

- Recall simulations and incident logs

- Test reports and certification records

All records are retained in line with ISO 9001 and customer requirements.

Continuous Improvement

- Annual customer feedback reviews on product safety and communication

- Quarterly review of regulatory updates affecting pharmaceutical chemicals and labeling

- Continuous substitution of hazardous substances with safer alternatives

- Mock recall drills conducted periodically to test responsiveness

- Ongoing internal training to strengthen customer health and safety culture

# SECTION 23: EMPLOYEE PERFORMANCE EVALUATION AND CAREER PROGRESSION FRAMEWORK

Introduction

Alchem International recognizes that its employees are the key drivers of operational excellence, product quality, compliance, and customer satisfaction within the pharmaceutical and phytochemical manufacturing industry. Consistent performance in areas such as extraction, purification, synthesis, quality control, production planning, compliance, health & safety, and supply chain management depends on a motivated, skilled, and continuously developing workforce.

This Standard Operating Procedure (SOP) establishes a structured framework for evaluating employee performance and enabling systematic career progression across all functions of Alchem International, including Production, Quality, Maintenance, EHS, HR, Compliance, Business Development, Supply Chain, Administration, and Management functions.

The framework is designed to ensure objective, transparent, consistent, and development-oriented performance evaluations that support individual growth while strengthening organizational capability, customer compliance, and long-term sustainability.

Purpose and Objectives

The Employee Performance Evaluation & Career Progression Framework at Alchem International aims to:

- Ensure Fair and Consistent Reviews: Apply a standardized and evidence-based appraisal process across all departments and job levels.

- Support Employee Development: Align individual career aspirations with organizational objectives through clearly defined growth pathways.

- Use Comprehensive Performance Indicators: Combine productivity metrics, quality outcomes, compliance behavior, teamwork, and behavioral competencies for a balanced performance view.

- Encourage a Feedback-Driven Culture: Promote open communication, regular coaching, and constructive feedback.

- Ensure Legal and Policy Alignment: Adhere to internal HR policies, labor laws, equal opportunity commitments, and customer code of conduct requirements.

- Enable Data-Informed Talent Decisions: Maintain structured documentation to support promotions, training needs, succession planning, and workforce stability.

Scope and Applicability

This SOP applies to all permanent, contractual, probationary, and supervisory employees across all Alchem International factories, offices, and operational locations. It covers:

- Quarterly, half-yearly, and annual performance appraisals

- Goal setting and KPI definition

- Performance monitoring and documentation

- Individual Development Plans (IDPs)

- Technical, operational, behavioral, and leadership competency evaluation

- Training, upskilling, and cross-functional exposure

- Career progression, promotions, and internal mobility

Roles and Responsibilities

| Role | Responsibilities |
| --- | --- |
| Managing Director / Senior Management | Provide strategic direction for talent development and approve major policy or framework updates. |
| Department Heads | Align departmental goals with business objectives, monitor team performance, and support career growth initiatives. |
| HR Manager / HR Department | Design and manage appraisal cycles, maintain records, ensure fairness and compliance, and identify training needs. |
| Supervisors / Line Managers / Section In-Charges | Conduct performance reviews, track employee progress, provide coaching, and document evaluations. |
| Employees | Participate in self-assessments, engage in feedback discussions, and take ownership of personal development plans. |

Feedback and Communication Channels

To promote transparency and continuous dialogue, Alchem International uses multiple feedback mechanisms:

- Internal Communication (Email / HR Systems): For appraisal schedules, self-assessments, and performance-related updates.

- One-on-One Review Meetings: Regular discussions between employees and supervisors for performance tracking and development planning.

- Standardized Appraisal Formats: Uniform templates ensure consistency across departments and shifts.

- Suggestion Box & Confidential Feedback: Employees may raise concerns, suggestions, or improvement ideas anonymously.

Evaluation Process Overview

1. Goal Setting and Planning

At the beginning of each appraisal cycle, goals and KPIs are jointly defined by employees and supervisors. KPIs may include:

- Productivity and efficiency

- Quality performance and defect reduction

- Compliance with SOPs, customer requirements, and safety standards

- Attendance, discipline, and punctuality

- Teamwork, communication, and problem-solving

- Adherence to EHS, ethical, and social compliance practices

2. Mid-Year Review (Where Applicable)

- Conducted to review progress, identify challenges, and realign expectations.

- Enables timely corrective actions and performance support.

3. Annual Performance Review

Conducted at the end of the appraisal cycle involving:

- Employee

- Immediate Supervisor

- HR (where required)

Review inputs include:

- Self-assessment

- Supervisor evaluation

- Cross-functional or peer feedback (where relevant, such as Production-Quality or Business Development-Supply Chain coordination)

4. Rating and Recommendations

Performance is evaluated using a defined rating scale supported by written feedback.

Ratings and recommendations are reviewed and acknowledged by:

- Employee

- Supervisor

- Department Head

- HR

5. Career Development Planning

High-performing employees may be considered for:

- Skill enhancement

- Role expansion

- Supervisory or leadership roles

- Succession planning pipelines

Employees requiring improvement receive:

- Individual Development Plans (IDPs)

- Targeted training and mentoring

- Periodic follow-ups to monitor progress

Training and Development Support

Alchem International promotes continuous skill enhancement through:

- Technical & Operational Training: Extraction techniques, purification methods, quality inspection, machine handling, lean manufacturing, compliance requirements.

- Functional Training: Production planning, documentation, customer compliance, inventory control.

- Behavioral & Soft Skills Training: Communication, teamwork, leadership, conflict management.

- Mentoring & On-the-Job Learning: Guidance from experienced supervisors and managers.

- Cross-Training Opportunities: Exposure across production, quality, compliance, and support functions to build flexibility and internal mobility.

Recordkeeping and Documentation

HR securely maintains all performance-related records, including:

- Performance appraisal forms

- KPI tracking sheets

- Training and skill development records

- Promotion and role-change approvals

- Individual Development Plans (IDPs)

- Performance improvement plans (if applicable)

These records support internal audits, customer assessments, and long-term workforce planning.

Continuous Improvement Measures

The performance evaluation framework is reviewed annually to:

- Refine evaluation criteria and weightages

- Incorporate employee and supervisor feedback

- Align KPIs with production targets, compliance needs, and customer expectations

- Introduce digital tracking tools or improved templates

- Strengthen transparency, consistency, and engagement

Employee surveys, HR analytics, and audit findings are used to enhance effectiveness.

Compliance and Fairness Assurance

Alchem International ensures that all performance evaluations are conducted with confidentiality, respect, and impartiality. The company is committed to:

- Equal opportunity for all employees regardless of gender, caste, religion, ethnicity, age, or background

- Compliance with labor laws, social compliance standards, and internal HR policies

- Regular internal reviews to identify and address bias, documentation gaps, or process deviations

Employees may raise appraisal-related grievances with HR. Unresolved cases are reviewed by a neutral committee to ensure fair resolution.

Documentation and Grievance Handling

Each grievance case is documented with:

- Nature of concern

- Review and investigation notes

- Resolution status

- Corrective and preventive actions

Access to grievance records is restricted to HR and authorized management personnel.

Continuous Improvement and Ethical Culture

Alchem International continuously strengthens its people management practices by:

- Reviewing this SOP periodically

- Integrating performance and compliance indicators into internal audits

- Collecting employee feedback through surveys and forums

- Benchmarking against industry best practices

- Updating procedures in response to regulatory, customer, and operational changes

Insights from these efforts reinforce a workplace culture built on fairness, growth, accountability, and long-term employee development.

# SECTION 24: INTERNAL COMMUNICATION PROTOCOL

Introduction

Effective internal communication is critical to ensuring operational alignment, production efficiency, worker safety, compliance with customer requirements, and ethical workplace practices across all units of Alchem International. As a pharmaceutical and phytochemical manufacturing organization involved in extraction, purification, synthesis, quality control, packing, warehousing, and administrative operations, Alchem International relies on clear, timely, and structured communication to support productivity, quality consistency, regulatory compliance, and employee engagement.

Given the multi-shift, compliance-driven nature of the pharmaceutical industry, accurate dissemination of information related to production planning, quality standards, health & safety, audits, and employee welfare is essential. This SOP establishes a formal internal communication framework to ensure transparency, coordination, accountability, and continuous improvement across all departments and levels.

Purpose and Goals

The Internal Communication Protocol at Alchem International aims to:

- Ensure timely and accurate communication of operational, safety, quality, and compliance-related information across production floors and support functions.

- Enhance employee awareness and engagement regarding policies, customer codes of conduct, audit requirements, training programs, welfare initiatives, and performance expectations.

- Strengthen emergency preparedness through reliable communication mechanisms during fires, accidents, machine failures, medical emergencies, or security incidents.

- Standardize communication channels to ensure consistent use of emails, notice boards, internal messaging platforms, ERP systems, and meetings.

- Support documentation and audit readiness through structured recordkeeping of internal communications.

- Encourage two-way communication, enabling employees to safely share feedback, suggestions, and concerns without fear of retaliation.

Scope and Applicability

This SOP applies to:

- All permanent, contractual, temporary, and probationary employees

- Trainees, apprentices, and interns

- Employees working in production, quality, maintenance, EHS, HR, compliance, business development, procurement, stores, logistics, finance, and administration

It governs all forms of internal communication, including:

- Emails, circulars, and official notices

- Digital communication platforms and ERP systems

- Inter-departmental coordination

- Safety alerts and operational instructions

- Shift briefings, toolbox talks, and structured meetings

Roles and Responsibilities

| Role | Responsibilities |
| --- | --- |
| Managing Director / Senior Management | Approves strategic communications, major policy updates, and company-wide announcements. |
| Department Heads | Communicate departmental instructions, align teams with production and compliance objectives, and ensure effective information flow. |
| HR & Administration Manager | Issues HR circulars, policy updates, training communications, and maintains communication records. |
| Line Supervisors / Floor In-Charges / Team Leaders | Communicate daily work allocation, production targets, quality alerts, safety instructions, and shift-related updates. |
| EHS & Compliance Officers | Issue safety alerts, emergency instructions, compliance updates, and audit-related communications. |
| IT / Systems Support | Maintain email systems, ERP platforms, messaging tools, digital displays, and access controls. |
| All Employees | Read internal communications, comply with instructions, attend meetings, and provide feedback when required. |

Communication Channels

Email Communication

- Used for official announcements, SOP revisions, policy changes, audit schedules, and management instructions.

- Emails must contain clear subject lines, concise content, and defined action points.

- Company-wide emails are circulated through designated internal mailing groups.

Notice Boards and Digital Displays

- Installed in production areas, corridors, canteens, quality sections, and administrative offices.

Used to display:

- Safety instructions

- Policy and SOP updates

- Audit schedules

- Training calendars

- Worker welfare and compliance notices

- Digital displays are used for urgent alerts and emergency communication.

Internal Messaging Platforms

- Departmental or shift-based groups are used for rapid operational coordination.

- Professional language, respectful conduct, and confidentiality are mandatory.

- Used for sharing schedules, visual work instructions, maintenance alerts, and real-time updates.

ERP / Intranet / Shared Drives

Centralized access for:

- HR policies and employee handbook

- SOPs and work instructions

- Customer compliance documents

- EHS procedures and emergency contacts

- Training and awareness materials

Employees are expected to review updates regularly.

Meetings, Shift Briefings & Toolbox Talks

Conducted daily, weekly, or as required based on operational needs.

Topics include:

- Production planning and targets

- Quality issues and defect prevention

- Safety instructions and incident learnings

- Customer and audit requirements

Briefing notes or meeting minutes are documented and followed up.

Communication Process

Drafting of Messages

- All communications must be factual, clear, and relevant to the intended audience.

- Safety, compliance, disciplinary, or policy-related messages require prior review and approval by HR, EHS, or Senior Management, as applicable.

Distribution

Communication channels are selected based on urgency and relevance:

- Routine updates → Email / ERP / Notice boards

- Operational instructions → Meetings / Messaging platforms

- Urgent alerts → Messaging groups, public address systems, alarms

HR maintains a communication log for audit and review purposes.

Crisis and Emergency Communication

Emergency communication is triggered during:

- Fire, smoke, or electrical incidents

- Workplace injuries or medical emergencies

- Machinery, power, or utility failures

- Security threats or disturbances

Emergency alerts are communicated through:

- Internal messaging groups and phone calls

- Public address systems

- Digital screens and alarms

- EHS emergency contact lists

A post-incident briefing is conducted to communicate root causes, corrective actions, and preventive measures.

Event and Program Communication

For audits, trainings, meetings, or internal campaigns:

- Invitations include agenda, date, time, venue, and expected participants.

- Attendance is recorded through registers or digital systems.

- Post-event feedback is collected to improve future communication effectiveness.

Documentation and Archiving

Alchem International maintains structured records of:

- Emails and circulars

- Departmental notices

- Meeting minutes

- Emergency communication logs

- Training and audit announcements

- SOP change notifications

All records are securely stored by HR / Administration with version control and restricted access.

Feedback and Continuous Improvement

- Employees may provide feedback through supervisors, HR, suggestion boxes, or digital feedback tools.

- Feedback is reviewed periodically, and improvement actions are discussed with management.

- Communication practices are refined based on employee inputs, audit findings, and operational requirements.

Regulatory and Buyer Compliance

Alchem International's internal communication practices are aligned with:

- ISO 9001 – Quality Management communication requirements

- ISO 14001 – Environmental communication requirements

- ISO 45001 – Occupational health & safety communication requirements

- Customer Codes of Conduct and social compliance standards

- Applicable labour and industrial regulations

Regular reviews ensure the internal communication system remains effective, transparent, and aligned with best practices in the pharmaceutical and phytochemical manufacturing industry.

# SECTION 25: EMERGENCY PREPAREDNESS & RESPONSE FOR ENVIRONMENT, HEALTH & SAFETY (EHS)

Introduction

Alchem International recognizes that pharmaceutical and phytochemical manufacturing operations involve inherent risks—including chemical fires, toxic releases, reactor runaway reactions, solvent explosions, and natural disasters—that can threaten employee safety, community health, environmental quality, and business continuity. The company is committed to maintaining a robust Emergency Preparedness & Response (EPR) framework that ensures swift, coordinated, and effective action during any emergency situation.

This SOP establishes the structured approach for identifying foreseeable emergencies, defining preventive and mitigation measures, and ensuring a coordinated response across all Alchem International facilities. It is aligned with:

- Factories Act, 1948 (Section 41B – Emergency Standards)

- Environment Protection Act, 1986

- Chemical Accidents (Emergency Planning, Preparedness, and Response) Rules, 1996

- Disaster Management Act, 2005

- ISO 14001 (Environmental Management)

- ISO 45001 (Occupational Health & Safety)

- National Disaster Management Authority (NDMA) guidelines

- Haryana State Disaster Management Authority directives

- Customer social compliance and audit requirements

Objectives

The objectives of this SOP are to:

- Establish a structured emergency preparedness and response system for all Alchem International facilities.

- Identify all foreseeable environmental, health, and safety emergencies.

- Define preventive, control, and mitigation measures for each emergency scenario.

- Ensure coordinated, timely, and effective response during any emergency.

- Protect employees, contractors, visitors, neighbouring communities, and the environment.

- Minimize damage to assets, production, and reputation.

- Comply with applicable legal, regulatory, and customer requirements.

- Periodically test, evaluate, and revise emergency controls through mock drills and reviews.

- Integrate emergency preparedness with business continuity planning.

Scope

This SOP applies to all Alchem International facilities, including:

- Manufacturing plant at 25/2, Main Mathura Road, Village Kaili, Ballabhgarh, Faridabad, Haryana 121004

- Warehouses and storage areas

- Quality control and R&D laboratories

- Utility areas (boiler, DG set, compressor, cooling tower)

- Effluent treatment plant (ETP)

- Administrative offices and canteen

- Any future Alchem International site

It covers emergencies arising from:

- Fire and explosion

- Chemical spills and leaks

- Toxic gas releases

- Reactor runaway reactions

- Electrical failures and shocks

- Natural disasters (earthquake, flood, storm, heat wave)

- Utility failures (power, water, compressed air)

- Medical emergencies

- Security threats

- External emergencies from neighbouring industries

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| EHS Department / Management Representative / HR | Implement this SOP across all sites; ensure readiness, communication, coordination, training, mock drills, and periodic reviews. |
| Department Heads / Area In-Charges | Execute emergency actions within their departments; ensure employee compliance with procedures; support investigations and preventive actions. |
| Security Team | Control access; activate sirens; guide evacuations; coordinate with external emergency services. |
| Supervisors / Floor In-Charges | Guide workers during evacuation; ensure attendance at assembly points; report status to the Emergency Controller. |
| Employees | Follow emergency instructions; evacuate safely; cooperate during drills and actual emergencies. |
| Emergency Response Team (ERT) | Trained personnel responsible for firefighting, first aid, spill response, and rescue operations. |
| Site Emergency Controller | Overall in-charge during an emergency; coordinates with external agencies; authorises evacuation and all-clear. |

Emergency Response Procedure

Identification of Emergency Scenarios

Alchem International has identified the following potential EHS emergency scenarios across production floors, utilities, warehouses, and offices:

| Category | Emergency Scenario |
| --- | --- |
| Fire & Explosion | Fire in solvent storage, reactor area, warehouse, or electrical panel; explosion due to dust, solvent vapour, or reactive chemicals |
| Chemical Release | Spill or leak of acids, bases, solvents, toxic chemicals; toxic gas release (e.g., chlorine, ammonia, H₂S) |
| Reactor Runaway | Uncontrolled exothermic reaction; pressure build-up; temperature excursion |
| Electrical | Electrical shock; panel fire; short circuit; power failure |
| Natural Disasters | Earthquake; flood; storm; heat wave; dust storm |
| Utility Failures | Boiler failure; cooling water failure; compressed air failure; DG set failure |
| Medical | Worker injury; chemical exposure; heart attack; heat stroke |
| Security | Intruder; bomb threat; civil unrest; labour unrest |
| Environmental | Effluent spill; ETP overflow; air emission exceedance |

Emergency Contact Information

A master list of emergency contact numbers is prominently displayed at:

- Main security gates

- Production floors

- Electrical rooms

- Offices and control rooms

- Canteen and common areas

The list includes contact details for:

- Fire brigade

- Police

- Ambulance and hospitals

- Local emergency services

- Alchem International internal emergency response team

- Site Emergency Controller

- EHS Head

Contact information is reviewed and updated periodically.

Activation of Emergency

Upon confirmation of a major emergency:

- The Security Officer / Security Guard activates the emergency siren upon instruction from the Site Emergency Controller.

- Emergency Siren Pattern:

  - Emergency Alert (Wailing Siren): 30 seconds ON, 10 seconds OFF, repeated for 2 minutes.

  - All-Clear Siren: Continuous sound for 2 minutes.

- Weekly Siren Testing: Conducted once every week at a fixed time. HR / EHS informs all department heads in advance. No evacuation is required during siren testing.

Onsite Emergency Plan / Disaster Management Plan

Alchem International maintains a documented Onsite Emergency Plan prepared by the Management Representative and Department Heads, covering:

- Emergency organization structure

- Roles and responsibilities

- Evacuation routes and emergency exits

- Assembly points

- Firefighting and first-aid resources

- Chemical hazards and environmental risks

- Communication and coordination mechanisms

- Environmental impacts and occupational health & safety hazards

Emergency Helpline Numbers

External Emergency Services

| Service | Contact Number |
| --- | --- |
| Fire Emergency | 101 |
| Police Emergency | 100 |
| Ambulance / Medical Emergency | 102 / 108 |
| National Highway Trauma Care | 1073 |
| Disaster Management Helpline | 1078 |

Internal Key Personnel – Alchem International

| Designation | Contact Number |
| --- | --- |
| EHS / Safety Officer | [To be filled] |
| Site Emergency Controller | [To be filled] |
| Security Head | [To be filled] |
| HR Head | [To be filled] |
| Maintenance Head | [To be filled] |

Emergency Categories & Response Measures

Fire Emergency

Prevention:

- Maintain fire extinguishers, hydrants, alarms, and detectors in working condition.

- Conduct routine inspections and housekeeping.

- Train employees and security staff in fire extinguisher usage.

- Ensure proper storage of flammable materials.

- Implement hot work permit system.

Mitigation:

- Inform Security and Supervisor immediately.

- Use appropriate extinguishers (electrical, fabric, oil-based fires).

- Evacuate through staircases; do not use lifts.

- Move to designated assembly points.

- Isolate affected area and disconnect power if safe.

- Call Fire Brigade if fire is uncontrollable.

- Assess and address environmental impact post-incident.

Chemical Spill / Leak

Prevention:

- Secondary containment for all chemical storage.

- Spill kits at strategic locations.

- Regular inspection of containers, pipelines, and valves.

- Training on safe chemical handling.

- Proper labelling and segregation.

Mitigation:

- Alert area personnel; evacuate if necessary.

- Identify the chemical (from label or SDS).

- Don appropriate PPE.

- Contain the spill using absorbent booms or sand.

- Neutralise if applicable.

- Collect contaminated material in hazardous waste containers.

- Clean area with appropriate cleaning agent.

- Report incident to EHS.

- Conduct root cause analysis and CAPA.

Reactor Runaway Reaction

Prevention:

- Process hazard analysis (PHA) for all reactions.

- Temperature and pressure monitoring with alarms.

- Emergency cooling systems.

- Addition rate control.

- Operator training.

Mitigation:

- Activate emergency cooling.

- Stop reactant feed.

- Vent pressure safely (if designed).

- Evacuate area.

- Notify EHS and Emergency Controller.

- If fire or toxic release, activate fire or spill response.

- Conduct investigation and CAPA.

Electrical Shock / Short Circuit

Prevention:

- Mandatory PPE for electrical work.

- Rubber mats at all electrical panels.

- Clear "Danger – High Voltage" signage.

- Proper earthing and protective devices (MCB, ELCB).

- Only authorized electricians allowed to work on electrical systems.

Mitigation:

- Switch off power immediately.

- Separate the victim safely from the power source.

- Provide first aid by trained personnel.

- Shift to medical facility if required.

Earthquake

Mitigation:

- Evacuate calmly through emergency exits to open assembly areas.

- Do not use elevators.

- Assist employees unfamiliar with procedures.

- Stay away from windows, shelves, and heavy equipment.

- After shaking stops, follow evacuation procedures.

- Check for injuries and damage.

- Report to Emergency Controller.

Flood / Heavy Rain

Mitigation:

- Move to higher floors if instructed.

- Avoid walking through moving water.

- Disconnect electrical equipment if safe.

- Store critical documents and equipment on elevated platforms.

- Follow evacuation instructions.

- Coordinate with local authorities.

Heat Wave

Mitigation:

- Provide hydration stations.

- Schedule outdoor work during cooler hours.

- Provide rest breaks in cool areas.

- Monitor workers for heat stress.

- Provide electrolytes.

- Train employees on heat stress symptoms.

Occupational Accidents

Includes slips, trips, falls, needle injuries, electrical shocks, and machine-related injuries.

- Immediate first aid provided.

- Incident reported to EHS and HR.

- Mock drills conducted periodically.

- Emergency maps and siren codes displayed at key locations.

- Medical surveillance for high-risk roles.

Corrective and Preventive Actions (CAPA)

For all emergency incidents:

- Root cause analysis is conducted.

- Corrective and preventive actions are documented.

- Regulatory notifications are made where required.

- Lessons learned are communicated to employees.

- CAPA effectiveness is verified.

- CAPA status is tracked in the central register.

Review and Disposition

- The Onsite Emergency Plan is reviewed at least once every three years or earlier if required due to process changes, incidents, or regulatory updates.

- Mock drills are conducted periodically and findings documented.

- All revisions are recorded and retained.

- The EHS Head presents an annual emergency preparedness report to the Management Review Committee.

Abbreviations

| Abbreviation | Full Form |
| --- | --- |
| EHS | Environment, Health & Safety |
| MCB | Miniature Circuit Breaker |
| ELCB | Earth Leakage Circuit Breaker |
| MR | Management Representative |
| ERT | Emergency Response Team |
| PHA | Process Hazard Analysis |
| CAPA | Corrective and Preventive Action |
| SDS | Safety Data Sheet |
| PPE | Personal Protective Equipment |

Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| Onsite Emergency Plan | EHS | 5 years |
| Mock Drill Reports | EHS | 3 years |
| Incident Reports | EHS | 5 years |
| CAPA Records | EHS | 5 years |
| Training Records | HR | 3 years |
| Emergency Contact List | EHS | Updated annually |
| Siren Testing Log | Security | 3 years |
| Medical Records | HR / Medical | As per law |

Continuous Improvement

- Annual review of emergency preparedness and response.

- Post-drill and post-incident reviews.

- Integration of lessons learned into the plan.

- Benchmarking against industry best practices.

- Employee feedback and suggestions.

- Technology upgrades for detection, communication, and response.

- Community awareness and coordination.

Compliance

This SOP aligns with:

- Factories Act, 1948

- Environment Protection Act, 1986

- Chemical Accidents (EPPR) Rules, 1996

- Disaster Management Act, 2005

- ISO 14001 and ISO 45001

- NDMA and Haryana SDMA guidelines

- Customer social compliance requirements

# SECTION 26: ENVIRONMENTAL DISRUPTION CONTINGENCY & FINANCIAL RESILIENCE MEASURES

Introduction

Alchem International recognizes that environmental disruptions—whether acute (floods, cyclones, heat waves, earthquakes) or chronic (water scarcity, rising temperatures, ecosystem degradation)—can cause significant operational, financial, and reputational damage. To withstand and recover from such disruptions, the company has established a structured framework of contingency measures that ensure financial stability, maintain basic operations, and protect stakeholder interests.

This SOP outlines the operational buffers, financial safety nets, and contingency mechanisms in place at Alchem International to maintain business continuity during and immediately after a severe climate event or disruption of ecosystem services. It complements the Operational Adaptation to Environmental Disruptions (Section 21) and the Emergency Preparedness & Response (Section 20) SOPs.

This SOP is aligned with:

- Disaster Management Act, 2005

- National Disaster Management Authority (NDMA) guidelines

- Insurance Regulatory and Development Authority of India (IRDAI) regulations

- Reserve Bank of India (RBI) guidelines on financial resilience

- ISO 22301 (Business Continuity Management)

- ISO 31000 (Risk Management)

- Customer supply chain resilience requirements

Objectives

The objectives of this SOP are to:

- Establish contingency measures to withstand and recover from environmental disruptions.

- Maintain financial stability during and immediately after a severe climate event.

- Ensure basic operations continue without immediate shutdown.

- Protect employees, assets, and stakeholder interests.

- Minimize downtime and revenue loss.

- Comply with regulatory and customer requirements for business resilience.

- Periodically review and update contingency measures based on evolving risks.

Scope

This SOP applies to all Alchem International facilities, operations, and functions, including:

- Manufacturing plant at Faridabad

- Warehouses and storage areas

- Utilities and infrastructure

- Supply chain and logistics

- Employees and contract workers

- Financial and administrative functions

It covers contingency measures for:

- Acute physical risks (floods, cyclones, heat waves, earthquakes, wildfires)

- Chronic physical risks (water scarcity, rising temperatures, sea-level rise)

- Ecosystem service disruptions (soil erosion, loss of natural buffers)

- Regulatory and market disruptions linked to environmental factors

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| Managing Director / Board | Approves contingency plans and financial safety nets; reviews resilience performance. |
| Finance Head | Maintains emergency cash reserves; manages insurance policies; tracks financial resilience metrics. |
| EHS Head | Identifies environmental risks; develops contingency plans; coordinates with authorities. |
| Operations Head | Ensures operational buffers (safety stock, backup equipment) are maintained. |
| Supply Chain Head | Maintains safety stock of critical supplies; identifies alternate suppliers. |
| Maintenance Head | Maintains backup equipment (generators, pumps, flood barriers). |
| HR Head | Ensures employee safety and communication during disruptions. |
| All Department Heads | Implement contingency measures within their areas; report gaps. |
| All Employees | Follow contingency procedures; report disruptions; support recovery. |

Operational Buffers and Contingency Measures

1. Safety Stock of Critical Operational Supplies

Alchem International maintains safety stock of critical supplies to prevent immediate shutdowns during external shortages caused by environmental disruptions.

| Category | Critical Items | Safety Stock Level | Storage Location |
| --- | --- | --- | --- |
| Raw Materials | Key botanical raw materials, solvents, reagents | 30–45 days | Covered warehouse |
| Utilities | Diesel for DG sets, boiler fuel, treatment chemicals | 15–30 days | Fuel storage, chemical store |
| Packaging | Drums, bags, cartons, labels | 30 days | Packaging store |
| Spare Parts | Critical pumps, valves, gaskets, instruments | 15–30 days | Maintenance store |
| PPE | Gloves, masks, goggles, suits | 30 days | PPE store |
| Water | Stored water for essential operations | 2 days | Overhead / underground tanks |
| Food & Essentials | Canteen supplies, drinking water | 7 days | Canteen store |

Safety stock levels are reviewed quarterly and adjusted based on risk assessment and supply chain reliability.

2. Backup Operational Equipment

Alchem International has installed backup equipment to maintain basic operations during utility failures or environmental disruptions.

| Equipment | Capacity | Purpose | Location |
| --- | --- | --- | --- |
| Diesel Generators (DG Sets) | 2 × 500 kVA | Backup power for critical loads (ETP, cold storage, control room, security) | Utility building |
| Emergency Power for ETP | 1 × 100 kVA | Maintain effluent treatment during power failure | ETP area |
| Backup Pumps | 2 × 10 HP | Water supply during main pump failure | Pump house |
| Portable Pumps | 4 × 5 HP | Flood dewatering | Store |
| Flood Barriers | 100 meters | Protect critical entry points | Gate, warehouse, ETP |
| Sandbags | 500 bags | Flood protection | Store |
| Emergency Lighting | 50 units | Evacuation and critical areas | Throughout facility |
| Portable Cooling Units | 4 units | Heat wave response | Store |
| Water Tankers | 2 × 10,000 L | Emergency water supply | Contracted |
| Standby Boiler | 1 × 2 TPH | Steam supply during main boiler failure | Utility building |
| Backup Chiller | 1 × 100 TR | Cooling during main chiller failure | Utility building |
| UPS Systems | Multiple | Critical controls, IT servers, labs | Control room, IT, QC |

Backup equipment is tested monthly or as per schedule and maintained in ready-to-use condition.

3. Alternate Supply Chain and Logistics

| Measure | Description |
| --- | --- |
| Alternate Suppliers | Qualified alternate suppliers for critical raw materials and services. |
| Alternate Routes | Pre-identified alternate transport routes to avoid flood-prone areas. |
| Local Sourcing | Preference for local suppliers to reduce logistics risk. |
| Inventory Buffers | Safety stock at plant and regional warehouses. |
| Logistics Partners | Multiple logistics providers to avoid single-point dependency. |
| Contractual Clauses | Force majeure and environmental disruption clauses in supplier contracts. |

4. Water Contingency

| Measure | Description |
| --- | --- |
| Water Storage | Overhead and underground tanks with 2-day backup for essential operations. |
| Rainwater Harvesting | Captured rainwater used for non-potable applications. |
| Water Tankers | Contracted tanker supply for emergencies. |
| Recycling | Treated ETP water reused for cooling, cleaning, gardening. |
| Water-Saving Measures | Low-flow fixtures, leak detection, process optimization. |
| Alternate Sources | Identification of alternate water sources (borewells, municipal, tankers). |

5. Energy Contingency

| Measure | Description |
| --- | --- |
| DG Backup | Diesel generators for critical loads. |
| Solar Power | Rooftop solar for partial daytime load. |
| Fuel Storage | 15–30 days of diesel storage. |
| Energy Efficiency | Measures to reduce load during disruptions. |
| Alternate Fuel | Evaluation of biodiesel, natural gas, or other fuels. |
| Power Purchase Agreements | Options for green power or alternate suppliers. |

6. Infrastructure Resilience

| Measure | Description |
| --- | --- |
| Flood Protection | Elevated storage, flood barriers, drainage systems. |
| Heat Resilience | Cooling systems, insulation, reflective roofing. |
| Wind Resilience | Structural strengthening, secured equipment. |
| Seismic Resilience | Earthquake-resistant design (as per BIS codes). |
| Ecosystem Buffers | Green belt, vegetation, natural drainage. |
| Maintenance | Regular inspection and repair of infrastructure. |

Financial Safety Nets

1. Business Interruption Insurance

Alchem International maintains comprehensive insurance coverage that includes:

| Insurance Type | Coverage | Provider |
| --- | --- | --- |
| Property Insurance | Fire, flood, earthquake, storm damage to assets | [To be filled] |
| Business Interruption | Loss of profit due to disruption | [To be filled] |
| Machinery Breakdown | Equipment failure due to environmental stress | [To be filled] |
| Marine Insurance | Goods in transit | [To be filled] |
| Environmental Liability | Pollution and cleanup costs | [To be filled] |
| Workmen's Compensation | Employee injury during disruption | [To be filled] |
| Directors & Officers | Liability for climate-related decisions | [To be filled] |

Insurance policies are reviewed annually with the Finance and Legal teams.

2. Emergency Cash Reserves

Alchem International maintains a dedicated emergency cash reserve to fund immediate recovery activities.

| Parameter | Details |
| --- | --- |
| Reserve Amount | [To be filled] (e.g., 5% of annual operating budget) |
| Liquidity | Held in liquid instruments (FDs, liquid mutual funds) |
| Access | Joint authorisation of Managing Director and Finance Head |
| Purpose | Emergency repairs, employee wages, supplier payments, logistics, insurance deductibles |
| Review | Quarterly review of adequacy |
| Replenishment | Replenished after each use |

3. Contingent Credit Lines

| Facility | Description |
| --- | --- |
| Working Capital Limit | Pre-approved bank limit for emergency working capital |
| Standby Letter of Credit | For critical supplier payments |
| Credit Insurance | For receivables during disruption |
| Supplier Credit | Negotiated credit terms with key suppliers |

4. Government and Institutional Support

| Source | Type of Support |
| --- | --- |
| Prime Minister's National Relief Fund | Disaster relief |
| State Disaster Response Fund (SDRF) | State-level disaster relief |
| National Disaster Response Fund (NDRF) | National-level disaster relief |
| MSME Schemes | Emergency credit lines for MSMEs |
| Industry Associations | Coordination and advocacy |
| Insurance Claims | As per policy |

5. Financial Risk Assessment

Alchem International conducts periodic financial risk assessments to evaluate:

- Potential financial impact of identified environmental disruptions.

- Adequacy of insurance coverage.

- Adequacy of cash reserves.

- Access to credit.

- Supplier and customer financial resilience.

- Market and regulatory risks.

Findings are integrated into the Enterprise Risk Management (ERM) framework.

Communication and Coordination

Internal Communication

- Emergency communication tree for rapid alerts.

- Designated spokespersons for internal and external communication.

- Regular updates to employees during disruptions.

- Post-event debriefings.

External Communication

- Coordination with local authorities (District Administration, Pollution Control Board, Fire Services).

- Communication with customers and suppliers on disruption status.

- Engagement with insurance providers for claims.

- Community communication where required.

Training and Awareness

| Training Module | Audience | Frequency |
| --- | --- | --- |
| Business Continuity & Contingency | Department Heads, EHS, Finance | Annual |
| Emergency Response | All employees | Annual |
| Insurance and Claims | Finance, Legal | Annual |
| Financial Resilience | Finance, Senior Management | Annual |
| Mock Drills | All relevant staff | Half-yearly |

Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| Contingency Plan | EHS | 5 years |
| Insurance Policies | Finance | Life of policy + 5 years |
| Cash Reserve Records | Finance | 7 years |
| Credit Line Documents | Finance | 7 years |
| Risk Assessment Reports | EHS / Finance | 5 years |
| Training Records | HR | 3 years |
| Mock Drill Reports | EHS | 3 years |
| Incident and Recovery Reports | EHS | 5 years |
| Communication Logs | Admin | 3 years |

Review and Continuous Improvement

- Annual review of contingency plans and financial safety nets.

- Post-event reviews after each disruption.

- Integration of lessons learned into plans.

- Benchmarking against industry best practices.

- Employee feedback and suggestions.

- Technology upgrades for resilience.

- Engagement with insurers, banks, and authorities for improved terms.

Compliance

This SOP aligns with:

- Disaster Management Act, 2005

- IRDAI regulations

- RBI guidelines

- ISO 22301 and ISO 31000

- Factories Act, 1948

- Environment Protection Act, 1986

- Customer supply chain resilience requirements

- Haryana State Disaster Management Authority directives

# SECTION 27: SENSITIVE TRANSACTION APPROVAL AND MANAGEMENT PROTOCOL

Introduction

Alchem International is committed to conducting all sensitive, exceptional, or high-risk transactions with the highest standards of integrity, transparency, accountability, and regulatory compliance. As a manufacturer and exporter operating within global pharmaceutical, phytochemical, and nutraceutical supply chains, Alchem International recognizes that certain transactions may pose elevated financial, ethical, operational, or reputational risks if not governed through a structured approval framework.

This protocol establishes a formal, traceable, and auditable system to evaluate, authorize, execute, and document sensitive transactions. It reinforces Alchem International's governance commitments under ISO 9001 (Quality Management), ISO 14001 (Environmental Management), ISO 45001 (Occupational Health & Safety), ISO 37001 (Anti-Bribery Management Systems), customer compliance expectations, and internationally accepted principles related to ethical conduct, anti-corruption, and responsible business practices.

This SOP is aligned with:

- Companies Act, 2013

- Prevention of Corruption Act, 1988

- Foreign Corrupt Practices Act (FCPA), 1977

- UK Bribery Act, 2010

- Indian Contract Act, 1872

- Customer Codes of Conduct and third-party due diligence requirements

Objectives

This protocol is designed to:

- Define a standardized, transparent, and traceable decision-making process for all sensitive transactions.

- Prevent bribery, favoritism, undue influence, or conflicts of interest that may compromise business integrity.

- Ensure thorough financial, legal, regulatory, and ethical evaluation prior to approval.

- Strengthen internal oversight while enabling timely and compliant decision-making.

- Maintain accurate documentation to support internal audits, external audits, buyer assessments, and stakeholder assurance.

- Ensure compliance with applicable anti-corruption laws and customer requirements.

- Protect Alchem International's reputation and stakeholder trust.

Scope and Applicability

This SOP applies to all Alchem International employees, managers, consultants, and contract personnel involved in initiating, reviewing, recommending, or approving sensitive transactions.

The protocol covers, but is not limited to:

| Transaction Type | Examples |
| --- | --- |
| High-Value Financial Transactions | Capital expenditure, large procurement, major contracts |
| Procurement or Sales Deviations | Transactions deviating from approved terms, pricing, or payment conditions |
| Gifts, Hospitality, Sponsorships, Donations | Any transfer of value to third parties |
| Third-Party Commissions, Consultancy Fees, Rebates, Incentives | Payments to agents, consultants, intermediaries |
| Exceptional Concessions, Waivers, or Pricing Deviations | Discounts, credit terms, or waivers outside standard policy |
| Non-Monetary Transactions | Bartering, professional favors, service exchanges |
| Vendor Onboarding or Renewal with Elevated Risk | Suppliers in high-risk categories or geographies |
| Related-Party Transactions | Transactions with related parties as defined under Companies Act, 2013 |

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| Initiator / Department | Prepare and submit complete transaction proposals with justification, supporting documents, risk assessment, and cost analysis. |
| Finance & Legal / Compliance Teams | Evaluate financial exposure, contractual obligations, regulatory risks, ESG implications, and potential conflicts of interest. |
| Senior Management | Review and approve high-value, high-risk, or strategically significant transactions; ensure alignment with Alchem International's ethical and sustainability commitments. |
| Board / Audit Committee | Oversee the approval framework; review escalated cases; approve policy changes. |
| Internal Audit | Periodically audit sensitive transactions for compliance with this SOP. |

Approval Process and Procedures

1. Initiation

The transaction initiator must submit a comprehensive transaction proposal that includes:

- Purpose and business justification

- Strategic, operational, or commercial relevance

- Financial implications and cost-benefit analysis

- Stakeholder influence and conflict-of-interest assessment

- Draft agreements, quotations, comparative bids, and supporting documents

- Risk assessment covering financial, ethical, environmental, operational, and reputational aspects

- Compliance declaration confirming no violation of anti-corruption laws

No sensitive transaction may proceed without a complete, verified, and documented proposal.

2. Preliminary Review

The relevant Department Head conducts the initial screening to:

- Verify operational necessity and alignment with departmental objectives

- Confirm compliance with internal policies and approved budgets

- Identify any immediate ethical, legal, safety, or customer-compliance concerns

- Assess feasibility and preliminary risk classification

Proposals meeting departmental requirements are forwarded to Finance and Compliance for due diligence.

3. Due Diligence and Compliance Checks

Finance and Compliance teams jointly conduct a detailed evaluation covering:

| Area | Evaluation Criteria |
| --- | --- |
| Financial Soundness | Cost justification, budget impact, comparison with standard commercial terms, payment terms, tax implications |
| Legal and Regulatory Compliance | Compliance with applicable Indian laws, labour regulations, tax obligations, FEMA, contractual enforceability |
| Ethical and ESG Risks | Alignment with anti-bribery requirements, customer codes of conduct, Supplier Code, human rights, environmental impact |
| Conflict of Interest Review | Verification that no employee, supplier, or related party holds an improper personal interest |
| Reputational Risk Assessment | Potential impact on buyers, certifications, audits, and brand credibility |
| Sanctions and Watchlist Screening | Verification against UN, OFAC, EU, and Indian sanctions lists |

If risks remain unresolved, the transaction may be escalated, deferred, or rejected.

4. Approval Based on Transaction Tier and Risk Level

Alchem International categorizes sensitive transactions based on financial value, nature, and risk exposure:

| Tier | Risk Level | Value Range (Indicative) | Approval Authority | Documentation Required |
| --- | --- | --- | --- | --- |
| Tier 1 | Low | < ₹5 lakh | Department Head | Proposal, budget approval, basic due diligence |
| Tier 2 | Medium | ₹5–25 lakh | Department Head + Finance Head + Compliance Head | Proposal, due diligence report, risk assessment, conflict-of-interest declaration |
| Tier 3 | High | > ₹25 lakh or high-risk category | Managing Director + Board / Audit Committee (as applicable) | Full due diligence, legal review, risk assessment, compliance certification, board approval |

Tier 3 applies to:

- Strategic supplier or customer contracts

- High-value procurement or service engagements

- Vendor changes with significant operational or compliance impact

- Transactions involving ESG-sensitive materials or services

- Gifts, sponsorships, donations, or non-monetary exchanges above threshold

- Any transaction flagged for ethical, legal, or compliance concerns

- Related-party transactions

All non-monetary transactions (bartering, professional favors, service exchanges) are evaluated based on qualitative risk and escalated when required.

5. Final Authorization and Execution

Once approved:

- Execution teams proceed with contract finalization, negotiation, and financial processing.

- All legal formalities, compliance validations, and approvals must be completed prior to execution.

- A Final Transaction Record is compiled, including:

  - Signed approval forms

  - Due diligence and risk assessment reports

  - Contracts, amendments, and correspondence

  - Financial reconciliation and payment records

  - Compliance verification documentation

  - Sanctions screening results

Transaction closure is completed only after final reconciliation and formal archival.

Documentation and Recordkeeping

All sensitive transaction records are securely maintained in digital repositories, including:

| Record | Responsibility | Retention |
| --- | --- | --- |
| Transaction Proposal | Initiator | 7 years |
| Due Diligence Reports | Finance / Compliance | 7 years |
| Risk Assessment Reports | Compliance | 7 years |
| Conflict-of-Interest Declarations | HR / Compliance | 7 years |
| Approval Forms | Approving Authority | 7 years |
| Contracts and Amendments | Legal | Contract term + 7 years |
| Payment Records | Finance | 7 years |
| Sanctions Screening Records | Compliance | 7 years |
| Final Transaction Record | Compliance | 7 years |

System backups are performed periodically to ensure data integrity and audit readiness.

Training and Awareness

| Training Module | Audience | Frequency |
| --- | --- | --- |
| Sensitive Transaction Approval Process | All employees involved in procurement, finance, sales, and approvals | Annual |
| Anti-Bribery and Anti-Corruption | All employees | Annual |
| Conflict of Interest | All employees | Annual |
| Gifts and Hospitality Policy | All employees | Annual |
| Third-Party Due Diligence | Procurement, Compliance | Annual |

Continuous Improvement

Alchem International strengthens this protocol through:

- Periodic internal and external audits

- Annual review of approval thresholds and risk criteria

- Feedback from employees, auditors, and customers

- Updates aligned with evolving compliance and customer expectations

- Increased automation and alerts within ERP systems for transaction flagging

- Benchmarking against industry best practices

Findings from these activities are used to refine controls, improve efficiency, and uphold Alchem International's governance, ethics, and transparency commitments.

Compliance

This SOP aligns with:

- Companies Act, 2013

- Prevention of Corruption Act, 1988

- FCPA, UK Bribery Act, and other applicable anti-corruption laws

- Indian Contract Act, 1872

- FEMA and RBI guidelines

- Customer Codes of Conduct

- ISO 37001 Anti-Bribery Management Systems

- Alchem International's Code of Conduct and Ethics Policy

Review

This SOP is reviewed annually or upon:

- Changes in laws or regulations

- Customer requirement changes

- Audit findings

- Significant incidents

- Organisational restructuring

# SECTION 28: INCIDENT RESPONSE PROCEDURE

Introduction

Alchem International is committed to maintaining the highest standards of information security, data privacy, operational continuity, and ethical business conduct across all its manufacturing, administrative, and supply-chain operations. As a pharmaceutical and phytochemical manufacturing organization serving international customers, brands, and compliance-driven markets, the protection of confidential business information, employee data, customer specifications, and operational systems is essential for maintaining trust, regulatory compliance, and uninterrupted production.

This Incident Response Procedure (IRP) establishes a structured, timely, and traceable approach for identifying, reporting, containing, investigating, and resolving information security, data privacy, and operational system incidents. The procedure safeguards sensitive information, digital infrastructure, physical records, and internal systems from unauthorized access, misuse, loss, or disruption.

This SOP is aligned with:

- Digital Personal Data Protection Act (DPDP Act), 2023

- Information Technology Act, 2000 and IT (Reasonable Security Practices) Rules, 2011

- Drugs and Cosmetics Act, 1940

- ISO 27001 (Information Security Management)

- ISO 22301 (Business Continuity Management)

- Customer data protection and confidentiality requirements

Objectives

The primary objectives of this procedure are to:

- Protect confidential and sensitive information, including employee personal data, payroll records, customer specifications, production plans, costing data, quality reports, and compliance documentation.

- Ensure rapid detection, containment, and mitigation of information security incidents or data breaches.

- Minimize operational disruption, production delays, and reputational risks.

- Conduct structured investigations and risk assessments to prevent recurrence.

- Enable clear, controlled, and accurate communication during any incident.

- Ensure compliance with applicable labour laws, customer data-protection requirements, and contractual confidentiality obligations.

- Preserve stakeholder confidence among customers, auditors, regulators, suppliers, and employees.

Scope & Applicability

This SOP applies to all:

- Permanent employees

- Contract and temporary workers

- Interns and apprentices

- Consultants and service providers

- Third-party vendors with system, data, or site access

It covers incidents involving:

- Digital systems (ERP, payroll systems, attendance systems, production planning software, email, shared drives)

- Physical records (personnel files, audit reports, contracts, production records)

- Portable devices (USB drives, laptops, tablets, external hard drives)

- Cloud-based storage and collaboration platforms

- Network infrastructure and CCTV systems

- Any medium used to store or process confidential information

This procedure applies during working hours, off-hours, remote work, field operations, audits, and emergency situations.

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| Incident Response Team (IRT) | Comprising IT, HR, Compliance, and EHS representatives; lead incident response, investigation, and coordination. |
| IT / Systems Manager | Detect, contain, and investigate technical incidents; maintain logs; implement technical controls. |
| HR Manager | Handle employee-related incidents; ensure confidentiality; coordinate with legal. |
| Compliance Head | Assess legal and contractual obligations; coordinate with regulators and customers; maintain documentation. |
| Department Heads | Support incident response within their departments; ensure employee cooperation. |
| Senior Management | Review major incidents; approve communication and corrective actions. |
| All Employees | Report incidents immediately; follow instructions; preserve evidence; maintain confidentiality. |

Incident Response Procedure

Step 1: Immediate Action by First Responder

Any employee who discovers or suspects an incident must act immediately.

Treat All Incidents as High Priority

Unusual system behavior, missing files, unauthorized access alerts, suspicious emails, data leakage, or physical document loss must never be ignored.

Contain the Incident (If Safe to Do So)

- Disconnect affected systems from the network.

- Log out of compromised user accounts.

- Stop active data transfers or unauthorized access.

- Remove external storage devices involved.

Notify the Incident Response Team (IRT) Immediately

Incidents must be reported through any available internal channel:

- Designated incident reporting email

- IT / Compliance contact number

- Internal reporting form or ERP alert

- Direct phone or WhatsApp message to IT, HR, or Compliance

Document Critical Details

The reporter must record:

- Date and time of discovery

- Location (office, factory floor, server, system, file cabinet, etc.)

- Systems or documents affected

- Actions already taken

- Visible or suspected impact

Preserve Evidence

- Do not delete files, logs, emails, or CCTV footage.

- Do not reset devices unless instructed.

- Do not attempt independent fixes or data recovery.

Step 2: Incident Response Team (IRT) Actions

Severity Assessment

Incidents are classified as:

| Severity | Description | Examples |
| --- | --- | --- |
| High | Data breach involving employee data, buyer information, payroll, cyberattack, ransomware, system sabotage, theft of records | Ransomware, data exfiltration, unauthorized access to sensitive files |
| Medium | Malware detection, unauthorized internal access, compromised credentials | Phishing, malware, unauthorized login |
| Low | Accidental email misdelivery, policy deviations, near-miss events | Wrong recipient email, policy violation |

Containment Measures

- Disable affected user accounts.

- Block unauthorized network access.

- Isolate affected devices or systems.

- Secure physical locations or document storage.

Evidence Collection

- System and access logs

- Email trails and screenshots

- ERP audit trails

- CCTV footage (for physical incidents)

Internal Notifications

Inform relevant stakeholders:

- HR

- IT

- Compliance

- Department Heads

- Senior Management

Escalation (If Required)

Engage:

- External IT security or forensic experts

- Legal advisors

- Law enforcement (for theft, sabotage, or criminal acts)

- Buyer-mandated reporting channels where contractually required

Step 3: Communication Protocols

Internal Communication

- Employees are informed strictly on a need-to-know basis.

- Instructions are issued to prevent further risk.

- Confidentiality is reinforced to avoid rumors or misinformation.

External Communication

Used only if buyer, supplier, or third-party data is affected. Communication includes:

- Brief incident description

- Immediate containment actions

- Required actions by the stakeholder (if any)

- Designated contact person

Media & Public Communication

Handled exclusively by:

- Managing Director / CEO

- Authorized Senior Management Representative

Unauthorized disclosure is strictly prohibited.

Step 4: Legal and Compliance Coordination

The Compliance and Legal teams will:

Assess Legal and Contractual Obligations

Including:

- National data protection and labour laws (DPDP Act, IT Act)

- Customer codes of conduct and confidentiality clauses

- Audit and disclosure requirements

Prepare Notifications (If required)

Within defined timelines, covering:

- Nature of the incident

- Data or systems impacted

- Mitigation actions taken

- Preventive steps planned

Maintain Documentation

All incident records are retained for audits, buyer reviews, and legal reference.

Step 5: Continuous Improvement

After incident resolution:

1. Post-Incident Review

Evaluate:

- Root cause

- Detection and response gaps

- Communication effectiveness

- System or procedural weaknesses

2. Corrective and Preventive Actions (CAPA)

May include:

- System upgrades

- Stronger access controls

- Password and MFA enforcement

- SOP revisions

- Process redesign

3. Training & Awareness

- Refresher training for employees

- Phishing and data-handling awareness

- Controlled sharing of lessons learned

4. Incident Simulations

Conducted annually or semi-annually:

- Cyber-incident drills

- Data-loss simulations

- Mock breach response exercises

5. Annual Review of IRP

This procedure is reviewed annually or upon:

- Major incidents

- Regulatory updates

- Buyer requirement changes

- Technology upgrades

Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| Incident Reports | IRT | 7 years |
| Investigation Reports | IRT | 7 years |
| Evidence Logs | IT / Compliance | 7 years |
| Communication Logs | Compliance | 7 years |
| CAPA Records | IRT | 7 years |
| Training Records | HR | 3 years |
| Incident Simulation Reports | IRT | 3 years |
| Regulatory Notifications | Compliance | 7 years |

Training and Awareness

| Training Module | Audience | Frequency |
| --- | --- | --- |
| Incident Response Procedure | All employees | Annual |
| Data Protection and Privacy | All employees | Annual |
| Phishing and Social Engineering | All employees | Annual |
| Incident Reporting | All employees | Induction |
| IRT Advanced Training | IRT members | Annual |
| Mock Drills | All relevant staff | Half-yearly |

Compliance

This SOP aligns with:

- Digital Personal Data Protection Act, 2023

- Information Technology Act, 2000

- IT (Reasonable Security Practices) Rules, 2011

- Drugs and Cosmetics Act, 1940

- ISO 27001

- ISO 22301

- Customer data protection requirements

# SECTION 29: INTERNAL ASSESSMENT PROCEDURE

Introduction

Alchem International recognizes that effective governance, regulatory compliance, and continuous improvement depend on a robust internal assessment mechanism. As a pharmaceutical and phytochemical manufacturer serving global customers with strict quality, environmental, social, and governance (ESG) expectations, the company must regularly evaluate the effectiveness of its internal controls, policy implementation, and compliance with applicable standards.

This Internal Assessment Procedure establishes a structured, transparent, and organization-wide mechanism for conducting internal assessments across all Alchem International facilities and functions. These assessments evaluate the effectiveness of internal controls, implementation of company policies, ESG commitments, regulatory compliance, and alignment with certified management systems. The process supports operational excellence, legal compliance, risk mitigation, and continuous improvement.

This SOP is aligned with:

- ISO 9001:2015 (Quality Management Systems)

- ISO 14001:2015 (Environmental Management Systems)

- ISO 45001:2018 (Occupational Health & Safety Management Systems)

- ISO 50001 (Energy Management Systems – where applicable)

- ISO 27001 (Information Security Management – where applicable)

- Good Manufacturing Practices (GMP) as per WHO and Indian Drugs and Cosmetics Act, 1940

- UN Global Compact principles

- Customer Codes of Conduct and social compliance standards

- Applicable Indian labour, environmental, and corporate laws

Purpose and Objectives

The purpose of this procedure is to:

- Establish a structured, transparent, and organization-wide mechanism for conducting internal assessments.

- Evaluate the effectiveness of internal controls, policies, and procedures.

- Assess compliance with applicable laws, regulations, and customer requirements.

- Identify gaps, risks, and improvement opportunities.

- Support operational excellence, legal compliance, and risk mitigation.

- Provide objective evidence for management review and decision-making.

- Drive continuous improvement across all departments and functions.

- Prepare for external audits, customer assessments, and regulatory inspections.

Scope and Applicability

This SOP applies to all departments, production units, extraction and purification sections, synthesis operations, warehouses, quality units, supply chain operations, administrative offices, and support functions at Alchem International.

Internal assessments cover, but are not limited to:

| Assessment Area | Key Focus |
| --- | --- |
| Anti-Corruption and Ethical Business Controls | Bribery prevention, conflict of interest, gifts and hospitality |
| Labour Standards and Human Rights | Fair treatment, wages, working hours, child labour prevention, forced labour prevention |
| Workplace Conditions and Employee Welfare | Working conditions, welfare facilities, grievance mechanisms |
| Occupational Health and Safety | Safety compliance, PPE, incident management, emergency preparedness |
| Environmental and Energy Management | Emissions, waste, water, energy, chemical management |
| Information Security and Data Confidentiality | Data protection, access controls, incident response |
| ESG Commitments | Sustainability, community engagement, supplier ESG |
| Harassment-Free Workplace | POSH compliance, anti-discrimination, whistleblower protection |
| Quality Management | GMP compliance, documentation, batch records, deviations |
| Regulatory Compliance | Drugs and Cosmetics Act, Environment Protection Act, Factories Act, labour laws |

Assessments are aligned with applicable labour laws, customer compliance requirements, ISO-aligned management systems, UN Global Compact principles, and Alchem International's internal governance framework.

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| Internal Assessment Coordinator (IAC) | Designs, administers, monitors, and reports on internal assessments. Coordinates assessment schedules, communicates timelines, and ensures consistency in tools and methodologies. Reviews completeness, credibility, and supporting evidence submitted by departments. |
| Department Heads & Line Managers | Ensure accurate, truthful, and evidence-backed responses for their respective areas. Participate in clarifications, root-cause analysis, and preparation of corrective actions. |
| Compliance, HR, EHS, QA, IT, and Utility Teams | Provide subject-matter expertise during assessment reviews. Validate alignment with legal, customer, and internal policy requirements. |
| Management Review Committee (MRC) | Reviews consolidated assessment results. Approves corrective actions, assigns accountability, and monitors closure. Integrates assessment outcomes into enterprise risk management and business planning. |
| Internal Audit Team | Conducts independent verification of assessment findings. Reports to the MRC. |
| All Employees | Participate in assessments as required. Provide accurate information. Support corrective actions. |

Procedure

1. Annual Planning

At the beginning of each calendar year, the IAC prepares an Annual Internal Assessment Plan covering all functional areas. The plan is reviewed and approved by Senior Management and aligned with:

- Customer audit calendars

- Internal and external audit schedules

- CSR and ESG reporting timelines

- Risk prioritization for the year

- Regulatory inspection schedules

2. Design of Assessment Tools

Each thematic assessment uses standardized and updated formats that include:

- Binary questions (Yes / No / Not Applicable)

- Scored questions (1–5 rating scale)

- Sections for comments, explanations, and observations

- Manager declaration and approval section

- Evidence submission requirements (records, registers, photos, training logs, corrective action files)

Assessment templates are reviewed annually with relevant subject-matter experts to ensure continued relevance and audit readiness.

3. Rollout and Communication

Assessment formats are circulated through official email, ERP systems, or internal dashboards. Employees and managers are briefed on assessment objectives, response expectations, and timelines. Standard response window: 10–15 working days.

Assessments related to ethics, harassment, discrimination, or whistleblowing are anonymized to encourage honest and unbiased participation.

4. Review and Analysis

After submission:

- The IAC and functional reviewers verify completeness, accuracy, and supporting evidence.

- Clarifications are sought for inconsistencies or insufficient documentation.

- Scoring is assigned using predefined categories:

  - Fully Compliant

  - Partially Compliant

  - Non-Compliant

- Findings are supported with objective evidence such as records, photographs, system logs, or registers.

5. Reporting and Departmental Scorecards

Within 15 working days of assessment closure:

- Department-wise assessment reports and compliance scorecards are prepared.

Reports include:

- Compliance percentage

- Key gaps and observations

- Good practices identified

- Risk exposure levels

- Recommended corrective and preventive actions (CAPAs)

Reports are shared with Department Heads, the Management Review Committee, and the Compliance function.

6. Corrective Actions and Tracking

Each department must submit a CAPA plan within 10 working days of receiving the assessment report.

CAPAs are tracked through a centralized tracker capturing:

- Responsible owner

- Target completion date

- Current status

- Verification remarks

Progress is reviewed monthly, and delays or recurring non-compliances are escalated to Senior Management.

7. Final Consolidation & Management Review

At the end of the assessment cycle:

- The IAC prepares a consolidated Internal Assessment Summary Report.

- The report is presented during the annual Management Review Meeting (MRM) and highlights:

  - Key compliance risks

  - Trend analysis across departments

  - Recurring gaps and systemic issues

  - Department-wise performance

  - Strategic improvement priorities for the next cycle

Confidentiality

All assessment data—especially related to ethics, discrimination, harassment, or whistleblower matters—is treated as strictly confidential. Access is restricted to the IAC, Compliance, HR, and Senior Management only.

Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| Annual Internal Assessment Plan | IAC | 5 years |
| Assessment Tools and Templates | IAC | 5 years |
| Completed Assessment Forms | Departments | 5 years |
| Assessment Reports and Scorecards | IAC | 5 years |
| CAPA Plans and Trackers | Departments / IAC | 5 years |
| Consolidated Summary Report | IAC | 5 years |
| Management Review Minutes | HR / IAC | 5 years |
| Evidence Files | Departments | 5 years |

Continuous Improvement

Alchem International strengthens its internal assessment process through:

- Annual refinement of assessment tools, scoring criteria, and timelines

- Integration of feedback from employees, auditors, and customers

- Periodic training to improve response quality and evidence management

- Progressive adoption of digital assessment and tracking platforms

- Benchmarking against industry best practices

- Sharing of lessons learned across departments

Compliance

This SOP aligns with:

- ISO 9001, ISO 14001, ISO 45001, ISO 50001, ISO 27001

- Good Manufacturing Practices (GMP)

- Drugs and Cosmetics Act, 1940

- Environment Protection Act, 1986

- Factories Act, 1948

- Labour laws and POSH Act, 2013

- Customer Codes of Conduct

- UN Global Compact principles

Review

This SOP is reviewed annually or upon:

- Changes in regulatory requirements

- Customer requirement changes

- Major audit findings

- Significant incidents

- Organisational restructuring

# SECTION 30: SOP MANUAL REVIEW MECHANISM

Introduction

The Standard Operating Procedure (SOP) Manual at Alchem International Private Limited is a living document. It must evolve with changes in regulations, technology, customer expectations, operational processes, and organizational structure. An outdated SOP is not merely ineffective—it can create compliance risks, safety hazards, and operational inefficiencies.

This SOP Manual Review Mechanism establishes a structured, periodic, and trigger-based review process to ensure that all SOPs remain current, relevant, effective, and compliant with applicable Bangladesh labour laws, corporate regulations, and industry best practices.

This mechanism supports operational consistency, risk management, and continuous improvement across all functions at Alchem International.

Purpose and Objectives

The purpose of this procedure is to:

- Ensure the SOP Manual remains current, relevant, and effective.

- Comply with applicable Indian labour laws, corporate regulations, and industry best practices.

- Identify and remove outdated or redundant procedures.

- Integrate employee feedback and operational insights.

- Strengthen governance, risk control, and operational alignment.

- Support audit readiness and customer compliance.

- Drive continuous improvement across all functions.

Scope and Applicability

This SOP applies to all SOPs contained within the Alchem International SOP Manual, covering:

- Manufacturing operations

- Quality control and quality assurance

- Environmental, health, and safety (EHS)

- Human resources and administration

- Procurement and supply chain

- Information technology and data management

- Finance and compliance

- Sustainability and ESG

- Any other documented procedure

It applies to all departments, functions, and personnel involved in developing, reviewing, approving, or implementing SOPs.

Organizational Responsibilities

| Role | Responsibility |
| --- | --- |
| Managing Director / Senior Management | Approves major policy or framework updates. Provides strategic direction for SOP review. |
| HR Manager | Leads the annual SOP review process. Issues review notifications. Maintains version control. Coordinates with stakeholders. |
| IT / Systems Manager | Reviews IT operations, data security, and cybersecurity-related SOPs. |
| Operations Manager | Reviews production processes, operational controls, and efficiency-related SOPs. |
| EHS Head | Reviews environmental, health, and safety SOPs. |
| Compliance Head | Reviews regulatory and customer compliance SOPs. |
| Department Heads | Review departmental SOPs. Provide input on operational changes. |
| All Employees | Provide feedback on SOP effectiveness and usability. |

Review Frequency

1. Annual Comprehensive Review

An annual comprehensive review of the SOP Manual is conducted. All SOPs are reviewed at least once every year to ensure continued relevance and effectiveness.

2. Trigger-Based Reviews

Additional reviews are initiated following significant organizational, regulatory, technological, or operational changes.

Review triggers include:

- Amendments to Indian labour, environmental, or corporate laws

- Changes in regulatory expectations or compliance frameworks

- Cybersecurity or data protection risks

- Organizational restructuring or process changes

- Updates to ISO certifications or management systems

- Customer requirement changes

- Major audit findings

- Significant incidents or accidents

- Introduction of new products, processes, or equipment

- Technology upgrades

- Feedback from employees, customers, or auditors

Review Process

Step 1: Review Notification

The HR Department issues a formal review notification to relevant stakeholders at least one month in advance. The notification includes:

- Scope of review

- Timeline

- Responsibilities

- Required inputs

Step 2: Review Meetings

Designated stakeholders review existing SOPs, assess gaps, and propose updates or improvements. Review meetings may be conducted:

- Department-wise

- Function-wise

- Cross-functional (for SOPs affecting multiple departments)

Step 3: Feedback Integration

Employee and departmental feedback is documented, evaluated, and incorporated where relevant. Feedback may be collected through:

- Written submissions

- Meetings

- Surveys

- Suggestion boxes

- Audit findings

Step 4: SOP Updates

Approved revisions are formally documented and integrated into the SOP Manual with version control. Updates may include:

- Minor revisions (typographical, formatting, minor procedural changes)

- Major revisions (process changes, regulatory updates, structural changes)

- New SOPs

- Withdrawal of obsolete SOPs

Step 5: Communication and Training

Updated SOPs are communicated through internal channels, including email and intranet, and training is conducted where required.

Communication includes:

- Summary of changes

- Rationale for changes

- Effective date

- Training requirements

- Access to updated documents

Documentation and Approval

1. Version Control

All SOP changes are recorded with:

- Revision number

- Date of revision

- Summary of changes

- Justification notes

- Approving authority

2. Approval Authority

| Type of Change | Approval Authority |
| --- | --- |
| Minor Revisions | Department Head + HR |
| Major Revisions | Department Head + HR + Managing Director |
| New SOPs | Department Head + HR + Managing Director |
| Withdrawal of SOPs | HR + Managing Director |

3. Final Approval

The updated SOP Manual receives final approval from the Head of HR before release.

4. Document Control

- Master copy maintained by HR

- Controlled copies distributed to departments

- Obsolete copies withdrawn and destroyed

- Digital copies on intranet with version control

Accessibility

The latest approved SOP Manual is:

- Available digitally on the company intranet or shared drive

- Maintained in physical copies at relevant departmental offices for reference

- Accessible to all employees as required for their roles

Training and Awareness

- All employees are informed of SOP updates relevant to their roles.

- Training is conducted for major revisions.

- Training records are maintained by HR.

- Awareness sessions are conducted for new SOPs.

Continuous Improvement

Alchem International strengthens its SOP review mechanism through:

- Annual review of the review process itself

- Integration of feedback from employees, auditors, and customers

- Benchmarking against industry best practices

- Progressive adoption of digital document management systems

- Regular training of SOP owners and reviewers

Compliance

This SOP Manual Review Mechanism aligns with:

- Applicable Indian labour and corporate laws

- ISO 9001, ISO 14001, ISO 45001, ISO 27001

- Good Manufacturing Practices (GMP)

- Customer Codes of Conduct

- Industry best practices

Documentation and Records

| Record | Responsibility | Retention |
| --- | --- | --- |
| Annual Review Plan | HR | 5 years |
| Review Meeting Minutes | HR | 5 years |
| Feedback Records | HR | 5 years |
| Revision Summaries | HR | 5 years |
| Approval Records | HR | 5 years |
| Training Records | HR | 3 years |
| Obsolete SOPs (Archived) | HR | 5 years |
| Master SOP Manual | HR | Current version + 5 years |

Review

This SOP is reviewed annually or upon:

- Changes in regulatory requirements

- Customer requirement changes

- Major audit findings

- Significant incidents

- Organisational restructuring

Effective Date: 01.06.2026

Next Review Date: 01.06.2027

Approved By:  Sunil Malik- Senior Vice President