INDEX

OCCUPATIONAL HEALTH, SAFETY AND EMPLOYEE WELLBEING GUIDELINES	3

INCIDENT REPORTING AND INVESTIGATION	9

WATER CONSERVATION AND SUSTAINABILITY	13

AIR EMISSIONS MANAGEMENT GUIDELINES	19

ENERGY EFFICIENCY AND CARBON MANAGEMENT GUIDELINES	24

MATERIAL SUSTAINABILITY AND HAZARDOUS SUBSTANCE MANAGEMENT GUIDELINES	27

WASTE MINIMIZATION AND SUSTAINABLE DISPOSAL GUIDELINES	37

PRODUCT SUSTAINABILITY AND SAFE HANDLING GUIDELINES	49

PRODUCT DISPOSAL AND SUSTAINABLE PACKAGING GUIDELINES	55

EMERGENCY PREPAREDNESS AND RESPONSE PROCEDURE	59

CUSTOMER FEEDBACK	62

COMPLIANCE, ETHICS, AND RISK ASSESSMENT GUIDELINES	63

DATA PRIVACY AND RECORDS MANAGEMENT GUIDELINES	68

SUSTAINABLE PROCUREMENT AND ETHICAL SOURCING GUIDELINES	73

## OCCUPATIONAL HEALTH, SAFETY AND EMPLOYEE WELLBEING GUIDELINES

- Purpose

To establish preventive measures for protecting employees from occupational health risks and promoting a safe and supportive workplace. This SOP provides guidelines for ergonomic risk management, prevention of repetitive strain injuries, workplace stress and psychological wellbeing, hazardous substance exposure control, and occupational noise exposure.

The SOP aims to minimize occupational health risks through appropriate risk identification, preventive controls, employee awareness, monitoring, and continual improvement.

- Scope

This SOP applies to all employees, contract personnel, trainees, and other persons working within the organization's operational areas, offices, laboratories, warehouses, maintenance areas, and other applicable workplaces.

- Responsibilities

- Safety / EHS Function – Identification and assessment of occupational health risks, implementation of preventive controls, monitoring, training, and compliance oversight.

- HR / Employee Welfare Function – Coordination of employee wellbeing initiatives, communication channels, and applicable employee support measures.

- Occupational Health / Medical Function – Support for occupational health assessments, medical evaluation, health surveillance, and appropriate follow-up, where applicable.

- Operations / Department Heads – Implementation of occupational health controls within their respective work areas and ensuring employee adherence to safe work practices.

- Maintenance Teams – Maintenance of equipment, engineering controls, ventilation systems, noise-control systems, and other occupational health-related infrastructure.

- Employees and Contract Personnel – Follow established safe work practices, use required PPE, participate in training and monitoring activities, and report occupational health concerns or unsafe conditions.

- Supervisors / Section Heads – Identification of task-level risks, implementation of controls, employee communication, and follow-up of identified corrective actions.

- Accountability

- HOD – Safety / EHS.

- Head – Human Resources / Employee Welfare.

- Factory Medical Officer / Occupational Health Function, where applicable.

- HOD – Operations.

- Concerned Department Heads / Section Heads.

- Senior Management for strategic planning and resource allocation.

- Procedures

- Ergonomics and Prevention of Repetitive Strain Injuries

- Ergonomic Risk Identification

- Identify work activities involving repetitive movements, prolonged static postures, awkward postures, forceful exertion, frequent lifting or other activities that may contribute to musculoskeletal discomfort or repetitive strain injuries.

- Conduct ergonomic risk assessments for relevant workstations, tasks and work processes.

- Consider factors such as task frequency, duration, posture, force, reach, workstation arrangement and available recovery periods when assessing ergonomic risks.

- Identify employees or work groups exposed to significant ergonomic risks and prioritize appropriate preventive measures.

- Workstation and Task Optimization

- Optimize workstations, tools, equipment and work layouts to reduce awkward postures, excessive reaching, unnecessary force and repetitive movements.

- Where technically feasible, provide suitable mechanical aids, adjustable workstations, appropriate tools or other engineering solutions to reduce ergonomic strain.

- Review task design and work methods to identify opportunities to reduce unnecessary repetition and excessive physical effort.

- Ensure that ergonomic improvements do not compromise process safety, product quality or operational requirements.

- Repetitive Task and Manual Handling Controls

- Evaluate repetitive tasks and manual handling activities and implement suitable controls based on the identified risk.

- Use appropriate handling aids or equipment where manual handling presents a significant ergonomic risk.

- Where applicable, organize work activities to reduce prolonged exposure to repetitive or physically demanding tasks.

- Provide employees with appropriate guidance on safe posture, task execution and manual handling practices.

- Employee Awareness and Reporting

- Provide awareness or training to employees performing tasks with identified ergonomic risks.

- Encourage employees to report early signs of musculoskeletal discomfort, repetitive strain or other work-related ergonomic concerns to their supervisor, Safety/EHS or Occupational Health function.

- Reported ergonomic concerns shall be evaluated and appropriate corrective or preventive actions shall be identified.

- Where required, work activities or workstation arrangements shall be reviewed following reported ergonomic concerns.

- Workplace Stress and Psychological Wellbeing

- Workplace Stress Risk Identification

- Identify workplace factors that may contribute to excessive work-related stress, including excessive workload, prolonged working hours, work organization, inadequate communication, interpersonal concerns or other relevant workplace factors.

- Where appropriate, assess identified psychosocial risks as part of workplace risk assessments or employee wellbeing reviews.

- Maintain appropriate confidentiality when handling individual employee wellbeing concerns.

- Employees shall be provided with appropriate channels to raise workplace concerns without fear of retaliation.

- Preventive Workplace Measures

- Promote a work environment that supports respectful communication, reasonable work organization and appropriate employee support.

- Where significant work-related stress risks are identified, evaluate appropriate organizational or work-practice improvements.

- Encourage supervisors and managers to identify workload, work-organization or workplace issues that may adversely affect employee wellbeing.

- Promote awareness of available employee support resources and encourage employees to seek assistance when required.

- Occupational Hazardous Substance Exposure Control

- Exposure Risk Identification and Assessment

- Identify work activities, processes and job roles where employees may be exposed to hazardous chemicals, substances, vapours, gases, dusts or other hazardous materials.

- Evaluate potential exposure routes, including inhalation, skin contact, eye contact or accidental ingestion, as applicable to the substance and task.

- Consider the hazardous properties of substances, quantity handled, duration/frequency of exposure, process conditions and existing controls when assessing exposure risks.

- Use applicable SDS/MSDS and chemical hazard information when evaluating occupational exposure risks.

- PPE and Safe Work Practices

- Provide appropriate PPE based on the identified hazardous-substance exposure risk and applicable SDS/assessment requirements.

- Employees shall use, maintain and store assigned PPE in accordance with established requirements.

- Employees handling hazardous substances shall follow applicable safe work procedures, SDS instructions and emergency-response requirements.

- Damaged or unsuitable PPE shall be replaced as required.

- Exposure Monitoring and Health Surveillance

- Where required based on risk assessment, applicable regulatory requirements or occupational-health considerations, conduct workplace exposure monitoring for relevant hazardous substances.

- Where applicable, establish appropriate occupational health or medical surveillance for employees with identified exposure risks.

- Training and Communication

- Provide employees with information and training on hazardous-substance hazards, exposure routes, safe handling practices, PPE requirements, emergency measures and applicable SDS information.

- Provide additional training when significant changes are made to chemicals, processes, equipment or exposure-control measures.

- Employees shall be informed of applicable workplace controls and the importance of reporting leaks, spills, unusual exposures or unsafe conditions.

- Occupational Noise Exposure Control

- Identification of Noise Exposure Areas

- Identify work areas, equipment and activities where employees may be exposed to elevated occupational noise levels.

- Consider equipment such as rotating machinery, compressors, pumps, boilers, production equipment and other relevant noise-generating sources during assessment.

- Identify employees or work groups who may require additional noise exposure controls based on workplace assessment.

- Maintain identification of designated high-noise areas, where applicable.

- Occupational Noise Monitoring

- Conduct workplace noise measurements in relevant employee work areas using suitable and calibrated noise-measuring equipment.

- Conduct periodic noise monitoring based on identified risks and applicable requirements.

- Engineering and Administrative Noise Controls

- Prioritize engineering controls to reduce occupational noise at the source, including equipment enclosure, acoustic treatment, muffling, isolation, vibration control or equipment maintenance, where technically feasible.

- Maintain noise-generating equipment in suitable operating condition to minimize unnecessary noise generation.

- Where engineering controls alone are insufficient, implement appropriate administrative or work-practice controls to reduce employee exposure duration.

- Restrict access to designated high-noise areas where appropriate and communicate applicable requirements to employees and contractors.

- Hearing Protection and Employee Training

- Provide suitable hearing protection to employees working in areas where such protection is required based on the occupational noise assessment.

- Provide employees with information on occupational noise hazards, hearing-protection requirements and appropriate use and maintenance of hearing protection.

- Ensure employees understand applicable requirements for designated high-noise areas.

- Replace damaged or unsuitable hearing-protection equipment as required.

- Emergency Response for Occupational Health Incidents

- Maintain appropriate emergency arrangements for incidents involving hazardous-substance exposure, significant chemical releases, acute occupational health events or other workplace emergencies.

- Provide appropriate emergency response equipment, PPE, first-aid facilities and other resources based on identified workplace risks.

- Employees shall immediately report occupational exposure incidents, significant injuries, chemical exposure events or other occupational health emergencies to the concerned supervisor and Safety/EHS function.

- Provide appropriate first aid and medical assistance based on the nature of the incident.

- For hazardous-substance exposure, follow the applicable SDS/MSDS emergency and first-aid instructions.

- Investigate significant occupational health incidents to identify root causes and corrective/preventive actions.

- Maintain applicable emergency and incident records in accordance with the Incident Reporting and Investigation procedure.

- Review and Continuous Improvement

- Identify opportunities to improve engineering controls, work practices, workplace arrangements and employee support measures.

- Update risk assessments and control measures following significant process, equipment, chemical or workplace changes.

- Review the effectiveness of implemented occupational health controls and initiate further improvements where required.

- Update this SOP based on assessment findings, applicable regulatory requirements, technological developments and identified occupational health risks.

## INCIDENT REPORTING AND INVESTIGATION

1.0	Purpose

To identify areas of improvement and ensure actions are implemented with regard to emergency preparedness, response to incidents/accidents, and managing information security breaches to prevent further damage and reassure affected parties.

2.0	Scope

This procedure applies to all incidents, accidents, and information security breaches taking place at the TIL Ekalbara site and Registered Office.

3.0	Responsibility

- Concerned Section Heads – for incident and security breach reporting.

- Section Head (Safety) & HOD (PSM) – for incident investigation.

- Information Security Officer (ISO) – for investigating and responding to information security breaches.

- Concerned Section Heads – for compliance with corrective actions.

4.0	Accountability

HOD (Safety)

HOD (Information Technology)

5.0      	Description

5.1 	Categories of Incidents:

Incident includes work-related injuries, occupational illness, property damage, leakage, spillage, fire, near-miss events, and information security breaches such as unauthorized data access, phishing, malware attacks, and data leaks.

- Near-miss, first-aid, non-reportable & reportable.

- Unusual occurrences / dangerous occurrences like leakage, spillage, fire, or cybersecurity breaches.

5.2.  Definitions:

- Incident (Accident): Work-related event(s) in which an injury, ill health, fatality, loss, or damage (to person, property, or equipment) occurred or could have occurred.

- Near-miss: An incident where no injury or damage occurs but had the potential to do so.

- First Aid: Initial care for an illness or injury, typically performed by non-experts.

- Non-reportable Incident: An incident causing bodily injury that allows the affected person to resume duty within 48 hours.

- Reportable Incident: An incident causing death or injury preventing work for 48 hours or more.

- Reportable Incident:

Any incident which causes death or any bodily injury, preventing affected person from working for period of 48 hours or more.

- Fatal Incident:

If any incident occurs which result in death.

- Information Security Breach: Any unauthorized access, disclosure, alteration, or destruction of sensitive data, including system intrusions, phishing attacks, or accidental data exposure.

5.3	Procedure:

5.3.1	Incidents Reporting (First-aid, Non-Reportable, Reportable):

- Report incidents/accidents immediately to the safety department.

- Fill the prescribed Incident/Accident Report Form (TIL-EKB-SFT-FF-04) for physical incidents and the Information Security Incident Report Form (TIL-EKB-IT-FF-01) for security breaches.

- Submit reports to respective departments for verification and action.

- Medical Officer and Safety Officer to document and evaluate the incident, suggesting corrective actions.

5.3.2	Incidents Reporting (Unusual Occurrence / Dangerous Occurrence):

- Report events such as fires, chemical spills, or significant cybersecurity breaches using the relevant forms (TIL-EKB-SFT-FF-05 or TIL-EKB-IT-FF-01).

- Involve relevant personnel, including safety officers, IT teams, and section heads, to document and mitigate the issue.

5.3.3	Incidents Reporting (Near miss):

- Witnesses of near-misses must document them using the Near-miss Form (TIL-EKB-SFT-FF-13).

- Report to the safety or IT department (for cybersecurity near-misses) within 24 hours.

5.3.4    Information Security Breach Reporting:

- Employees must immediately report suspected breaches, phishing attempts, or system anomalies to the IT Helpdesk.

- The Information Security Officer will log the incident, assess the breach’s scope, and initiate an investigation.

5.4 	Incident/Accident Investigation (Including Information Security Breaches)

5.4.1 	General Guidelines:

- Investigate all incidents and breaches within 24 hours of occurrence.

- Identify root causes and corrective actions.

- Track corrective actions until completed.

- Review trends, identify gaps, and develop improvement plans.

5.4.2    Investigation Process:

- Conduct a thorough investigation by visiting the site (or analyzing system logs in case of breaches).

- Interview affected individuals, witnesses, supervisors, or IT personnel.

- Document findings in the relevant Incident Investigation Report (TIL-EKB-SFT-FF-14 or TIL-EKB-IT-FF-02).

5.4.3 	Investigation Team Composition:

- Operations

- PSM

- Safety

- Engg. Services

- Section Head / Supervisor of the respective plant.

- Shift Incharge

- Factory Medical Officer (as & when required)

5.4.4   Legal and Regulatory Reporting:

- Submit reports to regulatory bodies (e.g., Factory Inspector) as per the law for physical incidents.

- For data breaches, notify affected parties and, if required, regulatory authorities as per applicable data protection laws (e.g., GDPR, IT Act).

5.4.5   Communication and Review:

- Share incident details and corrective actions with concerned personnel, safety committees, or IT governance teams.

- Include incidents in monthly safety and IT security meetings.

- Revise related policies, HIRA, or work instructions based on findings.

5.4.6   Cybersecurity Incident Containment and Recovery:

- Isolate affected systems to prevent further damage.

- Collaborate with the IT team to mitigate risks and recover data.

- Communicate transparently with affected stakeholders about the breach and measures taken.

- Review and update security controls and employee training programs to prevent recurrence.

6.0	Records

- Accident Reporting Form -TIL-EKB-SFT-FF-04

- Unusual occurrence / dangerous occurrence reporting form- TIL-EKB-SFT-FF-05

- Near Miss Reporting Form - TIL-EKB-SFT-FF-13

- Incident Investigation Report- TIL-EKB-SFT-FF-14

- Records of form no. 21, 21-A and 29

- IT incidents reporting formTIL-EKB-IT-FF-01

- IT incident investigation Report TIL-EKB-FF-02

## WATER CONSERVATION AND SUSTAINABILITY

- Purpose

To define and implement effective strategies, technologies, and operational practices aimed at conserving water resources. This SOP focuses on water recycling, reuse, advanced cooling systems, groundwater protection, water-risk assessment and the integration of sustainable water management solutions across company operations.

- Scope

This document applies to all processes and activities undertaken by the organization to reduce water consumption, maximize reuse, and prevent water resource contamination at all operational sites.

- Responsibility

- Functional head-EPD – Implementation and monitoring of technologies and practices.

- Functional legal compliance – Ensuring adherence to legal and sustainability standards.

- Maintenance Teams – Regular inspection and maintenance of water conservation systems.

- Operations Heads – Supervision of daily operations to ensure compliance.

- Accountability

- Departmental Head- EPD, Legal compliance

- HOD (Operations).

- Senior Management for strategic decision-making and resource allocation.

- Description

- Recycling and Reuse of Water

    - Operational Processes:

- Identify water sources for reclamation such as process water, wastewater.

- Implement advanced water treatment technologies (e.g., membrane filtration, reverse osmosis, UV disinfection) to reclaim water.

- Develop and maintain closed-loop water systems for internal reuse in production, cooling, and landscape irrigation.

- Integrate water recycling systems with ECMs to enhance energy efficiency in water treatment and distribution.

- Where technically and operationally feasible, identify additional opportunities for recycling and reuse of treated water to reduce freshwater consumption.

- Evaluate suitable treated-water streams for reuse in utilities, cooling systems, gardening, or other approved non-potable applications, subject to applicable quality requirements

    - Documentation Requirements:

- Maintain technology installation records (e.g., water treatment units).

- Regularly update work instructions for water recycling procedures.

- Include data on reclaimed and reused water in the Annual Report or Sustainability Report.

- Maintain records of water recycling and reuse activities, including available quantity/volume data and relevant treatment or water-quality records.

    - Monitoring:

- Conduct regular assessments of treated water quality to ensure compliance with safety standards.

- Implement continuous monitoring systems for critical water reuse operations.

- Periodically review the performance of water recycling and reuse systems and identify opportunities for further improvement.

- Cooling Tower Water Efficiency and Optimization

    - Cooling Tower Operation and Water Efficiency:

- Operate cooling towers to optimize water consumption and minimize avoidable water losses.

- Use RO-treated water as cooling tower make-up water, where technically and operationally appropriate.

- Maintain appropriate cooling tower operating conditions, including monitoring of water quality and cycles of concentration (CoC), to minimize unnecessary blowdown.

- Monitor cooling tower evaporation, blowdown, leakage, and other significant water losses.

- Integrate waste heat recovery systems as part of ECMs to reduce cooling demands and associated water usage.

- Identify opportunities for further reduction, recycling, or reuse of cooling tower water through water audits and periodic operational reviews.

- Conduct routine inspection and maintenance of cooling towers to maintain efficient operation and prevent water losses.

    - Cooling Tower Documentation and Records:

- Maintain installation and maintenance records for cooling systems.

- Develop SOPs for the operation and maintenance of water-efficient cooling equipment.

- Include water consumption data for cooling purposes in the Annual Report.

- Maintain records of cooling tower make-up water consumption and blowdown, where applicable.

- Maintain cooling tower operation, inspection, maintenance, and water-quality monitoring records.

- Maintain relevant cooling tower water-quality, CoC, inspection, and maintenance records.

- Document water-efficiency measures implemented for cooling tower operations and available evidence of resulting water savings.

    - Cooling Tower Monitoring and Performance Review:

- Conduct routine maintenance of cooling systems to ensure efficiency.

- Monitor and document water usage reduction achieved through adopted technologies.

- Periodically review cooling tower water-use trends and identify opportunities for further water conservation.

- Track measurable improvements resulting from cooling tower water-efficiency measures, where data is available.

- Include significant cooling-water conservation initiatives in periodic environmental and sustainability performance reviews.

- Groundwater Protection and Pollution Prevention

    - Operational Processes:

- Implement groundwater monitoring programs to detect potential pollutants, including heavy metals, nitrates, and organic compounds.

- Establish control measures, such as:

  - Sealing potential contamination sources.

  - Designing and maintaining leak-proof chemical storage systems.

  - Treating industrial effluents to meet discharge standards before release.

  - Incorporate energy-efficient pumping and monitoring systems as part of ECMs to minimize energy use in groundwater management.

- Conduct groundwater quality testing at defined locations and intervals to identify potential contamination and changes in groundwater quality.

- Conduct soil quality testing at relevant locations to identify potential contamination associated with operational activities, spills, leaks, waste handling, or other potential pollution sources.

- Review groundwater and soil testing results and initiate appropriate corrective or preventive actions where potential contamination or abnormal results are identified.

    - Documentation Requirements:

- Maintain groundwater analysis reports.

- Prepare SOPs for effluent treatment and pollution control.

- Conduct regular wastewater quality tests to ensure effluent discharge complies with legal and environmental regulations.

- Include groundwater protection measures in the CSR or Sustainability Report.

- Maintain soil quality testing reports and associated environmental monitoring records.

- Maintain records of identified groundwater/soil contamination risks and corrective or preventive actions, where applicable.

- Maintain records of groundwater and soil monitoring results for periodic environmental review.

    - Monitoring:

- Conduct monthly groundwater quality testing.

- Maintain records of pollutant levels and corrective actions implemented.

- Monitor soil quality at defined locations and intervals based on identified environmental risks and applicable monitoring requirements.

- Review groundwater and soil monitoring trends periodically to identify potential pollution risks and improvement opportunities.

- Advanced Water Conservation Technologies and Rainwater Harvesting

    - Implementation:

- Integrate water-saving devices such as low-flow fixtures, smart irrigation controllers, and water-efficient process equipment.

- Utilize automated leak detection systems to promptly identify and address water losses.

- Align the deployment of water conservation technologies with ECM objectives to maximize resource efficiency.

- Implement and maintain suitable rainwater harvesting structures to capture rainwater and support groundwater recharge or other approved uses, where applicable.

- Maintain suitable conveyance, filtration, collection, storage, or recharge arrangements for rainwater harvesting systems, as applicable to the installed system.

- Inspect and clean rainwater harvesting structures periodically to maintain their effective operation.

- Identify opportunities to enhance rainwater harvesting and groundwater recharge based on site conditions, water availability, and water-risk assessment findings

    - Documentation:

- Maintain records of water-saving technology deployments.

- Document system performance reviews and maintenance schedules.

- Report annual water savings in the sustainability report.

- Maintain records of rainwater harvesting structures, locations, inspection, cleaning, maintenance, and repair activities.

- Maintain relevant technical assessments, completion certificates, photographs, and other supporting records for rainwater harvesting and groundwater recharge initiatives, where applicable.

- Where monitoring is performed, maintain records of groundwater levels and/or water quality associated with recharge activities.

    - Monitoring:

- Conduct regular audits to evaluate the effectiveness of water conservation technologies.

- Monitor key performance indicators (KPIs) related to water usage efficiency.

- Periodically review the effectiveness and condition of rainwater harvesting structures.

- Monitor available data on rainwater harvesting, recharge, and water conservation performance and identify opportunities for improvement.

- Water Risk Assessment and Management

    - Water Risk Assessment

- Conduct periodic water-risk assessments to identify risks related to water availability, water consumption, groundwater dependence, water quality, wastewater discharge, and other relevant water-related issues.

- Consider operational water requirements, water sources, local water availability, groundwater conditions, regulatory requirements, and potential environmental impacts when assessing water-related risks.

- Identify significant water-related risks and establish appropriate mitigation or preventive measures.

- Review water-related risks following significant changes in operations, water availability, regulatory requirements, or environmental conditions.

    - Documentation and Monitoring

- Maintain records of completed water-risk assessments.

- Document identified water-related risks, mitigation measures, responsible functions, and follow-up actions.

- Periodically review the effectiveness of water-risk mitigation measures and update actions where required.

- Wastewater Quality Monitoring and Compliance

    - Wastewater Quality Monitoring

- Conduct regular wastewater quality testing at designated monitoring points.

- Monitor relevant wastewater-quality parameters based on applicable regulatory requirements and the characteristics of the wastewater.

- Assess wastewater quality before discharge or reuse, as applicable.

- Review wastewater monitoring results to identify deviations, environmental risks, and opportunities for improvement.

- Initiate corrective and preventive actions where wastewater-quality results do not meet applicable requirements.

    - Wastewater Monitoring Records and Compliance

- Maintain wastewater-quality laboratory reports and monitoring records.

- Maintain records of sampling dates, sampling locations, test parameters, results, and applicable standards or limits.

- Maintain records of corrective and preventive actions arising from wastewater-quality monitoring.

- Retain wastewater monitoring records for regulatory, environmental, and sustainability review.

- Review and Improvement

- Review all water conservation measures annually during sustainability audits.

- Update SOPs, technologies, and practices based on advancements and audit findings.

- Update this SOP when significant changes occur in operations, water-management technologies, regulatory requirements, or identified water-related risks.

## AIR EMISSIONS MANAGEMENT GUIDELINES

- Purpose

To define and implement actions, technologies, and monitoring mechanisms aimed at reducing emissions of atmospheric pollutants, controlling environmental nuisances (noise, odor, vibration, light, and road congestion), and mitigating their impact on the environment. This SOP aims to ensure compliance with applicable environmental regulations and promote sustainable practices.

- Scope

This SOP applies to all operational activities, equipment, and processes across the organization that have the potential to generate air or environmental pollutants, noise, odor, or other nuisances. It covers all facilities, production sites, warehouses, and associated infrastructure.

- Responsibility

- Facility Management Teams – Implementation and maintenance of pollution control systems.

- Functional Head - EPD – Oversight and adherence to environmental regulations, including reporting to relevant authorities.

- Operations Teams – Daily compliance with emission reduction practices.

- Maintenance Teams – Routine inspection and repair of emission control systems.

- Quality Assurance Teams – Verification of compliance with environmental standards.

- Accountability

- Departmental Head - EPD.

- HOD (Operations).

- Senior Management for overall strategy and resource allocation.

- Description

5.1 Prevention of Atmospheric Pollutants and Environmental Nuisances

5.1.1 Actions Implemented:

- Upgrade production processes to improve efficiency and reduce pollutant generation, such as low-emission burners and energy-efficient systems.

- Install advanced pollution control equipment such as scrubbers, bag filters, and electrostatic precipitators (ESP Boilers) to capture and neutralize emissions at the source.

- Transition from fire boilers to ESP boilers utilizing briquette and biomass fuels, significantly reducing carbon emissions and enhancing fuel efficiency.

- Develop traffic management plans to minimize road congestion, including dedicated routes and optimized vehicle schedules.

- Use shielded or directional lighting to prevent light pollution while maintaining operational safety.

- Implement vibration damping measures such as vibration isolators for machinery and structural reinforcements.

- Ensure noise barriers and other soundproofing measures are in place to mitigate noise at site boundaries.

5.1.2 Documentation Requirements:

- Maintain installation and operational records of pollution control equipment.

- Include actions taken and outcomes in Annual Reports or Sustainability Reports.

- Maintain logs of compliance with local environmental regulations.

5.1.3 Regulatory Compliance References:

- Compliance with The Air (Prevention and Control of Pollution) Act, 1981.

- Adherence to Central Pollution Control Board (CPCB) Guidelines for emission standards.

- Alignment with ISO 14001:2015 for Environmental Management Systems.

- Observance of The Environment (Protection) Act, 1986 for environmental safeguards.

5.2 Dust and Particulate Matter Mitigation

5.2.1 Technologies Implemented:

- Install dust collection and extraction systems, including cyclone separators, and wet scrubbers.

- Utilize ESP boilers to enhance particulate matter control, ensuring compliance with stringent emission norms.

- Use water sprinkler systems to suppress dust & gases in high-risk areas such as construction zones and bulk material handling.

- Implement close system to control dusty operations, including vacuum systems for removal of fine particulate & gases.

- Introduce vegetative cover or green belts in areas prone to dust generation to naturally reduce dust emissions.

5.2.2 Monitoring and Documentation:

- Conduct routine inspections of dust control measures and maintain calibration records for monitoring equipment.

- Keep detailed logs of particulate matter reduction activities and their effectiveness.

- Include dust control measures and monitoring results in quarterly environmental compliance reports.

5.3 Air Quality Monitoring

5.3.1 Monitoring Mechanisms:

- Establish internal systems equipped with sensors and real-time monitoring tools to measure PM, NOx, SO2, VOC, and heavy metal levels.

- Engage third-party services for ambient air quality analysis at predetermined intervals.

- Conduct emissions dispersion modelling to assess potential impacts on the surrounding environment.

5.3.2 Reporting:

- Prepare periodic air quality analysis reports and submit them to regulatory authorities as required.

- Include air monitoring data, trends, and mitigation efforts in Annual and Sustainability Reports.

- Display key air quality metrics on-site for transparency and public awareness.

5.4 VOC, SO2, NOx, and Heavy Metal Emission Mitigation

5.4.1 Technologies Implemented:

- Install scrubbers, adsorption systems, vapor recovery units, and airtight seals to capture and neutralize emissions.

- Use catalytic or thermal oxidizers for VOC destruction in high-emission processes.

- Optimize combustion processes to reduce NOx formation and introduce selective catalytic reduction (SCR) systems.

- Implement advanced material handling systems to minimize heavy metal emissions during production.

- Implement energy conservation measures (ECMs) such as optimizing steam usage and upgrading boilers to reduce SO2 emissions.

- Leverage ESP boiler technology to improve efficiency in controlling SO2, NOx, and particulate emissions from biomass and briquette combustion.

5.4.2 Documentation and Monitoring:

- Maintain installation and operational records of all emission control technologies.

- Regularly evaluate the performance of emission control systems using third-party audits.

- Include emission mitigation data and impact assessments in Sustainability Reports.

5.5 Noise Emission Monitoring and Mitigation

5.5.1 Monitoring:

- Conduct regular noise measurements at site boundaries using calibrated sound level meters.

- Analyze noise data against permissible levels and prepare compliance reports.

- Engage certified third-party agencies for periodic noise monitoring to validate internal findings.

5.5.2 Mitigation Actions:

- Install acoustic enclosures, muffling controls, and noise barriers at high-noise equipment locations.

- Develop a noise prevention maintenance plan to replace or repair faulty equipment that contributes to noise pollution.

- Create a greenbelt around the facility to act as a natural sound barrier.

- Train employees on noise mitigation practices and the importance of adhering to noise control measures.

5.5.3 Documentation:

- Maintain maintenance logs, noise control system records, and employee training records.

- Include noise monitoring data and mitigation strategies in CSR/Sustainability Reports.

5.6 Odor Emission Mitigation

5.6.1 Actions Implemented:

- Use isolated storage and advanced ventilation systems to contain and disperse odors.

- Implement odor-neutralizing agents in storage and processing areas where odor generation is prevalent.

- Employ activated carbon filters and biofilters in exhaust systems to remove odorous compounds.

5.6.2 Documentation:

- Maintain records of odor control technology deployment and performance reviews.

- Include odor mitigation efforts, results, and community feedback in Annual Reports.

5.7 Leak Detection and Repair (LDAR) Program

5.7.1 Implementation:

- Develop a structured LDAR program for identifying and repairing leaks in valves, pumps, compressors, and other equipment.

- Use advanced monitoring tools such as leak detectors with alarm and gas analyzers.

- Establish a database to track detected leaks, repair schedules, and verification tests.

5.7.2 Monitoring and Documentation:

- Maintain detailed repair logs, program implementation records, and follow-up actions.

- Conduct quarterly audits of LDAR effectiveness and submit findings in Sustainability Reports.

- Emergency Response Protocols

- Establish an Emergency Response Plan (ERP) specific to air emission incidents, including procedures for accidental releases of hazardous pollutants.

- Define clear roles and responsibilities during emergencies, including first responders, communication leads, and environmental officers.

- Equip facilities with emergency kits, personal protective equipment (PPE), and spill containment tools.

- Conduct regular emergency drills to test response readiness and improve procedures based on drill outcomes.

- Maintain an emergency contact list, including local regulatory bodies and environmental response agencies.

- Review and Improvement

- Annual review of pollution control measures and SOPs during environmental audits.

- Update technologies and practices based on audit findings, advancements in environmental protection, and feedback from stakeholders.

- Engage external consultants for expert recommendations on emerging pollutant control technologies.

- Review the performance of ESP boilers regularly to ensure optimal emission control and compliance with environmental standards.

## ENERGY EFFICIENCY AND CARBON MANAGEMENT GUIDELINES

1.0 Purpose

To define and implement strategies, technologies, and monitoring mechanisms aimed at improving energy efficiency, reducing carbon emissions, and optimizing resource utilization. This SOP supports sustainable operational practices and ensures compliance with energy management regulations.

2.0 Scope

This SOP applies to all facilities, production sites, equipment, and processes within the organization that impact energy consumption and carbon emissions. It covers both existing operations and new projects.

3.0 Responsibility

- Facility Management Teams – Implementation and monitoring of energy efficiency measures.

- Energy Management teams – Oversight of energy management strategies and regulatory compliance.

- Operations Teams – Adoption of energy-efficient practices during daily activities.

- Maintenance Teams – Maintenance and performance checks of energy-efficient equipment.

- Sustainability Teams – Reporting and continuous improvement of carbon reduction initiatives.

4.0 Accountability

- Head – Engineering Services and Site head

- HOD (Operations)

- Senior Management for strategic decisions and resource allocation.

5.0 Description

5.1 Technological Advancements for Energy Efficiency

5.1.1 Actions Implemented:

- Implementation of energy-efficient technologies such as variable frequency drives (VFDs), high-efficiency motors, and LED lighting.

- Upgrade to ESP boilers using briquette and biomass, replacing conventional fire boilers for enhanced fuel efficiency and reduced carbon emissions.

- Deployment of automation systems to optimize process control and energy utilization.

5.1.2 Documentation Requirements:

- Maintain detailed records of equipment upgrades, energy audits, and efficiency reports.

- Include energy efficiency improvements in sustainability reports.

5.2 Waste Heat Recovery and Combined Heat & Power Systems

5.2.1 Systems Implemented:

- Installation of waste heat recovery systems (WHRS) to capture and reuse thermal energy from industrial processes.

5.2.2 Monitoring and Documentation:

- Regular inspections and performance assessments of WHRS.

- Documentation of energy savings and system efficiency in annual energy reports.

5.3 Fuel Switching for Enhanced Efficiency

5.3.1 Strategies Implemented:

- Transition from fossil fuels to cleaner alternatives such as biomass and natural gas to reduce carbon intensity.

- Optimization of combustion processes to maximize fuel efficiency and minimize greenhouse gas emissions.

5.3.2 Documentation:

- Maintain records of fuel consumption, conversion efficiency, and emissions data.

- Report on the environmental impact of fuel-switching initiatives.

5.4 Energy Monitoring and Reporting

5.4.1 Monitoring Mechanisms:

- Installation of real-time energy monitoring systems to track consumption patterns and identify inefficiencies.

- Conduct periodic energy audits to assess performance and recommend improvements.

5.4.2 Reporting:

- Preparation of periodic energy performance reports highlighting key metrics and progress towards energy goals.

- Transparent reporting to regulatory authorities and stakeholders.

6.0 Emergency Response Protocols

- Establish an Emergency Response Plan (ERP) specific to energy system failures, including procedures for power outages, equipment malfunctions, and energy supply disruptions.

- Define clear roles and responsibilities during emergencies, including energy system operators, emergency coordinators, and maintenance teams.

- Equip facilities with emergency energy backup systems, such as generators and uninterruptible power supplies (UPS), along with personal protective equipment (PPE).

- Conduct regular emergency drills focused on energy system resilience to test response readiness and improve protocols based on drill outcomes.

- Maintain an emergency contact list, including energy suppliers, technical support teams, and local regulatory authorities.

7.0 Review and Improvement

- Annual review of energy efficiency measures, technologies, and SOPs during internal audits.

- Update strategies and technologies based on audit findings, advancements in energy management, and stakeholder feedback.

## MATERIAL SUSTAINABILITY AND HAZARDOUS SUBSTANCE MANAGEMENT GUIDELINES

- Purpose

To establish sustainable material management practices aimed at reducing material consumption, promoting the use of eco-friendly and recovered materials, and ensuring the safe handling, treatment, and disposal of hazardous substances. This SOP supports operational efficiency, environmental protection, and compliance with regulatory requirements.

- Scope

This SOP applies to all operations, production processes, and supply chain activities where materials are consumed, handled, or disposed of within the organization. It includes guidelines for minimizing raw material usage, managing hazardous substances, and adopting sustainable practices.

- Responsibility

- Facility Management Teams – Implementation of material sustainability and hazardous substance management practices.

- Legal Compliance Teams – Oversight of legal compliance and sustainability protocols.

- Production and Operations Teams – Daily adherence to material efficiency and hazardous substance protocols.

- Safety and Maintenance Teams – Regular inspections and monitoring of hazardous material storage and disposal systems.

- Process/Technical Teams – Identification and evaluation of process optimization opportunities for material consumption reduction, recovery, reuse, and waste minimization.

- Environment, Health & Safety (EHS) Teams – Monitoring of hazardous waste handling, treatment, disposal, and applicable environmental performance indicators.

- Accountability

- Head EPD, Energy Management

- HOD (Operations).

- Senior Management for strategic planning and resource allocation.

- Relevant Process/Technical and EHS Heads

- Description

- Process Optimization and Material Consumption Reduction

- Operational Strategies:

- Adopt process optimization technologies to reduce material consumption, such as lean manufacturing techniques and automation systems.

- Improve product design and synthesis techniques to minimize raw material usage without compromising product quality.

- Utilize advanced monitoring systems to track material usage and identify areas for reduction.

- Evaluate production processes and operating parameters to identify opportunities to reduce raw material consumption while maintaining required product quality, yield, safety, and performance.

- Optimize reaction conditions, process parameters, material yield, and production efficiency to minimize material losses and unnecessary consumption.

- Evaluate opportunities to improve catalyst efficiency, catalyst recovery/reuse, chemical recovery, and recycling of suitable process inputs, wherever technically, operationally, and economically feasible.

- Evaluate process modifications or technological improvements that can achieve the required product specifications with lower material input.

- Where process changes are implemented for material efficiency, their technical feasibility, environmental benefits, product quality, and operational performance shall be evaluated and documented.

- Documentation:

- Maintain records of process improvements and material consumption reports.

- Include material efficiency metrics in sustainability audits.

- Maintain records of significant process optimization initiatives undertaken for reduction of material consumption, improvement in yield, or reduction of material losses.

- Where measurable data are available, record material consumption and relevant production/yield indicators before and after significant process improvements

- Material Efficiency & Process Optimization Register

| Sr. No. | Process / Activity | Material / Input | Improvement / Optimization Implemented | Purpose / Benefit | Supporting Record |
| --- | --- | --- | --- | --- | --- |
| 1 | Production Process | Raw Material / Process Input | Process optimization / parameter optimization | Reduction in material consumption | Process records |
| 2 | Production Process | Catalyst | Catalyst efficiency / recovery / reuse | Reduction in fresh catalyst requirement | Production records |
| 3 | Chemical Recovery | Recoverable Process Material | Recovery / recycling | Reduction in virgin material requirement | Recovery records |

- Recovery and Reuse of Input Materials

- Recovery Methods:

- Implement systems for the recovery of useful by-products, and chemical agents from production processes.

- Adopt closed-loop recycling practices within operations to minimize raw material requirements.

- Incorporate SO₂ recovery using a closed-loop system to enhance resource efficiency and reduce emissions.

- Identify suitable process streams, by-products, recovered chemicals, and other materials that can be safely recovered and reused within operations, subject to technical feasibility, quality requirements, and applicable regulatory requirements.

- Prioritize recovery and reuse of materials where this can reduce the requirement for virgin/raw materials or reduce waste generation.

- Maintain appropriate controls to ensure that recovered materials are suitable for their intended reuse and do not adversely affect product quality, process safety, or environmental performance.

- Documentation:

- Maintain logs of recovered materials and their subsequent use.

- Report on material recovery efforts in sustainability reports.

- Maintain records of material recovery, reuse, recycling, and closed-loop recovery initiatives, including relevant quantities where data are available.

- Document significant changes or improvements to material recovery systems and their environmental/resource-efficiency benefits.

- Material Recovery and Reuse Register.

| Sr. No. | Material / Process Stream | Source | Recovery Method | Reuse / Recovery Application | Supporting Record |
| --- | --- | --- | --- | --- | --- |
| 1 | SO₂ | SO₂ Recovery Process | Closed-loop recovery | Reuse in applicable production process | Recovery log |
| 2 | Process By-product / Chemical | Production Process | Recovery / recycling | Internal reuse / approved recovery | Recovery record |
| 3 | Other Recoverable Material | Relevant Process | Approved recovery method | Reuse / recycling | Material recovery record |

- Sustainable and Safer Material Selection

- Sustainable Material Practices:

- Use eco-friendly, biodegradable, or bio-based input materials to reduce environmental impact.

- Replace non-renewable resources with sustainable alternatives where feasible.

- Evaluate opportunities to substitute hazardous or higher-impact materials with less hazardous, lower-toxicity, eco-friendly, or bio-based alternatives where technically feasible.

- Assess the potential environmental, health, safety, quality, and regulatory implications before introducing an alternative material.

- Give preference to alternatives that reduce hazardous characteristics, environmental impact, or dependency on non-renewable resources without compromising product quality and process safety.

- Documentation:

- Maintain procurement records detailing eco-friendly materials used.

- Include sustainable material usage data in environmental impact assessments.

- Maintain records of significant material substitutions, including the replaced material, alternative material, basis for substitution, and applicable technical/safety evaluation.

- Maintain relevant supporting records demonstrating the environmental or hazardous-substance reduction benefits of material substitution, where available.

- Hazardous Substance Labelling, Storage, Handling, and Transportation

- Safety Protocols:

- Implement standardized labelling systems for hazardous materials, ensuring clear hazard identification.

- Design and maintain secure, compliant storage facilities with proper ventilation, spill containment systems, and leachate collection systems to prevent soil contamination.

- Establish handling protocols with mandatory use of personal protective equipment (PPE).

- Develop safe transportation guidelines following regulatory standards to minimize risk during material movement.

- Ensure compliance with the Hazardous Waste (Management, Handling, and Transboundary Movement) Rules, 2016, for all hazardous waste activities.

- Documentation:

- Maintain Material Safety Data Sheets (MSDS) for all hazardous substances.

- Keep transportation and storage logs for traceability.

- Record hazardous waste handling, disposal manifests, and safety inspection reports as per regulatory requirements.

- Specialized Treatment and Disposal of Hazardous Substances

- Treatment and Disposal Methods:

- Utilize specialized treatment facilities for hazardous waste, including chemical neutralization, incineration, and secured landfill facilities (SLF).

- Partner with certified hazardous waste disposal companies to ensure safe and compliant disposal.

- Implement effluent monitoring and analysis protocols to ensure compliance with GPCB & VECL norms, including regular testing of parameters such as pH, COD, BOD, TDS, heavy metals, and other pollutants.

- Ensure that hazardous waste is appropriately classified and managed according to its characteristics and applicable regulatory requirements before treatment, recovery, recycling, co-processing, or disposal.

- Transfer hazardous waste only to authorized/certified facilities, transporters, recyclers, recovery facilities, co-processing facilities, treatment facilities, or disposal facilities, as applicable.

- Select treatment or disposal methods based on the nature, characteristics, regulatory classification, and environmental risks associated with the hazardous substance or waste.

- Where technically and legally feasible, evaluate recovery, recycling, or co-processing options before final disposal.

- Ensure appropriate controls are maintained during the collection, storage, transportation, treatment, and final disposal of hazardous waste to minimize environmental and health risks.

- Periodically review applicable authorization, consent, membership, or service-provider documentation for hazardous-waste treatment and disposal arrangements

- Documentation:

- Maintain records of hazardous waste treatment and disposal activities.

- Include hazardous waste management data in sustainability and compliance reports.

- Document effluent monitoring reports, internal manifests, external manifests, and third-party analysis as required.

- Maintain hazardous waste manifests, transporter records, disposal/treatment certificates, facility authorization records, and other applicable records to demonstrate traceability from generation to final treatment, recovery, co-processing, or disposal.

- Maintain records of inspections or evaluations of relevant hazardous-waste service providers and facilities, where applicable.

- Safer Substance Substitution and Management

- Alternative Material Strategies:

- Transition from hazardous chemicals to Less hazardous, less toxic alternatives in production processes.

- Promote the use of bio-based feedstocks, environmentally friendly less toxic additives.

- Identify and evaluate opportunities to replace substances presenting higher environmental or health hazards with less hazardous alternatives, where technically feasible and consistent with product and process requirements.

- Evaluate greener solvents, reagents, additives, feedstocks, and other process inputs where suitable alternatives are technically and commercially feasible.

- Consider the potential impact of proposed substitutions on worker safety, process safety, product quality, regulatory compliance, environmental performance, and waste generation before implementation.

- Document significant substitution decisions and the basis for selecting the safer alternative.

- Documentation:

- Maintain records of materials replaced with safer alternatives.

- Report progress on hazardous material reduction in sustainability reports.

- Maintain documented assessments or approvals supporting significant hazardous-substance substitutions, where applicable.

- Maintain records demonstrating the environmental or hazard-reduction benefits of implemented substitutions, where measurable data are available.

- Hazardous Substance Substitution Register –

| Sr. No. | Substance / Waste Stream | Source / Process | Reduction / Substitution Initiative | Action Type | Environmental / Hazard Reduction Benefit | Supporting Evidence / Record |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | SO₃ / SO₃-based process input | SO₂ / related production process | Adoption of SO₃ to SO₂ route to eliminate SO₃ production | Process Modification / Hazard Reduction | Elimination of SO₃ production, storage and handling hazards; reduction in associated energy consumption and liquid effluent generation | Process improvement / Circular Economy project records |
| 2 | Spent HCl | Acid Chloride Process | Utilization of spent HCl for Epichlorohydrin production through the applicable recovery route | Material Recovery / Waste Reduction | Reduces dependency on disposal of spent HCl and associated environmental impact | Process recovery records / Circular Economy project records |
| 3 | SO₂ | SO₂ Recovery Plant | Recovery and reuse of SO₂ for applicable production process | Recovery / Reuse | Reduces fresh material requirement and supports closed-loop resource utilization | SO₂ recovery records / production records |
| 4 | Hazardous Waste | Multiple production processes | Process improvement, recovery and co-processing initiatives to reduce hazardous waste requiring final disposal | Waste Reduction / Recovery | Reduction in hazardous waste generation and increased recovery/recycling opportunities | Hazardous waste performance records / Circular Economy records |
| 5 | Recyclable / Recoverable Waste | Multiple operational areas | Increase in recyclable waste through recovery and recycling arrangements | Recycling / Recovery | Improved resource recovery and reduced waste requiring disposal | Waste/recovery performance records |
| 6 | Total Waste | Multiple operational areas | Waste minimization and recovery initiatives | Waste Reduction | Reduction in overall waste generation through prevention, recovery and improved waste management | Waste performance / Circular Economy records |

- Hazardous Substance and Waste Generation Reduction

- Mitigation Techniques:

- Implement process improvements that reduce the generation of hazardous substances.

- Adopt physical or chemical treatment methods to reduce toxicity levels in waste.

- Utilize pollution prevention technologies to minimize emissions and discharges.

- Identify processes and activities contributing to hazardous substance or hazardous waste generation and evaluate opportunities for prevention or reduction at source.

- Optimize process conditions, material utilization, recovery systems, and treatment processes to minimize hazardous waste generation.

- Where hazardous waste cannot reasonably be prevented, evaluate opportunities for recovery, recycling, reuse, co-processing, or appropriate treatment before final disposal, subject to technical and regulatory requirements.

- Use appropriate physical or chemical treatment methods, where applicable, to reduce hazardous characteristics or toxicity of waste before its final management.

- Monitor hazardous waste generation in relation to relevant production activity, where data are available, to identify trends and opportunities for improvement.

- Document significant process or technology modifications undertaken to reduce hazardous substance generation, waste quantity, or toxicity

- Documentation:

- Maintain data on hazardous substance generation and toxicity reduction.

- Include mitigation efforts in annual environmental performance reports.

- Maintain records of hazardous waste quantities, relevant waste categories, treatment/recovery routes, and disposal routes.

- Where measurable data are available, monitor trends in hazardous waste generation and relevant intensity indicators such as hazardous waste per unit of production.

- Maintain supporting records for significant waste-reduction, recycling, recovery, co-processing, or treatment initiatives

- Sustainable Packaging and Material Reduction

- Packaging Optimization:

- Design packaging with minimal material usage while ensuring product safety.

- Adopt dispatch of products in reusable, safe and durable ISO tanks as a packaging material.

- Evaluate opportunities to reduce packaging material consumption while maintaining product protection, safety, quality, and regulatory requirements.

- Where technically feasible, consider reusable or recyclable packaging solutions that reduce material consumption and waste generation

- Documentation:

- Maintain records of packaging material consumption.

- Report sustainable packaging initiatives in environmental impact reports.

- Emergency Response Protocols for Hazardous Materials

- Develop an Emergency Response Plan (ERP) for hazardous material incidents, including spills, leaks, and exposure events.

- Define roles and responsibilities for emergency response teams, environmental officers, and safety personnel.

- Equip facilities with emergency response vehicle, emergency response kits, spill containment tools, and PPE.

- Conduct regular emergency drills to ensure readiness and improve response protocols.

- Maintain an emergency contact list with local authorities, hazardous material response teams, and medical facilities.

- Review and Continuous Improvement

- Conduct annual reviews of material sustainability and hazardous substance management practices.

- Update SOPs based on audit findings, regulatory changes, and technological advancements.

- Engage external experts to provide recommendations for improving material efficiency and safety.

## WASTE MINIMIZATION AND SUSTAINABLE DISPOSAL GUIDELINES

- Purpose

To establish waste minimization strategies and sustainable disposal practices aimed at reducing internal waste generation, enhancing waste segregation, and ensuring environmentally sound management of hazardous waste. This SOP supports efficient resource utilization, regulatory compliance, and environmental sustainability.

- Scope

This SOP applies to all facilities, operational units, and waste management activities within the organization. It includes guidelines for waste reduction, sorting, recovery, mapping of waste streams, and proper disposal of hazardous and non-hazardous waste.

- Responsibilities

- Facility Management Teams – Implementation of waste reduction and disposal strategies.

- Safety & Environmental compliance teams – Oversight of waste management compliance and sustainability initiatives.

- Operations Teams – Daily adherence to waste segregation and minimization practices.

- Maintenance Teams – Supervision of waste handling equipment and infrastructure.

- Concerned Department Heads / Waste Generating Departments – Identification of waste generation points, implementation of source segregation and identification of opportunities for reuse, recovery or repurposing.

- EPD / Environmental Function – Maintenance of waste inventories, waste-stream mapping, manifest tracking, review of disposal routes and monitoring of compliance with applicable requirements.

- Accountability

- HOD – EPD and HOD-Safety.

- HOD (Operations).

- Senior Management for strategic planning and resource allocation.

- Concerned Department Heads shall ensure implementation of waste minimization, segregation, recovery and disposal requirements within their respective areas.

- Procedure

- Waste Prevention and Resource Recovery

- Waste Prevention and Reduction Measures:

- Promote material reuse by replacing disposable items with reusable alternatives wherever feasible.

- Identify opportunities to prevent or reduce waste generation at the source through improved process practices, efficient material utilization and operational controls.

- Prioritize waste prevention and reduction before recovery, treatment or final disposal, wherever technically feasible and legally permissible.

- Evaluate process activities that generate significant quantities of waste and identify opportunities to reduce material losses and avoid unnecessary waste generation.

- Material Recovery and Reuse Practices

- Implement recovery systems to reclaim valuable materials from production by-products.

- Identify suitable process by-products, residues and internally generated materials for recovery, recycling or reuse within operations, where technically feasible and approved.

- Promote recovery of useful materials before sending waste for final disposal.

- Where internal recovery or reuse is not feasible, suitable authorized recovery or recycling routes shall be evaluated before disposal.

- Utilize recovered waste materials, such as spent lime sludge, sulphur sludge, and spent catalyst, in compliance with approved recycling or disposal practices.

- Repurposing and Beneficial Utilization

- Repurpose obsolete equipment and materials within operations to reduce waste generation.

- Where technically and operationally feasible, identify suitable materials and equipment that can be repurposed for another approved use rather than being discarded.

- Reuse, recovery or repurposing shall be carried out without compromising product quality, process safety, environmental requirements or applicable legal requirements.

- Contaminated materials and chemical packaging shall not be reused unless such reuse is specifically permitted by applicable safety, SDS and regulatory requirements.

- Records and Performance Monitoring

- Maintain records of materials reused, recovered, or repurposed.

- Report waste reduction achievements in sustainability performance reviews.

- Maintain records, where available, of the type and quantity of materials reused, recovered, recycled or repurposed.

- Review waste-reduction and material-recovery performance periodically to identify further opportunities for improvement.

- Waste Prevention, Recovery and Reuse Performance Register

| Sr. No. | Waste / Material | Source / Department | Prevention / Recovery / Reuse Initiative | Supporting Record | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Spent Lime Sludge | TC Plant / ETP | Recovery / approved beneficial utilization | Waste / recovery records | Ongoing |
| 2 | Sulphur Sludge | SOL / TC Plant | Recovery / approved beneficial utilization | Waste / recovery records | Ongoing |
| 3 | Spent Catalyst | TC Plant | Recovery / recycling evaluation | Waste / recovery records | Ongoing |
| 4 | Other Recoverable Waste | Concerned Department | Reuse / recovery / recycling | Relevant records | Ongoing |

- Waste Segregation and Handling Controls

- Source Segregation and Waste Identification

- Implement internal waste sorting systems categorized into paper, glass, metals, plastics, organic waste, and hazardous waste.

- Segregate waste at the point of generation according to its applicable waste stream before transfer to storage, treatment, recovery, recycling or disposal.

- Hazardous and non-hazardous waste shall be kept segregated to prevent cross-contamination, unsafe mixing and inappropriate disposal.

- Waste shall be identified according to its nature, physical form, hazard characteristics and applicable treatment/disposal requirements.

- Incompatible hazardous waste streams shall not be mixed where such mixing could create a safety, environmental or treatment risk.

- Container Labelling and Storage Controls

- Utilize color-coded bins and clear labelling to facilitate proper waste segregation.

- Ensure proper labelling of hazardous waste containers as per Form No-8 and maintain compliance with GPCB regulations.

- Waste containers shall be suitable for the waste characteristics and maintained in designated storage areas to prevent leakage, spillage or uncontrolled release.

- Hazardous chemical residues and contaminated packaging shall be handled and stored with reference to the applicable Safety Data Sheet (SDS) and approved waste-management requirements.

- Waste Treatment and Disposal Routes

- Adopt specialized disposal methods for each waste category, such as recycling, composting, or incineration for hazardous waste.

- Waste shall be directed to an appropriate recycling, recovery, treatment or disposal route based on its characteristics and applicable regulatory requirements.

- Where recovery or recycling is technically feasible and legally permissible, such routes shall be considered before final disposal.

- Hazardous waste shall be transferred only through approved and authorized waste-management arrangements.

- Disposal methods shall be selected to prevent adverse impacts on human health and the environment.

- Waste Segregation Records

- Maintain logs of waste sorted and quantities directed to various disposal methods.

- Include waste segregation data in environmental compliance reports.

- Maintain records of waste quantities by waste stream and the corresponding recovery, recycling, treatment or disposal route, as applicable.

- Records shall be reviewed periodically to identify abnormal increases in waste generation and opportunities for improved segregation or recovery.

- Waste Flow Identification and Traceability

- Waste Generation Point Identification

- Identify significant waste-generation points across operational areas and document the applicable waste streams generated from each source.

- Identify the responsible department/area associated with each waste stream to support proper segregation, handling and accountability.

- Waste characteristics, physical form, storage requirements and applicable treatment/recovery/disposal route shall be identified, where applicable.

- Waste Flow Tracking

- Develop comprehensive waste stream maps to track waste flow from generation to final disposal.

- Identify critical control points to optimize waste reduction and recovery efforts.

- Conduct regular audits to assess the effectiveness of waste stream management.

- Track hazardous waste streams using the internal and external manifest system for regulatory compliance and traceability.

- Waste-flow tracking shall cover, as applicable, the movement from generation → segregation → internal storage/handling → transportation → treatment/recovery/recycling → final disposal or other approved destination.

- Waste-flow information shall be reviewed to identify opportunities for waste prevention, recovery, recycling and resource efficiency.

- Significant changes in processes, raw materials, waste generation points or disposal/recovery arrangements shall be considered when updating waste-flow information.

- Waste Management and Disposal Register

- Waste Mapping & Disposal Table

| Sr. No. | Type of Waste | Permitted Generation | Source | Physical Form | Packing Type | Approval required | Process waste disposal form/ Manifest | Storage area | Disposal Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Spent Lime Sludge (TC Plant) | 101.5 MT/Month | TC Plant, ETP Plant | Solid | Bags | HOD-Operation In charge-ETP | Manifest | ETP | Disposal at TSDF |
| 2 | Sulphur Sludge | 150 MT/Year | SOL Plant, TC Plant | Solid | Bags | HOD-Operation Incharge-ETP | Manifest | SOL Plant | Disposal at TSDF |
| 3 | Used Spent Oil | 10 MT/Year | SO2 Recovery, Maintenance, Utility | Liquid | HDFC Screw cap Barrel | HOD-Operation Incharge-ETP | Manifest | Hazardous waste storage area | Sale to authorized Refiners |
| 4 | Discarded Containers | 155 MT/Year | Acid Chloride Plants, TC Complex, SOL Plant, Tarak Chemicals | Solid | Store in decontamination area | HOD-Operation Incharge-ETP | Manifest | Hazardous waste storage area | Sale to authorized Re-cycler |
| 5 | Process Waste |  |  |  |  |  |  |  |  |
| a. | Thionyl Chloride Process Waste | 5 MT/Month | TC Complex | Solid, Semi Solid | HDPE Screw cap barrel & Open mouth carboy | HOD-Operation Incharge-ETP | Manifest | Hazardous waste storage area | Incineration to authorized CHWIF/ Sent to Pre-processer |
| b. | Acid Chlorides | 64.65 MT/Month | Acid Chloride Plants, 4-CBC waste from Tarak | Solid, Semi Solid, Liquid | HDPE Screw cap barrel & Open mouth carboy | HOD-Operation Incharge-ETP | Manifest | Hazardous waste storage area | Incineration to authorized CHWIF/ Sent to Pre-processer |
| 6 | Spent Carbon | 0.5 MT/Year | TC Plant | Solid | HDPE Screw cap barrel & Open mouth carboy | HOD-Operation Incharge-ETP | Manifest | Hazardous waste storage area | Incineration to authorized CHWIF |
| 7 | Sodium Chloride | 167.41 MT/Month | Samba – 3 Plant | Solid | HDPE Bags | HOD-Operation Incharge-ETP | Manifest | Hazardous waste storage area | Disposal at TSDF |
| 8 | Spent Catalyst | 0.5 MT/Month | TC Plant | Solid | Open mouth carboy | HOD-Operation Incharge-ETP | Manifest | Hazardous waste storage area | Disposal at TSDF |
| 9 | Acid Sludge from SO2 Recovery Plant | 0.5 MT/Month | SO2 & SOL Plant | Solid | HDPE Bags | HOD-Operation Incharge-ETP | Manifest | SOL Plant | Disposal at TSDF |
| 10 | Incineration Ash | 6 MT/Month | Incinerator, Agro Waste Boiler | Solid | Bags | HOD-Operation Incharge-ETP | Manifest | Incinerator | Disposal at TSDF |
| 11 | Resin (Cation, Anion) | 2 MT/Year | SOL Plant | Solid | HDPE Bags | HOD-Operation Incharge-ETP | Manifest | SOL Plant | Disposal at TSDF |
| 12 | Solid waste generated from APMC system (SO3 - Oleum Project) | 10 MT/Year | SOL Plant | Solid | HDPE Bagas | HOD-Operation Incharge-ETP | Manifest | SOL Plant | Disposal at TSDF |
| 13 | Distillation process waste (imido bis Sulfuryl Chloride) | 7.6 MT/Month | Samba- 3 | Semi Solid | HDPE Drum | HOD-Operation Incharge-ETP | Manifest | Hazardous waste storage area | Incineration to authorized CHWIF/ Sent to Pre-processer |
| 14 | Hydrochloric Acid | 3001 MT/Month | Acid Chloride Plants | Liquid | Tanker | HOD-Operation Incharge-ETP | Manifest | Store in dedicated storage tank | Disposal at selling to actual users |
|  |  | 2026 MT/Month |  |  |  |  | Manifest |  |  |
| 15 | Aluminium Chloride Solution | 1330 MT/Month | Acid Chloride Plants | Liquid | Tanker | HOD-Operation Incharge-ETP | Manifest | Store in dedicated storage tank | Disposal at selling to actual users |
| 16 | Sodium Sulphite OR Sodium Bisulphite (by Product) | 10698 MT/Month | Acid Chloride Plants | Liquid | Tanker | HOD-Operation Incharge-ETP | Manifest | Store in dedicated storage tank | Disposal at selling to actual users |
|  |  | 3123 MT/Month |  |  |  |  |  |  |  |
| 17 | Electronics Waste | - | Instrument EDP, Electric | Solid | Store in designated area | HOD-Operation Incharge-ETP | Manifest | EDP | Sale to authorized re-cycler |
| 18 | Battery Waste | - | Electric Dept | Solid | Store in designated area | HOD-Operation Incharge-ETP | Manifest | Electric | Sale to authorized re-cycler |
| 19 | Cloth Waste Chindi | - | Acid Chloride Plants, SO2 recovery, TC, SOL Plant, ISO Tank, Utility, MNT. | Solid | Bags | HOD-Operation Incharge-ETP | Manifest | Plant area | Incineration |
| 20 | LRB Insulation Material | - | Various Plants during Insulation Activity | Solid | Store in designated area | HOD-Operation Incharge-ETP | Manifest | Scrap Yard | Disposal at TSDF |
| 21 | Bio Medical Waste | - | OHC | Solid | Open mouth carboy/ dustbin | HOD-Operation Incharge-ETP | Manifest | OHC | Incineration at GPCB approved BMW Incinerator |
| 22 | Empty Glass/ Plastic Bottles & Broken Glassware | - | QC Lab, ETP Lab, R&D Lab | Solid | Store in designated area | HOD-Operation Incharge-ETP | - | Scrap Yard | Sale to scrap recycler |
| 23 | Effluent Water | 287.28 KL/Day | ETP | Liquid | - | HOD-Operation Incharge-ETP | - | ETP Plant | Disposal at VECL Channel |

- The Waste Management and Disposal Register shall be periodically reviewed and updated to ensure that the information accurately represents the current waste-management arrangements.

- Where applicable, the register shall identify whether a waste stream is reused, recovered, recycled, treated, incinerated, co-processed, disposed of at a secured facility/TSDF, or managed through another approved route.

- Review and Documentation

- Maintain updated waste stream maps and audit reports.

- Include mapping outcomes in annual environmental performance assessments.

- Maintain supporting waste inventories, waste-management records, manifests and disposal/recovery records, as applicable.

- Review waste-flow information during environmental audits to identify gaps in segregation, traceability, recovery and disposal practices.

- Corrective actions identified during review or audit shall be tracked to completion.

- Hazardous Waste Transportation and Disposal Controls

- Hazardous Waste Movement Requirements

- Restrict transboundary movement of hazardous waste unless it aligns with environmentally sound management practices.

- Hazardous waste shall be managed through authorized and environmentally sound treatment, recovery, recycling or disposal routes.

- Hazardous waste shall not be transferred to unauthorized persons, facilities or disposal routes.

- Where transboundary movement of hazardous waste is applicable, such movement shall be prohibited or restricted unless all applicable legal requirements, approvals and environmentally sound management conditions are satisfied.

- The organization shall maintain appropriate information regarding the origin, quantity, nature, movement and final destination of hazardous waste, as applicable.

- Manifest and Transportation Controls

- Implement waste incentivization protocols to track hazardous waste from generation to final disposal.

- Ensure compliance with the Hazardous Waste (Management, Handling, and Transboundary Movement) Rules, 2016, including the use of Form-10 for manifest tracking.

- Maintain applicable manifests and transportation records to provide traceability of hazardous waste movement.

- Verify that hazardous waste is transported through approved arrangements and reaches the designated treatment, recovery or disposal destination.

- Maintain records of waste movement from the point of generation/storage to the final approved destination.

- Authorized Waste Handler and Final Destination Controls

- Establish agreements with third-party waste handlers to ensure traceability of hazardous waste to its final treatment or disposal location.

- Third-party waste handlers shall be appropriately authorized for the relevant waste stream, wherever such authorization is applicable.

- The final treatment, recovery, recycling or disposal destination of hazardous waste shall be identifiable from applicable records.

- Disposal/recovery arrangements shall be reviewed periodically to ensure continued suitability and traceability.

- Where hazardous waste is sent for recovery or recycling, records shall identify the relevant authorized receiving facility, where applicable.

- Compliance Verification and Records

- Maintain hazardous waste tracking records, manifests, and third-party agreements.

- Include hazardous waste movement data in compliance and sustainability reports.

- Maintain waste inventories and final disposal/recovery records, including applicable disposal certificates, manifests, transportation records or equivalent documentation.

- Conduct periodic internal reviews of hazardous waste storage, movement, treatment, recovery and disposal arrangements.

- Document any deviations identified during such reviews and track corrective actions to completion.

- Resource Efficiency and Circularity

- Resource Efficiency Measures

- Promote resource optimization by reducing raw material consumption and increasing product lifespan.

- Identify opportunities to improve material utilization efficiency and reduce the quantity of materials becoming waste during operations.

- Consider waste prevention, material recovery and reuse when evaluating resource-efficiency improvement opportunities.

- Circular Resource Utilization

- Integrate circular economy principles, encouraging the recycling and upcycling of materials within operations.

- Promote reuse, recycling, recovery and beneficial utilization of suitable materials within operational processes, subject to technical feasibility, safety and applicable regulatory requirements.

- Evaluate opportunities for closed-loop material recovery and reuse where technically feasible.

- Prioritize recovery and circular utilization of suitable resources before final disposal.

- Resource Efficiency Performance Records

- Maintain records of circular economy initiatives and resource efficiency projects.

- Report resource optimization achievements in sustainability disclosures.

- Maintain records of significant resource-efficiency, recovery, recycling and circularity initiatives and their outcomes, where measurable.

- Review identified opportunities periodically as part of environmental performance improvement.

- Waste Data Management and Performance Monitoring

- Waste Data Collection and Tracking

- Collect and maintain relevant information on waste generation, segregation, recovery, recycling, treatment and disposal.

- Track waste quantities by applicable waste stream and disposal/recovery route.

- Use waste data to identify significant waste streams and opportunities for reduction, recovery and resource optimization.

- Digital Records and Data Analysis

- Implement digital tracking systems for real-time monitoring of waste generation, segregation, and disposal.

- Utilize data analytics to identify waste reduction opportunities and improve operational efficiency.

- Where digital systems are available, use them to support traceability of waste-management activities and maintain reliable records.

- Analyze available waste data to identify trends, unusual increases in waste generation and opportunities for improvement.

- Environmental Performance Reporting

- Maintain digital records of waste management activities.

- Generate periodic waste management reports for internal and external stakeholders.

- Include relevant waste-reduction, recovery, recycling and disposal performance information in environmental or sustainability reviews, where applicable.

- Waste-management performance shall be reviewed against applicable environmental objectives and improvement actions.

- Emergency Response Protocols for Waste Incidents

- Develop an Emergency Response Plan (ERP) specific to waste-related incidents, including spills, leaks, and hazardous waste exposure events.

- Define roles and responsibilities for emergency response teams and environmental officers.

- Equip facilities with emergency spill kits, containment tools, and personal protective equipment (PPE).

- Conduct regular emergency drills to enhance preparedness and improve response procedures.

- Ensure emergency response plans address specific waste incidents, including chemical spills and hazardous waste leaks, in accordance with MSDS guidelines.

- Maintain an emergency contact list with local authorities, hazardous waste response teams, and environmental agencies.

- Review and Continuous Improvement

- Conduct annual reviews of waste minimization strategies, disposal practices, and compliance with regulatory standards.

- Update SOPs based on audit findings, technological advancements, and sustainability targets.

- Engage external experts to identify innovative waste management solutions.

- Continuously monitor the performance of waste management systems, including secured landfill facilities, hazardous waste treatment plants, and effluent monitoring programs, to identify areas for improvement and ensure sustainable practices.

## PRODUCT SUSTAINABILITY AND SAFE HANDLING GUIDELINES

- Purpose

To establish best practices for efficient product use, safe storage, and maintenance to extend product life, reduce environmental impact, and minimize hazardous emissions. This SOP ensures compliance with sustainability objectives while enhancing product safety and performance.

- Scope

This SOP applies to all chemical and durable products manufactured, stored, and distributed by the company. It includes guidelines on product efficiency, optimized design, safe handling, storage, labelling, and reduction of toxic discharges.

- Responsibility

- Product Development Teams – Implementation of sustainable product designs.

- Quality Assurance Teams – Verification of compliance with safety and environmental standards.

- Legal compliance teams – Oversight of regulatory adherence.

- Operations and Storage Teams – Proper handling, labelling, and storage of products.

- Accountability

- Head of Legal Compliance.

- HOD (Operations).

- Senior Management for strategic planning and resource allocation

- Procedures

- Efficient Product Use, Storage, and Maintenance

- Product Use and Handling Practices:

- Develop and maintain appropriate product guidelines for customers/users to support efficient and intended use of products and, where applicable, extend the useful life of durable products.

- Provide relevant instructions on recommended product use, handling and application practices to minimize unnecessary resource consumption, product loss and wastage while maintaining product effectiveness.

- For chemical products, provide applicable Safety Data Sheets (SDS), product labels and technical information covering safe handling, application and storage requirements.

- Ensure that product-use information supports appropriate application and handling practices to minimize leakages or unintended release of harmful substances.

- Ensure proper storage conditions to prevent degradation, leakages, and environmental contamination.

- Where applicable, establish and communicate maintenance requirements for durable products/equipment to maximize service life and reduce associated waste.

- Product Storage and Leakage Prevention:

- Specify appropriate storage conditions, including temperature, ventilation, container closure and protection from incompatible substances, as applicable to the product.

- Ensure that product containers are maintained in suitable condition and kept properly closed to prevent leakage, deterioration or uncontrolled release.

- Provide precautionary information for preventing product release to drains, soil or other environmental receptors, where applicable.

- Review product storage and handling information periodically and update it when product characteristics, regulatory requirements or safety information change.

- Product Information and Customer Communication:

- Provide relevant product-use and safety information through applicable product labels, SDS, technical documents or other approved customer communication channels.

- Ensure that information provided to customers/users is consistent with the approved product specification, SDS and applicable regulatory requirements.

- Maintain records demonstrating the provision and communication of applicable product-use, storage and handling information.

- Product Use, Storage and Handling Information Register:

- Maintain a Product Use, Storage and Handling Information Register for applicable products.

- The register may include product name, intended use/application, recommended handling/use information, storage conditions, leakage/release prevention measures, applicable SDS/label reference and customer/user information provided.

- Review the register periodically and update it following relevant product, process, regulatory or safety changes.

| Sr. No | Product | Recommended Use / Handling Information | Storage Conditions | Leakage / Release Prevention | SDS / Label Reference | User Information Provided | Record Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4-CBC | As per approved product information | As specified in label/SDS | Prevent uncontrolled release | GHS Label / SDS | Yes / To be recorded | Product record |
| 2 | Isophthaloyl Dichloride | As per approved product information | As specified in SDS | Prevent entry into drains/soil | SDS | Yes / To be recorded | Product record |

- Chemical Product Efficiency and Sustainable Design

- Product Efficiency and Application Optimization:

- Evaluate opportunities during product development, modification or improvement to enhance chemical product efficiency and achieve the intended performance with reduced dosage, quantity or application frequency, where technically feasible.

- Consider product characteristics, formulation, concentration, reaction/application efficiency and performance requirements that may enable reduced material consumption during intended use.

- Where an improved product demonstrates equivalent or improved performance at a lower dosage, quantity or application frequency compared with a reference or existing product, document the technical evaluation and comparison.

- Ensure that product efficiency improvements do not compromise product quality, safety, regulatory compliance or intended performance.

- Dosage and Application Efficiency Evaluation:

- Conduct appropriate technical trials, product comparison studies or performance evaluations, where applicable, to determine whether product improvements can reduce dosage, quantity or frequency of use.

- Document the basis for any claimed reduction, including the reference product/condition, improved product characteristic, dosage or application requirement and corresponding performance results.

- Use documented technical evidence to support communication of product efficiency benefits to relevant customers/users, where applicable.

- Documentation:

- Maintain formulation improvements and product comparison studies.

- Maintain records of technical evaluations, trials and test results demonstrating optimized dosage, quantity or application frequency, where such improvements are established.

- Record test results of optimized dosage and efficiency.

- Maintain records of product efficiency improvement initiatives and their implementation status.

- Chemical Product Efficiency and Dosage Optimization Register:

- Maintain a register, where applicable, identifying products evaluated for dosage, quantity or application-frequency optimization.

- The register should include the product/reference product, application, existing and improved product characteristics, dosage/quantity/frequency comparison, performance results and supporting technical records.

- Environmental Impact and Toxic Discharge Reduction in Product Design

- Sustainable Chemical Product Design:

- Evaluate product formulation and design options that may reduce toxic discharges, harmful residues or environmental releases during the intended use of chemical products, where technically feasible.

- Develop less hazardous chemical alternatives that reduce harmful emissions.

- Utilize less hazardous additives to ensure safe environmental degradation.

- Eliminate or substitute hazardous substances in formulations to comply with global safety standards.

- Consider product composition, hazardous substance content and potential environmental impacts when evaluating product improvements or alternative formulations.

- Hazardous Substance Substitution and Product Improvement:

- Evaluate the substitution or reduction of hazardous substances with less hazardous or less toxic alternatives, where technically feasible and consistent with product performance and regulatory requirements.

- Where a product modification demonstrates reduced toxic discharge potential or reduced environmental impact compared with an existing/reference product, document the technical basis and comparative assessment.

- Consider available product safety, environmental and technical information during product development and improvement activities.

- Environmental Performance Evaluation:

- Where applicable, assess relevant environmental characteristics of improved chemical products, including potential toxic discharges, harmful residues, emissions or environmental releases.

- Use appropriate technical studies, product comparison assessments, environmental assessments or other documented evidence to substantiate environmental improvements.

- Ensure that environmental improvements do not adversely affect product quality, safety, intended performance or applicable regulatory requirements.

- Documentation:

- Maintain chemical safety data sheets (MSDS/SDS) and environmental compliance reports.

- Maintain records of product formulation improvements, hazardous substance substitution and environmental performance evaluations.

- Document comparative evidence where product improvements demonstrate reduced toxic discharge or environmental impact.

- Include toxic discharge reduction measures in sustainability performance reviews.

- Chemical Product Environmental Improvement Register:

- Maintain a Chemical Product Environmental Improvement Register for applicable product-development and improvement initiatives.

- The register should include the product, environmental concern, existing product characteristic, design/formulation improvement, expected environmental benefit, supporting technical evidence and applicable safety/regulatory review.

- Product Labelling and Safety Information

- Labelling Requirements:

- Include hazard statements and precautionary statements for safe handling.

- Include relevant recommended-use, handling, storage, disposal and emergency response information, as applicable to the product.

- Clearly indicate applicable storage conditions, temperature requirements, container/handling precautions and expiration or retest details, where relevant, to prevent misuse or degradation.

- Ensure product labels provide information consistent with the applicable SDS and approved product information.

- Documentation:

- Maintain product labels and regulatory approval records.

- Maintain applicable SDS and technical/product information supporting the instructions provided to users.

- Ensure all products comply with national and international labelling standards.

- Digital Monitoring and Traceability

- Digital Product Information and Traceability:

- Use digital logs to track compliance with safety measures and environmental regulations.

- Where applicable, maintain digital records of product information, SDS, technical documents, product-use information and product improvement records to support traceability.

- Documentation:

- Maintain digital records of product lifecycle tracking and traceability reports.

- Maintain records of product efficiency evaluations, product design improvements and environmental performance assessments in the applicable document management system.

- Generate periodic compliance reports for internal and external stakeholders.

- Product Sustainability Performance Review

- Review of Product Efficiency and Environmental Performance:

- Periodically review product-use information, storage/handling controls, product efficiency improvement initiatives and environmental design measures.

- Review available evidence relating to dosage/quantity/frequency optimization and reduction of toxic discharges or harmful environmental releases.

- Identify opportunities for further product improvement based on technical evaluations, customer requirements, regulatory developments and environmental performance.

- Documentation:

- Maintain records of product sustainability reviews, identified improvement opportunities and actions taken.

- Include significant product efficiency and environmental improvement initiatives in relevant sustainability performance reviews.

- Review and Continuous Improvement

- Conduct annual reviews of product sustainability strategies and safe handling protocols.

- Update SOPs based on audit findings, technological advancements, and regulatory changes.

- Engage external experts to enhance sustainable product development initiatives.

## PRODUCT DISPOSAL AND SUSTAINABLE PACKAGING GUIDELINES

- Purpose

To establish best practices for proper product disposal, optimized packaging, and environmentally responsible chemical design. This SOP ensures compliance with sustainability goals, promotes circular material flow, and minimizes environmental impact.

- Scope

This SOP applies to all products manufactured, packaged, and distributed by the company. It includes guidelines on end-of-life disposal, packaging optimization, sustainable chemical formulation, and consumer information to support responsible waste management.

- Responsibility

- Product Development Teams – Implementation of sustainable product disposal and packaging designs.

- Environmental & Legal compliance teams – Oversight of regulatory adherence and waste management protocols.

- Quality Assurance Teams – Verification of compliance with environmental and packaging standards.

- Supply Chain and Logistics Teams – Integration of sustainable packaging and return systems.

- Accountability

- Head legal Compliance.

- HOD (Operations).

- Senior Management for strategic planning and resource allocation

- Procedures

- Responsible Product Disposal and Circular Material Flow

- End-of-Life Disposal Guidelines:

- Provide clear instructions on how consumers can return, recycle, or safely dispose of products after use.

- Establish collection programs to facilitate proper disposal and recycling of unused products.

- Collaborate with recycling and waste management facilities to ensure product components are repurposed efficiently.

- Reduce landfill waste by promoting reuse of materials in secondary manufacturing.

- Provide applicable product users/customers with information on appropriate return, recovery, recycling or disposal routes at the end of the product's useful life.

- Where applicable, communicate available collection, recycling or authorized waste-management arrangements relevant to the product.

- Disposal information shall aim to minimize landfill disposal, uncontrolled release of product residues and unnecessary consumption of virgin materials.

- For chemical products, disposal information shall be consistent with the applicable SDS, product label and regulatory requirements

- Documentation:

- Maintain records of product end-of-life information communicated to customers/users, where applicable.

- Maintain supporting records of applicable return, recycling, recovery or authorized disposal arrangements.

- Optimized Packaging Design for Sustainability

- Primary Packaging Optimization:

- Reduce the dimensions and weight of packaging drums to minimize material usage.

- Prioritize optimization of primary packaging that is in direct contact with the product while maintaining product quality, safety and integrity.

- Evaluate opportunities for lightweighting primary packaging by reducing unnecessary packaging material, dimensions or weight without compromising product containment or safe handling.

- Where technically feasible, evaluate simplified or monomaterial primary-packaging designs to facilitate recycling and end-of-life management.

- Packaging optimization shall consider product compatibility, leakage prevention, transport/storage requirements, user safety and applicable regulatory requirements.

- Secondary and tertiary packaging shall not be considered as primary-packaging optimization unless it is directly relevant to the product-contact packaging design.

- Packaging Material Selection and Circularity

- Evaluate recyclable, reusable or otherwise environmentally preferable primary-packaging materials where technically feasible and suitable for the product.

- Consider opportunities to reduce virgin material requirements through reusable or recyclable primary-packaging solutions while maintaining product safety and quality.

- Packaging design changes shall be evaluated for compatibility with the chemical/product characteristics and intended use.

5.2.2 Documentation:

- Maintain packaging design specifications and material reduction data.

- Maintain records of primary-packaging material, dimensions and weight, where applicable.

- Maintain records of packaging optimization studies, lightweighting initiatives, material simplification and recyclability assessments..

- Green Chemistry and Residue-Free Formulation

- Eco-Friendly Chemical Product Design:

- Evaluate chemical product formulations and design characteristics to reduce or eliminate the potential for harmful residues to leach into soil, water or other environmental receptors following intended use.

- Where technically feasible, prioritize formulation or product-design options that minimize hazardous residues and environmentally harmful substances.

- Evaluate the environmental fate and potential release of relevant product constituents during product development or significant product modification, where applicable.

- Consider the use of less hazardous or environmentally preferable ingredients where technically feasible and consistent with product performance and regulatory requirements.

- Product design improvements shall not compromise intended product performance, quality, safety or applicable regulatory requirements.

- Hazardous Residue Reduction and Substitution

- Identify opportunities to eliminate, reduce or substitute substances that may contribute to harmful environmental residues after intended product use.

- Where a formulation change is implemented to reduce potential leaching or harmful residues, document the technical basis and environmental benefit of the change.

- Use applicable technical studies, product comparison assessments, SDS information, environmental assessments or other relevant documented evidence to support significant formulation improvements.

- Documentation:

- Maintain records of chemical safety improvements and residue-free formulations.

- Include product safety compliance reports in sustainability audits.

- Maintain records of formulation changes, substance substitutions and assessments undertaken to reduce harmful residue or leaching potential.

- Maintain comparative or technical evidence where a product modification demonstrates reduced potential for environmental release.

- Product Labeling and Environmental Information

- Labeling Requirements:

- Include hazard statements and precautionary statements for safe handling and disposal.

- Provide supplemental information, such as recommended recycling options, return programs, and environmental benefits.

- Clearly indicate storage conditions, temperature requirements, and expiration details to ensure product integrity.

- Where applicable, provide end-of-life disposal, return, recycling or recovery information through product labels, manuals, SDS, technical documents or digital resources.

- Ensure that disposal information is consistent with the applicable product characteristics, SDS and regulatory requirements

- Documentation:

- Maintain product labels and regulatory approval records.

- Ensure compliance with national and international labeling standards.

- Digital Tracking and Consumer Awareness

- Digital Solutions:

- Develop an online database for product lifecycle information, including return locations and recycling facilities.

- Documentation:

- Maintain records of digital tracking initiatives and consumer engagement metrics.

- Generate periodic reports on user participation in sustainability programs.

- Review and Continuous Improvement

- Conduct annual reviews of product disposal strategies, packaging optimization, and compliance with environmental regulations.

- Update SOPs based on audit findings, technological advancements, and sustainability targets.

- Engage external experts to enhance waste reduction and sustainable packaging initiatives.

- Monitor customer feedback on disposal guidelines and packaging efficiency for continuous improvement.

## EMERGENCY PREPAREDNESS AND RESPONSE PROCEDURE

1.0 Purpose

To establish a structured emergency preparedness and response protocol, ensuring rapid action in case of product-related incidents such as contamination, safety risks, or regulatory non-compliance. This SOP includes a formalized recall and call-back process to proactively interact with customers regarding potential health and safety risks from product use.

2.0 Scope

This SOP applies to all manufactured and outsourced products of Transpek Industry Ltd., covering emergencies related to product quality, compliance failures, adverse customer feedback, and regulatory actions.

3.0 Responsibility

- HOD – Marketing – Initiation of product recall and customer communication.

- HOD – Operations – Oversight of recall logistics and corrective actions.

- HOD – QC and QA – Evaluation of defects, risk assessment, and quality assurance.

- Warehouse and Production Teams – Handling, inspection, and corrective measures for recalled materials.

- Legal compliance teams – Ensuring adherence to safety and environmental regulations.

4.0 Accountability

- HOD – Marketing and Senior Management for strategic decisions and customer interactions.

- HOD – Operations for execution and corrective actions related to recalled materials

5.0 Description

5.1 Emergency Identification and Assessment

5.1.1 Situations Triggering Emergency Response:

- Customer complaints regarding product quality or safety issues.

- Failure to meet compliance or technical parameters after dispatch.

- Findings from stability testing or internal quality assessments.

- Requests by regulatory authorities for product withdrawal.

- Incidents of counterfeiting, tampering, or adverse reactions.

- Leakage, spillage, or contamination during transportation or at customer premises.

- Strategic commercial decisions impacting product recall.

5.1.2 Documentation:

- Maintain incident reports, stability test records, and regulatory communications.

- Ensure real-time monitoring of quality deviations and risk factors.

5.2 Customer Call-Back and Recall Process

5.2.1 Initiation of Recall:

- HOD – Marketing, along with Operations, QC, and QA, will review the recall necessity.

- Immediate notification to affected customers and regulatory bodies.

- Formal recall communication issued to all stakeholders.

5.2.2 Logistics and Handling:

- Recalled consignments shall be received as per standard vehicle entry procedures.

- Warehouse and marketing teams will verify weight, confirm documentation, and inspect returned materials.

- Segregated storage and proper labelling of recalled materials.

5.2.3 Corrective Actions:

- QC and Production teams will inspect the product and determine the best course of action:

  - Repacking, regrading, or dispatch under deviation.

  - Reprocessing, redistillation, or reworking to meet quality standards.

  - Final quality assurance before re-release or disposal.

  - Reference to IMS manual (TIL-EKB-MR-IMS, Chapter 41 – Control of Non-Conforming Output).

5.2.4 Customer Communication:

- Formal communication letter sent to customers regarding recall status.

- Detailed corrective actions and replacement procedures provided.

- Follow-up verification to ensure customer safety and satisfaction.

5.3 Emergency Response for Health, Safety, and Environmental Risks

5.3.1 Response Protocols:

- Immediate containment and mitigation measures in case of contamination or spills.

- Use of protective equipment (PPE) and spill control measures by trained personnel.

- Engagement of emergency response teams for risk containment.

- Notification of local regulatory authorities as per compliance standards.

5.3.2 Documentation:

- Maintain records of emergency response actions, environmental impact assessments, and compliance reports.

- Conduct post-incident reviews to improve preparedness.

5.4 Product Labelling and Safety Information

5.4.1 Labelling Requirements:

- Include hazard statements and precautionary statements for product safety.

- Provide supplemental information on storage conditions, expiry dates, and emergency contacts.

- Indicate proper disposal methods for recalled or defective products.

5.4.2 Documentation:

- Maintain updated product labels in compliance with industry and regulatory standards.

- Store historical data for recall tracking and regulatory audits.

6.0 Emergency Drills and Training

- Conduct annual emergency response drills to test recall effectiveness and crisis management.

- Provide training to employees on safety protocols and recall handling procedures.

- Simulate contamination or product failure scenarios to enhance response preparedness.

- Regularly update emergency contact lists and regulatory response plans.

7.0 Review and Continuous Improvement

- Conduct periodic reviews of emergency response procedures to ensure efficiency.

- Update SOPs based on lessons learned from recall incidents and regulatory updates.

- Engage with external consultants to refine emergency response measures.

- Monitor industry best practices for product recalls and crisis management.

## CUSTOMER FEEDBACK

1.0 Purpose

The purpose of this SOP is to ensure that all information relevant to the quality of products or services, with respect to customer experiences and expectations, is collected, analyzed, interpreted, and communicated for appropriate management action regularly.

2.0 Scope

This SOP applies to all customers of our products.

3.0 Responsibility

All concerned salespersons are responsible for collecting customer feedback.

4.0 Accountability

The Head of Marketing (HOD-Marketing) is accountable for the overall customer feedback process.

5.0 Description

5.1 The feedback form from all Customer’s is collected in Customer Feedback Form, once in a year.

5.2 Customer Feedback forms are sent to customers by email, fax, or during customer visits.

5.3 The Feedback information is compiled and analysed using relevant criteria, which would help to created better understanding of the  customer’s requirements.

5.4 Customer feedback may also be obtained over the telephone, and the completed form is then sent to the customer's email for acknowledgment or comments if any.

5.5 The results of the analysis are interpreted and reviewed by concerned salespersons, led by the HOD-Marketing. An action plan is then drawn for various related functions.

5.6  Management approval and resources are obtained and the action plan is implemented.

6.0 Records

Customer feedback is recorded in the Customer's Feedback - TIL-EKB-MKT-FF-13 document.

## COMPLIANCE, ETHICS, AND RISK ASSESSMENT GUIDELINES

- Purpose

To establish a structured approach for conducting audits, risk assessments, and compliance reviews to prevent corruption, unethical practices, and other business risks. This SOP ensures adherence to legal, financial, ethical, and social responsibility standards while protecting corporate integrity.

- Scope

This SOP applies to all internal operational, financial, HR, and regulatory compliance activities at Transpek Industry Ltd. It includes guidelines for internal audits, external reviews, anti-corruption measures, social issue assessments, and ethical risk mitigation.

- Responsibility

- Audit & Compliance Teams – Conduct audits, internal control reviews, and risk assessments.

- Finance & Accounting Teams – Ensure financial transparency, prevent fraud, and manage internal controls.

- Human Resources (HR) Teams – Promote ethical hiring, workplace conduct, and social responsibility.

- Operations & Legal Teams – Ensure adherence to regulatory policies and ethical governance.

- Accountability

- HOD (Finance, HR, Operations, and Legal)

- Description

- Internal Audits and Control Procedures

- Audit Scope and Execution:

- Conduct periodic internal audits and control assessments covering relevant financial, ethical, anti-corruption, information security, operational, and compliance controls based on identified risks and the approved audit scope.

- Define the scope, assessment criteria, applicable departments/functions, and period covered for each audit or control assessment.

- Regular internal audits covering financial transactions, HR policies, workplace ethics, and operational compliance.

- Include anti-corruption and information security controls in internal assessments, as applicable.

- Accountability and Audit Responsibility:

- Assign the responsible person or function for planning, conducting, reviewing, and following up each audit or control assessment.

- The accountable function shall ensure that audit findings are documented, corrective actions are assigned, and follow-up is completed.

- Audit & Compliance Teams – Conduct audits, internal control reviews, and risk assessments.

- Relevant functional heads shall provide necessary information, support the assessment, and implement assigned corrective actions.

- Retrospective Review and Effectiveness Evaluation:

- Review previous audit reports, assessment results, identified non-conformities, and corrective actions to determine whether previously identified control weaknesses have been addressed.

- Evaluate the effectiveness of existing ethical, anti-corruption, and information security controls through review of previous findings, current assessment results, and implementation of corrective actions.

- Retrospective analysis of past audit reports to assess system improvements.

- Corrective Actions and Follow-up:

- Document audit and assessment findings and identify appropriate corrective actions for identified gaps or control weaknesses.

- Assign responsible persons and target completion dates for corrective actions, where applicable.

- Verify the implementation and effectiveness of corrective actions through follow-up review or subsequent assessment.

- Track corrective actions until completion.

- Documentation:

- Maintain audit reports, corrective action records, and compliance tracking logs.

- Maintain defined audit scope, assessment checklists, supporting evidence, findings, responsible persons, corrective actions, follow-up verification, and management review records.

- Include findings and improvements in annual compliance reports.

- Governance and Ethical Risk Management

- Governance Policies and Internal Controls:

- Ensure transparency and accountability in financial transactions, operational decision-making, and policy enforcement.

- Conduct internal audits and governance reviews to prevent financial misconduct, conflicts of interest, and unethical practices.

- Assess corruption and ethical risks associated with relevant business activities and third-party interactions.

- Implement governance training programs to educate employees on corporate integrity, ethical decision-making, and compliance obligations.

- Governance Compliance Documentation:

- Maintain governance audit reports, financial compliance records, and risk management assessments.

- Ensure adherence to corporate governance principles, ethical business conduct policies, and regulatory requirements.

- Maintain documented evidence of identified governance risks, assessments performed, corrective actions, and follow-up activities.

- Social Issue Risk Assessment

- Workplace Ethics and Labor Rights:

- Assess compliance with labor laws, workplace diversity, and fair treatment policies.

- Monitor working conditions, wages, and employee welfare.

- Identify and mitigate risks related to discrimination, harassment, and forced labor.

- Community and Environmental Impact:

- Evaluate company policies on environmental sustainability and social responsibility.

- Monitor community grievances related to corporate operations.

- Engage with local stakeholders to address social and environmental concerns.

- Documentation:

- Maintain records of workplace audits, social risk assessments, and stakeholder engagement reports.

- Include social responsibility performance in sustainability reports.

- Third-Party Ethical Due Diligence and Compliance Assessment

- Third-Party Risk Screening and Due Diligence:

- Identify third parties that may create ethical, corruption, compliance, or information security risks, including suppliers, vendors, consultants, agents, brokers, distributors, contractors, customs brokers, sales representatives, and other business intermediaries, as applicable.

- Conduct appropriate due diligence before onboarding or engaging relevant third parties, based on the nature of the business relationship and associated risks.

- Review available information relating to the third party’s business activities, ownership or management, ethical practices, regulatory compliance, and potential conflicts of interest, as applicable.

- Obtain and review relevant declarations, questionnaires, compliance commitments, or supporting documents where required.

- Document the due diligence performed and the outcome of the assessment.

- Supplier and Business Partner Assessment:

- Use Supplier Self-Assessments, Supplier On-Site ESG Audits, Internal Assessments, and other applicable assessment mechanisms to evaluate third-party compliance and ethical risks.

- Assess relevant third parties against applicable anti-corruption, ethical conduct, information security, and compliance requirements.

- Review identified gaps, non-conformities, observations, and risks arising from third-party assessments.

- Record appropriate corrective actions and follow-up requirements for identified gaps.

- Consider assessment results when reviewing the continuation or improvement of the business relationship, as applicable.

- Enhanced Review for Higher-Risk Third Parties:

- Where a third party is identified as having higher ethical, corruption, compliance, or information security risk, additional review or supporting information may be requested based on the nature and risk of the relationship.

- The extent of additional due diligence shall be determined based on the identified risk and business relationship.

- Document the additional review and its outcome where performed.

- Review, Approval and Follow-up:

- Review the results of third-party due diligence and assessments before approval or continuation of the relevant business relationship, as applicable.

- Communicate identified requirements or corrective actions to the concerned third party or responsible internal function.

- Track corrective actions arising from supplier or third-party assessments until closure or appropriate resolution.

- Conduct follow-up assessment or verification where required based on the identified risk or previous findings.

- Third-Party Due Diligence Records:

- Maintain records of third-party due diligence, Supplier Self-Assessments, Supplier On-Site ESG Audits, internal assessment results, supporting documents, identified risks, corrective actions, and follow-up verification.

- Maintain records in accordance with the Company's applicable document and records management requirements.

- Incident Reporting and Investigation

- Establish confidential whistleblower mechanisms for reporting unethical activities.

- Ensure all reports are reviewed by an independent ethics committee.

- Investigate allegations thoroughly and implement corrective actions.

- Protect employees from retaliation when reporting unethical behaviour.

- Review and Continuous Improvement

- Conduct annual reviews of compliance programs and audit procedures.

- Update SOPs based on regulatory changes and audit findings.

- Monitor new trends in corporate governance, and social responsibility.

## DATA PRIVACY AND RECORDS MANAGEMENT GUIDELINES

- Purpose

To establish best practices for data retention, protection, and stakeholder consent management to ensure compliance with privacy regulations, confidentiality standards, and corporate governance policies.

- Scope

This SOP applies to all third-party, customer, and internal confidential data collected, stored, processed, and retained by the company. It includes data handling procedures, access control measures, and retention policies to prevent unauthorized disclosure and ensure compliance.

- Responsibility

- Audit & Compliance Teams – Conduct audits, internal control reviews, and risk assessments.

- Finance & Accounting Teams – Ensure financial transparency, prevent fraud, and manage internal controls.

- Human Resources (HR) Teams – Promote ethical hiring, workplace conduct, and social responsibility.

- Operations & Legal Teams – Ensure adherence to regulatory policies and ethical governance.

- Accountability

- HOD (Legal, IT, and Operations)

- Description

- Data Protection and Access Control

- Security Measures for Data Protection:

- Implement access control mechanisms to restrict unauthorized physical or digital access to third-party or customer data.

- Use encryption and multi-factor authentication (MFA) for sensitive data protection.

- Monitor and log all access attempts to detect and prevent unauthorized data breaches.

- Establish data segregation protocols to ensure minimal data exposure between departments.

- Documentation:

- Maintain access logs and security compliance reports.

- Conduct regular penetration testing and security audits.

- Maintain records of identified information-security risks, control measures, assessment findings and corrective actions, as applicable.

- Stakeholder Consent and Data Processing Transparency

- Information Provided to Stakeholders:

- Provide clear and transparent information to customers, clients, employees and other relevant stakeholders regarding the collection, processing, sharing and retention of their personal or confidential information.

- Communicate the relevant purpose and nature of information collected before or at the time of collection, as applicable.

- Inform stakeholders, where applicable, about the categories or types of information collected and the intended use of such information.

- Inform stakeholders where personal or confidential information may be shared with authorized third parties or service providers, in accordance with applicable requirements.

- Provide information regarding applicable data retention requirements or periods, where required.

- Obtaining Consent for Data Processing:

- Ensure clear and transparent communication with stakeholders regarding the collection, processing, and retention of their data.

- Provide stakeholders with an explicit opt-in mechanism before collecting personal or confidential data.

- Disclose data-sharing policies to inform stakeholders about third-party data processing agreements.

- Obtain and document stakeholder consent where consent is the applicable legal or operational basis for collecting, processing or sharing personal information.

- Allow stakeholders to request modifications or deletions of their stored data in compliance with data privacy laws.

- Consent and Communication Records:

- Maintain records of consent agreements and privacy policy updates.

- Store communication logs related to data privacy disclosures.

- Maintain records demonstrating the information provided to stakeholders regarding data collection, processing, sharing and retention, where applicable.

- Maintain records of requests received from stakeholders and actions taken in response, where applicable.

- Records Retention and Management

- Implementation of Retention Schedules:

- Establish a structured records retention schedule specifying the duration for storing third-party, customer, and internal data.

- Classify records based on type (e.g., financial, legal, operational, personal data) and define retention timelines for each category.

- Automate record retention policies through digital management systems to ensure compliance.

- Data Retention Schedule:

| Record Type | Retention Period | Storage Method | Disposal Method |
| --- | --- | --- | --- |
| Financial Records | 7 years | Encrypted Digital Archive | Secure Shredding |
| Customer Data | 5 years (or per contract terms) | Secured Database | Permanent Deletion |
| Employee Records | 10 years after termination | HR Management System | Confidential Disposal |
| Legal Contracts | Lifetime + 5 years | Legal Document Storage | Secure Archival Destruction |
| Compliance Reports | 6 years | Compliance Repository | Secure Shredding |
| Email Communications | 2 years | Email Server Backup | Automated Digital Purging |
| Security Logs | 1 year | Cybersecurity Log System | Automatic Log Rotation |

- Documentation:

- Maintain records retention logs and periodic compliance audits.

- Ensure retention policies align with legal and regulatory requirements.

- Information Security Incident Response and Breach Management

- Breach Detection and Reporting:

- Establish a defined process for identifying and reporting suspected information-security breaches, unauthorized disclosures, data leaks, phishing attempts, malware incidents and other cybersecurity events.

- Employees and relevant personnel shall promptly report suspected information-security incidents through the designated reporting channel.

- The Information Security Officer or designated responsible person shall record and assess reported incidents to determine the nature and potential impact of the breach.

- Incident Response and Containment:

- Develop an Emergency Response Plan (ERP) for data breaches, unauthorized disclosures, and cyber threats.

- Define roles and responsibilities for IT security, legal, and compliance teams in the event of a data breach.

- Investigate the reported breach to determine its cause, affected information, affected systems and potential impact.

- Take appropriate containment measures to limit unauthorized access, disclosure, alteration, loss or further spread of confidential information.

- Implement recovery measures appropriate to the nature and severity of the incident.

- Communication and Stakeholder Protection:

- Notify affected stakeholders and regulatory authorities in accordance with applicable data protection laws.

- Where a breach affects customers, employees, suppliers or other stakeholders, provide appropriate communication regarding the incident and relevant protective or corrective measures, where required.

- Restrict further disclosure of confidential information and implement appropriate measures to protect affected information.

- Investigation and Corrective Action:

- Conduct root cause analysis and corrective actions to prevent future incidents.

- Document the incident, investigation findings, containment measures, corrective actions and closure status.

- Review existing information-security controls following significant incidents and strengthen controls where gaps or weaknesses are identified.

- Track corrective actions until completion and verify effectiveness where appropriate.

- Information Security Incident Records

- Documentation Requirements:

- Maintain information-security incident reports, investigation records, breach assessment records, communication records, corrective-action records and closure evidence, as applicable.

- Maintain records of information-security assessments and reviews used to evaluate the effectiveness of security controls.

- Ensure information-security records are protected against unauthorized access, alteration or disclosure.

- Review and Continuous Improvement

- Conduct annual reviews of data retention schedules, security controls, and consent management policies.

- Review information-security incident trends, stakeholder consent practices, data-processing arrangements and identified control weaknesses to identify opportunities for improvement.

- Update SOPs based on regulatory changes and cybersecurity trends.

- Monitor evolving global data protection laws to ensure proactive compliance.

- Implement improvements arising from incident investigations, security assessments, audits and corrective actions.

## SUSTAINABLE PROCUREMENT AND ETHICAL SOURCING GUIDELINES

- Purpose

To establish a structured approach for integrating sustainability objectives into procurement processes and buyer performance evaluations. This SOP ensures that ethical sourcing, environmental responsibility, and supplier sustainability are key factors in procurement decision-making.

- Scope

This SOP applies to all procurement activities, supplier assessments, and buyer performance evaluations at Transpek Industry Ltd. It includes guidelines for sustainable procurement strategies, ethical sourcing, and the integration of sustainability metrics into buyer performance reviews.

- Responsibility

- Procurement Teams – Implement sustainable procurement strategies and supplier assessments.

- Finance & Compliance Teams – Monitor financial integrity and ethical compliance in procurement.

- Buyer Performance Reviewer – Evaluate procurement personnel based on sustainability criteria.

- Environmental & Sustainability Teams – Ensure supplier adherence to environmental and ethical standards.

- Accountability

- HOD Procurement.

- HOD Finance.

- HOD legal compliance.

- Description

- Sustainable Procurement Strategies

- Key Sustainability Criteria:

- Preference for suppliers with strong environmental, social, and governance (ESG) policies.

- Adoption of sustainable raw materials and packaging by vendors.

- Evaluation of suppliers based on carbon footprint, water conservation, and waste management.

- Encouraging procurement from local and ethically certified sources.

- Documentation:

- Maintain supplier sustainability assessment reports.

- Include sustainability metrics in supplier contracts and agreements.

- Buyer Performance Evaluation Linked to Sustainability

- Integration of Sustainability in Buyer Performance:

- Assess buyers based on their ability to source from sustainable suppliers.

- Include supplier sustainability performance as a defined criterion in the performance evaluation of buyers responsible for supplier selection, management, and interaction.

- Evaluate buyers, as applicable, on their implementation of supplier sustainability assessment, monitoring, corrective-action follow-up, and engagement activities

- Include sustainability KPIs in buyer performance evaluations, such as:

  - Percentage of applicable suppliers assessed against defined sustainability criteria.

  - Percentage of applicable suppliers meeting required sustainability/compliance criteria.

  - Completion of supplier sustainability assessments and follow-up actions within defined timelines.

  - Reduction in procurement-related environmental impact.

  - Compliance with fair labor and ethical trade standards.

  - Buyers must demonstrate efforts in promoting sustainable supply chain initiatives.

- The sustainability-related performance of buyers shall be reviewed as part of the applicable buyer performance appraisal process

- Documentation:

- Maintain buyer performance evaluation forms incorporating sustainability metrics.

- Record corrective actions for non-compliance with sustainable procurement standards.

- Maintain evidence of supplier sustainability assessments, supplier audit results, corrective-action follow-up, and buyer performance evaluation records used to assess sustainability-related objectives

- Ethical Sourcing and Supplier Compliance

- Supplier Code of Conduct:

- Mandate adherence to ethical labor practices and fair-trade policies.

- Require transparency in supply chain practices to prevent human rights violations.

- Ensure compliance with hazardous substance regulations and responsible material sourcing.

- Documentation:

- Maintain supplier agreements outlining ethical sourcing expectations.

- Conduct periodic supplier audits and maintain compliance records.

- Digital Monitoring and Reporting

- Technology Integration:

- Implement digital tracking systems for monitoring supplier sustainability performance.

- Utilize procurement dashboards to analyze real-time sustainability metrics.

- Documentation:

- Maintain digital records of procurement decisions based on sustainability evaluations.

- Generate periodic sustainability reports for internal and external stakeholders.

- Review and Continuous Improvement

- Conduct annual reviews of sustainable procurement strategies and buyer performance evaluations.

- Update SOPs based on audit findings, regulatory changes, and emerging sustainability trends.

- Engage with external sustainability experts to enhance ethical sourcing initiatives.

- Monitor buyer compliance with sustainability KPIs and implement continuous training programs.

Approved By :    
last Review Date : 01/04/2026