Table of Contents

Introduction	3

Scope	4

Environmental Management Procedures	5

1. Waste Management Procedures	5

2. Material Management Procedures	19

3. Hazardous Substances Management and Reduction	24

4. Efficient Energy Management	27

5. Water Optimization Procedures	38

6. Air pollution management	47

7. Sustainable Packaging management and Optimization	52

Social Procedures	56

8. Provision of Emergency Equipment	56

9. Equipment Safety Inspection	59

10. Workplace Safety Practices	62

11. Employee Evaluation Practices	67

12. Supplier Selection, Engagement and Management	69

Governance Procedures	73

13. Procedure for Approving of Sensitive Transactions Compliance	73

14. Procedure for Confidential Data Handling and Breach Responses	76

15. Securing Third-Party Data from Unauthorized Access	79

16. Stakeholder Consent and Data Processing Retention and Disposal	82

17. Procedure for Employee Compliance and Awareness	86

18. Anti-Corruption Due Diligence Program on Third Parties	88

19. Information Security Due Diligence Program on Third Parties	91

20. Audits of Control Procedure to ensure Ethical Compliance	95

# Introduction

"Structured Systems for Seamless Performance — at Ballyfabs International Limited, our SOP Manual anchors our commitment to quality, sustainability and innovation."

At Ballyfabs, we believe that well-defined processes are the foundation of reliability and excellence. This Standard Operating Procedures (SOP) Manual is not just a set of guidelines—it is a framework that ensures consistency, accountability, and performance across all aspects of our operations.

As one of India’s leading exporters of jute goods, we are dedicated to ethical production, sustainable practices, and uncompromising quality. From raw jute procurement to finished product delivery, this manual supports our teams in maintaining efficiency, safety, and compliance with both national and international standards.

Each section of the manual has been designed to promote clarity, minimize ambiguity, and align our work with the expectations of global partners, buyers, and regulatory authorities. It empowers our workforce to uphold the highest standards while reflecting the values that define Ballyfabs: responsibility, craftsmanship, and progress.

This SOP Manual is a living document, evolving alongside our growth, integrating continuous feedback, and reinforcing our mission to produce world-class jute products that respect people, the planet, and our legacy of sustainable innovation.

# Scope

The ESG SOP Manual applies to all operations of Ballyfabs International Limited, covering employees, contractors, vendors, and key stakeholders. It establishes a clear framework for integrating Environmental, Social, and Governance (ESG) principles into every aspect of our business.

These manual outlines procedures to minimize environmental impact, promote employee well-being, uphold ethical practices, and strengthen transparent stakeholder engagement—demonstrating Ballyfabs commitment to sustainable growth and alignment with global standards.

| Site | Address |
| --- | --- |
| Ballyfabs International Limited | - 3, Haren Mukherjee Road, Belur, Howrah-711202, West Bengal, India. |
|  | - Vill-Gangarampur, PO-Dakshin Alipore, PS-Bishnupur, Dist-South 24 Parganas, Pin-743503, West Bengal, India |

# Environmental Management Procedures

## 1. Waste Management Procedures

Overview

Effective waste management is a key component of Bally Fabs International Limited’s commitment to environmental sustainability, workplace safety, and full regulatory compliance. The company has implemented structured and systematic waste management practices across all stages of its jute manufacturing operations, including cutting, stitching, printing, finishing, and maintenance activities.

The waste management process at Bally Fabs focuses on the following core procedures:

- Waste Stream Mapping– Identification and recording of all waste types generated across operations.

- Waste Segregation at Source – Separation of biodegradable, recyclable, and hazardous waste at the point of generation.

- Reduction of Total Waste Generation – Adoption of process efficiencies and material optimization to minimize waste.

- Safe Storage, Handling, and Transportation of Waste – Ensuring secure containment and movement of waste materials to prevent leaks, spills, or cross-contamination.

- Environmentally Sound Disposal Methods – Disposal of waste through authorized recyclers and government-approved facilities, in line with environmental regulations.

Purpose

This SOP establishes a comprehensive and standardized waste management procedure at Ballyfabs International Limited. It aims to ensure proper segregation, classification, storage, handling, transportation, and disposal of all waste streams, while emphasizing environmental sustainability, worker safety, and compliance with applicable Indian regulations, including guidelines issued by the Pollution Control Board and other relevant authorities.

Scope

This SOP applies to all jute manufacturing units and support functions at Ballyfabs International Limited, covering the entire waste lifecycle, from generation and segregation to final disposal or recycling. It includes provisions for production waste, maintenance waste, administrative waste, and special waste, including hazardous materials, in accordance with applicable Indian regulations and industry best practices.

Definitions

- Hazardous Waste: Waste that poses a significant risk to human health or the environment (e.g., chemicals, dyes, oil residues).

- Non-Hazardous Waste: General waste that does not pose immediate risks (e.g., fabric scraps, packaging materials).

- Recyclable Waste: Waste that can be reprocessed and reused (e.g., paper, plastic, metals).

- Organic Waste: Biodegradable waste like food waste and plant matter that can be composted.

Procedures

- Waste Stream Mapping

At Bally Fabs International Limited, all waste is segregated at source and tracked through a structured Waste Stream Mapping process. This enables accurate identification, classification, storage, handling, and disposal in compliance with:

- Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016

- Solid Waste Management Rules, 2016

- E-Waste (Management) Rules, 2022

A centralized Waste Stream Register is maintained and overseen by the EHS Officer in coordination with department heads. The register captures:

- Source of waste generation

- Type and classification (hazardous, recyclable, organic, general, etc.)

- Estimated and actual volumes generated

- Storage locations and disposal details

- Authorized vendor information and manifests

All records, including manifests, disposal certificates, and volume logs, are archived for minimum five years for audits, sustainability reporting, and regulatory inspections.

Waste Stream Mapping:

| Waste Category | Description | Origin (Department/Process) | Waste Included | Bin Color-Coding | Storage / Handling Method | Disposal Method | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Textile Fibre Waste | Natural fibre waste generated during manufacturing processes | Cutting, Stitching, Weaving Units | Jute off-cuts, fabric trimmings, yarn waste, weaving scraps | Blue | Stored in segregated fibre-waste bins; collected shift-wise | Recycled or sent to authorized fibre recovery facility | Production Manager |
| Recyclable Paper Waste | Clean and dry paper-based waste | Packaging Area, Offices | Cardboard boxes, kraft paper, paper sheets, cartons | White | Stored in dry paper-waste bins; kept away from moisture | Sent to paper recyclers | Warehouse Manager |
| Recyclable Plastic Waste | Plastic packaging material suitable for recycling | Raw Material Receiving, Packaging Area | Plastic films, wrappers, polybags | Yellow | Collected in dedicated plastic-waste bins; compressed if required | Sent to authorized plastic recyclers | Packaging Supervisor |
| Dry General Waste | Non-recyclable, non-hazardous dry waste | Office Areas, Production Floor | Used tapes, labels, mixed dry waste | Black | Collected daily in general waste bins; kept separate from recyclables | Sent to municipal waste collection or disposal | Admin Supervisor |
| Wet / Organic Waste | Biodegradable food waste and organic matter | Canteen, Pantry | Food scraps, tea waste, canteen leftovers | Green | Stored in leakproof bins; collected daily by housekeeping | Sent for composting or municipal organic waste collection | Canteen Supervisor |
| Hazardous Waste | Waste containing chemical residues requiring controlled handling | Chemical Storage, Maintenance (if applicable) | Empty chemical containers, adhesive containers | Red | Stored in sealed, labeled containers in a separate hazardous area | Handed to authorized hazardous waste handlers | EHS Officer |
| General Sweep Waste | Dust and floor residues collected during cleaning | Production Floors, Warehouse | Dust, lint, floor sweepings | Grey | Stored in closed bins; collected during routine cleaning cycles | Sent to municipal waste or waste-to-energy facility | Housekeeping Supervisor |
| Reject / Defective Product Waste | Damaged or unusable goods not suitable for sale or rework | QC Department, Warehouse | Defective bags, damaged stitched pieces | Orange | Stored in a marked “Rejects” zone; sorted for potential rework | Recycled where possible; remainder to authorized waste handlers | QC In-charge |

- Waste Segregation

- Procedure

- Segregation at Source: All waste must be placed immediately into the correct color-coded bins at the point of generation. No waste shall be mixed at any stage of handling, storage, or transport.

- Collection and Internal Transfer: Departmental staff or housekeeping personnel shall collect waste at scheduled intervals to avoid overflow, ensure cleanliness, and prevent cross-contamination.

- Temporary Storage: Each waste category shall be transferred to its designated interim storage area. Hazardous waste must remain in sealed and clearly labelled containers as per regulatory requirements until collected by authorized vendors.

- Documentation & Register Updating: All waste quantities, categories, and disposal details shall be recorded in the centralized Waste Stream Register. Department heads must verify weekly entries for accuracy.

- Final Disposal: Waste shall be disposed of through authorized recyclers, municipal authorities, or hazardous-waste vendors as per the disposal method listed in the Waste Stream Mapping Table.

- Monitoring: Supervisors and the EHS Officer shall routinely inspect segregation practices and provide refresher training to departments where non-compliance is observed.

- Corrective Actions

- Immediate Response: Any instance of mis-segregation or cross-contamination must be corrected immediately. Waste shall be re-segregated and handled according to the defined procedure.

- Employee Retraining: Targeted training sessions are held for departments or shifts where errors are identified.

- Root Cause Analysis: The EHS Officer shall investigate repeated non-compliance to identify process gaps and implement preventive measures.

  - Safety and Environmental Considerations

- Personal Protective Equipment (PPE): All staff involved in waste handling, especially hazardous and chemical waste, are equipped with appropriate PPE including gloves, safety goggles, masks, aprons, and closed footwear.

- Spill Management: Trained personnel follow a documented Spill Response Protocol, using spill kits, absorbents, and neutralizing agents. All incidents are recorded in the spill log.

- Total Waste Generation reduction

Bally Fabs International Limited adopts a proactive, multi-tiered approach to minimize waste generation throughout its jute product manufacturing operations. Aligned with India’s environmental regulations and sustainability principles, the company emphasizes source reduction, material reuse, recovery, and process optimization over disposal, aiming to reduce its environmental footprint and enhance circularity.

  - Promotion of Material Reuse and Recovery

Identification of Recoverable Materials

A periodic waste stream audit is conducted to identify recyclable and recoverable materials generated during the product manufacturing process. These materials include, but are not limited to:

- Textile Fiber Waste: Jute off-cuts, fabric trimmings, yarn waste, and weaving scraps generated in cutting, stitching, and weaving units.

- Recyclable Paper Waste: Clean cardboard, paper sheets, kraft paper, and cartons from packaging areas and offices.

- Recyclable Plastic Waste: Plastic films, wrappers, and polybags from raw material receiving and packaging areas.

- Reject / Defective Product Waste: Damaged or unusable finished goods segregated in the QC department and warehouse.

Collection and Segregation

- Designated bins and collection points are strategically placed within production, dyeing, and packaging areas for recoverable materials.

- Training sessions and awareness programs are conducted to ensure all staff understand the value of reuse and their role in achieving waste reduction goals.

- All recoverable waste is logged in the centralized Waste Stream Register, with departmental responsibility assigned for handling, storage, and disposal.

Recovery and Reuse Processes

Recoverable materials are processed to maximize reuse and minimize disposal. The process ensures materials are repurposed internally or sent to certified recyclers, as applicable.

| Material Type | Recovery Percentage | Collection Method (Origin / Department) | Reused For |
| --- | --- | --- | --- |
| Textile Fibre Waste | 85% | Bins in Cutting, Stitching, and Weaving Units | Sample products, stuffing, reusable cloths |
| Recyclable Paper Waste | 90% | Sorting at Packaging Area, Offices | Reused in shipments or sent to certified paper recyclers |
| Recyclable Plastic Waste | 90% | Sorting at Raw Material Receiving & Packaging Areas | Compressed for internal reuse or sent to authorized plastic recyclers |
| Reject / Defective Product Waste | 75% | QC Department, Warehouse | Repurposed as trial pieces, downgraded items, secondary products |

  - Source Reduction through Process Optimization

In addition to reuse and recovery, Bally Fabs International Limited employs process improvements to reduce waste at the point of generation:

- Right-First-Time (RFT) Production: Enhanced quality controls in knitting, dyeing, and finishing reduce reworks, rejections, and material loss.

- Computer-Aided Pattern Making (CAD): Optimizes fabric consumption during cutting, minimizing off-cuts and layout waste.

- Lean Manufacturing Practices: Reduces overproduction, idle inventory, and unnecessary handling, thereby minimizing waste from inefficiencies.

- Chemical Dosing Systems: Automated and precise dosing reduces chemical overuse and associated waste in dyeing and washing.

- Batch Optimization: Proper planning ensures full utilization of machine capacity, avoiding partial loads that result in higher resource consumption and waste.

4. Waste Storage, Handling, and Transportation

4.1. Storage Procedures

- Hazardous Waste

- Containers for Hazardous Waste
Hazardous waste is stored in leak-proof, sealed, and chemically compatible containers to prevent leaks and contamination. These containers are chosen based on the specific chemical properties of the waste to ensure maximum safety. All containers remain securely closed except during waste loading activities.

- Labeling of Hazardous Waste Containers

To ensure safe storage, handling, and disposal of hazardous waste, Bally Fabs International Limited follows mandatory labelling procedures in line with the Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016, issued by the Central Pollution Control Board (CPCB) and enforced by the State Pollution Control Board (SPCB).

Labelling Specifications

- Material:
Labels are made of chemical-resistant and weatherproof material, suited for high-temperature and humid textile environments.

- Size:
Minimum dimensions of 4 x 6 inches, visible from at least 2 meters.

- Font & Contrast:

  - Bold text in English

  - Font size: minimum 12-point for key information

  - High contrast colors (e.g., black on white or red) for clear readability

- Adhesion:
Labels must be securely affixed to resist heat, moisture, and chemical exposure.

- Placement:
Affixed to the upper front portion of each container for easy visibility during inspection and handling.

- Label Must Include:

  - “Hazardous Waste” and relevant hazard symbol(s)

  - Waste category and physical state (liquid, sludge, etc.)

  - Date of generation

  - Generator’s name and contact details (Bally Fabs International Limited)

  - Emergency response information or hazard classification code

- Information Included on the Label
Each hazardous waste container includes the following:

- Waste Type

- Waste Code

- Date of Generation

- Quantity

- Signal Word: DANGER

- Hazard Statement: Flammable liquid and vapor. Toxic if inhaled.

- Handling Instructions: Keep tightly closed. Store away from heat and flame.

- Generator Name: Bally Fabs International Limited

- Contact Person: Environmental Manager

- Contact Details: +91-9073988363| hr.bil@thejuteshop.com

d. Inspection and Maintenance

- Labels are reviewed and annual internal waste audits are conducted to ensure clarity and regulatory compliance.

- Damaged or unreadable labels are immediately replaced.

- Any unlabeled containers are promptly flagged and relabeled by trained personnel.

e. Designated Storage Areas for Hazardous Waste

- Hazardous waste is stored in clearly marked, secure areas away from operational zones.

- Access is restricted to trained personnel only.

- The storage zone features impervious flooring, bunds, proper ventilation, and fire suppression systems.

- Regular inspections are conducted for leaks or container damage, with spill kits and emergency plans in place.

- Non-Hazardous Waste Storage

- Containers: Non-hazardous waste is stored in standard, non-leaking containers. These containers are not required to meet the same stringent specifications as hazardous waste containers but must still be durable enough to withstand normal handling conditions.

- Labeling: Non-hazardous waste is not required to have detailed hazard labels, containers are clearly labeled to indicate the waste type (e.g., packaging waste, office waste).

- Storage Areas: Non-hazardous waste is stored in designated collection areas that are separate from hazardous waste storage zones. These areas should be kept clean and organized to avoid cross-contamination with hazardous or recyclable waste.

- Spill Containment: Spill containment materials are available near storage areas to address any accidental spills promptly.

- Recyclable Waste Storage

- Containers: Recyclable waste (e.g., plastics, paper, metals) are stored in separate bins marked clearly with the type of material.

- Sorting: Recycling bins are sorted according to the type of material (e.g., separate bins for plastics, paper, metal, and glass) to facilitate easy and efficient recycling.

- Storage Area: Recyclables are stored in areas that is dry, clean, and away from potential contamination sources. The area is well-organized to facilitate easy access and transportation to recycling facilities.

- Organic Waste Storage

- Containers: Organic waste, such as food scraps or biodegradable materials, are stored in compostable or biodegradable bags or bins.

- Storage Conditions: Organic waste are kept in a cool, ventilated area to prevent odor buildup and promote efficient decomposition.

- Separation: Organic waste is kept separate from non-hazardous and recyclable waste to avoid contamination.

4.2. Handling Procedures

- Hazardous Waste Handling

- PPE: Mandatory use of gloves, goggles, masks/respirators, and protective gear based on hazard level.

- Training: Staff receive regular training on safe handling, segregation, and emergency protocols.

- Spill Control: Secondary containment is used. Any spill is managed as per BallyFabs International Limited spill response plan.

- Non-Hazardous Waste Handling

- Personal Protective Equipment (PPE): While non-hazardous waste handling does not typically require specialized PPE, workers should wear gloves and other general protective equipment to prevent minor injuries.

- Handling Instructions: Non-hazardous waste should be handled with care to avoid contamination with hazardous or recyclable materials. The waste should be placed directly into designated containers to ensure segregation is maintained.

- Safety Considerations: The handling area for non-hazardous waste should be kept clean and organized to ensure safe handling and quick access to storage or disposal facilities.

- Recyclable Waste Handling

- Personal Protective Equipment (PPE): Employees should wear gloves and any other PPE deemed necessary to handle recyclables safely.

- Sorting: Employees must follow proper sorting instructions to ensure that recyclable materials are separated by type (e.g., plastics, paper, cloth) and are not contaminated by non-recyclable or hazardous materials.

- Handling Procedures: Recyclables should be handled carefully to prevent breakage and must be compacted if needed to reduce volume and prepare them for transportation.

- Organic Waste Handling

- Personal Protective Equipment (PPE): Employees handling organic waste should wear gloves and any other suitable PPE to avoid direct contact with potentially contaminated materials.

- Proper Handling: Organic waste must be handled in a manner that minimizes odor and contamination. Any spillage of organic material should be promptly cleaned up to prevent pests or other health risks.

4.3. Transportation Procedures

Bally Fabs International Limited ensures that hazardous waste is transported with the highest standards of safety, compliance, and traceability. All activities are performed in accordance with the Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016, Motor Vehicles Act, 1988, and the applicable 	State Pollution Control Board (SPCB) guidelines.

- Hazardous Waste Transportation

- Licensed Contractors

- All hazardous waste is transported exclusively by CPCB authorized hazardous waste transport contractors.

- Contractors are required to hold valid transport licenses, vehicle fitness certificates, and personnel training certifications as per Rule 19 of the 2016 Hazardous Waste Rules.

- Service-level agreements ensure contractors adhere to both national and state-level environmental laws and submit documentation before each dispatch.

- Container Sealing and Vehicle Safety

- Pre-Dispatch Inspection:
All hazardous waste containers undergo visual and physical inspection by the EHS Officer to ensure they are undamaged, leak-proof, and compliant with prescribed specifications.

- Container Specifications:

  - Constructed of HDPE or corrosion-resistant steel.

  - Equipped with tamper-evident lids and secure locking systems.

  - Labeled in accordance with Schedule II of the 2016 Rules, including waste category, generation date, and hazard symbols in English.

- Transport Vehicle Safety Features:

  - Equipped with spill containment trays, absorbent lining, and segregation compartments.

  - Clearly marked with “HAZARDOUS WASTE” placards, including appropriate UN hazard codes and color-coded labels.

  - Vehicle body inspected for container stability and tie-down security prior to dispatch.

- Documentation and Regulatory Compliance

Each shipment is accompanied by a full set of documentation as required under Rule 19 of the Hazardous Waste Rules and SPCB SOPs:

- Form 10 – Hazardous Waste Manifest:

  - Contains generator details, transporter information, quantity, physical state, and final disposal location

  - Signed by the generator (Bally Fabs International Limited), transporter, and TSDF upon completion

- Transport Authorization Certificate:

  - Verified copy of contractor’s waste transport approval issued by the SPCB

- Safety Data Sheets (SDS):

  - Specific to the hazardous material being transported

  - Includes chemical composition, physical properties, hazards, first aid measures, and firefighting precautions

- Emergency Preparedness and Response

Transport vehicles are equipped and staffed to handle emergency situations involving hazardous waste.

- On-Board Emergency Kit Includes:

  - Absorbent pads, neutralizing agents, spill containment socks, waste collection bags

  - Complete set of PPE: gloves, masks, aprons, goggles, face shields

  - Fire extinguisher and first-aid kit

- Personnel Training:

  - Drivers and accompanying staff are trained in:

    - Classification and handling of hazardous waste under Indian rules

    - On-site spill containment, evacuation procedures, and safe zone establishment

    - Immediate communication protocols with SPCB, local authorities, and emergency responders

    - First aid and hazard-specific response techniques

- Emergency Plan Adherence:

  - Transport staff must follow Bally Fabs International Limited’ Emergency Response Plan (ERP) including:

    - Rapid isolation of spill zone

    - Notification to Environmental Compliance Officer and SPCB

    - Activation of third-party emergency cleanup services, if necessary

- Transportation Execution and Post-Delivery Verification

- Route Planning and Pre-Dispatch Checks:

  - Transported via pre-approved, low-risk routes identified to minimize environmental and human health risks

  - Vehicles undergo pre-departure inspections for:

    - Container integrity

    - Proper label display

    - Documentation readiness

    - Secure cargo placement

- Segregation of Incompatibles:

  - Flammable, reactive, or corrosive substances are stored separately within the transport vehicle, using CPCB-compliant segregation barriers

- Delivery and Reconciliation at TSDF:

  - On reaching the authorized Treatment, Storage, and Disposal Facility (TSDF):

    - Waste is unloaded under supervision

    - Quantity and type are verified against manifest entries

    - Any discrepancies are documented, reported, and investigated

    - Form 10 is updated, signed, and one copy is returned to Bally Fabs International Limited for records

- Non-Hazardous Waste Transportation

- Transportation Vehicles: Non-hazardous waste is transported using regular vehicles designed to carry bulk waste. These vehicles should be equipped with secure and leak-proof containers to prevent contamination during transit.

- Route Planning: Non-hazardous waste should be transported directly to disposal or recycling facilities without unnecessary detours to minimize time and risk of spillage.

- Recyclable Waste Transportation

- Designated Vehicles: Recyclable materials should be transported in vehicles dedicated to recycling operations to ensure they are not contaminated by other waste streams.

- Proper Sealing: Recyclable materials should be securely stored in containers to prevent leakage, breakage, or contamination during transportation.

- Organic Waste Transportation

- Transport Containers: Organic waste should be transported in sealed, vented containers to prevent odors and exposure to contaminants. Containers should be easy to unload at composting or organic recycling facilities.

- Transportation Vehicles: Organic waste is transported using trucks or vehicles designed for organic waste, with a focus on maintaining a hygienic and controlled environment during transit.

- Efficient Treatment & Disposal Systems

- Hazardous Waste Treatment and Disposal

Bally Fabs International Limited is committed to minimizing the environmental and human health impacts of hazardous substances generated during production through the implementation of specialized treatment protocols and safe disposal systems. These procedures are aligned with the Hazardous and Other Wastes (Management and Transboundary Movement) Rules, 2016, the Environment (Protection) Act, 1986, and State Pollution Control Board (SPCB) directives.

1. Specialized In-House Treatment Techniques

Hazardous waste generated during manufacturing is treated on-site using tailored methods, depending on its type and hazard level:

- Neutralization Units:
Used to treat acidic or alkaline wastewater and chemical residues, ensuring safe pH levels before storage or further processing.

- Sludge Solidification:
Sludge containing chemicals is stabilized using binding agents (like fly ash or lime) to convert it into a dry, non-leachable form suitable for secure landfilling.

- Solvent Recovery and Segregation:
High-value spent solvents are separated and stored for off-site recovery, reducing environmental load and enabling circular use.

- Container Cleaning Units:
Used chemical drums are triple-rinsed and stored safely for return to suppliers or scrap recovery vendors.

- Contaminated PPE Handling:
Soiled gloves, rags, and aprons are collected in labelled containers and prepared for co-processing or incineration.

All treatment processes are managed by trained EHS personnel with regular monitoring to ensure effectiveness and safety.

2. Safe and Compliant Disposal

After treatment, hazardous waste is dispatched to CPCB/SPCB-authorized Treatment, Storage, and Disposal Facilities (TSDFs):

| Waste Type | Specialized Treatment | Final Disposal Method |
| --- | --- | --- |
| Spent Solvents | Segregated for recovery | Incineration or recovery units |
| Contaminated PPE | Collected & containerized | Co-processing in cement kilns |
| Discarded Chemicals/Drums | Rinsed and sorted | Returned or recycled |
| E-waste (batteries, tools) | Segregated and logged | E-waste dismantling facilities |

3. Documentation and Monitoring

- Form 10 (Manifest): Tracks each shipment from source to disposal

- Disposal Certificates and SDS records are maintained for each waste category

- Annual Form IV submitted to SPCB as per compliance norms

- Monthly waste reconciliation conducted to verify treatment vs. disposal ratios

Bally Fabs International Limited’ integration of specialized treatment techniques with regulatory-compliant disposal helps reduce long-term environmental impacts and supports its broader commitment to responsible production and chemical safety.

- Non-Hazardous Waste Disposal

- Municipal or Approved Landfills: Non-hazardous waste, if not reused or recycled, is disposed of via local municipal waste systems or sent to government-approved landfills, in accordance with local environmental guidelines.

- Recordkeeping: Disposal quantities and routes are recorded and retained to ensure traceability and compliance.

- Organic Waste Disposal

- Composting & Organic Processing Units: Organic waste is sent to authorized composting sites or organic recycling facilities where it is transformed into compost or energy-rich material. This supports soil health and reduces environmental burden.

- Documentation: Disposal volumes and destinations are recorded to maintain transparency and support sustainability reporting.

Review Mechanism

The Waste Management SOP is reviewed annually by Bally Fabs International Limited Environmental Manager to ensure compliance with regulations and evaluate progress on waste reduction. Feedback from employees, department heads, and waste contractors is used to assess effectiveness. If gaps are identified, corrective actions such as procedure updates, staff training, or changes in disposal methods are implemented. All review outcomes are documented, communicated to relevant teams, and followed by regular audits to maintain compliance and support continuous improvement.

## 2. Material Management Procedures

Overview
This Material Management Procedures outlines a structured approach to managing materials in production. It defines procedures for efficient and sustainable procurement, storage, handling, and utilization of raw materials. The SOP focuses on optimizing material consumption, minimizing waste, and enhancing operational efficiency while ensuring compliance with environmental, health, and safety regulations. It supports consistent and sustainable production practices across all departments.

Purpose
The purpose of this SOP is to improve material efficiency and sustainability in product manufacturing. It sets clear guidelines to reduce material wastage, enhance resource optimization, and limit the environmental impact associated with material handling and disposal.

Scope
This SOP applies to all stages of material management at Bally Fabs International Limited, including procurement, storage, handling, usage, and reuse or recycling. It covers all the raw materials used such as jute fabric, blended textiles, dyes, auxiliaries, and packaging materials. The SOP ensures efficient material use, waste reduction, and compliance with environmental standards and sustainability goals.

Procedure

- Material Consumption Reduction by process optimization:

- Process Innovation for Precise Material Use

    - All dyes, auxiliaries, and finishing agents are dispensed using automated dosing systems to ensure accurate quantities based on batch requirements, preventing overuse or underuse of materials.

    - During dyeing, washing, and finishing operations, process parameters such as temperature, time, and chemical concentration are closely controlled through advanced monitoring systems to optimize material usage and minimize wastage.

    - Continuous monitoring of production data is conducted allowing the operators to make precise adjustments in formulations and process conditions in real time, maintaining consistent product quality while reducing excess consumption of chemicals and water.

    - Equipment and recycling systems are routinely inspected and maintained to ensure proper functioning, consistent output, and adherence to efficiency and sustainability standards.

- Reintegration of Recovered Input Materials

    - Residual water, chemicals, sizing agents and dyes from previous batches are systematically collected, filtered, tested and reintegrated into the production process wherever feasible, reducing dependency on virgin materials and supporting sustainability goals.

    - The collected residues are passed through filtration units to remove impurities and treated as per standard chemical recovery protocols to ensure quality and safety for reuse.

    - Treated materials undergo quality testing to verify concentration, composition, and suitability for reuse in non-critical production processes.

    - Approved recovered materials are reintroduced into subsequent production batches in controlled quantities to replace up to 25% of virgin raw materials.

    - All recovery and reuse activities are recorded in material usage logs, including batch details, recovered volume, and percentage of reuse.

    - The EHS and Production teams jointly review recovery performance monthly to ensure efficiency, cost savings, and compliance with sustainability objectives.

- Enhanced Production Planning and Process Scheduling

    - Production batches are scheduled based on accurate demand forecasts and available machine capacity to ensure optimal utilization of resources and avoid overproduction or excess material use.

    - Scheduling is designed to reduce frequent machine changeovers, thereby minimizing the consumption of cleaning agents, purge materials, and water used between batches.

    - Similar color and material types are sequenced consecutively to reduce the need for intensive washing, re-dyeing, or equipment cleaning between runs.

    - By aligning batch size with operational capacity, idle machine time and material wastage are minimized, improving overall production efficiency.

    - Production schedules are periodically reviewed and adjusted based on actual performance data and material consumption trends to enhance planning accuracy and sustain material efficiency.

- Use of Eco-Friendly and Biodegradable Raw materials

Ballyfabs International Limited follows a focused strategy to minimize production waste and reduce environmental impact by integrating eco-friendly and biodegradable raw materials throughout its manufacturing operations. The following practices are implemented across all units:

- Biodegradable Fiber Inputs: Ballyfabs International Limited primarily uses organic cotton and cellulosic fibers (e.g., TENCEL™, modal) that decompose naturally without leaving toxic residues, reducing the volume of post-production textile waste.

- Low-Toxicity Chemical Inputs: Finishing agents and auxiliaries used during processing are biodegradable and free from heavy metals, APEOs, and formaldehyde, minimizing the chemical load in wastewater.

- Low-Impact Dyes: Only low-impact reactive and pigment dyes that have high fixation rates and generate minimal unfixed residues are used, thereby lowering color-related effluent discharge and reducing dye-related waste.

- Water-Based Processing Chemicals: Wherever applicable, petroleum-based solvents and resins have been replaced with water-based alternatives (such as water-based prints and finishes), which are easier to treat in wastewater systems and do not contribute to VOC emissions or toxic residues.

- Certified Inputs Only: All inputs used in production are vetted for certifications like GOTS, OEKO-TEX Standard 100, or ZDHC MRSL compliance, ensuring they are biodegradable, non-toxic, and environmentally responsible.

- Vendor Qualification and Re-Assessment: Suppliers are approved only after documentation and samples verify the biodegradability and chemical safety of materials. Vendor compliance is re-evaluated every quarter to phase out materials that may have waste-intensive profiles.

- Adoption of Safer alternative & less-hazardous Substances

- Identification of Hazardous Inputs
A quarterly review of all chemicals used in pretreatment, dyeing, printing, finishing, and washing is conducted to identify substances listed in the ZDHC MRSL, Annex XVII, and BIS safety lists.

- Substitution with Greener Alternatives

  - Solvents & Stain Removers: Petroleum-based stain removers have been replaced with water-based citrus terpenes (e.g., d-Limonene), which are biodegradable, low-toxicity, and safe for routine cleaning and stain removal operation.

  - Surfactants: Conventional synthetic surfactants are substituted with plant-derived APG surfactants (Alkyl Polyglucosides), which are biodegradable, non-ionic, and approved under GOTS for textile wet-processing.

  - Bleaching Agents: Chlorine-based bleaches are fully phased out and replaced with hydrogen peroxide activated by enzyme catalysts (e.g., catalase-assisted peroxide bleaching), reducing chemical load and improving worker safety.

  - Dyes: Azo and heavy-metal-based dyes are replaced with herbal dye extracts (turmeric extract-based yellow dye), which are non-toxic, biodegradable, and suitable for eco-friendly jute coloration.

  - Softeners: Synthetic cationic softeners have been substituted with palm- and soybean-based alternatives that are fully biodegradable.

- Pre-Approval and Testing
All alternative chemicals undergo compatibility testing with fabric types and process conditions before approval. Only inputs with proven performance and compliance certifications (e.g., OEKO-TEX, GOTS) are permitted for production use. Production staff are trained during implementation of each substitution.

- Monitoring and Recordkeeping
Effluent discharge data, chemical consumption records, and supplier MSDS are reviewed monthly to ensure substituted inputs are in continuous use and meet environmental benchmarks. Audit trails are maintained for verification.

- Reducing Hazardous By-Product Formation and Toxicity

Bally Fabs International Limited has implemented a series of proactive measures to limit the generation of hazardous by-products during production processes and reduce their impact on the environment:

- Conventional dyes and auxiliaries have been replaced with water-based alternatives that significantly reduce emissions of volatile organic compounds (VOCs), heavy metals, and Persistent Organic Pollutants (POPs).

- Process conditions such as temperature and pH are carefully controlled to prevent Persistent Organic Pollutants (POPs) formation.

- Advanced filtration and chemical recovery systems are installed to capture and reclaim residual substances before they enter the effluent stream.

- All hazardous wastes are treated and disposed of through government authorized facilities, ensuring full compliance with applicable national regulations.

- Routine audits of chemical usage and waste management practices are carried out to ensure continued compliance and to identify opportunities for improvement.

- Efficient Inventory Management and Material Tracking

A centralized, digitalized inventory management system is used to:

- Monitor real-time data on incoming, stored, and consumed materials.

- Prevent overstocking, expiry-related waste, and inefficiencies in storage.

- Ensure proper stock rotation using the First-In-First-Out (FIFO) principle.

- Provide usage forecasts to assist in precise procurement planning and minimize surplus stock.

- Enable tracking of eco-friendly and recovered inputs separately for performance evaluation and reporting.

- Training and Awareness on Sustainable Material Practices

To ensure consistent and knowledgeable implementation of sustainable practices:

- Training programs are conducted periodically for employees across departments, focusing on safe chemical handling, material conservation techniques, and proper dosing protocols.

- New employees receive induction training on environmentally responsible practices during onboarding.

- Awareness campaigns are held to introduce new sustainable or safer materials and reinforce the importance of minimizing chemical waste and maximizing resource efficiency.

- Training outcomes are measured through periodic assessments and refresher workshops.

Responsibilities

- Material Management Team: Responsible for overseeing the implementation of the SOP, maintaining accurate material usage records, ensuring efficient consumption, and supervising the reintegration of recovered input materials.

- Production Supervisors: Ensure strict compliance with material mixing, dosing, and chemical handling protocols at the operational level, as per defined process parameters.

- Sustainability Manager: Tracks usage of eco-friendly and safer chemical alternatives, monitors progress against environmental targets, and ensures alignment with regulatory frameworks and certifications.

Review Mechanism

The SOP is reviewed annually by the Sustainability Manager to evaluate the progress made in reducing hazardous material usage and enhancing the integration of eco-friendly alternatives. The review also involves analysing trends in material consumption and waste generation to identify opportunities for further optimization. In addition, the Sustainability Manager assesses supplier performance and the effectiveness of the existing vendor selection criteria. Based on the findings, recommendations for process improvements and policy updates are submitted to senior management. The review process is carried out in close coordination with internal audit and compliance teams to ensure timely implementation of any required corrective actions.

## 3. Hazardous Substances Management and Reduction

Overview

This SOP establishes the standardized system followed for labelling, storing, handling, transporting, and reducing hazardous substances used. It ensures safe operations, regulatory compliance, and minimization of environmental and health risks.

Purpose

To define clear procedures for hazard identification, labelling, safe storage, controlled handling, secure transport, and compliant disposal of hazardous substances, while reducing their use and toxicity wherever feasible.

Scope

Applicable to all locations where hazardous chemicals, solvents, adhesives, paints, and related materials are procured, stored, moved, handled, or disposed of.

Procedure

1. Hazard Identification, Labeling & Inventory Control

- All incoming materials are evaluated using Safety Data Sheets (SDS), GHS hazard symbols, and composition.

- Each hazardous material is labeled immediately upon receipt with:

  - Product name

  - Hazard classification (GHS pictograms)

  - Lot/batch number

  - Expiry date

  - Storage location

- Labels must remain clear, legible, and intact throughout the material’s lifecycle.

- Every item is barcode-tagged and entered into the hazardous material inventory system for traceability.

2. Storage Requirements

- Hazardous substances are stored only in designated, ventilated enclosures fitted with spill trays and fire-resistant structures.

- Incompatible materials are strictly segregated (e.g., acids away from solvents; oxidizers away from combustibles).

- Storage units include:

  - Flame-proof cabinets

  - Secondary containment trays

  - Clearly marked hazard zones

- Materials are arranged using First-Expiry-First-Out (FEFO) rotation.

- Safety Data Sheets are displayed or accessible in all storage rooms.

3. Safe Handling Procedures

- Only trained and authorized personnel may dispense, mix, transfer, or apply hazardous materials.

- Mandatory PPE includes: chemical-resistant gloves, goggles, respirators/masks, and protective clothing.

- Decanting or mixing is carried out in well-ventilated or locally exhausted areas.

- Emergency response equipment—eyewash stations, fire extinguishers, spill kits—is kept within reach.

- Controlled dispensing units and pre-measured packs are used to minimize material wastage and exposure.

4. Transportation & Movement Control

- Internal movement of hazardous materials follows barcode-based logistic tracking for real-time traceability.

- Materials are moved in sealed, labeled containers that prevent spills, leaks, and vapor release.

- Transport trolleys or carriers with secondary spill containment are used for internal transfers.

- External transport is conducted only through vendors compliant with hazardous waste transportation rules.

- Movement logs (issued quantity, recipient, time, and route) are maintained and verified in periodic audits.

5. Reduction of Hazardous Use & Toxicity

- Wherever possible, hazardous substances are substituted with lower-toxicity or eco-labeled materials such as:

  - Water-based paints and adhesives

  - Low-VOC cleaning formulations

- Application teams use standardized low-consumption methods to minimize chemical loading.

- Continuous collaboration between procurement, design, and operations ensures adoption of safer alternatives.

6. Disposal & Regulatory Compliance

- Hazardous waste (used rags, expired chemicals, residue cans, oil-soaked materials) is segregated at source into labeled, leak-proof containers.

- Waste is stored in a designated hazardous waste holding area with spill containment and GHS signage.

- Disposal is carried out exclusively through authorized hazardous waste agencies, following statutory rules.

- Manifests, tracking documents, and certificates of disposal are retained for minimum five years.

Training & Awareness
All handlers, supervisors, and warehouse personnel undergo annual training covering chemical hazard identification, labeling and SDS interpretation, safe handling and storage practices, and spill response procedures. Regular toolbox talks are conducted to reinforce key precautions, refresh awareness, and encourage proactive reporting of unsafe conditions or near-misses.

Review & Monitoring
This SOP is reviewed annually by the HSSE team, or earlier if there are changes in materials, processes, or regulatory requirements. The review covers material usage and movement logs, incident and near-miss records, and findings from internal compliance audits. Any improvements identified during the review are documented and implemented following management approval.

## 4. Efficient Energy Management

Overview

This Efficient Energy Management SOP outlines Ballyfabs International Limited’s structured approach to optimizing energy consumption across all jute manufacturing and processing operations. The strategy is aligned with national and international sustainability goals, incorporating energy-efficient machinery, renewable energy adoption, and waste heat recovery systems to reduce environmental impact.

Additional measures include fuel switching, smart monitoring technologies, and active employee participation to strengthen overall efficiency. This SOP supports Ballyfabs’ broader ESG objectives by ensuring continuous improvement, compliance with energy management standards, and measurable reductions in carbon emissions and operational costs. The following table summarizes the key energy management areas and practices currently implemented.

| Aspect | Description | Location | Method/Technology |
| --- | --- | --- | --- |
| Machine Optimization | Energy-efficient weaving, printing, cutting, stitching, and finishing machines are used to reduce electricity consumption. | Weaving/fabric preparation, printing & embroidery, cutting & stitching, finishing sections | Inverter-based drives, programmable logic controllers (PLCs). |
| Lighting Efficiency | Conventional lighting systems replaced with LED fixtures and smart controls to improve energy efficiency. | Production halls, warehouses, administrative offices | LED lights, motion sensors, daylight sensors, zonal lighting. |
| Waste Heat Utilization | Heat recovered from finishing unit is reused for water preheating and process heating. | Finishing sections | Heat exchangers, insulated pipelines, temperature control systems. |
| Fuel Switching | Transition from diesel-based boilers to cleaner alternatives like biomass for reducing emissions. | Boiler/utility areas | Emission monitoring devices, biomass boilers. |
| Compressed Air Optimization | Efficient compressors and preventive leak detection programs improve energy performance. | Utility/maintenance areas | Variable speed drive (VSD) compressors, energy meters. |
| Transportation and Logistics Efficiency | Sustainable logistics practices reduce energy use and enhance supply chain efficiency. | Dispatch section, transport yard, distribution network | Route optimization software, efficient vehicle fleets, adoption of electric vehicles (EVs). |

#### Purpose

To define a comprehensive energy management procedure that enhances operational efficiency, reduces environmental impact, and supports long-term sustainability across Jute Products manufacturing processes.

#### Scope

This SOP is applicable to all sites, departments and equipment within Bally Fabs manufacturing facilities that consume energy during daily operations.

#### Procedures

The following procedures reflects the established practices implemented to enhance energy efficiency, optimize resource utilization, and minimize environmental impact in our operations.

- Machine Optimization

To enhance energy efficiency and reduce overall electricity consumption, BallyFabs has implemented the following machine-level optimization strategies:

  - Upgrade to Energy-Efficient Equipment

- Ballyfabs production lines, including weaving looms, bag stitching machines, printing, and finishing units, are equipped with modern inverter-based motor drives that adjust speed and torque according to operational load.

- Variable frequency drives (VFDs) help machines run at optimal energy levels, reducing consumption during low-load conditions.

- Older, high-energy-consuming machineries are systematically replaced with BEE-rated or equivalent certified machinery to enhance energy efficiency.

  - Process Automation Using PLCs

- Programmable Logic Controllers (PLCs) are integrated into key production lines, such as automated weaving and finishing machines, to regulate operations based on real-time production requirements.

- Machines automatically start, pause, or stop based on load sensors and production cycle triggers, reducing idle running and standby energy use.

- Sequential operations are interlocked so that subsequent processes only activate when previous stages are complete, ensuring precise control over runtime and energy consumption.

  - Energy Performance Monitoring and Preventive Maintenance

- Critical machinery, including weaving, printing, and finishing units, is connected to energy meters to provide continuous data on energy usage, efficiency, and anomalies.

- The engineering and EHS teams review this data weekly to identify inefficiencies or abnormal consumption patterns.

- Preventive maintenance schedules are guided by energy performance data to:

  - Maintain optimal machine operation.

  - Prevent overheating, mechanical wear, and inefficiency due to misalignment or aging parts.

  - Extend the operational lifespan of machines, reducing the environmental footprint of equipment replacement.

- Lighting Efficiency Measures

Ballyfabs International Limited has upgraded its lighting infrastructure to reduce energy consumption while ensuring safe and well-illuminated workspaces across all operations.

  - LED Lighting Retrofit

- All outdated fluorescent, incandescent, and halogen lighting systems have been replaced with energy-efficient LED fixtures in production areas, bag assembly sections, warehouses, and administrative offices.

- LEDs are selected for high lumen-per-watt efficiency and long lifespan, delivering maximum light output with minimal power consumption.

- This retrofit has led to measurable reductions in lighting-related energy usage and lower maintenance requirements.

  - Motion and Daylight Sensors

- Motion sensors are installed in low-traffic areas such as storage rooms, corridors, and restrooms, ensuring lights are activated only when needed.

- Daylight sensors are installed near windows and skylights to automatically dim or switch off artificial lighting when sufficient natural light is available.

- Combined, these sensors prevent unnecessary energy use during non-operational hours and daylight periods.

  - Zonal and Smart Lighting Controls

- The facility is divided into independent lighting zones based on activity areas and occupancy schedules. For example:

  - Production and bag assembly zones operate on extended schedules.

  - Storage, loading, and utility zones follow reduced lighting cycles.

- Smart control systems use programmable timers and real-time data to dynamically adjust lighting intensity and duration based on:

  - Occupancy sensors

  - Time-of-day settings

  - Natural light availability

- Emergency and exit lighting systems remain exempt from automated controls to ensure continuous safety compliance.

- Renewable Energy Integration

  - Automated Monitoring and Performance Tracking

The installed rooftop solar PV system is equipped with advanced monitoring software that continuously tracks:

- Real-time energy generation and cumulative output.

- System efficiency and panel performance trends.

- Deviations from expected parameters, such as voltage irregularities or sudden output drops.

- Alerts are automatically triggered in the event of system anomalies, enabling prompt diagnostics and corrective measures.

  - Preventive Maintenance and Inspection Schedule

- Solar Panel Cleaning: Carried out at regular intervals (e.g., monthly), depending on local environmental conditions. This ensures removal of dust, dirt, bird droppings, or debris that can obstruct sunlight and reduce efficiency.

- Component Inspection: Routine inspections are conducted for:

- Inverters and Junction Boxes – checked for faults, overheating, or signs of internal wear.

- Cabling and Connectors – examined for physical damage, corrosion, or loose connections.

- Mounting Structures – assessed for structural integrity, rust, and anchoring stability.

  - Fault Response and Corrective Maintenance

- Any deviations or underperformance identified by the monitoring system are immediately reported to the maintenance team.

- On-site diagnostics are carried out to identify issues such as:

  - Malfunctioning inverters or failed modules.

  - Faulty connectors, wiring damage, or panel shading.

  - Identified faults are promptly addressed through repair or component replacement using certified parts, ensuring minimal system downtime.

  - Health, Safety, and Risk Control Measures

- Personal Protective Equipment (PPE): All personnel engaged in maintenance activities are required to wear appropriate PPE, including electrical-grade gloves, helmets, eye protection, non-conductive footwear, and harnesses for rooftop access.

- Lockout/Tagout (LOTO): Strict LOTO procedures are followed before performing maintenance to eliminate the risk of electrical shock or unintended activation.

- Fire Safety Compliance: Electrical installations are maintained in accordance with fire safety norms. Appropriate fire extinguishers (Class C-rated) are readily available near control panels and inverter stations.

  - Documentation, Recordkeeping, and Compliance

- Comprehensive records are maintained covering:

  - Daily, weekly, and monthly energy production data.

  - Scheduled maintenance, cleaning logs, and fault resolutions.

  - System performance trends and deviation reports.

  - Regulatory Compliance:

- Operations align with the Electricity Act, 2003, CEA Safety Regulations, and relevant Bureau of Indian Standards (BIS) for solar installations.

- Periodic internal audits are conducted to ensure continued compliance and system optimization.

- Heat Recovery and Thermal Energy Optimization

To enhance energy efficiency and reduce carbon emissions, Ballyfabs International Limited has implemented a waste heat recovery system within its finishing operations.:

  - Heat Recovery from DG Set

- The diesel generator (DG) set generates substantial heat from engine exhaust gases and the coolant system.

- This heat is captured using Exhaust gas heat exchangers. These exchangers extract thermal energy from the hot exhaust gases.

- The recovered thermal energy is then used for preheating process water and maintaining operational temperatures in production areas.

- Utilizing this recovered heat reduces reliance on external fuel-based heating systems, lowering energy consumption and cutting carbon emissions.

  - Thermal Insulation of Pipelines

- All hot water and steam pipelines connected to DG set heat recovery systems are fitted with high-quality insulation to minimize thermal losses.

- Insulation helps maintain consistent process temperatures, shortens heating cycle times, and improves overall thermal efficiency.

  - Reduced Dependence on External Energy Sources

- By reusing internally generated heat, the system reduces reliance on external heating systems such as electric boilers or fossil-fuel-based heaters.

- This results in:

  - Lower energy consumption

  - Reduced fuel usage

  - A measurable decrease in carbon dioxide emissions

- The initiative directly supports Ballyfabs sustainability and decarbonization goals, aligning with both internal targets and external environmental compliance frameworks.

- Cleaner Fuel Transition and Emissions Control

  - Transition to Cleaner Fuel Sources

- All operational diesel-fired boilers have been replaced with biomass-fired boilers, ensuring the use of cleaner, renewable fuel sources across production and processing areas.

- The biomass fuel consists of sustainable organic materials such as briquettes and agro-residues, which are routinely used to maintain consistent combustion efficiency and minimize environmental impact.

- Biomass-fired boilers are integrated into daily production schedules, providing stable and reliable heat for jute processing and associated manufacturing operations.

- Boiler operations follow optimized fuel usage protocols, which are consistently applied to maximize energy efficiency and maintain process reliability.

- Fuel consumption, emissions, and thermal efficiency are continuously monitored to ensure optimal boiler performance.

- Scheduled audits and routine maintenance are carried out to maintain consistent operation and prevent energy wastage.

- The use of biomass fuel has resulted in measurable reductions in CO₂, SO₂, and particulate matter emissions.

- Improved fuel efficiency contributes to reduced operating costs while sustaining production capacity.

- These practices support Ballyfabs’ ongoing decarbonization and sustainability initiatives.

  - Installation of Emission Monitoring Systems

- All new combustion systems are equipped with continuous emission monitoring systems (CEMS) that provide real-time data on:

  - Stack emissions (CO₂, NOₓ, SOₓ, and particulate matter)

  - Combustion temperature and efficiency

- These systems allow the engineering team to:

  - Adjust air-to-fuel ratios dynamically for clean combustion

  - Detect abnormal pollutant levels early

  - Maintain compliance with national and state-level pollution control norms

  - Performance Audits and Operational Oversight

- Routine energy and emissions audits are carried out quarterly by the EHS and engineering teams to:

  - Evaluate combustion performance

  - Identify maintenance needs

  - Recommend tuning or retrofitting where efficiency drops are observed

- All findings are documented in boiler performance logs and used to update the facility’s energy management action plan.

- Compressed Air Optimization

- Energy-efficient compressors, equipped with Variable Speed Drives (VSDs), have replaced traditional models, significantly reducing electricity consumption, especially during off-peak load periods.

- An air leak detection program is actively implemented, identifying and sealing leaks within the compressed air system to enhance overall system efficiency and reduce wastage.

- Usage patterns are monitored and logged through sub-meters, with detailed monthly reviews to identify trends and drive continuous operational improvements.

- Carbon Audits:

Carbon Footprint Calculation:

Ballyfabs International Limited undertakes comprehensive carbon footprint assessments to quantify and manage its greenhouse gas (GHG) emissions. The calculation process follows internationally recognized standards, primarily the Greenhouse Gas (GHG) Protocol, ensuring consistency, accuracy, and global comparability. Emissions are systematically categorized into:

  - Scope 1 – Direct emissions from sources owned or controlled by the company, such as emissions from boilers, generators, and manufacturing equipment.

  - Scope 2 – Indirect emissions from the consumption of purchased electricity, steam, or other forms of imported energy used in production and administrative operations.

  - Scope 3 – Other indirect emissions that occur in the value chain, including emissions from upstream activities (e.g., raw material transportation, supplier emissions) and downstream activities (e.g., product distribution, waste disposal, and employee commuting).

To support this process, Ballyfabs International Limited maintains energy consumption data, production volumes, transport logs, and supplier information. These data points are used to calculate CO₂-equivalent emissions across all scopes. Regular third-party carbon audits are performed to monitor emissions trends over time, assess progress toward emission reduction targets, and identify opportunities for further energy efficiency or cleaner technologies. These audits also aid in maintaining regulatory compliance, improving environmental transparency, and guiding sustainability initiatives across operations.

By embedding carbon footprint calculations into its environmental management system, Ballyfabs International Limited strengthens its strategic approach to sustainability and reaffirms its commitment to mitigating climate change.

- Energy Audits

To systematically assess and enhance energy efficiency across all the operations regular energy audits are conducted, focusing on identifying conservation opportunities and improving system performance.

- Planning and Preparation

- The energy audit is scheduled once in a three-year block, in coordination with an authorized third-party energy audit firm.

- Key energy systems and equipment at Ballyfabs International, including reactors, distillation units, thermal fluid heaters, process heaters, heat economizers, expansion tanks, pump skids, conveyors, mixers, air compressors, chillers, pumps, motors, and lighting systems, are identified in advance for detailed evaluation to optimize energy efficiency and support the company’s sustainability initiatives.

- The operation and maintenance teams are notified about the schedule and their roles in supporting data sharing, equipment access, and on-ground coordination.

- Historical energy data, including electricity bills, operational logs, and equipment specifications, are compiled to establish baseline performance.

- Data Collection and Measurement

- Real-time parameters such as voltage, current, power factor, temperature, and load are measured using calibrated instruments.

- Energy consumption records are collected for a period of 12 months to analyze trends in demand, unit consumption, and billing charges.

- Details of equipment load, usage hours, operating efficiency, and power system layout are recorded across departments.

- Special focus is given to compressors, chillers, pumps, and motors, where potential inefficiencies or energy loss are commonly observed.

- Performance Analysis

- All measured and recorded data is analyzed to identify energy losses, low-efficiency equipment, and systems with potential for upgrades.

- Equipment such as air compressors, Chillers, pumps, motors and transformers are evaluated for loading efficiency and specific power consumption.

- Power quality assessments, such as harmonics analysis and power factor checks, are conducted to evaluate system stability and penalties.

- Recommendation and Reporting

- The audit findings are compiled into a detailed Energy Audit Report, which includes:

  - Energy Conservation Measures (EnCon)

  - Cost-benefit analysis

  - Estimated savings in kWh and ₹

  - Required investment and payback periods

- Implementation and Follow-up

- All approved recommendations are scheduled for phased implementation.

- The plant team integrates energy-efficient solutions

- Post-implementation monitoring is conducted to compare actual performance against projections.

- A summary of improvements is maintained for regulatory compliance and internal review.

- Transportation and Logistics Efficiency

To reduce greenhouse gas emissions, optimize operational costs, and support sustainable logistics, Ballyfabs has implemented a comprehensive transportation efficiency program:

  - Fleet Modernization

- Legacy diesel-powered vehicles have been systematically phased out and replaced with BS-VI compliant, fuel-efficient commercial vehicles, in line with Indian vehicular emission norms.

- These upgraded vehicles emit significantly lower levels of nitrogen oxides (NOₓ), particulate matter (PM), and hydrocarbons, improving air quality and reducing carbon footprint per kilometres travelled.

- Routine engine diagnostics and emission testing are conducted to ensure ongoing compliance with Central Pollution Control Board (CPCB) standards.

  - Integration of Electric Vehicles (EVs)

- Bally Fabs has introduced electric vehicles (EVs) such as Tata Tigor EV (for mid-loads) and Mahindra Treo (for light-load, short-range deliveries).

- These vehicles are deployed primarily for intra-city transport, particularly around manufacturing hubs and distribution centres.

- The use of EVs contributes to:

  - Zero tailpipe emissions

  - Reduced operating costs per kilometre

  - Eligibility for government incentives under FAME-II

  - Smart Route Optimization

- AI-enabled route optimization software is used to design delivery schedules and dispatch routes that:

  - Minimize vehicle mileage and idle time

  - Reduce traffic-related delays

  - Optimize delivery sequencing based on customer location and load capacity

- The system dynamically adjusts routes in response to real-time traffic data, weather conditions, and vehicle availability.

  - Real-Time Fleet Monitoring and Driver Training

- All vehicles are equipped with GPS tracking and fuel consumption sensors that provide live data on:

  - Vehicle speed and route adherence

  - Engine idling time

  - Fuel usage trends

  - Driving behaviours such as harsh braking or over speeding

- Fleet managers use this data to:

  - Identify inefficiencies and reduce fuel wastage

  - Generate monthly performance reports

  - Conduct driver training programs focused on eco-driving techniques, such as:

    - Optimal gear shifting

    - Smooth acceleration

    - Avoidance of unnecessary idling

Responsibilities

- Sustainability Manager: Monitor energy performance and recommend improvements.

- Maintenance Team: Ensure regular upkeep of energy-efficient equipment.

- Operations Team: Adhere to energy-saving protocols and report inefficiencies.

- Procurement Team: Procure energy-efficient technologies and suppliers.

- Employees: Implement energy-efficient practices in daily tasks.

Review Mechanism
This SOP is reviewed annually by the Sustainability Manager to evaluate its effectiveness in meeting defined energy efficiency targets. Key indicators such as energy consumption, emissions, and machine performance are assessed to identify gaps and areas for enhancement. Findings are shared with the management team and relevant departments. Where required, corrective actions are implemented to further optimize resource use and reduce the facility’s carbon footprint.

## 5. Water Optimization Procedures

Overview

This Water Optimization Procedure sets out the framework adopted by Ballyfabs to achieve efficient water usage and consumption by optimizing water consumption, enhancing conservation, and maximizing recycling efforts across jute product manufacturing operations. It outlines strategies for continuous monitoring, efficient water usage, and integration of advanced water-saving technologies. The production processes are structured to reduce wastewater generation, with treated water being reused across non-critical applications to eliminate discharge. Regular water audits are conducted to evaluate consumption patterns, detect inefficiencies, and improve water use practices. The SOP ensures full compliance with environmental regulations, with routine maintenance of water infrastructure to prevent leaks or unnecessary loss. Performance indicators are reviewed periodically to uphold the water management standards, supporting long-term sustainability and resource conservation goals.

Scope

This SOP applies to all departments and operational units within Ballyfabs’ jute manufacturing facilities, including production lines, finishing units, utilities and water storage infrastructure. It covers procedures for monitoring, conservation, treatment, and reuse/recycling of water. The SOP ensures efficient water usage, minimal waste, and compliance with applicable environmental regulations across all operations.

| Aspect | Description | Location | Key Technology / Practice |
| --- | --- | --- | --- |
| 1. Water Reclamation | Wastewater recovery for reuse. | UF Plant | UF system, flow meters, cleaning cycles. |
| 2. Recycled Water Usage | Reuse in non-potable processes. | Cooling & Utility Areas | Reuse allocation, quality checks. |
| 3. Reduction in Water Use | Cut freshwater demand. | Production & Utilities | Closed loops, leak detection, low-flow nozzles. |
| 4. Rainwater Harvesting | Collect and reuse rainwater. | Rooftops & Tanks | First-flush system, filtration, storage. |
| 5. Water Audit | Review use and efficiency. | Entire Facility | Flow meters, testing, data analysis. |

Procedure

- Water Reclamation

Purpose:
To establish guidelines for the efficient operation and maintenance of the onsite Ultrafiltration (UF) system and the safe transfer of treated water to an external Effluent Treatment Plant (ETP). This ensures wastewater is effectively reclaimed for reuse where possible, reduces load on the external ETP, and minimizes environmental impact in compliance with regulatory standards.

Procedure:

a. Pre-Operation Checks

- The operations, maintenance, and EHS teams conduct daily inspections of the UF system, feed tanks, pumps, valves, and storage tanks before startup.

- Operators check for adequate chemical dosing levels if pre-treatment coagulants or pH adjusters are used.

- Safety officers verify emergency equipment, PPE, and spill containment readiness.

b. Daily Operations

Influent Monitoring

- Flow rate into the UF feed tank is monitored via flow meters.

- Key parameters such as pH, turbidity, TDS, and temperature are recorded to ensure optimal operation.

UF Treatment

- Wastewater is passed through the UF membranes to remove suspended solids, colloidal matter, and pathogens.

- Operators monitor Transmembrane Pressure (TMP) and permeate flow, ensuring performance is within acceptable limits.

- Periodic backwash and chemical cleaning cycles are conducted as per vendor recommendations to maintain efficiency.

Permeate Storage and Reuse

- UF permeate is stored in dedicated, covered tanks for reuse in non-critical processes such as floor cleaning, utility washing, and cooling systems.

- Concentrate is stored temporarily before being transferred to the external ETP according to scheduled dispatch.

Sampling & Testing

- UF permeate is sampled weekly for turbidity, TSS, pH, and COD to verify suitability for reuse.

- Concentrate is sampled prior to transfer to ensure compliance with external ETP acceptance criteria.

c. Non-Compliance Response Procedures

- Any deviation in water quality or system performance is immediately reported to the Environmental Compliance Officer (ECO).

- Corrective actions may include: adjusting chemical dosing, modifying operating parameters, performing membrane cleaning, retraining operators, or increasing monitoring frequency.

- All incidents are documented, and follow-up sampling ensures compliance is restored.

d. Post-Operation Procedures

- Operators clean UF membranes, tanks, pumps, and surrounding areas daily.

- Standard shutdown procedures are followed for the UF system, including isolation of valves, power shutdown, and securing storage tanks.

- Daily logs are maintained for operational data, water quality results, maintenance, and incident records.

e. Routine Maintenance

- UF membranes, pumps, valves, and pre-filters are serviced according to a preventive maintenance schedule:

Maintenance Schedule Table:

| Equipment | Task | Frequency | Responsible | Remarks |
| --- | --- | --- | --- | --- |
| UF Membranes | Chemical cleaning & inspection | Monthly | Maintenance Team | Follow vendor guidelines |
| UF Pumps | Lubrication & performance check | Weekly | Maintenance Team | Monitor abnormal vibrations |
| Pre-treatment Filters | Backwash / replacement | Biweekly | Operations Team | Replace if pressure drop occurs |
| Flow Meters | Calibration | Annually | Maintenance Team | Ensure accuracy |

f. Transfer to External ETP

- UF permeate/concentrate is transferred only through authorized transporters with valid permits.

- Each batch is accompanied by a manifest including: source, volume, water quality parameters, date/time, sender & receiver signatures, and vehicle details.

- External ETP acceptance criteria are strictly followed. Any batch exceeding agreed limits is quarantined and corrected before dispatch.

g. Emergency Procedures

- In case of UF system failure or accidental spillage, operators initiate emergency shutdown and containment procedures.

- ECO is notified immediately; corrective measures are documented and implemented.

- Critical spare parts are kept onsite for urgent replacement, and vendor support is available for technical assistance.

- Recycled Water Usage (Water Reuse)

Purpose:
To establish guidelines for the safe and efficient use of reclaimed water at Ballyfabs, promoting resource conservation, reducing freshwater demand, and minimizing environmental impact across manufacturing operations.

Procedure:

Reclaimed water from the onsite Ultrafiltration (UF) system is allocated to specific non-potable applications to optimize resource use and reduce dependency on external water sources. Typical applications include:

- Cooling Systems: Approximately 75% of the total water required for condenser recirculation is supplied using reclaimed UF water, ensuring efficient cooling while minimizing fresh water intake.

- Finishing and Lamination Processes: Nearly 60% of rinse water demand in finishing and lamination operations is met through reclaimed water, effectively reducing overall wastewater discharge.

- Equipment Cleaning: Around 50% of the water used for high-pressure equipment cleaning is sourced from reclaimed water, ensuring cleaning effectiveness while conserving fresh water resources.

- Utility Water Supply: Approximately 40% of utility water requirements—including washing of non-product-contact surfaces and general utility functions—is fulfilled using reclaimed water.

- Floor and Facility Cleaning: Nearly 45% of the water used for routine floor cleaning and facility hygiene is derived from reclaimed water, enabling effective housekeeping with a lower environmental footprint.

Reclaimed water from the UF system is allocated according to predefined usage priorities to ensure consistent supply for non-potable applications. Each receiving operation follows prescribed quality checks to confirm suitability before use. Water transfer valves, storage tanks, and pipelines are inspected routinely to prevent contamination and maintain flow efficiency. All reuse volumes are recorded daily in the Water Reuse Log Sheet, and deviations such as low permeate quality or reduced recovery are escalated to the Utility Supervisor for corrective action.

a. Record Keeping

- Operators maintain daily logs of reclaimed water volumes used, storage tank levels, and operational data.

- Water quality test results, system maintenance records, and any corrective actions are documented and securely stored for inspection and compliance verification.

b. Training

- All personnel receive comprehensive training on the proper handling, allocation, and monitoring of reclaimed water.

- Refresher sessions are conducted periodically to reinforce compliance with operational standards, safety protocols, and emergency procedures.

- Reduction in Water Consumption

Purpose:
To implement low-water or waterless methods in jute finishing, lamination, and coating to conserve water and reduce environmental impact.

Procedure:

- Water Usage Optimization

- Water-Efficient Washing & Cleaning Practices:

All washing and cleaning tasks are performed in accordance with standardized work instructions designed to ensure effective cleaning with minimal water use. These instructions specify:

- Required Water Volume: Predefined litres per task to prevent overconsumption.

- Nozzle Type & Spray Pattern: Use of high-pressure, low-flow nozzles and controlled-spray systems for optimal coverage.

- Cleaning Sequence: A structured top-to-bottom process, moving from cleaner to dirtier areas to avoid repeated washing.

- Maximum Duration: Fixed time limits for each cleaning task to maintain consistency and prevent water wastage.

To support these practices, all equipment is maintained routinely to ensure accurate flow control, avoid leakages, and sustain efficiency. Cleaning schedules are designed to balance hygiene needs with water conservation, preventing redundant or unnecessary washing activities. Additionally, all bleaching and surface-cleaning agents used in the facility are low-rinse formulations, enabling effective cleaning without the need for extra rinse cycles or additional water beyond the specified limits. Treated water recovered from the on-site water treatment system is reused for non-potable cleaning activities such as floor washing, equipment exterior cleaning, and housekeeping tasks. The quality of reused water is verified regularly as per internal water-recycling procedures.

- Water-Efficient Dyeing & Production Operations:

We follow an established procedure in which dyeing operations are conducted using low-liquor-ratio machines (1:4–1:6) and closed-loop rinse-recovery systems. As per this procedure, rinse water is consistently captured, filtered, and reused in pre-wash or initial wash cycles, while high-efficiency soft-flow machines are operated to achieve effective dyeing with minimal freshwater use. This procedure is in place and practiced routinely to ensure significant reduction in water consumption and wastewater discharge during the dyeing operations. Operations team continuously monitors water consumption in the dyeing department to identify areas for further optimization. Through periodic evaluations, we ensure that our dyeing processes are as efficient as possible, reducing waste and improving the sustainability of our operations. Comprehensive training is provided to the dyeing staff, ensuring they are aware of water-saving techniques and are equipped to operate water-efficient systems. Regular refresher courses are conducted to keep the team updated on the latest technologies and best practices.

- Monitoring, Review, and Continuous Improvement

Water consumption across all washing, cleaning, and dyeing units is tracked through calibrated flow meters and daily logbooks. Monthly reviews are conducted to:

- Identify deviations in water usage

- Assess performance of rinse-recovery systems

- Highlight equipment or operational inefficiencies

- Recommend corrective and preventive actions

Operator feedback, internal audits, and performance evaluations are used to continually refine water-efficient practices and further reduce dependency on freshwater resources.

- Leak Detection and Repair

- Regular Inspections: Monthly inspections of water pipelines, finishing and lamination machines, overhead tanks, and utility lines are conducted to identify potential leakages.

- Advanced Leak Detection: Critical areas are monitored using digital leak detection sensors and acoustic monitoring devices to detect hidden or slow leaks within pipelines.

- Timely Repairs: All identified leaks are repaired within 24 hours to minimize water loss and prevent operational disruption.

- Documentation: Inspection findings are digitally logged for traceability and trend analysis, supporting predictive maintenance planning.

- Rain Water Harvesting

To reduce dependency on groundwater and promote sustainable water use practices in line with government regulations, Bally Fabs International ltd. has implemented a robust rainwater harvesting system integrated into its facility infrastructure.

- Strategic Rooftop Collection:
Rainwater is collected from the extensive rooftop surfaces of the knitting, dyeing, and administrative buildings. These rooftops have been structurally designed or modified to ensure optimal water flow towards dedicated collection points without interference from exhaust outlets.

- First-Flush Diverter Mechanism:
To ensure clean water collection, the first 2–3 mm of rainfall, which may contain dust, bird droppings, and other rooftop contaminants, is automatically diverted through a first-flush diverter system. This mechanism protects the storage tanks from initial impurities, ensuring better water quality for reuse.

- Storage and Filtration Infrastructure:
post-diversion, the cleaner rainwater is routed through gravel and sand-based filtration units and stored in underground RCC tanks built with protective coatings to prevent leaching or microbial growth. These tanks are fitted with level indicators, overflow protection, and maintenance access points.

- Applications of Harvested Rainwater:
The stored rainwater is used for various non-potable industrial and facility-related purposes, including:

  - Cooling Tower Make-Up Water: Reduces demand on treated fresh water by supplementing cooling needs during high-load operations.

  - Toilet Flushing in Administrative and Production Areas: Helps save large volumes of water used in sanitary operations.

  - Landscape Irrigation: Ensures greener premises and cleaner surroundings without using borewell water.

- Monitoring and Maintenance:
Water quality is periodically monitored for basic parameters such as pH, turbidity, and microbial content, especially for water directed to equipment like cooling towers. The tanks and filters are cleaned as per a bi-monthly maintenance schedule to prevent contamination and ensure system efficiency.

- Water Audit

Purpose

The purpose of conducting a water audit is to systematically assess the facility’s water usage patterns, identify areas of excessive or inefficient consumption, and uncover opportunities for conservation and reuse. The audit aims to enhance water efficiency, reduce operational costs, ensure compliance with environmental regulations, and support long-term sustainability goals by promoting responsible water management across all operations.

Procedure

- Planning the Water Audit

- Define audit objectives, scope, and methodology in alignment with corporate sustainability targets.

- Assign responsibilities and establish a timeline for conducting the audit.

- Collect historical data on water consumption, wastewater generation, recycling efficiency, and previous audit recommendations.

- Develop a checklist of water-consuming activities, including cleaning, equipment cooling, and sanitation, ensuring adherence to water management principles.

- Data Collection

Water Consumption Monitoring:

- Measure water consumption at different points using flow meters, manual readings, and system logs.

- Conduct physical inspections of pipelines, valves, cooling towers, and equipment for leaks, corrosion, and inefficiencies.

- Review the effectiveness of existing water-saving technologies and operational best practices.

- Interview employees to gather insights on daily water usage habits and identify potential areas for improvement.

Wastewater Quality Assessment:

- Wastewater samples are collected from multiple critical points, including production unit discharge outlets, the treatments inlet, each treatment stage, and the final treated effluent. This enables a comprehensive evaluation of contaminant levels across the treatment process.

- Samples are tested for key environmental indicators such as pH, Biochemical Oxygen Demand (BOD), Chemical Oxygen Demand (COD), Total Suspended Solids (TSS), and Ammoniacal Nitrogen. These tests are performed using standard analytical methods to ensure accuracy and consistency.

- By testing the quality of treated effluent used internally (e.g., in gardening, toilet flushing, or cooling systems). Reuse water is routinely monitored to ensure it meets internal specifications and safety parameters.

- Detailed records are maintained for each sampling and testing activity. This includes sample logs, test reports, treatment system observations, and any corrective actions taken in case of non-compliance. These records support internal audits, statutory reporting, and continuous environmental performance improvement.

- Data Analysis and Identification of Inefficiencies

- Compare collected data against industry benchmarks, historical trends, and regulatory limits.

- Identify high-consumption areas and inefficiencies, such as excessive rinse cycles, unnecessary water usage, or system malfunctions.

- Evaluate the potential for integrating advanced water-saving measures, such as closed-loop systems and water-efficient fixtures.

- Identify areas where wastewater treatment can be improved to enhance reuse.

Responsibilities

- Environmental Manager: Ensures overall SOP implementation, regulatory compliance, and performance reporting.

- Maintenance Team: Maintains equipment and pipeline integrity to minimize water loss and prevent leakages.

- Operations Team: Implements water conservation practices during routine operations and monitors consumption.

- Quality Control Team: Verifies that treated water meets internal reuse criteria and external discharge standards.

Review Mechanism

The Environmental Manager conducts annual reviews to assess the effectiveness of water optimization practices. Annual audits are performed to ensure compliance with operational goals and regulatory standards. Performance data, including water usage efficiency and recycling rates, is analyzed, and improvements are proposed based on audit findings. Any necessary adjustments to processes, procedures, or technology are implemented to ensure continuous water resource optimization.

## 6. Air pollution management

Overview

The Air Management Standard Operating Procedure (SOP) outlines the steps taken to ensure effective control, monitoring, and reduction of air emissions in manufacturing processes. The primary focus is to minimize the impact of air pollutants such as particulate matter, volatile organic compounds (VOCs), and gases, in line with regulatory standards. The SOP includes the use of air filtration systems, regular emissions monitoring, maintenance protocols for air handling equipment, and corrective measures for any non-compliance. These actions contribute to maintaining a healthy work environment and reducing the company’s overall environmental footprint.

Purpose

To implement effective strategies and measures to prevent emissions of atmospheric pollutants and other environmental nuisances in jute product manufacturing.

Scope

The scope includes the implementation of measures to control and reduce atmospheric pollutants and other environmental impacts associated with production processes. This includes:

- Installation of air filtration systems, to capture particulate matter.

- Utilization of enclosed systems to reduce emissions.

- Adoption of noise-dampening equipment in high-noise machinery areas and vibration isolation techniques.

- Application of odor-neutralizing agents and ensuring proper ventilation in areas with volatile chemicals.

Procedure

- Emissions Reduction and Monitoring

- Installation of Control Technologies:

Wet Scrubbers at Heat Setting Machines and Boilers:
All heat setting machines and boilers are equipped with high-efficiency wet scrubbers to capture and neutralize sulfur dioxide (SO₂), volatile organic compounds (VOCs), and other acid gases generated during fuel combustion and finishing operations. Exhaust gases pass through an alkaline scrubbing solution, which reacts with acidic components to form neutral salts, ensuring emissions meet regulatory standards.

The scrubbers operate continuously during production, with regular monitoring of flow rates and solution levels to maintain optimal efficiency. Periodic maintenance is performed on pumps, nozzles, and tanks to ensure consistent performance. This system effectively reduces harmful emissions, prevents atmospheric pollution, and supports sustainable manufacturing operations.

Closed-Circuit Recirculation Systems in Boiler Units:
Boiler exhaust systems are equipped with closed-circuit gas recirculation (CCGR) units that continuously capture a portion of the flue gases generated during combustion and redirect them back into the boiler’s combustion chamber. By reintroducing controlled volumes of exhaust gas, the system stabilizes combustion temperatures, reduces peak flame temperatures, and limits the availability of excess oxygen, which in turn minimizes the formation of sulfur oxides (SOx) and nitrogen oxides (NOx).

The recirculated flue gases also promote more complete fuel combustion, improving thermal efficiency and reducing unburnt fuel losses. This process decreases the overall volume of exhaust gases released to the atmosphere, ensuring emissions are within regulatory limits. Regular monitoring and automated control mechanisms maintain optimal recirculation rates based on boiler load and fuel type, while scheduled maintenance ensures valves, ducts, and sensors function efficiently. The system supports operational consistency, energy savings, and reduced environmental impact from boiler operations.

Dust Collectors for PM Control:

High-efficiency dust collectors are installed across finishing, cutting, and material handling sections to control airborne particulate matter (PM) generated during various manufacturing operations. These systems use bag filters combined with pulse-jet cleaning mechanisms, which automatically remove accumulated dust from the filters and maintain consistent airflow and filtration efficiency throughout operations.

The collected dust is directed into sealed containers to prevent leakage or re-entrainment into the workspace. These containers are periodically emptied and transported to authorized waste management facilities, ensuring safe and compliant disposal in accordance with environmental regulations.

Continuous monitoring of system performance, including pressure differentials across filters and airflow rates, ensures that the dust collection units operate at optimal efficiency. Routine inspections and preventive maintenance are carried out on filter media, ducts, and collection bins to prevent blockages, reduce emissions, and maintain a safe and clean working environment.

The implementation of these dust collection systems effectively minimizes worker exposure to airborne particulates, prevents environmental contamination, and ensures compliance with applicable air quality standards.

Monitoring and Maintenance:

- Emission control systems are inspected monthly to ensure effective operation.

- Continuous Emissions Monitoring Systems (CEMS) are in place at emission sources to monitor parameters such as SO₂, NOx, CO, and particulate matter.

- Quarterly air sampling is performed by an accredited third-party laboratory.

Key Parameters Monitored:

| Parameter | Description |
| --- | --- |
| Particulate Matter | Fine particles from processing units |
| Sulfur Dioxide (SO₂) | Released during fuel combustion, contributes to acid rain |
| Nitrogen Oxides (NOx) | Gases from machinery combustion processes |
| Carbon Monoxide (CO) | Produced by incomplete fuel combustion |
| VOCs | Emitted from printing and finishing chemicals |
| Ozone (O₃) | Formed as a secondary pollutant |
| Lead (Pb) | Checked for compliance though rarely generated |

Maintenance and Inspection Schedule:

| Equipment | Inspection Frequency | Responsible Team | Purpose |
| --- | --- | --- | --- |
| Heat Setting Machines | Monthly | Maintenance Team | Verify emission control system performance |
| Generators | Monthly | Maintenance Team | Check combustion efficiency and emissions |
| Exhaust Systems | Quarterly | Environmental Officer | Confirm compliance with air quality norms |
| Adsorbers & Filters | Biannually | Maintenance Team | Prevent VOC leaks and ensure capture |
| Boilers | Monthly | Maintenance & Utilities | Ensure closed-loop system functionality |

All monitoring data are reviewed by the Environmental Manager and Quality Assurance Team for compliance and corrective action.

- Odor Control

- All chemical storage rooms, mixing sections, and printing areas are equipped with sealed enclosures and proper ventilation systems to prevent fugitive emissions of odorous compounds.

- Wet scrubbers and fixed-bed activated carbon filters are installed in exhaust lines to capture and neutralize VOCs, solvent vapors, and odorous gases before discharge. The carbon media is regenerated as per performance monitoring results.

- Ambient odor levels are checked periodically through sensory monitoring and VOC detectors in critical zones. Any odor-related complaints from employees or nearby communities are logged, investigated, and corrective actions are implemented immediately.

- Employees handling chemicals and solvents are trained on proper storage, transfer, and spill management procedures. Regular refresher sessions are conducted on odor control measures and emergency response protocols.

- Noise Control Procedures

- Foam-lined acoustic panels are installed around high-noise equipment such as air compressors, jet dyeing machines, dryers, and sewing clusters to absorb and dampen sound levels.

- All machinery is inspected and maintained routinely to ensure smooth operation and to minimize noise from mechanical wear, loose fittings, or vibrations. Faulty parts are replaced promptly.

- Noise mapping is carried out annually across all production areas during both day and night shifts using calibrated sound level meters. Areas exceeding permissible limits are prioritized for additional controls.

- Work rotation schedules are implemented to reduce prolonged exposure to high-noise zones. Ear protection (earplugs) is provided to employees in designated areas.

- Workers are trained on hearing conservation practices and correct use of PPE. Monitoring results, maintenance logs, and corrective actions are documented and reviewed periodically.

Noise Management Table:

| Equipment | Source of Noise | Inspection Frequency | Responsible Team | Maintenance Action |
| --- | --- | --- | --- | --- |
| Sewing Machines | Vibrating motors | Monthly | Maintenance Team | Replace worn motors |
| Compressors | Leaky valves | Monthly | Maintenance Team | Repair/replace faulty parts |
| Dryers | Fan imbalance | Quarterly | Operations Team | Clean filters and balance fans |
| Exhaust Fans | Vibration | Monthly | Maintenance Team | Check alignment and mounting |

Earplugs are provided in high-noise areas and their usage is mandatory.

- Actions to Avoid Road Congestion

- Delivery and dispatch schedules are planned to avoid peak traffic hours, ensuring smoother transit, reducing time spent in traffic, and minimizing unnecessary fuel consumption and emissions. This also helps in maintaining timely deliveries without adding pressure on city traffic systems.

- Dedicated entry and exit lanes, along with vehicle holding areas, are established within the premises to streamline vehicle movement, reduce waiting times at the gate, and avoid traffic congestion both inside and outside the facility. This organized flow significantly enhances safety and efficiency in loading and unloading operations.

- Use of high-capacity trucks helps reduce the number of trips required, leading to lower overall fuel usage and emissions. By maximizing load capacity per trip, transportation becomes more efficient and cost-effective.

- GPS tracking systems are installed in vehicles to monitor their real-time location, allowing for smarter route planning, quicker responses to unexpected delays, and minimizing idle times. This contributes to reduced fuel wastage and better overall fleet management.

Roles and Responsibilities

- Management: Approves environmental objectives and allocates necessary resources.

- Environmental Manager: Oversees the execution of the SOP, ensures regulatory compliance, and conducts periodic reviews.

- Department Heads: Enforce SOP measures within their departments and ensure that all staff are properly trained.

- Employees: Adhere to SOP practices and promptly report any issues or concerns to their supervisors.

Review Mechanism

This Standard Operating Procedure (SOP) is subject to an annual review, or earlier if a significant environmental incident occurs, to maintain its effectiveness and relevance. The Environmental Manager is responsible for conducting the review process, ensuring that all aspects of the SOP align with the latest national environmental regulations, internal operational changes, and the company’s evolving sustainability goals. The review aims to identify any gaps or areas for improvement, incorporate lessons learned from incidents, and uphold the company’s commitment to continuous environmental performance enhancement.

## 7. Sustainable Packaging management and Optimization

Purpose
The purpose of this SOP is to outline the procedures and responsibilities related to managing packaging materials and processes at Bally Fabs International Limited, ensuring that product quality is preserved, packaging waste is minimized, and all practices comply with applicable regulatory and environmental standards. The procedure highlights how packaging designs and workflows have been optimized to simplify disposal for end users and promote sustainability.

Scope
This procedure applies to all departments involved in the handling, storage, usage, and disposal of packaging materials, including procurement, production, quality assurance, and logistics.

Definitions

- Primary Packaging: Materials in direct contact with product (e.g., individual polybags).

- Secondary Packaging: Materials that enclose primary packaging for grouping or protection (e.g., cardboard cartons).

- Tertiary Packaging: Outer packaging used for transport and bulk handling (e.g., pallets, straps, shrink wraps).

- Eco-friendly Packaging: Packaging materials that are biodegradable, recyclable, or made from recycled content.

- Lightweighting: Reducing the volume or thickness of packaging without affecting its protective function.

- Monomaterial Packaging: Packaging made from a single material to facilitate easier recycling.

Procedure

- Packaging Material Selection

- Packaging materials such as paper wraps, tissue paper, cardboard boxes, and shrink wrap on pallets are selected in accordance with buyer specifications, ensuring that labelling and presentation meet customer expectations.

- Packaging solutions are designed to comply with export requirements while supporting Bally Fabs International Limited sustainability goals, including reducing material use and promoting recyclability.

- Eco-friendly alternatives such as recyclable paper wraps and certified cartons are prioritized where feasible, in alignment with buyer approvals and environmental standards.

- Secondary and tertiary packaging are chosen to ensure product remain protected during handling, storage, and international shipping, with a focus on durability and minimal waste.

- Packaging Optimization

- Lightweighted paper wraps through thickness reduction and size optimization, ensuring product safety during transit without unnecessary material usage. This approach reduces overall packaging waste while maintaining product protection.

- Mono-material packaging, such as using 100% paper-based, is implemented to facilitate easier recycling for end-users and align with buyer expectations for sustainable packaging.

- Standardized packaging dimensions are followed across similar product categories, which helps in efficient stacking, maximizes storage space in cartons and containers, and reduces transportation costs by enabling better loading ratios.

- Clear disposal guidance is printed on both paper wraps and outer cartons, using simple icons and text to help consumers and retailers handle the packaging responsibly, promoting proper recycling and waste segregation.

- Packaging Process

- Products are carefully, tagged, and wrapped in paper-based packaging materials in accordance with customer-specific guidelines, which may include folding styles, tag placement, and branding requirements. This ensures the final presentation meets buyer expectations and protects product during transit.

- Automated systems are utilized for sealing, labelling, and boxing, which helps maintain consistency in packaging quality, reduces manual errors, and improves overall operational efficiency in the packing line.

- In-line quality checks are performed at multiple stages during and after the packing process, including verification of correct product placement, tag accuracy, packaging integrity, and compliance with buyer instructions. Any deviations are immediately flagged and corrected.

- Each carton is clearly labelled with shipment-specific details such as product codes, SKU numbers, quantity, product care instructions, and disposal symbols, ensuring smooth handling during logistics and enabling end-users to manage packaging waste responsibly.

- Waste Management

- All packaging waste is sorted into recyclable, reusable, and non-recyclable streams at designated collection points to support effective waste management.

- Cartons, plastic wraps, hangers, and pallets are inspected and reused within the facility to reduce material consumption and environmental impact.

- Non-recyclable waste is handed over to certified disposal partners, with records maintained to ensure compliance with local regulations.

- Scrap materials from packaging operations are tracked and analyzed regularly to identify reduction opportunities and improve process efficiency.

- Sustainability Measures

- Packaging materials and processes are continuously improved to reduce environmental impact.

- Employees involved in packaging operations receive training on best practices for waste reduction and recycling.

- Routine evaluations and audits ensure packaging aligns with Bally Fabs International Limited sustainability policies and buyer requirements.

Documentation and Records

- Records of incoming packaging material inspections, supplier audits, and waste management logs are maintained for accountability.

- Deviations and corrective actions are documented for future process improvement.

Training

All relevant employees undergo regular training that includes:

- Safe and efficient handling of packaging materials.

- Regulatory and buyer-specific packaging requirements.

- Techniques for reducing packaging waste and improving recyclability.

Responsibilities

- Packaging Team: Ensures proper packaging design, compliance with buyer specifications, and adoption of sustainable materials.

- Procurement Team: Sources cost-effective and environmentally friendly packaging from approved vendors.

- Quality Assurance Team: Verifies packaging materials meet quality and buyer requirements through inspections and tests.

- Production Team: Uses packaging materials efficiently, ensuring minimal wastage during packing operations.

- Logistics and Warehouse Team: Manages the safe transport and storage of products with minimal packaging damage.

Compliance and Review Mechanism

Internal audits are carried out to ensure adherence to this SOP, and any identified non-conformance is promptly addressed through corrective actions. The Packaging Head is responsible for reviewing the SOP on an annual basis, incorporating necessary updates, with final approval obtained from senior management to ensure ongoing relevance and compliance.

# Social Procedures

## 8. Provision of Emergency Equipment

Purpose

To establish a standardized procedure for the identification, provision, inspection, and maintenance of emergency equipment at Bally Fabs International Limited to ensure employee safety and preparedness during emergencies.

Scope

This SOP applies to all employees, non-employee workers, and subcontractors across all operational areas, administrative buildings, and production units of Bally Fabs, covering equipment related to fire safety, first aid, chemical spills, and natural disasters.

Responsibilities

- HSE Department
Ensures all emergency equipment is identified, installed, and maintained as per risk assessments and legal standards.

- Unit Heads / Area Supervisors
Monitor the presence and readiness of emergency equipment in their areas and coordinate with HSE for periodic checks.

- All Employees
Are oriented during induction to the emergency equipment in their work areas and are responsible for immediately reporting any deficiencies.

Emergency Equipment in Place

The following emergency equipment is already installed and routinely maintained at Bally Fabs International Limited:

- Fire Safety Equipment:
ABC and CO₂ fire extinguishers, fire hydrants with hose reels in high-risk zones, fire alarms, smoke detectors, fire blankets, and sand buckets.

- First Aid and Medical Response:
Fully stocked first aid kits in all departments (checked monthly), eye wash stations and emergency showers in chemical zones, and trained first aiders on each shift.

- Spill Control and Containment:
Chemical spill kits near storage/handling areas and secondary containment for hazardous substances.

- Evacuation & Rescue Equipment:
Emergency torches, clearly marked exits with emergency lighting, and a functional PA system tested during drills.

Procedure

  - Equipment Mapping and Placement

- Emergency equipment is deployed following thorough location-specific risk assessments conducted by the HSE Department in coordination with departmental heads.

- Equipment placement considers:

  - Nature of activity (e.g., dyeing, cutting, chemical storage)

  - Density of personnel

  - Proximity to potential hazards (fire, chemicals, machinery)

- Fire extinguishers, spill kits, first aid boxes, and emergency lighting are mounted at eye level, within easy reach, and clearly labelled with glow-in-the-dark signage.

- Each floor and department maintains an Emergency Equipment Layout Map, which is:

  - Displayed at prominent locations such as entrance corridors and evacuation points

  - Updated annually or after any layout/operational changes

- A centralized Emergency Equipment Register is maintained by the HSE team, capturing location, equipment ID, type, and inspection due dates.

  - Inspection and Maintenance

- A monthly inspection schedule is implemented by the HSE Officer, and the inspection checklist includes:

  - Physical condition (corrosion, leakage, damage)

  - Expiry dates (first aid items, fire extinguishers, spill absorbents)

  - Visibility and accessibility (not blocked or relocated)

  - Tagging of inspection date and initials

- Fire extinguishers undergo:

  - Monthly visual inspections

  - Annual servicing and pressure testing by a licensed vendor

  - Replacement if pressure gauge is below standard range or any damage is noted

- Eye wash stations and emergency showers are flow-tested weekly to ensure functionality and cleanliness.

- A preventive maintenance log is reviewed quarterly to plan upcoming servicing and replacements proactively.

- Inspection findings are documented in Form BFIL/EHS/001 and deviations are flagged for immediate corrective action.

  - Restocking and Replacement

- First aid kits are restocked every month by the EHS representative using a standardized checklist that includes sterile gauze, bandages, antiseptic cream, eye drops, burn ointments, painkillers, and gloves.

  - Used or expired items are replaced immediately.

  - Each kit includes a restocking card that logs previous usage and refill date.

- Spill kits are replenished after any usage or at minimum on a quarterly basis.

  - Contents include chemical absorbents, neutralizers, PPE (gloves, goggles), and disposal bags.

- Emergency torches and batteries are checked and replaced semi-annually or sooner if found non-functional during drills.

- All damaged or expired equipment is marked with “DO NOT USE” tape and immediately removed from the active area.

- Disposal of such items, especially chemicals and medical supplies, is carried out as per Hazardous Waste Management Rules and documented with vendor disposal slips.

  - Training and Drills

- All new employees receive mandatory training during induction on:

  - Location and use of fire extinguishers, eye wash stations, and first aid kits

  - Emergency evacuation procedures and emergency contact points

- Refresher training sessions are held every 6 months and include:

  - Live demonstrations by certified trainers

  - Department-wise hands-on practice for operating fire extinguishers and spill kits

- Mock evacuation drills are conducted annually in coordination with local fire and emergency services.

  - Drills are unannounced to simulate real scenarios

  - Includes timed evacuation, equipment use, and headcounts at designated assembly points

- Drill outcomes and training participation are recorded and reviewed during the HSE committee meetings for continuous improvement.

Records Maintained

- Emergency Equipment Inspection Logs (Monthly)

- Maintenance and Servicing Reports (Annual)

- First Aid Kit Restocking Records (Monthly)

- Training and Drill Attendance Sheets (Quarterly)

All documents are retained in digital formats for a minimum of three years.

Review Mechanism

The emergency equipment provision procedure at Bally Fabs International Limited is reviewed by the HSE Manager in coordination with the Internal Safety Committee, with final approval by the Operations Head. Reviews are conducted annually, and after any incidents to ensure equipment effectiveness and compliance. Feedback from drills and employees is incorporated for continuous improvement. All findings are recorded in the Emergency Equipment Review Log and the SOP is updated annually or as needed based on changes or audit results.

## 9. Equipment Safety Inspection

Purpose

To establish a structured and auditable process for regularly inspecting machinery and equipment to ensure:

- Workplace safety for all employees

- Early detection and resolution of mechanical or electrical hazards

- Compliance with applicable legal and client-specific safety standards

- Reduced risk of equipment-related incidents and downtime

Scope

This SOP covers all operational machinery and equipment located in:

- Production areas (e.g., sewing, dyeing, finishing)

- Utility and support systems (e.g., boilers, compressors, dryers)

- Storage and material handling zones

- Electrical and mechanical service areas

Responsibilities

At Bally Fabs International Limited, equipment safety inspections are carried out through coordinated efforts:

- The Safety Officer oversees the entire inspection system, ensures correct use of the Equipment Safety Inspection Audit Form (BFIL/EHS/001), maintains records, and conducts internal audits.

- The Maintenance Team performs scheduled inspections, completes inspection forms, and carries out timely repairs to ensure safe equipment operation.

- Operators conduct daily visual checks before use, report any issues immediately, and follow all safety and training protocols.

- The Engineering Lead supports technical diagnostics, suggests system improvements, and coordinates external equipment audits when needed.

Inspection Frequency & Types

| Frequency | Type of Inspection | Conducted By | Tools/Forms Used |
| --- | --- | --- | --- |
| Daily | Visual Safety Check | Operators | Daily Logbook |
| Weekly | Functional Test | Maintenance | Weekly Checklist (BFIL/EHS/003) |
| Quarterly | Detailed Technical Review | Maintenance/EHS | Equipment Safety Inspection Audit Form (BFIL/EHS/001) |
| Annually | Full Safety Audit | Internal/Third Party | Annual Safety Review Report |

Procedure Overview

- Daily Visual Checks

- Conducted by operators before using machines

- Check for loose guards, fluid leaks, damage, and unsafe wiring

- Faults are logged in the Daily Log and reported

- Weekly Maintenance Checks

- Performed by maintenance team

- Checks include moving parts, switches, gauges, and control systems

- Logged in Weekly Maintenance Form

- Quarterly Safety Inspection

- Involves mechanical, structural, and electrical assessment

- Safety systems such as interlocks, emergency stops, insulation, and motors are inspected

- All findings and actions are recorded in the Equipment Safety Inspection Audit Form (BFIL/EHS/001)

- Annual Equipment Audit

- Comprehensive review by internal team and/or certified third-party

- Includes risk assessment and regulatory conformity checks

- Consolidated into an Annual Equipment Safety Report

Corrective Action Protocol

- Unsafe equipment is tagged “DO NOT USE” and immediately isolated

- All identified risks are logged, prioritized, and repaired as per expert guidance

- Re-inspection is mandatory before recommissioning

Recordkeeping

- All inspection records are maintained digitally for a minimum of five years

- This includes:

  - Inspection type and date

  - Equipment ID and location

  - Findings and corrective actions

  - Inspector’s name and designation

- The Equipment Safety Inspection Audit Form (BFIL/EHS/001) is the primary tool for quarterly inspections and is reviewed during internal audits

Training and Awareness

- All relevant personnel are trained in:

  - Pre-use inspection

  - Safe equipment handling

  - Proper completion of inspection forms

- Refresher training is conducted annually or after machinery upgrades

Review & Continuous Improvement

Inspection Review Meetings

- Monthly: Maintenance and EHS teams review inspection logs and the audit forms

- Quarterly: Trends are analyzed to update inspection strategies or training needs

Annual SOP Review

- EHS Officer leads review with inputs from Maintenance and Engineering

- Revisions made based on:

  - New equipment

  - Audit findings

  - Regulatory or buyer requirements

- Approved changes are communicated and re-trained

CAPA Tracking

- Non-conformities are tracked using a Corrective and Preventive Action Register

- Resolutions and re-verification are documented for closure

Compliance Assurance

This SOP and the associated inspection practices using the Equipment Safety Inspection Audit Form ensure that Bally Fabs International Limited:

- Meets the safety obligations under The Factories Act, 1948, and applicable State rules

- Is aligned with BIS/IS safety standards, buyer audit expectations, and internal safety policies

- Actively prevents equipment-related injuries and operational hazards.

## 10. Workplace Safety Practices

Purpose

To define safe and efficient work procedures for all manufacturing and operational activities at Bally Fabs International Limited, covering jute bag production, printing, inspection, packing, storage, and dispatch.

Scope

Applicable to all employees, contractual staff, temporary workers, drivers, and visitors across all Ballyfabs operational areas, including production, processing, warehousing, or logistics areas.

General Safety Rules (Site-wide)

- Always wear area-specific PPE (safety shoes, gloves, high-visibility vests where required).

- Do not tamper with machine guards.

- Keep work areas clean and obstruction-free, especially in packing and storage zones.

- Report unsafe conditions, near-misses, and spills immediately.

- Follow floor markings, signage, and designated zones (storage, walkways, loading bays).

- Smoking, running in the warehouse, and misuse of equipment are strictly prohibited.

PPE (Aligned to Operations)

- Head Protection: Helmets in loading/unloading or high-rack areas (if applicable).

- Hand Protection: Cut-resistant or general-purpose gloves during material handling and packing.

- Foot Protection: Safety shoes mandatory.

- Eye Protection: Safety glasses where dust or flying particles may occur (inspection, unpacking).

- Body Protection: High-visibility jackets in loading zones and during vehicle movement.

1. Safe Work Practices

1.1 Material Handling & Equipment

- Only trained and authorized personnel may operate material-handling equipment such as pallet jacks, hand trolleys, bale trolleys, or fabric-roll lifters used for jute rolls, finished bags, and packaging materials.

- Conduct pre-use safety checks for equipment such as brakes, wheels, handles, safety locks, and alarms (where applicable).

- Do not exceed load capacity when stacking jute rolls, finished jute bags, or cartons on pallets, racks, or trolleys.

- Use proper lifting techniques—bend knees, keep back straight, hold loads close to the body—to prevent strain when handling jute bundles, jute fabric rolls, and finished goods cartons.

- Use mechanical aids for heavy jute bales or large bulk consignments to avoid manual overexertion.

1.2 Jute Product Storage & Handling

- Store jute rolls, jute fabrics, shopping bags, pouches, and accessories in clearly labelled shelving systems or bin locations.

- Keep different product types segregated (fabrics, stitched bags, printed bags, accessories) to prevent mix-ups or cross-contamination.

- Handle heavy rolls and bulky jute bundles with caution, ensuring proper grip to avoid hand injuries.

- Ensure bins, racks, and storage shelves are not overfilled to prevent collapse or spilling of stored items.

- Maintain dry, well-ventilated storage conditions to prevent moisture absorption by jute materials.

1.3 Packaging & Kitting Operations

- Keep packing tables clean, organized, and free from loose threads, fibres, or packaging scraps.

- Use ergonomic work tables and anti-fatigue mats for prolonged packing of jute bags, pouches, and accessory sets.

- Dispose of packaging waste including paper tags, cardboard, and jute fibre waste only in designated bins.

- Avoid excessive force when sealing cartons, tying bundles, or strapping packages to prevent strain injuries.

- Ensure correct labelling and barcoding of each product type (shopping bag, bag, storage bag, fabric roll) before dispatch.

1.4 Electrical Safety

- Do not use damaged electrical cords, plugs, extension boards, or portable equipment used in packing/dispatch areas.

- Switch off sealing machines, printing touch-up equipment, weighing scales, and stitching station utilities when not in use.

- Report any sparking, burning smell, overheating, or abnormal noise immediately to the Safety Officer.

- Keep all electrical panels unobstructed and ensure dry surroundings, especially in storage zones where jute (an easily combustible natural fibre) is stored.

1.5 Loading & Unloading Activities

- Ensure delivery trucks, vans, and cargo vehicles are completely stationary and secured before loading jute products.

- Do not stand beneath suspended loads during handling of bulk jute cartons or jute bale movements.

- Use wheel stoppers, chocks, and dock levelers where required.

- Maintain a safe distance from moving vehicles, forklifts, or trolleys in dispatch areas.

- Ensure cartons of jute bags are stacked evenly and secured properly to avoid tipping during transport.

1.6 Hazardous Substance Exposure Control

- Ensure safe handling, dispensing, and storage of chemicals used for printing, finishing, cleaning, or adhesive application.

- Maintain up-to-date Material Safety Data Sheets (MSDS) for all chemicals at easily accessible locations.

- Provide chemical-specific PPE such as chemical-resistant gloves, safety goggles, aprons, and face masks whenever handling solvents, dyes, printing inks, or cleaning agents.

- Ensure proper ventilation or local exhaust systems in areas where vapours, fumes, or solvent-based inks are used.

- Label all chemical containers clearly; never store chemicals in unmarked bottles or food containers.

- Store chemicals in designated, well-ventilated storage areas away from heat, ignition sources, and jute materials.

- Follow the trainings provided on safe chemical handling, spill response, and emergency procedures.

- Use spill kits immediately in case of chemical leaks; isolate the area and report to the supervisor.

- Strictly prohibit eating, drinking, or smoking in areas where chemicals are stored or used.

1.7 Slip, Trip & Fall Prevention Measures

- Keep floors dry at all times; immediately clean water, oil, or chemical spills to prevent slips, especially as jute fibres increase floor slipperiness.

- Use anti-slip mats in high-traffic zones such as packing, dispatch, and storage walkways.

- Ensure adequate lighting in jute storage rooms, inspection areas, stairways, loading bays, and corridors.

- Use handrails when ascending/descending stairs or ramps.

- Secure loose cables, ropes, straps, and packaging materials away from pathways.

- Do not climb on jute rolls, pallets, or unstable stacks.

- Use certified ladders or platforms for stacking/unstaking high rack items; ensure stability before use.

- Wear slip-resistant safety shoes in all operational zones.

2. Housekeeping & Hygiene

- Immediately clean loose jute fibres, threads, fabric scraps, and packaging waste to prevent slip hazards.

- Keep emergency exits, fire extinguishers, and aisles clear at all times.

- Maintain cleanliness in resting and eating areas; wash hands before breaks to avoid ingesting jute dust or fibres.

- Dispose of jute waste only in assigned collection bags/bins as part of Ballyfabs’ eco-friendly waste management practices.

3. Health & Wellness

- Inform supervisors immediately if feeling unwell, exhausted, or experiencing difficulty due to dust exposure.

- Take scheduled rest breaks to reduce fatigue during repetitive packing or material-handling tasks.

- Stay hydrated, especially during high-activity periods or hot weather.

- Use dust masks (where required) when handling large quantities of jute fibres or loose jute materials.

Ergonomic & RSI (Repetitive Strain Injury) Prevention

1. Workstation Setup & Posture

- Ensure packing stations, inspection tables, cutting areas, and stitching stations are adjusted to appropriate working height.

- Maintain neutral posture: straight spine, relaxed shoulders, wrists aligned with forearms.

- Use chairs with lumbar support and footrests where seated tasks are required for longer periods. (quality checking, tagging, folding).

2. Task Rotation & Breaks

- Rotate employees between tasks such as folding, tagging, packing, inspection, and handling to reduce continuous strain.

- Take micro-breaks every 30–60 minutes to stretch shoulders, wrists, fingers, and back.

- Encourage short walking breaks for employees performing prolonged standing or sitting tasks.

3. Tool & Equipment Ergonomics

- Use ergonomic hand tools, scissors, tape dispensers, and cutters with cushioned grips to minimize force.

- Keep tools within easy reach to avoid overextension.

- Use magnifying tools & proper lighting for detailed inspection or tagging tasks.

4. Safe Work Practices

- Avoid excessive force while folding thick jute fabrics, cutting multiple layers, or sealing cartons.

- Keep wrists straight and avoid prolonged twisting while performing repetitive packing movements.

- Follow the 20-20-20 eye rule for employees working with detailed quality checks or labelling tasks.

Incident Reporting

- Report all incidents, including minor injuries, falls, fibre-related reactions, spills, or near-misses, to enable root cause analysis. Safety Officer will conduct the investigation and implement corrective measures.

- All incidents will be recorded, reviewed, and discussed in monthly safety committee meetings.

Compliance & Improvement

- Non-compliance with safety rules may result in disciplinary action (warnings, retraining, suspension depending on severity).

- All safety procedures will be reviewed annually or earlier in case of a major incident, audit finding, or statutory update.

- Employees are encouraged to provide suggestions for improvement based on operational observations and safety needs.

Work Activity – Risk & Safety Matrix

| Work Activity | Key Risks | Safety Focus |
| --- | --- | --- |
| Loading & unloading | Crush injuries, vehicle movement | PPE, safe distance, clear communication |
| Packing & Kitting | Cuts, repetitive strain, fibre irritation | Ergonomics, gloves, workstation setup |
| Material Handling | Back/shoulder injuries | Mechanical aids, proper lifting |
| Cutting & Stitching | Cuts, needle injuries, RSI | PPE, guards, ergonomics, training |
| Printing & Finishing | Solvent/dye exposure, burns | PPE, ventilation, MSDS, SOP compliance |
| Electrical Equipment | Shock, fire | Authorized use, inspection |

Review Mechanism

This procedure will be reviewed annually by the Ballyfabs Safety Officer along with department heads to ensure continued suitability. Reviews will also be conducted immediately following any serious incident, audit observation, or regulatory amendment. All corrective and preventive actions will be documented and communicated to employees.

## 11. Employee Evaluation Practices

Purpose
Defines the process for evaluating employee performance at Bally Fabs International Limited, ensuring fair, structured, and transparent assessments aligned with company objectives and individual growth.

Scope
Applicable to all full-time and contractual employees across departments at Bally Fabs International Limited.

Evaluation Components

- Employee Evaluation Form – Assesses role-specific performance and behavioral competencies.

- Individual Career Development Plan (ICDP) – Outlines goals, training needs, and growth paths.

- Stakeholder Interaction Section – For roles with external communications (e.g., clients, suppliers).

- Buyer Performance Evaluation (for Merchandisers and Sales Teams) – Specific evaluation based on effectiveness in handling buyer communications, responsiveness, order management, compliance adherence, relationship building, and satisfaction metrics.

Evaluation Criteria
Rated on a 1–5 scale

Review Timelines

| Review Type | Timing |
| --- | --- |
| Annual Performance Review | January month each year |
| Stakeholder Interaction Review | With annual review or post key events |
| ICDP Review | Every year or role change |
| Mid-Year Review (optional) | July or as required |
| Buyer Performance Evaluation | Annually, or post major buying season/event |

Documentation & Recordkeeping
HR maintains secure records, including:

- Evaluation Forms

- ICDP Documents

- Reviewer & Employee Comments

- Buyer Performance Feedback Form (if applicable)

Outcomes & Follow-Up

| Rating Range | Action Plan |
| --- | --- |
| 1–2 | Performance Improvement Plan (PIP) |
| 3 | Maintain with skill-building opportunities |
| 4–5 | Promotion consideration and incentives |

Review and Continuous Improvement
The HR department will review this SOP annually, incorporating feedback from department heads, employee representatives, and key performance outcomes. Adjustments may be made to reflect evolving business needs, role definitions, and buyer expectations.

## 12. Supplier Selection, Engagement and Management

Purpose

This procedure defines the systematic approach taken by Bally Fabs International Limited to integrate environmental, social, and ethical sustainability into the selection, onboarding, and ongoing management of suppliers. The objective is to ensure responsible sourcing, mitigate ESG risks, and promote continuous sustainability improvement across the supply chain.

Scope

Applies to all suppliers, subcontractors, and service providers involved in materials procurement, manufacturing, packaging, logistics, and support services for Bally Fabs International Limited.

Responsibilities

- Procurement Team: Screens and selects suppliers, embeds sustainability clauses into contracts, and ensures documentation and review.

- EHS Team: Evaluates environmental and social performance; leads capacity-building and engagement programs.

- Compliance Team: Monitors contractual adherence, oversees audits, and manages corrective action plans.

- Suppliers: Must comply with all contractual sustainability requirements and actively participate in improvement initiatives.

Procedure

- Initial Screening of Potential Suppliers

All prospective suppliers must complete a Sustainability Pre-Assessment Questionnaire as part of the onboarding process. This screening assesses the supplier’s current practices in the following areas:

- Energy efficiency and GHG management

- Water and waste handling practices

- Labor rights, occupational safety, and non-discrimination

- Anti-corruption and responsible sourcing

- Commitment to inclusive hiring and supplier diversity

Only those meeting Bally Fabs International Limited’s minimum ESG thresholds advance to the evaluation stage.

- Supplier Diversity Program

Bally Fabs International Limited is committed to fostering an inclusive and equitable supply chain by prioritizing partnerships with diverse suppliers, including:

- Women-owned businesses

- Social enterprises

- Minority-owned or rural enterprises

- Small-scale, community-based suppliers

To ensure measurable progress, annual procurement targets are established to enhance diversity across all supplier tiers. Bally Fabs International Limited provides diverse suppliers with:

- Mentorship and training programs to build commercial and compliance capabilities

- Capacity-building support to meet sustainability and quality requirements

- Enhanced visibility and preference during sourcing and contract award decisions

Additionally, a formal Supplier Diversity Program Report is prepared annually. This report outlines:

- The percentage of procurement spend directed to diverse suppliers

- Progress against set diversity goals

- Outreach and inclusion initiatives undertaken

- Future actions to expand the impact of the program

The report is reviewed by senior management and shared with relevant stakeholders to demonstrate accountability and continuous improvement in promoting supplier diversity.

- Evaluation and Final Selection

Shortlisted suppliers are further assessed based on:

- Historical performance and existing certifications (e.g., GOTS, OEKO-TEX, ISO 14001, SA8000)

- Documented sustainability progress and transparency in reporting

- Willingness to engage in ongoing improvement and alignment with industry best practices

Preference is given to suppliers who demonstrate proactive sustainability leadership.

- Contractual Integration of Sustainability Requirements

Approved suppliers sign a Supplier Code of Conduct and a formal contract containing enforceable sustainability clauses, including:

- Environmental Compliance: Adherence to laws on emissions, effluents, water usage, chemical handling, and waste disposal.

- Labor Rights: Upholding safe, fair, and inclusive working conditions; strict prohibition of child or forced labor.

- Ethical Conduct: Zero tolerance for corruption, bribery, or non-transparent business practices.

- Diversity & Inclusion: Encouragement of gender equality, inclusive hiring, and supplier diversity.

- Capacity Building and Knowledge Sharing

To enable suppliers to meet sustainability expectations, Bally Fabs International Limited facilitates:

- Training sessions on energy efficiency, water conservation, chemical safety (e.g., ZDHC), anti-harassment, anti-discrimination within the workforce and social compliance.

- Distribution of toolkits and guidelines, including sample policies, audit checklists, and reporting templates.

- Industry forums, webinars, and peer-learning workshops in collaboration with buyers and sustainability experts.

- Monitoring, Reporting, and Verification

Suppliers are required to submit regular sustainability reports upon request detailing:

- Energy and water usage

- Emissions and waste management

- Labor practices and compliance certifications

- Incident reports and corrective actions

Bally Fabs International Limited’s EHS and Compliance teams conduct annual on-site audits to verify reported data and assess adherence to ESG expectations. Where non-compliance is observed:

- A Corrective Action Plan (CAPA) is developed with the supplier

- Closure timelines are defined and tracked

- Repeat non-compliance may result in contract suspension or review.

- Supplier Sustainability Assessment and Incentive Program

Each supplier undergoes an Annual Sustainability Assessment that evaluates performance across key ESG categories:

- Carbon footprint and GHG reduction

- Energy and water efficiency

- Ethical sourcing and traceability

- Worker health, safety, and fair treatment

Top-performing suppliers receive:

- Sustainability Awards (e.g., “Best Sustainable Supplier of the Year”)

- Certificates of ESG Excellence

- Financial incentives, preferential contract renewals, or volume increases

- Public recognition in sustainability reports.

Underperforming suppliers are required to undergo additional training and monitored closely until compliance is restored.

- Supplier Grievance Mechanism

Bally Fabs International Limited maintains a formal grievance mechanism that allows suppliers and subcontractors to raise concerns or complaints related but not limited to:

- Unfair treatment or contract disputes

- Labor rights violations

- Environmental or safety concerns

- Business ethics, bribery, or corruption

- Discrimination or retaliation in procurement practices

Submission Channels:
Suppliers may submit grievances through the following secure and confidential channels:

- Designated email: p.gupta@ballyfabs.com

- Supplier portal grievance submission form

- Confidential feedback boxes available during audit visits

Handling Procedure:

- Grievances are logged by the Compliance Team and acknowledged within 3 business days.

- An initial assessment is conducted within 7 days, and full resolution (including investigation or mediation) is targeted within 30 days.

- Serious issues are escalated to the senior management and may involve third-party review.

- All grievances are documented and tracked in a Grievance Log, including actions taken and closure status.

Non-Retaliation Commitment:
Bally Fabs International Limited guarantees that no supplier will face retaliation or reduced business opportunities for raising a grievance in good faith.

Review and Continuous Improvement

This procedure is reviewed annually or when required by the Sustainability officer to ensure:

- Regulatory updates or buyer requirements

- Audit findings and performance trends

- Supplier feedback

Regular supplier consultations, surveys, and engagement meetings are conducted to identify improvement opportunities, promote transparency, and foster long-term collaboration.

# Governance Procedures

Definitions

- Sensitive Transactions: Transactions that carry potential ethical, legal, or reputational risks, including but not limited to gifts, travel, entertainment, charitable donations, political contributions, and significant procurement activities.

- Confidential Information: Any proprietary or sensitive data related to the company, employees, customers, or third-party vendors that is protected from unauthorized access, use, or disclosure. This includes business strategies, financial data, trade secrets, and personal information.

- Third-Party Due Diligence: The process of evaluating the compliance, ethical standing, and risk profile of external vendors, suppliers, and partners before and during business engagements.

- Data Breach: An incident where confidential or sensitive information is accessed, disclosed, or used by unauthorized individuals or systems, potentially leading to harm or financial loss.

- Anti-Bribery Protocols: Company policies and legal requirements designed to prevent the offering, giving, receiving, or soliciting of bribes to influence decisions or secure business advantages.

- Employee Compliance: The adherence of employees to company policies, ethical standards, and legal requirements, particularly concerning sensitive transactions, anti-bribery measures, and data security protocols.

- Incident Reporting: The formal process for notifying the company of any breaches, risks, or irregularities regarding sensitive transactions or data security, which triggers an internal investigation.

## 13. Procedure for Approving of Sensitive Transactions Compliance

Purpose

To establish clear guidelines for the approval and oversight of high-risk or sensitive transactions, including donations, sponsorships, procurement, travel, gifts, and entertainment, ensuring alignment with Bally Fabs International Limited ethical standards, anti-bribery commitments, and compliance requirements.

Procedure

| Sr. No. | Step | Action | Details | Responsible Department | Status/Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Identify Transaction | Determine whether the transaction qualifies as sensitive. | Sensitive transactions include gifts, travel, entertainment, charitable donations, sponsorships, political contributions, and high-value procurement activities. | Initiating Department / Employee | Review transaction details, consult immediate supervisor, and seek initial guidance from the Compliance Officer. Document findings. |
| 2 | Approval Process | Submit transaction details for review and approval. | Contact the Compliance Officer either in person or via the official email: p.gupta@ballyfabs.com. For high-value transactions, a formal approval request must be submitted. | Compliance Officer | Review context, purpose, and risk level. Record approval and retain in the sensitive transaction file. |
| 3 | Documentation | Maintain complete documentation of the transaction. | Documentation should include transaction purpose, involved parties, estimated costs, and written approvals. These must accompany any expense reimbursement requests. | Finance Department | Ensure documents are securely filed and available for internal/external audits. |
| 4 | Ethical Considerations | Conduct a post-approval ethical review. | Verify that the transaction cannot be perceived as influencing business decisions or violating anti-bribery and anti-corruption laws. | Compliance Department | Document ethical evaluation and retain record. |
| 5 | Prohibited Transactions | Validate transaction legality and compliance with internal policies. | Ensure transaction does not involve prohibited items like cash equivalents, personal payments, or violations of local/international laws. | Compliance Department | Confirm and record compliance. Escalate any red flags immediately. |

Annex: Prohibited Transactions

The following types of transactions are strictly prohibited at Bally Fabs International Limited, regardless of value or intent:

| Category | Prohibited Examples |
| --- | --- |
| Cash or Cash Equivalents | Direct cash payments, personal cheques, money orders, or prepaid gift cards to employees, partners, or vendors. |
| Gifts and Entertainment | Lavish or frequent gifts, gifts given during bidding or contract periods, or gifts that may influence decisions. |
| Donations and Sponsorships | Political contributions, donations without documentation, or charitable support to unverified organizations. |
| Travel & Hospitality | Payment for family members’ travel, first-class travel not aligned with policy, luxury accommodations. |
| Procurement | Awards to vendors without due diligence, split purchases to bypass thresholds, or conflicts of interest. |
| Personal Benefits | Any transaction providing personal gain, services, or financial benefit to an employee or associate. |

Note: Violations may result in disciplinary action, including termination or legal action under government laws.

Review & Oversight

The Compliance and Ethics Committee at Bally Fabs International Limited will annually  review approvals for sensitive transactions to ensure adherence to policies, assess risks, and recommend updates as needed.

## 14. Procedure for Confidential Data Handling and Breach Responses

Purpose

This procedure outlines the steps for managing confidential information, including responding to data breaches and ensuring third-party data is protected from unauthorized access or disclosure, in alignment with Bally Fabs International Limited commitment to security and privacy.

Procedure

- Data Classification:
Data must be classified into three categories:

| Types of Data | Details |
| --- | --- |
| Confidential | Highly sensitive data requiring strict access controls, e.g., personal information, financial records. |
| Sensitive | Data that is important to the organization but not classified as confidential, e.g., employee information, operational procedures. |
| Public | Information intended for public consumption, e.g., marketing materials, press releases. |

- Access Control:

- Access rights are granted based on the principle of least privilege (PoLP), ensuring users receive only the access rights necessary for their role.

- Roles and responsibilities are clearly defined, and access is allocated accordingly.

- A quarterly review of access rights will be conducted by the Data Protection Officer, using automated access logs and manual checks to ensure compliance. Any discrepancies will be investigated, corrective actions will be implemented, and follow-up reviews will ensure compliance.

- Data Handling and Storage

- Confidential information is stored securely, such as in locked cabinets or encrypted servers, ensuring regulatory compliance and protection.

- Transmission of confidential data is conducted through secure channels, such as VPNs and encrypted emails.

- All confidential data is encrypted both in transit and at rest, adhering to industry-standard encryption protocols like AES-256.

- Access to storage areas with confidential data is restricted to authorized personnel only, using physical access controls such as keycards.

4. Incident Detection and Reporting

4.1 Identification:

- IT Department: Regular analysis of system logs to identify unusual access patterns, failed login attempts, or other signs of potential data breaches.

- Employees: Employees are responsible for monitoring system logs for abnormalities and should alert the IT department if suspicious activities, such as failed logins or unauthorized access, are detected.

4.2 Reporting:

- Employees must report any suspected data breaches immediately to their supervisor and the IT department.

- An Incident Reporting Form must be completed and submitted to the IT department for documentation.

4.3 Escalation Protocol:

- Depending on the severity of the breach, it should be escalated to management and legal counsel for further action.

5. Incident Response Plan (IRP)

5.1 Containment:

- The IT Security Manager will immediately isolate affected systems to prevent further unauthorized access. This could include disconnecting affected systems from the network, disabling user accounts, or restricting access until the issue is resolved.

5.2 Investigation:

- The Incident Response Team (IRT) will conduct a thorough investigation to understand the cause and scope of the breach. They will collect and analyze relevant logs and evidence, assess vulnerabilities, and document findings. An incident report will be prepared with recommendations for remediation.

5.3 Communication:

- Affected stakeholders will be notified via official email or in person. The communication will explain the nature of the breach, the data affected, and the associated risks. Guidance will include actions to protect their information, such as changing passwords and monitoring for suspicious activities. Updates will be provided regularly.

5.4 Internal Communication:

- Management and the legal team will be kept informed through progress updates, emails, meetings, and incident reports. All developments, decisions, and actions are documented and communicated promptly to ensure alignment with Bally Fabs International Limited policies.

6. Resolution and Follow-Up

6.1 Remediation:

- Corrective actions will be implemented based on the investigation findings. These include revising data access controls, strengthening encryption protocols, updating security policies, retraining employees, and improving incident detection systems to prevent future breaches.

6.2 Post-Incident Review:

- A review meeting will be conducted by the Incident Response Team, led by the IT Security Manager, to assess the effectiveness of the response. The team will suggest improvements to communication protocols, incident response processes, and security monitoring tools.

7. Compliance and Record Keeping

- Documentation Retention: Detailed records of the incident, including timelines, actions taken, and communications, will be retained for at least three years.

- Annual Review: The IT Security Officer will review and update these procedures annually or following a significant incident to ensure they remain effective and compliant with evolving security threats and legal requirements.

## 15. Securing Third-Party Data from Unauthorized Access

Purpose

To protect sensitive information shared with or received from third parties by implementing strict access controls, ensuring privacy, and minimizing the risk of unauthorized disclosure or data breaches.

Procedure

- Data Access Control

- Access Restrictions: Access to third-party sensitive data is restricted to authorized personnel only, based on their job responsibilities. Role-Based Access Control (RBAC) is utilized to assign permissions according to specific roles, ensuring that individuals only access the data necessary for their job functions.

- Access Reviews: Access permissions are reviewed quarterly by the Data Protection Officer to ensure compliance with data protection policies. Any non-compliance or changes in role will trigger a reassessment of access rights.

- Data Encryption

- Encryption during Transmission: All third-party sensitive data transmitted externally is encrypted using secure protocols such as SSL/TLS to protect the data from unauthorized interception.

- Encryption at Rest: Third-party data is encrypted when stored, using industry-standard encryption algorithms like AES-256 to ensure data security even when stored on servers or cloud storage.

- Key Management: Encryption keys are stored securely in an access-controlled environment, with access granted on a need-to-know basis only.

- Secure Data Storage

- Storage Practices: All third-party sensitive data is stored on secure, access-controlled servers, with encryption applied both during storage and transmission. Physical and logical security measures are in place to protect against unauthorized access.

- Access Control Reviews: Regular audits and access control reviews are conducted by the IT Security Team to ensure compliance with internal data protection policies, including reviewing logs for unauthorized access attempts and ensuring encryption standards are met.

- Data Sharing & Transmission

- Secure Sharing: Third-party sensitive data is only shared using secure methods, such as encrypted emails. The identity and authorization of recipients are verified before sharing sensitive data.

- Activity Logging: All instances of data sharing are logged, including the recipient’s details, the type of data shared, and the time and date of the transaction. These logs are reviewed periodically by the Compliance Team for potential security gaps.

- Third-Party Agreements

- Data Protection Agreements (DPA): All third parties handling or accessing sensitive information are required to sign a Data Protection Agreement (DPA). The DPA outlines the third party's obligations, including security measures and timely reporting of any data breaches.

- Contractual Obligations: Contracts with third parties include clauses that mandate the implementation of equivalent security measures and compliance with relevant data protection laws, ensuring that third parties protect the data as diligently as Bally Fabs International Limited.

- Employee Training & Awareness

- Regular Training: Employees receive regular training on data protection policies and practices, emphasizing the importance of safeguarding third-party data. The training includes handling procedures, secure sharing methods, and the potential consequences of a data breach.

- Role-Specific Training: Employees in roles with direct access to sensitive third-party data receive enhanced training, ensuring they understand their specific responsibilities and how to implement security measures effectively.

- Monitoring & Auditing

- Continuous Monitoring: A monitoring system is in place to detect any unauthorized access attempts or unusual access patterns related to third-party data. The IT Security Team conducts real-time monitoring and analysis of network traffic and system logs for potential security incidents.

- Regular Audits: Data access logs, storage practices, and third-party compliance with security measures are audited quarterly. These audits are performed by the Compliance and Audit Team, who investigate any discrepancies and report on findings to senior management.

- Incident Response

- Breach Detection: If a data breach involving third-party information is detected, the Incident Response Team (IRT) follows the established Incident Response Procedure (IRP) to contain and mitigate the breach.

- Notification of Affected Parties: Affected third parties are promptly notified, detailing the breach's nature, the data impacted, potential risks, and the steps taken to resolve the incident. Bally Fabs International Limited ensures full transparency in communications and provides guidance on mitigating further risks.

- Remediation and Follow-Up: Following the breach, the Data Protection Officer leads efforts to implement corrective actions and improve existing security measures. This includes revising access control measures, enhancing encryption practices, and reviewing third-party agreements to ensure similar incidents do not occur in the future.

- Review Mechanism

The review mechanism includes annual review by the Data Protection Officer and relevant teams to verify access, encryption, and compliance, an annual audit to assess overall data protection effectiveness, and immediate reviews triggered by role changes, breaches, or regulatory updates, with findings documented, reported to management, and used to improve procedures and safeguards.

## 16. Stakeholder Consent and Data Processing Retention and Disposal

Purpose

To ensure that data is managed in compliance with stakeholder permissions, with clear retention periods and secure disposal practices that adhere to privacy standards and regulatory requirements.

Procedure

1. Obtaining Consent

- Formal Consent Mechanism: Consent is obtained through formal, documented processes, such as signed consent forms and digital agreements. These forms explicitly outline the purpose of data collection, the intended use of the data, and the rights of stakeholders regarding their data.

- Confirmation Process: After consent is granted, a confirmation email and electronic signature are used to ensure stakeholders have fully understood the terms before providing their consent. This process confirms that the stakeholders are aware of their rights and the data handling practices.

2. Transparency

- Privacy Notice: Stakeholders are provided with clear and concise information about the data collection process, how their data will be used, retention periods, and their rights to access, correct, or delete their data. This is communicated through a Privacy Notice available in both print and digital formats at the time of data collection.

- Rights Information: The Privacy Notice also outlines the stakeholders' rights to access their personal data, request corrections, and request deletion in accordance with applicable data protection laws.

3. Data Retention and Secure Disposal

- Retention Timelines: Each type of data is categorized based on its sensitivity and relevance. Retention timelines are determined based on legal requirements, business needs, or contractual obligations, ensuring compliance with regulatory requirements and industry standards.

| Category | Record Type | Retention Period | Disposal Method | Review Cycle | Responsible Person | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Employee Records | Personnel files | Employment duration + 6 years | Shredding / Digital Wipe | Annual | HR Department | Includes appointment letters, appraisals, resignation/exit records |
|  | Payroll and salary records | 6 years | Shredding / Digital Wipe | Annual | Accounts / HR | Required under Income Tax Act and for internal/external audits |
|  | Training and certification | Employment duration + 2 years | Shredding / Digital Wipe | Annual | Training Officer | Covers skill-building, statutory, and external certifications |
| Financial Records | Tax returns, audit reports | 8 years | Shredding | Biannual | Finance Manager | Per Income Tax Act and Companies Act audit retention guidelines |
|  | Invoices and payment records | 6 years | Shredding | Annual | Accounts Officer | Required for reconciliation, GST compliance, and audits |
| Operational Records | Production logs | 5 years | Shredding | Annual | Production Manager | Daily output reports, rework/defect logs, shift-wise summaries |
|  | Machine maintenance logs | 3 years | Shredding | Annual | Maintenance Head | As per ISO 9001 and safety compliance norms |
| Contracts & Legal | Vendor and client agreements | Contract term + 6 years | Shredding / Digital Wipe | Annual | Commercial / Legal Officer | Includes renewal history, amendments, and termination letters |
|  | NDAs, MOUs | 10 years from date of expiry | Shredding / Digital Wipe | Biannual | Legal Officer | In case of future disputes, audits, or investigations |
| Legal Records | Litigation and legal case files | Case duration + 10 years | Shredding | Biannual | Legal Advisor | For documentation of legal proceedings and outcomes |
|  | Intellectual property records | Life of IP + 6 years | Secure Archival / Shredding | Biannual | Compliance Officer | Patents, trademarks, copyrights, design registrations |
| Health & Safety | Incident/accident investigation reports | 5 years | Shredding | Annual | Safety Officer | For compliance with Factories Act and insurance claims |
|  | Safety training records | 3 years | Shredding | Annual | EHS / Training Officer | Evidence of compliance with safety training standards |
| Environmental | environmental reports | 10 years | Shredding | Biannual | Environmental Officer | For SPCB inspections and sustainability certifications |
|  | Waste disposal/ monitoring data | 5 years | Shredding | Annual | Environmental Officer | Daily logs, manifests, and monthly summaries as per SPCB requirements |

- Tracking Retention Periods: Automated systems, i.e.: Microsoft SQL Server, are used to track the retention period of stored data. The system ensures that data is flagged for review or deletion once the retention period expires.

- Secure Disposal: Once data is no longer required, it is securely deleted following appropriate methods for its sensitivity. For digital data, secure deletion methods, such as data wiping or cryptographic erasure, are employed. For physical records, documents are securely shredded or incinerated in a controlled environment to prevent unauthorized access to discarded data.

- Compliance Checks: The Data Protection Officer (DPO) conducts regular audits to verify that data is being retained or disposed of in accordance with established policies and legal requirements.

4. Monitoring and Audit Controls

- Regular Audits: The Data Protection Officer (DPO) conducts annual audits of data access and handling practices. The audit process includes reviewing access logs, evaluating data handling procedures, interviewing relevant personnel, and assessing compliance with internal policies. Random sampling of data access and handling activities may also be performed to ensure that the company is adhering to standards.

- Continuous Monitoring: Automated monitoring tools, such as an Intrusion Detection System (IDS), are implemented to detect any unauthorized access attempts or anomalies in data access patterns. Continuous monitoring ensures that any potential data security incidents are identified promptly, allowing for swift action to mitigate risks.

Review Mechanism

The DPO will review this SOP annually, or sooner if triggered by regulatory changes, new practices, audits, or incidents, with all updates approved by senior management, communicated to relevant personnel, and recorded in a review log retained for six years.

## 17. Procedure for Employee Compliance and Awareness

Purpose

To ensure that all employees at Bally Fabs International Limited are well-informed, understand, and comply with company policies, cultivating a culture of accountability and awareness regarding relevant regulations and ethical standards.

Procedure

- Mandatory Training Programs

- Onboarding & Annual Training: All employees must complete mandatory training on data protection, confidentiality, and security protocols during onboarding and annually thereafter.

- Training Topics: The training covers company data protection policies, recognizing security threats such as phishing, and immediate actions to take in the event of a data breach.

- Training Materials

- Employee Handbook: Employees are provided with an updated handbook that includes all relevant policies, ensuring they are aligned with current laws and best practices.

- Data Breach Response: Training includes specific instructions on how to report data breaches and the necessary containment measures.

- Awareness Campaigns

- Periodic Campaigns: Awareness campaigns are conducted periodically through newsletters, posters, and meetings, reinforcing the importance of data protection and security protocols.

- Simulated Phishing Exercises: To assess employee awareness, simulated phishing exercises are carried out, followed by feedback and improvements based on performance.

- Reporting Mechanisms

- Non-Retaliatory Reporting: Employees are encouraged to report compliance concerns or suspicious activities in a non-retaliatory environment.

- Clear Reporting Channels: Reporting channels, including the dedicated email: p.gupta@ballyfabs.com  are clearly communicated to all employees.

- Monitoring and Assessment

- Training Participation: Training participation and completion records are maintained to track compliance.

- Regular Assessments: Regular assessments are conducted to evaluate the effectiveness of the training programs. Necessary modifications are made based on feedback and audit findings.

Responsibilities

- Employees: Complete mandatory training on data protection and compliance procedures, and report any data breaches or compliance issues via the designated channels.

- Managers: Ensure team members understand and follow the policies and procedures, regularly reviewing compliance with training and reporting requirements.

- Data Protection Officer (DPO): Oversee the implementation of policies, enforce data protection standards, conduct audits, and serve as the contact point for data protection issues.

- IT Department: Implement necessary safeguards to protect data security, monitor systems for breaches, and assist in root cause analysis.

Compliance and Monitoring

- DPO Audits: The DPO conducts regular audits to monitor adherence to policies and identify areas for improvement.

- Tracking Training: IT Department tracks employee participation in training, with annual completion being mandatory.

Enforcement and Penalties

- Disciplinary Action: Non-compliance may lead to disciplinary actions as determined by the HR Department, including warnings, suspension, or termination depending on the severity of the violation.

Review and Update Procedures

- Annual Review: The Governance Practices SOP is reviewed annually by the DPO to ensure compliance with current laws.

- Emergency Reviews: Changes in regulations or policies trigger emergency reviews, and revisions are communicated to all employees for acknowledgment.

## 18. Anti-Corruption Due Diligence Program on Third Parties

Purpose:
This SOP defines the framework followed by Bally Fabs International Limited for conducting anti-corruption due diligence on third parties, including suppliers, contractors, distributors, agents, and other business partners. The aim is to mitigate corruption-related risks, ensure compliance with anti-bribery laws, and maintain the integrity and reputation of Bally Fabs International Limited.

Scope:
This procedure applies to all third-party engagements initiated or renewed by Bally Fabs International Limited. It covers activities such as procurement, contracting, and partnerships in all geographic regions where the company operates.

Third Parties Include:

- Vendors and suppliers of raw materials and machinery

- Contractors involved in manufacturing processes.

- Agents or intermediaries acting on behalf of the company

- Distributors and channel partners

Procedure:

Step 1: Identification of Third Parties

- Applicability: Any third party intending to establish a business relationship with Bally Fabs International Limited undergoes anti-corruption due diligence.

- Preliminary Assessment: Third parties are identified based on their role, geographic location, and potential risk exposure.

Step 2: Risk Categorization
Third parties are classified into risk categories based on factors such as:

- Nature of services (e.g., agents handling government contracts)

- Location of operation (e.g., high-risk regions with weak anti-corruption enforcement)

- History of compliance violations or legal disputes

Risk Levels:

- Low Risk: Routine suppliers with minimal exposure to corruption risks.

- Medium Risk: Vendors in regions with moderate corruption risks.

- High Risk: Third parties involved in high-value contracts or working in high-risk jurisdictions.

Step 3: Due Diligence Process

- Background Check: A thorough review of the third party is conducted, including:

  - Business registration and licenses

  - Ownership and management structure

  - Previous track record, including compliance history

- Questionnaire: The third party completes an anti-corruption questionnaire, detailing:

  - Their compliance policies and procedures

  - Known conflicts of interest or affiliations with government entities

  - Past involvement in corruption allegations or investigations

- Screening: Third-party databases or screening tools are used to verify:

  - Sanctions, blacklists, or adverse media reports

  - Connections to politically exposed persons (PEPs)

- On-Site Visits: For high-risk third parties, physical audits are conducted to verify compliance with ethical practices.

Step 4: Contractual Safeguards
Anti-corruption clauses are incorporated into all contracts, including:

- Prohibition of bribery, kickbacks, or other unethical practices

- Right to audit the third party’s records and operations

- Termination rights in case of non-compliance

Step 5: Approval Process

- Low Risk: Approved by the procurement team.

- Medium Risk: Approved by the department head and Legal/Compliance team.

- High Risk: Final approval is given by senior management and/or the Chief Compliance Officer (CCO).

Step 6: Monitoring and Review

- Ongoing Monitoring: Automated systems are used to monitor third parties for changes in risk profile or new compliance concerns.

- Regular Audits: Annual audits of high-risk third parties are conducted to ensure adherence to contractual and ethical obligations.

- Training: Periodic anti-corruption training is provided to third parties to reinforce compliance expectations.

Responsibilities:

- Procurement Team: Responsible for ensuring due diligence is conducted before onboarding third parties.

- Legal and Compliance Team: Reviews and approves medium- and high-risk third parties, drafts contractual safeguards, and oversees compliance programs.

- Chief Compliance Officer (CCO): Approves high-risk relationships and ensures the overall effectiveness of the program.

- Third Parties: Must comply with all anti-corruption policies and cooperate with due diligence and audit activities.

Review Mechanism:

This SOP is reviewed annually by the Compliance officer to ensure ongoing relevance. Updates reflect changes in regulatory requirements, company policies, or operational practice. Employees and third parties are notified of any revisions, and additional training is provided when necessary.

## 19. Information Security Due Diligence Program on Third Parties

Purpose

#### This SOP outlines the procedures for conducting information security due diligence on third parties, such as suppliers, contractors, and business partners, to ensure that all third-party engagements meet the company’s information security standards. The objective is to assess the security posture of third parties, mitigate risks associated with data breaches, and ensure the confidentiality, integrity, and availability of company data.

Scope

This procedure applies to all third parties with which Bally Fabs International Limited shares or receives sensitive information, including but not limited to:

- Service providers handling confidential or personal data (e.g., IT providers, data processors).

- Suppliers who have access to systems or infrastructure critical to operations.

- Contractors and agents who may handle internal or customer-related data.

- Partners and distributors who interact with customer or product data.

The SOP applies to third-party relationships in all geographic regions where Bally Fabs International Limited operates.

Procedure

Step 1: Identification of Third Parties

- Applicability: Third parties with access to sensitive or proprietary information (including production data, client information, and employee data) must undergo information security due diligence.

- Initial Assessment: Identify and categorize third parties based on the type of information they will have access to and the potential risk they pose to company information systems.

Step 2: Risk Categorization

Third parties will be classified into risk categories based on:

- The sensitivity of the data they handle (e.g., Personally Identifiable Information, confidential financial data).

- The level of access they have to the company’s systems or infrastructure (e.g., full access vs. limited access).

- The location and regulatory environment in which they operate (e.g., regions with strict data protection laws).

Risk Levels:

- Low Risk: Third parties with minimal or no access to sensitive data or systems.

- Medium Risk: Third parties with restricted access to sensitive data or systems.

- High Risk: Third parties with broad or unrestricted access to sensitive data or critical systems.

Step 3: Due Diligence Process

- Initial Evaluation:

- Review third-party company profiles, including their information security policies and certifications (e.g., ISO 27001).

- Verify their history regarding data breaches, security incidents, and compliance with applicable data protection regulations (e.g., GDPR, CCPA).

- Information Security Questionnaire:

- Require the third party to complete an information security questionnaire that covers:

  - Information handling practices.

  - Security controls (e.g., firewalls, encryption, access management).

  - Incident response procedures.

  - Compliance with industry standards (e.g., GDPR).

- Screening:

  - Conduct a screening for any history of security incidents, data breaches, or compliance failures.

  - Use reputable screening tools or third-party services to verify information.

- On-Site Security Audit (for high-risk third parties):

  - For high-risk third parties, conduct an on-site audit to assess the physical and digital security measures in place.

  - Verify compliance with agreed-upon security protocols and company standards.

Step 4: Contractual Safeguards

- All third-party agreements must include information security clauses that define:

  - Responsibilities for protecting sensitive data.

  - Access control measures and limitations.

  - Rights to audit and inspect security practices.

  - Notification requirements in the event of a data breach.

  - Termination clauses in case of security breaches or failure to meet security standards.

Step 5: Approval Process

- Low Risk: Approved by the procurement or department head based on the initial due diligence.

- Medium Risk: Requires approval from the Information Security Officer (ISO) and Legal/Compliance team.

- High Risk: Final approval by senior management and the Chief Information Security Officer (CISO).

Step 6: Monitoring and Ongoing Assessment

- Conduct annual reviews of third-party information security practices.

- Continuously monitor third-party activities to ensure compliance with security protocols.

- Use automated tools to detect unusual behaviour or potential vulnerabilities in third-party systems.

- Follow up on any changes in the third party’s security practices or infrastructure that may affect data security.

Monitoring and Review

- Continuous Monitoring: Use automated monitoring tools to track third-party access to sensitive data and systems.

- Periodic Audits: Schedule periodic security audits and vulnerability assessments to verify that third-party security measures remain effective.

- Incident Response: If a third-party security breach is identified, the incident response protocol should be activated to mitigate any damage and prevent recurrence.

Compliance

#### This SOP complies with applicable information security standards and legal frameworks, including:

- General Data Protection Regulation (GDPR)

- ISO 27001

#### Non-compliance by third parties may lead to the termination of business relationships and legal consequences.

Responsibilities

- Employees: Ensure that any third-party engagements requiring access to sensitive information follow the appropriate due diligence process.

- Information Security Officer (ISO): Lead the due diligence process for third-party security evaluations, especially for medium- and high-risk vendors.

- Procurement Team: Ensure that information security requirements are incorporated into contracts with third parties.

- Legal and Compliance Team: Review contracts to ensure they meet regulatory and security requirements.

- Chief Information Security Officer (CISO): Oversee the entire due diligence program, especially for high-risk third-party relationships.

- Third Parties: Comply with all agreed-upon security standards and participate in regular security reviews and audits.

Review Mechanism

- This SOP will be reviewed annually by the Information Security officer.

- Changes in regulations or the company’s security needs will be reflected in the SOP to maintain effectiveness.

- Updates will be communicated to relevant stakeholders, and training on the revised SOP will be provided as necessary.

## 20. Audits of Control Procedure to ensure Ethical Compliance

Purpose

This SOP outlines the procedure for conducting internal audits at Bally Fabs International Limited to ensure compliance with ethical standards, integrity controls, and responsible business practices across all departments. The aim is to proactively detect non-compliance, strengthen accountability, and support a culture of transparency and fair conduct.

Scope

Applies to all functional departments at Bally Fabs International Limited, including Finance, Procurement, HR, Marketing, Operations, IT, and Sustainability/Compliance. The procedure also covers controls related to anti-corruption, fair labor practices, responsible sourcing, and financial integrity.

Audit Planning

- Audits are conducted annually, and additionally following any major organizational, structural, or regulatory changes.

- The Internal Audit Team, in coordination with the Compliance Officer, develops an annual audit calendar with defined scope, objectives, timelines, and responsible persons.

Audit Schedule and Scope

| Audit Type | Department/Area | Objectives | Scope | Responsible Persons |
| --- | --- | --- | --- | --- |
| Financial Integrity Audit | Finance | Ensure accuracy of reporting and compliance with internal controls. | Financial records, ledger entries, and financial disclosures. | Audit Team Lead |
| Vendor Ethics & Compliance | Procurement | Verify supplier selection is ethical and fair. | Vendor contracts, tender records, ESG clauses, and sourcing traceability. | Procurement Manager |
| Labor Practices Audit | Human Resources | Assess compliance with labor laws, fair wages, and non-discrimination. | Hiring records, grievance logs, wage data, and exit interviews. | HR Manager |
| Marketing and Communication | Marketing | Validate responsible communication and compliance with branding ethics. | Advertising materials, digital campaigns, and buyer-facing collateral. | Marketing Manager |
| Operations Compliance | Production/Operations | Evaluate adherence to SOPs and ethical work environment standards. | SOP implementation, resource management, and incident logs. | Operations Manager |
| IT and Data Security | Information Technology | Assess systems for secure handling of sensitive and supplier data. | Access controls, cybersecurity protocols, data protection practices. | IT Manager |
| Anti-Corruption and Bribery | Compliance & Leadership | Detect bribery risks and corruption vulnerabilities. | Gifts/hospitality logs, whistleblower cases, Code of Conduct adherence. | Compliance Officer |

Control Procedure Evaluation

- The Audit Team reviews documentation and digital records associated with each area: including contracts, logs, approvals, training records, and incident reports.

- Reviews are conducted against Bally Fabs International Limited Ethical Code of Conduct, Supplier Code, Labor Compliance Framework, and relevant local regulations.

- Interviews with key employees and department leads are conducted to validate processes and uncover hidden gaps or informal practices.

Data Analysis

- Quantitative and qualitative data are analyzed to identify trends, irregularities, and early indicators of non-compliance.

- Internal benchmarks (e.g., labor compliance rate, supplier grievance rate, wage audit compliance %) are used for performance comparison.

- Ethical risk scoring may be applied for departments or suppliers with prior compliance issues.

Reporting of Audit Findings

- A detailed Ethical Compliance Audit Report is prepared after each audit, containing:

  - Summary of observations

  - List of non-conformities

  - Root cause analysis (if applicable)

  - Recommendations for process and control improvement

- Reports are submitted to senior management, and relevant department heads within 10 business days of audit completion.

Follow-Up and Corrective Actions

- A Corrective Action Plan (CAP) is formulated in coordination with each audited department, detailing:

  - Action items

  - Target completion dates

  - Accountable persons

- The Compliance Department schedules follow-up audits or spot checks to verify the implementation and effectiveness of corrective actions.

- Recurring non-compliances are escalated to top management and included in performance evaluations.

Documentation and Recordkeeping

- All audit materials (plans, checklists, reports, CAPs, and follow-up findings) are stored securely, both digitally and in hard copy where needed.

- The Audit Team, in coordination with the Compliance Department, maintains these records for a minimum of 5 years for internal reviews or third-party audits.

Review and Improvement of Audit Mechanism

- This SOP is reviewed annually by Compliance Officer to incorporate changes in legal frameworks, buyer codes of conduct, or Bally Fabs International Limited own ethical standards.

- Lessons learned from each audit cycle are used to update checklists, risk indicators, and training content to improve future compliance.

#### Effective Date: 01.04.2025

#### Next Review Date: 01.04.2026

Approved By: